3GPP TSG RAN WG1 #112bis-e	   	 	  	          R1-2303079

e-Meeting, April 17th – April 26th, 2023

Agenda item:		9.2.3.2

Source:		LG Electronics

Title:			Other aspects on AI/ML for beam management

Document for:	Discussion and Decision

Introduction

In the SID [1], three use cases are included as initial set of use cases for AI/ML for NR air interface.

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Also, in RAN#98-e, 6 sub-use cases including BM-case 1 and 2 were confirmed for the initial set of use case. In this contribution, we discuss on these BM sub-use cases.

Discussion

Assist information for training/inference

In previous meetings, there has been some progress on the assist information for UE-side AI/ML model training/inference as captured below.

Agreement Regarding the data collection for AI/ML model training at UE side, study the potential specification impact considering the following additional aspects. - Whether and how to initiate data collection - Configurations, e.g., configuration related to set A and/or Set B, information on association/mapping of Set A and Set B - Assistance information from Network to UE (If supported) Other aspect(s) is not precluded Conclusion Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information - NW-side beam shape information - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc. - Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion - e.g., some information following the same principle of Rel-17 positioning agreement Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused).

For this issue, we should note that the DL beams of DL resources is totally transparent to UE by current specification. In the case that Set B is a subset of Set A, for example, UE has no idea about the beams included in Set A but not in Set B since there may be no live measurement on those beams and no additional information from NW neither. According to the conclusion made in RAN1#112, on the other hand, providing explicit information about NW-side beam shape to UE is precluded in order to keep vendor-specific proprietary information outside 3GPP. In order to meet these competing demands, we can consider approaches where information of Set A/Set B beams are represented in a logical domain/space which may provide ‘relative’ information between beams. For an example, NW may provide 2D/3D coordinate information about Tx beam boresight direction, e.g. (x1, y1, z1) for RS#a, (x2, y2, z2) for RS#b, where the coordinate axes may be decided by NW implementation (i.e. not necessarily same as global coordinate axes) or may be defined in a logical space so that it does not provide ‘physical’ implementation of beams, rather it is about ‘relative’ Tx beam angle difference of RSs comprising Set A and/or Set B. For another example, Set A beams can be represented as a linear combination of Set B beams, analogous to type-II CSI codebook.

Proposal #1: For the UE AI/ML model training and inference, assist information on relation/association between Set A beams and Set B beams should be provided to UE. To represent beams in Set A and/or Set B while preserving sensitive proprietary information, consider following exemplary methods.

- Set A beams are represented by LC coefficients of Set B beams

- Tx beam directions are represented as ordered numbers on a 2D or 3D coordinate

General consideration

Based on discussion in previous meetings, companies seem to have different assumption on Set A beam availability to UE. In our view, it should be assumed that measurements of Set A beams are not available or is available but can be outdated at UE side. In addition, UE is not expected to manage a good Rx beam for each of the Set A beams. If this assumption is broken, we expect negligible or no gain with SD beam prediction compared to legacy BM due to the increased RS and BM overhead.

Proposal #2: It should be assumed that measurements of Set A beams are not available or is available but can be outdated at UE side, and UE is not expected to manage a good Rx beam for each of the Set A beams.

Specification enhancements for UE-sided model

For beam prediction with UE-sided model, some study points have been agreed.

Agreement For BM-Case1 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW - The beam(s) that is based on the output of AI/ML model inference - FFS: Predicted L1-RSRP corresponding to the beam(s) - FFS: other information Agreement For BM-Case2 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW - The beam(s) of N future time instance(s) that is based on the output of AI/ML model inference - FFS: value of N - FFS: Predicted L1-RSRP corresponding to the beam(s) - Information about the timestamp corresponding the reported beam(s) - FFS: explicit or implicit FFS: other information Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity, feasibility and the potential specification impact (if needed) of the following information reported from UE to network: - Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s) - Whether/how to differentiate predicted L1-RSRP and measured L1-RSRP - Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams) - FFS: Definition/content of confidence/probability information Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused).

When assistance information is provided to UE, one issue would be how to determine Set A. The size of Set A would impact UE complexity and feedback overhead, so it needs to be maintained in a reasonable size. Assuming that potential beams for Set A are measurable with a periodicity much longer than that of Set B beams, UE assistance/reporting for determining Set A can be considered.

Proposal #3: Consider UE assistance/reporting for determining Set A, e.g. UE to report preferred Set A among candidate beams of Set A.

Regarding the predicted L1-RSRP report, we think that it is useful for NW to compare beam quality of multiple UEs and/or multiple beams of a same UE. For BM-Case2, time-variation of predicted L1-RSRP can also be considered to be included in the report for helping intra-/extra-polation at NW side. For the predicted beam report with L1-RSRP, one challenge would be how NW can rely on the reported beam ID and/or L1-RSRP value. This issue could be addressed if UE reports confidence/probability information about the predicted beams. For this information, it needs to be clarified whether the confidence/probability is calculated per model/functionality, per report or per report parameter (e.g. beam ID, L1-RSRP).

Proposal #4: Support predicted L1-RSRP report together with beam(s). For BM-Case2, information on time-variation of L1-RSRP can also be included in the report for helping intra-/extra-polation at NW side.

Proposal #5: For predicted L1-RSRP report, confidence/probability information may be helpful for NW to decide whether/how to use the reported L1-RSRP. Further study whether the information is per model/functionality, per report or per report parameter.

Proposal #6: For BM-Case2 with UE-sided models, following beam reporting enhancements can be considered

- Report of beam(s) for each future time instance or beam(s) for a time duration, i.e. from the first time instance to the last time instance.

- Report of beam(s) for current time instance for fallback operation

- Report of timestamps by UE or NW to indicate timestamps

Specification enhancements for NW-sided model

For beam prediction with NW-sided models, some study points have been agreed.

Agreement For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on the following L1 reporting enhancement for AI/ML model inference - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered

For NW-sided models, more than 4 beam report from UE would be beneficial for NW-side data collection for training/inference/monitoring. For this, the following two approaches have been identified,

- Opt1. L1 beam report enhancement for >4 beams

- Opt2. Defining higher-layer based beam measurement collection procedure for >4 beams

Opt2 is one possible approach as 3GPP has similar measurement collection features, e.g. for Minimization of Drive Tests (MDT). As we are in SI phase, i.e. not necessarily down-select one option between the two, we suggest that RAN1 should focus on RAN1 solution and leave Opt2 for study in RAN2.

Proposal #7: RAN1 should focus on potential enhancement on L1 beam report, and leave the higher-layer based approach for beam measurement collection to RAN2.

In addition to the beam reporting enhancement, beam indication enhancement should also be considered. Similar to the observation/discussion previously, UE can measure and maintain its Rx beam only for Set B beams, and not possible for Set A beams. Thus, beam indication should also be based on Set B beams.

Proposal #8: For NW-sided models, in addition to beam reporting enhancements, beam indication enhancement should be considered that TCI/QCL RS should be represented based on Set B beams of which UE can measure and maintain its Rx beam.

For NW-side AI/ML models, NW can use its own measurement as training/validation/testing dataset for AI/ML model. In addition, beam related UE reporting quantity can be used as input parameter for the AI/ML model. For example, legacy beam reporting and/or CSI reporting can be used as input or assist data for NW-side AI/ML model to predict future DL beams. Also, positioning related UE reporting can also be used for DL beam prediction, since UE position is highly related with DL beam to be served for the UE. Additional UE reporting can also be considered for improving the DL beam prediction performance. For example, UE data attained from sensors such as velocity, orientation, or rotation can be formatted to a non-proprietary information and reported to NW so that it can be used as input for the NW-side AI/ML model. Also, more refined or detailed beam reporting, e.g., tendency/variance of best N beam(s) and/or past/present best N beam(s) per time stamp, can be considered, which can also be used as input data for NW-side AI/ML model. In addition to the UE reporting, enhancements on beam indication can also be considered. For example, in UE group based mobility scenarios like UEs in a train or bus, NW-side AI/ML could predict future beam for the group of UEs so that NW may indicate multiple beams for multiple time instances for one or multiple group(s) of UEs.

Proposal #9: For NW-sided AI/ML in BM-Case2, consider enhancements on UE reporting and beam indication.

Performance monitoring and LCM aspect

For model performance monitoring and LCM, the following agreements were made.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation Agreement For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the NW-side model monitoring: - NW monitors the performance metric(s) and makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation Agreement Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the potential specification impacts from the following aspects - Beam measurement and report for model monitoring - Note: This may or may not have specification impact Agreement Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the necessity and the potential specification impacts from the following aspects: - UE reporting of beam measurement(s) based on a set of beams indicated by gNB - Signaling, e.g., RRC-based, L1-based - Note: Performance and UE complexity, power consumption should be considered Agreement Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, study the following alternatives (including feasibility/necessity) with potential down-selection: - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - Other alternatives are not precluded - Note: At least the performance and spec impact should be considered Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding NW-side performance monitoring, study the following aspects as a starting point including the study of necessity: - Configuration/Signaling from gNB to UE for measurement and/or reporting - UE reporting to NW (e.g., for the calculation of performance metric) - Indication from NW for UE to do LCM operations - Other aspect(s) is not precluded - Note1: At least the performance and reporting overhead of model monitoring mechanism should be considered Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following aspects as a starting point including the study of necessity and feasibility: - Indication/request/report from UE to gNB for performance monitoring - Note: The indictation/request/report may be not needed in some case(s) - Configuration/Signaling from gNB to UE for performance monitoring - Other aspect(s) is not precluded

Among the three alternatives for UE-side AI/ML model, Alt1 should be supported as intensively discussed in our companion contribution [2]. UE can perform model performance monitoring, e.g. based on input/output data distribution, applicable condition, etc. by itself, and make decision of model selection/activation/deactivation/switching/fallback operation.

Proposal #10: For UE-sided AI/ML model, Alt1(UE-side model monitoring) should be supported.

For UE-side performance monitoring, request or report from UE to NW may be considered, e.g. to check and compare predicted L1-RSRP with actual L1-RSRP from Set A beam measurement. This may not necessarily be an on-demand procedure since NW could configure and transmit Set A with long periodicity based on UE capability report, thus UE can use the Set A measurement for the purpose of performance monitoring.

Proposal #11: Further study whether dedicated signaling or procedure for UE-side performance monitoring is necessary by considering that Set A beams could be provided based on UE capability report.

For NW-side monitoring for network-side AI/ML model, UE reporting based on Set A can be considered but with an assumption that measurement and reporting on Set A or potential beams of Set A should happen rarely, e.g. with tens or hundreds longer periodicity than that of Set B beams.

Proposal #12: For NW-sided model monitoring, UE reporting based on Set A can be considered but with an assumption that measurement and reporting on Set A or potential beams of Set A shall happen rarely.

Tx beam vs. BPL prediction for NW-sided model

In previous meetings, there have been different views on the benefit of BPL(beam pair link) prediction by NW-sided AI/ML model in comparison to gNB Tx beam prediction as being adopted currently. Actually, the same discussion had been done during Rel-14/Rel-15 NR study and work item time frame. At that time, proponents of BPL based BM showed performance gain over Tx beam based BM via simulation results but 3GPP finally agreed to adopt Tx beam based BM framework due to the following reasons.

1.	Difficulty on representing UE beams for diverse UE types/shapes

BPL would require UE to report Rx beam ID to NW. But it is nearly impossible to formulate UE beams in a standard form due to very diverse and irregular UE antenna form factor. Beam radiation pattern including beam gain, beam-width, and main/side-lope pattern of Rx beam can be quite different from what use for computer simulation in 3GPP according to different UE types/shapes.

2.	Losing UE implementation flexibility and implementation-based beam selection

For BPL based approach, UE should maintain its beam pattern for each beam ID which restrict UE implementation flexibility and precludes UE Rx beam pattern optimization. In addition, it also preclude UE to change to a good beam autonomously, e.g. by sensing UE rotation, blockage, hand-gripping, interference, etc. For example, UE can adjust its beamforming or change to a new Rx beam when UE rotates in order to maintain the serving gNB Tx beam, which is allowed operation by current specification. Note that QCL information is about gNB reference signal so that it is up to UE implementation on how to maintain a good Rx beam for the serving gNB Tx beam. By adopting BPL based BM, we should break this agreed principle of BM, risking a situation that UE should maintain a bad beam even if UE knows that it is now BAD.

As observed above, the performance of BPL based BM can be worse than Tx beam based BM in real world and these reasons remain unchanged regardless of applying AI/ML for BM or not. Thus, the agreed principle for BM in 3GPP should not be broken.

Proposal #13: For NW-sided model, Tx beam prediction should only be considered.

Conclusion

In this contribution, we provided our view on the potential use cases and specification impact on AI/ML for beam management and the followings are proposed.

Proposal #1: For the UE AI/ML model training and inference, assist information on relation/association between Set A beams and Set B beams should be provided to UE. To represent beams in Set A and/or Set B while preserving sensitive proprietary information, consider following exemplary methods.

- Set A beams are represented by LC coefficients of Set B beams

- Tx beam directions are represented as ordered numbers on a 2D or 3D coordinate

Proposal #2: It should be assumed that measurements of Set A beams are not available or is available but can be outdated at UE side, and UE is not expected to manage a good Rx beam for each of the Set A beams.

Proposal #3: Consider UE assistance/reporting for determining Set A, e.g. UE to report preferred Set A among candidate beams of Set A.

Proposal #4: Support predicted L1-RSRP report together with beam(s). For BM-Case2, information on time-variation of L1-RSRP can also be included in the report for helping intra-/extra-polation at NW side.

Proposal #5: For predicted L1-RSRP report, confidence/probability information may be helpful for NW to decide whether/how to use the reported L1-RSRP. Further study whether the information is per model/functionality, per report or per report parameter.

Proposal #6: For BM-Case2 with UE-sided models, following beam reporting enhancements can be considered

- Report of beam(s) for each future time instance or beam(s) for a time duration, i.e. from the first time instance to the last time instance.

- Report of beam(s) for current time instance for fallback operation

- Report of timestamps by UE or NW to indicate timestamps

Proposal #7: RAN1 should focus on potential enhancement on L1 beam report, and leave the higher-layer based approach for beam measurement collection to RAN2.

Proposal #8: For NW-sided models, in addition to beam reporting enhancements, beam indication enhancement should be considered that TCI/QCL RS should be represented based on Set B beams of which UE can measure and maintain its Rx beam.

Proposal #9: For NW-sided AI/ML in BM-Case2, consider enhancements on UE reporting and beam indication.

Proposal #10: For UE-sided AI/ML model, Alt1(UE-side model monitoring) should be supported.

Proposal #11: Further study whether dedicated signaling or procedure for UE-side performance monitoring is necessary by considering that Set A beams could be provided based on UE capability report.

Proposal #12: For NW-sided model monitoring, UE reporting based on Set A can be considered but with an assumption that measurement and reporting on Set A or potential beams of Set A shall happen rarely.

Proposal #13: For NW-sided model, Tx beam prediction should only be considered.

Reference

[1] RP-221348 “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm

[2] R1-2303075, General aspects on AI/ML framework, LG Electronics