3GPP TSG RAN WG1 #112bis-e	                           R1-2303080

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.2.4.1

Source:	LG Electronics

Title:	Evaluation on AI/ML for positioning accuracy enhancement

Document for:	   Discussion and Decision

- Introduction

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

New SI on artificial intelligence (AI)/machine learning (ML) for NR air interface was approved and three use cases were captured in the SID as follows [1].

In this contribution, we discuss and provide our views on evaluation methodology and metrics for AI/ML positioning accuracy enhancement.

- Discussions on evaluation methodology and metrics

In RAN1#112 meeting, the agreements were made as follows for AI/ML model monitoring methods and ground truth label as captured below. In the following subsections, we discuss the AI/ML model monitoring metric not only considering the intermediate performance metric on AI/ML assisted positioning in MTRP construction but also depending on the availability of ground truth label with labelling error,

Agreement For direct AI/ML positioning, study the impact of labelling error to positioning accuracy - The ground truth label error in each dimension of x-axis and y-axis can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. - Value L is up to sources. - Other models are not precluded - [Whether/how to study the impact of labelling error to label-based model monitoring methods] - [Whether/how to study the impact of labelling error for AI/ML assisted positioning.] Agreement For direct AI/ML positioning, study the performance of model monitoring methods, including: - Label based methods, where ground truth label (or its approximation) is provided for monitoring the accuracy of model output. - Label-free methods, where model monitoring does not require ground truth label (or its approximation). Agreement For AI/ML assisted approach, study the performance of label-free model monitoring methods, which do not require ground truth label (or its approximation) for model monitoring.

  - AI/ML model monitoring based on ground truth label

As shown above, ground truth label error can be considered to evaluate and study the impact on positioning accuracy at least for direct AI/ML positioning method. The labelling error is mainly originated from considering the case on normal UE as a PRU and/or PRU with mobility. According to this, the impact on the ground truth label error also shall be taken into account on AI/ML model monitoring especially for direct AI/ML positioning method where the AI/ML model output estimates UE location, and the corresponding model monitoring performance can be generally determined by the amount of the difference between the estimated UE location and the given ground truth label. In this case, there may exist an ambiguity whether the AI/ML model works well or not via the monitoring performance itself since the monitoring performance degradation may come from the ground truth label error while the estimation of UE location is relatively accurate. To address this problem, it would be helpful to monitor the model performance depending on the quality indicator of the ground truth label, which is based on the label error. For example, when the quality of the label is poor, the difference between the estimated UE location and the ground truth label is not adopted for model performance calculation or an offset value can additionally be considered on threshold on the monitoring performance for model fine-tuning/transfer. Above all, the most important simulation aspect of model monitoring for AI/ML based positioning is to align the procedure (e.g. model fine-tuning/transfer) with respect to the model monitoring performance regardless considering the ground truth label error or not.

Proposal #1: For model monitoring evaluation, consider to align the procedure (e.g. model fine-tuning/transfer) depending on model monitoring metric/performance among companies.

Regarding the monitoring metric when the ground truth label is not provided/needed, it can be based the model input or UE location/mobility. For model input based monitoring, it can be designed via the similarity check between a model input and that of AI/ML model training. The amount of the difference in term of distribution can be MSE or mean/variance of the distribution. To monitor the AI/ML model, the UE can report the similarity information with soft value to NW or LMF side. Based on this monitoring approach, NW can indicate the model update (e.g. maintain/fine-tuning/model transfer) to the UE depending on the similarity information value.

Proposal #2: Consider model monitoring metric based on model input depending on the amount of similarity between input and training data distribution

On UE location/mobility based monitoring, a form of IDs representing real environment (e.g. zone ID, scenario ID etc.) for TRPs where UE is located can be a criterion that the deployed environment is different from that of pre-trained AI/ML model. In this case, depending on the UE location, the amount of impact on monitoring performance on each TRP can be different. For example, when the monitoring performance of a certain TRP is poor but the distance between the UE and the corresponding TRP is relatively far compared to that of other TRPs, the overall monitoring performance can be reasonable to maintain the AI/ML model. Also, environment change where UE is to be located based on the prediction of UE mobility can be considered. Then, for the performance generalization, the model can be updated depending on the status of environment and/or the pair, e.g., fine-tuning is operated when UE moves from zone ID#1 to zone ID#2 but model update and/or model transfer/delivery if needed is operated when UE moves from zone ID#1 to zone ID#3.

  - AI/ML model monitoring with intermediate performance metric

In RAN1#110bis-e, it was agreed to use intermediate performance metrics for evaluating AI/ML assisted positioning as follows.

Agreement For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used: - LOS classification accuracy, if the model output includes LOS/NLOS indicator of hard values, where the LOS/NLOS indicator is generated for a link between UE and TRP; - Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD). - Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD). - Companies provide info on how LOS classification accuracy and timing/angle estimation accuracy are estimated, if the ML output is a soft value that represents a probability distribution (e.g., probability of LOS, probability of timing, probability of angle, mean and variance of timing/angle, etc.)

Regarding the output performance of AI/ML assisted positioning, the quality of intermediate performance is related to output performance (i.e. UE location) and it can be utilized as an assistance information for estimating the UE position. Moreover, it is necessary to monitor AI/ML model by considering intermediate/output performance at the same time to perform model fine-tuning and update as mentioned above. Based on the performance monitoring approach, the condition for AI/ML model fine-tuning/update seems to be considered further. In this sense, the meaning of the intermediate performance of AI/ML assisted positioning can be regarded as a performance indicator of the AI/ML model monitoring performance on output performance (i.e. UE location)

As shown above, intermediate performance metrics are used depending on the sub use cases. For LOS classification accuracy, it needs to be discussed further at least on how the LOS classification accuracy is estimated when the ML output is not a hard value to represent LOS/NLOS indication. To our understanding, the LOS/NLOS identifier can be estimated with hard/soft value and the corresponding statistical information to provide the probability distribution. It means that the AI/ML output with the soft value can indicate the corresponding LOS/NLOS accuracy in itself and it would be replaced to the intermediate performance metric on that and/or provides an additional information to calculate AI/ML output to improve the robustness on the estimation. Moreover, details of reporting the intermediate performance metric is a considerable point where it is reported with the intermediate performance metric with a specific value or the amount of difference between intermediate performance and the actual value obtained via ground-truth labelled dataset etc.

Proposal #3: For LOS/NLOS classification of AI/ML assisted positioning, consider also to utilize a soft value of the ML output as a LOS classification accuracy and model monitoring metric.

- Conclusion

In this contribution, we discussed evaluation methodology and metrics for AI/ML positioning accuracy enhancement. Based on the above discussion, we have following proposals:

Proposal #1: For model monitoring evaluation, consider to align the procedure (e.g. model fine-tuning/transfer) depending on model monitoring metric/performance among companies.

Proposal #2: Consider model monitoring metric based on model input depending on the amount of similarity between input and training data distribution

Proposal #3: For LOS/NLOS classification of AI/ML assisted positioning, consider also to utilize a soft value of the ML output as a LOS classification accuracy and model monitoring metric.

Reference

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm