3GPP TSG RAN WG1 #112bis-e                                   R1-2303081

e-Meeting, April 17th – April 26th, 2023

Agenda item:		9.2.4.2

Source:		LG Electronics

Title:			Other aspects on AI/ML for positioning accuracy enhancement

Document for:	Discussion and Decision

Introduction

In RAN1#112 meeting, the agreements on sub use cases and potential specification impact of AI/ML for positioning were made as follows.

Agreement Regarding training data generation for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified - At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a) - At least LMF with known PRU location is identified to generate ground truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a) - FFS whether and if so, applicable conditions and potential specification impact for the following options to generate ground truth label - UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods - Network entity generates ground truth label based on positioning methods - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: transfer of training data from the entity generating training data to a different entity is not precluded and associated potential specification impact is for further study Agreement Regarding training data collection for AI/ML based positioning, study benefit(s) and potential specification impact (including necessity) at least for the following aspects - Associated information of training data - Quality indicator at least for ground truth label (if needed) - Other information associated with training data is not precluded. E.g., information related training dataset/samples, information related to scenario, resource configuration & mapping, timing for training data, information on implementation imperfections, etc. - Assistance signaling and procedure to facilitate generating/collecting training data - Potential determination of the UE/PRU/TRP which can provide the training data - Configuration of reference signal (for measurement and/or label) - Signaling other than above 2 for data collection - E.g., requested quality of training data Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - FFS PRU for Case 1 and 2a - gNB at least for Case 3a (with gNB-side model) - FFS gNB for Case 3b (with LMF-side model) - LMF at least for Case 2b and 3b (with LMF-side model) - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b) - If model monitoring does not require ground truth label (or its approximation). - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc. - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc. - report of the calculated metric and/or model monitoring decision - If model monitoring requires and is provided ground truth label (or its approximation) - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc. - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc. - report of the calculated metric and/or model monitoring decision - Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded Agreement Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement - For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead - Potential new measurement: CIR/PDP - existing measurement: e.g., RSRP/RSRPP/RSTD - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied - Note2: study the impact of model input for other cases are not precluded - For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF - new measurement report: e.g., ToA, path phase - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - RS configurations - Other assistance information is not precluded Note: Companies are encouraged to report their assumption of functionality and their assumption of information element(s) of AI/ML functionality identification for AI/ML based positioning with UE-side model (Case 1 and 2a).

In this contribution, we discuss on other aspects on AI/ML for positioning accuracy enhancements such as representative sub use-cases that are worth considering and related potential specification impacts in terms of RAN1 point of view.

Discussions on potential sub use cases and their specification impact

Based on above agreements, we firstly discuss model monitoring performance metric for AI/ML based positioning accuracy enhancement in the following sub-section 2.1. In sub-section 2.2, we suggest two sub use cases (i.e. LOS/NLOS indication, PRU prediction) on the perspective of positioning accuracy enhancement and the corresponding potential specification impact. In addition, regarding data collection point of view, it is considered that utilizing the PRU prediction would make a normal UE possible as an entity to obtain ground truth label.

Model monitoring performance metric

For AI/ML based positioning accuracy enhancement, the offline-trained AI/ML model is delivered to UE or NW for utilizing it to actual operating environment. In this case, mismatch between the offline training data and the field data is inevitable and the output performance may be degraded when the AI/ML model is applied. To address this problem, how to update and/or fine-tune the AI/ML model can be considered.

Regarding the AI/ML model monitoring, the intermediate performance which is related to output performance also needs to be taken into account. As an intermediate performance, it would be estimated LOS/NLOS identifier, TDoA, AoA and so on. In other words, it means 2-step monitoring performance metric where the output performance degradation of the AI/ML model is originated from whether the poor intermediate performance or from discrepancy on environment, scenario and the assumption of the offline trained AI/ML model compared to actual operating environment. Based on this monitoring method, the detailed condition is further investigated to perform fine-tuning or update of the AI/ML model. For example, if the output performance is less than a certain level, fine-tuning can be operated on the intermediate performance to obtain a specific quality on that. Depending on the output performance after the fine-tuning, the AI/ML model can be maintained or updated or can be switched by another suitable AI/ML model.

Proposal #1: Consider AI/ML model fine-tuning or update based on model monitoring performance metric by taking into account the intermediate performance and output performance jointly.

- Condition of fine-tuning or update with respect to a quality of intermediate/output performance

To this end, the related assistance data can be requested by UE at least for UE-side model such as the distance among TRP(s) and the beam information, e.g., beam ID, beam RSRP, etc., covering an area per TRP, or labelled dataset etc. In addition, the detailed method or contents on model switching/update can be discussed further, e.g., AI/ML model in itself or the corresponding parameter/structure only, etc.

Proposal #2: Consider the followings for potential specification impact on AI/ML model monitoring.

- Assistance signalling for UE-sided model (e.g. distance between TRPs, beam information per TRP)

- Contents of model switching/update (e.g. AI/ML model itself, AI/ML model parameter or structure only)

In addition to the AI/ML model monitoring, it was agreed in the last meeting to study the model monitoring method depending on the availability of ground truth label. For the ground truth label, the label error is considered from a data collection and monitoring point of view as in AI 9.2.4.1. In this sense, it can be captured in the agreement above to investigate an associated information of the ground truth label with a quality indicator. By utilizing this, it can be beneficial on AI/ML model life cycle management as well as that of the output/intermediate performance aspects.

Regarding how to configure/quantify the quality of ground truth labels from a certain entity, there are two ways based on LMF/NW side and normal UE/PRU side, respectively. In case of LMF/NW with known UE/PRU location, the ground truth label and its quality can be generated through multiple instances of the measurement report from TRPs for positioning and the corresponding measurement quality information (e.g. LOS/NLOS information, TDoA etc.) during a certain window. Based on this, LMF/NW can generate the ground truth label with the corresponding quality indicator Q and it can be utilized to AI/ML model (re)training/fine-tuning and monitoring with respect to each case. For the model training, the ground truth label can be delivered to AI/ML model training entity and it can be included in training dataset depending on the value of Q. Also for the model monitoring for AI/ML based positioning, the value of Q can give a criterion whether the difference between the estimated UE location and the ground truth label is used for model performance calculation or not. In addition, it can be used to adjust model monitoring threshold for fine-tuning/model-transfer.

Proposal #3: Consider the quality of ground truth label generation based on the measurement reports from TRPs for LMF/NW with known UE/PRU case.

- Based on this, study AI/ML model training and monitoring methods further

Accuracy improvements

In Rel-17 positioning enhancements, reporting of path specific measurement and providing LOS/NLOS indicator for first arrival path were agreed since LOS/NLOS identification is critical factor for accuracy performance. However, the indication has been without definition and it relies on UE implementation. In this sense, LMF does not know the reliability of the indication and then, it may be difficult to utilize it to improve the performance accuracy. Therefore, it should be required to improve the reliability of LOS indication information and to improve the performance in terms of positioning accuracy and resource efficiency.

Observation #1: In Rel-17, LOS/NLOS indication for first path can be reported but the detailed algorithm is up to UE implementation (reliability issue per UE).

To improve the reliability of LOS/NLOS information, it can be based on AI/ML approach at UE (i.e. Case 1) or LMF side (i.e. Case 2a/2b). When applying the AI/ML method at UE-side, the LOS/NLOS probability value for first arrival path and/or the reliability information such as probability can be derived based on the UE capability and various parameters related to LOS estimation such as UE antenna configurability, capability of PRS measurement, AI/ML model for LOS estimation, indication of PRU, the number of PRS measurement, noise variance, etc. Based on the above, the output can be a LOS/NLOS identifier with hard/soft value and the corresponding statistical information, e.g., mean/variance of the identifier as an assistance information to LMF. Regarding Case 2a for UE-side AI/ML, based on the measurement results and some assistance information (e.g. LOS probability and/or reliability information) from UE, LOS/NLOS identification can be achieved. In addition, LMF can feedback the estimation accuracy of the LOS/NLOS identifier or UE location to the UE and the subsequent LOS/NLOS identification or the UE location at UE-side model can be adjusted by the feedback (Case 2b).

Proposal #4: Consider assistant information including LOS probability and/or reliability information for the AI/ML based LOS/NLOS identification at least for Case 2a

- Assistance information: LOS/NLOS identifier with hard/soft value and the corresponding statistical information.

Based on the LOS/NLOS related information from the above, NW can collect/process the LOS/NLOS identification at a specific location on coordinate system for supporting UEs. Through this, it is possible to determine which PRS resource is most likely to be LOS for a specific UE in the form of a boundary between specific regions on the coordinate as shown in Figure 1 below. Then, the PRS configuration for the location of the UE can be adaptively configured via the above information.

Figure 1. Example of LOS/NLOS probability and the corresponding PRSs on coordinate system supporting UEs

Proposal #5: Consider PRS priority configuration based on AI/ML based LOS/NLOS indication.

Regarding the above LOS/NLOS based PRS configuration, it can be also considered on MTRP construction. According to this construction, PRS(s) can be configured and transmitted by each TRP corresponding to LOS indication and the related probability information. When UE measures the PRSs from N TRPs in this case, near-far problem can happen that a signal power of PRS from a certain TRP is too strong compared to that of other TRPs and the UE fails to detect the rest of PRSs due to the large relative power difference. Especially, this problem can frequently occur since the PRS configuration for each TRP is based on LOS indication while it enhances the accuracy of positioning accuracy.

To mitigate/avoid this problem, there are two approaches: 1) PRS transmit power adaptation based on the distance between the UE and the TRPs and 2) adaptive PRS muting pattern. For 1st approach, the UE can report power limitation information on a specific TRP(s) as an assistance information where the candidate value on the power limitation is configured by network. For 2nd approach, the muting pattern for PRS per TRP among N TRPs can be set based on the PRS measurement report from the UE.

Proposal #6: Consider PRS power control and PRS muting pattern on MTRP scenario via LOS/NLOS based PRS configuration.

In the Rel-17 positioning enhancement, PRU (positioning reference unit) was introduced by utilizing the exactly known position of a specific UE or gNB (i.e. reference UE or gNB) at LMF for accuracy improvement. Regarding a UE as PRU, there is a limitation since the UE generally has mobility and the location is correspondingly changed. If the LMF can predict the UE location with mobility based on AI/ML method, it would be possible that which UE can be used as a PRU. Then, the LMF can use the UE dynamically as PRU to calculate/estimate the position of target UE.

Observation #2: When LMF can predict UE location with mobility, it is possible that which UE can be used as PRU, the LMF can use the UE dynamically as PRU to calculate the position of target UE.

For LMF-based PRU prediction (i.e. Case 2b), based on the measurement report of a normal UE as an input data set of the AI/ML algorithm, it is possible to predict/determine the output whether the normal UE is utilized as a PRU or not at the LMF side. If the UE is determined as the PRU, the position of target UE can be estimated/calculated with the PRU. Meanwhile, for UE-based PRU prediction (i.e. Case 2a), it is based on AI/ML model in the UE-side with providing assistance information for AI/ML model at LMF. Depending on the AI/ML model output, the UE can report identification information as PRU and/or assistance information, e.g. PRU expiration time, de-registration request on PRU, etc.

With these consideration, the potential specification impact can be related signaling on measurement report or information on deciding the UE as the PRU. For example, duration and the amount of distance change in duration for sensor based information and/or RSRP based on RAT dependent method can be considered as measurement or assistant information. Based on AI/ML model and assistant information, predicted value of the UE location with the corresponding time instance of the future, PRU related information such as PRU indication, the probability of the conditions, etc. can be obtained.

Proposal #7: Consider PRU prediction on LMF-/UE-side based on measurement report in addition to PRU identification and/or assistance information utilized for PRU determination at least for Case 2a/2b.

Regarding the perspective of data collection, it was intensively discussed in the last meeting whether and how to determine an entity to obtain ground truth label. For an exact UE location as an applicable ground truth label, PRU has a crucial role. However, a normal UE cannot be assumed with static or fixed location in general. In this sense, the PRU prediction based on AI/ML approach as mentioned above makes the normal UE feasible to consider the entity to collect ground truth label on the UE location accurately.

Proposal #8: Consider a normal UE an entity used to obtain ground truth label based on the AI/ML based PRU prediction

Conclusion

In this contribution, we discussed about several aspects on AI/ML for positioning enhancement, and our observations and proposals are summarized below:

Observation #1: In Rel-17, LOS/NLOS indication for first path can be reported but the detailed algorithm is up to UE implementation (reliability issue per UE).

Observation #2: When LMF can predict UE location with mobility, it is possible that which UE can be used as PRU, the LMF can use the UE dynamically as PRU to calculate the position of target UE.

Proposal #1: Consider AI/ML model fine-tuning or update based on model monitoring performance metric by taking into account the intermediate performance and output performance jointly.

- Condition of fine-tuning or update with respect to a quality of intermediate/output performance

Proposal #2: Consider the followings for potential specification impact on AI/ML model monitoring.

- Assistance signalling for UE-sided model (e.g. distance between TRPs, beam information per TRP)

- Contents of model switching/update (e.g. AI/ML model itself, AI/ML model parameter or structure only)

Proposal #3: Consider the quality of ground truth label generation based on the measurement reports from TRPs for LMF/NW with known UE/PRU case.

- Based on this, study AI/ML model training and monitoring methods further

Proposal #4: Consider assistant information including LOS probability and/or reliability information for the AI/ML based LOS/NLOS identification at least for Case 2a

- Assistance information: LOS/NLOS identifier with hard/soft value and the corresponding statistical information.

Proposal #5: Consider PRS priority configuration based on AI/ML based LOS/NLOS indication.

Proposal #6: Consider PRS power control and PRS muting pattern on MTRP scenario via LOS/NLOS based PRS configuration.

Proposal #7: Consider PRU prediction on LMF-/UE-side based on measurement report in addition to PRU identification and/or assistance information utilized for PRU determination at least for Case 2a/2b.

Proposal #8: Consider a normal UE an entity used to obtain ground truth label based on the AI/ML based PRU prediction