3GPP TSG RAN WG1 #112bis-e	   	 	  	                	      R1-2303083

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.12.2

Source:	LG Electronics

Title:	Enhancements on TA management for mobility

Document for:	Discussion and Decision

Introduction

The approved WID [1] includes enhancements on Rel-18 LTM (L1/L2 triggered mobility), including L1 enhancements for inter-cell beam management and timing advance management. Specifically, in order to achieve latency reduction in mobility scenario, timing advance management is targeting enhancement on TA acquisition/maintenance for candidate serving cell.

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3]

In this contribution, we discuss the enhancement on timing advance management.

Discussion

TA acquisition for candidate cell

The following agreements were made in previous meetings.

Agreement Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility. - FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2) Agreement On mechanism to acquire TA of the candidate cells, the following solutions can be further studied: • RACH-based solutions e.g., PDCCH ordered RACH, UE-triggered RACH, higher layer triggered RACH from NW other than L3 HO cmd • RACH-less solutions e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell) Agreement On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH.  The PDCCH order is only triggered by source cell  FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc.  Note: any other RACH-based solutions are for discussion separately Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

In RAN1 #110bis-e meeting, it was agreed to support TA acquisition of candidate cell(s) before cell switch command is received in Rel-18 LTM. Specifically, regarding mechanism to acquire TA of the candidate serving cell(s), it was agreed to support RACH-based approach by PDCCH ordered RACH triggering. In the last meeting, regarding whether to support RACH-less approach, UE-based TA measurement was to be supported as working assumption. In our view, the entire procedure of TA acquisition for candidate cell(s) can be quite diverged according to RACH-based/less solution. In RACH-based solution, gNB may have knowledge of acquired TA for candidate cell(s) from UE RACH transmission. However, up to whether RAR is transmitted or not, UE may or may not have knowledge of acquired TA for candidate cell(s). On the other hand, in RACH-less solution, for example in case of UE based TA measurement, gNB doesn’t need to know TA value and may only need to know that the UE is capable of UE based TA measurement for candidate cell(s). So, considering limited TU of Rel-18 LTM in RAN1, we prefer to focus on already agreed RACH-based approach and further discuss on RACH-less approach if time allows.

Proposal #1: Prefer to firstly complete RACH-based approach (i.e., PDCCH ordered RACH) for TA acquisition.

Details on TA acquisition with RACH-based approach

The following agreements were made in previous meetings.

Agreement For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell: - Alt1: Associate TA/TAG and candidate cell implicitly, e.g., - the association between TA/TAG and TCI states can be configured - Alt2: Associate TA/TAG and candidate cell explicitly, e.g., - the association is provided as a part of candidate cell(s) configuration - the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration Agreement For PDCCH ordered RACH in LTM, at least the following enhancements are supported - Introduce indication of candidate cell and/or RO of candidate cell in DCI - configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order - FFS: whether/how to transmit RAR Agreement - TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW. - same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell Agreement For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell. Note: the detailed signalling is left to RAN2 Agreement The PDCCH order from the source cell contains the indication of candidate cell. - The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity Agreement For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated - If reception of RAR is not configured/indicated (without RAR) - TA value of candidate cell is indicated in cell switch command - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order - If reception of RAR is configured/indicated (with RAR), FFS - whether RAR is received from serving cell or candidate cell - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE - content of RAR - FFS: signaling for configuration/indication of whether RAR needs to be received - UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM - Send LS to RAN2 and RAN3 to check the feasibility about this agreement - Note: Definition of candidate cells is up to RAN2 Agreement - For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported Agreement on whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives. - Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) - Alt 2: UE autonomous Re-transmission of PRACH is allowed, - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax Agreement If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell. - The maximum number of TA values memorized by UE is a UE capability - FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR Agreement Whether RAR needs to be received is configured by RRC. Agreement study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured - Whether gap between the DCI and PRACH longer than timeline defined in spec is needed - Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx

In previous meeting, PDCCH ordered RACH was agreed to be supported for TA acquisition of candidate cell(s). It was agreed that each PRACH configuration can be configured per candidate cell, and gNB can trigger PDCCH ordered CFRA with indication of candidate cell id for a UE to acquire TA of the candidate cell. After TA acquisition, acquired TA can be explicitly associated with (TAG of) each candidate cell configuration. Furthermore, for several meetings, it had been discussed regarding the number of acquired TAs/TAGs for candidate cell(s). There was a discussion that up to two TA/TAG can be associated with a candidate cell, considering two TA operation in the candidate cell. However, in our view, it is not necessary to consider two TA for M-DCI based M-TRP in Rel-18 LTM based handover procedure, so it is not preferred to mix up Rel-18 LTM TA management and Rel-18 MIMO two TA. We think that up to one TA/TAG per candidate cell is sufficient and multiple candidate cells can be associated with the same TAG when the multiple candidate cells have the same TA value.

Proposal #2: After TA acquisition for candidate cell(s), TA/TAG can be explicitly associated with the candidate cell(s).

  - Up to one TA/TAG per candidate cell configuration is sufficient, and not prefer to mix up Rel-18 LTM TA management and Rel-18 MIMO two TA.

In the last meeting, it was agreed that RAR reception can be configured by RRC signaling. If RAR reception is configured, it should be decided whether the RAR is received from serving cell or candidate cell. In the current specification, in order to receive RAR scheduling DCI(DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI) in response to a PRACH(CFRA) initiated by a PDCCH order, i) UE assume that the RAR scheduling DCI has the same QCL properties with the PDCCH order(if the PRACH is for the SpCell), or ii) UE assume that the RAR scheduling DCI has the same QCL properties with the CORESET associated with the Type1-PDCCH CSS(if the PRACH is for a SCell). In that regard, additional type-1 PDCCH CSS should be configured for a candidate cell to receive RAR from the candidate cell, where the additional type-1 PDCCH CSS is associated with a CORESET which has QCL properties corresponding to the candidate cell. However, there could be a number of configured candidate cells for a UE, then multiple additional type-1 PDCCH CSS configuration would be a burden for the UE from blind detection complexity perspective. So, in our view, receiving RAR from serving cell can be more backward compatible and simple solution. Regardless of whether RAR is received from serving cell or candidate cell, it should be investigated that which UE ID is to be used for CRC scrambling of RAR scheduling DCI. Since PRACH transmission to acquire TA of candidate cell is not for serving cell anymore, it should be clarified whether the RAR scheduling DCI can be scrambled by the RA-RNTI calculated from the serving cell or another RNTI for candidate cell(s). In addition, as discussed in the last meeting, the contents of RAR MAC CE also should be discussed. Lastly, QCL assumptions for RAR scheduling DCI and RAR PDSCH should be also considered up to whether RAR is received from serving cell or candidate cell, since the current QCL assumptions for RAR is considering only the case when RAR is received from serving cell.

Furthermore, there was a discussion regarding retransmission of PRACH when reception of RAR is not configured/indicated. In our view, the number of retransmission can depend on channel quality of candidate cell(s) which is dynamically changing. So, the number of retransmission or the repetition number (as discussed in Rel-18 CE) for PRACH transmission can be indicated by ordering DCI adaptively.

Proposal #3: Support RAR received from serving cell.

Proposal #4: Further study indication of the number of PRACH retransmission or the PRACH repetition number in PDCCH order.

Conclusion

In this contribution, we have discussed the enhancement on Rel-18 LTM TA management, and provided the following observations and proposals.

Proposal #1: Prefer to firstly complete RACH-based approach (i.e., PDCCH ordered RACH) for TA acquisition.

Proposal #2: After TA acquisition for candidate cell(s), TA/TAG can be explicitly associated with the candidate cell(s).

  - Up to one TA/TAG per candidate cell configuration is sufficient, and not prefer to mix up Rel-18 LTM TA management and Rel-18 MIMO two TA.

Proposal #3: Support RAR received from serving cell.

Proposal #4: Further study indication of the number of PRACH retransmission or the PRACH repetition number in PDCCH order.

References

[1] RP-213565, New WID on Further NR mobility enhancements