3GPP TSG RAN WG1 #112bis-e	R1-2303084

e-Meeting, April 17th – April 26th, 2023

Agenda item:		9.1.2

Source:		Mavenir

Title:	Discussion on CSI enhancement

Document for:		Discussion and Decision

# 1	Introduction

Agreement

For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows:

- Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported

  - FFS: Candidate values for delay

- Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported

  - For Y>1, the phase can be configured to be absent for all the Y delays

  - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource

  - TBD: Candidate value(s) for Y>1

- FFS: Value of Dbasic

Agreement

For the Rel-18 TRS-based TDCP reporting, the priority of the CSI report(s) associated with TDCP reporting is down-selected from the following alternatives:

- Alt1. Lower than other CSI reports

- Alt2. Same as CSI report(s) not carrying L1-RSRP or L1-SINR

- Alt3. Higher than other CSI reports

- Other alternatives are not precluded

Agreement

For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, by RAN1#112bis-e, decide between the following alternatives:

- Alt1. Fully reuse legacy TRS

- Alt2. Study enhancements on TRS (e.g. periodicities)

Note. If there is no consensus on Alt2, Alt1 is the default outcome

The following agreements about TRS-based TDCP report was made during RAN1 #112 [1].

In this contribution, we provide some views and simulation results about TRS-based TDCP reporting.

# TRS-based TDCP reporting

Agreement

For the Rel-18 TRS-based TDCP reporting, the priority of the CSI report(s) associated with TDCP reporting is down-selected from the following alternatives:

- Alt1. Lower than other CSI reports

- Alt2. Same as CSI report(s) not carrying L1-RSRP or L1-SINR

- Alt3. Higher than other CSI reports

- Other alternatives are not precluded

TDCP reporting priority

CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where

-	  for aperiodic CSI reports to be carried on PUSCH  for semi-persistent CSI reports to be carried on PUSCH,  for semi-persistent CSI reports to be carried on PUCCH and  for periodic CSI reports to be carried on PUCCH;

-	  for CSI reports carrying L1-RSRP or L1-SINR and  for CSI reports not carrying L1-RSRP or L1-SINR or TDCP; k=2 for CSI reports carrying TDCP.

-	c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells;

-	s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations.

In current 3GPP 38.214 [2], CSI reports associated with a priority are given. Due to introducing TRS-based TDCP reporting, its priority should be defined. It is suggested that the priority of CSI reports including TRS-based TDCP reporting should be the lowest priority, as follows.

Proposal 1. Support Alt1, the priority of TDCP reporting is lower than other CSI reports.

Agreement

For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows:

- Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported

  - FFS: Candidate values for delay

- Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported

  - For Y>1, the phase can be configured to be absent for all the Y delays

  - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource

  - TBD: Candidate value(s) for Y>1

- FFS: Value of Dbasic

TDCP parameters

According to RAN1#112 [1], normalized time correlation has been agreed for TDCP reporting. Corresponding with various delays, the normalized time correlation values in specific UE velocities are following in Table-1.

Table-1. The values of normalized time correlation corresponding with various delays in specific UE velocities.

Other relevant parameters are complied with Section 5.1.

| UE velocity | Max. Doppler Shift | Normalized time correlation Tac |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Delay = 4 symbols | Delay = 1 slot | Delay = 5 slots | Delay = 20 slots |
| 3 km/h | 9.72 Hz | 1.0000 | 0.9992 | 0.9942 | 0.9229 |
| 30 km/h | 97.22 Hz | 0.9976 | 0.9711 | 0.5189 | 0.2198 |
| 120 km/h | 388.89 Hz | 0.9657 | 0.6873 | 0.1715 | -0.0144 |

Observation 1.  For given UE velocity, different delays result in different time correlations.

There are several results of previous simulations in [3]. Some latest results have been plotted in the below.

Updated results of estimated Doppler compared with different velocities are seen as below.

Figure-1. SNR 0dB, UE velocity vs estimated Doppler spread by TD correlation over 100ns delay spread TDL-A.

Figure-2. SNR 30dB, UE velocity vs estimated Doppler spread by TD correlation over 100ns delay spread TDL-A.

Above figures has been illustrated that different lags between TRS pairs would take distinct performance with changing UE velocity, particularly in SNR0dB.

Observation 2. 20-slot delay has shown worse accuracy. Delay <= 5 slots can ensure the estimation for time variation of channel. 5-slot delay is better for smaller UE velocity (<=30km/h), whereas 1-slot delay is suitable in scenario of higher velocity.

For basic feature Y=1 with delay≤ Dbasic symbols, simulation results are shown that the compromised choice of Dbasic is 5 slots, then the estimated accuracy can be assured in various velocities.  For example, lower than roughly 30km/h, the choice in this scenario is 5-slot delay.

Proposal 2. For basic feature Y=1 with delay≤ Dbasic symbols, Dbasic is 5 slots.

Agreement

For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, by RAN1#112bis-e, decide between the following alternatives:

- Alt1. Fully reuse legacy TRS

- Alt2. Study enhancements on TRS (e.g. periodicities)

Note. If there is no consensus on Alt2, Alt1 is the default outcome

TRS configuration

Computing channel time-domain correlation with smaller than 5-slot delay, we can reuse legacy multiple TRS resource sets. Multiplexing with timing offset and different/same periodicity, two periodic TRSs can meet requirement for various delays.   
Proposal 3. Support Alt1. We can set two periodic TRSs with timing offset and different/same periodicity.

## Analysis among agreed use case

We select one agreed use case to determine SRS periodicity in the UL-SRS reciprocity-based precoding scheme from single-user throughput (LLS) performance comparison.

TDD system use the same frequency band for uplink (UL) and downlink (DL) transmission. Due to common channel characteristics in both UL and DL, radio channel ban be reciprocal. UL-SRS reciprocity-based precoding/beamforming scheme is to obtain channel estimation from UL-SRS transmission, then use channel estimation to calculate parameters for DL transmission.

In our simulation, we schedule SRS transmissions for single UE in the UL part of special slot, calculate precoding/beamforming by channel estimation from SRS, and schedule PDSCH transmission for this UE in DL and special slots.

Figure-3. UL-SRS reciprocity-based precoding scheme

Figure-10 shows single user throughput versus multiple SRS overhead (with different SRS period) under 0/30dB SNR based on LLS simulation.  The carrier frequency is 2 GHz, and subcarrier spacing is 15kHz, we select three velocities around 3/50/100 km/h corresponding with maximum Doppler shift 5/100/200 Hz in multipath channels for comparison.

Figure-4. Single user throughput versus SRS overhead

When time variation in channel with maxDplrShift 200Hz is stronger, SRS would be aging faster, then higher SRS overhead of 10-slot (5ms) SRS period would take better user throughput.

When channel tends to static with maxDplrShift 5Hz, lower SRS overhead of 40-slot (20ms) SRS period is enough to get better throughput.

According to Table-1, thresholds can be setting based on TDCP reporting of 5-slot delay,

Table-2. The threshold of SRS period based on normalized TD correlation

| Threshold in 5-slot delay Normalized TD correlation | Max. Doppler shift |
| --- | --- |
| thr0 = 0.7 | 90.03 Hz |
| thr1 = 0.1 | 120.79 Hz |

If we have TDCP-reporting TD correlation Tac  and SRS period Psrs, then

Psrs=5 ms,                  Tac≤thr010 ms, thr1≤Tac≤thr1 20 ms,               Tac≥thr1

After TDCP reporting, PDSCH throughput would be stable with setting threshold. Even in scenario of high velocity, throughput would not be dropped down dramatically.

Observation 3. Higher Doppler scenario means higher time-varying channel, SRS should be scheduled more frequently to overcome aging issue; but lower SRS overhead is enough to get better throughput when channel is under lower UE velocity.

Proposal 4. SRS periodicity should be lower with lower time-varying channel, which is reported by UE with smaller value of normalized time domain correlation.

# Conclusion

In this contribution, we discussed TRS-based TDCP reporting. The following proposals have been made:

Proposal 1. As good trade-off between complexity and accuracy, Doppler spread is sufficient enough for TDCP report.

Proposal 2. As TDCP report, the overhead of time correlation is higher due to the presence of multiple lags in time correlation calculations than that of Doppler shift.

Proposal 3. To improve accuracy of estimation Doppler, advanced segmented algorithms on UE side based on specific UE velocity and delay spread, need to further study.

Proposal 4. SRS periodicity should be lower with lower time-varying channel, which is reported by UE with lower Doppler spread.

The following observations have been made:

Observation 1.  For given UE velocity, different delays result in different time correlations.

Observation 2. 20-slot delay has shown worse accuracy. Delay <= 5 slots can ensure the estimation for time variation of channel. 5-slot delay is better for smaller UE velocity (<=30km/h), whereas 1-slot delay is suitable in scenario of higher velocity.

Observation 3. Higher Doppler scenario means higher time-varying channel, SRS should be schedule more frequent to overcome aging issue; but lower SRS overhead is enough to get better throughput when channel is under lower UE velocity.

# Reference

| [1] | “Chairman’s Notes 3GPP RAN1 #112,” 27th February – 3rd March 2023. |
| --- | --- |
| [2] | 3GPP, TR38.214 Physical layer procedures for data. |
| [3] | R1-2300900, “Discussion on CSI enhancement”. |
| [4] | 3GPP, TR38.901 Study on channel model for frequencies from 0.5 to 100 GHz. |

# Appendix

## Link level assumptions

### TRS measurement

There is link level assumption for TRS measurement as Table-2. Based on our understanding, we have chosen parts of parameter for channel model. For delay spread 10, 100, 1000ns are corrsponding to very short, nominal, very long delay spread according to [4]. For UE velocity, we assum 3km/h, 30km/h, 120km/h are low, medium, high speed.

Table-2. Link level assumption for TRS based doppler accuracy

| Parameters | Values |
| --- | --- |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20 MHz |
| TRS bandwidth | 20 MHz |
| Channel model | TDL-A |
| Delay spread | 10, 100, 1000ns |
| UE velocity | 3 km/h, 30 km/h, 120 km/h |
| Antenna at UE | 1 |
| Antenna at gNB | 1 |
| Evaluation metrics for measurement accuracies | RMS error, Standard deviation (STD) |

### Simulation analysis among agreed use case

There is link level assumption for agreed use case as Table-3.

| Basic configuration |  |
| --- | --- |
| Parameters | Values |
| TDD Pattern | "D" "D" "D" "D" "D" "D" "D" "S" "U" "U" |
| Special Slot | [D G U] = [6 4 4] |
| System bandwidth | 20 MHz, 51 PRBs |
| Carrier frequency and subcarrier spacing | 2 GHz with 30 kHz SCS |
| Channel model | CDL-A |
| Delay spread | 10ns |
| UE velocity | 3 km/h, 50 km/h, 100 km/h |
| Number of UE(s) | 1 |
| Number of layer(s) per UE | 2 |
| PDSCH configuration |  |
| Parameters | Values |
| MCS | 8, 38.214 Table 5.1.3.1-1 |
| DMRS configuration type | 1 |
| Front-loaded DMRS length | 2 |
| Additional DMRS | 0 |
| SRS configuration |  |
| Parameters | Values |
| Resource type | Periodic |
| Frequency hopping | OFF |
| Repetition | 1 |
| BSRS | 0 |
| BHop | 1 |
| Frequency start | 0 |
| NRRC | 0 |
| Number of SRS symbol(s) | 1 |
| CSRS | 13 |
| KTC | 2 |
| SRS period (slot) | 10/20/40 |
| Number of SRS port(s) | 2 |