3GPP TSG RAN WG1 #112bis-e	R1-2303085

e-Meeting, April 17th – April 26th, 2023

Agenda item:		9.12.3

Source:		Mavenir

Title:	Discussion on solutions for NR dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:		Discussion and Decision

# 1	Introduction

3GPP Work Item Description [1] provides a potential target in R18. The objective of this work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM. Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1).

In the previous meeting (RAN WG1 #111 and #112), there was broad agreement that supported dynamically scheduled PUSCH dynamic waveform switching indication from UL scheduling DCI and assumed that supporting new 1-bit field for dynamic waveform indication from UL scheduling DCI.

Agreement

For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI.

Note: no change of the current size alignment procedure between UL DCI and DL DCI.

Conclusion

There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18.

Agreement

Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant.

Conclusion

The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

Agreement

Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching:

- Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform

  - Target waveform can be same or different from waveform of an actual PUSCH transmission

  - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission

  - FFS determination of target waveform, target RB allocation, target modulation order

  - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms

- PHR triggering enhancements, e.g.

  - Network-triggered PHR

  - PH becomes lower (higher) than a threshold

  - PHR triggered by waveform switching

- Reporting of recommended waveform or request to switch waveform

- Other solutions not precluded

We provide a waveform indication definition of the 1-bit for both DCI 0_0 and 0_1, besides, we consider the CG type 2, UL CA and assistant information reporting.

# 2	Dynamic switching between DFT-S-OFDM and CP-OFDM

we focus our attention on how to achieve dynamic switching between DFT-S-OFDM and CP-OFDM. This contribution proposes that not only DCI 0_1 but also DCI 0_0 could be used for dynamic switching, for DCI 0_0 could cover more scheduling scenarios than DCI_0_1 and DCI 0_2, for example, Msg3 and coverage limitation cases.

This contribution provides a UL waveform indicator option when 1-bit adding for dynamic switching to DCI format 0_0 and 0_1, as follows:

Table 1: UL waveform indicator

| Value of UL waveform indicator | UL waveform |
| --- | --- |
| 0 | CP-OFDM |
| 1 | DFT-S-OFDM |

Propose1: Not only DCI 0_1 but also DCI 0_0 could be used for dynamic switching and UL waveform indicator: 1 bit according to Table 1.

It has been agreed that new 1-bit field for dynamic waveform indication from UL scheduling DCI. So if the 1-bit is used for configured grant type 2, the effort can not be added for specification. In addition, configured grant type 2 is usually for URLLC service, and it is necessary to switch the UL waveform for this kind of service. we suggest dynamic switching is applicable in R18 for CG type 2.

Propose2: Dynamic switching is applicable in R18 for CG type 2.

A company proposes to discuss whether dynamic switching should consider UL CA scenario [2]. We support considering UL CA scenario because it is effective for defining specifications possibly. First, we should analyze the influence, for example, If DCI 0_0 or DCI 0_1 or DCI 0_2 is used for new 1-bit and a DCI only schedules one cell, it will be the same for non-CA scenario and CA scenario.

Propose3: UL CA scenario should be considered.

As we all know, CP-OFDM waveform is more suitable for cell center user and DFT-S-OFDM waveform is more suitable for cell edge user, and gNB has capacity to differentiate cell center and cell edge user, for example, SRS SINR or RSRP. In addition, if UE reports assistant information for gNB to switch UL waveform, it will increase the UL overhead.

Propose4: It is not necessary to report assistant information for switching UL waveform, i.e. gNB has capacity to differentiate cell center and cell edge user, not need additional information.

After switching UL waveform, the transmitted power of UE may change because UL waveform will impact the max transmitted power, and MCS and RB allocation may different for different UL waveforms. We think gNB can estimate max transmitted power according to MPR and AMPR, and gNB can calculate the transmit power of UE according to the new MCS and RB.

Even though gNB can not estimate PHR of the target waveform accurately, after switching UL waveform, UE can report PHR of target waveform when phr-ProhibitTimer expires. It is worth discussing whether reduce the time of phr-ProhibitTimer or report PHR of the target waveform after switching UL waveform.

Propose5: It should be further study whether reduce the time of phr-ProhibitTimer or report PHR of the target waveform after switching UL waveform.

# 3	Conclusions

Propose1: Not only DCI 0_1 but also DCI 0_0 could be used for dynamic switching and UL waveform indicator: 1 bit according to Table 1.

Table 1: UL waveform indicator

| Value of UL waveform indicator | UL waveform |
| --- | --- |
| 0 | CP-OFDM |
| 1 | DFT-S-OFDM |

Porpose2: Dynamic switching is applicable in R18 for CG type 2.

Propose3: UL CA scenario should be considered.

Propose4: It is not necessary to report assistant information for switching UL waveform, i.e. gNB has capacity to differentiate cell center and cell edge user, not need additional information.

Propose5: It is worth discussing whether reduce the time of phr-ProhibitTimer or report PHR of the target waveform after switching UL waveform.

# 4. References

[1] RP-220937  Revised WID on Further NR coverage enhancements

[2] R1-2300864 Discussion on dynamic waveform switching