3GPP TSG RAN WG1 #112bis-e	R1-2303087

e-Meeting, April 17th – April 26th, 2023

Agenda item:		9.2.2.1

Source:		Mavenir

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:		Discussion and Decision

# 1  Introduction

According to discussion in 3GPP RAN1#112 meeting, some progress has been made on evaluation methodology for CSI feedback enhancement. The following agreements and conclusions were extracted from the Chair’s note [1]:

Working Assumption

For the initial template for AI/ML-based CSI compression without generalization/scalability verification achieved in the working assumption in the RAN1#111 meeting, X, Y and Z are determined as:

- X is <=80bits

- Y is 100bits-140bits

- Z is  >=230bits

Agreement

For how to separate the templates for different training types/cases for AI/ML-based CSI compression without generalization/scalability verification, the following is considered:

- The determined template in the RAN1#111 working assumption is entitled with “1-on-1 joint training”

- A second separate template is introduced to capture the evaluation results for “multi-vendor joint training”

    - Note: this table captures the results for the joint training cases of 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, or N>1 NW part models to M>1 UE part models. An example is multi-vendor Type 2 training.

- A third separate template is introduced to capture the evaluation results for “separate training”

- FFS: additional KPIs for each template, e.g., overhead, latency, ect.

In this contribution, we provide our views on further details for evaluation methodology and share some initial evaluation results.

# 2  CSI compression

## 2.1 AI/ML Model Description

An autoencoder architecture is applied to compress and reconstruct CSI matrix. There are encoder and quantization modules at the UE side, de-quantization and decoder modules at the BS side. The encoder takes the original eigenvectors V as the input and compresses it into a codeword. Then, the codeword is sent to quantization module and fed back to the BS as a bit stream. Dequantization module recovers the codeword from the received bit stream. After that, the BS reconstructs original eigenvectors from codeword.

Figure 1 The structure of AI/ML based CSI compression

Specifically, the input of the encoder includes eigenvectors for N subbands, which are formulated as V=V1…VN, where Vn denotes the eigenvector for the n-th subband. The codewords can be formulated as VEn=fencoder(V), where fencoder(∙) represents the function of the encoder. The quantizer at the UE side maps the floating-point vector to a quantized bit stream for CSI feedback. The quantization module can be formulated as VQ=fquantizer(VEn), where fquantizer(∙) represents the function of the quantizer. In our simulation, scalar quantization is used. The de-quantizer recovers the compressed CSI from the feedback CSI bit stream and sends it as the input to the decoder. The de-quantized CSI can be formulated as VDe=fdequantizer(VQ) where fdequantizer(∙)represents the function of the de-quantizer. The decoder recovers the eigenvectors. The recovered eigenvectors can be formulated as V=fdecoder(VDe), where fdecoder(∙) represents the function of the decoder.

## 2.2 Generalization performance

The main focuses is to study the generalization impact of different scenario(CDL_A/CDL_B).  The simulation assumption show in Table2.

Case 1:  the training data set is generated for CDL_A channel model with 30k samples, and testing with the same  CDL_A channel model. The SGCS and GCS performance is shown in Fig. 2.

Case 2:  the training data set is generated for CDL_B channel model with 30k samples, and testing with the same  CDL_B channel model. The SGCS and GCS performance is shown in Fig. 2.

Case 3:  the training data set is generated for mix  CDL_A and CDL_B channel model with 30k samples, and testing with the  CDL_A channel model. The SGCS and GCS performance is shown in Fig. 2.

Case 4:  the training data set is generated for mix  CDL_A and CDL_B channel model with 30k samples, and testing with the same  CDL_B channel model. The SGCS and GCS performance is shown in Fig. 2.

Case 5:  the training data set is generated for CDL_A channel model with 30k samples, and testing with the same  CDL_B channel model. The SGCS and GCS performance is shown in Fig. 2.

Case 6:  the training data set is generated for CDL_B channel model with 30k samples, and testing with the same  CDL_A channel model. The SGCS and GCS performance is shown in Fig. 2.

Figure 2 Evaluation results for CSI compression with model generalization

Observation 1: For different scenarios, the SGCS degradation is about 1%~2% when training set and testing set are mismatching.

Observation 2: Training on mixing dataset with CDL_A and CDL_B can improve the generalization performance (about 1.4%/0.6%) of AI/ML model for CDL_A and CDL_B.

Proposal1: Mixed dataset should be considered as a effective way to increase AI model generation.

Proposal2: AI model performance should be evaluated on datasets with different scenario and configuration.

# 3 Conclusions

In this contribution, we discuss the evaluations on AI/ML for CSI feedback enhancement and provide preliminary simulation results. We have the following observations and proposals.

Observation 1: For different scenarios, the SGCS degradation is about 1%~2% when training set and testing set are mismatching.

Observation 2: Training on mixing dataset with CDL_A and CDL_B can improve the generalization performance (about 1.4%/0.6%) of AI/ML model for CDL_A and CDL_B.

Poposal1: Mixed dataset should be considered as a effective way to increase AI model generation.

Proposal2: AI model performance should be evaluated on datasets with different scenario and configuration.

Table 1. Evaluation results for CSI compression with model generalization/scalability, [Max rank value], [Scenario/configuration]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone | Transformer |  |  |
|  | Pre-processing | / |  |  |
|  | Post-processing | / |  |  |
|  | Number of parameters/M | 8 |  |  |
| CSI reconstruction part | AL/ML model backbone | Transformer |  |  |
|  | [Pre-processing] | / |  |  |
|  | [Post-processing] | / |  |  |
|  | Number of parameters/M | 8 |  |  |
| Common description | Input type | Raw channel matrix |  |  |
|  | Output type | Raw channel matrix |  |  |
|  | Quantization /dequantization method | Scalar quantization |  |  |
|  | Generalization/Scalability method description if applicable, e.g., truncation, adaptation layer, etc. | / |  |  |
|  | Input/output scalability dimension if applicable, e.g., N>=1 NW part model(s) to M>=1 UE part model(s) | / |  |  |
| Dataset description | Ground-truth CSI quantization method | Float32 |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Generalization Case 1 | Train (setting#A, size/k) | CDL_A#30k |  |  |
|  | Test (setting#A, size/k) | CDL_A#12k |  |  |
| GCS, layer 1 | CSI feedback payload X(64bit) | 0.9759 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9802 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9865 |  |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) | 0.9524 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9608 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9732 |  |  |
| Generalization Case 2 | Train (setting#B, size/k) | CDL_B#30k |  |  |
|  | Test (setting#B, size/k) | CDL_B#12k |  |  |
| GCS, layer 1 | CSI feedback payload X(64bit) | 0.9698 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9784 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9843 |  |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) | 0.9405 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9573 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9678 |  |  |
| Generalization Case 3 | Train (setting#A+#B, size/k) | CDL_A&CDL_B#30k |  |  |
|  | Test (setting#A, size/k) | CDL_A#12k |  |  |
| GCS, layer 1 | CSI feedback payload X(64bit) | 0.9683 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9731 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9802 |  |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) | 0.9376 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9469 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9610 |  |  |
| Generalization Case 4 | Train (setting#A+#B, size/k) |  | CDL_A&CDL_B#30k |  |
|  | Test (setting #B, size/k) |  | CDL_B#12k |  |
| GCS, layer 1 | CSI feedback payload X(64bit) |  | 0.9614 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9716 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9812 |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) |  | 0.9243 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9440 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9628 |  |
| Generalization Case 5 | Train (setting#A, size/k) |  | CDL_A#30k |  |
|  | Test (setting #B, size/k) |  | CDL_B#12k |  |
| GCS, layer 1 | CSI feedback payload X(64bit) |  | 0.9569 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9602 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9783 |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) |  | 0.9157 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9220 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9571 |  |
| Generalization Case 6 | Train (setting#B, size/k) |  | CDL_B#30k |  |
|  | Test (setting #A, size/k) |  | CDL_A#12k |  |
| GCS, layer 1 | CSI feedback payload X(64bit) |  | 0.9432 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9567 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9734 |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) |  | 0.8896 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9153 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9475 |  |

# 4. References

- Chairman’s notes, RAN1 #112.

# Appendix

Table 2 Simulation Assumption

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz |
| Nt | 32 |
| Nr | 4 |
| Channel model | CDL-A, CDL-B |
| Delay spread | 300ns |