3GPP TSG RAN WG1 #112bis-e	        		              R1-2303103

e-meeting, April 17th – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on timer expiration for SSSG switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ UE_pow_sav_enh |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - It is ambiguous whether switching to SSSG 0 applies upon timer expiration in searchSpaceGroupIdList-r17 The specifications capture the following: “When a UE is provided searchSpaceGroupIdList, the UE resets PDCCH monitoring according to search space sets with group index 0, if provided by searchSpaceGroupIdList.” and “When the timer expires in a first slot, the UE monitors PDCCH on the serving cell according to search space sets with group index 0 starting in a second slot that - is not earlier than Pswitch symbols after the first slot when μ∈0, 1, 2, 3, - is a first slot in a slot group of Xs slots that is not earlier than Pswitch symbols after the first slot when μ∈5, 6, - is not earlier than a slot where a PDCCH skipping duration expires, if applicable” Although the latter text is generic, the former text is ambiguous as it includes only searchSpaceGroupIdList and does not clarify that the reset is upon the timer expiration. It is suggested to add “if provided by searchSpaceGroupIdList” in the latter text and to remove the earlier text. - The Rel-17 parameter, searchSpaceGroupIdList-r17, is missing from the following statement “If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList.” - Incorrect RRC parameter name for PDCCHSkippingDurationList. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove duplicated/incomplete text to clarify applicability of the timer. - Add searchSpaceGroupIdList-r17 as a condition for applicability of the procedures. - Correct RRC parameter name from PDCCHSkippingDurationList to pdcch-SkippingDurationList. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous/incomplete specifications, incorrect reference to an RRC parameter. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR clarifies existing behavior has no impact on UE/gNB procedures. It can be considered as an alignment CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.4	Search space set group switching and skipping of PDCCH monitoring

A UE can be provided

-	a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell,

-	a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell.

If the UE is not provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17 for a search space set, the following procedures that are based on search space set group switching are not applicable for PDCCH monitoring according to the search space set.

A UE can be provided a set of durations by pdcch-SkippingDurationList for Type3-PDCCH CSS set or USS set for PDCCH monitoring on an active DL BWP of a serving cell. If the UE is not provided PDCCHSkippingDurationList, the following procedures related to skipping of PDCCH monitoring are not applicable.

If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17.

A UE can be provided by searchSpaceSwitchDelay a number of symbols Pswitch where a minimum value of Pswitch is provided in Table 10.4-1 for UE processing capability 1 and UE processing capability 2 and SCS configuration μ. UE processing capability 1 for SCS configuration μ applies unless the UE indicates support for UE processing capability 2.

*** Unchanged text omitted ***

If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring an active DL BWP of on a serving cell and the timer is running, the UE

-	resets the timer after a slot of the active DL BWP of the serving cell if the UE detects a DCI format in a PDCCH reception in the slot for with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI

-	otherwise, decrements the timer value by one after a slot of the active DL BWP of the serving cell

When the timer expires in a first slot, the UE monitors PDCCH on the serving cell according to search space sets with group index 0, if provided by searchSpaceGroupIdList, starting in a second slot that

-	is not earlier than Pswitch symbols after the first slot when μ∈0, 1, 2, 3,

-	is a first slot in a slot group of Xs slots that is not earlier than Pswitch symbols after the first slot when μ∈5, 6,

-	is not earlier than a slot where a PDCCH skipping duration expires, if applicable

*** Unchanged text omitted ***