3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303105

e-Meeting, April 17th – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for BWP switching with CBG-based transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If codeBlockGroupTransmission is configured, in DCI format 0_1, CBGTI field is 0 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1 and 2, 4, 6, 8 bits if the number of scheduled PUSCH indicated by the Time domain resource assignment field is equal to 1. In case where the DCI has 0 bit CBGTI field (if the number of scheduled PUSCH indicated by the Time domain resource assignment field in the active BWP is larger than 1) and indicates a new BWP where more than 0 bits CBGTI field is required (if the number of scheduled PUSCH indicated by the Time domain resource assignment field in the indicated BWP is equal to 1), zeros are padded to CBGTI field for interpretation according to Clause 12 of TS38.213. After the zero padding, the CBGTI field is all-zeros so that UE’s interpretation is the scheduled PUSCH contains no CBGs in the indicated BWP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following condition to allow that all CBGs are scheduled in the indicated BWP. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part, - if codeBlockGroupTransmission is configured, and - if CBGTI field is 0 bit and the number of scheduled PUSCH indicated by the Time domain resource assignment field in the indicated bandwidth part is equal to 1, the UE assumes all CBGs in the scheduled PUSCH are scheduled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BWP switching with CBG-based PUSCH transmission is broken |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.3.1.1.2	Format 0_1

=============================== Unchanged Text Omitted ===================================

-	CBG transmission information (CBGTI) – 0 bit if higher layer parameter codeBlockGroupTransmission for PUSCH is not configured or if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1; otherwise, 2, 4, 6, or 8 bits determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock for PUSCH.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

- if codeBlockGroupTransmission is configured, and

- if CBGTI field is 0 bit and the number of scheduled PUSCH indicated by the Time domain resource assignment field in the indicated bandwidth part is equal to 1,

the UE assumes all CBGs in the scheduled PUSCH are scheduled.

=============================== Unchanged Text Omitted ===================================