3GPP TSG RAN WG1#112-bis-e			R1-233114

Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.2

Source: 	Samsung

Title: 	Views on CSI enhancements

Document for:	Discussion and Decision

# Introduction

This contribution provides Samsung’s view on the key issues related to Rel.18 CSI enhancements that are prioritized for RAN1#112-bis-e (as announced by the FL). It also discusses other issues that are relevant for further discussions.

# Type II codebook refinement for coherent-JT

## Key issues

    - Issue 1 (W2 quantization, working assumption)

[3] Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer: - Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table enhancement - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported - Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111) - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

Regarding the working assumption on Alt3 for amplitude grouping, the main argument from the proponents of Alt3 is the power imbalance across TRPs in case of inter-site scenarios with large ISD (e.g. 500m), which may require reporting of this power imbalance via different amplitude groups, one per TRP (2N groups in total). In our view, since the SCI is across TRPs, coefficients associated with different TRPs need to be normalized with respect to the strongest coefficient across all TRPs, implying that the W2 coefficients after normalization in some sense shall already capture the power level of different TRPs. This is regardless of the scenarios or ISD values. We verify this via simulation results provided in Section 2.2, wherein we show that there is no gain with Alt3 over Alt1 in inter-site scenarios with small (200m) as well as large (500m) ISDs. We therefore propose to revert the working assumption.

Observation 1: due to one SCI across TRPs, the W2 coefficients for different TRPs (after normalization with the strongest coefficient) capture the different power level across TRPs, implying that there is no need for per-TRP per-polarization reference amplitude reporting (i.e. Alt3)

Proposal 1: Revert the working assumption on the additional support for Alt3 for the amplitude grouping

- Note that the working assumption was included with the understanding that the gain of Alt3 over Alt1 (already agreed to be supported) can be demonstrated – not simply that “as long as Alt3 is not broken, the WA should be confirmed as agreement.”

    - Issue 2 (mode-1: Alt1 vs 2)

[1] Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FD basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

We support Alt1. For mode1, independent per-TRP FD basis reporting can be w.r.t. a reference FD basis (of a reference TRP). The reporting of a relative FD basis offset for each TRP ϕn (except the reference, which can be set to 0) can help align FD basis (or the delay profile) of different TRPs. Furthermore, the relative FD basis Wf (hence delay profile) after cyclic shift or alignment can be similar. The main advantage of Alt1 over Alt2 is that it can unify the functionality of mode 1 and mode 2 (which is a design principle included in the agreement on the support of mode 1 and mode 2), reduce overhead (when compared with Alt2), and reduce UE computational complexity (due to one vs per-TRP Wf). Simulation results comparing these alternatives are provided in Section 2.2. The results demonstrate that there is no clear performance gap between Alt1/2.

Additionally, the simulation results show that Alt 1 with TRP-specific q3 (oversampling/rotated value) yields a 2~3% average UPT gain over Alt 1 with TRP-common q3. Hence, we prefer to introduce TRP-specific q3.

Regarding layer-common vs layer-specific φn and q3, in our view, it is sufficient to support layer-common φn and q3, since they can be utilized to preprocess/compensate the FD offset (mainly caused from delay difference) before layer extraction via computing SVD for the per-TRP channel.

We can have a joint indicator for (φn,q3), which can indicate a value from an alphabet 0,1,⋯,N3O3-1 or 0,1O3,⋯,O3-1O3,1,⋯,N3-1O3. However, in our view, for reporting, a subset of the alphabet (e.g. a window) should be sufficient, since the delay difference across TRPs favorable for CJT operation should not be large, hence the size of alphabet can be reduced (implying overhead can be reduced) especially when N3 is large. Furthermore, for the case of Rel-17 based CJT codebook, the gNB performs beamforming (for delay compensation or channel shortening) on CSI-RS ports, where the beamforming can be designed to compensate for delay difference across TRPs assuming FDD partial reciprocity. In this case, the window size can be very small (e.g. around 4). Additionally, a reference CSI-RS resource indicator is also needed to indicate the reference CSI-RS resource for reporting φn.

Observation 2: on FD basis reporting for mode-1, when compared with Alt 2, Alt1 can

- unify the functionality of mode 1 and mode 2 (per agreement on support of mode1/2),

- achieve similar performance with lower overhead of FD basis reporting as well as lower complexity of FD basis calculation

Observation 3: the simulation results show that Alt 1 with TRP-specific q3 (oversampling/rotated value) yields a 2~3% average UPT gain over Alt 1 with TRP-common q3. Hence, we prefer to introduce TRP-specific q3.

Observation 4: a subset (e.g. window) of 0,1,⋯,N3O3-1 (or 0,1O3,⋯,O3-1O3,1,⋯,N3-1O3) should be enough as an alphabet for φn reporting, since

- the delay difference across TRPs favorable for CJT operation should not be large, and

- the delay difference can be compensated by gNB implementation when Rel-17 based CJT operation is used with delay/angle reciprocity.

Proposal 2:

- support Alt1 for mode-1 FD basis reporting, where layer-common TRP-specific φn (and q3) is reported.

- support a subset (e.g. window) of 0,1,⋯,N3O3-1 as an alphabet for φn reporting.

- support reporting reference CSI-RS resource n* for indicating the relative FD offset values, i.e., φn for ∀n≠n*

    - Issue 3 (parameter combination linkage)

[1] Agreement On the Type-II codebook refinement for CJT mTRP, only support NL ={2,4} as additional candidate values to NL=1. - FFS: Additional restriction(s) depending on the configured value for NTRP [1] Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {Ln} for the higher-layer-configured value of NTRP (FFS by RAN1#112: whether the bracketed permutations are also supported): - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {n} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement FFS: Whether the total number of Ln is a UE capability NTRP; {Ln} combination. 1; {2}. {4}. {6} (analogous to legacy, only for total # ports =32, rank 1-2, R=1. 2; {2,2}. {2,4}, [{4,2}]. {4,4}. 3; {2,2,2}. {2,2,4} [and its other permutations]. {4,4,4}. 4; {2,2,2,2}. {2,2,2,4} [and its other permutations]. {2,2,4,4} [and its other permutations]. {4,4,4,4}. [1] Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {pv,} from where the value of {pv,} is gNB-configured via higher-layer (RRC) signaling: - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {M} for Rel-17-based refinement are derived from the supported combinations of {pv ,} for Rel-16-based refinement pv for layers 1-4; ; Condition(s). {1/8, 1/8, 1/16, 1/16}; ¼; --. ½; --. {1/4, 1/4, 1/8, 1/8}; ¼ (*); --. ½ (*); --. {1/4, 1/4, 1/4, 1/4}; ¾ (*); --. {1/2, 1/2, 1/2, 1/2}; ½; - Only applicable when NTRP≤3 and NL=1 - Optional. (*) Supported by legacy Rel-16 [1] Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, regarding the list of supported combinations of {Ln}, only support the following additional combinations: NTRP; {Ln} combination. 2; {4,2}. 3; {2,4,2}, {4,2,2}. No other permutations are supported. FFS: For NTRP>1, in addition to the supported combinations/permutations, whether to support at least one additional combination where at least one of the Ln values (n=1, …, NTRP) is 6 [1] Agreement On the Parameter Combination of Type-II codebook refinement for CJT mTRP, support linkage between the list of supported {Ln} combinations and list of supported {pv,} combinations via pairing each combination for {pv,} with at least one combination for {Ln}, for each NTRP value. - FFS (by RAN1#112bis-e): The exact list of supported pairs/linkage, or restriction of {Ln} when paired to each of {pv,} - FFS (by RAN1#112bis-e): Whether/How to support configuration signalling for indicating the linkage - Note: While no additional codebook parameter will be introduced, the total number of SD basis vectors across CSI-RS resources can still be used as a criterion for choosing the supported pairs/linkage Offline Proposal 1.C.1: On the Parameter Combination of Type-II codebook refinement for CJT mTRP, only the following linkages are supported (marked ‘x’) - For NTRP =1, - For Rel-16 eType-II based: fully reuse seven out of the eight Parameter Combinations from Rel-16 eType-II as indicated in the table below - FFS (by RAN1#112bis-e): whether to add one more Parameter Combination for L=4 based on the legacy Rel-16 eType-II FD combo {½, ½, ¼, ¼; ½} or the agreed FD combo {½, ½, ½, ½; ½}, or not to add from the indicated seven below - For Rel-17 FeType-II based, fully reuse the eight Parameter Combinations from Rel-16 eType-II - For NTRP >1, only the following linkages are supported (marked ‘x’) NTRP; SD combo; FD combo {pv},. {1/8, 1/8, 1/16, 1/16}, ¼; {1/8, 1/8, 1/16, 1/16}, ½; {1/4, 1/4, 1/8, 1/8}, ¼; {1/4, 1/4, 1/8, 1/8}, ½; {1/4, 1/4, 1/4, 1/4}, ¾; {1/2, 1/2, 1/2, 1/2}, ½. 1; 2; X; X. 4; X; X; X. 6 w restriction; X; X. 2; {2,2}; x. {2,4} {4,2}; x. {4,4}; x; x; x. 3; {2,2,2}; x; x. {2,2,4} {2,4,2} {4,2,2}; x; x; x. {4,4,4}; x; x; x; x; x; x. 4; {2,2,2,2}; x; N/A. {2,2,2,4}; x; N/A. {2,2,4,4}; x; x; N/A. {4,4,4,4}; x; x; x; N/A.

We support the offline proposal 1.C.1 as we have verified that the selected linkages yield good performance overall compared to other linkages and the overhead of them are well uniformly-spaced. The detailed analysis on the parameter combinations are along with the SLS results shown in Section 2.2.

On additional {Ln} combo including 6 for NTRP>1, we don’t support due to 1) the high UE processing complexity, and 2) worse performance than other combination using Ln={2,4} under a same Ltot constraint that we have observed in our SLS results.

Proposal 3: for parameter combination,

- support offline proposal 1.C.1

- Do not support Ln=6 for NTRP>1

    - Issue 4 (CBSR)

[1] Agreement On the Type-II codebook refinement for CJT mTRP, regarding CBSR, at least for restricting SD basis selection, the legacy CBSR scheme is fully reused for each of the RRC-configured NTRP CSI-RS resources (resulting in CSI-RS-resource-specific SD beam group restriction) - FFS: Whether amplitude restriction is CSI-RS-resource-common or specific, and soft vs hard restriction - FFS: Whether CBSR can be configured to be off for a CSI-RS resource The same rank restriction is applied across NTRP CSI-RS resources

We support TRP-common soft amplitude restriction, averaging over TRPs and FD beams since

- The max overhead for per-TRP amplitude restriction is large, (2N1N2 bits per beam group  8N1N2 bits for 4 beam groups  4 TRPs x 8N1N2 bits = max 512 bits (when N1N2=16))

- Hard amplitude restriction only is too restrictive from NW perspective, and

- For co-located TRPs, the amplitude restriction can be the same across TRPs.

There is one issue regarding (N1,N2) configuration: (N1,N2) is configured jointly with CBSR in legacy (n1-n2-codebookSubsetRestriction), but for CJT (N1, N2) is the same for all TRPs. It will be up to RAN2 decision, but we suggest to consider either one of the followings for the configuration:

- Opt1: separated out n1-n2 from CBSR parameter, since (N1, N2) is TRP-common.

- Opt2: reuse legacy, and include the restriction that (N1, N2) is the same for each n1-n2-codebookSubsetRestriction.

Proposal 4: For CBSR,

- support TRP-common soft amplitude restriction, averaging over TRPs and FD beams.

- support a separate configuration of (N1,N2) from CBSR

    - Issue 5 (CSI omission)

[1] Agreement On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, down-select between the following three alternatives (by RAN1#112-bis where n denotes the n-th CSI-RS resource): - Alt1. Prio(,l,m,n)=( k=0n-12Lk) .N.RI.P(m)+N.RI.l(n)+N.n - Note: This implies that CSI-RS resource is designated the highest priority - Alt2. Prio(,l,m,n)=2L’.Q(n).RI.N3+2L’.RI. P(m)+RI.l(n)+ - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis) - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI. - Alt3. Replace SD basis index l in legacy Prio calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+ FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m Offline Proposal 1.E.1: On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, support (Alt3) replacing SD basis index l in legacy Prio calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+ - FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m

We support offline proposal 1.E.1.

We have verified that joint SVD is more beneficial than per-TRP SVD in CJT operation. This means that NZCs/SD beams/FD beams are selected based on the composite channel of mTRP and the selected NZCs across TRPs are intertwined with each other in terms of priority (since it is not separately selected in per-TRP manner). In this regard, we don’t think Alt2 has a technical merit, where the design principle of Alt2 is to prioritize keeping all NZCs (across all layers) of the TRP including the SCI, which conflicts the joint-SVD principle of precoder selection (hence it can’t be a proper fallback mode).

Also, we already have the dynamic TRP selection feature at the UE side, which can be used instead when all NZC omission for some TRPs is needed. For example, the UE doesn’t have to use UCI omission for certain TRPs under the conflicted precoder selection assumption, but can use dynamic TRP selection feature and select the precoder (NZCs) for the composite channel of the selected TRPs. If UCI omission is needed even in this case, Alt3 or Alt1 (i.e., at least having the same priority of SD and TRP) should be more aligned with the precoder selection. The SLS results shown in Section 2.2 verify our statement.

Finally, the UCI omission is an emergency solution to a problem (i.e. insufficiency of UL resources) that happens rarely. There is no need to introduce any complicated solution such as Alt2 since it does not bring any benefits in return.

Proposal 5: for UCI omission, support offline proposal 1.E.1.

    - Other issues (UCI parameters)

The list of UCI parameters has been agreed, except for the FD basis reporting in Mode 1. As discussed, we support N-1 indicators for φn for n≠n* and a CSI-RS resource indicator (in addition to FD basis vector selection indicator). These indicators can be located in Group 0 of CSI part 2.

Proposal 6: for mode1, support including indicator(s) indicating φn and a reference CSI-RS resource indicator in Group 0 of CSI part 2.

## Simulation results

We provide system-level simulation (SLS) results on (1) performance comparison for all supported linkages/pairs of {Ln} and (pv,β), (2) performance comparison of UCI omission Alt2 and Alt3, (3) performance comparison for two alternatives on FD basis selection offset for Mode 1, and Mode 2, and (4) performance comparison for different reference grouping methods, i.e., Alt1, and Alt3, of Issue 1, in inter-site inter-cell scenarios with ISD=500m and 200m. The simulation assumptions are summarized in Appendix A.

Evaluation 1: performance comparison for all supported linkages/pairs of {Ln} and (pv,β)

We performed SLS simulations to compare Mode 2 performance for all supported linkages/pairs of (pv,β) for each {Ln,n=1,⋯,NTRP} combination for each case of NTRP=1,2,3,4.

We considered

- intra-site inter-cell scenarios for NTRP=1 and NTRP=3 cases and

- intra-site intra-cell scenarios (D-MIMO) for NTRP=2 and NTRP=4 cases.

- The number of ports per TRP is shown in each figure.

- UE is allowed to select dynamic rank up to 4.

- UE is allowed to perform dynamic TRP selection.

Based on the results shown in Figure 1 and Figure 2, for NTRP=2,3,4, we down-select linkages that yield good performance in each scenario. The linkages shown in Table 1 are what we don’t want to remove from the default/supported linkages/pairs derived from the agreed SD and FD parameter values.

Table 1: selected linkages/pairs based on the SLS results

| NTRP | SD combo | FD combo {pv}, |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | {1/8, 1/8, 1/16, 1/16}, ¼ | {1/8, 1/8, 1/16, 1/16}, ½ | {1/4, 1/4, 1/8, 1/8}, ¼ | {1/4, 1/4, 1/8, 1/8}, ½ | {1/4, 1/4, 1/4, 1/4}, ¾ | {1/2, 1/2, 1/2, 1/2}, ½ |
| 2 | {2,2} | o |  |  |  |  |  |
|  | {2,4} {4,2} | o |  |  |  |  |  |
|  | {4,4} |  | o |  | o |  | o |
| 3 | {2,2,2} | o | o |  |  |  |  |
|  | {2,2,4} {2,4,2} {4,2,2} | o | o |  | o |  |  |
|  | {4,4,4} | o | o | o | o | o | o |
| 4 | {2,2,2,2} | o |  |  |  |  | N/A |
|  | {2,2,2,4} | o |  |  |  |  | N/A |
|  | {2,2,4,4} |  |  |  | o | o | N/A |
|  | {4,4,4,4} |  | o |  | o | o | N/A |

Figure 1: Average UPT gain vs overhead w.r.t. supported {Ln} and (pv, beta) for 16 ports per TRP for NTRP>1 and 32 ports per TRP for NTRP=1

Figure 2: Average UPT gain vs overhead w.r.t. supported {Ln} and (pv, beta) for 8 and 32 ports per TRP for NTRP>1

Evaluation 2: performance comparison of UCI omission Alt2 and Alt3

We considered intra-site intra-cell (D-MIMO) scenario with allowing dynamic TRP selection at the UE side. We modelled, for the sake of simplicity, UCI omission happens for every CSI report for each UE.

Figure 3: Average UPT loss of Alt 2 over Alt 3 w.r.t. legacy parameter combination

Based on the results shown in Figure 3, we can make the following observation.

Observation 5: UCI omission with Alt3 is more beneficial than Alt2 in CJT operation.

Evaluation 3: performance comparison for two alternatives on FD basis selection offset for Mode 1, and Mode 2

Figure 4: Average UPT gain vs overhead w.r.t. different FD basis selection offset methods, i.e., Mode 1 with Alt1 and Alt2, and Mode 2 in inter-site inter-cell scenario

To compare the performance of Alt1 and Alt2 for Mode 1 (and Mode 2 as a reference), we considered R=2 to satisfy N3>19. The detailed selection schemes of Alt1/Alt2 in our SLS results are as follows:

- Alt1: one Minitial∈{-2Mv+1,-2Mv+2,⋯,0} value for a reference TRP + one FD relative offset in {0,1,,⋯,N3-1} for each of remaining N-1 TRPs + common Mv FD basis vector selection across N TRPs

- Alt2: one Minitial∈{-2Mv+1,-2Mv+2,⋯,0} value for each of N TRPs + independent Mv FD basis vector selection for each of N TRPs

For Mode 1 with Alt1 (as well as Mode 2), we considered the case of joint SVD for mTRP channels, whereas for Mode 1 with Alt2, we considered both the cases of joint SVD and per-TRP SVD for mTRP channels. Note that during the discussion of Mode 1 vs Mode 2, the proponents of Mode 1 strongly argued (despite the absence of evidence that Mode 1 provides performance benefit over Mode 2) that the Alt2 structure lends itself to a simpler PMI calculation as per-TRP PMI search is possible (i.e. per-TRP SVD).

Here, we use the UPT with Alt1 of paraComb=1 as the reference UPT value (which we regard as 100%). As shown in Figure 4, we can make the following observation.

Observation 6:

- Mode 1 with Alt 2 per-TRP SVD (the advocated lower complexity benefit for Alt2) incurs ~4% UPT loss (for the same PMI overhead) over Mode 2.

- Overall, Mode 2 and Mode 1 with Alt 1 and Alt 2 using joint-SVD operation yield similar performance.

- Mode 1 with Alt 2 needs additional UE processing to find per-TRP FD basis vectors for the case of joint-SVD operation, compared to Mode 2 or Mode 1 with Alt 1 – thereby resulting in higher UE complexity

Figure 5: Average UPT gain vs overhead w.r.t. different FD basis selection offset methods, i.e., Mode 1 with Alt1 and Alt2, and Mode 2 in inter-site inter-cell scenario, TRP-common q3 and O3=4

Figure 6: Average UPT gain vs overhead w.r.t. different FD basis selection offset methods, i.e., Mode 1 with Alt1 and Alt2, and Mode 2 in inter-site inter-cell scenario, TRP-specific q3 and O3=4

In addition, we further enhance the performance of Mode 1 Alt1/Alt2 by optimizing the oversampling factor O3 for FD basis selection. Figure 4 and Figure 5 show the SLS results when considering the cases of TRP-common q3 for Mode 1 and TRP-specific q3 for Mode 1, respectively. Note that TRP-common q3 for Mode 1 can be spec-transparently implemented but TRP-specific q3 for Mode 1 needs additional spec impact. As seen in Figure 4 and Figure 5, we can make the additional observations as follows:

Observation 7:

- With TRP-common q3, Mode 1 Alt1 performs slightly better than Alt2, and Mode 1 Alt 1 and Mode 2 perform similarly.

- With TRP-specific q3 (additional spec impact to be needed), the performance of the both Mode 1 Alt1 and Alt2 can be improved and they yield a small gain (~2% average UPT gain) over Mode 2. Regardless, Mode1 Alt1 and Alt2 perform similarly.

Evaluation 4: performance comparison for different reference grouping methods, i.e., Alt1, and Alt3, of Issue 1, in inter-site inter-cell scenarios with ISD=500m and 200m

Figure 7: Average UPT gain vs overhead w.r.t. reference grouping methods, Alts 1 and 3, in inter-site intercell scenario with ISD = 500m (Mode 1)

Figure 8: Average UPT gain vs overhead w.r.t. reference grouping methods, Alts 1 and 3, in inter-site intercell scenario with ISD = 500m (Mode 2)

Figure 9: Average UPT gain vs overhead w.r.t. reference grouping methods, Alts 1 and 3, in inter-site intercell scenario with ISD = 200m (Mode 1)

Figure 10: Average UPT gain vs overhead w.r.t. reference grouping methods, Alts 1 and 3, in inter-site intercell scenario with ISD = 200m (Mode 1)

To see any benefit of Alt3 for W2 quantization scheme, we considered inter-site inter-cell scenarios with ISD=500m and ISD=200m (detailed assumptions are described in Table 6). As seen in Figure 7 - Figure 10, there is no benefit of Alt3 over Alt1 shown in our SLS results for both mode 1 and mode 2 cases even in the inter-site inter-cell scenarios. The Alt1 scheme sufficiently performs well with the least overhead. Here, we use the UPT for Alt1 scheme with paraComb=1 as the reference UPT value (which we regard as 100%).

Observation 8: there is no benefit of Alt3 over Alt1 shown in our SLS results for both mode 1 and mode 2 cases even in the inter-site inter-cell scenarios.

Type II codebook refinement for high/medium velocities

## Key issues

    - Issue 6 (CQI format of X=2)

[1] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrix - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix - FFS: Whether/how to include CQI overhead reduction for X=2 Offline Proposal 2.F.1: For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI is located in UCI part 2 Offline Proposal 2.F.2: For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI includes 4-bit wideband CQI and 2-bit sub-bands CQIs calculated independently from the 1st CQI

Regarding CQI, there are two open issues:

- CQI format of the 2nd TD CQI: there are 3 alternatives: (Alt1.1) independent of 1st TD CQI: 4-bit WB CQI and 2-bit SB CQI; (Alt1.2) Differential reference CQI relative to the 1st TD CQI: A BR-bit wideband CQI and 2-bit sub-band CQIs; and (Alt1.3) Differential reference and sub-band CQIs relative to the 1st TD CQI: A BR-bit wideband CQI and BSB-bit sub-band CQIs. The overhead saving for these alternatives are shown in Table 2. We can observe that the overhead saving is very small when compared with the total overhead (PMI, CQI, RI). Besides, Alt1.2/1/3 incur loss in avg. UPT when compared with Alt1.1, as shown in Section 3.2. We therefore support Alt1.1 and offline proposal 2.F.2.

- Location of the 2nd TD CQI: there are three alternatives: (Alt2.1) UCI part 1; (Alt2.2) UCI part 2; and (Alt2.3) WB CQI in UCI part 1 and SB CQI in UCI part 2. In our view, the 2nd TD CQI has a lower priority that the 1st TD CQI, hence it should be located in UCI part 2. Besides, the legacy Type I CSI for rank > 4 already includes a 2nd CQI located in UCI part 2. In particular, the WB CQI is located in Group 0, even-numbered SB CQIs are located in Group 1, and odd-numbered SB CQIs are located in Group 2. The same can be used for the 2nd TD CQI. We thus support offline proposal 2.F.1 and propose to use legacy (Type I CSI for rank > 4) solution.

Table 2

| Alt | WB CQI1 | SB CQI1 | WB CQI2 | SB CQI2 | NSB=19 | Overhead | Max saving |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | 4 | 2NSB | 4 | 2NSB | 84 | 84 |  |
| 1.2 | 4 | 2NSB | BR | 2NSB | 80+BR | [81,83] | 3 |
| 1.2A | 4 | 2NSB | 0 | 2NSB | 76+BR | [77,81] | 7 |
| 1.3 | 4 | 2NSB | BR | BSBNSB | 46+BR+BSBNSB | [66,68] | 18 |

Observation 9:

- Alt1.2/1.3 is worse than Alt1.1 in UPT vs overhead trade-off

  - Overhead saving with Alt1.2/1.3 is very small

  - Alt1.2/1.3 incur performance loss

- The 2nd TD CQI has a lower priority than the 1st TD CQI.

Proposal 7:

- support offline proposal 2.F.1, and reuse legacy solution (2nd CQI for Rel.15 Type I rank > 4)

- support offline proposal 2.F.2.

    - Issue 7 (NNZC bitmap design)

[1] Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives (no later than RAN1#112bis-e): - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L – s1-s0+ mint1-t0, Q – t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively. Nokia/NSB, Samsung, vivo, and ZTE raised concerns that, in their understanding, Alt3A violates previous agreements for “Q different two-dimensional bitmaps” and/or common DD basis selection across SD/FD basis pairs and hence, to some extent, objective 1 of the WID. (From offline) Alt 4’: simplified Alt4 For NZC bitmap in Type II CSI refinements for high/medium UEs, Q different bitmaps are supported for each layer, each of the Q bitmaps corresponds to DD basis q = 0 or 1. - For each polarization, each of the Q bitmaps contains bits included in a set of SD basis and FD basis pairs {(s, f)}, satisfying min(f,Mv-f)+ min(¦s-sref ¦, L-¦s-sref ¦)≤D, where - s∈{0,…,L-1}, f∈{0,…,M-1} - sref∈{0,…,L-1} is the SD basis indicated by SCI - Two polarizations have same set of {(s, f)} in the bitmap [1] Agreement For the Type-II codebook refinement for high/medium velocities, regarding the down-selection of bitmap(s) for indicating the locations of the NZCs (in RAN1#112bis-e), the following is used as a guidance for evaluation: - Following the agreed EVM, use “UPT vs. overall overhead (including CQI and PMI)” to compare across alternatives, assuming at least FTP1 traffic model and Rel-16 Parameter Combinations (L, beta, pv) - Use only the supported codebook parameter values (e.g. Q, K, m, d, delta, N4) - Companies are to state their assumptions on UE-side prediction (e.g. ideal or realistic, CSI-RS type, CSI-RS overhead calculation in relation to UPT, assumptions on WCSI and l) and the use of rank adaptation [1] Agreement For the Type-II codebook refinement for high/medium velocities, - The constraint on the maximum number of non-zero coefficients (NZCs) per-layer (K0) is defined jointly across all Q DD basis vectors. - FFS: How K0 is calculated - Also support a constraint on the total number of non-zero coefficients (NZCs) summed across all Q DD basis vectors and across all layers: - Following the legacy specification, the maximum total number is 2K Offline Proposal 2.G.1: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, - When the UE is configured with Q=1: for each layer, one 2-dimensional bitmap of size-2LM reusing the legacy design is used - When the UE is configured with Q=2: for each layer, - Basic feature: two 2-dimensional bitmaps, each of size-2LM reusing the legacy design for each of the two selected DD basis vectors, are used - Optional feature (for higher CSI overhead, FFS: definition), if the following down-selection succeeds: down-select from the following two alternatives in RAN#112bis-e: - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt4’: Q different bitmaps are supported for each layer, each of the Q bitmaps corresponds to DD basis q = 0 or 1. - For each polarization, each of the Q bitmaps contains bits included in a set of SD basis and FD basis pairs {(s, f)}, satisfying min(f,Mv-f)+ min(¦s-sref ¦, L-¦s-sref ¦)≤D, where - s∈0,…,L-1, f∈0,…,M-1 - sref∈{0,…,L-1} is the SD basis indicated by SCI - Two polarizations have same set of {(s, f)} in the bitmap

The Rel-16/17 based bitmap to indicate indices of NZ coefficients should be reused for Doppler codebook. However, since the coefficient matrix ( W2) comprises 2LMυQ coefficients, the total number of coefficients increases to Q times if Rel-16/17 parameter combinations are reused, which can be large (especially in high overhead regime corresponding to paramCombination = 6,7,8). Thus, mechanisms to reduce the W2 reporting payload can be considered. There are three alternatives:

- Alt1: it is a simple extension of legacy bitmap, i.e., Q bitmaps, each 2LMυ bits. The value of β can be smaller than legacy when Q=2. The rationale behind reducing beta value is that the ‘weak’ coefficients will add to overhead, but may not contribute to the UPT due to prediction and quantization errors. This has been demonstrated via SLS results in Section 3.2. In particular, it has been shown that a lower beta value can achieve better UPT vs overhead tradeoff than legacy Rel.16 beta for Q=2.

- Alt3A: there are two bitmaps, a first bitmap of size 2LS bits and a second bitmap of size MQ bits. The second bitmap selects S out of MQ basis vector pairs, hence is equivalent to a joint basis across (FD, DD), which changes the FD basis from legacy (hence, violates WID). In addition, this alternatives is not aligned with the previous agreement, which says “Q different two-dimensional bitmaps”. This alternative therefore should not be removed.

- Alt4’: there are Q bitmaps, they are identical and have a non-rectangular (rhombus) shape. The bitmap can be partitioned into four parts or submatrices. The reduction in the bitmap size is R1+R2+R3+R4, where Ri= number of coefficients omitted (removed) from the i-th of the four parts/submatrices of the bitmap. Note that Ri=1+2+…+vi and vi is the number of coefficients with ±s±f≤D-1 or D-2 in the i-th part/submatrix.

Simulation results comparing these alternatives are provided in Section 3.2. When compared with Alt1, there is no gain with Alt3A, and Alt4’ can beneficial for some parameter combinations.

Considering the above, we can support the offline proposal 2.G.1 and support Alt4’ for parameter combinations that incur large overhead (bitmap size).

The value of K0 can be similar to legacy, i.e., K0=cβ2LMQ where c≤1 is a parameter to quantify the overhead reduction. For example, the value of c∈{0.5,1}. When c=1, there is no overhead reduction. When c<1, there is overhead reduction. In case of Alt1, c can be used to lower the value of legacy β to cβ. In case of Alt4’, c can quantify the overhead reduction for a given (L,M,D). For example, c=1-R2LMQ and R= reduction in the bitmap size.

Observation 10: Regarding bitmap

- Alt3A violates the previous agreements for “Q different two-dimensional bitmaps”, and the objective 1 of the WID (without any modification of the SD and FD bases).

- Compared with Alt1, Alt3A can save some overhead, but gains in terms of UPT vs overhead trade-off is unclear.

- Alt4’ can improve UPT vs overhead trade-off

Proposal 8: Regarding the bitmap design, support offline proposal 2.G.1

- support Alt1 with cβ values, where c∈{0.5,1} and β is legacy

- support Alt4’ in high overhead regime (e.g. when 2LM is large)

- K0=cβ2LMQ where c≤1

    - Issue 8 (parameter combination)

[1] Agreement On the Type-II codebook refinement for high/medium velocities based on Rel-16 regular eType-II codebook (if supported), for the purpose of choosing the supported Parameter Combinations - Regarding the codebook parameter pv, in addition to the supported values from the legacy specification, introduce as additional candidate values - pv =1/8 for v=1,2 (hence 1/16 for v=3,4) - pv =1/2 for v=1,2,3,4 - Regarding the codebook parameter , in addition to the supported values from the legacy specification, introduce as an additional candidate value = 1/8 - Regarding the codebook parameter L, the supported values from the legacy specification apply [1] Agreement The Rel-18 Type-II codebook refinement for high/medium velocities comprises refinement of the following codebooks: - Refinement of the Rel-16 eType-II regular codebook, with N4>=1 - Refinement of the Rel-17 FeType-II port selection (PS) codebook, based on the common design with the Refinement of the Rel-16 eType-II regular codebook, except for the supported set of parameter combinations, with N4=1 - Time-/Doppler-domain reciprocity is not assumed

The legacy parameter combinations should be reused, except for the lower β values should also be supported, i.e., cβ, as described above. If c=12, then β=18 should be supported in addition to the legacy values.

Proposal 9: Regarding the parameter combination, reuse the legacy parameter combination tables, and support both legacy and lower β values, i.e., cβ, where c=1 for legacy and c=12 for lower β.

    - Issue 9 (CBSR)

The legacy (Rel.16) CBSR can be reused and extended, i.e. amplitude restriction includes DD in addition to FD.

Proposal 10: Support a simple extension of legacy CBSR, i.e., amplitude restriction includes both FD and DD

    - Issue 10 (UCI omission)

[1] Agreement On the Type-II codebook refinement for high/medium velocities, regarding UCI omission, down-select between the following three alternatives (by RAN1#112bis-e where q denotes the q-th DD basis vector): - Alt1. Prio(,l,m,q)=2L. Q.RI.P(m)+Q.RI.l+Q.q - Note: This implies that DD basis is designated the highest priority - Alt2. Prio(,l,m,q)=2L.S(q).RI.N3+2L.RI. P(m)+RI.l+ - Note: This implies that DD basis is designated the lower priority (after FD basis) - FFS: S(q) maps the index q according to a rule - Alt3. Prio(,l,m,q)=2L.RI.Mv.q + 2L.RI.P(m)+ RI.l +  - Note: This implies that DD basis is designated the least priority - Alt4. Prio(,l,m,q)=2L.P(m).RI.Q+2L.RI.S(q)+RI.l+ - Note: This implies that DD basis is designated with lower priority (after SD basis) and higher priority (before FD basis) - FFS: S(q) maps the index q according to a rule FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m q=0,…,Q-1

There are four alternatives regarding the priority equation. Alt2 and Alt4 are unnecessarily complicated due to the remapping Sq. The UCI omission is an emergency solution to a problem (i.e. insufficiency of UL resources) that happens rarely. There is no need to introduce any complicated solution such as Alt2 and Alt4 since they bring no benefits in return. Between Alt1 and Alt3, we prefer Alt3, since the DD basis selection relies on the UE-side prediction, which can be less accurate when compared with SD and FD basis selection, implying DD bases should have the least priority.

The UCI omission mechanism can be the same as the legacy mechanism, i.e., UCI parameters can be located in UCI part 1, and (G0, G1, G2) of UCI part 2, the same way as in legacy.

There are two new parameters:

- DD basis indicator: since SCI indicating (il*,0,dl*) is located in G0, the DD basis indicator should also be in located in G0.

- 2nd TD CQI: as discussed above, it can be located in UCI part 2, following the solution for the 2nd CQI in case of Rel.15 Type I, rank > 4.

Proposal 11: Regarding UCI omission

- Support Alt3 for priority, i.e. DD has the least priority

- Reuse legacy UCI omission mechanism

  - DD basis indicator is located in G0

  - 2nd TD CQI is located in UCI part 2: following legacy (the CQI for Rel.15 Type I, rank > 4), WB CQI is located in G0, even-numbered SB CQIs in G1, and off-numbered SB CQIs in G2.

## SLS results

The SLS results based on the agreed EVM assumptions (Table 7 in Appendix A) are provided in this section.

Evaluation 1: CQI format

In this evaluation, we first compare UPT vs overhead tradeoffs of the three alternatives (Alt1.1, Alt1.2, Alt1.3) for the 2nd TD CQI format. The results are shown in Figure 11 for the following simulation setting:

- CSI report window

  - d=2, N4=8  WCSI=dN4=16

  - 1st slot: l=nref

- CSI-RS measurement:

  - K=4, m=1

  - Measurement overhead modelled

- Q=2

- CSI periodicity P = 20 slots

- UE speed = 10 kmph

We can make the following observation.

Observation 11: there is no benefit with Alt1.2/1.3 (differential w.r.t. the 1st CQI) over Alt1.1 (independent of the 1st CQI)

Figure 11

Evaluation 2: bitmap

In this evaluation, we compared the three alternatives for the bitmap (Alt1, Alt3A, and, Alt4’).

- Alt1: β and μβ

- Alt3A: S=μMQ

- Alt4’: D=12L2+M2 and two values of K0 are considered

  - K0=β2LMQ

  - K0=βX, where X= reduced bitmap size

- Here, β= legacy (Rel.16) and two values μ= 0.5, 0.75 are considered.

The simulation setting is summarized as follows.

- CSI report window

  - d=2, N4=8  WCSI=dN4=16

  - 1st slot: l=nref

- CSI-RS measurement:

  - K=4, m=1

  - Measurement overhead modelled

- Q=2

- CSI periodicity P = 20 slots

- UE speed = 10 kmph

To compare Alt1, and Alt3A, the simulation results for avg., 50%, and 5% UPT vs overhead tradeoffs are shown Figure 12 through Figure 14. The simulation results comparing Alt1 and Alt4’ are shown in Figure 15 through Figure 17. We can make the following observation.

Observation 12:

- Alt3A and Alt1 are similar in UPT vs overhead trade-off for all of avg. UPT, 50% UPT, and 5% UPT.

- For any (UPT, overhead) achieved by Alt3A, there is a similar (UPT, overhead) achieved by Alt1

- Alt4’ can improve UPT vs overhead trade-off

Figure 12

Figure 13

Figure 14

Figure 15

Figure 16

Figure 17

Evaluation 3: parameter combination

The simulation setting for comparing parameter combinations is summarized as follows:

- CSI report window

  - WCSI=24

  - 1st slot: l=nref

- CSI-RS measurement:

  - K=4, m=1

  - Measurement overhead modelled

- CSI periodicity P = 20 slots

- Q=2

- UE speed = 10 kmph

- (Rel.16) paraComb=1-4 with cβ16

  - c=1: Rel.16 beta

  - c=0.5: Smaller beta

The UPT vs overhead tradeoffs is shown Figure 18. We can make the following observation.

Observation 13:

- Smaller β than legacy can be beneficial

- Weak coefficients increase overhead, but don’t provide UPT gain ( beta can be small)

Figure 18

TDCP reporting

## Key issues

    - Issue 11 (TRS configuration)

[1] Agreement For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, by RAN1#112bis-e, decide between the following alternatives: - Alt1. Fully reuse legacy TRS - Alt2. Study enhancements on TRS (e.g. periodicities) Note. If there is no consensus on Alt2, Alt1 is the default outcome Proposal 3.A: For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, - KTRS ≥1 TRS resource set(s) can be configured in the CSI reporting setting when ReportQuantity is ‘tdcp’ - Note: the TRS resource set(s) configured for TDCP report do not impact or impose any new requirements on the UE behavior when processing TRS used as QCL type A/D source for reception of PDxCH. - No further spec enhancement on TRS is supported - FFS: Whether to add further restrictions on the TRS resource set(s) on, e.g. QCL relationship, power control, RE location, relation with resource set used for legacy usage

In legacy, a UE can be configured with S≥1 TRS resource sets. The delay values for TDCP reporting depends on the value of S and the number of TRS resources in these sets.

- When S=1 TRS resource set, the number of TRS resources can be 2 or 4. The delay values depends on the number (N) of TRS resources out of 2 or 4 TRS resources configured for the TDCP reporting.

  - When N=1 TRS resource, the min periodicity is 10 slots, which is large for a high speed UE.

  - When N= 2 or 4 TRS resources, the TRS resources have the same periodicity and a fixed offset (intra-slot: 4 symbols, inter-slot: 1 slot), implying that the min possible delay (based on 10 slots periodicity) = [4 symbols, 8 symbols, 1 slot] + X, where X = 0, 9 slots, 10 slots, 11 slots, and so on. Even with multiple TRS resources, some relevant delay values (e.g. 5 slots) can’t be supported with only 1 TRS resource set. A possible solution is to introduce lower periodicity values (e.g. 4, 5 slots) for a TRS resource sets configured for TDCP reporting. This, however, requires enhancement on TRS.

- When S>1 TRS resource sets, different delay values (including 5 slots) can be supported by configuring different offsets for different TRS resource sets. Since the spec already supports multiple TRS resource sets, there is no spec enhancement needed on TRS.

Observation 14: regarding enhancement on TRS,

- If only one TRS resource set is supported for TDCP reporting, then enhancement on TRS is needed in order to support delay values such as 5 slots.

- If multiple TRS resource sets are supported for TDCP reporting, then enhancement on TRS is not needed.

Based on the above, we support the offline proposal 3.A on multiple TRS resource sets.

Proposal 12: support offline proposal 3.A

    - Issue 12 (Quantization)

Offline Proposal 3.B: For the Rel-18 TRS-based TDCP reporting, regarding the quantization of wideband normalized amplitude value, - At least the following size-Q quantization alphabet is supported: 1-2-N-qs, q=0,1,…,2Q-1 - TBD: supported value(s) of N (e.g. 2Q-1), Q, s (e.g. ½, ¼, 1/8, …), whether a center threshold is also supported (and if so, higher-layer configured) - FFS: Whether different schemes can be supported for different use cases

The (un-quantized) amplitude of the auto-correlation values corresponding to different delay values (assuming 5ms inter-delay separation) are shown in Figure 19. We can observe that the amplitude for the 1st delay/tap @5ms decreases with speed, implying that the value of Y can be reduced.

- For Y=1, the amplitude quantization range can be within [0.9,1] corresponding speed range [3-20]. The quantization levels can be 1-v2, where v is one of the first 8 values (indicator 0-7) of the legacy Rel.16 4-bit reference amplitude codebook.

- For Y>1, the amplitude quantization range can be within [0.9,a] where a depends on the speed and delay values. In this offline proposal 3.B, this corresponds to the value of N. Note that when N=2Q-1, then the value of 1-2-N-qs=0. In our view, we don’t need support amplitude values close to 0. We hence propose to choose a value of N>2Q-1.

Figure 19

Proposal 13: support the offline proposal 3.B

    - Other issues

[1] Agreement For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - FFS: Candidate values for delay - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 - FFS: Value of Dbasic [1] Agreement For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter Y for Y>1, down-select from the following alternatives: - Alt1. The value of Y is gNB-configured via higher-layer (RRC) signalling - Alt2. The value of Y follows the delays from the configured TRS resource - Alt3. The value of Y is UE-selected and reported The value of Y is a UE capability [1] Agreement For the Rel-18 TRS-based TDCP reporting, the priority of the CSI report(s) associated with TDCP reporting is down-selected from the following alternatives: - Alt1. Lower than other CSI reports - Alt2. Same as CSI report(s) not carrying L1-RSRP or L1-SINR - Alt3. Higher than other CSI reports - Other alternatives are not precluded

Our view about the other issues are as follows.

- Issue 13 (Y values, D_basis, delay values)

  - Y>1 value(s): the value of Y should depend on UE speed and delay values. The NW can configure the different delay values by configuring multiple TRS resource sets. The value of Y should decrease with increasing UE speed. At least the set of values {2,4} should be supported. Whether Y>4 is needed require further study.

  - D_basic: since delay values depends on the periodicity and offset of TRS resources in different TRS resource sets, and the minimum granularity of the delay values can be in slots. So, D_basic should be in slots, in particular, it can be equal τmin, the min delay values possible with the configured TRS resource sets.

  - Delay values: the delay values can be based on periodicity and offset of TRS resources in different TRS resource sets. In particular, the values can be the Y smallest delay values possible with the configured TRS resource sets.

- Issue 14 (Signaling/configuration of Y): for a given Y value, whether all Y values need reporting depends on UE speed and delay values. For low speed, all Y values can be reported, but for high speed, some of the Y correlation values can be ‘weak’ (small amplitude), and can be below the quantization level, hence they don’t need reporting. Since whether a correlation is ‘weak’ can only be determined by the UE, the UE can report a value y≤Y. We therefore support Alt3 (UE-selected).

- Issue 15 (Priority rule): in our view, the TDCP reporting should have lower priority than other CSI reports, we hence support Alt3.

Observation 15:

- The value of Y depends on UE speed and delay values.

- Whether all Y correlation values need reporting can only be determined by the UE.

Proposal 14:

- Support Y∈{2,4}

- Regarding signaling/configuration of Y, support Alt3 (UE-selected), the UE can report y≤Y

- Regarding priority rule, support Alt1 (lower than other CSI reports)

## Simulation results

Two sets of results are provided for the amplitude quantization for the use case of Type I/II switching based on a threshold. The first set of results is based on a simplified SLS setup wherein average UPT vs UE speed performance has been considered as a metric, and the second set of results is based on a LLS, wherein BLER vs UE speed performance has been considered as a metric.

SLS results

The SLS setup is as follow: 1 ring (21 cells), 1 UE per cell, full buffer, 16 BS ports, 2 UE ports, dynamic rank 1-2, Rel. 16 T2 PC 6, CSI period = 20, TRS period = 5. The following settings are considered to compare different amplitude quantization codebooks.

- Speed: 3, 5, 10, 15, 20, 30 [kmph]; 3kmph is the reference (100%)

- Delay: 5, 10, 15, 20 [ms]

- The following amplitude codebooks are compared.

  - 3-bit R16-based, 1-v, v = 1st 8 values from 4-bit R16 ref amp CB

  - 3-bit R16-based, 1-v2, v = 1st 8 values from 4-bit R16 ref amp CB

  - 4-bit R16-based, 1-v, v =4-bit R16 ref amp CB

  - 4-bit R16-based, 1-v2, v =4-bit R16 ref amp CB

  - 5-bit R16-extended, 1-v

  - 4-bit, configured threshold (N), 1-v

  - 4-bit, configured threshold (N), 1-v2

The results (Avg. UPT vs speed) are shown in Figure 20. We can observe the following.

Observation 16: for T1/T2 CB switch based on threshold = 0.86, and Y=1

- 3-bit R16-based quantization is sufficient

  - 1-v^2 is the best at low speed (<=10kmph)

  - 1-v is good overall

- 4-bit/5-bit doesn’t offset any gain over 3-bit

Figure 20

LLS results

Table 3

| Index | Level (R16 ref. amp. CB) | Level (1dB step) | Level (2dB step) |
| --- | --- | --- | --- |
| 0 | 1-1128 | 0.990 | 0.992 |
| 1 | 1-1819212 | 0.987 | 0.987 |
| 2 | 1-164 | 0.984 | 0.980 |
| 3 | 1-1204812 | 0.980 | 0.968 |
| 4 | 1-132 | 0.975 | 0.950 |
| 5 | 1-151212 | 0.968 | 0.921 |
| 6 | 1-116 | 0.960 | 0.874 |
| 7 | 1-112812 | 0.950 | 0.8 |

Case 1 (Y=1): in this section, we have provided the LLS results comparing different quantization scheme for Y=1 magnitude of autocorrelation based on threshold of 0.98, switching between Type and Type II. The three amplitude quantization codebooks, as shown in Table 3, are compared. Based on our results, the performance of quantization schemes is almost same with minor variation at higher speeds. Since step size in quantization schemes as are very small and close to threshold there is almost no performance difference. At switching speed, Rel-16 legacy based quantization and 1 dB step quantization performs almost same while 2 dB step has little higher BLER. Comparing the legacy based and 1 dB step quantization to un-quantized case, their performance is very close. The simulation assumptions are provided in Table 4.

Table 4

|  | Y=1 | Y=4,8 |
| --- | --- | --- |
| Threshold | 0.98 | 22.2 Hz |
| #TRS lags | 2 | 8 |
| Feedback | 1 amplitude of autocorrelation lag 1 | 7 amplitude and phase of all autocorrelation lags |
| TRS period | 5 slots (5ms) | 5 slots (5ms) |
| CSI period | 20 slots (20ms) | 20 slots (20ms) |

Observation 17: based on LLS evaluations,

- The BLER performance of un-quantized and 1st 8 levels from Rel-16 legacy 4-bit reference codebook is almost same

Figure 21 : BLER Vs UE speed

Case 2 (Y=8): in this section, we have provided the LLS results comparing different quantization for phase for Y=8 with magnitude quantized using 1st 8 levels of Rel-16, 4-bit quantization reference codebook, and phase with QPSK, 8- PSK and 16 PSK. The performance of 16-PSK and un-quantized is same. While 8-PSK performs almost same as un-quantized for lower speeds. Among 3, QPSK has highest BLER. The performance of phase quantization with above amplitude quantization schemes are almost same.

Observation 18: based on LLS evaluations,

- The BLER performance with 16-PSK for phase quantization is least, provides close match with un-quantized performance

- QPSK has highest BLER among 3 phase quantization methods.

Conclusion

In this contribution, the following observations and proposals are made:

Type II C-JT

Observation 1: due to one SCI across TRPs, the W2 coefficients for different TRPs (after normalization with the strongest coefficient) capture the different power level across TRPs, implying that there is no need for per-TRP per-polarization reference amplitude reporting (i.e. Alt3)

Observation 2: on FD basis reporting for mode-1, when compared with Alt 2, Alt1 can

- unify the functionality of mode 1 and mode 2 (per agreement on support of mode1/2),

- achieve similar performance with lower overhead of FD basis reporting as well as lower complexity of FD basis calculation

Observation 3: the simulation results show that Alt 1 with TRP-specific q3 (oversampling/rotated value) yields a 2~3% average UPT gain over Alt 1 with TRP-common q3. Hence, we prefer to introduce TRP-specific q3.

Observation 4: a subset (e.g. window) of 0,1,⋯,N3O3-1 (or 0,1O3,⋯,O3-1O3,1,⋯,N3-1O3) should be enough as an alphabet for φn reporting, since

- the delay difference across TRPs favorable for CJT operation should not be large, and

- the delay difference can be compensated by gNB implementation when Rel-17 based CJT operation is used with delay/angle reciprocity.

Observation 5: UCI omission with Alt3 is more beneficial than Alt2 in CJT operation.

Observation 6:

- Mode 1 with Alt 2 per-TRP SVD (the advocated lower complexity benefit for Alt2) incurs ~4% UPT loss (for the same PMI overhead) over Mode 2.

- Overall, Mode 2 and Mode 1 with Alt 1 and Alt 2 using joint-SVD operation yield similar performance.

- Mode 1 with Alt 2 needs additional UE processing to find per-TRP FD basis vectors for the case of joint-SVD operation, compared to Mode 2 or Mode 1 with Alt 1 – thereby resulting in higher UE complexity

Observation 7:

- With TRP-common q3, Mode 1 Alt1 performs slightly better than Alt2, and Mode 1 Alt 1 and Mode 2 perform similarly.

- With TRP-specific q3 (additional spec impact to be needed), the performance of the both Mode 1 Alt1 and Alt2 can be improved and they yield a small gain (~2% average UPT gain) over Mode 2. Regardless, Mode1 Alt1 and Alt2 perform similarly.

Observation 8: there is no benefit of Alt3 over Alt1 shown in our SLS results for both mode 1 and mode 2 cases even in the inter-site inter-cell scenarios.

Proposal 1: Revert the working assumption on the additional support for Alt3 for the amplitude grouping

- Note that the working assumption was included with the understanding that the gain of Alt3 over Alt1 (already agreed to be supported) can be demonstrated – not simply that “as long as Alt3 is not broken, the WA should be confirmed as agreement.”

Proposal 2:

- support Alt1 for mode-1 FD basis reporting, where layer-common TRP-specific φn (and q3) is reported.

- support a subset (e.g. window) of 0,1,⋯,N3O3-1 as an alphabet for φn reporting.

- support reporting reference CSI-RS resource n* for indicating the relative FD offset values, i.e., φn for ∀n≠n*

Proposal 3: for parameter combination,

- support offline proposal 1.C.1

- Do not support Ln=6 for NTRP>1

Proposal 4: For CBSR,

- support TRP-common soft amplitude restriction, averaging over TRPs and FD beams.

- support a separate configuration of (N1,N2) from CBSR

Proposal 5: for UCI omission, support offline proposal 1.E.1.

Proposal 6: for mode1, support including indicator(s) indicating φn and a reference CSI-RS resource indicator in Group 0 of CS part 2.

Type II Doppler

Observation 9:

- Alt1.2/1.3 is worse than Alt1.1 in UPT vs overhead trade-off

  - Overhead saving with Alt1.2/1.3 is very small

  - Alt1.2/1.3 incur performance loss

- The 2nd TD CQI has a lower priority than the 1st TD CQI.

Observation 10: Regarding bitmap

- Alt3A violates the previous agreements for “Q different two-dimensional bitmaps”, and the objective 1 of the WID (without any modification of the SD and FD bases).

- Compared with Alt1, Alt3A can save some overhead, but gains in terms of UPT vs overhead trade-off is unclear.

- Alt4’ can improve UPT vs overhead trade-off

Observation 11: there is no benefit with Alt1.2/1.3 (differential w.r.t. the 1st CQI) over Alt1.1 (independent of the 1st CQI)

Observation 12:

- Alt3A and Alt1 are similar in UPT vs overhead trade-off for all of avg. UPT, 50% UPT, and 5% UPT.

- For any (UPT, overhead) achieved by Alt3A, there is a similar (UPT, overhead) achieved by Alt1

- Alt4’ can improve UPT vs overhead trade-off

Observation 13:

- Smaller β than legacy can be beneficial

- Weak coefficients increase overhead, but don’t provide UPT gain ( beta can be small)

Proposal 7:

- support offline proposal 2.F.1, and reuse legacy solution (2nd CQI for Rel.15 Type I rank > 4)

- support offline proposal 2.F.2.

Proposal 8: Regarding the bitmap design, support offline proposal 2.G.1

- support Alt1 with cβ values, where c∈{0.5,1} and β is legacy

- support Alt4’ in high overhead regime (e.g. when 2LM is large)

- K0=cβ2LMQ where c≤1

Proposal 9: Regarding the parameter combination, reuse the legacy parameter combination tables, and support both legacy and lower β values, i.e., cβ, where c=1 for legacy and c=12 for lower β.

Proposal 10: Support a simple extension of legacy CBSR, i.e., amplitude restriction includes both FD and DD

Proposal 11: Regarding UCI omission

- Support Alt3 for priority, i.e. DD has the least priority

- Reuse legacy UCI omission mechanism

  - DD basis indicator is located in G0

  - 2nd TD CQI is located in UCI part 2: following legacy (the CQI for Rel.15 Type I, rank > 4), WB CQI is located in G0, even-numbered SB CQIs in G1, and off-numbered SB CQIs in G2.

TDCP reporting

Observation 14: regarding enhancement on TRS,

- If only one TRS resource set is supported for TDCP reporting, then enhancement on TRS is needed in order to support delay values such as 5 slots.

- If multiple TRS resource sets are supported for TDCP reporting, then enhancement on TRS is not needed.

Observation 15:

- The value of Y depends on UE speed and delay values.

- Whether all Y correlation values need reporting can only be determined by the UE.

Observation 16: for T1/T2 CB switch based on threshold = 0.86, and Y=1

- 3-bit R16-based quantization is sufficient

  - 1-v^2 is the best at low speed (<=10kmph)

  - 1-v is good overall

- 4-bit/5-bit doesn’t offset any gain over 3-bit

Observation 17: based on LLS evaluations,

- The BLER performance of un-quantized and 1st 8 levels from Rel-16 legacy 4-bit reference codebook is almost same

Observation 18: based on LLS evaluations,

- The BLER performance with 16-PSK for phase quantization is least, provides close match with un-quantized performance

- QPSK has highest BLER among 3 phase quantization methods.

Proposal 12: support offline proposal 3.A

Proposal 13: support the offline proposal 3.B

Proposal 14:

- Support Y∈{2,4}

- Regarding signaling/configuration of Y, support Alt3 (UE-selected), the UE can report y≤Y

- Regarding priority rule, support Alt1 (lower than other CSI reports)

# References

- Chairman’s notes, RAN1#112

- Chairman’s notes, RAN1#111

- Chairman’s notes, RAN1#110bis-e

- Chairman’s notes, RAN1#110

# Appendix A

Table 5: EVM for Type II C-JT

| Parameter |  | Value (Intra-cell scenario) | Value (Inter-cell scenario) |
| --- | --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM | FDD, OFDM |
| Multiple access |  | OFDMA | OFDMA |
| Scenario |  | RMa (Rural Macro) N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP) for each UE Outdoor1 | Dense Urban (Macro only) N_TRP (#TRPs): 2, 3, (N_TRP is semi-statically chosen based on, e.g. RSRP) Outdoor2 OptA |
| Frequency Range |  | FR1, 700Hz | FR1, 2GHz |
| Inter-BS distance |  | 1.7km | 200m |
| Channel model |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT |
| Number of Rings |  | 2 rings (57 sectors) - Each sector has N TRP as a cooperating mTRP set. | 2 rings (57 sectors): - The three sectors of each site is a cooperating mTRP set. |
| Number of UEs per sector |  | 30 | 30 |
| Antenna setup and port layouts at gNB |  | For each TRP, - 4 ports: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.8)λ - 8 ports: (2,2,2,1,1,2,2), (dH,dV) = (0.5, 0.8)λ Total #ports in mTRP = N TRP x {4,8} | For each TRP, - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32} |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 46 dBm for 10 MHz | 41 dBm per TRP for 10 MHz |
| BS antenna height |  | 25m | 25m |
| UE antenna height & gain |  | Follow TR36.873 | Follow TR36.873 |
| UE receiver noise figure |  | 9dB | 9dB |
| Modulation |  | Up to 256QAM | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot | 14 OFDM symbol slot |
|  | SCS | 15kHz | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots | Slot Format 0 (all downlink) for all slots |
| MIMO and scheduling scheme |  | MU-MIMO PF scheduling (User Rank 1 or 2 or 3 or 4) | MU-MIMO PF scheduling (User Rank 1 or 2 or 3 or 4) |
| MIMO layers |  | Up to 4 | Up to 12 |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Based on Alt1A/B, Alt2, Rel-16 eType-II | Based on Alt1/B, Alt2B, Rel-16 eType-II |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | RU 30~40% or 70~80% | RU 30~40% or 70~80% |
| UE distribution |  | 50% indoor (3km/h), 50% outdoor (120km/h) | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC | MMSE-IRC |
| Feedback assumption |  | Realistic | Realistic |
| Channel estimation |  | Realistic | Realistic |
| Evaluation Metric |  | User perceived throughput and CSI feedback overhead | User perceived throughput and CSI feedback overhead |

Table 6: EVM for Type II C-JT

| Parameter |  | Value (Inter-cell scenario) |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) N_TRP (#TRPs): 2, 3, (N_TRP is semi-statically or dynamically chosen based on, e.g. RSRP) (*Each same color indicates each collaborating mTRP set) Outdoor2 OptA - Inter-site inter-cell scenario |
| Frequency Range |  | FR1, 2GHz |
| Inter-BS distance |  | 200m or 500m |
| Channel model |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT |
| Number of Rings |  | 2 rings (57 sectors): - The three sectors with each same color above is a cooperating mTRP set. |
| Number of UEs per sector |  | 30 |
| Antenna setup and port layouts at gNB |  | For each TRP, - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32} |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm per TRP for 10 MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO and scheduling scheme |  | MU-MIMO PF scheduling (User Rank 1 or 2 or 3 or 4) |
| MIMO layers |  | Up to 12 |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Based on Alt1/B, Alt2B, Rel-16 eType-II |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | RU 30~40% or 70~80% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | User perceived throughput and CSI feedback overhead |

Table 7: EVM for Type-II Doppler

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. UE speed: 10 kmph Mobility model: - Spatial consistency procedure not modeled - No trajectory is assumed - Doppler spectrum model is not needed Single TRP |
| Frequency Range |  | FR1 only, 2GHz |
| Duplexing gap (b/w DL and UL) |  | 200MHz |
| Inter-BS (site) distance |  | 200m |
| Channel generation model |  | According to the TR 38.901 O2I car penetration loss per TS 38.901 can be assumed |
| Reciprocity model |  | Not applicable |
| Antenna setup and port layouts at gNB |  | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm for 10MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 20 MHz (10 MHz DL + 10 MHz UL) for 15kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | Dynamic SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Up to 4 MU layers |
| CSI feedback |  | Baseline scheme: CSI feedback periodicity (full CSI feedback): p1=5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling): D=4 ms eType II Doppler: CSI feedback periodicity : W=p2=20 ms CSI-RS burst: window of B measurement instances with separation d - B=4 - d=1 CSI reporting window: WCSI=16 Speen = 10kmph UE-side prediction: LMMSE |
| Overhead |  | DMRS, CSI-RS (including CSI-RS burst overhead) |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 50% for SU/MU-MIMO with dynamic rank 1-2 adaptation |
| UE distribution |  | 100% outdoor |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead |
| Baseline for performance evaluation |  | Rel-16 regular eTypeII with CSI feedback periodicity 5 ms |

Table 8: simulation assumptions for TDCP LLS

| Parameter | Value |
| --- | --- |
| Channel Model | CDL-C |
| Delay spread | 100 ns |
| Frequency | 4 GHz |
| Subcarrier Spacing | 15 KHz |
| FFT | 1024 |
| No of gNB ports | 32 |
| No of UE ports | 2 |
| Type II parameter combination | 6 |
| CSI periodicity | 20 slots (20 ms) |
| Bandwidth | 20 MHz |
| TRS periodicity | 5 slots(5 ms) |
| Autocorrelation lags for Alt. A2 | 8 and 4 (Including Zero lag) |
| Autocorrelation lags for Alt. B | 2 lags (Including Zero lag) |
| Threshold for Alt. A | 22.2 Hz (~ 6 km/hr) |
| Threshold for Alt. B | 0.98 |
| UE Antenna Pattern | Omnidirectional |
| BS Antenna Pattern | According to TR36.873 |
| Channel Estimation | Ideal |
| DMRS estimation | 2DMMSE |
| MIMO mode | SU-MIMO with rank=1 |
| SNR | 15 dB |

# Appendix B

We used the Rel-16 parameter combination table unless otherwise noted.