3GPP TSG RAN WG1#112b-e			R1-2303115

e-Meeting, April 17th – 26th, 2023

Agenda item:	9.1.3.1

Source: 	Samsung

Title: 	Views on DMRS enhancements

Document for:	Discussion and Decision

Introduction

The Rel-18 WID [1] includes the following objectives regarding the Rel-18 DMRS enhancements.

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

This contribution provides Samsung’s view regarding the Rel-18 DMRS enhancements highlighted above.

Increased number of DMRS ports for DL/UL MU-MIMO

  - Spatial multiplexing between legacy UE and Rel-18 UE

In current specification, when gNB would like to schedule some UEs by MU-MIMO manner, there is a restriction that the UE is not expected to assume co-scheduled UE(s) with different DM-RS configuration with respect to the followings.

  - DM-RS configuration type

  - DM-RS symbol location

  - actual number of front-loaded DM-RS symbol(s)

  - actual number of additional DM-RS

That means, if one UE is configured with DMRS type 1 and the other UE is configured with DMRS type 2, then two UEs are not scheduled by MU-MIMO together. It is natural to restrict MU-MIMO scheduling between DMRS type 1 and type 2 since RE mapping of each CDM group and applied OCC are different between DMRS type 1 and type 2. Also, if new DMRS types are defined in Rel-18, then it is natural to schedule MU-MIMO among UEs which are configured with new DMRS type 1 only or new DMRS type 2 only. To summarize, the following 4 cases of MU-MIMO scheduling will be supported in Rel-18.

  - (current specification) among UEs which are configured with the DMRS type 1

  - (current specification) among UEs which are configured with the DMRS type 2

  - (natural extension in Rel-18) among UEs which are configured with a new DMRS type 1

  - (natural extension in Rel-18) among UEs which are configured with a new DMRS type 2

However, when we define new DMRS types on top of the current DMRS type 1 and type 2 for supporting increased number of orthogonal DMRS ports for MU-MIMO scheduling, it should be considered whether spatial multiplexing between legacy DMRS types and new DMRS types is supported or not. Since the main objective to increase the number of DMRS ports for both DL and UL cases is to obtain more gain on spectral efficiency by scheduling MU-MIMO, it would be beneficial to allow spatial multiplexing not only between same DMRS types but also between legacy and new DMRS types. Therefore, in addition to the above cases, the following two cases are additionally supported.

  - (additionally supported case 1 in Rel-18) among UEs which some UEs are configured with current DMRS type 1 and the other UEs are configured with a new DMRS type 1

  - (additionally supported case 2 in Rel-18) among UEs which some UEs are configured with current DMRS type 2 and the other UEs are configured with a new DMRS type 2

In RAN1#110 [2], it was agreed to support MU scheduling between Rel-15 and Rel-18 DMRS ports as follows:

Agreement - Support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports. - For MU-MIMO by different CDM groups, no MU-MIMO scheduling restriction of PUSCH/PDSCH (i.e. MU-MIMO between Rel.15 UE and Rel.18 UE is allowed). - For MU-MIMO within a CDM group, study whether and how to support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH. - Note: the study includes MU-MIMO between Rel.15 UE and Rel.18 UE, and between Rel.18 UEs. - Note: PUSCH above is CP-OFDM waveform.

Our view is that allowing MU-MIMO by both same and different CDM groups is needed and beneficial to increase system throughput. Obviously, if we consider MU-MIMO scheduling within a CDM group for Rel-15 UE and Rel-18 UEs, the orthogonality between length 2 FD-OCC for Rel-15 and length M (=4 or 6) FD-OCC for Rel-18 should be hold which does not impact on legacy Rel-15 UE.

Proposal 1. Support MU-MIMO between Rel-15 DMRS ports and Rel-18 DMRS ports by using same or different CDM groups.

  - Compensation on degraded channel estimation performance

In order to achieve the main goal of this agenda item, the important constraint is not to utilize additional DMRS resources, i.e., keeping the DMRS overhead. Therefore, if we utilize FD-OCC with longer length than 2 (e.g., 4), the channel estimation performance of new DMRS types would be degraded as a side effect rather than the current DMRS types. Hence, in order to compensate the expected performance degradation, two approaches can be considered.

    - Switching between Rel-15 DMRS type and Rel-18 DMRS type

First approach is dynamic switching between current DMRS type and new DMRS type as current DMRS type may have shorter OCC length, better channel estimation performance can be achieved. However, in RAN1#112 [5], it was concluded that DCI based dynamic switching between Rel-15/18 DMRS type, i.e., indicating between length-2 or length-4 FD-OCC, is not supported. The main reason not supporting the feature is that even though a UE is configured with Rel-18 DMRS type, the UE can freely use or apply one of or both of two different length of FD-OCC by the UE’s own decision and freedom, so it is not needed to indicate a certain length of FD-OCC from gNB to UE.

However, our view is that this operation is not possible from all UEs, and shall require additional channel estimation cost and specific implementation from UE side. Hence, it is not clear to assume all UEs could do this operation by default. Hence, the optional UE capability should be defined for this specific UE operation.

Proposal 2. Support to apply length-2 and/or length-4 FD-OCC by UE’s own decision regardless of gNB’s indication and/or configuration as UE optional feature.

Then, if a UE supports the above UE capability, since the UE can apply length-2 and/or length-4 FD-OCC based on UE’s own decision and/or freedom regardless of gNB’s indication and/or configuration, the UE does not need dynamic switching between Rel-15/18 DMRS type based on gNB’s dynamic indication. However, if a UE does not support the above UE capability, the UE cannot apply the FD-OCC which is not configured. For example, if a UE without the above UE capability is configured with Rel-18 DMRS, the UE can only apply length-4 FD-OCC, which is configured one, but cannot apply length-2 FD-OCC. Then, for these particular UEs, if gNB could dynamically change DMRS type of UEs depending on the scheduling/CSI reporting situation (e.g., channel quality, orthogonality between other UE’s channel based on the UE’s subband/wideband CSI reporting), it could be better to have more flexible and adaptive scheduling gain. Hence, switching between Rel-15/18 DMRS type is beneficial for these UEs.

Therefore, at least for UEs who cannot support to apply length-2 and/or length-4 FD-OCC freely, we suggest MAC-CE based switching between Rel-15/18 DMRS type. Even for this feature, i.e., MAC-CE based switching, since the UE shall prepare two different channel estimation procedure, it also requires additional UE capability.

Proposal 3. Support MAC-CE based switching between Rel-15 DMRS and Rel-18 DMRS as UE optional feature.

    - Re-using the concept of OCC disabling scheme

Second approach is reusing the concept of OCC disabling scheme which has been adopted in Rel-17 above 52.6 GHz agenda item. The motivation of this scheme is to achieve better channel estimation performance in case of large subcarrier spacing (e.g., 480kHz, 960kHz) in 52.6 GHz band.

In TS38.214, the concept of OCC disabling scheme is implemented as follows: “If a UE is configured with higher layer parameter [dmrs-FD-OCC-disableForRank1PDSCH] and the UE is scheduled with PDSCH with single DM-RS port, the UE may assume that set of orthogonal DM-RS antenna ports from the same CDM group using different set of wf(k') codes are not associated with the transmission of PDSCH to another UE.” This means that OCC for a certain DMRS port may not be used (i.e., disabled) for the CDM group containing the DMRS port for the UE when a RRC parameter is configured and rank-1 PDSCH scheduling is indicated. Then, the CDM group including the scheduled DMRS port is only used for the UE, so the UE does not need to apply OCC to distinguish other UE’s co-scheduled DMRS port. Hence, despite of longer OCC or sparser DMRS RE used by new DMRS type, channel estimation performance of UE can be mitigated. Although the number of co-scheduled UEs, especially for MU-MIMO, may be decreased, this concept is beneficial for gNB as well as UE to make scheduling simple when a UE disabling OCC is included in the scheduling, since gNB does not always schedule large number of UEs by MU-MIMO. Therefore, our view is that it would be better to study/reuse the concept of OCC disabling scheme for new DMRS type.

Proposal 4. Study on OCC disabling scheme for new DMRS type to compensate degraded channel estimation performance.

  - Remaining issues on orphan RE

There are several remaining issues on scheduling restriction of the orphan RE case. In RAN1#110b-e [3], the following agreement was made.

Agreement in RAN1#110b-e For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH, support the following: - Introduce UE capability to report whether UE can be scheduled PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS. - If this capability is not supported by the UE, UE expects that gNB shall apply the scheduling restriction for PDSCH for FD-OCC length 4 in Rel.18 eType 1 DMRS. - The scheduling restriction above means satisfying all of the following at least for other than M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - 1) The number of consecutively scheduled PRBs for PDSCH is even. - 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even. - 3) FFS: Restriction on scheduling of different UEs in case of MU-MIMO. - FFS: Scheduling restriction for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - Note1: Up to UE how to implement DMRS channel estimation. - Note2: No further RAN1 specification enhancement is introduced to handle the orphan REs (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs) for UE that is scheduled PDSCH without the scheduling restriction. - Note 3: Other scheduling restrictions, if identified in future meetings, are not precluded.

The 1st issue is whether additional scheduling restriction on single-DCI based multi-TRP PDSCH transmission with FDM scheme A or B. Since it is common understanding that multi-TRP scheme could be supported by Rel-18 DMRS, it is natural to be extended supporting single-DCI based multi-TRP PDSCH transmission with FDM scheme A or B. Hence, additional scheduling restriction is needed for supporting single-DCI based multi-TRP PDSCH transmission with FDM scheme A or B without orphan RE issue. Then, the above scheduling restrictions, i.e., 1) The number of consecutively scheduled PRBs for PDSCH is even, and 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even, could be extended per each TRP or per each TCI state.

Proposal 5. Support additional scheduling restriction on single-DCI based multi-TRP PDSCH transmission with FDM scheme A or B.

The 2nd issue is for FDRA type 1 with interleaved VRB-PRB mapping. In the above agreement, the expression like “scheduled PRBs” or “the number of PRBs” in two scheduling restrictions is not clear whether those mean VRB or PRB. Hence, we suggest to clarify that the above two scheduling restrictions is applied to VRB.

Proposal 6. Support to clarify that the expression “scheduled PRBs” or “the number of PRBs” in the previous agreement means VRB.

UL DMRS enhancement enabling 8TX operation

  - UL DMRS 8 ports design

In RAN1#110 [2], the maximum number of layers for UL transmission was agreed as follows:

Agreement Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability - For uplink transmission with rank<=4, single CW is supported - For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e The above applies only with regards to the work scope of this agenda item.

Then, considering UL DMRS up to 8 ports (i.e., for the case when a UE reports UE capability supporting X=8), since the current specification supports up to 8 ports DMRS with 2 codewords for the DL aspect, it can be a framework which is a good starting point as a reference. Hence, it would be good to start UL DMRS 8 ports design based on the DL design. The possible aspects to be considered would be at least the followings.

  - Enabling higher layer parameter

    - For DL case, there is a higher layer parameter, maxNrofCodeWordsScheduledByDCI in PDSCH-Config, meaning that the maximum number of codewords that a single DCI may schedule. Similarly, a new higher layer parameter can be defined to enable the second TB based on the similar signalling granularity, e.g., configured in PUSCH-Config.

  - Codeword to layer mapping

    - For DL DMRS up to 8 ports, layers from 1 to 4 can be mapped onto the first codeword, and layers from 5 to 8 can be mapped onto the second codeword. This principle can be also simply re-used for UL DMRS up to 8 ports. Hence, we prefer to have 2 codewords when more than 4 layers are supported.

  - Possible indicated rank

    - In DL case, all rank values from 5 to 8 can be indicated when maxNrofCodeWordsScheduledByDCI is configured. Similarly, in UL case, all rank values from 5 to 8 can be supported as a starting point. Also, in order to reduce work load, it is possible to preclude some rank values and some frequently used rank values can be only defined, and this can make less specification effort.

  - DMRS table entries supporting larger than 4 layers

    - In DL case, there is only one entry for each rank larger than 4 (i.e., one entry for each rank 5, 6, 7, and 8, respectively). Similar DMRS entry design is applied for UL case.

  - Additional MCS, NDI, RV fields for second TB in DCI format 0_1

    - If maxNrofCodeWordsScheduledByDCI is configured, then additional MCS, NDI, RV fields for the second TB are defined in DCI format 1_1. Hence, additional MCS, NDI, RV fields can be also defined for the second TB of UL data transmission.

  - Enabling/disabling mechanism of each TB using MCS and RV fields

    - For DL DMRS up to 8 ports, if one of two TBs is disabled for the UE to schedule only one TB, then MCS index 26 and RV index 1 for the corresponding TB is used, and MCS index 26 and RV index 2 for the corresponding TB is used when the UE is configured with multi-PDSCH scheduling. This mechanism can be re-used for UL DMRS up to 8 ports to switch between scheduling 1 TB and 2 TBs.

Proposal 7: For UL DMRS up to 8 ports, the following aspects can be re-used from DL DMRS design.

  - Enabling higher layer parameter

  - Codeword to layer mapping (support 2 codewords for more than 4 layers)

  - Possible indicated rank

  - DMRS table entries supporting larger than 4 layers

  - Additional MCS, NDI, RV fields for second TB in DCI format 0_1

  - Enabling/disabling mechanism of each TB using MCS and RV fields

  - PTRS-DMRS association

Although almost every aspect from DL 8 ports DMRS design can be re-used as a starting point for designing UL 8 ports DMRS design, the principle of PTRS-DMRS association is different between DL and UL in the current specification. For the case of DL, the association between PTRS and DMRS can be fixed, since the gNB can switch the layers by implementation based on the Layer Indicator (LI) feedback by CSI report from UE. However, for the case of UL, since the gNB can know which DMRS port is the strongest one, based on the channel estimation of SRS transmission from UE side, the gNB selects the appropriate DMRS port which PTRS will be associated and indicates the corresponding information by PTRS-DMRS association field in DCI. Hence, it would be better to enhance PTRS-DMRS association considering up to 8 ports UL DMRS and 2 codewords, and also considering both codebook and non-codebook based PUSCH transmissions.

Proposal 8: Study enhancement on PTRS-DMRS association

  - Considering up to 8 ports UL DMRS and 2 codewords.

  - Considering both codebook and non-codebook based PUSCH transmissions.

In RAN1#110 [2], it was agreed to study on potential enhancement of PTRS-DMRS association as follows:

Agreement - For support of more than 4 layers SU-MIMO PUSCH, study the following potential enhancements for PTRS-DMRS association. - Whether to support more than 2-port UL PTRS. - Whether to increase the DCI size of PTRS-DMRS association field in DCI format 0_1/0_2.

Regarding the number of UL PTRS ports, we prefer not to have more than 2-port UL PTRS. The reasons can be shown as follows:

  - Since the main target scenario of up to 8 layers would be FR1 (it is hard to see the feasible case of more than 4 layers in FR2) and PTRS is mainly used in FR2 due to higher carrier frequency (mandatory with capability signaling in FR2, and optional with capability signalling in FR1), it is not necessary to have more number of PTRS ports rather than current specification.

  - Also, we are not sure whether multiple panels are feasible in FR1, even for more than a single panel, up to 2 panels are reasonable.

  - In AI 9.1.4.1, it was agreed that up to 2 PT-RS ports can be supported for single-DCI based STXMP (simultaneous transmission by multi-panel) scheme even in FR2.

  - If more than two PTRS ports are used, we can expect significant throughput loss due to additional overhead of PTRS resources, and this is not good for more than 4 layers, because the main purpose of supporting up to 8 layers is to increase UL throughput.

Based on the above, we prefer to keep the maximum number of UL PTRS as 2.

Proposal 9: Do not support more than 2-port UL PTRS.

Regarding the DCI size of PTRS-DMRS association field in DCI format 0_1/0_2, the following agreement has been made in RAN1#112 [5] to down select among two alternatives.

Agreement in RAN#112 - For full-coherent PUSCH with rank 5-8 with one port PTRS, support Alt.1 in the RAN1#111 agreement with the following update - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2. - FFS: Association with The CW with the higher MCS is selected in case of two CWs. - If the MCS is the same for two CWs, the PTRS port is associated with the first CW. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW with the higher MCS. 1; 2nd scheduled DMRS port the CW with the higher MCS. 2; 3rd scheduled DMRS port the CW with the higher MCS. 3; 4th scheduled DMRS port the CW with the higher MCS.

The remaining issue is how to support partial-/non-coherent PUSCH with rank 5-8 with one or two port PTRSs. Even for these remaining cases, our view is when a UE is indicated to select DMRS port associated with PTRS port, the concept of choosing one CW first (e.g., CW with higher MCS, or first CW if two CWs have same MCS) could be re-used based on properly allocating coherent PUSCH antenna port group and corresponding DMRS ports within a certain CW. Since some discussion points on partial-/non-coherent PUSCH have not been resolved yet (e.g., UL TPMI codebook design, coherent antenna group on PUSCH antenna port, etc), we think that the discussion on partial-/non-coherent PUSCH with rank 5-8 with one or two port PTRSs could be postponed.

Proposal 10: For partial-/non-coherent PUSCH with rank 5-8 with one or two port PTRSs, strive to have similar concept of choosing one CW first and select DMRS port(s) within the CW.

  - Indication between Rel-15/18 DMRS for supporting UL DMRS more than 4 ports

In RAN1#110b-e [3], the following was agreed for supporting more than 4 layers on UL PUSCH transmission.

Agreement For more than 4 layers SU-MIMO PUSCH, support - Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports. - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports. - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed. - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only.

As above, both Rel-15 and Rel-18 DMRS have been adopted for supporting more than 4 layers on UL PUSCH transmission. The main reason why both Rel-15 and Rel-18 DMRS are agreed to use is to support similar structure with DL DMRS up to 8 ports, and to support by using only 1 front loaded DMRS symbol with more than 4 layers for PUSCH transmission, which may have lower overhead rather than 2 front loaded DMRS symbols. So far, since 1 or 2 front loaded DMRS symbols can be semi-statically configured, we think that DMRS of which release (i.e., between Rel-15 and Rel-18 DMRS) can be indicated by RRC parameter (i.e., semi-statically configured), and dynamic indication by DCI is not needed. Hence, we only support RRC based indication between Rel-15/18 DMRS for supporting UL DMRS more than 4 ports.

Proposal 11: Support RRC based indication between Rel-15/18 DMRS for supporting UL DMRS more than 4 ports.

Conclusion

In this contribution, the following observations and proposals are made:

Increased number of DMRS ports for DL/UL MU-MIMO

Proposal 1. Support MU-MIMO between Rel-15 DMRS ports and Rel-18 DMRS ports by using same or different CDM groups.

Proposal 2. Support to apply length-2 and/or length-4 FD-OCC by UE’s own decision regardless of gNB’s indication and/or configuration as UE optional feature.

Proposal 3. Support MAC-CE based switching between Rel-15 DMRS and Rel-18 DMRS as UE optional feature.

Proposal 4. Study on OCC disabling scheme for new DMRS type to compensate degraded channel estimation performance.

Proposal 5. Support additional scheduling restriction on single-DCI based multi-TRP PDSCH transmission with FDM scheme A or B.

Proposal 6. Support to clarify that the expression “scheduled PRBs” or “the number of PRBs” in the previous agreement means VRB.

UL DMRS enhancement enabling 8TX operation

Proposal 7: For UL DMRS up to 8 ports, the following aspects can be re-used from DL DMRS design.

  - Enabling higher layer parameter

  - Codeword to layer mapping (support 2 codewords for more than 4 layers)

  - Possible indicated rank

  - DMRS table entries supporting larger than 4 layers

  - Additional MCS, NDI, RV fields for second TB in DCI format 0_1

  - Enabling/disabling mechanism of each TB using MCS and RV fields

Proposal 8: Study enhancement on PTRS-DMRS association

  - Considering up to 8 ports UL DMRS and 2 codewords.

  - Considering both codebook and non-codebook based PUSCH transmissions.

Proposal 9: Do not support more than 2-port UL PTRS.

Proposal 10: For partial-/non-coherent PUSCH with rank 5-8 with one or two port PTRSs, strive to have similar concept of choosing one CW first and select DMRS port(s) within the CW.

Proposal 11: Support RRC based indication between Rel-15/18 DMRS for supporting UL DMRS more than 4 ports.

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94-e.

- Chair’s note, RAN1#110.

- Chair’s note, RAN1#110b-e

- Chair’s note, RAN1#111

- Chair’s note, RAN1#112