3GPP TSG RAN WG1#112-bis-e			R1-2303118

Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.4.2

Source: 	Samsung

Title: 	Views on TPMI/SRI enhancements for 8Tx UL transmission

Document for:	Discussion and Decision

# Introduction

This contribution provides Samsung’s view regarding the remaining issues on 8Tx UL operation.

# Potential enhancements

## Codebook-based transmission

Agreement [2] Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook - Precoding matrices generated according to (O1, O2) = (1, 1) is supported - Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1 - Subject to UE capability - FFS: Different O1, O2 values for different ranks Agreement [2] For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), - Study whether/how to support (O1, O2) = (2,1), (2,2) - whether for all rank, or rank 1-2, or rank 3-8 - applicability of different (O1, O2) values per agreed (N1, N2) - companies are encouraged to submit simulation results Agreement [2] For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2 - Precoding design is based on Rel-15 UL 4TX codebook, - Full-coherent precoders are used - FFS whether partial-coherent precoders are needed - When Ng=4, down-select from, - Alt1: - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Alt2: - Precoding design is based on Rel-15 UL 4TX codebook, - Partial-coherent precoders are used Agreement [2] For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, - Following rank and layer splitting cases are supported Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 1; (1,0), (0,1). 8; (4,4). - Select from the following cases based on the performance and overall DCI overhead Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (2,1), (1,2). 4; (4,0), (0,4). 4; (2,2), (3,1), (1,3). 5; (4,1), (1,4), (2,3), (3,2). 6; (4,2), (2,4), (3,3). 7; (4,3), (3,4). Note: Above is not relevant to how precoders are indicated Agreement [4] For CB-based 8TX PUSCH transmission, for rank indication, down-select among the following - Separate indication of TRI and TPMI - Joint indication of TRI and TPMI Agreement [3] Support the following cases for codebook design for 8TX precoders - Full coherent precoders with Ng=1 - FFS: Full coherent precoders with Ng=2, Ng=4 - Partial coherent precoders with Ng=2 and Ng=4 - This does not imply any relation with the number of TPMI indications for 8TX precoder - Non-coherent precoders Agreement [3] For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource - For when Ng=2, down-select of the following convention for assumption of port coherency scheme is used - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7} - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7} - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7} - For when Ng=4, down-select of the following convention for assumption of port coherency scheme is used - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7} - Alt3: four coherent groups of {0, 2}, {4, 6}, {1, 3} and {5, 7} Note: Other alternatives which are not foreseen are not precluded Agreement [3] For SRI and/or transmitter precoder matrix indication for codebook-based uplink transmission by an 8TX UE, study - Whether/how to indicate one or multiple TPMI/SRI, according to the number of antenna groups, coherence capability, codebooksubset configuration, etc. - Whether/how to extend Rel-17 framework, e.g., TPMI/SRI indication in MTRP PUSCH - Whether/how to separate/joint indication of rank and precoding information. - Whether/how to indicate n (<=Ng) selected antenna group(s) separately from TPMI/TRI indication Agreement [3] For SRS configuration supporting codebook -based UL transmission for an 8TX UE , - Support configuration of 1 SRS resource set containing up to X 8-port SRS resource(s), where X = 2 - FFS : Other values for X, if needed - FFS : Configuration of at least one SRS resource set, configured with more than one SRS resources where each SRS resource may have the same or different number of SRS ports, e.g., for support full power operation, if supported - FFS : Configuration of at least one SRS resource set, configured with 8/M of M-port SRS resources, for example, - Configuration of an SRS resource set, configured with at least 4 of 2-port SRS resources - Configuration of an SRS resource set, configured with at least 2 of 4-port SRS resources Agreement [4] Study full TX power uplink codebook-based transmission by a partially/non-coherent 8TX precoder, - Reuse Rel-16 UE capability definitions for discussion purpose, i.e., UE Capability 1, 2 and 3 - For full TX power transmission by UE Capability 2/3, at least, following exemplary PA architectures can be considered - Other cases of interest are not precluded, down-select preferred potential architecture for the purpose of 8TX full power study in RAN#112. - This can be used for other UE Power Classes as well. 8TX UE, Power class 3 (23 dBm) Pi= Nominal power rating of each PA. Regular UE; P1=P2= …=P8=14 dBm (Full power supported by Mode1). Full-power capable UE; Full power capability with any PA comb. (CAP1) Example: P1=P2= …=P8= 23 dBm. Full power capability with 1 PA (CAP3) Example: P1=P2= …=P7= 14 dBm P8= 23 dBm. (lower priority) Full power capability with 2 PAs (CAP2) Example 2a: P1=P2= …=P6= 14 dBm, P7=P8 ≥ 20 dBm Example 2b: P1=P2= …= P8= 20 dBm. (lower priority) Full power capability with 4 PAs (CAP2) Example 3a: P1=P2= …=P4= 14 dBm, P5=P6= …=P8 ≥ 17 dBm Example 3b: P1=P2= …= P8 = 17 dBm. (lower priority) Full power capability with 6 PAs (CAP2) Example 4a: P1=P2= 14 dBm, P3=P4= …=P8 ≥ 15.3 dBm Example 4b: P1=P2= … = P8≥ 15.3 dBm. Agreement [4] For an 8TX partial/non-coherent precoder, for study on full power codebook-based PUSCH transmissions, use Rel-16 full power modes as the starting point for the design. Note: This does not mandate support of all Rel-16 modes

For a UE with 8 antenna ports, the number of antenna groups Ng=1,2,4,8. It was agreed in RAN1#110bis-e that full coherent precoders correspond to Ng=1, and partial coherent precoders correspond to Ng=2,4. It is FFS whether full coherent precoders can be associated with Ng=2,4. In our view, if two antenna groups are FC, they essentially can be combined into one antenna group. Therefore, the support for full coherent precoders for Ng=2,4 is not needed.

Observation 1: if two antenna groups comprise coherent antennae, they essentially can be combined into one antenna group for codebook design

Proposal 1: antennae within a group are coherent, and antennae across multiple groups are non-coherent

- Do not support full coherent precoders with Ng=2,4

Figure 1

A parameterized approach can be used to describe the codebook and associated signalling. This would simplify the design and is scalable to >8Tx UL operations, if needed, in future. A similar approach has been used in legacy Rel. 15 Type I codebook, wherein parameters (Ng,N1,N2) are used to describe the codebook design and associated configurations.

Observation 2: a parameterized approach similar to Rel. 15 DL Type I codebook can be adopted to simply codebook design and to have a solution scalable to >8Tx, if needed, in future

In particular, different 8Tx antenna structures can be described based on the following parameters.

- P=2 for cross-pol.

- Ng= number of antenna groups.

- N1= Number of antennae in 1st dimension.

- N2= Number of antennae in 2nd dimension.

The number of groups (Ng) can be used to describe antenna coherence types, i.e. full-coherent (FC), partial-coherent (PC), and non-coherent (NC). In particular,

- FC: Ng=1

- PC: Ng=2,4

- NC: Ng=8

Examples of the possible antenna structures are shown in Figure 1. The tuple Ng,N1,N2,P can take following values.

- FC: Ng,N1,N2,P=1,4,1,2, 1,2,2,2

- PC: Ng,N1,N2,P=2,2,1,2,(4,1,1,2)

- NC: Ng,N1,N2,P=(8,-,-,-)

Proposal 2: reuse DL Type I codebook parameters (Ng,N1,N2) to describe/configure 8Tx UL codebook for different coherence types

- FC: Ng,N1,N2,P=1,4,1,2, 1,2,2,2

- PC: Ng,N1,N2,P=2,2,1,2,(4,1,1,2)

- NC: Ng,N1,N2,P=8,-,-,-

- FC/PC precoders: comprises two components

  - selection of antenna group(s)

  - precoder associated with the selected antenna group(s)

- NC precoders: selection of antenna group(s), where a group comprises single antenna

Codebook design

Considering that the TPMI indication is WB and it is indicated via UL-DCI, the TPMI payload (i.e., overall codebook size) should be restricted, for example, 1-2 bits more than 4Tx TPMI overhead (which is at most 6 bits). Hence, the codebook design should be based on the UPT gain vs CB size (TPMI overhead) trade-offs. Note that this is also the metric used in evaluation of Type II codebooks since Rel.16.

Proposal 3: similar to Rel.16-18 Type II codebook design, the metric for evaluating different 8Tx codebook alternatives should be UPT gain vs codebook size (TPMI overhead).

Full coherent precoder design

There are two aspects for further discussion.

- Codebook parameters:

  - CodebookMode: in DL Type I codebook, codebookMode1 corresponds to L=1 (single DFT beam) and codebookMode2 corresponds to L=4. Similar to FC precoders in Rel. 15 UL 4Tx codebook, the number of DFT beams can be fixed to 1 (i.e., L=1). This can be save up to 2 bits when compared with L=4.

  - Oversampling factors (O1,O2): (1,1) has been agreed, and it is FFS whether to support (2,1) and (2,2). In our view, similar to Rel.15 4Tx UL codebook (cf. Appendix A), the oversampling can be less (e.g. 1 or 2) than DL Type I CB (i.e. 4). In particular, the oversampling factor Oi=2 can be supported at least for rank 1-2, and Oi=1 can be fixed for rank > 2.

- Codebook subsampling: to reduce the TPMI payload, the Rel. 15 Type I codebook can be subsampled by a factor of N, implying that a subset of Rel. 15 Type I codebook is used as FC precoders in 8Tx UL codebook. For instance, the subsampling factor can be N=1 for rank 1-2, and N=2 for rank 3-4, and N=4 for rank 5-8. As shown in Appendix, a similar approach was adopted while designing the Rel. 15 UL 4Tx codebook. An example of the subsampling can be as follows:

  - (i11,i12):

    - {0,1,2,3} for rank 1-2

    - {0,2} for rank 3-4

    - {0} for rank 5-8

  - i13 (rank 2-4): only i13=0

  - i2:

    - {0,1,2,3} for rank 1,

    - {0,1} for rank 2-4, and

    - {0} for rank 5-8

Simulation results comparing FC precoders with or without subsampling are provided in Section 3. It has been shown that the number of rank 1-4 FC precoders can be around 64 in order to achieve the best UPT vs TPMI overhead trade-off.

Proposal 4: support the following regarding full-coherent precoder design

- Codebook parameters

  - L=1

  - Oversampling factor Oi=2 can be supported at least for rank 1-2

- Codebook subsampling: Rel. 16 DL Type I single codebook is subsampled by a factor N, i.e., a subset of Rel. 15 Type I codebook is used as FC precoders in 8Tx UL codebook

  - Rank 1-2: N=1 (no subsampling)

  - Rank 3-4: N=2 (subsampling by 2)

  - Rank 5-8: N=4 (subsampling by 4)

  - (i1,1,i1,2,i2): number of supported values decreases with rank (e.g. rank 1-2 supports all values, and rank > 2 supports a subset of values)

  - i1,3: support only i1,3=0

Partial coherent precoder design

The partial coherent precoders for Ng=2 is agreed to be based on the full-coherent (FC) 4Tx precoders. There are two alternatives for further discussion.

- Alt1: based on one FC 4Tx TPMI indicating a rank-R 4Tx precoding matrix PR

  - Rank 1: one 4Tx rank 1 TPMI + indication of 1 of the 2 groups

  - Rank 2-4: depending on distribution of layers across groups (L1,L2)

    - When Li=0, one 4Tx rank 2-4 TPMI + indication of 1 of the 2 groups

    - When Li>0, Li out of R layers (columns) of the 4Tx precoding matrix PR are assigned to Group i. For example,

      - Group 1: Layers (columns) 1,…L1 of the 4Tx precoding matrix PR

      - Group 2: Layers 4-L2+1,…4 of the 4Tx precoding matrix PR

  - Rank 5-8: depending on distribution of layers across groups (L1,L2). The value of R=4, and Li out of R layers (columns) of the 4Tx precoding matrix PR are assigned to Group i. For example,

    - Group 1: Layers (columns) 1,…L1 of the 4Tx precoding matrix PR

    - Group 2: Layers 4-L2+1,…4 of the 4Tx precoding matrix PR

- Alt2: based on two FC 4Tx TPMIs (one per group)

  - Group 1: rank-L1 4Tx FC TPMI

  - Group 2: rank-L2 4Tx FC TPMI

Alt1 has much lower overhead than Alt2, but, can incur performance loss. However, the performance loss is expected to be not large. Two examples of the PC codebook design based on Alt1 are shown in Table 1, where the notation PR(I) indicates 4Tx precoders corresponding to layers with indices I of a rank-R 4Tx precoder.

Table 1

| (L1,L2) | 4Tx FC TPMIs (I) | 8Tx precoders | 8Tx precoders (without scaling) |
| --- | --- | --- | --- |
| (1,1) | 14-21 | P2(1)O OP2(2)) | 11110000 000011-1-1,11jj0000 000011-j-j,1j1j0000 00001j-1-j,1jj-10000 00001j-j1, 1-11-10000 00001-1-11,1-1j-j0000 00001-1-jj,1-j1-j0000 00001-j-1j,1-jj10000 00001-j-j-1 |
| (2,3) | 3-4 | P4(1,2)O OP4(2,3,4)) | 11110000 1-11-10000 00001-11-1000011-1-1 00001-1-11,11jj0000 1-1j-j000000001-1j-j000011-j-j 00001-1-jj |

For Ng=4, there are two alternatives to down-select from:

- Alt1: based on Rel. 15 UL 2Tx FC precoders

- Alt2: based on Rel.15 UL 4Tx partial-coherent (PC) precoders

Either alternative seems plausible as the starting point. One advantage of Alt2 though is that Rel.15 4Tx PC precoders based design can reduce the number of candidate precoders significantly, when compared with 2Tx full-coherent based design. So, we support Alt2.

Figure 2

An example of the PC precoder design is shown in Figure 2. For Ng=4, based on PC 4Tx precoders (shown in blue boxes).

- Rank 1: one rank 1 PC 4Tx precoder

  - Group 1 g0 (g from TPMI4-7)

  - Group 2 g0 (g from TPMI8-11)

  - Group 3 0g (g from TPMI4-7)

  - Group 4 0g (g from TPMI8-11)

- Rank >1: similar to Ng=2

Proposal 5: support the following regarding partial coherent precoder design

- Ng=2: based on one Rel. 15 UL 4Tx full-coherent precoder PR

  - Group 1: Layers (columns) 1,…L1 of the 4Tx precoding matrix PR

  - Group 2: Layers (columns) 4-L2+1,…4 of the 4Tx precoding matrix PR

- Ng=4: Alt2 (based on Rel. 15 UL 4Tx partial-coherent precoders)

TPMI overhead reduction

As agreed, mechanisms to reduce TPMI payload need to be considered in order to keep TPMI payload reasonable. We can consider the following example mechanisms to keep the codebook size hence the TPMI payload reasonable.

Mechanism 1: based on codebook parameters

The values of the codebook parameters can be one approach to reduce payload.

- Single DFT beam: the number of DFT beams can be fixed to 1 (i.e., L=1 which corresponds to codebookMode1 in DL Type I codebook).

- Lower oversampling factors (O1,O2): similar to Rel. 15 4Tx UL codebook (cf. Appendix A, [5]), the oversampling can be less (e.g. 1 or 2) than DL T1 CB (i.e. 4). In particular, Oi=2 for rank 1-2 and Oi=1 for rank > 2.

Mechanism 2: based on efficient signalling

The UL precoding indication indicates two components: (A) selection of antenna group(s), and (B) for FC or PC antenna group(s), a TPMI indicating a precoding matrix across the selected group(s). The indication about (A) and (B) can be according to the following examples:

- Ex1: two separate indicators, one for (A) and another for (B).

- Ex2: a joint indicator, e.g. TPMI

Note that SRI and TPMI fields already exist in legacy UL-related DCI. The legacy UL precoding indication corresponds to Ex2. However, for 8Tx, Ex1 can be more efficient UL precoding indication scheme from spec simplicity perspective, since it can keep the TPMI payload small and decouples antenna selection aspect from the precoding indication, and couples it with SRI (which is intended to indicate selection of antennae/resources). Therefore, SRI can be used indicating (A) and TPMI can be used for indicating (B).

Proposal 6: support the following mechanisms to reduce TPMI payload

- Mechanism 1: based on codebook parameter

  - L=1

  - Lower oversampling factors: Oi=2 for rank 1-2 and Oi=1 for rank > 2

- Mechanism 2: based on efficient signalling for the indication of (A) antenna group(s), and (B) UL precoding matrix, e.g. two separate indicators, e.g. SRI for (A) and TPMI for (B)

Full power modes

The Rel. 16 supports 3 different full power modes. A summary of potential enhancements for different full power modes is provided in Table 2.

Table 2

| Rel.16 full power modes | Use case/feature | Potential enhancements for 8Tx |
| --- | --- | --- |
| fullPower (or Mode0) | All full-rated PAs | Reuse (no spec change expected) |
| fullPowerMode1 | No full-rated PAs - Introduce a TPMI (x) to combine non-full-rated PAs to achieve full power | 8Tx TPMI (x) is needed |
| fullPowerMode2 | SRS resources with different #SRS ports in a set - SRS virtualization to achieve full power | UE capability on SRS resource with different #ports |
|  | Full power TPMI group(s) reported by UE - Full power only for reported TPMIs - Legacy power scaling for remaining TPMIs | UE capability on 8Tx TPMI group(s) |

Since the full power design depends on the PC and NC 8Tx codebook design, the full power modes for 8Tx should start after the 8Tx codebook designed is almost complete.

Proposal 7: Discussion on full power modes can start after the 8Tx codebook design is sufficiently mature

## Non-codebook-based transmission

Agreement [2] For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set, - All SRS port combinations are supported - For SRI indication, down-select from, - Option 1: Use an NSRS bit length bitmap - Option 2: Use a legacy-based solution - Consideration of Lmax for SRI indication For NSRS≤4, Rel-15 SRI indication is reused Agreement [3] For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt1 is supported, that is - Alt1: A single SRS resource set configured with up to 8 single-port SRS resources - FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively

For NCB based UL, there are two open issues.

- Number of SRS resource set: when NSRS≤4, the legacy (Rel.15) NCB based UL can be reused, and hence one SRS resource set with NSRS resources can be configured. When NSRS>4, then the NSRS SRS resources can be configured via one or two SRS resource sets. One resource set corresponds to simple extension of legacy Rel. 15 solution, whereas two SRS resource sets can provide more flexibility, and can also be used for mTRP schemes (e.g. Rel. 17 mTRP PUSCH). We therefore prefer to exploit number of SRS resource sets, and consider both one and two SRS resource sets for NCB based UL.

- SRI indication: based on the agreement in RAN1#112, when NSRS>4, the following aspects can be considered regarding the SRI indication.

  - Lmax: the configured value of the max number of UL layers (Lmax) can be 4 or >4.

  - Indication scheme: the SRI indication scheme is according to one of the following options.

    - Option1: a bitmap comprising NSRS bits is used, where the location of ‘1’s indicate the indicated SRS resource(s). The bitmap based scheme can be simpler (when compared with a combinatorial index based scheme).

    - Option2: the legacy scheme based on the combinatorial index is used (extension of Rel. 15 SRI indication tables). The issue with the alternative is the spec-complexity due to (a) extending legacy tables for Lmax=1,…,4 for NSRS=5,…,8, and (b) introducing new tables for Lmax=5,…,8. The spec-complexity can save only 1-2 bits at the most when Lmax is small, and there is no saving when Lmax is large.

Observation 3: when compared with a bitmap approach, SRI indication for non-codebook-based 8Tx UL operation based on combinatorial indexing has large spec-complexity, and can save 1-2 bits only when Lmax is small, and there is no saving for large Lmax.

Proposal 8: regarding 8Tx NCB based UL transmission,

- Number of SRS resource sets: support two SRS resource sets in addition to one SRS resource set

- When NSRS>4, support Option1 (the SRI indication is based on a length-NSRS bitmap)

## 2 CWs

Agreement [2] To support dual CW PUSCH transmission for rank>4 by an 8TX UE, for MCS indication, support - Alt.2: A second MCS field (5 bits) is indicated for the second codeword Agreement [2] To support dual CW PUSCH transmission for rank>4 by an 8TX UE, a second set of NDI (1 bit) and RV (2 bits) fields are indicated. - FFS: Details on how to signal Agreement [2] To support dual CW PUSCH transmission for rank>4 by an 8TX UE, reuse DL PDSCH scrambling mechanism to initialize the scrambling sequence generator for codeword q∈{0,1}, cinit=nRNTI∙215+q∙214+nID where nRNTI, and nID are defined similar to the legacy single CW PUSCH transmission. Agreement [2] To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the CWs, down-select from, - Alt1: First CW - Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW) Working Assumption [1] For uplink transmission with rank>4, support dual CW transmission. Agreement [4] For PUSCH transmission with rank>4 by an 8TX UE, to support dual CW transmission, - specify MCS, NDI, RV indication for the second CW - specify PUSCH Scrambling for the second CW - specify UCI multiplexing on PUSCH for dual CW transmission - study whether/how Enabling/Disabling the second CW FFS: Optimization of DCI to indicate the above Note: Strive to reuse Rel-15 NR DL schemes where possible.

For >4 layers PUSCH transmission, we support to confirm the working assumption on 2 CWs with completely reusing legacy DL CW-layer mapping.

Our view about the remaining issues to support 2CW is as follows.

- UCI multiplexing: there are two alternatives to down-select from. Among these, we support Alt1 since it is simple, and has the least spec impact. Besides, the need/benefit of Alt2 (UCI mux on the CW with the highest MCS) is unclear.

- Whether/how to enable/disable 2nd CW: we do not support any new signaling for this. Since the 2nd CW has to be enabled when rank > 4, the higher layer configuration of maxRank (as in legacy) is sufficient.

Proposal 9: regarding 2 CWs for > 4 layers

- confirm the working assumption

- UCI multiplexing: support Alt1 (UCI is multiplexed on the 1st CW)

- Enable/disable 2nd CW: do not support any new signalling, and higher layer indication of maxRank can be used to enable/disable the 2nd CW.

SLS results

The SLS results according to the agreed EVM in RAN1#109-e (Table 4 in Appendix B) are provided in this section for the following setting:

- 8Tx antenna layout: Alt 1-b (Ng=1, (M, N ,P)=(1,4,2)

- Full-coherent precoders based on DL Type I codebook: only full coherent precoding matrices are considered, and the following alternatives for the codebooks (CBs) are compared. Two oversampling factors (O) are also considered. Three different evaluations are performed. A summary is provided in Table 3.

  - Evaluation 1: rank 1 only

    - O=1: CB0, CB4, CB5, CB6

    - O=2: CB0, CB4, CB5, CB6

  - Evaluation 2: Dynamic rank 1-2

    - O=1: CB0, CB4, CB5, CB6

    - O=2: CB0, CB4, CB5, CB6

  - Evaluation 3: Dynamic rank 1-4

    - O=1: CB0-CB9, i1,3=0

    - O=2: CB0-CB9, i1,3=0

    - O=1: CB0-CB9, i1,3=0,1,2

    - O=2: CB0-CB9, i1,3=0,1,2

- UE Tx power: 23dBm, 32dBm

- Modulation: 64QAM

Table 3

|  | Rank 1-1 | Rank 3-4 | Evaluation 1: rank 1 only | Evaluation 2: dynamic rank 1-2 | Evaluation 3: dynamic rank 1-4 |
| --- | --- | --- | --- | --- | --- |
| CB0 | No subsampling | No subsampling | Y | Y | Y |
| CB1 | No subsampling | Subsampling by 2, even-numbered |  |  | Y |
| CB2 | No subsampling | Subsampling by 2, odd-numbered |  |  | Y |
| CB3 | No subsampling | Subsampling by 2, random |  |  | Y |
| CB4 | Subsampling by 2, even-numbered | Subsampling by 4, even-numbered | Y | Y | Y |
| CB5 | Subsampling by 2, odd-numbered | Subsampling by 4, odd-numbered | Y | Y | Y |
| CB6 | Subsampling by 2, random | Subsampling by 2, random | Y | Y | Y |
| CB7 | Subsampling by 2, even-numbered | Subsampling by 8, even-numbered |  |  | Y |
| CB8 | Subsampling by 2, odd-numbered | Subsampling by 8, odd-numbered |  |  | Y |
| CB9 | Subsampling by 2, random | Subsampling by 8, random |  |  | Y |

The avg. UPT gain vs CB size trade-offs are shown in Figure 3 - Figure 8, for the three evaluations. In Figure 3 - Figure 6, two sets are results are provided, one for O=1 and another for O=2, and each set of results includes four points for CB0, CB4, CB5, CB6. We can observe the following. In Figure 7 and Figure 8, four sets are results are provided, one for O=1,2, and i1,3=0 and i1,3=0,1,2, and each set of results includes ten points for CB0-CB9. We can observe the following.

Observation 4: for full-coherent precoder design, when comparing avg. UPT gain and codebook size (or PMI overhead) overhead trade-offs, the following codebook sizes achieves the best trade-offs

- For rank 1 only: codebook size = 14-16 bits (CB0, CB4, or CB6)

- Dynamic rank 1-2: codebook size = 24 (CB0 or CB4)

- Dynamic rank 1-4: rank 1-4 codebook size = 64 (CB1 or CB2)

Figure 3

Figure 4

Figure 5

Figure 6

Figure 7

Figure 8

Conclusion

In this contribution, the following observations and proposals are made:

Observation 1: if two antenna groups comprise coherent antennae, they essentially can be combined into one antenna group for codebook design

Observation 2: a parameterized approach similar to Rel. 15 DL Type I codebook can be adopted to simply codebook design and to have a solution scalable to >8Tx, if needed, in future

Observation 3: when compared with a bitmap approach, SRI indication for non-codebook-based 8Tx UL operation based on combinatorial indexing has large spec-complexity, and can save 1-2 bits only when Lmax is small, and there is no saving for large Lmax.

Observation 4: for full-coherent precoder design, when comparing avg. UPT gain and codebook size (or PMI overhead) overhead trade-offs, the following codebook sizes achieves the best trade-offs

- For rank 1 only: codebook size = 14-16 bits (CB0, CB4, or CB6)

- Dynamic rank 1-2: codebook size = 24 (CB0 or CB4)

- Dynamic rank 1-4: rank 1-4 codebook size = 64 (CB1 or CB2)

Proposal 1: antennae within a group are coherent, and antennae across multiple groups are non-coherent

- Do not support full coherent precoders with Ng=2,4

Proposal 2: reuse DL Type I codebook parameters (Ng,N1,N2) to describe/configure 8Tx UL codebook for different coherence types

- FC: Ng,N1,N2,P=1,4,1,2, 1,2,2,2

- PC: Ng,N1,N2,P=2,2,1,2,(4,1,1,2)

- NC: Ng,N1,N2,P=8,-,-,-

- FC/PC precoders: comprises two components

  - selection of antenna group(s)

  - precoder associated with the selected antenna group(s)

- NC precoders: selection of antenna group(s), where a group comprises single antenna

Proposal 3: similar to Rel.16-18 Type II codebook design, the metric for evaluating different 8Tx codebook alternatives should be UPT gain vs codebook size (TPMI overhead).

Proposal 4: support the following regarding full-coherent precoder design

- Codebook parameters

  - L=1

  - Oversampling factor Oi=2 can be supported at least for rank 1-2

- Codebook subsampling: Rel. 16 DL Type I single codebook is subsampled by a factor N, i.e., a subset of Rel. 15 Type I codebook is used as FC precoders in 8Tx UL codebook

  - Rank 1-2: N=1 (no subsampling)

  - Rank 3-4: N=2 (subsampling by 2)

  - Rank 5-8: N=4 (subsampling by 4)

  - (i1,1,i1,2,i2): number of supported values decreases with rank (e.g. rank 1-2 supports all values, and rank > 2 supports a subset of values)

  - i1,3: support only i1,3=0

Proposal 5: support the following regarding partial coherent precoder design

- Ng=2: based on one Rel. 15 UL 4Tx full-coherent precoder PR

  - Group 1: Layers (columns) 1,…L1 of the 4Tx precoding matrix PR

  - Group 2: Layers (columns) 4-L2+1,…4 of the 4Tx precoding matrix PR

- Ng=4: Alt2 (based on Rel. 15 UL 4Tx partial-coherent precoders)

Proposal 6: support the following mechanisms to reduce TPMI payload

- Mechanism 1: based on codebook parameter

  - L=1

  - Lower oversampling factors: Oi=2 for rank 1-2 and Oi=1 for rank > 2

- Mechanism 2: based on efficient signalling for the indication of (A) antenna group(s), and (B) UL precoding matrix, e.g. two separate indicators, e.g. SRI for (A) and TPMI for (B)

Proposal 7: Discussion on full power modes can start after the 8Tx codebook design is sufficiently mature

Proposal 8: regarding 8Tx NCB based UL transmission,

- Number of SRS resource sets: support two SRS resource sets in addition to one SRS resource set

- When NSRS>4, support Option1 (the SRI indication is based on a length-NSRS bitmap

Proposal 9: regarding 2 CWs for > 4 layers

- confirm the working assumption

- UCI multiplexing: support Alt1 (UCI is multiplexed on the 1st CW)

- Enable/disable 2nd CW: do not support any new signalling, and higher layer indication of maxRank can be used to enable/disable the 2nd CW.

# References

- RAN1#110, Chairman’s notes

- RAN1#112, Chairman’s notes

- RAN1#110bis-e, Chairman’s notes

- RAN1#111, Chairman’s notes

- RAN1#91, Chairman’s notes

# Appendix A: Rel. 15 4Tx UL codebook

Agreement:

Agreement:

Agreement:

Agreement:

# Appendix B

Table 4

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 OFDM symbol slot SCS , 30 KHz |
| Scenario | Outdoor FWA (38.901): UMa (ISD = 500 m), 100% Outdoor, 3Km/h |
| Channel model | 38.901 |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | Outdoor FWA : (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 |
| gNB antenna radiation pattern parameters | Outdoor FWA : 38.901 Table 7.3-1, 8 dBi , 65° HPBW |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE TX antenna configuration | Case 1-b: Ng=1, (M,N,P) = (1,2,4) |
| Traffic model | - FTP model 1: Packet size 500KB, RU= 50% and suggested medium RU of value of 50% |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50 |
| UE power rating | 23 dBm (UE, 38.101) 32 dBm (FWA, 38.101) |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |