3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303127

e-Meeting, April 17th -26th, 2023

Agenda Item:	9.3.2

Source:	Samsung

Title:	On SBFD for NR duplex evolution

Document for:	Discussion

# Introduction

The SID for NR duplex evolution aims to identify solutions and evaluate their feasibility to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation in unpaired spectrum [1].

In the Rel-18 study item, it is assumed that the duplex enhancements are deployed at the gNB side and that the TDD UE operates in half-duplex. Subband full-duplex (SBFD) operation on a time-domain resource is limited to FDM on the NR carrier. gNB-side full-duplex operation when scheduling or configuring the DL transmissions to UEs and the UL receptions from UEs use non-overlapping SBFD DL and UL subbands in frequency-domain. Note that in the remainder of this contribution we will simply refer to SBFD for subband full-duplex operation using non-overlapping subband allocations, e.g., as assumed by the Rel-18 SID.

In Section 2 we discuss backwards compatibility aspects and benefits of gNB-side SBFD operation when legacy UEs are present in the NR TDD cell, e.g., SBFD transparent mode. In Section 3, we discuss SBFD design requirements.

In Section 4, we discuss SBFD operation with respect to configuration and scheduling on legacy downlink and flexible symbols. In Section 5, we discuss potential benefits of UE-side SBFD enhancements to provide support for gNB-side SBFD operation. Annex A shows results and discusses findings based on the Samsung FR1 and FR2-1 testbeds to demonstrate that gNB-side SBFD operation is feasible and viable for commercial NR TDD network implementations.

# SBFD and legacy UEs

When SBFD operation is supported by the gNB in the TDD NR cell, backwards compatibility for legacy UEs is required. Functional UE backwards compatibility and operational backwards compatibility must be considered.

Functional UE backwards compatibility implies that legacy Rel-15 to Rel-17 UEs can still connect to the NR TDD cell and establish service. Transmission and reception of data and the RRC/NAS control signaling should still be possible without undue performance impact. In addition, legacy UEs can potentially benefit from the availability of SBFD operation in the NR TDD cell, e.g., using tdd-UL-DL-ConfigCommon configured with DFFFU and scheduling simultaneous transmissions and receptions for legacy UEs on the flexible symbols.

Operational backwards compatibility means that legacy UEs should still be able to use any existing deployed NR network features such as NR intra-band or inter-band or FR1-FR2 CA, EN-DC, or NR-DC when SBFD operation is introduced in the NR TDD cell.

Legacy UEs using SA mode on the NR TDD cell with gNB-side SBFD operation access the NR TDD cell as their serving cell. These UEs must be able to acquire the NR TDD cell with gNB-side SBFD operation during initial cell-selection and complete initial network access involving NAS and AS. These UEs perform cell re-selection procedures when in RRC_IDLE or RRC_INACTIVE modes when camped on the NR TDD cell.

Using n78, NR channel BW = 100 MHz, SCS = 30kHz and DXXXU with gNB-side SBFD operation in slots #1, #2, and #3 as example in Figure 1, the gNB can configure SIB1 for the legacy UEs with tdd-UL-DL-ConfigCommon as {dl-UL-TransmissionPeriodicity, nrofDownlinkSlots, nrofDownlinkSymbols, nrofUplinkSlots, nrofUplinkSymbols} = {P = 2.5ms, 3 DL slots, 12 DL sym, 0 UL sym, 1 UL slot}, or DDDDU. The legacy NR UEs then consider only the last 2 symbols in slot #3 as flexible symbols. This disables any possibility to schedule UL transmissions for the legacy UEs in the SBFD UL subband. When SIB1 tdd-UL-DL-ConfigCommon for the legacy UEs is configured with {dl-UL-TransmissionPeriodicity, nrofDownlinkSlots, nrofDownlinkSymbols, nrofUplinkSlots, nrofUplinkSymbols} = {P = 2.5ms, 1 DL slot, 0 DL sym, 0 UL sym, 1 UL slot}, or DFFFU, the gNB can schedule or configure UL transmissions using the SBFD UL subband for the legacy UEs in the flexible symbols of slots #1, #2 and #3. The UL BWPs of the legacy UEs must be configured inside the BS channel BW such that they contain the SBFD UL subband. There is no specification impact.

Note that we assume here that no UE-dedicated, e.g., no UE-specific RRC UL-DL frame configuration is provided to the UEs by RRC. Similarly, the available but optional Rel-15 SFI feature, e.g., DCI format 2_0 is not assumed as configured. This assumption corresponds to typical real-life NR TDD deployments. The Rel-15 FG 5-1 (Basic scheduling/HARQ operation) is UE mandatory, but FG 5-1a (UE-specific RRC UL-DL frame configuration) and FG 3-6 (SFI) are optional UE features and not usually found implemented.

Different Rel-15 based UE modem chipsets and stacks may implement inconsistent behavior when following the gNB scheduling assignments for the semi-static symbols/slots configured as F based on Rel-15 specifications. It is not a safe assumption to make that SBFD transparent mode can always be used for scheduling any legacy UE. Unexpected behavior may result in some cases.

Figure 1: SBFD operation for legacy UEs, e.g., SBFD transparent mode

When SBFD transparent mode based on DFFFU is configured in the NR TDD cell, the gNB can schedule UL transmissions to the legacy UEs using the SBFD UL subband of slots #1, #2 and #3. PUSCH resource allocation type 1 must be used. Some limitations exist with respect to the CSI-RS resource configurations when configured in these slots. For legacy UEs, UL scheduling by the gNB using the SBFD UL subband cancels reception for the RRC configured DL signals. Similar considerations apply to RRC configured UL signals/channels. Reception of a scheduling DCI by the UE will cancel the configured UL transmission and the UE transmits or receives the scheduled signal/channel.

- Observation 1: Functional backwards-compatibility for legacy UEs when SBFD-operation is supported in the NR TDD cell can be achieved by configuring DDDDU in the SIB1 tdd-UL-DL-ConfigCommon.

- Observation 2: It cannot be assumed that SBFD transparent mode for legacy UEs when configuring DFFFU in the SIB1 tdd-UL-DL-ConfigCommon will always results in consistent UE behavior.

When SBFD operation is supported in a TDD cell in the NR band, the legacy NR UEs can be configured for NR intra-band CA, NR inter-band CA, FR1-FR2 CA, EN-DC, or NR-DC. For operational backwards compatibility, the NR TDD cell with gNB-side SBFD support must be able to operate as PCell, SCell or SpCell at least for the legacy UEs. In addition, it is very desirable that the same principle holds also for the SBFD-aware UEs in order to not impact existing operator deployments. Introduction of gNB-side SBFD support in selected network region for a given NR band should not affect the existing operator deployment assumptions with respect to SA or NSA mode supported for network access or the availability of band-specific CA or DC features for an existing assumed user base.

Using the example of NR (FR1-FR1) inter-band CA in Figure 2, it can be expected that a mid-band NR TDD cell with SBFD support will often be configured as an SCell. Some relevant deployment cases are n3+n78 or n41+n78. The n78 (3.5 GHz) NR TDD cell with SBFD support can use up to 100 MHz channel BW. An n3 (1.7 GHz) NR FDD carrier or an n41 (2.5 GHz) NR TDD carrier can provide the coverage layer and are then configured as the PCell for the legacy UEs. Note that simultaneous Tx/Rx capability is mandated for the UE for n3+n78, but not for n41+n78. In this case, the PCell is located in a different band and the NR TDD SCell with SBFD support will then carry DL data. The NR TDD SCell with SBFD support will then mostly be used for DL data transmission, so the NR TDD SCell UL does not benefit from the availability of gNB-side SBFD support.

Using the example of NR FR1-FR2 CA, it is possible that the mid-band NR TDD cell with SBFD support is configured as the PCell. One relevant deployment case is n78+n257. For example, the n257 (28 GHz) NR TDD cell is then configured with 200 MHz and SCS=120 kHz. Note that simultaneous Tx/Rx capability is mandatory for the UE in this band combination (like for any FR1-FR2 band combination). The FR1 NR TDD PCell with SBFD support carries the DL and UL control signaling in L1, RRC, and NAS for both CCs. The FR1 NR TDD PCell with SBFD support can also carry DL and UL data, so improved UL performance in the PCell with SBFD support is beneficial.

Figure 2: SBFD operation on NR SCell with FR1 inter-band CA (n41+n78)

For the EN-DC or NR-DC deployment cases, an example for EN-DC is DC_1A_n78A and for NR (FR1-FR2) inter-band DC an example is DC_n78A-n257A using 2 NR bands. Note that UL data on the n78 NR TDD cell with SBFD support is then present in both example deployment cases. Any potential benefits or penalties resulting from the introduction of SBFD operation on the NR TDD cell can affect the effective network and/or the perceived UE performance.

In summary, SBFD deployment in an NR TDD band in a network region by the operator must not result in the need for the operator to separate or operate the legacy and the SBFD-aware UEs using different frequency layers of the NR deployment. It is necessary to consider the potential use and impact of the SBFD UL subband in a TDD cell with SBFD support when the cell is configured as PCell, SCell or SpCell for CA or for DC for either legacy UEs or SBFD-aware UEs.

- Proposal 1: RAN1 should evaluate the potential impacts from SBFD when the NR TDD cell with SBFD support is configured as PCell, SCell or SpCell for a legacy UE or for an SBFD-aware UE.

# SBFD design principles

When gNB-side SBFD is supported in the NR TDD cell, two different approaches exist for the network to configure the UE for SBFD operation. One approach and the focus of the Rel-18 SID is that the gNB configures the SBFD UL and DL subband(s) for the UE inside the UE channel BW, e.g., using NR single-carrier operation. Single-carrier based SBFD is relevant for NR operation in either FR1 or FR2-1. The other approach is the use of intra-band (contiguous) carrier aggregation where an SBFD UL subband is configured by the gNB using one of the component carriers for UL transmissions from UEs and the other component carriers for DL receptions by UEs. Similar to SBFD transparent mode for legacy UEs, CA-based SBFD can be considered supported by existing core specifications and for some Rel-17 FR2-1 band combinations.

Using NR single carrier for SBFD operation, full-duplex transmission/reception by the gNB on a symbol/slot is configured in the BS channel bandwidth. In an SBFD slot, the gNB schedules or assigns the DL and UL resources to UEs using non-overlapping frequency-domain resources.

When a half-duplex TDD UE is scheduled for DL reception in a slot, no simultaneous UL transmission from the same UE can then occur on the same time-domain resource. The gNB must account for sufficient Rx-Tx and Tx-Rx switching times for the UE when scheduling the DL and UL transmissions in different symbols/slots. The UL transmission from a UE in an SBFD slot occur in the configured SBFD UL subband. The DL transmissions to UEs in the SBFD slot occur in one or both of the SBFD DL subband(s) located at higher and lower range of the BS channel BW using ‘DUD’ as shown in Figure 3.

Figure 3: SBFD configuration using ‘DUD’

In frequency-domain, it can be expected that the SBFD UL subband is often located at the center of the BS channel BW, e.g., using ‘DUD’. An SBFD configuration of type ‘DUD’ provides most frequency-domain separation and coexistence isolation towards the adjacent channels in band segments of another operator. In cases where an operator holds an NR band segment at the edge of the NR band, or for specialized TDD deployment types such as isolated/controlled NR industrial deployment, configuring the SBFD UL subband at the edge of the BS channel BW, e.g., using ‘DU’ or ‘UD’, is possible. For the SBFD configurations of type ‘DU’ or ‘UD’, there will then be only 1 SBFD DL subband located at the upper or at the lower edge of the BS channel bandwidth.

It is not meaningful to assume a single fixed size of the SBFD UL subband will always be sufficient for all deployment needs. At least for FR1 evaluation purposes, it may be assumed that the operator owns a contiguous 100 MHz band segment in n78 and that the SBFD UL subband is from 20 to 25 MHz wide. However, many operators own n78 band segments of smaller size, e.g., 40 or 60 MHz. Some operators may only hold 2 disjoint band segments in n78. The need to configure an SBFD UL subband of smaller size than 20 MHz exists. Similar considerations with respect to the need for flexible SBFD UL subband configuration suitable to deployment needs hold true for FR2-1 or when considering the NR industrial type deployments. A configurable size and location for the SBFD UL subband in frequency-domain does not imply that the size and the location, e.g., the frequency-domain occupancy, of the SBFD UL subband needs to change on a per-symbol allocation basis. It may be expected that the gNB-side SBFD implementation imposes practical limits with respect to the feasibility of UL receptions outside a maximum possible reception bandwidth while respecting the KPIs due to antenna design, SIC implementation and/or filtering. These constraints will then be the same for all symbols/slots where SBFD operation is possible.

Another approach for the network to configure the UE for SBFD operation is the use of intra-band CA as shown in Figure 4. The latter approach is of particular interest when SBFD is deployed in FR2-1.

Using intra-band carrier aggregation in FR2-1, either 3 or 4 contiguous CCs can be configured for the UE in the aggregated BS channel BW. With 3-CC contiguous intra-band CA, the SBFD UL subband can be configured in the center of the aggregated transmission BW. Using 3-CC intra-band CA in FR2-1, the SBFD UL subband is configured with an UL-heavy UL-DL frame configuration in one of the CCs and the SBFD DL subband(s) are configured on the 2 remaining CCs.

Figure 4: SBFD using 3 CC contiguous intra-band CA in FR2-1

Using FR2-1 intra-band carrier aggregation for SBFD, more flexibility to deploy SBFD support in the NR TDD cell exists when compared to SBFD in the FR1 upper mid-band. When FR2-1 intra-band CA is configured for the UE, it is in principle possible to configure the DL BWPs corresponding to the SBFD DL subbands independently from the size of the SBFD UL subband, or to use features such as R16 SCell Dormancy for reception in the SBFD DL subbands, or to configure the CCs with unaligned frame boundaries and the slot offset. NR FR2-1 introduced Independent Beam Management (IBM) and Common Beam Management (CBM) features with simultaneous DL reception on different CCs from the co-located and non-collocated TRPs in Rel-16 and Rel-17. IBM implies that a UE is capable of DL simultaneous reception on different UE panels/chains using separate beams on different CCs and requires improved UE BB and RF capabilities, e.g., multiple BB chains and support of multiple antenna panels. The Rel-18 FR2 enhancements are ongoing and will introduce requirements for FR2-1 UEs capable of multi-beam/multiple Rx chain simultaneous DL reception on a single CC to achieve improved RF, RRM and UE demodulation performance. Note that the use of FR1 intra-band TDD CA for purpose of enabling SBFD operation is significantly more challenging than using a single NR carrier for SBFD operation in FR1 for a variety of reasons.

While the focus of the Rel-18 SID is on SBFD operation using a single NR carrier, feasibility aspects and potential enhancements for SBFD operation using intra-band contiguous CA in FR2-1 should also be investigated at a later stage of the Rel-18 SID. One area for such potential enhancements in FR2-1 is the Rel-16 directional collision handling.

# SBFD configuration and scheduling

In RAN1#109-e it was agreed that the time and frequency location of subbands within a TDD carrier are not fixed in the specification. In RAN1#111 it was agreed that the understanding for the RAN1 study is that for semi-static configuration of subband frequency locations for SBFD operation, the frequency location of SBFD DL and UL subbands is with reference to CRB grid.

In RAN1#110bis-e, it was agreed that SBFD operation Alt.4 is baseline. It was also agreed that for semi-static configuration of the subband frequency locations for SBFD operation, at least the explicit indication of frequency location of the UL subband is required. It was left FFS if the frequency location(s) of other SBFD subbands types are explicitly indicated or implicitly determined, e.g., for the SBFD DL subband(s) or the guard band(s). It was also agreed that for semi-static configuration of the SBFD subband time locations for SBFD operation, explicit configuration of SBFD subband time locations within a period is the baseline.

In RAN1#111 and RAN1#112, several RAN1 agreements were made with respect to SBFD configuration and UE scheduling options for semi-statically configured downlink and flexible slots/symbols, respectively.

In Section 4.1, we first provide our views on the SBFD DL subband and guard band configurations, i.e., using explicit indication or implicitly derived by the UE. In Section 4.2 we consider UE scheduling for DL and/or UL when legacy DL and F symbols/slots are configured with an SBFD UL subband for SBFD-aware UEs. In Section 4.3 we discuss SBFD configuration and related signaling design aspects, e.g., per-symbol or per-slot granularity for SBFD time-domain configuration, need and potential benefits of group-common signaling and the handling of special DL signals/channels such as SSBs, the CORESET#0, or TRS.

In our view, most of the detailed signaling aspects related to Stage 3 design for SBFD UL and DL subband configuration can be left to the later WID. One important decision point for the Rel-18 SID is the need to agree if signaling design for SBFD configuration should assume that UE operation is supported only in RRC_CONNECTED mode, or additionally is supported also for UEs in RRC_IDLE/INACTIVE mode.

Note that we consider potential time domain collision handling rules for the SBFD-aware UEs as relevant. However, we consider that more Rel-18 SID agreements on SBFD configuration and the supported UE scheduling options for SBFD-aware UEs on legacy D and/or F symbols are required before possible new collision cases arising from SBFD operation can be meaningfully identified and concluded upon. We note that the set of SBFD-specific collision handling rules needs to account for presence of all DL or UL signals or channels. Therefore, more Rel-18 SID agreements which are related to principles of configuration and use of CSI-RS for CSI reporting and of SRS for CLI reporting are also prerequisite to identify additional collision cases, if any, arising from SBFD operation.

## 4.1	Guard bands and DL subband(s)

It can be expected that guard band(s) with a size of up to 5 RBs between the SBFD DL subband(s) and the SBFD UL subband are required by the gNB to avoid inter-subband CLI and to control the DL-UL self-interference levels for many gNB-side SIC implementations. In Annex A we show that time-domain digital SIC for example reduces the number of required guard RBs. SBFD operation assuming the possibility for the gNB to use a number of guard band RBs is therefore very meaningful, but the size of the guard band should remain configurable because it is a function of the gNB SIC implementation.

Depending on the gNB-side SIC implementation, guard bands can be required for SBFD operation in legacy DL and F symbols, i.e., symbols configured as D or F by TDD-UL-DL-ConfigCommon. If the gNB-side SIC implementation requires guard bands between the SBFD UL subband the SBFD DL subband(s), it will then need to use the guard band(s) irrespective of the SBFD operation using a legacy D or F symbol. The ability of the gNB-side SBFD implementation to schedule a legacy F symbol as DL-only or UL-only depends on the gNB-side antenna configuration and the gNB-side SIC implementation. When the gNB supports scheduling of the legacy F symbol as a DL-only or as an UL-only symbol, SBFD operation doesn’t occur on the symbol. The guard band(s) on the D or F symbol are then not required.

Fixed-size guard bands, e.g., introduced and defined for a particular RF band combination and function of a given SBFD UL subband size can potentially reduce cell throughput in the SBFD deployment. The resources of the guard band are then always reserved and cannot be scheduled by the gNB. Also, the guard band would then need to be present even if neighbor cells do not schedule the SBFD UL subband for UL transmission in the SBFD slot, e.g., un-schedulable RBs would exist even if the guard band size could be reduced due to less CLI.

In principle, it is first possible to consider that the gNB creates the guard band(s) through the DL and UL scheduling assignments and the presence or size of the guard band(s) is not known to the SBFD-aware UEs (Alt.1 in Figure 5). The gNB schedules or configures the DL or UL signals/channels and creates guard band(s) according to its needs by not assigning the desired number of guard band RBs next to the configured SBFD UL subband. UEs follow the gNB DL/UL scheduling decision or follow the RRC configured grants and do not know about the existence of the SBFD guard band(s). Note that for legacy UEs in a TDD cell with SBFD support, such implicit gNB scheduling-created guard bands must always be used.

When the presence and size of the SBFD guard band is not known to the UEs, this gives increased flexibility to the gNB implementation. For example, the gNB can then schedule UL or DL signals/channels from/to UEs in the SBFD symbol when much spatial isolation is possible using a smaller or no guard band. In cases where only small isolation for a DL and UL UE pair scheduled in the SBFD slot is available, a larger guard band can be used by the gNB. Different time-domain symbols can use a different number of guard band RBs. There is no specification impact.

Figure 5: SBFD operation and SBFD guard bands

When the size and frequency-domain locations of the guard band(s) are known to SBFD-aware UEs (Alt.2 in Figure 5), then the UE implementation can potentially benefit from additional UE Rx and Tx-side BB filtering on the PDSCH and PUSCH allocation BW in the SBFD DL and UL subband(s) when transmitting or receiving in SBFD slots, respectively. While it should not be assumed that SBFD-aware UE implementations must use more stringent Tx/Rx BB filtering than what is currently necessary to meet the existing Rel-15 NR UE RF requirements to improve their in-channel selectivity and in-band blocking performance, it is at least desirable to enable such UE functionality as a capability.

On the SBFD symbols, e.g., symbols where an SBFD UL subband is configured for the UE, the UE operational UL bandwidth can be scaled down to the size of the SBFD UL subband when compared to the normal UL slot where the UE operational UL bandwidth is given by the size of the UL BWP. UE modem functional blocks such as D/A converter and BB can use a lower voltage and/or a lower clock rate due to less data sample for processing. This is beneficial for the UE Tx PA regime and reduces UE power consumption.

When the UE knowns the frequency-domain occupancy of the guard band RBs, the UE implementation can also implement BB filtering of the Tx-side waveform in the PUSCH allocation BW and treat the guard band RBs as excess bandwidth. We note however that UE Tx-side BB filtering to make the waveform “gentler” for purpose of optimizing the UE PA is subject to many constraints, e.g., the available UL transmit power and PAPR must also be controlled with respect to EVM, ACLR, IBE and SEM. It can therefore not be assumed that potential gains) are uniform or are always available even when the UE knows the presence and size of the SBFD guard band(s).

Two approaches then exist to make the SBFD guard bands on the SBFD symbol known to the UE. The UE can be explicitly configured with the SBFD guard band configuration (Alt.2a), or the guard band configuration can be implicitly derived by the UE (Alt.2b) based on the frequency-domain occupancies of the SBFD DL and UL subbands and/or the NR DL carrier BW. It has already been agreed that the frequency-domain occupancy of the SBFD UL subband is explicitly configured. Explicit configuration or indication of the SBFD guard band(s) should be considered in conjunction with explicitly configured or implicitly derived SBFD DL subband(s).

When the frequency-domain locations of the SBFD DL subband(s) are explicitly configured by RRC, an implicitly derived SBFD guard band can be created by not configuring a number of RBs separating the SBFD DL or UL subband(s). When the SBFD DL subband(s) are not explicitly configured by RRC for the UE, the UE must use its knowledge of the DL and UL NR carrier BW and the SBFD UL subband size to determine the remaining RBs in an SBFD symbol as SBFD DL subband(s) and guard band(s). One possibility is then to rely on explicitly configured SBFD guard band(s) and to implicitly derive the SBFD DL subband(s). We do not consider this approach as sufficiently flexible. It must be considered that future SBFD operation using (partially or fully) overlapping subband configurations will require both explicit SBFD DL and UL subbands configuration.

In summary, we consider it beneficial for the UE implementation to know the presence and size of the guard band on the SBFD symbols. The SBFD DL subband(s) configuration should be explicitly configured for the UE on the SBFD symbol, i.e., on a symbol indicated as D or F by TDD-UL-DL-ConfigCommon. No explicit signaling for the SBFD guard band configuration is needed. The UE derives the SBFD guard band configuration implicitly as any unused RBs not explicitly configured or indicated as SBFD UL or DL subband(s) on the SBFD symbol.

- Proposal 2: The SBFD-aware UE is configured with the frequency-domain occupancy of the SBFD DL subband(s) on an SBFD symbol.

- Proposal 3: The frequency-domain occupancy of the SBFD guard band(s) on an SBFD symbol is determined by the SBFD-aware UE as any RB not configured as SBFD UL or DL subband(s) in the NR carrier bandwidth.

## 4.2	DL and UL scheduling

When introducing SBFD support in an NR TDD cell, one key Rel-15 NR feature to preserve is the support for dynamic TDD operation where the gNB scheduler can dynamically determine the transmission direction. NR can use 3 different signaling mechanisms to provide the information to the UE whether a radio resource is assigned for the UL or the DL transmission direction. The basic principle is that the UE monitors the PDCCH and then transmits in the UL or receives in the DL according to the received scheduling DCI. The TDD UL-DL frame configuration can be signaled to the UE as a cell-common pattern, e.g., tdd-UL-DL-ConfigCommon in SIB1. In addition, a UE-specific TDD UL-DL frame configuration for the F symbols in the cell-common pattern can be configured by tdd-UL-DL-ConfigDedicated or SFI, i.e., DCI format 2_0, can be configured. Note that the Rel-15 FG 5-1 (Basic scheduling/HARQ operation) is UE mandatory, but FG 5-1a (UE-specific RRC UL-DL frame configuration) and FG 3-6 (SFI) are optional UE features and not usually found implemented.

The following RAN1 agreement was taken in RAN1#112 with respect to SBFD configuration and UE scheduling options for semi-statically configured downlink and flexible slots/symbols, respectively.

Agreement

For dynamic SBFD,

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are not allowed

    - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are allowed

Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency.

For each option, additional conditions may apply to determine whether the option is applicable.

In this section, we first provide our views on UE scheduling when legacy DL symbols/slots are configured then for the case when legacy F symbols/slots are configured with an SBFD UL subband for SBFD-aware UEs.

### 4.2.1	SBFD operation on DL symbols

It is desirable to avoid that the introduction of the SBFD support in the NR TDD cell results in a hard UL-DL resource partitioning to avoid DL throughput and spectral efficiency penalties. When the SBFD DL and UL subband configuration is signaled to the UE by the gNB, such a configuration determines where an UL transmission or DL reception may occur. gNB scheduling then actually assigns the corresponding radio resources to actual UL transmissions from the UE or DL receptions by the UE as needed.

One important decision point for the Rel-18 SID is to evaluate the potential benefits and UE modem impacts when allowing for DL scheduling by the gNB inside the configured SBFD UL subband. One option is not to allow DL receptions outside the configured SBFD DL subband(s), e.g., DL-symbol Option 1 (or D-1) identified in the RAN1#112 agreement. The other option is to allow DL receptions in the configured SBFD UL subband, e.g., DL-symbol Option 2 (or D-2). D-1 results in semi-static UL-DL resource portioning at least from the UE perspective. Any DL reception on the SBFD symbol remains confined to the SBFD DL subband(s) and UL transmission is confined to the SBFD UL subband. D-1 is baseline for SBFD operation at least in the sense that SBFD operation in absence of a known and explicitly configured SBFD UL subband configuration as agreed would not otherwise be possible for the SBFD-aware UEs. D-2 allows for DL scheduling of UEs inside a configured SBFD UL subband in addition to D-1.

DL symbol Option 1 (D-1) and Option 2 (D-2) are shown in Figure 6. SBFD operation is configured for the SBFD-aware UE in slots #1, #2 and #3 using ‘DUD’. DL and UL scheduling for the UE configured with an SBFD UL subband is shown in slots #1 and #2 for DL symbol option 1 (D-1). Slot #3 shows DL scheduling when DL symbol Option 2 (D-2) is additionally allowed.

Figure 6: SBFD operation on a symbol configured as DL in TDD-UL-DL-ConfigCommon

In our view, the gNB scheduler should still be able to schedule the SBFD UL subband for DL transmissions to UEs as needed during SBFD operation. Based on our system-level evaluation results, substantial DL throughput and spectral efficiency losses are observed if semi-static, i.e., hard, UL-DL resource partitioning must be used on the NR carrier in the SBFD deployment. We expect that SBFD system level gains, or rather the avoidance of DL throughput and SE penalties, are substantial for low and even medium RU loads if the configured SBFD UL subband on the legacy DL symbols can be used for DL-only scheduling of the symbol by the gNB.

Allowing for the possibility of DL scheduling by the gNB in the configured SBFD UL subband must be considered from the UE perspective and from the gNB perspective, respectively. The observed higher cell aggregate throughput and increased spectral efficiency when allowing for DL scheduling by the gNB in the configured SBFD UL subband are possible for as long as the gNB can schedule some UE.

The ability of the gNB scheduler to exploit the flexibility of DL scheduling using the configured SBFD UL subband according to D-2 will depend on inter-cell intra-subband DL-UL CLI levels. Gains depend on the network load, offered DL and UL traffic, UE distribution and inter-site/inter-sector isolation, and other factors. For many low-load and small RU usage scenarios, it can be expected that CLI levels during SFBD operation are manageable.

When allowing DL scheduling of the configured SBFD UL subband according to DL-symbol Option 2 (D-2), a distinction should be made if, from the gNB perspective, DL scheduling in the configured SBFD UL subband UE should remain possible for the same SBFD-aware UE (Option D-2a in Figure 6) or only for other UEs (Option D-2b in Figure 6).

If D-2a is supported, UE-side Tx/Rx BB filtering depending on the frequency-domain locations of the configured SBFD DL or UL subband(s) inside the UE DL or UL BWP may be used. Sufficient filter re-configuration and UE switching time must be accounted for. DCI-based scheduling of UL transmission in the SBFD UL subband benefits from the larger allowed PUSCH processing delays. When same-slot scheduling is used to schedule PDSCH in the SBFD UL subband for the SBFD-aware UE on the SBFD symbol, filter re-configuration can be necessary and proper switching time must be accounted for. For the UE, support of D-2a is meaningful only if expected served DL and UL traffic from/to the UE allows to exploit the flexibility of DL scheduling using the configured SBFD UL subband. This is not always the case. For example, when SBFD is deployed for purpose of improved UL coverage in the NR TDD cell, it can be expected that the UE transmitting at maximum configured output power will then use most of the SBFD slots for UL transmissions most of the time to increase its UL:DL slot utilization ratio and the resulting operating UL SINR.

If D-2b is supported, the possibility of DL scheduling in the configured SBFD UL subband is restricted to other UEs, e.g., UEs not configured with the SBFD UL subband on the SBFD symbol. This is a reasonable assumption for many expected SBFD deployment scenarios and use cases. For example, SBFD-aware UEs in medium or good SINR conditions benefiting from the reduced latency when scheduling the DL and/or UL in the NR TDD cell with SBFD support may not need to be configured with an SBFD UL subband on all available SBFD slots/symbols in the NR TDD cell. We recommend to further evaluate these aspects in the Rel-18 SID.

If DL-symbol Option 2 (D-2) is supported, it might then be reasoned that the configured SBFD UL subband is in fact an SBFD “semi-flexible” subband, i.e., the symbol can be scheduled by the gNB as SBFD symbol or as a DL-only symbol. We think that such functionality does not require the formal introduction of a separate ‘flexible’ SBFD subband type in addition to the existing SBFD UL and UL subband type(s).

- Proposal 4: On a symbol configured as DL in TDD-UL-DL-ConfigCommon, Option 2 is supported at least from the gNB perspective.

### 4.2.2	SBFD operation on F symbols

For SBFD operation on symbols configured as F in TDD-UL-DL-ConfigCommon, the latest RAN1#112 agreement defines 3 options.

F-symbol Option 1 (F-1), Option 2 (F-2) and Option 3 (F-3) are shown in Figure 7 when SBFD operation is configured for the SBFD-aware UE in slots #1, #2 and #3 using ‘DUD’. DL and UL scheduling for the UE configured with an SBFD UL subband is shown in slots #1 and #2 for F-symbol option 1 (F-1). Slot #3 shows the possible additional DL and UL scheduling options when F-symbol Options 2 (F-2) and 3 (F-3) are additionally allowed. D-1/F-1 and D-2/F-2 are equivalent in the sense that the configured SBFD UL subband in the legacy DL or the legacy F symbol can now also be used for DL-only scheduling. F-3 allows to convert and use the legacy F symbol into a full UL-only symbol. This possibility does not exist for the legacy UEs on the legacy DL symbols because SBFD transparent mode cannot be configured for legacy UEs on the legacy DL symbols.

The use of either D-1/F-1 or D-2/F-2 or both results in the same behavior for SBFD-aware UEs from the implementation perspective from the specification perspective when legacy DL and F symbols are configured with an SBFD UL subband on the NR TDD cell. The additional use of F-3, not possible on legacy DL symbols, would result in a different UE implementation and distinct UE behavior from the specification perspective when SBFD operation on legacy DL and F symbols is supported.

Figure 7: SBFD operation on a symbol configured as F in TDD-UL-DL-ConfigCommon

One important decision point for the Rel-18 SID is to evaluate the potential benefits and UE modem impacts when allowing for DL scheduling by the gNB inside the configured SBFD UL subband. Similar to the case of SBFD operation on legacy DL symbols, the gNB scheduler should still be able to schedule the SBFD UL subband for DL transmissions to UEs as needed during SBFD operation on legacy F symbols. Based on our system-level evaluation results, substantial DL throughput and spectral efficiency losses are observed if semi-static, i.e., hard, UL-DL resource partitioning must be used on the NR carrier in the SBFD deployment.

Therefore, on a symbol configured as F in TDD-UL-DL-ConfigCommon, F-symbol Option 2 (F-2) should be supported at least from the gNB perspective.

The additional support of F-3, e.g., conversion of the legacy F symbol into an UL-only symbol for the SBFD-aware UE would result in different UE behavior for the legacy DL and F symbols when SBFD operation is introduced.

The ability of the gNB-side SBFD implementation to schedule a legacy F symbol as DL-only (F-2) or UL-only (F-3) depends on the gNB-side antenna configuration and SIC implementation. However, when the gNB supports scheduling of the legacy F symbol as DL-only (F-2) or UL-only (F-3) symbol, the gNB antenna configuration is switched accordingly, and SBFD operation doesn’t occur from the gNB perspective.

If D-2/F-2 is supported, the SBFD-aware UEs determine the DL reception parameters for the legacy D or legacy F symbol  DL-only symbol using the DL BWP. If F-3 is supported, the SBFD-aware UEs must also be able to determine the UL transmission parameters for the legacy F  UL-only symbol using the active UL BWP. However, SBFD-aware UEs use the active DL BWP in legacy DL symbols to set the DL reception bandwidth and can use the SBFD UL subband configuration to set the UL transmission bandwidth in SBFD symbols configured as legacy DL symbol. This results in a number of UE design complications for SBFD-aware UEs if additional flexibility needs to be supported to convert the F symbols to UL-only if allowed by F-3. For example, FDRA/FH for PUSCH repetition then needs to determine dynamically if a legacy F symbol where a PUSCH transmission of a PUSCH repetition occurs has been scheduled as UL-only (F-3) or has been scheduled as SBFD symbol (F-1). While UL transmissions from SBFD-aware UEs can be expected to occur across non-SBFD and SBFD slots, a semi-statically configured UL symbol or slot allows to determine the applicable UL transmission configurations ahead of time.

From the point of view of the gNB and SBFD-aware UEs, SFBD operation when the SBFD UL subband is configured on a legacy D or legacy F symbol should remain the same. In particular, the gNB configured frequency-domain occupancy of the SBFD UL subband inside the gNB channel BW can be expected to be the same for a symbol configured as DL or F by TDD-UL-DL-ConfigCommon. The SBFD UL subband is configured by the gNB according to fixed deployment needs, e.g., depending on the operator band segment. The frequency-domain location and size of the SBFD UL subband therefore mostly depends on the spectrum allocation and the capabilities of the gNB-side SBFD implementation. For many WA or MR base station class implementations, it can be expected that gNB-side SIC capabilities will often restrict the possibility of scheduling UL transmissions outside the configured SBFD UL subband due to RF and BB constraints and the possibility to use dynamic TDD operation according to F-3 is not meaningful when considering inter-gNB CLI. Therefore, scheduling UL transmissions for SFBD-aware or the legacy UEs on SBFD symbols outside the configured UL subband as by F-3 is precluded for any SBFD time-domain resource in such scenarios. In the case of some LA base station class implementations or for FR2-1 operation, fewer constraints may exist. For the LA BS classes and less hardware constraints arising from SIC implementation, the SBFD UL subband can be configured sufficiently large by RRC, e.g., up to the size of the NR channel BW. The SBFD-aware UE can then be either DL or UL scheduled up to the full NR channel BW in the SBFD symbol if desired. Still, even when it is possible to configure almost the entire NR carrier BW as SBFD UL subband, the same gNB implementation capabilities still apply to legacy DL and legacy F symbols, so there is no need to distinguish these symbol types with respect to SFBD operation.

It must also be considered that the potential specification impact from introduction of SBFD operation is significantly increased if DL and UL resource allocations in time- and frequency-domain must account for possibly different scheduling behaviors across symbols/slots configured as legacy DL or legacy F by TDD-UL-DL-ConfigCommon when the SBFD UL subband is configured.

- Proposal 5: On a symbol configured as F in TDD-UL-DL-ConfigCommon, Option 2 is supported at least from the gNB perspective.

- Proposal 6: On a symbol configured as F in TDD-UL-DL-ConfigCommon, Option 3 is not supported.

## 4.3	Signaling design aspects

### 4.3.1 SBFD and special DL signals/channels

One open design question for SBFD operation is if SBFD subband configuration for SBFD-aware UEs is meaningful or should be supported on any legacy DL or if certain DL signals such as SSBs need to be precluded. In time-domain, it may not always be possible that all symbols or slots are configured for SBFD operation by the gNB. Particular consideration must be given to DL symbols carrying the SSB(s) or configured for CORESET#0 or scheduled for PDSCH carrying SIB1 and the system information messages. In addition, TRS may be present on some DL symbols in the NR DL carrier BW.

In RAN1#110bis-e an agreement was made to study if SBFD operation in SSB symbols is supported or not. In RAN1#111, a FL proposal was made to at least consider the impact on SSB detection/measurement due to UE-to-UE CLI and potential limitations arising out of UE-side constraints for simultaneous UL transmission and DL reception of the SSBs.

The SSB(s) determine the DL cell coverage using the common DL beams in the deployment grid. Legacy UEs are allowed to make EPRE assumptions for the DL power relationships when measuring the SSS and DMRS on PBCH symbols during Cell (Re-)Selection. Legacy UEs also assume that the DL EPRE is constant over SSS carried in different SSBs for the purpose of SS-RSRP, SS-RSRQ and SS-SINR measurements.

When introducing SBFD support in commercial NR networks, any change to the DL link budget dimensioning determining the DL coverage of the cell assumed by the operator for the deployment grid should be avoided. Similar to the approach taken for the Rel-18 Network energy saving SID, it is desirable that the new functionality introduced by a later NR release does not affect DL coverage.

Allowing for the maximum possible number of symbols to be configured with an SBFD UL subband maximizes the theoretically achievable UL coverage gain when SBFD is introduced. For the case of FR1, SCS = 30 kHz, ‘DDDDU’, 8 SSBs, and using PUSCH repetition Type A with 14 symbols per UL slot, around 1 dB UL coverage gain from SBFD would be lost if the SSB carrying symbols cannot be configured with an SFBD UL subband when compared to the case that all symbols in all slots can be used.

The impact of a configured SBFD UL subband on the SSB symbols depends on the gNB-side SBFD antenna configuration. Using antenna configuration Option 1 where the number of antenna elements and number of TRXs in the SBFD DL subband(s) is reduced when compared to TDD, the SSB power allocation per RE is then also reduced, e.g., by 3 dB when only half of the DL TRXs are available for DL transmissions in the DL subband containing the SSB(s). A reduced DL EPRE can in principle be compensated for by the gNB, but practical limits exist with respect to the legacy UE EPRE assumptions when implementing the SS-RSRP or SS-RSRQ measurements. For example, using SBFD antenna configuration Option 2 where the same total number of antenna elements and the same number of DL TRXs as in TDD are available for the SBFD DL subband(s), there is no immediate impact to the available DL EPRE.

Because of these dependencies on the gNB-side SBFD antenna configuration, it should therefore not be expected that the SSB-carrying DL symbols can always be configured with the SBFD UL subband due to UE-side backwards compatibility considerations.

When the gNB-side SBFD antenna configuration allows to configure the SBFD UL subband on the SSB carrying DL symbols, the SSB symbols in an SBFD DL subband may be subjected to inter-UE CLI. To some extent, inter-UE CLI on the SSB symbols configured with the SBFD UL subband can be controlled by the gNB scheduler through the scheduling assignments. In the case of inter-UE CLI from PUSCH or PUCCH transmissions in the SBFD UL subband and PDSCH or PDCCH receptions in the SBFD DL subband on an SBFD symbol, the gNB scheduler can choose the UE pairings sufficiently separated and isolated, e.g., by pairing UEs which are located in different clusters. However, SSBs must also be received by UEs in RRC_IDLE/INACTIVE for Cell (Re-)Selection and the SSB coverage is provisioned for cell/sector coverage. The gNB scheduler can only freely select UEs for UL transmissions in the SBFD UL subband of an SSB carrying symbol because the locations of UEs in RRC_IDLE/INACTIVE in the cell receiving the SSB in an SBFD DL subband are not precisely known to the gNB. When the SBFD UL subband can be configured by the gNB on the SSB carrying symbols, the existing Rel-15 NR procedures including need or not for measurement gaps and switching time for SSB based measurements apply. For example, the UE performs intra-frequency SSB-based measurements with or without measurement gaps subject to the conditions and procedures in 38.133. Any potential constraints for a half-duplex UE with respect to handling of simultaneous UL transmission and DL reception of SSBs are handled according to Rel-15 measurement procedures.

More flexibility to configure SBFD operation exists for DL symbols which are configured with the CORESET#0 or which are used to schedule the PDSCH carrying SIB1 and the system information messages. The bandwidth of the CORESET#0, i.e., the Initial DL BWP, can be set to 24, 48 or 96 RBs. More than 1 symbol can be configured and indicated by MIB for the CORESET#0 allocation. Link adaptation settings for the PDSCH carrying the system information can be adjusted by the gNB using FDRA, TDRA and MCS settings in order to account for a potentially reduced number of available DL TRXs and different antenna areas in the SBFD DL subband when needed. The possibility of configuring the CORESET#0 exclusively inside an SBFD DL subband however depends on the size available NR band segment. For example, a number of operator band segments in n78 only allows for 40 or 60 MHz NR channel BW.

For TRS, the possible symbol locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots) are restricted for FR1 and FR2-1. Not being able to configure the SBFD UL subband on the symbols configured by trs-Info also means that the 3-symbol gap between a pair of CSI-RS resources and for most symbol pair configuration options, and the gap between the two pairs of CSI-RS resources cannot be scheduled using SBFD. For example, using FR1 n78, SCS = 30 kHz, Xp = 40 msec, at least 18 symbols then become not-schedulable for SBFD transmissions using the SBFD UL subband, which corresponds to a loss of 6% of available time-domain resources. Note that the bandwidth of a CSI-RS resource configured as TRS is at least 52 RBs. The DL EPRE of the four (NZP) CSI-RS resources designated as TRS can be configured by existing RRC signaling. For the TRS, more flexibility exists than in the case of the SSBs to adjust the measurement configuration to the gNB-side SBFD antenna configuration.

In summary, we consider that SBFD DL and UL subband configuration(s) should be supported on all DL symbols including SSB(s), CORESET#0, PDSCH carrying SIB1 or system information messages and the TRS. RRC configuration of the SBFD UL subband time-domain allocation can then set the allowed symbols for SBFD operation and protect the DL symbols as necessary as a function of the gNB-side SBFD antenna configuration.

- Proposal 7: The SFBD-aware UE can be configured with an SBFD UL subband on DL symbols indicated for SSB(s), CORESET#0 and TRS.

### 4.3.2 Cell common and UE specific SBFD configuration

In our view, most of the detailed signaling aspects related to Stage 3 design for SBFD UL and DL subband(s) configurations can be left to the later WID. One important decision point for the Rel-18 SID is the need to agree on signaling design supporting SBFD for UEs in RRC_CONNECTED mode only, or also for UEs in RRC_IDLE/INACTIVE. A related question is if SBFD DL and UL subband configuration can be indicated to the UE with symbol or slot level resolution.

If SBFD operation in the WID is only introduced for SBFD-aware UEs in RRC_CONNECTED mode, SIB1 reception by SBFD-aware UEs on the NR serving cell should not become prerequisite for introduction of SBFD.

For legacy UEs in RRC_CONNECTED mode, the network provides system information for the serving cell through dedicated signaling using the RRCReconfiguration message, i.e., using TDD-UL-DL-ConfigCommon in ServingCellConfigCommon. There is no immediate need to require an SBFD UL and DL subband(s) configurations using TDD-UL-DL-ConfigCommon (or a later release extension of it). It is clear that for backwards-compatibility reasons, any RRC provided SBFD subband configuration independent of the Stage 3 signaling design to be decided upon must remain compatible with the pattern1/pattern2 configured for the legacy UEs.

It is necessary to provide the SBFD UL and DL subband(s) configuration for the SBFD-aware UE independently from the legacy tdd-UL-DL-ConfigCommon, independent from the cell common signaling using SIB1 or ServingCellConfigCommon. For example, only then is it possible to configure UL transmissions for SBFD-aware UEs in legacy DL symbols/slots in the SBFD UL subband. It then also becomes possible to configure and schedule UL transmissions for SBFD-aware UEs only in SBFD slots, but not in the normal UL slot. For SBFD-aware UEs, it then also becomes possible to define new UE behaviors not currently supported in Rel-18 such as CSI-RS resource configuration, CSI reporting behavior, SRS transmissions or CLI reporting for purpose of inter-subband CLI measurements in the NR TDD cell with SBFD support.

It should also be considered that the SBFD UL subband configuration in frequency-domain on an SBFD symbol may often be expected to be the same for UEs because of its dependency on the gNB-side SBFD antenna configuration and SIC implementation.

However, it should not be expected that the SBFD UL subband configuration in time-domain will need to the same for all SBFD-aware UEs in a cell. For example, when SBFD is deployed for purpose of improved UL coverage in the NR TDD cell, it can be expected that the UE will be configured with an SBFD UL subband in most of the available SBFD slots for UL transmissions to increase the UL:DL slot utilization ratio and the resulting operating UL SINR. An SBFD-aware UE in medium or good SINR conditions benefiting from the reduced latency when scheduling DL and/or UL in the NR TDD cell with SBFD support may not need to be configured with an SBFD UL subband on all available SBFD slots/symbols. From the DL-perspective, most latency reduction gains can already be achieved when configuring the UE with only one SBFD slot.

In consequence, it is not meaningful to impose the same cell-common time-domain configuration for the SBFD DL and UL subbands for all SBFD-aware UEs in the NR TDD cell supporting SBFD. The gNB should retain the ability to configure DL receptions by UEs in the SBFD DL subbands and configure UL transmissions from UEs in the SBFD UL subband per-UE, i.e., on some but not necessarily all available SBFD time-domain resources. We note that such a UE-specific approach to time-domain SBFD configuration can also benefit the UE power consumption when in RRC_CONNECTED mode because not all SBFD slots/symbols must then be processed by the UE during DL and UL scheduling on SBFD slots.

Support for cell-common SIB1-based indication of the SBFD subbands and time-domain resources becomes only necessary when SBFD operation is also supported during Initial Access, i.e., for RACH msg1/msg-A, when the SBFD-aware UE is in RRC_IDLE/INACTIVE mode. The needs of SBFD DL and UL subband configuration(s) and the corresponding Stage-3 signaling solutions should therefore be considered separately for SBFD operation of the SBFD-aware UEs in RRC_CONNECTED mode and for the SBFD-aware UEs in RRC_IDLE/INACTIVE modes.

For SBFD DL and UL subband configuration in frequency-domain, several approaches are possible. Further consideration and decisions can be left to a later WID. The frequency location of an SBFD DL subband or UL subband has a start RB and an allocation bandwidth. The SBFD subband configuration is signaled to the UE with respect to the CRB grid. For example, the SBFD-aware UE only considers RBs of an SBFD DL or UL subband within its active DL or UL BWP as by Rel-15 NR. Such a principle, i.e., a frequency-domain occupancy of a UE-specific DL signal configuration is configured by RRC with respect to the CRB grid, but only RBs within the active UE DL BWP are then considered for DL receptions by the UE is applied already for CSI-RS resource configuration in Rel-15 NR. The SBFD DL and UL subband configuration can follow a similar principle.

- Proposal 8: SIB1 based cell-common indication of SBFD subband configuration(s) for SBFD-aware UEs is only supported if SBFD operation during random access for UEs in RRC_IDLE/INACTIVE modes is supported.

- Proposal 9: UE-specific SBFD subband and SBFD time-domain configuration for SBFD-aware UEs is the baseline for SBFD operation in RRC_CONNECTED mode.

### 4.3.3 SBFD symbol or slot level resolution

For SBFD subband configuration in time-domain, the following agreement was made in RAN1#112.

RAN1#112 agreement

Agreement

Study whether or not a slot can consist of both SBFD and non-SBFD symbols including

  - Benefits

  - Use cases

  - Scheduling flexibility

  - Implementation complexity

  - Compatibility with legacy TDD DL/UL configuration

The example of NR band n78, NR carrier BW of 100 or 60 MHz, SCS = 30kHz and DXXXU with gNB-side SBFD operation in slots #1, #2, and #3 is considered in Figure 8. For simplicity, it is assumed that slot #0 is not configured for SBFD operation due to the presence of the SSBs. Note that when ssb-periodicityServingCell = ms20 is configured, the slots #0 and #1 would then not be configured for SBFD operation in every 8th frame.

Even if the SBFD UL subband is configured in each of the slots #1, #2, and #3, a different number of SBFD symbols is available in each SBFD slot. For example, a number of F symbols, e.g., 4 symbols, must be configured in slot #3 for purpose of providing a guard period to legacy UEs. Only 8 SBFD symbols are available. For NR DL carrier BW of 100 MHz, slots #1 and #2 allow for 11 and 12 SBFD symbols, respectively. A different SBFD time-domain configuration must applied to the case of NR DL carrier BW = 60 MHz. Slots #1 and #2 are each configured with 14 SBFD symbols. A different numbers of available SBFD symbols in the SBFD slots should be expected.

When SBFD configuration in time-domain can use only slot-level resolution, the introduction of SBFD operation for the SBFD-aware UEs would result in an undue NR specification effort. For example, a large number of collision handling rules would then be needed for specification of the expected UE behavior in the case that CORESET#1 for PDCCH reception is configured either inside an SBFD DL subband or across the SBFD DL and UL subbands and these rules would then also need to define UE behavior for a number of resulting subcases such as when the SBFD UL subband in the SBFD slot is scheduled for UL transmission or not, using PUSCH repetition or not, etc.

In our view, it is therefore necessary that SBFD configuration in time-domain is supported with symbol-level resolution at least for the case when the SBFD-aware UE operates in RRC_CONNECTED mode. For UE-specific RRC configurations in RRC_CONNECTED mode, fewer payload constraints exist with respect to the signaling payload than in RRC_IDLE/INACTIVE modes. Symbol-level resolution is needed to configure different numbers of symbols available for SBFD DL or UL subband configuration in time-domain in the different SBFD slots.

However, frequent symbol-level switching in a slot of the gNB-side SBFD antenna configuration and SIC implementation, i.e., from/to {DL-only, UL-only, SBFD} should not be expected. If SBFD operation is possible in a slot, it should be expected that often a number of consecutive SBFD symbols is then configured. Another advantage of symbol-level resolution is that DL special signals/channels such as SSBs or CORESET#0 can be set as protected DL symbols when necessary and required without hard-coding the SBFD-aware UE behavior that SSBs can never be configured with the SBFD UL subband.

In summary, it may be expected that not all symbols in an SBFD slot are configured with the SBFD UL subband and that the number of available SBFD symbols in an SBFD slot can often be expected to be consecutive. Most of the time, PUSCH allocations using the SBFD UL subband can be scheduled using a sufficient number of consecutive SBFD symbols. We note that SBFD operation using few available SBFD symbols, e.g., 2, in a slot is still meaningful if PUCCH is considered. PDSCH/PUSCH allocations using less than a minimum of 4 symbols allocated by TDRA are not supported by Rel-15 NR and should not be introduced for purpose of SBFD operation. Short PUCCH transmissions using 1 or 2 symbols can be used even if the SBFD UL subband in an SBFD slot is configured using only 1 symbol or 2 consecutive SBFD symbols.

Figure 8: SBFD time-domain configuration: symbol level resolution

Symbol-level resolution by RRC for the SBFD time-domain configuration does not imply that SBFD operation across SBFD and non-SBFD symbol must be supported by the UE implementation.

The possibility to allocate and map a PDSCH, PUSCH or PUCCH across both non-SBFD and SBFD symbols in an SBFD slot depends on the gNB-side SBFD antenna configuration. Using SBFD antenna configuration Option 1 where the number of antenna elements and number of TRXs in the SBFD DL subband(s) is reduced when compared to TDD, the possibility to adjust the DL EPRE across the non-SBFD and SBFD symbols of the same PDSCH, PUSCH or PUCCH allocation may not always be possible. The UE implementation impact and specification delta with respect to QCL, beam settings, UL power control and UL timing advance procedures must be considered. Using SBFD antenna configuration Option 2 where the same total number of antenna elements and the same number of DL TRXs as in TDD are available for the SBFD DL subband(s), fewer such issue exists are expected. Because of these dependencies on the gNB-side SBFD antenna configuration and to avoid undue UE complexity, the UE should therefore not be expected to support PDSCH, PUSCH or PUCCH allocations across non-SBFD and SBFD symbols in an SBFD slot in the general case. Such allocations should only be considered for the case and under the condition that the gNB-side SBFD antenna configuration allows to the UE to apply the same QCL and EPRE assumptions when processing the PDSCH, PUSCH, or PUCCH symbol allocation across non-SBFD and SBFD symbols. Additional constraints exist for PUSCH and PUCCH when considering UL transmit power control or UE transmit timing across slot or symbol groups.

It should also be considered that symbol-level resolution of the SBFD configuration is inherently forward-compatible and flexible. Such a principle is already used in the Rel-15 NR core specifications. Rel-15 NR allows to assign and signal the DL or UL transmission direction with symbol-level resolution. Limitations such as the number of supported UL-DL switching points per slot, or the number of allowed UL signal/channel combinations per slot are then imposed by the UE capabilities. The same approach can be used when introducing support for SBFD operation in the WID.

- Proposal 10: For SBFD subband configuration in time-domain, a slot can consist of both SBFD and non-SBFD symbols.

- Proposal 11: For SBFD-aware UEs, SBFD subband configuration in time-domain with symbol level resolution is the baseline at least in RRC_CONNECTED mode.

### 4.3.4 SBFD and group-common signalling

One open design question for SBFD operation is if existing group-common signaling, e.g., SFI and DCI format 2_0 or an extended SFI functionality, can provide benefits for the UE implementation or from the gNB scheduling perspective.

In our view, it is first necessary to decide on the SBFD configuration and the supported UE scheduling options for SBFD-aware UEs on legacy D and/or F symbols before meaningful scenarios for support of SFI or extended SFI functionality for SBFD operation can be identified and concluded upon. For example, DL-symbol Option 1 (D-1) and F-symbol Option 1 (F-1) can be considered baseline and result in semi-static configuration and use of the SBFD DL and UL subbands from the UE perspective. If the SBFD configuration in time-domain is also semi-static, it may not be needed to be consider group-common signaling. When DL-symbol Option 2 (D-2) and F-symbol Option 2 (F-2) are considered, more scheduling flexibility exists. For example, from the UE perspective, consideration must be given to the UE processing timeline when scheduling PDSCH in the SBFD UL subband on the legacy D and legacy F symbols. The use of group-common signaling is one possible approach. We note that group-common, e.g., extended SFI signaling, if needed and found beneficial, may be considered primarily from the time-domain perspective, e.g., indicating symbol allocations of type {D, F, U, or SBFD}.

- Proposal 12: Study the potential benefits of group-common signaling for SBFD operation.

# UE enhancements for improved SBFD performance

SBFD operation in the NR TDD network can benefit from gNB coordinated scheduling in time/frequency domain and from gNB-to-gNB spatial domain coordination. In addition, power-domain enhancements such as gNB DL transmit power coordination or UE UL Tx power restrictions can be used to mitigate CLI. Some of these features require new Xn/F1AP signaling support. Network-side coordinated transmission of CSI-RS resource(s) set(s) for purpose of CLI signal power measurements and interference estimation is of particular interest when considering advanced gNB scheduler implementations. We also consider enhancements to the Rel-16 CLI reporting feature such as L1 aperiodic UE reporting and CLI reporting with associated spatial domain information as beneficial to improve the UE-side support for d/f-TDD and SBFD operation in the network.

We do not consider the introduction of non-backwards compatible RE-level blanking solutions for the UE as meaningful for SBFD operation. Such solutions would necessarily affect L1 processing in the UE modem and result in undue complexity to the modem implementation for SBFD-aware UEs. In addition, for the network to benefit from availability of RE-level blanking solutions to support gNB side advanced receivers, a meaningfully high penetration rate of SBFD aware UEs in the commercial network is required.

The performance of SBFD-aware UEs and the TDD network with SBFD support can benefit from a number of low-impact and low-complexity enhancements. In this section, we first provide our views on resource allocation enhancements for PDSCH, PUSCH and PUCCH. We then consider potential enhancements to CSI-RS resource configuration and CSI reporting. We further discuss the potential usefulness to enable SBFD operation on RACH resources, SBFD impacts on the UE transmission and reception timing, and UE transmit power control.

## 5.1	Resource allocation for PDSCH, PUSCH and PUCCH

### 5.1.1	Time-domain

In RAN1#112, the following agreement was made.

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols)

  - Study the following options for SBFD-aware UEs:

    - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

    - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

  - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

    - PDSCH/PUSCH/PUCCH repetitions

    - SPS PDSCH/CG PUSCH

    - TBoMS

    - Multi-PUSCH/PDSCH scheduled by a single DCI

    - Periodic/semi-persistent SRS/CSI-RS/PUCCH

    - PDCCH

For UL and DL scheduling in the NR TDD cell with SBFD support, both single slot scheduling for PUSCH and PDSCH and multi-slot scheduling for PUSCH should be considered. One important use case for SBFD is improved UL coverage for the existing NR TDD deployment grid. SBFD enables to significantly increase the UL:DL slot utilization ratio by making more time-domain resources available to UEs in bad coverage conditions. These UEs will usually have reached the maximum configured UE output power. Using DDDDU, the UE can only transmit its UL in 20% of the time-domain resources at maximum configured UE output power. Using SBFD, the UE can potentially transmit in up to 80% of the time-domain resources.

Rel-15 PUSCH aggregation and Rel-16/Rel-17 PUSCH repetition Type A/B features are therefore important for SBFD when scheduling the UE across multiple slots. It can be expected that in the NR TDD cell with SBFD support, the UEs which require improved UL coverage are configured with Rel-15 PUSCH aggregation or with Rel-16/Rel-17 PUSCH repetition Type A. PUSCH repetition Type B is of relevance from the perspective of URLLC services.

PUSCH transmission using single-slot PUSCH scheduling is shown in Figure 9. CORESET#1 is configured in the SBFD DL subband #1 of the SBFD slot for the SBFD-aware UEs. Search Space Set#1 is configured to occur once per slot. For AL8 (48RBs) @30kHz and using a 1 symbol CORESET, some 17-18 MHz are required. CORESET#1 can be configured inside the DL subband. Benefits of the per-slot PUSCH scheduling approach include good DL and UL scheduling latency. For smaller NR channel BW such as n78 using 40 MHz or 60 MHz, CORESET#1 can still be configured across one of the SBFD DL subband and the SBFD UL subband. However, not all symbols may then be available for UL transmissions using the SBFD UL subband in an SBFD slot. For example, SBFD UL coverage gains are reduced by around 1 dB when not all 14 symbols in the slot can be scheduled for the PUSCH.

When PUSCH repetition is configured for the UE, CORESET#1 must then be configured in the SBFD DL subband of the SBFD slot such that all SBFD per slot can be used. The UL link budget is improved because up to 14 symbols can be used for PUSCH transmission in an SBFD slot. Potentially, TDD can now also harvest the additional Rel-17 JCE gains because 4-slot DMRS bundling across contiguous slots without DL/UL switching is enabled by SBFD.

Figure 9: SBFD operation with single slot PUSCH scheduling and PUSCH repetition

Different antenna and panel design options are possible for the gNB side SBFD implementation. For example, the SBFD antenna panel to provide spatial separation of the Tx and Rx paths and for the gNB side SIC can be based on a fully split design where the TRX ports are either dedicated to DL transmission or dedicated to UL reception. Another possibility is shared panel design where a certain number of TRX ports can be used for both DL transmission and UL reception in non-SBFD symbols, but a number of TRX ports is dedicated to either DL transmission or dedicated to UL reception by the gNB in SBFD symbols. Therefore, it cannot be expected that the number of TRXs and the available Tx or Rx aperture area used for Tx and/or Rx in a DL or UL slot is necessarily the same when compared to the DL transmission or UL reception using the SBFD subbands in SBFD slots. The link gain and resulting link budget when transmitting an UL signal or channel in the SBFD UL subband of a slot is then different when compared to transmitting the same UL signal or channel in the UL slot. Similar considerations apply to transmission of DL signals/channels using the DL subbands of SBFD slots.

PUSCH transmissions of a PUSCH repetition when transmitted in the SBFD UL subband of SBFD slots and when transmitted in the UL slot can therefore be subjected to mis-matched link adaptation settings. One feature for enhanced SBFD support in the TDD network is to restrict the PUSCH transmissions of a PUSCH repetition with respect to the time-domain resources. It should be possible to transmit the PUSCH repetitions only in the SBFD UL subband of SBFD slots, but not in the UL slot. Note that when PUSCH repetitions are restricted to the SBFD UL subband, the resulting UL slot utilization ratio is still higher than currently possible with DDDSU.

Another important design consideration for SBFD operation is that the separation of SBFD symbols/slots and the UL slot(s) with respect to the resource allocation in time-domain when introducing gNB-side SBFD operation also allows to increase the achievable UL spectral efficiency and UL cell throughput in the normal UL slot(s). UL transmissions from low-SINR UEs can be offloaded to the SBFD slots. UEs in good geometry can then be scheduled with high MCS settings using large contiguous RB allocations in the normal UL slot(s). Given that most existing UE implementations only support UL contiguous resource allocation, e.g., PUSCH resource allocation type 1, offloading of the low-SINR UEs to the SBFD UL subband in the SBFD slots may be expected to result in system level gains due to reduced resource fragmentation in the UL slot.

Similar considerations apply to the case of PUCCH repetition despite a smaller number of repetitions for PUCCH and differences in available slot counting for PUCCH repetition and PUSCH repetition Type A/B.

Our views on SBFD configuration with symbol or slot level resolution and expected SBFD slot configurations are summarized in Section 4.3.3. In our view, SBFD configuration using symbol-level resolution in time-domain should be supported at least for SBFD-aware UEs in RRC_CONNECTED mode. It can be expected that different SBFD slots will often need to be configured with a different number of available SBFD symbols in each slot. PDSCH/PUSCH allocations using less than a minimum of 4 symbols allocated by TDRA are not supported by Rel-15 NR. Short PUCCH transmissions using 1 or 2 symbols can be used even if the SBFD UL subband in an SBFD slot is configured using only 1 symbol or 2 consecutive symbols. Most of the time, PUSCH or PUCCH allocations using the SBFD UL subband can be scheduled using a sufficient number of consecutive SBFD symbols in a slot. PDSCH, PUSCH or PUCCH allocations across non-SBFD and SBFD symbols in a slot should only be considered for the case and under the condition that the gNB-side SBFD antenna configuration allows to the UE to apply the same QCL and EPRE assumptions when processing the PDSCH, PUSCH, or PUCCH symbol allocation across non-SBFD and SBFD symbols. Additional constraints exist for PUSCH and PUCCH when considering UL transmit power control or UE transmit timing across slot or symbol groups.

- Proposal 13: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, support Option 1 and Option 2.

- Proposal 14: For UL transmissions and DL reception across non-SBFD and SBFD symbols in a slot, support Option 1 as baseline

- Proposal 15: For UL transmissions and DL reception across non-SBFD and SBFD symbols in a slot, support Option 2 under the condition that the same QCL and EPRE assumptions can be applied by the UE for the allocation on the non-SBFD and SBFD symbols.

### 5.1.2	Frequency-domain

In RAN1#112, the following agreements were made.

Agreement

For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary,

  - Option 1:

    - Part of the DL RBG inside the DL subband can be used

    - Part of the UL RBG inside the UL subband can be used

  - Option 2:

    - Part of the DL RBG inside the DL subband cannot be used

    - Part of the UL RBG inside the UL subband cannot be used

FFS: The part of the RBG outside.

Agreement

For SBFD-aware UEs, study at least the following issues for PDSCH:

  - PRG(s) with size of 2 and 4 that overlaps with subband boundary

  - Wideband precoder in case of non-contiguous DL subbands

When considering resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subband, we note that the expected specification effort can be considered low if existing Rel-15 UE procedures defined already for the case that the UE BWP overlaps with an RBG is re-used for the SBFD subband configuration case. RBs of the overlapping RBG are then schedulable by the gNB. It can be avoided that the SBFD subband configuration on the NR carrier must account for additional constraints when configuring the frequency-domain occupancy of the SBFD UL subband. SBFD-aware UEs can use the active DL BWP to set the DL reception bandwidth and can use the SBFD UL subband configuration to set the UL transmission bandwidth similar to the existing Rel-15 NR defined with respect to the UE BWP.

When considering the example of PDSCH resource allocation type 0 where a bitmap indicating which RBGs are allocated to the UE is used, the RBG size depends on the UE BWP. One of two possible sizes can be signaled by RRC using Rel-15 NR. For the case of n78 and 100 MHz carrier BW and a UE DL BWP of 100 MHz, the nominal RBG size is 16 RBs. DL scheduling to UEs in the 2 DL subbands of the SBFD slot then may suffer from some granularity issues. When the UL subband occupies center 51 RBs, 4*16 = 64 RBs of 273 RBs cannot be DL scheduled to the UE because an RBG would then partially overlap with the SBFD UL subband. The effective loss may be reduced when considering that part of these partial overlap RBGs may be used as guard band(s). It is possible to consider a smaller configurable RBG set size indicated by RRC for the SBFD-aware UE.

We see little potential that handling the case of PRG(s) of size 2 or 4 overlapping with an SBFD subband results in meaningful improvements for SBFD operation. Similarly, we see no issue with the wideband precoder in case of non-contiguous SBFD DL subbands.

- Proposal 16: For resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands, support Option 1.

- Proposal 17: For SBFD-ware UEs configured with PDSCH resource allocation type 0, support a new configurable RBG set size.

### 5.1.3 Frequency-hopping

When frequency-hopping is configured by the gNB for single slot PUSCH scheduling, for PUSCH repetition and PUCCH repetition, the same frequency-hopping configuration is applied to the non-SBFD and the SBFD slots if existing Rel-15 NR specification behavior is followed. FH in the SBFD slots must be configured by the gNB such that the hops remain confined inside the SBFD UL subband. The same configured hop size as part of the RRC provided PUSCH configuration must then also apply to the normal UL slot (Option 1 in Figure 10).

During SBFD operation in the NR TDD cell, it can be expected that the SBFD UL subband is placed in the center of the NR carrier. The available contiguous UL scheduling bandwidth in the UL slot is then fragmented which penalizes cell-aggregate and UL throughput in the UL slot where UEs in good and medium SINR are best scheduled using large contiguous UL scheduling bandwidth.

Figure 10: SBFD and PUSCH frequency-hopping

One solution with low design impact and specification effort is to introduce one or two separately configured RB offset values for PUSCH and PUCCH frequency hopping for the SBFD aware UEs such that the hops in the normal UL slot can be shifted outside the UL subband subject to the UE UL BWP configuration (Options 2a/2b in Figure 10).

- Proposal 18: For SBFD-aware UEs, support additionally configured RB offset value(s) for PUSCH and PUCCH frequency-hopping.

## 5.2	CSI-RS resource set configuration and CSI reporting

When SBFD operation is introduced in a TDD cell, the potential impact and limitations of CSI-RS resource set configuration and CSI reports must be considered.

An example for possible CSI-RS resource set configurations is shown in Figure 11. When SBFD operation is supported in the NR TDD cell, a CSI-RS resource set in the SBFD slot can be configured with a frequency-domain occupancy located entirely outside the SBFD UL subband or the CSI-RS resource set can contain the SBFD UL subband of the SBFD slot, e.g., is configured across the SBFD DL and UL subbands. Partial overlap scenarios, e.g., the CSI-RS resource set is configured with a frequency occupation comprising parts of the SBFD UL subband of the SBFD slot is also possible. In Rel-15 NR, the CSI-RS frequency occupancy must be configured in integer multiples of 4 RBs and has size of minimum DL BWP or 24 RBs. Either every or every other RB is occupied by CSI-RS REs inside the configured CSI-RS bandwidth. Special rules apply to the case of the CSI-RS resource set with density = 3 (TRS) where the minimum occupied BW must be 52 RBs. The CSI-RS bandwidth configuration applies independently of type, e.g., it applies to both CSI-RS designated by RRC as NZP or ZP. Note that CSI-IM occupy every RB inside the configured CSI-RS bandwidth. The CSI-RS bandwidth is configured independently for different (multiple) CSI-RS resource sets. The UE implementation must support a minimum number of multiple configured CSI-RS resource configurations per frame.

Figure 11: SBFD operation and configuration of CSI-RS resource sets

When the CSI-RS bandwidth of the RRC configured CSI-RS resource set is confined within the SBFD DL subband of the SBFD slot, no CSI reporting is possible for the UE for outside the CSI bandwidth. Configuration of multiple CSI-RS resource sets becomes necessary, e.g., 2, to cover both DL subbands of the SBFD slot. Another consideration is then that the gNB scheduler cannot obtain CSI reports from the UE for purpose of DL scheduling in the UL subband. There is no immediate impact on aperiodic, semi-persistent and periodic CSI-RS resources or the CSI reports. If the gNB scheduler requires CSI reporting inside the configured SBFD UL subband of the SBFD slot from the UE, aperiodic triggered CSI reporting and associated CSI-RS resource set must be used. Dependencies exist with respect to time-domain reference slot for the CSI reports.

When the CSI-RS bandwidth of the configured CSI-RS resource set contains the SBFD DL and UL subbands of the SBFD slot, either full or partially, CSI reporting can be impacted. Note that Rel-15 NR already allows to toggle on or off individual CSI reporting subbands through the CSI reporting configuration (38.214). The reportFreqConfiguration contained in the CSI-ReportConfig indicates the frequency granularity of the CSI report. A CSI Reporting Setting configuration defines a CSI reporting band as a subset of subbands of the bandwidth part, where the reportFreqConfiguration indicates the csi-ReportingBand as a contiguous or non-contiguous subset of subbands in the bandwidth part for which CSI shall be reported. When wideband CQI reporting is configured, a wideband CQI is reported for each codeword for the entire CSI reporting band. When subband CQI reporting is configured, one CQI for each codeword is reported for each subband in the CSI reporting band. In consequence, even when the CSI-RS resource set is configured across the DL and UL subbands of the SBFD slot, the CSI report configuration linked to the CSI-RS resource set can blank out the UL SB. Granularity issues exist similar to the case of PDSCH RA Type 0. The CSI reporting subband size can be configured from two possible values depending on the UE DL BWP size. For example, 16 or 32 PRBs subband size can be configured for a UE DL BWP of 273 RBs.

Another consideration when introducing SBFD operation in the TDD network is the need to apply different CSI-RS Tx EPRE settings by the gNB in the two SBFD DL subbands of the SBFD slot. One reason is that even when digital BF is used, a different number of TRX may be available to the gNB for DL beamforming operation in the upper SBFD DL subband of the SBFD slot and the lower SBFD DL subband of the SBFD slot. Tx power imbalances can exist across the SBFD DL subbands. Another reason is the direct Tx-Rx leakage interference cancellation by the gNB in presence of DL and UL scheduling. Equal RB occupancy in the first and second SBFD DL subbands of the SBFD slot is not always possible due to varying UE traffic loads. Similarly, an equal or balanced RB occupancy in the SBFD UL subband may not always be possible. The Tx EPRE of the DL signal(s) or channel(s) can be adjusted accordingly by the gNB accounting for DL and/or UL scheduling in the SBFD slot, but Rel-15 NR forces to set the same EPRE setting for both SBFD DL subbands in the SBFD slot for the UE configured with the CSI-RS resource set.

The following agreement was made in RAN1#112.

Agreement:

Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options:

  - Option 1: Two contiguous CSI-RS resources that are linked

  - Option 2: One CSI-RS resource

    - Option 2-1: Non-contiguous CSI-RS resource allocation

    - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

Agreement:

For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols):

  - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols

  - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

In our view, the possibility to configure multiple CSI-RS resource sets and multiple CSI reports and trigger conditions as by Rel-15 NR provides much flexibility to configure per-SBFD subband CSI-RS resource sets and associate these to the CSI reports configured to the UE. Option 1 for CSI-RS resource configuration and Option 1 for CSI reporting can be considered baseline and supported by Rel-15 NR specifications.

However, limitations exist with respect to the configuration of multiple CSI-RS resource sets with a frequency occupancy limited to either the first or the second SBFD DL subband and when configured across the SBFD DL and UL subbands. UE complexity can be increased, and CSI reporting restrictions can result due to the need for the UE to support more concurrently active CSI measurement and reporting configurations during SBFD operation. SBFD-aware UEs supporting Option 2-2 for CSI-RS resource configuration can benefit from enhancements. For CSI reporting, Option 2 can consider the use of separate reporting hypothesis, e.g., for SBFD symbols and non-SBFD symbols, respectively.

For CSI-RS resource set configuration and CSI reporting in the context of SBFD, it is important to consider that any change to the CSI-RS RE level processing at RB level, i.e., RE location on a symbol, mapping to one or more symbols in a slot, sequence generation across RBs (38.211) would result in significant specification impact and implementation complexity for gNB and UE. We therefore think that if enhancements to CSI reporting are motivated and to be pursued for SBFD, these should remain mostly limited to the CSI reporting (38.214). Skipping rules, i.e., the UE disregards RBs configured as SBFD UL subband during CSI reporting on an SBFD symbol may be used and can be considered low impact specification-wise.

- Proposal 19: For frequency-domain resource allocation of CSI-RS across SBFD DL subbands, support Option 1 and Option 2-2.

- Proposal 20: For CSI reporting associated with P/S-P CSI-RS, support Option 1 as baseline and Option 2 using separate reporting hypothesis for SBFD symbols and non-SBFD symbols.

## 5.3	Power and spatial domain enhancements

The following agreement was made in RAN1#112.

Agreement:

Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots:

  - Whether/how to have separate resources

  - Whether/how to have separate FH parameters

  - Whether/how to have separate UL power control parameters

  - Whether/how to have separate beam/spatial relation

Similar to the case of resource allocation enhancements in time- and frequency-domain for PDSCH, PUSCH and PUCCH, we consider separate UE configurations for UL power control and beam/spatial relations in the SBFD and non-SBFD slots are necessary for the SBFD enhancements.

For transmit power control in the SBFD deployment, solutions to adjust the DL gNB transmit power allocations and configure the UE with corresponding DL EPRE assumptions should be considered separately from solutions to control the UE UL transmit power in the SBFD and non-SBFD slots.

When considering the FR1 NR upper mid-band deployments, gNB-based CLI interference mitigation in power-domain can be used to better control gNB-to-gNB CLI levels across slots, e.g., time-domain ICIC, and for the frequency-domain resources in a slot. In the intra-operator case, inter-cell coordination using the Xn/F1AP is possible.

With respect UE UL transmit power control, an important consideration for SBFD is that only a single UE configured maximum output power value p-Max can be configured in Rel-15 NR for the serving cell. In addition, a single value for p-NR-FR1 and/or p-UE-FR1 can be configured for the total maximum configured output power of a cell group of which the serving cell is part or for all cells configured for the UE in FR1. A consequence is that the same UE configured maximum output power value must be used for UL transmit power control by the UE to determine the maximum transmission power value for PUSCH, PUCCH, SRS or PRACH irrespective of the slot type, e.g., SBFD slot or normal UL slot. For example, when p-Max is configured such that the UE is allowed 23 dBm nominal output power and adjusted by RF tolerances and A-MPR(s), the UE when scheduled will then transmit using up to the maximum configured value of 23 dBm in any time-domain resource of the serving cell. Such UE behavior is not necessarily desired in the NR TDD cell with SBFD support.

For SBFD operation, it should be possible to separately set the allowed UE configured maximum output power for the UL slot and the UL transmissions in the SBFD UL subband. The maximum allowed UL transmit output power of a UE determines its interference range with respect to the co-scheduled UEs in the SBFD DL subband(s) of the same cell. For SBFD operation in the serving cell, it is often useful to limit the interference range of the aggressor UE in average or good SINR conditions when transmitting in the SBFD UL subband of SBFD slots. The aggressor UE transmitting in the SBFD UL subband (mostly) interferes the victim UE receiving DL transmissions in the DL subband(s) of the same serving cell. The aggressor UE transmitting in the UL in the normal UL slot does (usually) not interfere with the DL transmissions to UE(s) in the same and in adjacent cells assuming the same TDD UL-DL frame configuration is configured for the TDD cells in the deployment and assuming that the guard period is configured sufficiently large. Note that similar considerations apply to d/f-TDD in F slots which can be assigned to either DL or UL by the gNB scheduler versus UL slots where the transmission direction cannot be changed. The configured maximum UE output power in a slot determines the UEs co-channel interference range.

- Proposal 21: For SBFD-aware UEs, support separately configured maximum UE output power in SBFD and non-SBFD slots on the serving cell.

Use of the Rel-15 NR open-loop and closed-loop parameter sets or the configurable DL pathloss reference signals can also benefit from separate UE configurations in SBFD and non-SBFD slots, respectively.

For example, the UE can be configured with multiple, e.g., up to 30, open-loop power-control parameter sets {P0(.), alpha(.)} in Rel-15 NR where each parameter set corresponds to a pair of values for a normalized target receive power level and a fractional pathloss compensation coefficient. The gNB can configure each value of the SRI field in a DCI format with one of the open-loop power control parameter set pairs. The gNB can select and indicate an open-loop parameter set with separate values for the normalized target receive power level and the fractional pathloss compensation coefficient, respectively, for each UL beam that can be used by the UE to transmit a PUSCH. Note that for a PUSCH transmission before the UE receives dedicated configuration for the open-loop power-control parameters, such as for a Msg3 PUSCH transmission or for a PUSCH transmission scheduled by a DCI format with CRC scrambled by TC-RNTI, fractional power control is not used, e.g., alpha(.) = 1, and P0(.) is then determined by the UE based on received information in the configuration of the random-access procedure. When DCI F0_1 or DCI F0_2 is configured to include an open-loop parameter set indication field as by Rel-16 NR, the associated index values are indicated by RRC signaling such as P0-PUSCH-Set-r16. However, the open-loop parameter set indication field can only indicate the normalized target receive power setting P0(.) but not the fractional pathloss coefficient alpha(.). However, the SRI cannot always be assumed available as UE feature in FR1.

For some SBFD slots/symbols, full pathloss compensation is preferable to achieve a large UL link budget improvement for the cell-edge UE, while partial pathloss compensation is preferable in other SBFD slots/symbols or for UEs benefiting from reduced latency with SBFD to control the system-level throughput and spectral efficiency while accepting a smaller UL link budget for the UE. When SRI is not available, separate open-loop power control parameter sets cannot be indicated to the SBFD-aware UE based on existing Rel-15/16 NR specification behavior. It is useful for SBFD deployments to introduce the ability to signal the fractional pathloss coefficient setting alpha(.) for SBFD-aware UEs for the SBFD and non-SBFD slots. For the two closed-loop components and the configurable, up to 4, DL pathloss reference signals, similar considerations apply.

- Proposal 22: For SBFD-aware UEs, support separate UL power control parameters and separate beam/spatial SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots.

## 5.4	Random Access signals/channels

Using the SBFD UL subband for PRACH transmissions (Figure 12) has the benefit that fragmentation of the contiguous UL scheduling BW in the UL slot can be avoided. UL throughput and spectral efficiency is increased in the TDD cell with SBFD. UEs in good SINR conditions can be scheduled by the gNB with high instantaneous UL data rates and benefit from the availability of a larger available contiguous UL scheduling BW in the UL slot. UEs in good SINR conditions or UEs requiring high UL data rates are best scheduled in the UL slot(s) where CLI is controlled network wide.

It must be considered that RACH occasions in the UL slot are rarely configured at the carrier edges in practical NR TDD deployments for various reasons. RACH in the UL slot segments part of the schedulable UL bandwidth. Most UEs support only the mandatory PUSCH resource allocation type 1, e.g., contiguous PUSCH allocations. When placing PRACH ROs in SBFD slots, PRACH creates CLI only when actually used by a UE selecting a random-access resource. A PRACH RO which is configured, but not used by any UE in the SBFD slot does not create CLI towards other UEs in the slot. Note that when PRACH ROs are configured in the normal UL slot, they can seldom be re-used for UL data scheduling and shared data/random access reception by the gNB, e.g., when configured, such resources are not available for UL data.

Figure 12: SBFD operation using the SBFD UL subband or normal UL slot(s)

Allowing for configuration of PRACH in the SBFD UL subband and use of the SBFD UL subband for RRC and NAS signaling exchanges during the 5G Initial Attach procedure also reduces UE initial acquisition time in NR TDD. It must be considered that about 8-10 DL and UL messages must be exchanged by network and UE during initial attach. These signaling exchanges are mostly “atomic” and “in sequence”, e.g., successful completion of a NAS procedure such as Authentication Request/Response is required before a subsequent NAS procedure such as NAS Identity Request can be initiated. NR TDD 30 kHz when compared to NR FDD 15 kHz in the NR low band is penalized by the average UL frame alignment delay incurred for each of the UL RRC and NAS messages due to limited number of UL slots. This typically results in 2x longer initial acquisition duration when comparing NR TDD to FDD.

While we see several potential benefits in allowing for the possibility to allow for PRACH transmission using the SBFD UL subband of SBFD slots, SBFD operation at least initially should attempt to evaluate feasibility for full-duplex operation in RRC_CONNECTED mode. RACH procedures are of relevance to RRC_CONNECTED mode in a more limited context, e.g., for network-controlled cell-level mobility handling procedures using contention-free random-access procedures where access information to the target cell is provided through RRC Handover Command message.

- Proposal 23: For SBFD-aware UEs, support contention-free random access using SFBD symbols in RRC_CONNECTED mode.

- Proposal 24: RAN1 to study potential benefits and specification impacts to support random access using SBFD slots/symbols for SBFD-aware UEs in RRC_IDLE/INACTIVE mode.

## 5.6	Transmission and reception timing

In our view, it is beneficial to further study and evaluate enhancements to UE transmission and reception timing for SBFD. The DL and UL symbol alignment at the gNB with SBFD is one key aspect affecting the gNB SIC and demodulation performance in the receiver.

The existing Rel-15 UE timing advance procedure controls the UE UL transmit timing with respect to the propagation delay of the UE to the gNB. The purpose of the existing timing advance procedure is to align the Rx timings of multiple UL signals transmitted by multiple UEs at the location of the gNB, e.g., ideally within a CP.

The UL reception timing at the gNB location for a particular UE can be made to align with the DL transmission timing symbol boundaries during SBFD operation. This however then creates issues when UE DL reception timing in the DL subband(s) is considered. In the TDD cell using SBFD operation, the desired DL signal received by the victim UE will be subjected to a relative Rx timing difference with respect to the interfering UL signal transmitted by an aggressor UE transmitting in the UL subband of the SBFD symbol. The relative Rx timing difference experiences by the victim UE depends on the 3 relative distances, i.e., between the victim UE and the gNB, between the aggressor UE and the gNB and the distance between victim UE and aggressor UE. The relative Rx timing difference observed by the victim UE in the DL subband then actually becomes largest when the victim UE and UE are co-located (or at least very close) and can become zero even at non-zero distance between the victim UE and UE. When multiple aggressor UEs transmit in the UL, the Rx timing spread of the undesired interfering signals experienced by the DL victim UE is larger. Different SBFD slots are subject to different UE pairings with gNB scheduling. It can often be expected that different SBFD slots co-schedule different UL and DL UEs. The relative Rx timing differences experiences by victim UEs in the DL subband of a slot will generally be different and vary per-SBFD slot. Misalignment of the Tx and Rx symbol boundaries beyond the CP length occurs during SBFD operation. This negatively affects UE demodulation performance.

One possible approach for UE-side transmission and reception timing enhancements which has been proposed by some companies is to use different Toffset settings during SBFD operation (Figure 13). In order to better align the symbol boundaries of the SBFD DL subband(s) and the SBFD UL subband at the gNB receiver, a zero Toffset offset value can be used for UL transmissions using the SBFD UL subband in SBFD symbols.

Figure 13: UE UL transmit timing using different TA offset values in SBFD/non-SBFD slots

Another possible approach which has been proposed is to use multiple TAGs per UE on a serving cell to separately control and align the desired UL transmission timing from a particular UE with respect to the DL co-scheduled UEs in the SBFD slot and the gNBs. We think either of these are promising techniques are suitable because they can introduce some degree of per-slot transmission timing control for SBFD operation.

Using either approach, due to the effective use of different TA values by the UE for its UL transmissions in the SBFD and non-SBFD slots, respectively, overlap of the last symbol in an SBFD slot with the first symbol of the UL slot can result. For example, such overlap can be handled by symbol dropping rules. Feasibility and specification impact of such solutions should be considered in the Rel-18 SID.

- Proposal 25: For SBFD-aware UEs, support configuration with different TA offset values or with multiple timing advance groups in the SBFD and non-SBFD slots.

# Conclusion

In this contribution, we made the following proposals:

- Proposal 1: RAN1 should evaluate the potential impacts from SBFD when the NR TDD cell with SBFD support is configured as PCell, SCell or SpCell for a legacy UE or for an SBFD-aware UE.

- Proposal 2: The SBFD-aware UE is configured with the frequency-domain occupancy of the SBFD DL subband(s) on an SBFD symbol.

- Proposal 3: The frequency-domain occupancy of the SBFD guard band(s) on an SBFD symbol is determined by the SBFD-aware UE as any RB not configured as SBFD UL or DL subband(s) in the NR carrier bandwidth.

- Proposal 4: On a symbol configured as DL in TDD-UL-DL-ConfigCommon, Option 2 is supported at least from the gNB perspective.

- Proposal 5: On a symbol configured as F in TDD-UL-DL-ConfigCommon, Option 2 is supported at least from the gNB perspective.

- Proposal 6: On a symbol configured as F in TDD-UL-DL-ConfigCommon, Option 3 is not supported.

- Proposal 7: The SFBD-aware UE can be configured with an SBFD UL subband on DL symbols indicated for SSB(s), CORESET#0 and TRS.

- Proposal 8: SIB1 based cell-common indication of SBFD subband configuration(s) for SBFD-aware UEs is only supported if SBFD operation during random access for UEs in RRC_IDLE/INACTIVE modes is supported.

- Proposal 9: UE-specific SBFD subband and SBFD time-domain configuration for SBFD-aware UEs is the baseline for SBFD operation in RRC_CONNECTED mode.

- Proposal 10: For SBFD subband configuration in time-domain, a slot can consist of both SBFD and non-SBFD symbols.

- Proposal 11: For SBFD-aware UEs, SBFD subband configuration in time-domain with symbol level resolution is the baseline at least in RRC_CONNECTED mode.

- Proposal 12: Study the potential benefits of group-common signaling for SBFD operation.

- Proposal 13: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, support Option 1 and Option 2.

- Proposal 14: For UL transmissions and DL reception across non-SBFD and SBFD symbols in a slot, support Option 1 as baseline

- Proposal 15: For UL transmissions and DL reception across non-SBFD and SBFD symbols in a slot, support Option 2 under the condition that the same QCL and EPRE assumptions can be applied by the UE for the allocation on the non-SBFD and SBFD symbols.

- Proposal 16: For resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands, support Option 1.

- Proposal 17: For SBFD-ware UEs configured with PDSCH resource allocation type 0, support a new configurable RBG set size.

- Proposal 18: For SBFD-aware UEs, support additionally configured RB offset value(s) for PUSCH and PUCCH frequency-hopping.

- Proposal 19: For frequency-domain resource allocation of CSI-RS across SBFD DL subbands, support Option 1 and Option 2-2.

- Proposal 20: For CSI reporting associated with P/S-P CSI-RS, support Option 1 as baseline and Option 2 using separate reporting hypothesis for SBFD symbols and non-SBFD symbols.

- Proposal 21: For SBFD-aware UEs, support separately configured maximum UE output power in SBFD and non-SBFD slots on the serving cell.

- Proposal 22: For SBFD-aware UEs, support separate UL power control parameters and separate beam/spatial SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots.

- Proposal 23: For SBFD-aware UEs, support contention-free random access using SFBD symbols in RRC_CONNECTED mode.

- Proposal 24: RAN1 to study potential benefits and specification impacts to support random access using SBFD slots/symbols for SBFD-aware UEs in RRC_IDLE/INACTIVE mode.

- Proposal 25: For SBFD-aware UEs, support configuration with different TA offset values or with multiple timing advance groups in the SBFD and non-SBFD slots.

In this contribution, we made the following observations:

- Observation 1: Functional backwards-compatibility for legacy UEs when SBFD-operation is supported in the NR TDD cell can be achieved by configuring DDDDU in the SIB1 tdd-UL-DL-ConfigCommon.

- Observation 2: It cannot be assumed that SBFD transparent mode for legacy UEs when configuring DFFFU in the SIB1 tdd-UL-DL-ConfigCommon will always results in consistent UE behavior.

Observation 3: 80 dB in FR1 and 87 dB in FR2-1 antenna isolation using spatial separation and RF barrier can be achieved.

Observation 4: Stopgap performance of the RF barrier for FR1 100 MHz and FR2-1 100 MHz channel BW is feasible.

Observation 5: 45 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources with digital pre-distortion can be achieved in FR1

Observation 6: 28 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources can be achieved in FR2-1.

Observation 7: Both in FR1 and FR2-1, SBFD can operate with only a few guard RBs between DL and UL subband when sufficient spatial isolation is guaranteed.

Observation 8: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.9dB for SFBD in FR1.

Observation 9: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.7 dB with 1T1R and 1 dB with 2T2R panel configurations for SBFD in FR2-1.

Observation 10: Additional Rx filtering can be applied for FR1 and FR2-1 receivers to increase robustness of the gNB Rx path with respect to ADC and LNA dynamic range without incurring undue insertion losses.

Observation 11: gNB power consumption aspects are considered in the SBFD feasibility analysis.

# References

- RP-223041 “Revised SID: Study on evolution of NR duplex operation”; RAN#98-e

- R1-2303126 “Discussion on evaluation for NR duplex evolution”; Samsung

- R1-2301262 “SBFD feasibility and design considerations for NR duplex evolution”; Samsung

- R1-2303128 “Dynamic and flexible TDD for NR duplex evolution”; Samsung

# Annex A: SBFD feasibility and implementation aspects

When SBFD is implemented at the gNB, the received UL signal at the gNB is subject to co-channel cross-link interference (CLI) from the gNB side transmitter. Methods to cancel the CLI include passive methods which rely on the antenna isolation between Tx and Rx antennas, active methods which utilize RF or digital signal processing, hybrid methods using a combination of these, and filtering.

Achieving a sufficient level of SIC is the most critical part when implementing SBFD at the gNB. Without adequate SIC capability, the interference from the transmitted DL signal would corrupt the received UL signal in Figure 14(a). To solve this problem, various SIC schemes can be used. Using the example of Figure 14(b), SIC capability can be provided through the antenna or panel design (A), can be applied in RF domain to the RF signal (B) or in digital signal domain (C), or a combination of these.

Figure 14: gNB transceiver architecture with self-interference cancellation capability

For example, antenna SIC can be used to minimize the leakage power from the Tx ports to the Rx ports of the panel, and digital SIC is then used to handle any residual interference after antenna SIC. DL out-band signal power flowing into the UL Rx path can be effectively suppressed below the noise floor level to guarantee the UL receiver performance. Also, by combining digital pre-distortion (DPD) at the Tx path and digital SIC at the Rx path, the out-band interference from the DL signal to the UL signal can be effectively mitigated by the gNB such that the need for a guard band between the UL and DL signals is minimized. In FR2, the use of separate antenna panels can provide additional spatial isolation.

To check and demonstrate the feasibility and viability of SBFD, Samsung has developed and tested two different testbeds, one operating at FR1 3.5 GHz and one for FR2-1 26 GHz. These validate the feasibility of SBFD operation when implemented at the gNB-side.

## A.1	Self-interference Cancellation

### A.1.1	Spatial-domain and antenna isolation

To simultaneously transmit and receive in the TDD gNB radio unit using SBFD, sufficient Tx/Rx isolation is required. NR TDD radio units use duplexers and multiplexers for the antenna panels which are shared for Tx-mode and Rx-mode in TDM. Such existing RF components alone do not provide sufficient isolation when introducing SBFD in the TDD gNB radio unit.

One solution to increase the Tx/Rx isolation during simultaneous transmission and reception by the gNB is to physically separate the Rx panel and the Tx panel, e.g., separation in antenna domain. Tx/Rx isolation can be increased first simply by increasing the spatial distance. Tx/Rx isolation performance can be further increased when an additional RF barrier structure is used. Using the RF barrier between the Tx and Rx panels affects the required spatial distance separating the Tx and Rx panels. A well-designed RF barrier can minimize the need for large spatial separation and mostly preserve the existing antenna form factor and enclosed volume when compared to legacy TDD. To design an efficient RF barrier, various electromagnetic resonator structures can be incorporated into the antenna design, e.g., wall(s), gap(s), or a combination of them. These result in surface wave nulling and can further block the undesired leakage signals from the Tx panel to the Rx panel.

Figure 15 shows measurement results with respect to the distance between upper and lower antenna panels in our FR1 3.5 GHz SBFD testbed.

While it can be expected that spatial isolation numbers vary depending on the form and particular layout configuration of antenna elements in the upper and lower panels, we have shown that >80 dB antenna isolation is possible between the Tx and Rx panels in FR1.

Figure 15: FR1 testbed and SIC performance when varying distance between upper and lower panel

We observe similar and even better antenna isolation performance with the FR2-1 26 GHz testbed where panel separation can be exploited.

Figure 16 shows the FR2-1 testbed using 2 Tx panels and 2 Rx panels. Unlike the FR1 3.5 GHz testbed where SBFD performance is verified for a single NR carrier setup, the FR2-1 gNB-side testbed uses intra-band contiguous CA with 4 (or 3 CCs). Total aggregated BW is 400 (or 300) MHz with 100 MHz per CC. 3 (or 2) CCs are used for the DL and 1 CC for the UL.

Like described in the case of the FR1 3.5 GHz testbed, the Tx panel and the Rx panel in the FR2-1 26 GHz testbed are separated by a separation distance. Additional Tx/Rx isolation performance is then enabled by using an RF barrier, e.g., an additional EM resonant between the panels. In the case of FR2-1 26 GHz, since each panel can perform more directive beamforming in analog domain than possible in FR1 using mMIMO panels, the FR2-1 antenna isolation performance is better than what is achievable in FR1. An average of 87 dB antenna isolation can be observed based on the measurement results obtained from our FR2-1 testbed.

Figure 16: FR2-1 testbed and SIC performance when varying the operating frequency

An important design consideration for increased spatial isolation provided by the RF barrier is whether such stopband performance is stable over a wide enough frequency range. EM isolators and resonant structures are designed around a specific center frequency, e.g., 3.5 GHz. Therefore, design of the resonant structure must account properly for the channel BW and NR operating band under consideration to provide a sufficiently large stopband between Tx and Rx panel. Another consideration is that undesired Tx/Rx interference is created by multiple EM sources, e.g., antenna elements in the Tx panel. Therefore, diffusion of the corresponding surface waves is more challenging when isolating the Tx and Rx panel. Despite these challenges, our FR1 3.5 GHz and FR2-1 26 GHz testbeds have achieved isolation performance that show almost uniform antenna and panel isolation performance with respect to frequency for the 100 MHz channel BW of the NR carrier in 3.5 GHz and 100 MHz CC BW in 26 GHz. Figure 14 shows measurement results from the FR2-1 testbed with respect to achievable antenna isolation as a function of the operating frequency.

Observation 3: 80 dB in FR1 and 87 dB in FR2-1 antenna isolation using spatial separation and RF barrier can be achieved.

Observation 4: Stopgap performance of the RF barrier for FR1 100 MHz and FR2-1 100 MHz channel BW is feasible.

### A.1.2	Frequency-domain and digital cancellation

On top of the spatial isolation to prevent undesired Tx-Rx interference from the Tx panel to the Rx panel during simultaneous transmission and reception by the gNB radio unit, additional Tx/Rx isolation can be achieved in frequency-domain.

The Tx signal and the Rx signal are respectively allocated to non-overlapping frequency-domain resources on the same time-domain symbol during simultaneous transmission and reception, e.g., SBFD. At least the waveform roll-off therefore reduces the magnitude of the Tx-Rx interference to which the Rx signal is subjected. Additionally, BB filtering can be applied to further increase the achievable isolation.

The use of frequency-domain isolation between the Tx and Rx signal allocations is primarily an approach that serves the purpose of reducing the amount of self-interference which must be further cancelled by a digital cancellation stage. Note that TDD gNB radio unit design must also account for ADC and LNA in the receiver path, e.g., to prevent Rx saturation or blocking by the spectral leakage created from the undesired Tx signal.

In the case of gNB-side SBFD operation, the SBFD UL subband can be considered as out-of-channel with respect to the 1 or 2 SBFD DL subband(s). Undesired spectral leakage from the DL Tx signal in the gNB into the Rx path are reduced similar to the case of out-of-channel leakage, e.g., comparable to the gNB Tx-side Adjacent Channel Leakage Ratio (ACLR) for coexistence between two operators on adjacent channels in the same NR band. Note that ACLR is determined by the non-linear characteristics of the PA and corresponding RF requirements are set by RAN4, e.g., 45 dBc for the gNB Tx.

While it can be considered to assume that the achievable Tx-to-Rx interference from the SBFD DL subband to the UL subband can only guarantee performance according to the less stringent in-channel RF requirements, our FR1 3.5 GHz testbed implementation shows that the use of digital pre-distortion (DPD) techniques to improve upon the non-linearity characteristics of the PA can achieve 45 dBc isolation between the SBFD DL and UL subbands.

Figure 17 shows the achievable isolation in frequency domain for FR1 SFBD when Tx-to-Rx leakage is also compensated for by DPD based on the FR1 3.5 GHz testbed.

Observation 5: 45 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources with digital pre-distortion can be achieved in FR1.

Figure 17: FR1 testbed and PSD for SBFD DL and UL SBs after antenna isolation and digital pre-distortion

In the case of FR2-1, frequency-domain isolation for SBFD is of particular importance. Non-linear characteristics of mmWave PAs are worse than those of FR1 mid-band PAs. 3GPP ACLR requirements are more relaxed in FR2-1 when compared to FR1. This is due to beamforming providing isolation in FR2-1, implying that the probability of a blocker coming from the same direction is much lower than in FR1. Another consideration is that in FR1, the difference between the out-of-channel requirements like the ACLR and in-channel requirements like EVM is large. The PA linearity requirement is therefore dominated by out-of-channel requirements, e.g., ACLR. In FR2-1, these are at comparable levels. Spectral regrowth due to IM3 is dominant for in-channel requirements and as such, PA linearity requirements are rather driven by EVM and possibly in-band emissions. Another design challenge for DPD in FR2-1 is that PA characteristics must be carried through a feedback link from the output of the PA. In the case of mmWave, it is more difficult than in FR1 to create such a feedback link due to signal attenuation. Therefore, it is significantly more challenging to exploit DPD in FR2-1 such as done for FR1.

Despite these design challenges for gNB-side SBFD operation, our FR2-1 26 GHz testbed measurement results in Figure 18 show that 28 dBc leakage ratio between DL and UL subband (or component carriers) are still possible, e.g., similar to ACLR as existing out-of-channel requirement for FR2-1.

Observation 6: 28 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources can be achieved in FR2-1.

Figure 18: FR2-1 testbed and PSD for SBFD DL and UL subbands after antenna isolation and filtering

### A.1.3	Additional design aspects

Frequency-domain separation to achieve a 45 dBc leakage ratio in FR1 benefits from the presence of a few RBs guard-band, e.g., 5 RBs, between the SBFD DL and UL subband. Since DPD is affected not only by the non-linear PA characteristics but also by noise caused from memory effects and PA temperature, absence of guard RBs is not meaningful to assume. In addition, presence of a few guard RBs between the DL and UL subband helps to balance the interference power per subcarrier in the UL subband which helps digital SIC performance.

Time-domain digital SIC can be used to further reduce the number of guard RBs. For example, if there is sufficient spatial isolation such that no ADC or LNA saturation in the Rx path occurs, the residual amount of interference power per subcarrier does not impact the digital SIC performance with time-domain SIC. Instead of using the frequency-domain digital SIC after FFT, time-domain filter taps which adapt according to the self-interference channel estimation can be applied to the Tx signal and remove the estimated leakage signal from the time-domain received signal. Using time-domain SIC, the guard-band size does not impact the digital SIC performance. The guard band can be smaller or the gNB radio unit can even operate without any guard band between the SBFD DL and UL subband.

Figures 18 (FR1) and 19 (FR2-1) show the achievable performance using time-domain SIC in our FR1 3.5 GHz and FR2-1 26 GHz testbeds respectively. Note that no guard band between the SBFD DL subband and UL subband was used.

Observation 7: Both in FR1 and FR2-1, SBFD can operate with only a few guard RBs between DL and UL subband when sufficient spatial isolation is guaranteed.

Another consideration is that the desired received signal is mixed with the undesired DL leakage signal in the Rx path of the gNB radio, e.g., after ADC. The unwanted DL leakage signal must be removed by receiver processing using digital SIC. It is necessary to estimate the interference channel between the Tx panel and the Rx panel. Digital SIC performance is helped when synchronization to accurately remove the Tx signal from the Rx signal can be obtained. In principle, two methods exist to estimate the interference channel. One approach is to store information on a Tx signal that has passed through the PA with a feedback link and then estimate the interference channel over-the-air to remove the interference from the Rx signal. Another approach is to use only over-the-air estimation. Without a feedback link, the whole combined channel can still be estimated through the Rx panel. We used the first approach in the FR1 3.5 GHz testbed and the second approach in the FR2-1 26 GHz testbed.

As we observed, both approaches can effectively estimate the interference channels and eliminate undesired Tx-to-Rx interference. Based on the measurement results in the testbeds, multiple interference channels can be removed simultaneously. No more than 0.9 dB noise rise (INR) was measured in our FR1 testbed. In the case of FR2-1, noise rise performance of 0.7 dB for 1T1R and 1 dB for 2T2R configurations were measured as shown in Figures 18 and 19.

Observation 8: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.9dB for SFBD in FR1.

Observation 9: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.7 dB with 1T1R and 1 dB with 2T2R panel configurations for SBFD in FR2-1.

Figure 19: FR1 testbed and Rx signal after digital SIC for INR < 0.9 dB

Figure 19: FR2-1 testbed and Rx signal after digital SIC for 1T1R (left) and 2T2R (right)

To prevent ADC saturation in the Rx path of the gNB radio unit supporting SBFD, Rx filtering can be used to suppress the leakage from the Tx side interfering signal. Additional Rx filters can provide protection to avoid potential dynamic range and saturation issues for ADC or LNA when demodulating the UL subband in the Rx path of the gNB. Note that for RF filters with sharp roll-off’s, the order of the filter must increase, and so must then the size of the filter. Additional insertion losses are incurred which negatively affect the link budget. Additionally, analog filters such as IF and BB filters can be employed. For example, when the receiver is designed to use zero IF architecture, the receiver can use the lowpass filter to further remove the leakage signal after applying the mixer. By combining multiple LNAs, filter loss can be compensated more easily.

Observation 10: Additional Rx filtering can be applied for FR1 and FR2-1 receivers to increase robustness of the gNB Rx path with respect to ADC and LNA dynamic range without incurring undue insertion losses.

## A.2	FR1 and FR2-1 testbed performance

We show end-to-end performance results for SBFD based on our FR1 and FR2-1 testbeds in Table 1 and Table 2. Note that FR2-1 results were obtained using an outdoor test environment.

In the case of SBFD in FR1 n78, performance is compared for 1-slot PUSCH transmission without DL interference for the legacy TDD case and 5-slots repeated PUSCH transmissions with DL interference for the first 4 slots (Table 1). For PUSCH transmission, MCS index 4 is selected and PUSCH transmitted over a 20 MHz UL subband placed in the center of the 100 MHz channel BW. When all repeated PUSCH transmissions are combined by the gNB receiver, an SNR gain of 6.1 dB is observed. Note that the FR1 testbed used the fully implemented SIC capability.

In the case of SBFD in FR2-1, we tested throughput performance over 4x100 MHz carriers. 1 CC among 4 CCs is used for the SBFD UL transmissions from the UE. PUSCH uses MCS index 24. For the legacy TDD case, only one FR2-1 CC is used for the PUSCH transmission without DL self-interference. Despite DL leakage somewhat degrading the UL receiver performance, we observe that SBFD using SIC allows for 4.13x throughput gain compared to legacy TDD in our FR2-1 testbed.

Table 1: FR1 testbed subband configuration for PUSCH and SNR performance

| Uplink throughput | TDD | SBFD |
| --- | --- | --- |
| SBFD Subband setting | DDDDU (100 MHz) | DDDD-- (40 MHz) UUUUU (20 MHz) DDDD-- (40 MHz) |
| SNR gain using repetitions | 0 dB | 6.1 dB |

Table 2: FR2-1 testbed CC configuration for PUSCH and throughput performance

| Uplink throughput | TDD | SBFD |
| --- | --- | --- |
| SBFD carrier setting | DDDD-- (4th CC, 100MHz) DDDDU (3rd CC, 100MHz) DDDD-- (2nd CC, 100MHz) DDDD-- (1st CC, 100MHz) | DDDD-- (4th CC) UUUUU (3rd CC) DDDD-- (2nd CC) DDDD-- (1st CC) |
| Throughput gain with new transmission | 75 Mbps (1x) | # bps (4.13x) |

## A.3	gNB power consumption

When evaluating the feasibility and performance of SBFD schemes for Local Area (LA), Medium Range (MR) and Wide Area (WA) base station classes in the Rel-18 SID NR duplex evolution, gNB power consumption must be considered as part of the feasibility analysis.

The SBFD implementation approach selected by a network vendor may affect the observed gNB power consumption profile during sustained system operation in a variety of ways, e.g., RFFE components, gNB scheduling, and site deployment aspects.

The direct Tx-Rx self-leakage link budgets for LA, MR and WA BS classes should be considered first when analyzing the need for higher linearity Tx RFFE or more RF components in the gNB implementation. In the case of the LA BS class, FR1 antenna isolation of 75 – 80 dB or more avoids Rx desensitization. There is little need for additional SI mitigation measures. The MR BS class somewhat more benefits from increased Tx linearity but relying on Tx-side analog/digital DPD or Rx-side digital SIC are meaningful design alternatives while preserving existing gNB implementations. In both cases, we do not expect significantly impacted gNB power consumption incurred by the RFFE components. WA BS classes are more challenging not only because of the larger Tx-Rx self-leakage budget, but also due to the very high Tx power regime of the PAs. In the Rx path, analog filtering may be required.

We would like to point out however, that even for the WA BS class, expected hardware and power consumption impacts when implementing SBFD are highly dependent on the vendor baseline. For example, the complexity of the analog/digital DPD component does not necessarily scale linearly with the number of Tx and Rx ports, respectively, depending on the selected SBFD implementation. Pre-distortion to increase the linearity in the self-interference channel(s) can be implemented with respect to the Rx ports only to reduce complexity. Power consumption then scales accordingly. Similar considerations apply to the eventual need of higher linearity or high-power filtering components. If the existing baseline implementation uses a PA supporting ACLR = -45 dBc for the 100 MHz channel bandwidth to meet out-of-band requirements, and the vendor design already maintains the -45 dBc leakage power ratio even for the in-band case, then no additional components and no corresponding power consumption increase for PA and pre-distortion components is incurred.

When the base station implementation supports SBFD operation, the impact of gNB scheduling must be considered. SBFD in a slot uses a smaller DL transmission BW, e.g., 2x40 or 80 MHz than the TDD channel BW, e.g., 100 MHz in a DL-only slot. The Tx PA efficiency is increased with SBFD when compared to TDD due to less DC bias which in turn increases the linearity of Tx PA. The need to run pre-distortion and digital SIC components in the base station also depends on the DL-UL co-scheduling allocations. gNB power consumption is increased in slots with simultaneous DL and UL transmissions. SBFD DL slots scheduled for UL-only allocations may see decreased gNB power consumption.

Site deployment aspects such as the selected antenna and panel architecture when SBFD is deployed affect the gNB power consumption primarily through the number of TRXs. However, the use of antenna option 1 where half of the antenna panel is used for Tx when compared to the TDD baseline would result in less power consumption with SBFD. The use of antenna option 2 and 3 would result in a higher power consumption, but this increase can be considered equivalent to the deployment of a second antenna panel in the TDD baseline case.

In summary, gNB power consumption should be considered as part of the SBFD feasibility analysis required by the Rel-18 SID. When SBFD is implemented, the gNB power consumption profile can be affected by several contributing factors, e.g., gNB RFFE components, gNB scheduling, and site deployment aspects. Based on our results presented in this section for the FR1 and FR2-1 testbeds, we consider that gNB power consumption and complexity when implementing SBFD is very manageable. We also note that some of these such as the potential need for more high-linearity RFFE components highly depend on the existing vendor baseline and their selected SBFD implementation approach.

Observation 11: gNB power consumption aspects are considered in the SBFD feasibility analysis.