3GPP TSG RAN WG1 #112bis-e	R1-2303134

# e-Meeting, April 17th – 26th, 2023

Agenda item:	9.5.1.2

Source: 	Samsung

Title: 	On Measurements and Reporting for SL Positioning

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about measurements and reporting for SL positioning for Rel-18 work on expanded and improved NR positioning and provide our view.

- Discussion

In the WID [1], the following objective is included regarding the measurements and reporting for SL positioning as:

- Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF.

In the following subsections, we discuss about measurements and reporting of SL RTT, SL AoA, and SL TDOA methods. Also, we discuss about other measurements and necessary procedure for SL positioning in order.

  - SL RTT

PRS transmission order for SL RTT

As shown in Fig. 1(a), for single-sided RTT, time of flight (ToF) can be estimated by UE-A when it receives SL-PRS2 and Rx-Tx time difference (i.e. Treply) from UE-B. Specifically, ToFestimated=12Tround1+eA-Treply(eB+1) where eA and eB are clock source errors (in ppm) for UE-A and UE-B, respectively. When there is no clock source errors, ToF=12(Tround-Treply). Therefore, the ToF estimation error by clock drift would be ToF-ToFestimated≈12eA-eBTreply [2]. This means that the ToF estimation error depends on Treply (decided by UE-B’s transmission timing of SL PRS as depicted in Fig. 1(a)) and the clock timing difference between UEs. On the other hand, it is well known that the double-sided RTT can mitigate the ToF estimation error by clock drift [2]. However, As shown in Fig. 1(b), the proceure ends at UE-B and the ToF can be estimated by UE-B. For UE-A to estimate the ToF, UE-B needs to provide Tround2 information additionally into UE-A and this can increase the time for the ToF estimation. Also, in the double-side RTT, UE-A may need to perform resource selection for SL-PRS1 and SL-PRS3, independently.

| (a) Single-sided RTT | (b) Double-sided RTT |
| --- | --- |

Figure 1: Single-sided and double-sided RTT methods.

| (a) Enhanced single-sided RTT method 1 | (b) Enhanced single-sided RTT method 2 |
| --- | --- |

Figure 2: Enhanced single-sided RTT methods.

To reduce the ToF estimation time in double-sided RTT and to estimate the ToF by UE-A, we can consider enhanced single sided RTT methods as depicted in Fig. 2. At first, for method 1 as shown in Fig. 2(a), UE-A selects and transmits two SL PRS resources (i.e. SL-PRS1 and SL-PRS2). UE-B then calculates Rx-Tx time differences (i.e. Treply1 and Treply2) between the two received SL PRS resources (i.e. SL-PRS1 and SL-PRS2) and the transmitted SL PRS resource (i.e. SL-PRS3). Finally, UE-A then estimates the ToF when it receives SL-PRS3 and Rx-Tx time differences from UE-B by compensating the ToF estimation error from the clock drift as:

ToF= 12(Tround1-δ ∙Treply1),                                 (1)

where δ=Tround1-Tround2Treply1-Treply2 is the compensating factor to mitigate the clock timing difference between UEs. Here, Tround1-Tround2 and Treply1-Treply2 are the time differences of two SL PRS resources (i.e. SL-PRS1 and SL-PRS2) calculated by UE-A and UE-B, respectively. Therefore, δ becomes the clock drift rate between two UEs.

Similarly but in an opposite way, for method 2 as shown in Fig. 2(b), UE-B selects and transmits two SL PRS resources (i.e. SL-PRS2 and SL-PRS3) when it receives SL PRS (i.e. SL-PRS1) from UE-A. UE-B then calculates Rx-Tx time differences (i.e. Treply1 and Treply2) between the received SL PRS resource (i.e. SL-PRS3) and the two transmitted SL PRS resources (i.e. SL-PRS2 and SL-PRS3). Finally, UE-A then estimates the ToF when it receives two SL PRS resources (i.e. SL-PRS2 and SL-PRS3) and Rx-Tx time differences from UE-B by compensating the ToF estimation error from the clock drift. Equation (1) can be used also but the compensating factor would be δ=Tround2-Tround1Treply2-Treply1.  Here, Tround2-Tround1 and Treply2-Treply1 are the time differences of two SL PRS resources (i.e. SL-PRS2 and SL-PRS3) calculated by UE-A and UE-B, respectively.

Notice that the enhanced single sided RTT methods in Fig. 2 can provide the same advantage of the double-sided RTT (i.e., reducing ToF estimation error by clock timing difference between UEs). However, the enhanced single-sided RTT methods can reduce ToF estimation time over the double-sided RTT even though it allows for UE-A to estimate the ToF. The reduced time for RTT procedure would be beneficial for SL positioning. Moreover, it can minimize the specification impact to change needed for the double-sided RTT. As described in Fig. 2, the enhanced single-sided RTT methods are just required for UE to select two resources for SL PRS transmission (UE-A for method 1 and UE-B for method 2) while the single-sided RTT is required to select only one resource.

In this regard, the following agreement was made for SL RTT in RAN1#112 meeting as:

Agreement For SL-PRS based Rx-Tx measurement for double sided RTT, consider sidelink PRS transmission without order restriction between multiple rounds of PRS transmission of involved UEs. - FFS on how to differentiate different PRS transmissions for sidelink PRS Rx-Tx measurement and report - FFS on impact of Scheme 2 resource allocation when the different orders in double sided RTT is considered and whether and how to minimize number of different orders - Aspects related to scheme 2 resource allocation are to be discussed in agenda 9.5.1.3

Compared with the original-double sided RTT in Fig. 1(b), PRS transmission order change in Fig 2 can provide further benefits as we explained above. Therefore, we propose to specify at least two RTT methods of Fig. 1 (a) and Fig. 2 (a). The double-sided RTT in Fig. 1(b) can be specified when it’s additional advantage is clarified (e.g., minimizing the impact of UE mobility). As shown in Fig. 1 (a) and Fig 2. (a), the difference of two RTT methods is the number of SL PRS transmission from UE-A (i.e. target UE). Specifically, UE-A (i.e. target UE) transmits one SL PRS resource in Fig. 1 (a) while UE-A (i.e. target UE) transmits two SL PRS resources in Fig. 2 (a). Assuming that there is no SL PRS retransmission and the second SL PRS resource is reserved and indicated by PSCCH, UE-B (i.e. anchor UE) can identify the RTT method requested by UE-A (i.e. target UE) according to the number of SL PRS transmitted by UE-A (i.e. target UE). Then, the UE-B (i.e. anchor UE) can reply on the identified RTT request by transmitting SL PRS and reporting Rx-Tx time difference. We need to discuss further about container for reporting Rx-Tx time difference.

Proposal 1: At least, two RTT methods are supported for SL positioning where

- In RTT method 1, a target UE transmits one SL PRS resource to anchor UE(s).

- In RTT method 2, a target UE transmits two SL PRS resources to anchor UE(s).

  - The second SL PRS resource is reserved and indicated by PSCCH. From this, the anchor UE can identify whether RTT method 1 or 2 is used.

- In RTT method 1, the anchor UE transmits SL PRS and reports one Rx-Tx time difference.

- In RTT method 2, the anchor UE transmits SL PRS and reports two Rx-Tx time differences.

- FFS: container for reporting Rx-Tx time difference.

- FFS: double-sided RTT and the impact of UE mobility

Definition of SL PRS based Rx-Tx difference measurement

In RAN1#112 meeting, the following agreement was made for the definition on Rx-Tx difference as:

Agreement For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu.

In our understanding, actual SL PRS transmission time in Alt 1 becomes ‘A.Tx’ of UE-B’s Tx timeline in Fig. 3. Therefore, the Rx-Tx difference can be measured as multiple slots in Alt 1 as depicted in Fig. 3. On the other hand, the SL-PRS transmission time in Alt 2 becomes ‘Ref.Tx’ of UE-B’s Tx timeline in Fig. 3. Specifically, to determine SL RTT between UE-A and UE-B, UE-A transmits SL PRS in slot m1, while UE-B transmits SL PRS in slot m2 as illustrated in Fig. 3. The transmission from UE-A in slot m1 arrives at time at Tm1-RxUEB at UE-B, time Tm1-RxUEB is measured in UE-B’s timeline. There is no transmission in slot m1 for UE-B, but a would-be (reference) transmission from UE-B would have taken place at starting at time Tm1-TxUEB. UE-B can measure the Rx-Tx difference between its reference transmit time (Tm1-TxUEB) and the actual receive time of a transmission from UE-A (Tm1-RxUEB). i.e.,

TRx-TxUEBm1=Tm1-RxUEB-Tm1-TxUEB

The reporting range for the Rx-Tx difference time can be 1msec and we can reuse the resolution step size defined for Uu Rx-Tx difference time in TS 38.133.

Figure 3: SL PRS for round trip measurement.

Proposal 2: For definition of SL-PRS based Rx-Tx measurement, SL-PRS transmission time based on the SL PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement.

- The reporting range for the Rx-Tx difference for SL RTT is 1msec.

- Reuse the resolution step defined in TS 38.133 for Uu Rx-Tx difference time.

  - SL AoA

Definition of SL PRS based AoA measurement

In RAN1#112 meeting, the following agreement was made for SL AoA as:

Agreement Support both GCS and LCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement. - FFS on the applicable scenario/service for AoA/ZoA relative to LCS without translation of the LCS to GCS

One remaining issue is to define SL AoA. In TS 38.215, UL AoA is defined as:

- UL Angle of Arrival (UL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relatize to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15]. - The UL AoA is determined at the gNB antenna for an UL channel corresponding to this UE.

We think that above definition can be reused for SL AoA. Thus, we propose:

Proposal 3: For SL AOA, both LCS and GCS are supported and the measurement is defined as:

- SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined:

  - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction

  - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The x, y and z axis form a right-handed co-ordinate system. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15].

- The SL AoA is determined at the UE antenna for an SL channel corresponding to a peer UE communicating each other.

  - SL TDOA

Definition of SL-PRS based RTOA

In RAN1#112 meeting, the following agreement was made for SL RTOA as:

Agreement SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 or DFN0. - FFS on how to select between SFN 0 or DFN 0 for determination of T0. - FFS: the source for the reference timing - tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively - FFS on how to select between SFN or DFN

Based on the TS 38.331, for SL communication, DFN is used when a UE selects GNSS as the synchronization reference source. We think that the same principle can be applied for SL positioning. Therefore, we propose

Proposal 4: For SL positioning, the synchronization reference source becomes the source for the reference timing and DFN is used when a UE selects GNSS as the synchronization reference source.

Definition of SL-PRS based RSTD

In RAN1#112 meeting, we discussed about the following proposals in [3] but could not reach a consensus.

- SL reference signal time difference (SL RSTD) is defined as the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi, Where: - TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j. - TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j. For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE. Note: how UE i is selected is a separate discussion.

In the above note, the reference UE i needs to be decided/selected for SL RSTD measurement. We can consider the following alternatives as

- Alt 1: (Pre-) configuration. The reference UE is (pre-) configured.

- Alt 2: Determined by the UE performing the RSTD measurement. The reference UE is selected with the highest SL PRS-RSRP.

- Alt 3: Indicated to the UE performing RSTD. For example, an RSU can indicate to UEs in its vicinity that it is the reference UE.

Among the alternatives, we think Alt 2 is the most reasonable approach since the reference UE can be decided and adjusted quickly according to varying channel condition of SL communication.

Proposal 5: SL reference signal time difference (SL RSTD) is defined as the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi, Where:

- TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j.

- TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j.

- The reference UE i is determined by the UE performing the RSTD measurement and the reference UE i is selected with the highest SL PRS-RSRP.

- For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE.

Synchronization error for SL-TDOA

In RAN1#110bis-e meeting, the following agreement was made for SL TDOA as:

Agreement With regards to SL-TDOA positioning method for SL-only positioning, - It corresponds to a method where SL-PRS is being transmitted from multiple anchor UEs to a target UE (i.e., DL-TDOA-like operation), and/or from a target UE to multiple anchor UEs (i.e. UL-TDOA-like operation) at least for the purpose of absolute positioning estimation of the target UE Study the detailed procedure, necessary signalling for SL-TDOA, method(s) to mitigate impact of synchronization error between multiple anchor UEs including whether such method(s) are needed.

In RAN1#111 meeting, the following observation was captured for SL TDOA as:

Observation SL absolute positioning performance may be degraded due to uncertainty in the anchor UEs’ location coordinates and synchronization error (for SL-TDOA) between anchor UEs.

Actually, SL-TDOA requires very accurate time synchronization between anchor UEs. However, achieving high synchronization accuracy between multiple anchor UEs is very difficult and expensive. Unlike in Uu, we cannot simply assume that perfect synchronization between anchor UEs transmitting or receiving SL-PRS. The anchor UEs can have a difference clock source with different accuracy level. Specifically, depending on the oscillator type equipped to anchor UEs, its clock accuracy level will be different [4]. It is important to note that clock source’s accuracy should be guaranteed in certain level for SL-TDOA in order to satisfy SL positioning accuracy requirement. Therefore, we propose

Proposal 6: For SL-TDOA, the clock source’s accuracy should be taken into account to mitigate impact of synchronization error between multiple anchor UEs.

- FFS: (pre-)configuration of the clock source’s accuracy requirement/threshold or signalling of UE’s clock source accuracy status

- FFS: How to select anchor UEs to mitigate impact of synchronization error

SL TDOA procedure

Based on the SL TDOA methods discussed in the previous section, we need to discuss about SL TDOA procedure as a next step. For SL TDOA, it is assumed that target UE and anchor UE(s) are synchronized with a same sync source where the sync source can be gNB, GNSS, or sycRef UE. In addition, as shown in Fig. 4, two different SL TDOA methods can be considered. Namely, SL RSTD in Fig. 4(a) and SL RTOA in Fig. 4(b). The entity for triggering SL positioning can be decided by RAN2. If SL positioning is triggered and SL RSTD is configured, we can consider the following two steps as:

- Step 1: A target UE transmits a RSTD request to anchor UE(s).

- Step 2: Anchor UE(s) replies on the RSTD request by transmitting SL PRS to the target UE.

On the other hand, if SL positioning is triggered and SL RTOA is configured, we can consider the following two steps as:

- Step 1: A target UE transmits SL PRS with RTOA request to anchor UE(s).

- Step 2: Anchor UE(s) replies on the RTOA request by providing SL RTOA measurement to the target UE.

We need to discuss further about signalling details for RSTD and RTOA requests and RTOA reporting.

Proposal 7: For SL TDOA, we propose the following procedure as:

- In SL RSTD, a target UE transmits a RSTD request to anchor UE(s). Then, anchor UE(s) replies on the RSTD request by transmitting SL PRS to the target UE.

- In SL RTOA, a target UE transmits SL PRS with RTOA reqeust to anchor UE(s). Then, anchor UE(s) replies on the RTOA request by providing SL RTOA measurement to the target UE.

- FFS: detailed signalling for RSTD and RTOA request and RTOA reporting.

| (a) SL RSTD | (b) SL RTOA |
| --- | --- |

Figure 4: Two SL TDOA methods.

  - SL positioning measurement reporting

In RAN1#112 meeting, the following agreement was made for SL RTOA as:

Agreement Study measurement report content for both the cases of sidelink positioning measurement reported to LMF and UE. Agreement Companies are encouraged to provide expected measurement report content in the following table to facilitate discussion in RAN1 #112bis-e. Note: this does not imply a different measurement report content for reporting to LMF or to UE. Table 6.2 Collection of measurement report content reporting to LMF; reporting to UE. SL-PRS based Rx-Tx measurement. SL-PRS based RSTD measurement. SL-PRS based RSRP measurement. SL-PRS based RSRPP measurement. SL-PRS based RTOA measurement. SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement. etc.

For the measurement report contents, we think all contents can be reported to LMF and UE. For the additional contents, we think LOS/NOS indicator can be included based on the following agreement we made in RAN1#112 meeting as:

Agreement LoS/NLoS indicator can be included in a sidelink positioning measurement report, considering different reporting targets (LMF and UE). - LOS/NLOS indicator specified in Rel-17 positioning is reused as much as possible. - No specification impact for how to set this indicator. - From RAN1 perspective, no performance requirements are expected to be defined for setting indicator in Rel-18.

Also, time stamp can be included as one of measurement report contents. In addition, SL-PRS resource ID/SL-PRS resource set ID can be included if it is supported. Lastly, the quality of synchronization information (e.g., clock source’s accuracy) can be included considering synchronization error for SL-TDOA. Further details about this was discussed in Section 2.3. In this regard, we propose:

Proposal 8: For SL positioning, the following measurement contents are reported to LMF and UE as:

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| SL-PRS based Rx-Tx measurement | yes | yes |
| SL-PRS based RSTD measurement | yes | yes |
| SL-PRS based RSRP measurement | yes | yes |
| SL-PRS based RSRPP measurement | yes | yes |
| SL-PRS based RTOA measurement | yes | yes |
| SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement | yes | yes |
| LOS/NLOS indicator | yes | yes |
| time stamp | yes | yes |
| SL-PRS resource ID/SL-PRS resource set ID | yes | yes |
| quality of synchronization information (e.g., clock source’s accuracy) | yes | yes |

SL positioning measurement can be reported by higher layer signaling with similar principle to Uu positioning. In addition to higher layer signaling, we prefer to also study the feasibility of reporting SL positioning measurement by PHY layer signaling, e.g. 2nd stage SCI.

The key motivation is that in a stand-alone resource pool dedicated for SL positioning, UE should also have available approach to report SL positioning measurement to other SL UEs. Otherwise for the purpose of SL positioning, resource pool configuration always needs to associate a dedicated resource pool for transmitting SL PRS with a shared pool/a data pool for reporting SL positioning measurement, which is quite non-flexible. If the available approach is higher layer signaling in dedicated pool, then dedicated pool is similar to shared pool and loss its gain of simplicity and resource efficiency. Therefore, it is beneficial to discuss PHY layer signaling as container of SL positioning measurement in RAN1.

Proposal 9: Discuss the feasibility of reporting SL positioning measurement by using 2nd stage SCI in RAN1.

For triggering of SL measurement reporting, a straightforward solution is by request signaling from gNB or from other SL UE. However, if there are not enough SL PRS resource for measurement before UE being triggered to report, the reliability requirement of SL positioning measurement cannot be satisfied. Considering SL PRS transmission and measurement are more risky than Uu positioning e.g. impacted by collision or half duplex, it can be studied if further conditions to trigger PRS measurement report should be introduced. For example, UE is triggered to report the measurement results when the number of SL PRS detected/measured by UE is greater than a (pre)configured value. Alternatively, in Scheme 2 resource allocation for SL PRS, UE is triggered to measure SL PRS or report measurement results only when sufficient SL PRS resources is indicated to the UE e.g. in IUC information.

Proposal 10: Support request signaling to trigger SL positioning measurement reporting, and study further trigger conditions to satisfy reliability requirement of SL positioning measurement.

The principle of Uu positioning measurement window can be also introduced to SL positioning measurement. SL positioning measurement can be configured and confined within a measurement window, UE measures SL PRS within the window and report measurement result at end of this window. Considering high moving speed of SL UE, the latency requirement of SL positioning measurement might be more sensitive than Uu positioning, thus it is beneficial to consider enhancements on latency reduction of SL positioning measurement report. For example, if SL PRS measurement can be performed in the first N SL PRSs within a configured SL PRS measurement window, UE can report starting from N+1 to the end of the window by PSSCH or PSCCH, rather than keep measuring remaining SL PRSs and wait for reporting until end of measurement window. On the contrary, if the latency requirement can be satisfied, a single measurement report can indicate multiple measurement results to reduce the signalling overhead.

Proposal 11: Study further enhancements on latency reduction and overhead reduction for SL positioning measurement reporting.

  - Other SL measurements and necessary procedure

Other necessary procedure for SL measurement

When a UE synchronized to a network, the clock of the UE depends on the downlink propagation delay between the network and UE. UEs with different distances (i.e., different round trip propagation delays) from the network have different clock offsets. In a system with two UEs synchronized to the network that are performing ranging based on SL-TDOA, doing ranging can help determine the distance between the two UEs. For example, these can be UEs traveling on a remote road that isn’t covered by RSUs. A timing diagram for this scenario is illustrated in Figure 3. The gNB transmits a DL reference signal. The DL reference signal arrives at UE1 with a delay equal to half the round-trip-time (RTT1/2), this determines the reference time of UE1. The DL reference signal arrives at UE2 with a delay equal to half the round-trip-time (RTT2/2), this determines the reference time of UE2.

Figure 5: Time diagram for ranging with two UEs synchronized to the network.

UE1 transmits a SL PRS relative to its reference time, assume that UE1 advances the signal by a time T1 relative to its reference timing, and UE2 measures the time of arrival of the SL PRS relative to its reference time, the measured time is Tm2. As illustrated in Fig. 5, the measured time at UE2 can be expressed as:

Tm2=T1-RTT1-RTT22-T12

where, T12=d12c, and d12 is the distance between UE1 and UE2. By measuring Tm2 and knowing T1, RTT1 and RTT2, it is possible to determine T12 and d12.

Proposal 12: Consider for a UE to measure and report the time of arrival of a SL signal relative to time of arrival of a DL reception from a gNB.

In RAN1#111 meeting, the following observation was captured about the uncertainty in the anchor UEs’ location coordinates as:

Observation SL absolute positioning performance may be degraded due to uncertainty in the anchor UEs’ location coordinates and synchronization error (for SL-TDOA) between anchor UEs.

Actually, the uncertainty in the anchor UEs’ location coordinates becomes an error source for positioning. Therefore, when the anchor UEs’ location coordinates is provided, the selection of anchor UE(s) with reliable location coordinate has a big effect on SL absolute positioning accuracy as we discussed in our companion contribution [5]. Therefore, when the anchor UEs’ location coordinates is transferred into target UE, the UE need to provide a reliability of the location information. Therefore, we propose:

Proposal 13: The reliability of the anchor UEs’ location coordinates is indicated to the target UE for SL positioning.

Depending on SL environments, the number of available measurement sources for positioning can be limited especially for OoC scenario and its quality can deteriorate when both anchor UEs and a target UE are moving. The evaluation results in our companion contribution [5] shows that the target UE’s selection of measurement sources (i.e. anchor UE/RSU) is important for achieving high positioning accuracy. Similarly, it would be necessary for the target UE to decide whether to perform absolute positioning or relative positioning or ranging depending on availability and quality of measurement source(s).

Proposal 14: In SL positioning, a target UE decides whether to perform absolute positioning or relative positioning or ranging depending on availability and quality of measurement source(s).

- Conclusions

This contribution discusses about measurements and reporting for SL positioning. Based on the discussion, the following proposals are provided:

Proposal 1: At least, two RTT methods are supported for SL positioning where

- In RTT method 1, a target UE transmits one SL PRS resource to anchor UE(s).

- In RTT method 2, a target UE transmits two SL PRS resources to anchor UE(s).

  - The second SL PRS resource is reserved and indicated by PSCCH. From this, the anchor UE can identify whether RTT method 1 or 2 is used.

- In RTT method 1, the anchor UE transmits SL PRS and reports one Rx-Tx time difference.

- In RTT method 2, the anchor UE transmits SL PRS and reports two Rx-Tx time differences.

- FFS: container for reporting Rx-Tx time difference.

- FFS: double-sided RTT and the impact of UE mobility

Proposal 2: For definition of SL-PRS based Rx-Tx measurement, SL-PRS transmission time based on the SL PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement.

- The reporting range for the Rx-Tx difference for SL RTT is 1msec.

- Reuse the resolution step defined in TS 38.133 for Uu Rx-Tx difference time.

Proposal 3: For SL AOA, both LCS and GCS are supported and the measurement is defined as:

- SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined:

  - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction

  - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The x, y and z axis form a right-handed co-ordinate system. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15].

- The SL AoA is determined at the UE antenna for an SL channel corresponding to a peer UE communicating each other.

Proposal 4: For SL positioning, the synchronization reference source becomes the source for the reference timing and DFN is used when a UE selects GNSS as the synchronization reference source.

Proposal 5: SL reference signal time difference (SL RSTD) is defined as the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi, Where:

- TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j.

- TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j.

- The reference UE i is determined by the UE performing the RSTD measurement and the reference UE i is selected with the highest SL PRS-RSRP.

- For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE.

Proposal 6: For SL-TDOA, the clock source’s accuracy should be taken into account to mitigate impact of synchronization error between multiple anchor UEs.

- FFS: (pre-)configuration of the clock source’s accuracy requirement/threshold or signalling of UE’s clock source accuracy status

- FFS: How to select anchor UEs to mitigate impact of synchronization error

Proposal 7: For SL TDOA, we propose the following procedure as:

- In SL RSTD, a target UE transmits a RSTD request to anchor UE(s). Then, anchor UE(s) replies on the RSTD request by transmitting SL PRS to the target UE.

- In SL RTOA, a target UE transmits SL PRS with RTOA reqeust to anchor UE(s). Then, anchor UE(s) replies on the RTOA request by providing SL RTOA measurement to the target UE.

- FFS: detailed signalling for RSTD and RTOA request and RTOA reporting.

Proposal 8: For SL positioning, the following measurement contents are reported to LMF and UE as:

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| SL-PRS based Rx-Tx measurement | yes | yes |
| SL-PRS based RSTD measurement | yes | yes |
| SL-PRS based RSRP measurement | yes | yes |
| SL-PRS based RSRPP measurement | yes | yes |
| SL-PRS based RTOA measurement | yes | yes |
| SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement | yes | yes |
| LOS/NLOS indicator | yes | yes |
| time stamp | yes | yes |
| SL-PRS resource ID/SL-PRS resource set ID | yes | yes |
| quality of synchronization information (e.g., clock source’s accuracy) | yes | yes |

Proposal 9: Discuss the feasibility of reporting SL positioning measurement by using 2nd stage SCI in RAN1.

Proposal 10: Support request signaling to trigger SL positioning measurement reporting, and study further trigger conditions to satisfy reliability requirement of SL positioning measurement.

Proposal 11: Study further enhancements on latency reduction and overhead reduction for SL positioning measurement reporting.

Proposal 12: Consider for a UE to measure and report the time of arrival of a SL signal relative to time of arrival of a DL reception from a gNB.

Proposal 13: The reliability of the anchor UEs’ location coordinates is indicated to the target UE for SL positioning.

Proposal 14: In SL positioning, a target UE decides whether to perform absolute positioning or relative positioning or ranging depending on availability and quality of measurement source(s).

References

RP-223549, “New WID on Expanded and Improved NR Positioning,” RAN#98-e, Dec., 2022.

"IEEE Standard for Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Peer Aware Communications (PAC)," in IEEE Std 802.15.8-2017, vol., no., pp.1-322, 7 Feb. 2018.

R1-2301952, “Summary #1 on Measurements and reporting for SL positioning,” Moderator (vivo).

Best Microcontroller Projects. Clock accuracy in ppm, https://www.best-microcontroller-projects.com/ppm.html.

R1-2212049, “Discussion on Evaluation for SL Positioning,” Samsung.