3GPP TSG RAN WG1 #112bis-e			R1-2303139

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.5.5

Source:	Samsung

Title:	Discussion on Positioning for RedCap UEs

Document for:	Discussion and Decision

# Introduction

In the newly agreed WID [1], the following objective is included regarding the support of positioning for RedCap UEs

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs)

  - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2].

    - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs.

  - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

This contribution discusses the aspects related to the specification support of positioning for RedCap UEs.

# Hopping pattern design consideration

In last meeting, the following hoping related to agreement have been reached.

Agreement

For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap

- FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG

- FFS: the use of a single or multiple instances of a MGs

- FFS: the use of PPW

Agreement

For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource

- FFS: whether there is specification update needed for RAN1

- FFS: remaining details

Agreement

For RedCap UEs, support SRS for positioning frequency hopping by

- Using a configuration separate from the existing BWP configuration

  - FFS: hopping is configured within a SRS resource or across SRS resources

Basically, the general framework for redcap positioning is clear that, for PRS Rx FH, the measurement gap based method is used; and some additional configuration/indication will be considered to facilitate SRS Tx FH. In the following subsections, we would like to discuss further considerations on the hop design in Redcap Pos.

## MG and/or PPW

The MG was agreed to be used for PRS RX FH. There is open study point on the PPW usage. Considering now one measurement on the complete PRS resource needs several instances, e.g., 5 measurement instances. It’s not desirable to interrupt the reception in between the PRS instances. For this reason, we do not think the PPW without PRS as higher priority is the suitable operation. That’s to say, since the PPW when configured with PRS as higher priority has similar handling for PRS reception as in MG, we think the PPW only with PRS as higher priority could to be used.

Proposal 1: PPW only with PRS as higher priority could to be used for PRS Rx FH.

## FH based measurement report

Impact by RACH related signals

By legacy behavior in MG or the new description for MG-less window (when PRS is higher priority), the UE will suspend most of the other signal/channel reception or transmission except for the interruption of RACH related signals, e.g., msg.1/A, msg3 and msg2/4 reception.

The handling of these interrupting RACH signals is not specified, which could be because these signals may interrupt the measurement of one or two PRS(s) but since the during the MGs, there could be plenty of other PRS for use, such interruption might not be a serious issue. However, for the PRS Rx FH in redcap Pos, the issue will be much more severe. As we discussed above, since one measurement on the complete PRS resource needs multiple instances, e.g., 5 measurement instances, so if one or multiple of these instances are interrupted, it will impact the whole reception and even make the whole reception useless, the UE might need to find another instance group for the complete reception of one PRS. But finding the instance group is also limited by total duration of MG(s). So it could end up with either not enough qualified measurements or quite long measurement period for the positioning which make the final results less useful. Neither of these situations are desired.

Observation 1: the interruption of the RACH related signals has severe damage to the Redcap Pos in PRS Rx FH.

Fig.1 the illustration of impact of RACH related signal

Thus, we think that, first, the impact on the PRS Rx reception should be studied to determine in which condition the FH part of the PRS could be received, e.g., the overlapping with RACH related signal or the RAR widow used for the msg2 reception. Secondly, the reporting should be optionally configured/selected from complete reporting (e.g., PRS resource based) or partial reporting (e.g., PRS FH based). In case UE could finish the complete measurement on the PRS resource, one single report could be used like the legacy. If UE is unable to finish the complete PRS resource, some of the PRS FH reception is interrupted by RACH related signal/channels, UE could still report the measurement on the measured FH part, and UE needs to report the measurement associated with corresponding FH part index, so the receiving node could understand what the measurement refers to.

Proposal 2: RAN1 to study the impact of RACH related signal to the PRS Rx reception FH.

Proposal 3: support FH based reporting and associated FH part index within one complete PRS.

Agreement

For Positioning enhancements for redcap UEs for UL SRS Tx and DL PRS Rx frequency hopping, from the RAN1 perspective, short switching time to allow RF retuning between adjacent hops may be beneficial in terms of accuracy and latency performance.

- Send an LS to RAN4 requesting feedback on the feasible values for the switching time between hops, at least when numerology and bandwidth for each hops can be the same, and the Tx/Rx antennas used in all hops can be the same.

Time Gap

The size of the time gap between hops may also have issue on the coherence time duration (see Annex), e.g., if the gap has surpassed the coherence time, the benefits to have FH could be reduced. Thus the candidate value of the time gap and the applicable condition should also be studied. As RAN1 has send the LS to RAN4 to request the feasible value, the work should proceed after the reply by RAN4.

Proposal 4: a time gap configuration should be considered between hops, FFS the candidate value and applicable condition.

On the other hand, for the applicability of using FH based method to improve the Redcap positioning with smaller bandwidth, the impact of channel timing varying should be handled. For the simulated impact of UE speed, for the given simulation has been submitted, some claims the benefits of having overlapped REs to estimate and compensate the phase offset.

Fig.2 illustration of overlapped F-domain between hops

We think the benefits could be expected from the simulation in case the phase error/offset was added on and without considering not much about another time varying channel property, e.g., amplitude change, which is hard to compensate. Especially when the time gap between two hops is large. Each RE may experience different frequency selective fading, the method of using the overlap REs for phase offset estimation and compensation need more study to check the feasibility and effectiveness.

Proposal 5: FFS how to compensate the impacts on phase offset and/or amplitude change, from the time varying channel property.

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: PPW only with PRS as higher priority could to be used for PRS Rx FH.

Observation 1: the interruption of the RACH related signals has severe damage to the Redcap Pos in PRS Rx FH.

Proposal 2: RAN1 to study the impact of RACH related signal to the PRS Rx reception FH.

Proposal 3: support FH based reporting and associated FH part index within one complete PRS.

Proposal 4: a time gap configuration should be considered between hops, FFS the candidate value and applicable condition.

Proposal 5: FFS how to compensate the impacts on phase offset and/or amplitude change, from the time varying channel property.

# Reference

[1] RP-223549, New WID on Expanded and Improved NR Positioning, Intel

# Annex – coherence calculation

The coherence time and Doppler spread are inversely related, i.e.

Coherence Time ≈ 14×Doppler spread

Table 1 lists the values for the Doppler spread and associated channel coherence time for two frequency bands evaluated for RedCap UE positioning.

Table 1: Doppler Spreads and Coherence Times for RedCap UE positioning

| Carrier Frequency | UE speed | Max Doppler Spread | Channel Coherence Time |
| --- | --- | --- | --- |
| 3.5GHz | 3 km/h | 9.72Hz | 25.7ms |
| 3.5GHz | 30 km/h | 97.2Hz | 2.57ms |
| 3.5GHz | 60km/h | 194.4 Hz | 1.285ms |
| 28GHz | 3 km/h | 77.8 Hz | 3.21 ms |
| 28GHz | 30 km/h | 777.8 Hz | 0.321 ms |
| 28GHz | 60km/h | 1555.6 Hz | 0.161 ms |