3GPP TSG RAN WG1 #112bis-e			R1-2303140

e-Meeting, April 17th – April 26th, 2023

Agenda Item:		9.6.1

Source:			Samsung

Title:			Further UE complexity reduction for eRedCap

Document for:		Discussion and decision

1 Introduction

In RAN Plenary #97e, a new WID on enhanced support of reduced capability NR devices was approved [1]. And following agreements were made in last RAN1 meeting [2]:

Agreement

Revise the earlier agreement by removing the square brackets like this:

- The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306.

- The same value for X is used for DL and UL

Agreement

For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable:

- Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS

For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot:

- Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS

Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number

Agreement

For the earlier RAN1 agreement achieved in RAN1#111 as following,

For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot.

- When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied.

- When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot,

  - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms.

    - FFS: value(s) of X

  - Otherwise, the UE behavior is up to the UE implementation.

- Note: it does not mean early indication is needed

- Note: it will not be used as example for unicast PDSCH

For the “FFS: value(s) of X”

- X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS

- Note: Single Value pair for X is to selected for SCSs

Conclusion

There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell”.

Conclusion

For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR.

Agreement

For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2.

- UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X

Agreement

Update the agreement for PDSCH paging with the clarification as follows:

- From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Agreement

For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

Working Assumption

- For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

  - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

This contribution discusses remaining issues for unicast PDSCH/PUSCH and UE capability report.

2 Discussion

2.1 Remaining issues for unicast PDSCH/PUSCH

For baseband bandwidth reduction, frequency resource assignment is one important aspect with potential specification impact. For unicast channel, including PDSCH and PUSCH, FDRA optimization can be considered, e.g., 5MHz sub-band based frequency resource assignment can be considered to reduce DCI size. Considering that the total PRBs in 5MHz sub-band is less than in a 20MHz BWP, the bit length used for frequency resource assignment can be reduced.  Thus, a 5MHz sub-band can be indicated, and frequency resource assignment can be based on the 5MHz sub-band. Regarding the indication of 5MH sub-band, there are following options:

- One option is to indicate 5MHz sub-band in DCI. For example, BWP can be divided into multiple 5MHz sub-bands based on a predefined pattern, and sub-band index can be indicated in DCI. In this case, same-slot scheduling may impact the degree of complexity reduction on post-FFT data buffering since 5MHz sub-band location cannot be pre-known. However, one benefit is that scheduling gain can be achieved with dynamically indicating to UE any location within 20MHz bandwidth.

- Another option is to indicate 5MHz sub-band by RRC. In this case, the total bit number on frequency resource assignment can be significantly reduced. In addition, even if same-slot scheduling is applied, maximum degree of complexity reduction on post-FFT data buffering can be achieved since 5MHz sub-band location can be pre-known. However, frequency selective gain across 20MHz bandwidth cannot be achieved. Regarding the two options, further discussion is needed.

Proposal #1: For unicast PDSCH/PUSCH, frequency resource assignment can be based on 5MHz sub-band to save DCI overhead. Following options can be considered for 5MHz sub-band indication:

Option.1:	5MHz sub-band is indicated in DCI

Option.2:	5MHz sub-band is preconfigured by RRC

In addition, the RBG size can be considered as one specified for 5MHz instead of one for 20MHz. If the RBG size follows 20MHz BWP, scheduling granularity is very large for 5MHz scheduling bandwidth. For example, assuming rbg-Size configuration 2 is applied, if RBG size follows 20MHz BWP, RBG size is 16 and there are only 1 full RBG within 5MHz for 15kHz SCS. If RBG size follows 5MHz, RBG size is 4 and there are 6 full RBGs within 5MHz for 15kHz SCS. More RBGs can provide finer scheduling.

Proposal #2: For unicast PDSCH/PUSCH, RBG size should follow 5MHz to provide a finer scheduling granularity.

Moreover, frequency hopping within 20MHz should be considered. Within 5MHz bandwidth, frequency diversity gain may be limited considering some cases with special environment, e.g., indoor, factory-like environments. In these cases, the channel delay spread is typically low, so having frequency diversity in a bandwidth larger than 5 MHz is beneficial to maintain coverage. Therefore, at least for unicast PUSCH with the resource allocation spanning a bandwidth of not larger than 5MHz, frequency hopping of 5MHz sub-band should be considered to maintain coverage. Note that, in addition to providing frequency diversity, this solution also averages the interference over the entire 20MHz. This is beneficial if one 5MHz sub-band happens to be experiencing higher loads than the others. For unicast PDSCH, although the resource allocation can be spanned over a bandwidth of more than 5MHz, frequency diversity may still be limited, and frequency hopping can also be considered.

Proposal #3: Support frequency hopping at least for unicast PUSCH.

2.2 UE capability report

There are three possible options for early indication on UE capability:

- Option 1: support separate early indication in Msg1 only

- Option 2: support separate early indication in Msg3 only

- Option 3: support separate early indication in both Msg1 and Msg3

If separate early indication in Msg1 is not supported, scheduling will be very restricted for RAR and Msg3. For example, if the minimum timeline of Msg3 is not relaxed assuming X=0, NW should always schedule RAR within 5MHz for Rel-17 Redcap UE and Rel-18 eRedCap UE. If the minimum timeline of Msg3 is relaxed to NT,1 + NT,2 + 0.5 + X ms, NW should always schedule a long timeline for Msg3 for Rel-17 Redcap UE and Rel-18 eRedCap UE. In addition, NW should always schedule Msg3 with resource allocation spanning a bandwidth of not larger than 5 MHz for Rel-17 Redcap UE or Rel-18 eRedCap UE. These scheduling restrictions may severely impact the random access experience of Rel-17 Redcap UE. If separate early indication in Msg1 is supported, proper scheduling of RAR/Msg3 can be separately applied for Rel-17 Redcap UE and Rel-18 eRedcap UE.

Considering above benefits on scheduling flexibility of RAR/Msg3, separate early indication in Msg1 should be supported. NW can configure a dedicated PRACH resource pool for Rel-18 eRedcap UE via current PRACH resource partitioning mechanism. And, this feature can be optional and up to NW implementation on PRACH resource configuration.

Since separate early indication in Msg1 is an optional feature, separate early indication in Msg3 should be supported as a supplement which is similar to early indication for Rel-17 Redcap UE. Currently, a dedicated LCID is defined for early indicate for Rel-17 Redcap UE. Similarly, a new LCID can be defined for early indication for Rel-18 eRedcap UE. When NW doesn’t enable separate early indication in Msg1, separate early indication in Msg3 should be enabled. Then, all of scheduling restrictions on PDSCH/PUSCH after Msg3 transmission can be applied only for Rel-18 eRedcap UE.

Therefore, option 3 (support separate early indication in both Msg1 and Msg3) is preferred. And, enabling which early indication can be up to NW implementation. For example, network can enable early indication in either Msg1 or Msg3, and network can also enable early indication in both Msg1 and Msg3.

Proposal #4: Support separate early indication in both Msg1 and Msg3.

3 Conclusion

This paper discussed remaining issues for unicast PDSCH/PUSCH, and UE capability report. Based on the discussions, we have the following proposals:

Proposal #1: For unicast PDSCH/PUSCH, frequency resource assignment can be based on 5MHz sub-band to save DCI overhead. Following options can be considered for 5MHz sub-band indication:

Option.1:	5MHz sub-band is indicated in DCI

Option.2:	5MHz sub-band is preconfigured by RRC

Proposal #2: For unicast PDSCH/PUSCH, RBG size should follow 5MHz to provide a finer scheduling granularity.

Proposal #3: Support frequency hopping at least for unicast PUSCH.

Proposal #4: Support separate early indication in both Msg1 and Msg3.

Reference

RP-222675, “New WID on enhanced support of reduced capability NR devices”, Ericsson

Chairman’s notes for RAN1#112 Meeting