3GPP TSG RAN WG1 #112bis-e	        		               R1-2303143

e-meeting, April 17th – 26th, 2023

Agenda item:		9.8.1

Source:			Samsung

Title:			Capacity Improvements for XR

Document for:		Discussion and decision

# Introduction

In RAN1#112, design aspects related to “a CG PUSCH configuration with multiple CG PUSCH transmission occasions within a period of the CG PUSCH configuration”, referred to as “multi-PUSCH CG” for brevity, were discussed and several options for each design aspect were identified [1]. This contribution considers the options for each design aspect of “multi-PUSCH CG”.

# Discussion

# TDRA/FDRA/MCS for “multi-PUSCH CG”

The following options were identified in RAN1#112 for configuring the TDRA for “multi-PUSCH CG”.

Agreement For determination of the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG, the following alternatives for further study: - Alt-A: TDRA determination based on repetition framework. - Alt-A1: Follow the time domain resource mapping of Type A repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The same SLIV in N PUSCH in consecutive slots per CG period - FFS for non-consecutive slots - FFS details, including related RRC parameters - Alt-A2: Follow the time domain resource mapping of Type B repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The SLIV used for 1st PUSCH per CG period. - N consecutive nominal PUSCHs with same duration per CG period - Note: N is not necessarily the repetition factor. FFS details, including related RRC parameters - Alt-B: TDRA determination based on NR-U framework - N and M configured by higher layers - Single SLIV is determined from TDRA. - The SLIV used for 1st PUSCH per CG period. - M consecutive PUSCH TOs with same duration in slot. The M PUSCH TOs are used in N consecutive slots per CG period - Note: N and M are configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. M and N configuration is independent from cgRetransmissionTimer configuration. - FFS details, including related RRC parameters - Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs - Alt-C1: Follow Rel-16 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with k2-r16 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs should be consecutive PUSCH TOs in consecutive slots. - FFS details, including related RRC parameters - Alt-C2: Follow Rel-17 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with extendedK2-r17 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs can be non-consecutive PUSCHs and/or in non-consecutive slots. - FFS details, including related RRC parameters

The justification from the SI for introducing normative work for “multi-CG PUSCH” was to reduce latency for transmissions of first TBs of a video frame by avoiding a delay that is primarily associated with “SR + UL grant”. The benefit over “SR + UL grant” occurred for a rather atypically small PDB of 10 msec for a DDDSU configuration while there was practically no benefit for a more typical PDB of 30 msec. It is noted that “multi-CG PUSCH” does not improve overall system capacity as, unlike DG PUSCH, there are no SE gains from link adaptation. Also, “multi-CG PUSCH” would lead, on average, to substantial resource waste when reserved resources are not used by a UE prior to the generation of a video frame. Therefore, in a potential realization, “multi-CG PUSCH” would be associated with few transmissions, either due to the small PDB for the video frame or due to a serving gNB, after obtaining and processing BSR, reverting to using DG PUSCH for the remaining of the video frame, when any.

Observation 1: Use of “multi-CG PUSCH” is expected to be associated with few transmissions.

The alternatives identified in RAN1#112 are mainly different ways for configuring “multi-CG PUSCH”. Alt-A2 is not appropriate for XR as Type-B repetitions are appropriate for IIoT applications with very small TBs and much smaller PDBs than XR and are not meaningful for XR video frames. Alt-C2 is also not meaningful as the SI objective for minimizing latency relative to “SR + UL grant” would require the PUSCH TOs, or slots with PUSCH TOs, to be consecutive (in valid UL slots) and there is no reason for a gNB to deliberately increase latency while using “multi-CG PUSCH”. Therefore, Alt-A2 and Alt-C2 are not appropriate for the small PDBs or the large frame sizes associated with “multi-CR PUSCH”.

The remaining alternatives (Alt-A1, Alt-B, and Alt-C1) are different realizations for configuring “multi-CG PUSCH” TOs and TDRA. The additional flexibility provided by Alt-C1 is unnecessary particularly when considering that after few CG-PUSCH transmissions, the gNB can obtain BSR and switch to DG-PUSCH to improve spectral efficiency, and that SLIVs are semi-statically configured for a video frame size that is not known in advance and has non-integer periodicity and for a first CG-PUSCH transmission that is also not known in advance due to the jitter associated with video encoder delays. Given that Alt-B has been specified for NR-U and that 3GPP avoids duplicated specifications for a same functionality, there is no reason for Alt-A1 or Alt-C1 (RRC overhead and complexity are also larger) and therefore Alt-B (with same SLIV per CG PUSCH) is default. There is also no need to introduce modifications to the NR-U configuration, such as support of non-consecutive slots, as CG-PUSCH TOs where a UE cannot transmit a CG-PUSCH due to collisions (e.g. DL slots) are handled by existing specifications and there is also no impact from having non-integer periodicity (i.e. the consecutive slots can be assumed to be valid slots).

Proposal 1: Extend the Rel-16 NR-U design using cg-nrofSlots and cg-nrofPUSCH-InSlot to non-shared spectrum to support “multi-CG PUSCH”. No enhancement is necessary.

The following was agreed for the parameters of the “multi-CG PUSCH”.

Agreement For the PUSCHs parameters in a multi-PUSCHs CG configuration, the configuration/indication parameters except MCS and FDRA of CG PUSCHs in a multi-PUSCHs CG configuration are the same - FFS: For MCS and FDRA, study further to decide whether/how to be different. - FFS: Applicability to type-1 and type-2 - Note: TDRA and HP ID are not in this scope of the above statement.

Different MCS or different FDRA (or different TDRA/SLIV) for different CG-PUSCH TOs of a “multi-CG PUSCH” would result to different BLERs for same TBS or to different TBS for same BLER. That would be in a “semi-static” manner, without any knowledge of the number of possible TB retransmissions, or of the video frame size, or even of the CG-PUSCH TOs where the UE would provide the video frame due to jitter associated with the video frame encoder. For example, and by ignoring the jitter, compromising earlier CG-PUSCH TOs by having a larger BLER (e.g. larger MCS or smaller FDRA or smaller TDRA) would lead to a larger probability of retransmissions and revert a possible gain by targeting a lower BLER at later CG-PUSCH TOs as the PDB may not be possible to meet. The same can occur for the later CG PUSCH TOs when a smaller BLER target applies for the earlier CG PUSCH TOs. In general, “multi-CG PUSCH” aims to reduce latency and there is no motivation to have uneven BLER targets for different CG-PUSCH TOs. For improving spectral efficiency, DG PUSCH is always a far superior choice for later TBs (e.g. based on BSR in a first TB) and easier for a gNB to manage than the rather ad-hoc semi-static adjustments of target BLERs for CG-PUSCHs based on different MCS or FDRA (or TDRA).

Proposal 2: TDRA, FDRA, and MCS are same for all CG-PUSCHs of a “multi-CG PUSCH”.

# HPN Determination for “multi-CG” PUSCH

The following options were identified in RAN1#112 for HPN determination for “multi-PUSCH CG”.

Agreement For determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH, consider the following alternatives: - Alt. 1: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period. - Alt 1-1; X = 1 - Alt 1-2: X is the number of configured PUSCHs in a period - Alt 1-3: X is provided by RRC configuration. - FFS details - Alt. 2: Support that UE can decide, as in NR-U, the HARQ IDs for the multiple CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - FFS details - Alt. 3: The HARQ process ID for the configured PUSCHs in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - FFS on potential enhancements different from previous alternatives - Alt 3-1: Note: Same HP ID would be used for all PUSCHs within a period. - FFS details - Alt 3-2: Note: Different HP ID could be used for all PUSCHs within a period. - FFS details - Alt. 4: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS on potential enhancements different from previous alternatives - Alt 5: Support that UE can decide, as in NR-U, the HARQ IDs for the first CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS details - Alt 6: FFS other solutions Note: The case of one TB map to multiple PUSCHs is not considered here.

Alt-2 is the simplest alternative based on NR-U (although there is no need for the UE to indicate HPN as there is no LBT failure in non-shared spectrum) but does not allow a gNB to be aware of the HPN used by a UE for a given CG-PUSCH transmission (and incurs a negligible overhead for XR). However, it is not clear why that is an issue that needs to be addressed. A problem can occur when the UE misses an UL grant for a PUSCH that overlaps with a CG-PUSCH, the TB in the PUSCH and the TB in the CG-PUSCH use same HPN, and the gNB fails to detect the PUSCH DTX (and may incorrectly detect DTX for the CG-PUSCH) for example when the DM-RS of the PUSCH and the DM-RS of the CG-PUSCH are in same symbols and the corresponding FDRAs are similar. However, HARQ buffer corruption from such event has smaller probability than for a typical NACK-to-ACK error (in the DL).

Alt-4 is also a simple alternative based on legacy mechanisms and enables the gNB to deterministically know the HPN for each CG-PUSCH transmission. Whether it is a configured or a valid CG-PUSCH TO is not important (can be for valid).

Alt-5 is a mix of Alt-2 and Alt-4 but there is no benefit for it over Alt-2 or Alt-4. For Alt-3 with same HPN for all CG-PUSCHs within a period, a similar error case as Alt-2 can occur but with larger probability as an UL grant for a TB retransmission in a PUSCH needs to use same HPN as for an initial TB transmission in a CG-PUSCH and is therefore not preferable over Alt-2. For Alt-3 with different HPN for all CG-PUSCHs within a period, one solution can be Alt-4 which is a simple alternative. Alt-1 is unnecessarily complex and may also default to another alternative. Overall, the HPN determination issue is a simple one with existing solutions in the specifications and either Alt-2 or Alt-4 suffice.

Proposal 3: For HPN determination for “multi-CG PUSCH”, apply Alt-2 or Alt-4.

# Definition of UCI indicating unused CG PUSCH transmission occasions

The following was agreed for the UCI indicating unused CG PUSCH transmission occasions.

Agreement The physical channel that carries the UCI that provides information about unused CG PUSCH transmission occasions is CG PUSCH. Agreement Encoding and multiplexing for “the UCI that provides information about unused CG PUSCH transmission occasions” in a CG PUSCH applies encoding and multiplexing procedures for CG-UCI as baseline. - FFS on details Agreement Consider the following alternatives for “the UCI that provides information about unused CG PUSCH transmission occasions” for down-selection or revision - Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI. - FFS on details - Alt. 2: “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI. - FFS on details - Alt. 3: “The UCI that provides information about unused CG PUSCH transmission occasions” replaces/re-purposes some field(s) of the CG-UCI. - FFS on details

There has not been any consideration in the SI or the WI for XR operation in shared spectrum and therefore Alt. 1 is default. If the specifications are to also support XR operation in shared spectrum (that may also be conditioned on using the NR-U mechanism for TDRA configuration), the WI scope needs to be updated. In any case, the present UCI can be added as separate UCI to CG-UCI in a same manner as for HARQ-ACK. In other words, for operation in non-shared spectrum, the UCI can be appended to HARQ-ACK information, if any, prior to encoding and multiplexing (already agreed). For operation in shared spectrum when CG-UCI is also configured, the UCI can be appended to HARQ-ACK information, if any, and to CG-UCI prior to encoding and multiplexing.

Proposal 4: The UCI indicating unused CG PUSCH transmission occasions is a new UCI that is encoded and multiplexed as HARQ-ACK. If operation on shared spectrum is to be supported and if CG-UCI is configured, the UCI is also appended to CG-UCI prior to encoding and multiplexing.

# CG PUSCHs with UCI indicating unused CG PUSCH transmission occasions

The following was agreed for the CG-PUSCHs that include UCI indicating unused CG PUSCH transmission occasions.

Agreement For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, the following options for further down-scoping with possible revision, are considered for the transmission occasion of the UCI: - Option 1: A transmitted CG PUSCH, includes the UCI. - FFS details - Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC. - FFS details - Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion - FFS details - Example of a pre-determined occasion: 1st configured PUSCH TO in a CG period or 1st configured PUSCH TO in a multiple CG periods - Option 4: A transmitted CG PUSCH includes the UCI, if it is transmitted in a transmission occasion determined satisfying given condition(s). - FFS details - Examples of a condition: A first transmitted PUSCH in a CG period, or a first PUSCH transmission within a multiple of CG periods. Other options are not precluded. Proponent companies to provide details.

Option 1 ensures robust delivery of the UCI regardless of jitter and regardless of incorrect decoding. It also allows for the video frame size to vary after the first CG-PUSCH transmission (in that case, the UCI values can change among CG-PUSCH transmissions). There is no issue with overhead as the UCI will be less than 10 bits, e.g. 3-4 bits, while the TB size is in the order of tens of thousands bits.

Options 2-4 fail to account for jitter due to the video frame encoder and fail to support indication of unused CG PUSCH transmission occasions, at least in a timely manner, if the UCI is incorrectly decoded or if the CG-PUSCH transmission is dropped due to any collision resolution procedure. Therefore, if for a CG PUSCH configuration a UE is configured to provide the UCI, restrictions for which CG-PUSCH transmission can include the UCI are neither necessary for overhead purposes nor beneficial for the purposes intended by the functionality of the UCI.

Proposal 5: If a UE is configured to provide the UCI for a CG PUSCH configuration, there are no restrictions on the CG-PUSCH transmissions that include the UCI (Option 1).

# Information of UCI indicating unused CG PUSCH transmission occasions

The following was agreed for the information provided by the UCI indicating unused PUSCH transmission occasions.

Agreement For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the following options for further down-scoping, are considered for the information provided by the UCI: - Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused” - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain. - Applicable numbers can be determined from information obtained from configuration. - FFS details - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain) - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - FFS whether/how the unused TO(s) can be associated to multiple CG configuration. - Other options are not precluded. Proponent companies to provide details.

Option 2 involves a bitmap and therefore implies that some of the consecutive CG PUSCH TOs will not be used for CG-PUSCH transmission. That is against the “minimize latency” rationale for introducing “multi-CG PUSCH” and a gNB has no reason to not switch to DG-PUSCH in order to improve spectral efficiency. There is also no reason for a UE to indicate gaps in CG PUSCH TOs with CG PUSCH transmissions and prolong the delivery of a video frame, particularly since TB retransmissions may also be needed. It is noted that for TDD operation, a bitmap is unnecessary as there is no ambiguity between the gNB and the UE of which consecutive CG-PUSCH TOs will have dropped CG-PUSCH transmissions – e.g. the indication can be either for CG-PUSCH TOs in valid slots or for all consecutive CG-PUSCH TOs while accounting for ones with dropped CG-PUSCH transmissions due to Rel-17 collision resolution.

In Option 1-2 the UE indicates a time duration that includes the consecutive TOs, while in Option 1-1 the UE indicates the number of consecutive TOs.  A motivation for Option 1-2 is unclear as it would generally require more bits (e.g. if the time duration is a number of slots that is a superset of the consecutive TOs) and does not provide any additional information to the gNB. It is noted that Option 1-1 is a form of L1 BSR as, based on the FDRA/TDRA/MCS and the indicated number of PUSCH TOs, the gNB can derive the video frame size, determine the PDB, and accordingly adjust scheduling including for TB retransmissions or for switching to DG-PUSCH. As the UE does not have any information for a next CG period corresponding to another video frame, the indication is applicable to a single CG period.

Proposal 6: The UCI provides the number of consecutive CG-PUSCH TOs (Option 1-1). The indication is applicable to a single CG-PUSCH period.

For the FFS of the above agreement, it is unclear whether the FFS is for the indication of unused CG-PUSCH TOs to be for multiple CG configurations, or whether the FFS is for the indication of unused CG-PUSCH TOs for a first CG PUSCH configuration to (also) be provided by CG-PUSCHs associated with a second CG PUSCH configuration. For the former interpretation, no such use case has been identified in the SI and the WID. For the latter interpretation, XR-related use cases and corresponding functionality need to first be identified. Even though the WID is not explicit, the discussions/decisions from the SI were based on the indication for unused CG-PUSCH TOs of a CG-PUSCH configurations to be included in CG-PUSCH transmissions for the CG-PUSCH configuration.

Observation 2: There is no need to support indication of unused CG-PUSCH TOs for multiple CG configurations.

Observation 3: XR-related use cases and benefits need to first be concluded for a decision to support indication of unused CG-PUSCH TOs for a first CG-PUSCH configuration in CG-PUSCH transmissions of a second CG-PUSCH configuration.

# Other topics

Several other topics were briefly discussed:

- One topic was the issue addressed by “Observation 2” above.

- Another topic was a need for a timeline but that is unnecessary as a gNB is aware of the requirements for UE processing timelines and specifically of the PUSCH preparation timeline.

- There is no reason to consider PUSCH skipping as such event will practically never occur for video frames (other than the indication for unused CG-PUSCH TOs).

- Providing a single TB over multiple CG-PUSCHs may be beneficial for improving coding gains for very small individual TBs (and for some TB CRC overhead reduction) but that is not the case for the very large TBs associated with XR while TB retransmissions will be practically impossible to handle and a resulting TB size may be larger than what a UE can currently support.

- Re-transmission of multiple TBs by a single DCI was effectively discussed during the SI and is not supported.

- CBG retransmission for multiple CG PUSCHs was discussed during the SI and is not supported. Although in principle a beneficial enhancement, the low target BLER required to meet PDB would limit any potential gain.

- There is no need to support repetitions as the PDB cannot be met in such cases.

- Overriding a previous indication for unused CG-PUSCH TOs can be possible via providing UCI in every CG-PUSCH occasion where a later indication can be different from an earlier indication.

One aspect that requires resolution for completeness of the specifications is the collisions of CG-PUSCHs associated with different CG PUSCH configurations (video and pose/control) which, due to the different periodicities, cannot be avoided. A similar issue exists for SPS PDSCHs in Rel-16 and a similar mechanism as for resolving collisions among SPS PDSCHs of different SPS PDSCH configurations can apply.

Proposal 7: Extend the collision resolution procedure for SPS PDSCHs to CG-PUSCHs.

# Conclusions

This contribution considered aspects for supporting “multi-CG PUSCH” and proposes the following.

Proposal 1: Extend the Rel-16 NR-U design using cg-nrofSlots and cg-nrofPUSCH-InSlot to non-shared spectrum to support “multi-CG PUSCH”. No enhancement is necessary.

Proposal 2: TDRA, FDRA, and MCS are same for all CG-PUSCHs of a “multi-CG PUSCH”.

Proposal 3: For HPN determination for “multi-CG PUSCH”, apply Alt-2 or Alt-4.

Proposal 4: The UCI indicating unused CG PUSCH transmission occasions is a new UCI that is encoded and multiplexed as HARQ-ACK. If operation on shared spectrum is to be supported and if CG-UCI is configured, the UCI is also appended to CG-UCI prior to encoding and multiplexing.

Proposal 5: If a UE is configured to provide the UCI for a CG PUSCH configuration, there are no restrictions on the CG-PUSCH transmissions that include the UCI (Option 1).

Proposal 6: The UCI provides the number of consecutive CG-PUSCH TOs (Option 1-1). The indication is applicable to a single CG-PUSCH period.

Proposal 7: Extend the collision resolution procedure for SPS PDSCHs to CG-PUSCHs.

In addition, the following observations are made.

Observation 1: Use of “multi-CG PUSCH” is expected to be associated with few transmissions.

Observation 2: There is no need to support indication of unused CG-PUSCH TOs for multiple CG configurations.

Observation 3: XR-related use cases and benefits need to first be concluded for a decision to support indication of unused CG-PUSCH TOs for a first CG-PUSCH configuration in CG-PUSCH transmissions of a second CG-PUSCH configuration.

# References

- R1-2301902, “Moderator Summary#3 - XR Specific Capacity Improvements”, Moderator (Ericsson)