3GPP TSG RAN WG1 #112bis-e	        		               R1-2303144

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.9.1

Source: 	Samsung

Title: 	On coverage enhancement for NR NTN

Document for:	Discussion and Decision

Introduction

In RAN1#112, it was discussed on coverage enhancement for NR NTN, and related results are captured as follows.

Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g. DMRS ports Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. - Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design) Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

In this contribution, it discusses on PUCCH enhancements for Msg4 HARQ-ACK and DMRS bundling for PUSCH, respectively, in case of LEO-1200&LOS with parameter set-1 which was captured in the revised WID as reference scenario.

PUCCH enhancements for Msg4 HARQ-ACK

Definition of PUCCH enhancements

In past meetings, there was a discussion on whether PUCCH enhancement can include only Msg4 PUCCH or other cases such as PUCCH transmission before RRC connection. Although “Msg4 PUCCH” is defined in WID, “Msg4” itself is not defined in the specifications and the PDSCH providing contention resolution is not necessarily the first PDSCH after Msg3 PUSCH is correctly received. Also, the issue is applicable in general until a UE is provided dedicated PUCCH resources which is an optional configuration (i.e. a NW is not required by the specifications to provide dedicated PUCCH resources to a UE).

Proposal 1: Support of PUCCH repetitions in Rel-18 NTN is about the case where a UE does not have dedicated PUCCH resources.

Support PUCCH format 1

For now, for simplicity, reference will be to “Msg4 HARQ-ACK” under the assumption that repetitions are supported for any PUCCH with HARQ-ACK when a UE does not have dedicated PUCCH resources. Regarding PUCCH format 1, in RAN1#110, it was observed that PUCCH format 1 can meet the performance requirement with proper repetition level and frequency hopping with the assumption of 2-bit payload.

Proposal 2: Support at least PUCCH repetition for PUCCH format 1.

Repetition request/report

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g. DMRS ports

Regarding options for repetition request/report, option A is preferred with the following reasons:

- Option B and C require further gNB blind detections.

- Option B and C require larger specification impact(s) (possibly required for other working groups)

- For option A, the partitioning issue can be avoided by gNB implementation such as joint configuration (RACH resources associated with multiple features) and/or making small spot beam to minimize SNR variation.

Proposal 3: Support Option A for Msg 4 PUCCH repetition request.

Dynamic repetition indication

Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. - Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design)

Alt. 1 is the cleanest solution to provide dynamic PUCCH repetitions. For Alts 2 and 4, it cannot be applicable for UEs only supporting Msg 4 HARQ-ACK repetitions. Therefore, other complementary solution(s) are still needed. For Alt. 3, it requires additional UE processing and additional resources. Also, it is likely to increase false alarm probability compared to legacy behavior since it is similar to introduce new RNTI for special purpose (i.e., indicating repetition number using CRC bits).

Proposal 4: Support Alt. 1 for dynamic repetition indication.

RSRP threshold

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

Regarding the determination of RSRP threshold, Alt. B is slightly preferable since it is more flexible and Alt. B can provide Alt. A by gNB implementation.

Regarding FFS on value of X, it should not be a configurable value in order to avoid different UE behaviors depending on the relative values of two parameters set by gNB for the same feature. Thus, we prefer to have the same value (i.e., 127) as for the Msg.3 PUSCH repetition threshold.

Proposal 5: Support Alt. B for RSRP threshold.

Proposal 6: Value of X should be 127.

DMRS bundling for PUSCH

Feasibility of pre-compensation for DMRS bundling

RAN1#112 made following observation.

Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit

There were lengthy discussions on the impact of phase difference limit, but there is no consensus on how to capture as an observation. We think that sending RAN4 LS is the only way to move forward because there has been no RAN4 requirement(s) specified for NTN-specific DMRS bundling.

With the assumption that NTN-specific DMRS bundling is needed, it should be discussed whether the UE can do pre-compensation of TA value or not during a DMRS bundling window before discussing events that can cause power consistency and phase continuity not to be maintained. Although it is unlikely to do DMRS bundling if UE applies same TA value over PUSCH repetition in NTN, it is not clear whether DMRS bundling can be applicable if TA pre-compensation is applied since this information is purely based on UE implementation. For example, if two consecutive PUSCH transmissions cannot maintain phase continuity due to 1) large delay difference and 2) large phase difference than the requirement, it is not clear whether phase difference after TA pre-compensation can be within the maximum allowable phase difference for DMRS bundling. Therefore, we need to send LS to RAN4 in order to check whether the issue is negligible or not.

Proposal 7: Sends LS to RAN4 asking the followings.

- The feasibility of DMRS bundling for NTN when the UE applies TA pre-compensation.

- Maximum allowable phase difference for NTN-specific DMRS bundling and related assumptions

DMRS bundling condition

In RAN1#111 meeting, candidate schemes to support NTN-specific DMRS bundling with the following options were discussed.

Proposal 2-2_v4 For NTN-specific PUSCH DMRS bundling, - Down-select from the following options of applying time/frequency compensation update with DMRS bundling - Option 1: Actual/Nominal TDW is determined based on when UE performs pre-compensation update causing phase discontinuity and power inconsistency - Option 1-1: UE reports information regarding when UE performs pre-compensation update causing phase discontinuity and power inconsistency - FFS: details of information report - Option 1-2: NTN-specific configuration/indication is provided regarding when UE can perform pre-compensation update causing phase discontinuity and power inconsistency - FFS: details of configuration/indication - Option 2: Actual/Nominal TDW is determined as in the existing specification and UE can perform pre-compensation update causing phase discontinuity and power inconsistency only at boundaries of two adjacent actual TDWs - Note: No NTN-specific configuration/indication to determine actual/nominal TDW is assumed

It is premature to discuss detailed solutions since the feasibility study of DMRS bundling for NTN has not been concluded yet. Specifically, it is not clear which aspects of current specifications cannot support proper DMRS bundling for NTN service. Thus, problem statement should be clear before discussing any new NTN-specific design.

Proposal 8: RAN1 to identify and justify the problem statement for supporting NTN-specific DMRS bundling based on existing specifications.

Regarding the options in the above table, all options are assuming that UE should maintain phase continuity and power consistency during an actual TDW. With this understanding, the motivation of option 1-1 is not clear because

- why/how reporting such information is beneficial compared to existing method,

- this information is needed before performing DMRS bundling and it degrades UL performance loss, and

- Rel-17 Coverage provides similar UE capability such as how many slots the UE can support for DMRS bundling.

For option 1-2, it assumes that gNB configures NTN-specific timeline so that DMRS bundling can be performed during the timeline. However, it is not clear why this timeline is necessary since existing DMRS bundling window can be controlled by gNB scheduling. For example, before applying actual TDW, nominal TDW size (for PUSCH repetition type A and PUSCH repetition type B) can be determined by gNB. Thus, it is understood that current specification already provides full flexibility of DMRS bundling window. With this understanding, the motivation option 1-2 is not clear.

Observation 1: Rel-17 coverage enhancement design for DMRS bundling provides fully flexible window size for DMRS bundling.

Conclusions

The following observations and proposals have been made for coverage enhancement of NR NTN.

Observation 1: Rel-17 coverage enhancement design for DMRS bundling provides fully flexible window size for DMRS bundling.

Proposal 1: Support of PUCCH repetitions in Rel-18 NTN is about the case where a UE does not have dedicated PUCCH resources.

Proposal 2: Support at least PUCCH repetition for PUCCH format 1.

Proposal 3: Support Option A for Msg 4 PUCCH repetition request.

Proposal 4: Support Alt. 1 for dynamic repetition indication.

Proposal 5: Support Alt. B for RSRP threshold.

Proposal 6: Value of X should be 127.

Proposal 7: Sends LS to RAN4 asking the followings.

- The feasibility of DMRS bundling for NTN when the UE applies TA pre-compensation.

- Maximum allowable phase difference for NTN-specific DMRS bundling and related assumptions

Proposal 8: RAN1 to identify and justify the problem statement for supporting NTN-specific DMRS bundling based on existing specification.