3GPP TSG RAN WG1 #112bis-e	R1- 2303146

# e-Meeting, April 17th – April 26th, 2023

Agenda Item:		9.9.3

Source:				Samsung

Title:					Disabling of HARQ feedback for IoT NTN

Document for:	Discussion

# Introduction

A WID on further enhancements for NB-IoT/eMTC in non-terrestrial networks (NTN) was approved in RAN#92-e [1]. One objective is to specify disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1, RAN2].

Good progress was achieved in RAN1#112 [2]. This contribution discusses some remaining issues for disabling HARQ feedback.

# Discussion

Signaling for disabling HARQ feedback

In RAN1#112 [2], the following WA was agreed:

Working assumption For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission: Support Option 1 in case only per-HARQ process bitmap signaling is configured - Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured - Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes. - FFS #2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration. RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”. For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

In the above,

Option 1: per HARQ process via UE specific RRC signaling

Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field)

FFS#1 – The main purpose of introducing this feature for IoT NTN is to disable the HARQ-ACK feedback as specifically stated in the WID objective. In addition, it is not clear the need for using the DCI-based signaling mechanism to enable a semi-statically HARQ feedback disabled process.

In RAN1#112, another related agreement states that the selection among the following alternatives:

- Alternative 1: applies to both semi-statically HARQ enabled and disabled processes

- Alternative 2: only applied to semi-statically HARQ disabled processes

- Alternative 3: only applied to semi-statically HARQ enabled processes

should be done based on the criteria of DCI overhead, PDCCH monitoring/power consumption, HARQ timer, impact on scheduling flexibility, UE implementation complexity.

Thus, considering the above, it seems sufficient that the DCI-based signaling is applied to disable semi-statically HARQ feedback enabled processes.

Proposal 1: The DCI-based signaling is applied to disable semi-statically HARQ feedback enabled processes.

FFS#2 – Whether/how to support the DCI-based signalling overriding Option 3 for transmission of multiple TBs scheduled by single DCI can be discussed after the design of the DCI scheduling a single TB is complete.

Proposal 2: The discussion on whether/how to support the DCI-based signalling overriding Option 3 for transmission of multiple TBs scheduled by single DCI is postponed until after the design of the DCI scheduling a single TB is complete.

DCI design

Regarding the indication in the DCI format, it was agreed in RAN1#112 to down select between adding one field in the DCI format and reusing/reinterpreting an existing field in DCI based on the criteria of DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc. Although adding a new 1-bit field in DCI seems to be a clean solution, it adds an overhead that may not be justified for such simple feature, and creates the need for further DCI design that would have a larger specification impact and also increase UE complexity. Thus, if a simple solution with minimal specification impact can be found by reusing existing fields, it should be preferred.

Number of HARQ feedbacks to disable

The number of HARQ processes for NB-IoT can be 1 or 2, hence the benefits of disabling HARQ-ACK reports for NB-IoT might be smaller than for eMTC or NR. Whether to allow disabling the HARQ-ACK reports for the case of single HARQ process can be further discussed. Also whether to allow disabling HARQ-ACK reports for all processes when more than one HARQ process is configured can be further discussed. A disadvantage of disabling HARQ-ACK reporting is an inability of the gNB to perform open-loop link adaptation and also having to rely on costly higher layer ARQ for retransmissions and/or higher PDSCH reception reliability while an advantage can be some UE power savings. That can be left to gNB implementation and enabling/disabling all/some of the HARQ-ACK reports for PDSCH receptions would be a gNB choice.

Proposal 3: For IoT NTN, enabling/disabling HARQ feedback for downlink transmission is supported for single HARQ process and for multiple HARQ processes.

Potential enhancements to assist the network for enabling/disabling HARQ feedback

Another aspect to consider is to report additional information by a UE in order to assist the network in configuring enabling/disabling HARQ-ACK reporting. As discussed during the Rel-17 SI, the additional information can be signaling that a sufficient number of repetitions has already been transmitted, requesting a number of repetition, reporting buffer status for HARQ operation, explicit indication to request enabling/disabling HARQ reports, etc.

Proposal 4: Further discuss the reporting of additional information by the UE.

# Conclusion

This contribution discusses remaining aspects related to timing relationship enhancements. Proposals made in this contribution are summarized below.

Proposal 1: The DCI-based signaling is applied to disable semi-statically HARQ feedback enabled processes.

Proposal 2: The discussion on whether/how to support the DCI-based signalling overriding Option 3 for transmission of multiple TBs scheduled by single DCI is postponed until after the design of the DCI scheduling a single TB is complete.

Proposal 3: For IoT NTN, enabling/disabling HARQ feedback for downlink transmission is supported for single HARQ process and for multiple HARQ processes.

Proposal 4: Further discuss the reporting of additional information by the UE.

# References

- RP-220979, WID on IoT NTN enhancements, RAN#95-e, March 2022.

- RAN1 Chair’s Notes, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023