3GPP TSG RAN WG1 #112bis-e	        		             R1-2303149

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.10.2

Source: 	Samsung

Title: 	Candidate cell TA acquisition for NR L1/L2 mobility enhancement

Document for:	Discussion and Decision

Background

In RAN1 #112 meeting, for candidate cell TA acquisition, two scheme, PDCCH-ordered PRACH and UE-based TA calculation are supported. It needs further clarified how UE receives RAR for candidate cell TA acquisition, and also needs conclusion whether other schemes can be supported for candidate cell TA calculation or acquisition. Detailed design of signaling is also need. In this contribution, we discuss remained details on candidate cell TA issue, and suggest our proposals.

Agreemment For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell. Note: the detailed signalling is left to RAN2 Agreement The PDCCH order from the source cell contains the indication of candidate cell. - The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity Agreement For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated - If reception of RAR is not configured/indicated (without RAR) - TA value of candidate cell is indicated in cell switch command - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order - If reception of RAR is configured/indicated (with RAR), FFS - whether RAR is received from serving cell or candidate cell - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE - content of RAR - FFS: signaling for configuration/indication of whether RAR needs to be received - UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM - Send LS to RAN2 and RAN3 to check the feasibility about this agreement - Note: Definition of candidate cells is up to RAN2 Agreement - For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported Agreement on whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives. - Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) - Alt 2: UE autonomous Re-transmission of PRACH is allowed, - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax Agreement If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell. - The maximum number of TA values memorized by UE is a UE capability - FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR Agreement Whether RAR needs to be received is configured by RRC. Agreement study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured - Whether gap between the DCI and PRACH longer than timeline defined in spec is needed - Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

Discussions

A strong consensus was observed at CBRA as way to manage candidate cell specific TA. One small concerns was observed whether there is any restriction at gNB configuring the dedicated resource for this specific RACH.

Proposal 1: RACH for candidate cell RA is only via pre-configured dedicated RACH resource

- UE would not expect overlapped resource with PRACH of other purpose.

We assume RAR as valid tools sharing candidate cell TA value, when TA acquisition is done by PRACH transmission. To avoid UE’s additional operation monitoring candidate cell, RAR should be received via the same resource UE is relying on for the reception of serving cell PDCCH. RAR transmission from serving cell should be considered as baseline. But in limited cases, RAR transmission from candidate cell can be allowed if additional PDCCH monitoring is not required. For example, if DL synchronization is possible between serving cell and candidate cell, UE would be possible to receive RAR from candidate cell if that RAR is received via serving cell CSS.

Proposal 2: Support RAR reception via PDCCH resource configured for serving cell, e.g., CSS Tye-1, as a way to share candidate cell TA value with UE

- RAR transmission via candidate cell is allowed if additional PDCCH monitoring is not required.

For further details, we don’t see any dedicated information should be loaded on RAR for the purpose of candidate cell TA indication. UE would be able to recognize whether received RAR is for regular purpose or for candidate cell TA acquisition. In addition, we don’t think PRACH toward multiple of candidate cell will be triggered by serving cell, since only up to 2 TA per UE is agreed.

Proposal 3: Serving cell does not indicated candidate cell ID in RAR.

We expect RACH-based TA acquisition as a concrete solution working in various scenarios. But in a case UE is configured with multiple of candidate cells, e.g., a UE with a high mobility or a UE in a dense network, for further overhead reduction, RACH-less solution can be considered.  In synchronized cell, UE oriented TA, e.g., LTE like solution would work well and may no need for further discussion required for the feasibility check. In other cases, it would be a natural expectation that UE oriented TA provides rough UL timing for limited usecases while further refinement can be done via RACH, if needed.

Proposal 4: RACH-less TA acquisition is supported for candidate cell

- At least LTE-like UE oriented TA, i.e., UE oriented TA in synchronized network is supported.

For the association of TA value to candidate cell, RAN1 made 2 alternatives as below:

Agreement (RAN1 #110b) For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell: - Alt1: Associate TA/TAG and candidate cell implicitly, e.g., - the association between TA/TAG and TCI states can be configured - Alt2: Associate TA/TAG and candidate cell explicitly, e.g., - the association is provided as a part of candidate cell(s) configuration - the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

We see both of alternative as possible solution, but first alternative as familiar solution aligned with recent RAN1 design. With alternative 1, candidate cell TA management is associated with the framework of beam management or UL transmission condition indication. Candidate cell TA information is provided via serving cell RRC or MAC CE. In other words, candidate cell TA value updates without reconfiguration of candidate cell RRC. It helps simplified operation when frequent candidate cell changing or large number of candidate cells are considered. With the second alternative, as captured in two sub-bullets, TA value is included in candidate cell RRC configuration with the association to SSB or TRS. We wonder whether alternative 2 assumes UE to refer candidate cell RRC configuration before the cell switching. For example, if PDCCH order PRACH transmission is supported for candidate cell TA acquisition, UE needs information on candidate cell SSB and PRACH resource. We prefer this information achieved by UE without referring candidate cell RRC, and regard alternative 2 as duplicated configuration without necessity.

In addition, for the management of TRP specific TA, RAN1 made agreement as below:

Agreement (RAN1 #112) For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel-15/16 spatial relation framework is used for - PUCCH - DG/CG Type 1/Type 2 PUSCH - AP/SP/P SRS

Considering the similarity of inter-cell MTRP operation and LTM, we propose to keep unified solution between two scenarios.

Proposal 5: Support unified solution for the TA information sharing with UE for the case of TA per TRP and TA per cell

- Support association between TA of candidate cell and configured/activated TCI state.

Conclusions

In this contribution, we provide following observations and propose proposals as below:

Proposal 1: RACH for candidate cell RA is only via pre-configured dedicated RACH resource

- UE would not expect overlapped resource with PRACH of other purpose.

Proposal 2: Support RAR reception via PDCCH resource configured for serving cell, e.g., CSS Tye-1, as a way to share candidate cell TA value with UE

- RAR transmission via candidate cell is allowed if additional PDCCH monitoring is not required.

Proposal 3: Serving cell does not indicated candidate cell ID in RAR.

Proposal 4: RACH-less TA acquisition is supported for candidate cell

- At least LTE-like UE oriented TA, i.e., UE oriented TA in synchronized network is supported.

Proposal 5: Support unified solution for the TA information sharing with UE for the case of TA per TRP and TA per cell

- Support association between TA of candidate cell and configured/activated TCI state.

- References

[1] RP-222332 “Revised WID on Further NR mobility enhancements”, 3GPP RAN#97e, e-Meeting, September, 2022.

[2] R1-2208331 (R2-2209257) “LS on L1/L2-based inter-cell mobility”, 3GPP RAN2#119, e-Meeting, October, 2022

[3] R1-2209755, “Non-serving cell TA management for NR L1/L2 mobility enhancement”, 3GPP RAN1 #110bis-e, October, 2022

[5] R1-2212069, “Candidate cell TA acquisition for NR L1/L2 mobility enhancement”, 3GPP RAN1 #111, November, 2022

[6] Chair note, 3GPP RAN1 #112, March 2023