3GPP TSG RAN WG1 #112bis-e	R1-2303155

# e-Meeting, April 17th – April 26th, 2023

Agenda Item:		9.12.3

Source:				Samsung

Title:					Dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion

# Introduction

In RAN#96 a Rel-18 WID on Further NR coverage enhancements [1] was approved. One objective is “Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)”.

Agreements for the signaling and applicability of the dynamic indication are listed below.

Agreement Dynamic waveform switching enhancement in R18 is only applicable to PUSCH channel. Agreement Dynamic waveform switching enhancement in R18 is applicable to PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1. Note: The above does not imply that dynamic switching enhancement in R18 is applicable or not applicable to other cases of PUSCH (e.g. PUSCH transmission with a Type 1 or Type 2 configured grant, PUSCH scheduled by DCI format 0_0). Agreement For DCI based solution, - For supported dynamically scheduled PUSCH, support dynamic waveform switching indication from UL scheduling DCI Note: “Supported dynamically scheduled PUSCH” is to be confirmed in further discussion Note: It does not imply that the waveform switching indication applies to other transmission or not - Indication from non-UL scheduling DCI is not supported. Agreement For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. Note: no change of the current size alignment procedure between UL DCI and DL DCI. Agreement Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant. Conclusion There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18. Conclusion The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

This contribution discusses remaining issues to support dynamic switching between DFT-S-OFDM and CP-OFDM.

# Discussion

DCI size alignment

It was agreed that dynamic waveform indication applies to a PUSCH scheduled by a DCI format 0_1 or 0_2 with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1. Some fields of the DCI format have sizes that depend on the waveform, and normally the size is larger for CP-OFDM compared with the size of the corresponding field for DFT-S-OFDM. The waveform indication in the DCI format needs to be before any of the fields with size depending on the waveform so that the UE can first determine the indicated waveform and then the size of the subsequent fields. The size of the DCI format that includes the waveform indication needs to be the same when the DCI format indicates DFT-S-OFDM or CP-OFDM otherwise a total number of sizes of DCI formats would exceed a predetermined maximum number of sizes of DCI formats (DCI format budget size). Thus, the size of the DCI format that includes the waveform indication should be set to the largest size between the two sizes corresponding to the two waveforms, and padding bits are added when needed.

For a PUSCH scheduled by a DCI format 0_1 or 0_2 with CRC scrambled by C-RNTI, the DCI size alignment can be done by adding padding bits per DCI format or per field, when needed.

If padding bits are added per DCI format, a number of bits with value set to '0' are added at the end of the DCI format until the size of the DCI format equals the size of the DCI format with the largest bit widths (among the bit widths required for DFT-S-OFDM and CP-OFDM) for fields with bit width depending on the waveform.

If padding bits are added per field, a number of most significant bits with value set to '0' are inserted to a field in DCI format until the bit width equals that of the field when the waveform is the one that requires the largest number of bits for that field. The UE assumes that the bit width of the field is equal to the largest bit width among the bit widths for DFT-S-OFDM and CP-OFDM, which may or may not be the same bit width corresponding to the configured waveform.

The waveform indication can be included in a DCI format scrambled by CS-RNTI with NDI =1, and the size alignment between the DCI format scrambled by CS-RNTI and the DCI format scrambled by C-RNTI is done according to the procedure in TS 38.212, 7.3.1.1.2 as follows: “A UE does not expect that the bit width of a field in DCI format 0_1 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_1 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_1 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell.”

It is assumed that the waveform indication field of a DCI format scrambled by C-RNTI or CS-RNTI precedes fields with bit widths depending on the waveform and that the waveform indication field precedes the NDI field in a DCI format scrambled by CS-RNTI. If the waveform field is present in the DCI format with CS-RNTI, the value of the NDI bit would be 1 (NDI=0 is an error case).

- If CP-OFDM is configured, and DFT-S-OFDM is indicated in the DCI format

- Padding per DCI format

  - For C-RNTI: UE reads the waveform indication, and assumes that the sizes of fields in the DCI format with C-RNTI are according to the indicated waveform (smaller size) and that padding bits are added at the end of the DCI format

  - For CS-RNTI: UE reads the waveform indication, then reads the NDI bit positioned after fields, if any, depending on the waveform and with sizes according to the indicated waveform

- Padding per field

  - For C-RNTI: UE reads the waveform indication, and assumes that the sizes of the fields in the DCI format with C-RNTI are according to the configured waveform (larger size) and that padding bits are added for the MSB(s) of such field of the DCI format

  - For CS-RNTI: UE reads the waveform indication, then reads the NDI bit positioned after fields, if any, depending on the waveform and with sizes according to the configured waveform

- If DFT-S-OFDM is configured, and CP-OFDM is indicated in the DCI format

  - For C-RNTI: UE reads the waveform indication and assumes that the sizes of the fields in the DCI format with C-RNTI are according to the indicated waveform (larger size), no padding is needed

  - For CS-RNTI: UE reads the waveform indication, then reads the NDI bit positioned after fields, if any, depending on the waveform and with sizes according to the indicated waveform

From the analysis above, both a) padding per DCI format and b) padding per field for DCI size alignment of a DCI format 0_1 or 0_2 scrambled by C-RNTI are feasible. DCI size alignment for CS-RNTI is done as per existing procedure.

If the presence of the waveform indication in the DCI format scrambled by CS-RNTI depends on whether the NDI field is 1 (waveform indication field is present) or 0 (waveform indication field is not present), then the NDI field can be before the field that includes the waveform indication.

Observation 1: For DCI size alignment of a DCI format 0_1 or 0_2 scrambled by C-RNTI that includes a waveform indication field, when needed, both padding per DCI format and padding per field are feasible.

Observation 2: The DCI size alignment of a DCI format 0_1 or 0_2 scrambled by CS-RNTI that includes a waveform indication field is done according to the existing procedure.

Proposal 1: The field of a DCI format 0_1 or 0_2 scrambled by C-RNTI or CS-RNTI that includes the waveform indication precedes any field of the DCI format that has a bit width depending on the waveform.

Proposal 2: The field of a DCI format 0_1 or 0_2 scrambled by CS-RNTI that includes the waveform indication precedes the NDI field.

Whether the DCI format scheduling the PUSCH transmission includes a field for the dynamic waveform indication depends on a configuration. For example, an RRC parameter for transform precoder can be set to {enabled, disabled, dynamic}, and the field dynamic can also indicate “DFT-S-OFDM” or “CP-OFDM”.

Proposal 3: Whether the dynamic indication field is present in a DCI format is configured by a higher layer parameter, e.g., transformPrecoder can indicate the configured waveform and whether the waveform is dynamically indicated.

Applicability of dynamic waveform switching to Msg3 PUSCH

For Msg3 PUSCH, the need for a dynamic waveform switching is questionable because the gNB may not be aware of UE experiencing different channel conditions and of changes to its operating SINR. In addition, there is the difficulty that the UE capability of supporting dynamic waveform switching for Msg3 PUSCH is not known to gNB before UE is in connected mode, and designing the system to ensure that the gNB knows the UE capability during initial access would require an additional partition of PRACH resources and a large specification impact, which are both not desirable for the simple feature of dynamic waveform switching.

Proposal 4: Indication of dynamic waveform switching for Msg3 PUSCH is not supported.

Assistance information for switching waveform

A UE determines the power headroom report (PHR) for an activated serving cell based on a reference PUSCH transmission that uses the configured waveform. It was suggested that for a PHR to be more useful to a gNB, the PHR could provide information corresponding to the actual waveform and to the other candidate waveform (e.g., UE can provide PHR values for both waveforms, or PHR value for one waveform and the difference between the two PHR values). For the calculation of the PHR, either the scheduled PUSCH transmission or a reference PUSCH transmission can be used. In addition, other enhancements for the triggering of the PHR and for the UE to request to switch waveform were also discussed in past RAN1 meetings.

A benefit on system/UE throughput from a gNB having a PHR for a candidate waveform prior to switching, versus the gNB assuming a baseline MPR difference for the two waveforms and obtaining, if needed, a PHR at a later time is not clear. Further, such enhancements would have a large impact to the specifications (also for RAN2 specifications) and this seems not justified for a simple feature as dynamic waveform switching. Moreover, the WID objective does not require to introduce enhancements for the PHR as such enhancements are not relevant to dynamic waveform switching and could be as applicable to RRC-based waveform switching.

Proposal 5: Enhancements to mechanisms for PH report and triggering are not further discussed.

Additional scheduling timeline for switching waveform

For UL dynamic scheduling, there is a timeline requirement to ensure UE has enough time to prepare the PUSCH. The time distance between the ending of the scheduling PDCCH and the starting of PUSCH should be no less than Tproc,2. For dynamic waveform switching, UE may need additional timeline to prepare the PUSCH depending on UE capability, it is then necessary to study the impact on scheduling timeline due to dynamic waveform switching.

Proposal 6: RAN1 to study whether additional timeline is needed for UL scheduling in case of dynamic waveform switching.

UCI multiplexing

DFT-S-OFDM is used for coverage limited scenario while CP-OFDM is used in good coverage scenario. If a PUCCH transmission overlaps with two PUSCHs with different waveforms, it is possible that the UE would multiplex the UCI in the PUSCH with DFT-S-OFDM. The reliability of the UCI could be worse compared with the case where the UE multiplexes the UCI in the other PUSCH with CP-OFDM. A similar scenario exists in legacy, where the waveform is semi-static configured and the serving cells with good coverage can be configured with smaller index compared with serving cells with poor coverage in legacy. Now we are targeting the scenario with channel conditions that can dynamically change for a serving cell, hence it is necessary to study whether enhancement is needed for UCI multiplexing in case a PUCCH overlaps with multiple PUSCHs with different waveforms to ensure the reliability of UCI transmission.

Proposal 7: RAN1 to study whether enhancement is needed for UCI multiplexing in case a PUCCH overlaps with multiple PUSCHs with different waveforms.

# Conclusion

This contribution discusses potential enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM. The proposals and observations made in this contribution are summarized as below:

Observation 1: For DCI size alignment of a DCI format 0_1 or 0_2 scrambled by C-RNTI that includes a waveform indication field, when needed, both padding per DCI format and padding per field are feasible.

Observation 2: The DCI size alignment of a DCI format 0_1 or 0_2 scrambled by CS-RNTI that includes a waveform indication field is done according to the existing procedure.

Proposal 1: The field of a DCI format 0_1 or 0_2 scrambled by C-RNTI or CS-RNTI that includes the waveform indication precedes any field of the DCI format that has a bit width depending on the waveform.

Proposal 2: The field of a DCI format 0_1 or 0_2 scrambled by CS-RNTI that includes the waveform indication precedes the NDI field.

Proposal 3: Whether the dynamic indication field is present in a DCI format is configured by a higher layer parameter, e.g., transformPrecoder can indicate the configured waveform and whether the waveform is dynamically indicated.

Proposal 4: Indication of dynamic waveform switching for Msg3 PUSCH is not supported.

Proposal 5: Enhancements to mechanisms for PH report and triggering are not further discussed.

Proposal 6: RAN1 to study whether additional timeline is needed for UL scheduling in case of dynamic waveform switching.

Proposal 7: RAN1 to study whether enhancement is needed for UCI multiplexing in case a PUCCH overlaps with multiple PUSCHs with different waveforms.

# References

- RP-221858, Revised WID on Further NR coverage enhancements, China Telecom, RAN#96.