3GPP TSG RAN WG1 #112bis-e			R1-2303158

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.17.4

Source:	Samsung

Title:	Discussion on UE features for NR NCR

Document for:	Discussion and Decision

- Introduction

In this contribution, our views on UE feature for NR network-controlled repeater (NCR) are presented.

- Discussion

In previous RAN1 meetings [1-4], agreements related to UE feature for NR NCR are provided as below.

Agreement (RAN1#110) For the timing of NCR, the following assumption is captured into TR 38.867. - The DL transmitting timing of the NCR-Fwd is delayed after the DL receiving timing of the NCR-MT (or the NCR-Fwd) by the internal delay; - The UL receiving timing of the NCR-Fwd is advanced before the UL transmitting timing of the NCR-MT (or the NCR-Fwd) by the internal delay. Agreement (RAN1#110b-e) The following aspects should be NCR capability: - Simultaneous UL transmission of C-link and backhaul link - Adaptive beam for C-link/backhaul-link - Note-1: Fixed beam for C-link/backhaul link is default capability - Note-2: TDMed UL transmission of C-link and backhaul link is default capability. - FFS: How to define the capability for adaptive beam for C-link/backhaul-link Agreement (RAN1#110b-e) If adaptive beams are adopted for C-link and backhaul link, new signaling is supported to indicate a beam(s) used for backhaul link from the set of beams for C-link. - Predefined rule is used to define the beam in case there is no indication via the new signalling - FFS: Details of the predefined rule - FFS: Application of predefined rule for other cases - Note: The beam(s) used for backhaul link should be from the RRC-configured list of beams for C-link. - The new signalling, if needed, is an optional NCR capability Agreement (RAN1#111) As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported - FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link. Agreement (RAN1#112) For the aperiodic beam indication, the reference of slot offset for each time resource is defined as the slot n+k where n refers to the slot that NCR-MT receive the DCI carrying the indication and: - Option-2: k refers to the offset value [defined by NCR-MT capability and/or declared by vendor]. - Note: This k is different from the parameter used to define the Slot offset for the time resource.

As agreed in RAN1#110b-e, it is subject to NCR capability to support simultaneous transmission of the UL of C-link and UL of backhaul link. In addition, TDMed UL transmission of C-link and backhaul link was agreed as default capability.

Proposal 1: Introduce a capability for NR NCR to indicate support for simultaneous transmission of the UL of C-link and UL of backhaul link.

- TDMed UL transmission of C-link and backhaul link is the default capability.

For an NCR that supports simultaneous transmission of the UL of C-link and UL of backhaul link, an additional capability can be considered to indicate whether the NCR: (i) is not subject to a joint power limit across the C-link and BH-link (e.g., using separate PAs / power resources for the C-link and BH-link), or (ii) is subject to a joint power limit across the C-link and BH-link and supports power sharing when power limited, or (iii) is subject to a joint power limit across the C-link and BH-link and does not supports power sharing when power limited (e.g., transmit only one of C-link or BH-link when power limited). Corresponding NCR behavior can be further discussed.

Proposal 2: For an NR NCR indicating a support for simultaneous transmission of the UL of C-link and UL of backhaul link, introduce an additional capability to indicate support for power allocation and power sharing among the C-link and backhaul link.

As agreed in RAN1#110b-e, it is subject to NCR capability to support adaptive beam for C-link/backhaul-link. In addition, fixed beam for C-link/backhaul link was agreed as default capability.

Proposal 3: Introduce a capability for NR NCR to indicate support for adaptive beam for C-link/backhaul-link.

- Fixed beam for C-link/backhaul link is the default capability.

As agreed in RAN1#110b-e, it is subject to NCR capability to support dedicated signaling of beam indication for backhaul link.

Proposal 4: Introduce a capability for NR NCR to indicate support for dedicated signaling of beam indication for backhaul link.

In our view, whether to introduce NCR capability to support aperiodic beam indication for access link can be discussed.

Proposal 5: In terms of UE feature for NR NCR, further discuss the necessity of a capability for aperiodic beam indication for access link.

As agreed in RAN1#112, for the aperiodic beam indication, the reference of slot offset for each time resource is defined as the slot n+k. Here, n refers to the slot that NCR-MT receive the DCI carrying the indication and k refers to the offset value. A further study point is whether k is defined by NCR-MT capability and/or declared by vendor. In our view, k should be subject to NCR-MT capability. A value set for k can be larger than that for a similar UE capability for DCI processing (e.g., parameter N_pdsch) in order to support simplified implementation of NCR and/or to incorporate beam switching delay of the NCR-Fwd.

Proposal 6: Introduce a capability for NR NCR for defining the offset between the slot that NCR-MT receive the DCI carrying aperiodic beam indication and the reference of slot offset for the time resource for aperiodic beam indication.

- A value set for NR NCR capability can be larger than that for a UE capability for DCI processing (e.g., parameter N_pdsch).

As agreed in RAN1#110, NCR forward DL transmitting timing of the NCR-Fwd is delayed after the DL receiving timing of the NCR-MT (or the NCR-Fwd) by the internal delay; and UL receiving timing of the NCR-Fwd is advanced before the UL transmitting timing of the NCR-MT (or the NCR-Fwd) by the internal delay. A discussion points is the internal delay. In our view, it is beneficial for gNB to acquire the information on DL/UL internal delay of NCR. By knowing the value of DL/UL internal delay, gNB can identify whether a UE is being served by the NCR more accurately. A straightforward option for gNB to get internal delay of NCR is from NCR capability report. Also, same DL internal delay and UL internal delay can be assumed.

Proposal 7: Introduce a capability for the internal delay for NR NCR.

- DL internal delay and UL internal delay is the same.

As agreed in RAN1#111, support for BFD/BFR/RLM is an optional capability for NCR-MT, with Rel-15 procedures as baseline.

Proposal 8: Introduce a capability for NR NCR to indicate support for BFD/BFR/RLM.

- The capability can additionally indicate a supported release (e.g., Rel-15, or Rel-16/17/18) for these procedures. Rel-15 is the default capability.

Per agreements below from RAN1#110bis-e, legacy mechanisms for CSI measurement and reporting, uplink sounding (SRS), and HARQ-ACK feedback are supported by an NCR-MT, while details of the supported mechanisms are left open. It is reasonable to introduce corresponding UE capabilities to indicate the supported release for such features.

Proposal 9: Introduce capabilities for NR NCR to indicate the supported release for CSI measurement and reporting, uplink sounding (SRS), and HARQ-ACK feedback procedures.

- As starting point, a value set can consider {Rel-15, or Rel-16/17/18}. Rel-15 is the default capability.

Agreement (RAN1#110bis-e) To support CSI measurement/reporting mechanisms for NCR-MT in C-link - The necessary legacy mechanism for receiving CSI-RS is reused for NCR-MT. - The necessary legacy mechanism for reporting CSI is reused for NCR-MT. - FFS: The details of the necessary mechanisms will be further discussed and decided. - Note: this does not mean all the legacy procedures for receiving CSI-RS and reporting CSI will be supported. Agreement (RAN1#110bis-e) To support the sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported. - FFS: The details of the necessary mechanism of legacy UE sounding procedure. - Note: This does not mean all legacy UE sounding procedure will be supported. Agreement (RAN1#110bis-e) HARQ-ACK feedback for PDSCH carrying the side control information from higher layer (e.g., MAC-CE, RRC) is supported. The legacy HARQ-ACK feedback mechanism is reused. - FFS: Whether HARQ-ACK feedback for PDCCH carrying side control information is supported - Note: This does not mean all legacy HARQ-ACK feedback mechanism will be supported.

- Conclusion

The observations and proposals made in this contribution are summarized below.

Proposal 1: Introduce a capability for NR NCR to indicate support for simultaneous transmission of the UL of C-link and UL of backhaul link.

- TDMed UL transmission of C-link and backhaul link is the default capability.

Proposal 2: For an NR NCR indicating a support for simultaneous transmission of the UL of C-link and UL of backhaul link, introduce an additional capability to indicate support for power allocation and power sharing among the C-link and backhaul link.

Proposal 3: Introduce a capability for NR NCR to indicate support for adaptive beam for C-link/backhaul-link.

- Fixed beam for C-link/backhaul link is the default capability.

Proposal 4: Introduce a capability for NR NCR to indicate support for dedicated signaling of beam indication for backhaul link.

Proposal 5: In terms of UE feature for NR NCR, further discuss the necessity of a capability for aperiodic beam indication for access link.

Proposal 6: Introduce a capability for NR NCR for defining the offset between the slot that NCR-MT receive the DCI carrying aperiodic beam indication and the reference of slot offset for the time resource for aperiodic beam indication.

- A value set for NR NCR capability can be larger than that for a UE capability for DCI processing (e.g., parameter N_pdsch).

Proposal 7: Introduce a capability for the internal delay for NR NCR.

- DL internal delay and UL internal delay is the same.

Proposal 8: Introduce a capability for NR NCR to indicate support for BFD/BFR/RLM.

- The capability can additionally indicate a supported release (e.g., Rel-15, or Rel-16/17/18) for these procedures. Rel-15 is the default capability.

Proposal 9: Introduce capabilities for NR NCR to indicate the supported release for CSI measurement and reporting, uplink sounding (SRS), and HARQ-ACK feedback procedures.

- As starting point, a value set can consider {Rel-15, or Rel-16/17/18}. Rel-15 is the default capability.

References

- Chair notes, RAN1#110-e.

- Chair notes, RAN1#110bis-e.

- Chair notes, RAN1#111.

- Chair notes, RAN1#112.