3GPP TSG RAN WG1 #112bis-e	R1-2303159

# e-Meeting, April 17th – 26th, 2023

Agenda item:	9.17.10

Source:	Samsung

Title:	Discussion on UE features for multi-carrier enhancements

Document for:	Discussion and decision

# Introduction

This contribution discusses UE features for joint scheduling of PDSCHs/PUSCHs on multiple serving cells using a single DCI format, as considered in the following objective from the Rel-18 WI for multi-carrier enhancements [1].

1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1] - Identify the maximum number of cells that can be scheduled simultaneously - Consider both intra-band and inter-band CA operation - Consider both FR1 and FR2 - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

Throughout this document, cells that are jointly scheduled by a single DCI format are referred to as “co-scheduled” cells.

# Supported scheduling combinations

Based on the following RAN1 agreements, a UE can be configured with up to 4 sets of cells for multi-cell scheduling, with each set of cells including up to 4 cells. The up to 4 configured sets of cells for multi-cell scheduling can be associated with different scheduling cells, while only up to N sets of cells can be from a same scheduling cell, where N can be a UE capability.

A DCI format 0_X/1_X can schedule one or more PUSCHs or PDSCHs on one or more cells in one set of cells, from the configured sets of cells. The cell combinations, in a set of cells, that a DCI format 0_X/1_X schedules can be configured in a table and associated with an index that is indicated in the DCI format 0_X/1_X, or can be arbitrary and indicated by FDRA values, where a non-reserved FDRA value for a cell indicates that the cell is scheduled, and a reserved FDRA value for a cell indicates that the cell is not scheduled.

Accordingly, the following parameters can be potentially considered as UE capability:

- Maximum number of sets of cells in a PUCCH group (i.e., across all scheduling cells)  since different scheduling cells are considered, there seems little motivation to introduce a new UE capability for this parameter. The maximum value of 4 sets of cells, as already agreed, can be assumed for all Rel-18 UEs that support multi-cell scheduling;

- Maximum number of sets of cells from a same scheduling cell  this is already agreed in RAN1 to be a UE capability. This parameter can be further considered in conjunction with parameter (5) below for a total number of configurable cells for co-scheduling from a same scheduling cell;

- Maximum number of cells in a set of cells  since a DCI format schedules only subsets / combinations of cells from a set of cells, there seems little motivation to introduce a UE capability for this parameter. The maximum value of 4 cells in each set of cells, as already agreed, can be assumed for all Rel-18 UEs that support multi-cell scheduling;

- Maximum number of cells in a/any configured cell combination from a set of cells  this parameter is related to a legacy UE capability for UL/DL CA. For example, if a UE reports a legacy capability for 2-cell UL CA (e.g., by FG 6-6 “Basic UL NR-NR CA operation”), the UE does not expect to be scheduled by an UL MC-DCI format 0_X on a cell combination that includes more than 2 UL cells. Therefore, a new UE capability for MC-DCI may not be necessary.

  - When a UE determines a co-scheduled cell combination based on FDRA values, there is no pre-configured list/table of cell combinations provided to the UE. Therefore, the legacy UE capability for DL/UL CA implies a maximum number of non-reserved values for FDRA in a DCI format 0_X/1_X supported by the UE.

- Maximum total number of configurable cells for co-scheduling from a same scheduling cell  Since Rel-17 supports up to 8 scheduled cells from a same scheduling cell, the UE does not expect to be configured a total number of cells across the N sets of cells that exceeds 8 cells. However, each set of cells can include fewer cells, such as 4 sets of cells, each including 2 cells. Therefore, it can be further discussed whether parameter (2) can be combined with or replaced by parameter (5);

- Maximum total number of cells across cell combinations that are co-scheduled by a DCI format 0_X/1_X from a same scheduling cell in a PDCCH monitoring occasion (or in a same slot)  This parameter is related to a total number of DCI formats that the UE can process in a slot/MO for a same scheduling cell and prepare corresponding PUSCHs/PDSCHs. This parameter is related to FG 18-5 and 18-5b “DL/UL cross-carrier scheduling with different SCS”, and can be further discussed as potential UE capability;

- Maximum total number of configurable cells for co-scheduling across all sets of cells / scheduling cells  Since different scheduling cells are considered, similar to parameter (1), there may be little motivation for a new UE capability. This parameter appears to be related to legacy DL/UL CA capability, such as FG 6-5 and 6-6 for “Basic DL/UL NR-NR CA operation”.

Proposal 1: Further discuss whether/how to introduce UE capability for the following parameters for multi-cell scheduling:

- Maximum number of sets of cells for multi-cell scheduling from a same scheduling cell;

- Maximum total number of configurable cells for co-scheduling, across different sets of cells for multi-cell scheduling, from a same scheduling cell;

- Maximum total number of cells across cell combinations that are co-scheduled by a DCI format 0_X/1_X from a same scheduling cell in a PDCCH monitoring occasion (or in a same slot).

Agreement (RAN1#109-e) - For a UE, the maximum number of cells scheduled by a DCI format 0_X can be same or different to the maximum number of cells scheduled by a DCI format 1_X. Agreement (RAN1#109-e) - One value for the maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is selected from {3, 4, 8}. - For a UE, the maximum number of co-scheduled cells by a DCI format 0_X can be smaller than or equal to the maximum number supported in Rel-18. Agreement (RAN1#109-e) - One value for the maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is selected from {3, 4, 8}. - For a UE, the maximum number of co-scheduled cells by a DCI format 1_X can be smaller than or equal to the maximum number supported in Rel-18. Agreement (RAN1#110bis-e) Confirm the following working assumption reached in RAN1#110 meeting. Working Assumption - The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4. - The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4. - FFS: The maximum number of configurable cells for co-scheduling Agreement (RAN1#111) For a set of cells which is configured for multi-cell scheduling, up to 4 cells within the set of cells are supported. - A DCI format 0_X/1_X can schedule PUSCH(s)/PDSCH(s) on a combination of co-scheduled cells among the same set of cells. Agreement (RAN1#112) Following is supported in Rel-18 multi-cell scheduling - A UE can be configured one or multiple sets of cells with each set configured for multi-cell scheduling using DCI format 0_X/1_X. - Up to 4 sets of cells can be configured per PUCCH group. - When multiple sets of cells are configured, - a cell in one set of cells can’t be included in another set of cells. - n_CI value is independently configured for each set of cells. - reference cell for counting DCI size and BD/CCE of DCI format 0_X/1_X is independently determined for each set of cells. - search space configuration of DCI format 0_X/1_X is independently configured for each set of cells. - DCI size of DCI format 0_X is independently determined for each set of cells. - DCI size of DCI format 1_X is independently determined for each set of cells. - The multiple sets of cells can be scheduled by DCI format 0_X/1_X from different scheduling cells. - Up to N sets of cells can be configured and respectively scheduled by DCI format 0_X/1_X from a same scheduling cell. - The value of N is reported as UE capability. - An indicator is included in the DCI to indicate the scheduled set of cells, - The size of the indicator is equal to ceil(log2(N)), where N is the number of sets of cells. - Unique n_CI value is configured for each set of cells.

The following agreement was reached as a compromise for indication of a co-scheduled cell combination in a DCI format 0_X/1_X using one of two methods: table-based and FDRA-based. If selection between the two methods is to be based on a UE capability, the first method based on RRC-configured table should be considered as the baseline method and default UE capability, as it leads to more efficient DCI size.

Proposal 2: If a UE capability is to be introduced for selecting the method for indication of co-scheduled cells in a DCI format 0_X/1_X, adopt the “table-based” method as the default UE capability.

Agreement (RAN1#112) For a set of cells which is configured for multi-cell scheduling using DCI format 0_X and DCI format 1_X, support the following: - If table defining combinations of co-scheduled cells for the set of cells is configured, - an indicator in the DCI is included and points to one row of the table. - The table is configured by RRC signaling for the set of cells. - Separate tables are configured for downlink scheduling and uplink scheduling - The size of the indicator is equal to ceil(log2(N)), where N is the number of rows in the table. - The max number of rows in the table is 16 - The size of the per-cell Type 2 fields for each co-scheduled cell does not change according to the indicated co-scheduled cell combination - The payload size of DCI format 1_X is derived by UE based on RRC configuration of the active BWP(s) of co-scheduled cell combinations within the set of cells. - The payload size of DCI format 1_X is the same for the active BWP(s) of all the co-scheduled cell combinations and equal to the largest payload size among the active BWP(s) of all the co-scheduled cell combinations determined by the co-scheduled cell combination table. - The payload size of DCI format 0_X is derived by UE based on RRC configuration of the active BWP(s) of co-scheduled cell combinations within the set of cells. - The payload size of DCI format 0_X is the same for the active BWP(s) of all the co-scheduled cell combinations and equal to the largest payload size among the active BWP(s) of all the co-scheduled cell combinations determined by the co-scheduled cell combination table. - Otherwise, - The UE determines the actually scheduled cell(s) based on the FDRA field of each cell of the set of cells. - For Type 0 FDRA, all 0s indicates the cell is not scheduled. - For Type 1 FDRA, all 1s indicates the cell is not scheduled. - The size of the Type 2 fields for each cell does not change according to actually co-scheduled cells. - The payload size of DCI format 0_X is derived by UE based on RRC configuration of the active BWP(s) of all cells within the set of cells. - The payload size of DCI format 1_X is derived by UE based on RRC configuration of the active BWP(s) of all cells within the set of cells.

# UE features for PDCCH monitoring

The following RAN1 agreement describes the UE behavior for monitoring legacy single-cell scheduling DCI (SC-DCI) formats in parallel with the new DCI format 0_X/1_X for multi-cell scheduling for a same scheduled cell. In Rel-17, the UE can be configured to monitor different SC-DCI formats for a same scheduled cell in same or different monitoring occasions, without any restriction or UE capability. Since RAN1 has agreed to maintain the Rel-17 PDCCH monitoring limits when DCI formats 0_X/1_X are configured, there is no reason to make an exception or restriction for monitoring DCI formats 0_X/1_X, so an additional UE capability is not necessary.

Proposal 3: Do NOT introduce a UE capability for monitoring, for any scheduled cell, both DCI formats 0_X/1_X and DCI formats 0_0/1_0, 0_1/1_1, and/or 0_2/1_2 (if supported by the UE), either simultaneously or non-simultaneously, from a same scheduling cell.

Agreement (RAN1#110bis-e) Confirm below working assumption reached in RAN1#110 meeting with revision. Working Assumption - For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and DCI format 0_0/1_0, 0_1/1_1, and/or 0_2/1_2 (if supported by the UE), if configured from a same scheduling cell. - The DCI format 0_X/1_X and the DCI format 0_0/1_0/0_1/1_1/0_2/1_2 can be monitored simultaneously. - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. Agreement (RAN1#111) Confirm the RAN1#110bis-e working assumption with the following changes: Working Assumption For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells. - DCI size of the DCI format 0_X/1_X is counted on the reference cell. - BD/CCE of DCI format 0_X/1_X is counted on one cell among the set of cells. - BD/CCE of the DCI format 0_X/1_X is counted on the reference cell. - Same reference cell is used for both DCI format 0_X and DCI format 1_X. - The reference cell is - the scheduling cell if the scheduling cell is included in the set of cells and search space of the DCI format 0_X/1_X is configured only on the scheduling cell; - one cell of the set of cells which search space of DCI format 0_X/1_X is configured on and associated with the search space of the scheduling cell with the same search space ID if search space of the DCI format 0_X/1_X is configured on the cell in addition to the scheduling cell. - It is up to gNB on which cell the SS of the DCI format 0_X/1_X is configured on. - To address Rel-17 BD/CCE limit for any given cell (operating the feature under Rel-17 BD/CCE limit) - For the reference cell, a total number of configured BD/CCEs for both DCI formats 0_X/1_X and legacy DCI formats (if configured) does not exceed the Rel-17 limits. - For other cells in the sets of cells, Rel-17 limits for PDCCH/DCI monitoring and BD/CCE counting rules for legacy DCI formats (not including DCI formats 0_X/1_X) apply

# Conclusions

This contribution considered UE features for multi-cell scheduling of PDSCHs/PUSCHs using a single DCI format and proposed the following.

Proposals

Proposal 1: Further discuss whether/how to introduce UE capability for the following parameters for multi-cell scheduling:

- Maximum number of sets of cells for multi-cell scheduling from a same scheduling cell;

- Maximum total number of configurable cells for co-scheduling, across different sets of cells for multi-cell scheduling, from a same scheduling cell;

- Maximum total number of cells across cell combinations that are co-scheduled by a DCI format 0_X/1_X from a same scheduling cell in a PDCCH monitoring occasion (or in a same slot).

Proposal 2: If a UE capability is to be introduced for selecting the method for indication of co-scheduled cells in a DCI format 0_X/1_X, adopt the “table-based” method as the default UE capability.

Proposal 3: Do NOT introduce a UE capability for monitoring, for any scheduled cell, both DCI formats 0_X/1_X and DCI formats 0_0/1_0, 0_1/1_1, and/or 0_2/1_2 (if supported by the UE), either simultaneously or non-simultaneously, from a same scheduling cell.

# References:

- RP-213577, New WID on Multi-carrier enhancements, NTT DOCOMO, INC.