3GPP TSG RAN WG1 #112bis-e	        		          R1-2303160

e-meeting, April 17th – 26th, 2023

Agenda item:		9.17.14

Source:			Samsung

Title:			UE features for eDSS

Document for:		Discussion and decision

# Discussion

This contribution considers the UE features for supporting PDCCH receptions in symbols that include CRS. UE features for supporting additional rate matching patterns for CRS were agreed in RAN1#110.

# UE capabilities for eDSS

The baseline UE capability for DSS is whether a UE can receive a PDCCH candidate when REs used for the PDCCH candidate overlap with LTE CRS REs. At the most basic realization, the UE can operate by ignoring the presence of LTE CRS REs. It is noted that such operation can also be achieved for a Rel-15 UE by the network indicating 2 CRS ports (symbol 0), when there are actually 4 CRS ports (symbols 0 and 1), for example for a CORESET located in symbols 1 and 2. A problem with such approach is that PDSCH BLER will be affected, especially for QAM modulations. That problem can be avoided by eDSS as the pattern with 4 CRS ports can be indicated. The impact on PDCCH BLER is under the control of the network and can be addressed by a lower code rate for the DCI or even, at least in principle, by reducing LTE CRS power in affected REs or symbols of a PDCCH transmission.

To improve PDCCH BLER with reduced requirements on network operation, a first UE capability is to receive/decode DCI formats by adjusting the LLRs for symbols in REs overlapping with LTE CRS REs (e.g. exclude those LLRs from use in decoding). Another UE capability is to do the same for channel estimation – adjust the weighting of DM-RS symbols in REs overlapping with LTE CRS REs (e.g. excluding those DM-RS symbols from use in channel estimation). To enable a simplified implementation for channel estimation that essentially re-uses the legacy channel estimator, another UE capability was agreed for a UE can perform channel estimation using only DM-RS REs in symbols where no DM-RS RE overlaps with LTE CRS RE. It is noted that PDCCH BLER based on channel estimation using DM-RS REs only in symbols where no DM-RS RE overlaps with LTE CRS RE will be worse than using all DM-RS REs except for those overlapping with CRS REs and, therefore, if a UE meets BLER requirements defined for the former channel estimator, the UE will meet those BLER requirements using the later channel estimator.

Finally, the gNB needs to be aware of each UE capability in order to be able to determine link budget requirements for a DCI format detection with a desired BLER by the UE.

Proposal 1: Consider the UE FGs in Table 1 for eDSS.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N-1 | PDCCH monitoring on REs with LTE CRS | Support PDCCH reception on REs that overlap with LTE CRS REs |  | Yes | Does not support PDCCH receptions in symbols with LTE CRS | Per UE | No | N/A | N/A | Only in FR1, only for 15 kHz SCS | Optional without capability signalling |
| N-2 | Optimized DCI format decoding | Support decoding without using DCI symbols in REs that overlap with LTE CRS REs | N-1 | Yes | Larger DCI BLER | Per UE | No | N/A | N/A |  | Optional with capability signalling |
| N-3-1 | Optimized channel estimation 1 | Support channel estimation without using DM-RS in REs that overlap with LTE CRS REs | N-1 | Yes | Larger DCI BLER | Per UE | No | N/A | N/A |  | Optional with capability signalling |
| N-3-2 | Optimized channel estimation 2 | Support channel estimation without using DM-RS in symbols where DM-RS REs overlap with LTE CRS REs | N-1 | Yes | Larger DCI BLER | Per UE | No | N/A | N/A |  | Optional with capability signalling |

Table 1: UE FGs for eDSS