3GPP TSG RAN WG1 #112bis-e		                                         R1-2303166

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.18

Source: 	Spreadtrum Communications

Title: 	Discussion on remaining issues for eDSS and MC

Document for:	Discussion and decision

Introduction

At RAN1#110-e meeting [1], the main work for RAN1 was completed and the following agreement were achieved.

Agreement If the feature “Reception of NR PDCCH candidates that overlap with LTE CRS REs “is supported, it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. Agreement Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side. Note: depends on UE capability Following options can be used for PDCCH-DMRS channel estimation - legacy CE assumption - RAN1 consider support this, if no RAN4 performance requirements are defined - CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET) Note: Restriction on the symbols and/or LTE CRS patterns applicable for above agreements can be considered during UE capability session. Conclusion RAN1 understands above agreements applies to 15kHz SCS only.

In this document, we provide our views on remaining issues for eDSS.

Discussion

Remaining issues for eDSS

RRC configuration

At RAN1#110-e meeting, two agreements and one conclusion were achieved. According to the agreement of RAN1#110-e meeting, whether to support reception of NR PDCCH candidates that overlap with LTE CRS REs depends on UE capability. In addition, there are two options can be used for PDCCH-DMRS channel estimation: legacy CE or CE on clean symbol(s) only. There are some remaining issues, including CE related issues, applicable search space set and LTE CRS patterns, etc.

In RAN1#110b-e meeting [2], whether to introduce UE capabilities to indicate the supported CE methods was discussed. However, there is no consensus on them. From our perspective, legacy CE should be a default UE behaviour. If no new RAN4 performance requirements will be defined, it is strange to introduce different CE options as UE capability, which totally can be UE implementation as long as RAN4 performance requirements can be met. From the perspective of UE implementation, there is no difference between these two CE. No additional algorithm complexity are introduced in CE on clean symbol(s) only compared to legacy CE. So the two channel estimation cannot be regarded as two UE capabilities. If no new RAN4 performance requirements is defined, there is no need to report the CE capability. Thus, it is unreasonable to introduce two CE capabilities when RAN4 decision on related requirements is not available.

Observation 1. There is no difference for legacy CE or CE on clean symbol(s) only in terms of UE capability and complexity.

Observation 2. The CE method depends on UE implementation as long as RAN4 performance requirements can be met.

Someone argued that CE capability can help NW identify whether the clean symbol is needed or not. We think it is not necessary and unreasonable. The gNB can allocate resources freely, and the UE can use appropriate methods to perform channel estimation, which depends on UE implementation. The configuration of CORESET should not be linked with “CE capabilities”.

If both legacy CE and CE on clean symbol are supported, which CE method is used should be determined. The CE method indicated by gNB is the most direct and simplest method. Network can have full knowledge about the situation of LTE deployment and its interference level to NR side. Thus, it is suitable for network to control the whole DSS transmission including UE channel estimation method, which will potentially achieve the best NR-PDCCH performance. For example, a new RRC parameter (i.e., PDCCH-DMRS-CE-r18) in ServingCellConfig can be introduced to indicate the channel estimation method. Alternatively, a bit information is carried in the system information to indicate the channel estimation method [3].

Proposal 1. Recommendation on NR-PDCCH channel estimation methods is indicated by the network:

- By introducing a new RRC parameter (i.e., PDCCH-DMRS-CE-r18) in ServingCellConfig

Restriction on LTE CRS pattern lists

According to the discussions in AI 9.9.2 in RAN1#110-e meeting, the UE can be configured with one or two CRS rate matching pattern lists, i.e. only legacy list 1, legacy list 1 and list 2, only new list 3, new list 3 and list 4. One LTE CRS pattern case has been evaluated by simulation while others never evaluated by RAN1.There would be limited REs (i.e., 4 RE) besides LTE CRS if two CRS patterns overlapping in frequency can be configured to puncture NR PDCCH, which will have a great impact on NR PDCCH decoding performance.  Thus, it should be clarified only one LTE CRS pattern is applicable to NR PDCCH candidate.

Proposal 2. Only one LTE CRS pattern is applicable to NR PDCCH candidate.

Restriction on Search space types

It’s not clear that how gNB should handle cell-specific CSS coexistence between legacy UEs and Rel-18 DSS UEs. For the CSS such as Type 0/0A/1/2, it is typically cell specific and they might be shared by a group of UEs such as UE is before RRC connection setup. Thus, it is not feasible to do LTE CRS puncture NR PDCCH associated with Type 0/0A/1/2 CSS. Besides, it may include legacy UEs and Rel-18 DSS UEs. In order to maintain consistent UE behavior, reception of NR PDCCH candidates that overlapping with LTE CRS REs cannot apply to cell-specific CSS. NR PDCCH in Type 0/0A/1/2 CSS are transmitted using legacy PDCCH/CORESET on non-LTE CRS symbol(s).

Proposal 3. Reception of NR PDCCH candidates in Type 0/0A/1/2 CSS overlapped with LTE CRS REs is not supported.

Clarification on narrow/wideband CORESET

In R17, PDCCH candidate overlaps with LTE CRS is not allowed in case of precoderGranularity = allContiguousRBs. It can be allowed when precoderGranularity = sameAsREG-Bundle and UE does not monitor the PDCCH candidate. We think same rules should be maintained for wideband CORESET, i.e., it is not expected to collide with LTE CRS REs for a CORESET with precoderGranularity = allContiguousRBs. For narrow CORESET, if PDCCH candidate overlap with LTE CRS, the UE is required to monitor the PDCCH candidate. We think it is not necessary to link UE monitoring behavior with CE choice.

Proposal 4. For a CORESET with precoderGranularity = allContiguousRBs, NR PDCCH candidates is not expected to be overlapped with LTE CRS REs.

Restriction on symbols

Based on the previous discussion, it does not have a clear clarification that locations of the NR PDCCH symbols applied to this feature. And it is not clear whether or not NR PDCCH can extend to symbol#0.

RAN1 never evaluated the overlapped symbol on Symbol#0 by simulation and this case should be excluded. Otherwise, the PDCCH decoding performance will deteriorate if NR PDCCH extend to symbol#0 and symbol #1. We think NR PDCCH can be allowed to overlap with LTE CRS on symbol#1, but not allowed to symbol#0.

Proposal 5. Specify restrictions on certain symbols where reception of NR PDCCH overlapped with LTE CRS REs is not supported e.g. symbol#0.

Text proposal for eDSS

Based on the above discussion, the following TP for clause 10 of TS 38.213 is proposed.

Proposal 6. Adopt the TP for clause 10 of TS 38.213.

10 UE procedure for receiving control information < Unchanged parts are omitted > If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. If the UE does not indicate capability [NR-PDCCHReceptionWithLTE-CRS], and Iif at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, the UE is not required to monitor the PDCCH candidate. If a UE is provided availableRB-SetsPerCell, the UE is not required to monitor PDCCH candidates that overlap with any RB from RB sets that are indicated as unavailable for receptions by an available RB set indicator field in DCI format 2_0 as described in clause 11.1.1. If the UE does not obtain the available RB set indicator for a symbol, the UE monitors PDCCH candidates on all RB sets in the symbol. < Unchanged parts are omitted >

Remaining issues for MC

RRC configuration for a single DCI scheduling multiple cells

Since a UE can be configured one or multiple sets of cells with each set configured for multi-cell scheduling using DCI format 0_X/1_X, those multiple sets of cells can be configured under PhysicalCellGroupConfig, as a list. Each entry/ID in the list points to a set of cells, containing detailed configurations for each set of cells.

In each set of cells configuration, there can be some fields are newly configured:

- the cells in the set (a list of ServCellIndex),

- n_CI,

- Scheduling cell ID (ServCellIndex),

- Search space for DCI format 0_X/1_X (SearchSpace)

For DCI fields in DCI format 0_X/1_X, if the field can be configured as Type-1A or Type-2 field, new RRC parameters are needed to introduced, including two value {Type-1A, Type-2 field}. If the field is a Type-1B field, new RRC-configured table are introduced. Each table of Type-1B field needs to discuss separately.

Table 1: DCI fields for DCI format 1_X

| Field | Type | Bits length | New RRC parameters or not |
| --- | --- | --- | --- |
| Identifier for DCI formats | Type 1A | 1 bit | No If it is set to 0, indicating an UL DCI format. If it is set to 1, indicating an DL DCI format |
| Indicator of co-scheduled cells | NA or Type 1B | 0,1,2,3,4 bits | Yes, for a RRC-configured table defining combinations of co-scheduled cells |
| Bandwidth part indicator | Type 1A | 0,1,2 bits | No |
| Frequency domain resource assignment | Type 2 |  | Yes A new RBG size configuration “Configuration 3” for RA type 0 New RRC parameter of RBG granularity for RA type 1 |
| Time domain resource assignment | Type 1B | 1,2,3,4,5 bits | Yes, A RRC-configured table for joint TDRA table |
| VRB-to-PRB mapping | Type 1A | 0, 1 bit | No |
| PRB bundling size indicator | Type 1A | 0, 1 bit | No |
| Rate matching indicator | Type 1B (up to 4 bits) | 0,1,2,3,4 bits | Yes, A RRC-configured table |
| ZP CSI-RS trigger | Type 1B (up to 3 bits) | 0,1,2,3 bits | Yes, A RRC-configured table |
| TB1/2: Modulation and coding scheme | Type 2 (without compression) | N*5 bits | No |
| TB1/2: New data indicator | Type 2 | N bits | No |
| TB1/2: Redundancy version | Type 2 |  | Yes Size of RV field can be configured per BWP per cell for DCI format 0_X/1_X. |
| HARQ process number | Type 2 |  | Yes Size of HPN field can be configured per BWP per cell for DCI format 0_X/1_X. |
| Downlink assignment index | Type 1A | 0 or 2 or 4 bits | No |
| TPC command for scheduled PUCCH | Type 1A | 2 bits | No |
| PUCCH resource indicator | Type 1A | 3 bits | No |
| PDSCH-to-HARQ timing indicator | Type 1A | 0 or 1 or 2 or 3 bits | No |
| One shot HARQ ACK request | Type 1A | 0 or 1 bit | No |
| Enhanced Type 3 codebook indicator | Type 1A | 0, 1, 2, or 3 bits | No |
| HARQ-ACK retransmission indicator | Type 1A |  | No |
| Antenna port(s) | Configurable between Type 1A and Type 2 |  | Yes Two values {Type-1A, Type-2 field} |
| Transmission configuration indication | Type 1B (up to 4 bits) | 0, 1, 2, 3, 4 bits | Yes, A RRC-configured table |
| DMRS sequence initialization | Type 1A | 0, 1 bit | See Section 3.2.2 |
| SRS request | Type 1B (up to 4 bits) | 0, 1, 2, 3, 4 bits | Yes, A RRC-configured table |
| SRS offset indicator | Type 1B (up to 3 bits) | 0, 1, 2, or 3 bits | Yes, A RRC-configured table |
| DMRS sequence initialization | Type 1A | 0, 1 bit | No |
| Priority indicator | Type 1A | 0, 1 bit | Yes A RRC parameter to configure the presence of priority indicator |
| PDCCH monitoring adaptation indication |  |  | Yes Inclusion of PDCCH monitoring adaptation indication in DCI format 0_X/1_X is configurable |
| Minimum applicable scheduling offset indicator |  |  | Yes Inclusion of minimum applicable scheduling offset indicator in DCI format 0_X/1_X is configurable |
| PUCCH Cell indicator | Type-1A | 0 or 1 bit | No |
| ChannelAccess-Cpext | Type 1A |  | No |

Table 2: DCI field types for DCI format 0_X

| Field | Type | Bits length | Details |
| --- | --- | --- | --- |
| Identifier for DCI formats | Type-1A | 1 bit | No If it is set to 0, indicating an UL DCI format. If it is set to 1, indicating an DL DCI format |
| Indicator of co-scheduled cells | Type 1B | 1,2,3,4 bits | Yes, for a RRC-configured table defining combinations of co-scheduled cells |
| Bandwidth part indicator | Type 1A | 0,1,2 bits | No |
| Frequency domain resource assignment | Type 2 |  | Yes A new RBG size configuration “Configuration 3” for RA type 0 New RRC parameter of RBG granularity for RA type 1 |
| Time domain resource assignment | Type 1B | 1,2,3,4,5 bits | Yes, A RRC-configured table for joint TDRA table |
| Frequency hopping flag | Type 1A | 0, 1 bit | No |
| Modulation and coding scheme | Type 2 (without compression) | N*5 bits | No |
| New data indicator | Type 2 | N bits | No |
| Redundancy version | Type 2 |  | Yes Size of RV field can be configured per BWP per cell for DCI format 0_X/1_X. |
| HARQ process number | Type 2 |  | Yes Size of HPN field can be configured per BWP per cell for DCI format 0_X/1_X. |
| Downlink assignment index | Type-1A | 0 or 2 or 4 bits | No |
| TPC command for scheduled PUSCH | Type-2 | N*2 bits | No |
| SRS resource set indicator | Configurable between Type 1A and Type 2 |  | Yes Two values {Type-1A, Type-2 field} |
| SRS resource indicator | Configurable between Type 1A and Type 2 |  | Yes Two values {Type-1A, Type-2 field} |
| Precoding information and number of layers | Configurable between Type 1A and Type 2 |  | Yes Two values {Type-1A, Type-2 field} |
| Antenna port(s) | Configurable between Type 1A and Type 2 |  | Yes Two values {Type-1A, Type-2 field} |
| SRS request | Type-1B | 0 or 1 or 2 or 3 or 4 bits | Yes, A RRC-configured table |
| SRS offset indicator. | Type-1B | 0 or 1 or 2 or 3 bits | Yes, A RRC-configured table |
| CSI request | Type 1C | 2 bits | No |
| PTRS-DMRS association | Configurable between Type 1A and Type 2 |  | Yes Two values {Type-1A, Type-2 field} |
| beta offset indicator | Type 1A |  | No |
| DMRS sequence initialization | Type-1A |  | No |
| UL-SCH indicator | Type 1C | 1 bit | No |
| ChannelAccess-CPext-CAPC | Type 1A | NA | No |
| Priority indicator | Type 1A | 0, 1 bit | Yes A RRC parameter to configure the presence of priority indicator |
| Minimum applicable scheduling offset indicator |  |  | Yes Inclusion of minimum applicable scheduling offset indicator in DCI format 0_X/1_X is configurable |
| PDCCH monitoring adaptation indication |  |  | Yes Inclusion of PDCCH monitoring adaptation indication in DCI format 0_X/1_X is configurable |

- Proposal 7.  Set(s) of cell specific parameters can configured under PhysicalCellGroupConfig.

Proposal 8. For a set of cells, it can configure the following parameters:

- Cells in the set (a list of ServCellIndex),

- n_CI,

- Scheduling cell ID (ServCellIndex),

- Search space for DCI format 0_X/1_X (SearchSpace)

Proposal 9. All the Type-1B fields need a RRC-configured table which requires new RRC parameters.

Proposal 10. For all the Type-3 fields, new RRC parameters are needed to introduced, including two value {Type-1A, Type-2 field}.

Proposal 11. New RRC parameters are introduced for some Type-2 fields,, such as FDRA, RV and HARQ process.

RRC configuration for UL Tx switching of 3/4 bands

According to the agreement below for Case#2 [4], the new RRC parameter is configured to determine the associated band for another Tx chain, while oneT is indicated and switched from band A and B to band C. This new RRC parameter for associated band is configured for each band.

Agreement Following working assumption is confirmed with updates. Working Assumption At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases - Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported - if twoT is indicated, both of two Tx chains are switched to band B - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A - Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported - if twoT is indicated, both of two Tx chains are switched to band C - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS - Alt.1: based on gNB’s configuration/indication e.g., new RRC parameter Agreement In Case#2 where two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, if oneT is indicated via uplinkTxSwitching-DualUL-TxState, one Tx chain is switched to band C and associated band for another Tx chain is determined by new RRC parameter which is down-selected from following alternatives. - An associated band is configured for each band so that another Tx chain is associated with the configured band (as associated band for the transmitting band) - E.g., associated band for each transmitting band is configured as {B for A}, {A for B}, {A for C} and {C for D}. - When 1 port transmission on band C is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band B is switched to band C while another Tx chain associated with band A remains unchanged (because band A is associated band for band C) - When 1 port transmission on band D is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band A (or B) is switched to band D while another Tx chain associated with band B (or A) is switched to band C (because band C is associated band for band D) - If there is one band where concurrent transmission with any other band is not supported, NW does not configure an associated band for the band. In such case, even if oneT is configured, UE performs switching as twoT is configured when 1 port transmission on the band is scheduled

New RRC parameter of the priority for each band is needed according to the following agreement. This new RRC parameter can be configured in the order from low to high for each UL Tx switching bands.

Agreement Alt.5: gNB configures priorities to each carrier/band. - The gNB configures priority for each band. The UE determines the switching period location on either switching-from band(s) or switching-to band(s) that is involved in the UL Tx switching and is not with the highest priority band.

Proposal 12. New RRC parameter is configured to determine the associated band for another Tx chain, while oneT is indicated and switched from band A and B to band C.

- This new RRC parameter for associated band is configured for each band.

Proposal 13. New RRC parameter of the priority for each band is needed.

- This new RRC parameter can be configured in the order from lowest to highest for all UL Tx switching bands.

Conclusion

In this contribution, we have the following observations and proposals.

Observation 1. There is no difference for legacy CE or CE on clean symbol(s) only in terms of UE capability and complexity.

Observation 2. The CE method depends on UE implementation as long as RAN4 performance requirements can be met.

Proposal 1. Recommendation on NR-PDCCH channel estimation methods is indicated by the network:

- By introducing a new RRC parameter (i.e., PDCCH-DMRS-CE-r18) in ServingCellConfig

Proposal 2. Only one LTE CRS pattern is applicable to NR PDCCH candidate.

Proposal 3. Reception of NR PDCCH candidates in Type 0/0A/1/2 CSS overlapped with LTE CRS REs is not supported.

Proposal 4. For a CORESET with precoderGranularity = allContiguousRBs, NR PDCCH candidates is not expected to be overlapped with LTE CRS REs.

Proposal 5. Specify restrictions on certain symbols where reception of NR PDCCH overlapped with LTE CRS REs is not supported e.g. symbol#0.

Proposal 6. Adopt the TP for clause 10 of TS 38.213.

Proposal 7. Set(s) of cell specific parameters can configured under PhysicalCellGroupConfig.

Proposal 8. For a set of cells, it can configure the following parameters:

- Cells in the set (a list of ServCellIndex),

- n_CI,

- Scheduling cell ID (ServCellIndex),

- Search space for DCI format 0_X/1_X (SearchSpace)

Proposal 9. All the Type-1B fields need a RRC-configured table which requires new RRC parameters.

Proposal 10. For all the Type-3 fields, new RRC parameters are needed to introduced, including two value {Type-1A, Type-2 field}.

Proposal 11. New RRC parameters are introduced for some Type-2 fields,, such as FDRA, RV and HARQ process.

Proposal 12. New RRC parameter is configured to determine the associated band for another Tx chain, while oneT is indicated and switched from band A and B to band C.

- This new RRC parameter for associated band is configured for each band.

Proposal 13. New RRC parameter of the priority for each band is needed.

- This new RRC parameter can be configured in the order from lowest to highest for all UL Tx switching bands.

Reference

3GPP RAN1, RAN1#110-e, Chairman Notes.

R1-2210358, Moderator Summary for Rel. 18 NR DSS, Moderator (Ericsson).

R1-2208578,“Maintenance for Rel-18 DSS”, Spreadtrum Communications

3GPP RAN1, RAN1#112, Chairman Notes.