3GPP TSG RAN WG1 #112bis-e		R1-2303167

e-Meeting, April 17th – 21th, 2023

Source:	Panasonic

Title: 	Discussion on potential enhancements on dynamic/flexible TDD

Agenda Item:		9.3.3

Document for:	Discussion

Introduction

A study item “Study on evolution of NR duplex operation” has been agreed for Rel.18. One of key objectives is to study gNB-to-gNB CLI measurement/UE-to-UE co-channel CLI measurement and reporting in SBFD (subband non-overlapping full duplex) and dynamic/flexible TDD operation. This document provides our view on the objective.

Discussion

UL resources muting for gNB-to-gNB CLI measurement

In RAN1#110bis-e, the following agreement was reached.

Agreement For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further. Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not.

In order to accurately measure the gNB-to-gNB CLI, victim gNB measure the gNB-to-gNB CLI without interference from UL (from UE served in victim gNB) using muting resources. For example, when victim gNB measures SSB from aggressor gNB, victim gNB can mute UL resource as SSB. In other usage, UL resources muting can also be used to avoid the gNB-to-gNB CLI from specific DL signal(s)/channel(s). The following descriptions are the characteristics of UL resources muting in UE transparent and non-UE transparent schemes.

- UE transparent scheme

  - UL resources muting can be allocated by gNB scheduling. The following Figure 1(a) shows an example of symbol/RB level UL resources muting allocation in UL transparent scheme. Symbol level UL resources muting is that all RBs in symbol are muted such as symbol #5, i.e., nothing is allocated in this symbol. RB level UL resources muting is that RBs in multiple symbols are muted such as RB #1. Therefore, symbol/RB level UL resources muting can be realized by gNB scheduling.

- Non-UE transparent scheme

  - UL resources muting can be allocated by gNB indication (e.g., indicating the unavailable resource for PUSCH). The following Figure 1(b) shows an example of RB/RE level UL resources muting allocation in non-UE transparent scheme. RB level UL resource muting is that some of the RB in symbol are muted such as symbol #8. RE level UL resource muting means is that some of the REs in RB are muted such as symbol #11. If RB/RE level resources muting can be allocated with PUSCH such as Figure 1(b), the total transit power or PSD among symbols within PUSCH varies with and without muting (in following right figure, total transit power or PSD among symbol#8 and #9 varies). It has the issue for higher order QAM reception and the required backoff in PA is increased if PA setting is same over a PUSCH. It means UE design are more complex.

Figure 1 Example of UL resources muting in UE transparent and non-UE transparent schemes

Observation 1: When the number of RE in PUSCH allocation varies between symbols in non-UE transparent scheme, UE design becomes more complex since the total transit power or PSD among symbols within PUSCH varies with and without muting.

Proposal 1: For UL muting resource for gNB-to-gNB CLI measurement, UE transparent scheme should be supported.

Mechanism of L1-based CLI measurement and reporting

In RAN1#112, the following agreements were reached. Further details need to be discussed.

Agreement For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the following measurement and report framework. - Use existing CSI framework as the baseline. - Others are not precluded. Agreement For L1/L2 based UE-to-UE CLI measurement, SRS-RSRP and CLI-RSSI are to be further studied as baseline metrics.

For reusing CSI report framework, CLI measurement metrics can be reported in conjunction with other CSI measurement metrics, i.e., SRS-RSRP or CLI-RSSI is mapped to UCI bits together with other CSI report. The following Figure 2 is an example of conjunction with CSI report for UCI bit sequence generation. UCI bit sequence (a0, a1, ⋯, aA) consists of multiple CSI reports and CLI reports. Multiple measurement results corresponding to measurement resources can be included in one report (e.g., CLI report in Figure 2). In SBFD operation, assuming the case where there can be many aggressor UEs around the victim UE (because aggressor UEs are in the same cell as the victim UE), UCI overhead would be a potential issue. Therefore, details of CLI measurement metrics need to be studied such as report fields of SRS-RSRP/CLI-RSSI in CSI report and configuration of SRS-RSRP/CLI-RSSI resource for CSI report.

Figure 2 Example of conjunction with CSI report

Proposal 2: For L1-based UE-to-UE CLI measurement and reporting, the conjunction between CLI measurement metrics and CSI measurement metrics should be discussed.

Subband-based CLI measurement and reporting for UE-to-UE CLI handling

In RAN1#112, the following proposal was discussed but not agreed.

Moderator Proposal #21-3 (1) For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the benefit of applying narrower frequency granularity of CLI measurement reporting.

UE-to-UE CLI leakage can be non-uniform distribution in frequency domain (orange line in Figure 3). In SBFD operation, DL subband can be affected by non-uniform CLI leakage from adjacent UL subband. In dynamic/flexible TDD operation, DL can be affected by non-uniform CLI leakage from UL transmitted by other cell in adjacent channel. In other case, when DL symbols overlap with UL subband in SBFD symbols in other cell, DL can be affected by non-uniform CLI leakage from UL subband. Rel.16 CLI framework only support wideband-based CLI measurement and reporting (i.e., does not support subband-based CLI measurement and reporting). Rel.16 CLI measurement and reporting are not suitable for reporting non-uniform CLI leakage. Therefore, subband-based CLI measurements and reporting need to be studied. For subband-based CLI measurement, CLI measurement bandwidth can be divided into multiple subbands and UE-to-UE CLI is measured in each subband (blue line in Figure 3). For subband-based CLI reporting, multiple measurement results corresponding to subband can be included in one report. If UE reports the information of non-uniform CLI (e.g., locations of large CLI values in the frequency domain), gNB can determine MCS and allocate DL considering the non-uniform CLI leakage. In order to report CLI values for multiple subbands, how to reduce UCI overhead needs to be discussed.

Proposal 3: Study subband-based CLI measurement and reporting for UE-to-UE CLI handling.

Figure 3 Example of subband-based CLI measurement and reporting

Spatial domain coordination method for UE-to-UE co-channel CLI handling

When beamforming is used, the interference could be dramatically different depending on the beam directions. Consider the Figure 4 as an example. UE #1 may not suffer from strong CLI from UE#2 if UE #1 applies Rx beamforming and UE #2 applies Tx beamforming toward gNB respectively. Therefore, spatial domain enhancement can be considered to facilitate efficient UE paring to avoid UE-to-UE interference. For example, UE can report one or several strongest interfered beam directions together with CLI-RSSI.

Proposal 4: Study how to include spatial domain information to facilitate efficient UE pairing to avoid UE-to-UE CLI

Figure 4 CLI is impacted by beamforming

Regarding how to report spatial domain information, the following options can be considered.

- Option 1: L1-based reporting

  - UE reports the CLI associated with each beam (e.g., SSB index, TCI state) using L1-based reporting. L1-based measurement and reporting are useful for enabling fast beam selection in response to interference variation as compared to L3-based measurement and reporting.

- Option 2: L2-based reporting

  - In Rel-17 eIAB restricted beam indication, the parent node indicates to the child node the beams associated with the SSB/CSI-RS of the child IAB-DU via MAC CE. To adjust/avoid UE-to-UE CLI using restricted beam indication similar to Rel-17 eIAB by gNB. Therefore, if L2-based report from UE is sufficient, the Rel-17 eIAB restricted beam indication could be reused.

Proposal 5: UE-to-UE reporting for spatial domain coordination using L1 or L2 reporting should be studied.

Conclusion

In this contribution, we made the following observation and proposals.

Observation 1: When the number of RE in PUSCH allocation varies between symbols in non-UE transparent scheme, UE design becomes more complex since the total transit power or PSD among symbols within PUSCH varies with and without muting.

Proposal 1: For UL muting resource for gNB-to-gNB CLI measurement, UE transparent scheme should be supported.

Proposal 2: For L1-based UE-to-UE CLI measurement and reporting, the conjunction between CLI measurement metrics and CSI measurement metrics should be discussed.

Proposal 3: Study subband-based CLI measurement and reporting for UE-to-UE CLI handling.

Proposal 4: Study how to include spatial domain information to facilitate efficient UE pairing to avoid UE-to-UE CLI

Proposal 5: UE-to-UE reporting for spatial domain coordination using L1 or L2 reporting should be studied.

- Reference

- RP-221352, “Revised SID: Study on evolution of NR duplex operation”, CMCC