3GPP TSG RAN WG1 #112bis-e		R1-2303177

e-Meeting, April 17th – April 26rd, 2023

Agenda item:	9.17.4

Source:			   Comba

Title:	Discussion on UE features for NCR

Document for:	Discussion

Introduction

In RAN1#112 Meeting [1], the following agreements were achieved.

Agreement Once beam failure is detected in C link by NCR-MT, NCR-Fwd is OFF until the beam failure recovery is completed.

RAN1 will discuss Rel-18 NCR UE features in RAN1-112bis-e. In this contribution, we present our views on other aspects of side control information, including control plane signaling and procedures, UE features for NCR relevant to RAN1.

Discussion

HARQ-ACK feedback for PDCCH

Agreement HARQ-ACK feedback for PDSCH carrying the side control information from higher layer (e.g., MAC-CE, RRC) is supported. The legacy HARQ-ACK feedback mechanism is reused. - FFS: Whether HARQ-ACK feedback for PDCCH carrying side control information is supported Note: This does not mean all legacy HARQ-ACK feedback mechanism will be supported.

In RAN1#110bis-e meeting, HARQ-ACK feedback for PDSCH carrying the side control information was agreed, and the HARQ-ACK feedback for PDCCH was left as an open issue.

Our preference is to support the HARQ-ACK feedback for PDCCH, considering the C-link does not always satisfy the performance requirement for PDCCH carrying side control information, when the NCR is deployed in relative low elevation. Consider the diversity of deployment scenarios, the network can control this feature based on the scene.

Proposal 1: HARQ-ACK feedback for PDCCH carrying side control information is supported, and the network can enable/disable this feature as needed.

Capabilities/features of NCR-MT and NCR-Fwd

There are two kinds of NCR capability, the NCR-MT capability and the NCR-Fwd capability. The NCR-MT capability includes capabilities related to beams in C-link, e.g. SSB/CSI-RS for beam measurement, SRS based beam management, et.al, can reuse the legacy capability reporting mechanism. For capabilities only related to NCR-Fwd, e.g. beam related capabilities for NCR-Fwd, mimo related capabilities for NCR-Fwd, the information is informed to gNB and NCR via OAM.

Proposal 2: The NCR-MT capabilities are informed to NW via RRC signalling, and the only NCR-Fwd capabilities are informed to NW via OAM.

Rel-18 NCR feature list

According to the previous agreement for NCR, the following feature list can be considered as a starting point for discussion:

Table 1: Rel-18 NCR feature list for discussion

| Features | Index | Feature group | Components | Note | Mandotory/ Optional |
| --- | --- | --- | --- | --- | --- |
| NCR | 1 |  | Support simultaneous UL transmission of C-link and backhaul link | NCR-MT impact | Optional with capability signaling |
| NCR | 2 |  | Support adaptive beam for C-link/backhaul-link | NCR-MT impact | Optional with capability signaling |
| NCR | 3 |  | Support dynamic beam indication for NCR-Fwd access link | NCR-MT impact | Optional with capability signaling |
| NCR | 4 |  | Support semi-static beam indication for NCR-Fwd access link | NCR-MT impact | Optional with capability signaling |
| NCR | 5 |  | - Support the aperiodic beam indication for NCR-Fwd access link - Slot offset k | NCR-MT impact | Optional with capability signaling |
| NCR | 6 |  | Support ON/OFF information explicit indication | NCR-MT impact | Optional with capability signaling |
| NCR | 7 |  | Support ON/OFF information implicit indication | NCR-MT impact | Optional with capability signaling |
| NCR | 8 |  | Support HARQ-ACK for PDCCH carrying SCI | NCR-MT impact | Optional with capability signaling |

Proposal 3: Consider Table 1 as the starting point for discussion for Rel-18 NCR feature list.

Conclusions

In this contribution, we present the discussion on side control information to enable NR network-controlled repeaters, UE features for NCR relevant to RAN1, we have the following proposals:

Proposal 1: HARQ-ACK feedback for PDCCH carrying side control information is supported, and the network can enable/disable this feature as needed.

Proposal 2: The NCR-MT capabilities are informed to NW via RRC signalling, and the only NCR-Fwd capabilities are informed to NW via OAM.

Proposal 3: Consider Table 1 as the starting point for discussion for Rel-18 NCR feature list.

- References

[1] Draft Report of 3GPP TSG RAN WG1 #112 v0.2.0, Athens, Greece, 27th February -3rd March 2023

[2] RP-223505, “Revised WID on NR network-controlled repeaters,” ZTE, Sanechips, Dec. 2022

[3] R1-2301832, “Summary#1 of discussion on other aspects,” Moderator (Fujitsu)

[4] R1-2300703, “Discussion on other aspects for NCR,” ZTE

[5] Draft Report of 3GPP TSG RAN WG1 #110bis-e v0.2.0, Online, 10th - 19rd October 2022

[6] Draft Report of 3GPP TSG RAN WG1 #111 v0.2.0, Toulouse, France, 14th -18rd November 2022