3GPP TSG RAN WG1 #112bis-e	R1-2303178

e-Meeting, April 17th – 26th, 2023

Source:	Sharp

Title:	Unified TCI framework extension for multi-TRP

Agenda Item:	9.1.1.1

Document for:	Discussion and Decision

Background

In RAN#94e, the working item to enhance both downlink and uplink MIMO operations in Rel-18 was agreed [1].

According to the WID, the following item needs to be studied, and if justified, specified:

- Extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

- the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

- Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

Discussions

  - Multi-DCI based MTRP

Mapping of the indicated TCI state

For PDCCH, it was agreed that the indicated joint/DL TCI state specific to a CORESET pool index is applied to a PDCCH on a CORESET associated with the same CORESET pool index.

For PDSCH/PUSCH, it was agreed that the indicated TCI state specific to a CORESET pool index is applied to a PDSCH/PUSCH scheduled/activated by a PDCCH on a CORESET associated with the same CORESET pool index.

PUCCH

Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set () or the second BFD-RS set () when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set () and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported

For Opt2, the RRC configuration in a PUCCH resource informs “first” or “second”, where “first” corresponds to CORESET pool index 0 and “second” corresponds to CORESET pool index 1. Therefore, in our view, it is simpler that one of CORESET pool index values is directly configured in each PUCCH resource. Namely, we prefer Opt1 to Opt2. However, we have a concern that PUCCH resources are consumed due to configuring CORESET pool index. Straightforwardly, the number of PUCCH resources for a given CORESET pool index is halved.

For Opt3, some companies raised issues of a case that ackNackFeedbackMode = joint. In the case, the UE transmits a PUCCH with a HARQ-ACK for two PDSCHs to first of two TRPs even if both PDSCH are received from both TRPs. However, for multi-DCI based MTRP, ackNackFeedbackMode set to “joint” is corner case because MTRP is with non-ideal backhaul. Therefore, when ackNackFeedbackMode = joint, it is enough to apply the legacy/fixed rule. Namely, the indicated joint/UL TCI state specific to the first CORESET pool index (i.e., the CORESET pool index value 0) needs to be applied to a PUCCH with a HARQ-ACK scheduled by a PDCCH.

Observation 1: For multi-DCI based MTRP, since it is corner case that ackNackFeedbackMode = “joint”, in this case, it is enough that fixed rules is applied to a PUCCH (e.g., the indicated joint/UL TCI state specific to the CORESET pool index value 0 is applied to a PUCCH scheduled by a PDCCH).

For these reasons, we support Opt3 when a PUCCH is scheduled by a PDCCH.

Proposal 1: For multi-DCI based MTRP, the indicated TCI state specific to a CORESET pool index should be applied to a PUCCH scheduled by a PDCCH on a CORESET associated with the same CORESETE pool index.

Otherwise (i.e., if the PUCCH is not scheduled by a PDCCH), we prefer Opt1 to Opt2.

Proposal 2: For multi-DCI based MTRP, the indicated TCI state specific to a CORESET pool index should be applied to a PUCCH on a PUCCH resource configured with the same CORESET pool index, if the PUCCH is not scheduled by a PDCCH.

PUSCH

For Type 1 CG-PUSCH, we think there are two alternatives for mapping of the indicated TCI state as the following:

- Alt 1: The indicated TCI state specific to a CORESET pool index is applied to a Type 1 CG-PUSCH corresponding to a CG configuration associated with the same CORESET pool index.

- Alt 2: The indicated TCI state specific to a CORESET pool index is applied to a Type 1 CG-PUSCH corresponding to a CG configuration including SRS_resource_set_index of a SRS resource set associated with the same CORESET pool index.

In AI 9.1.4.1, it was agreed that one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig. Therefore, this SRS resource set index can be simply reused, and we support Alt 2.

Proposal 3: The indicated TCI state specific to a CORESET pool index is applied to a Type 1 CG-PUSCH corresponding to a ConfiguredGrantConfig including SRS_resource_set_index of a SRS resource set associated with the same CORESET pool index.

SRS

For SRS, it was agreed that the first SRS resource set is associated with CORESET pool index 0, and the second SRS resource set is associated with CORESET pool index 1 in AI9.1.4.1. Likewise, for unified TCI framework extension for multi-DCI based MTRP, the indicated TCI state specific to a CORESET pool index should be applied to a SRS on a SRS resource set corresponding to the same CORESET pool index if the SRS resource set is configured to follow the indicated joint/UL TCI state by followingUnifiedTCIState.

Proposal 4: For multi-DCI based MTRP, the indicated TCI state specific to a CORESET pool index should be applied to a SRS on a SRS resource set corresponding to the same CORESET pool index if the SRS resource set is configured to follow the indicated joint/UL TCI state by followingUnifiedTCIState.

  - Single-DCI based MTRP

TCI state activation and indication

In RAN1#111 meeting, the following agreement was made in terms of single-DCI based MTRP [2]:

Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS

In our view, increase on the size of the TCI field is not needed and up to 8 single or pair of TCI state(s) are enough in one codepoint of a TCI field. This is because MAC entity can flexibly select 8 combinations of TCI states among all configured TCI states, and we think it is not significant problem that a TCI state cannot be indicated among more 8 combinations by DCI.

Proposal 5: The size of the TCI field should remain 3 bits.

Furthermore, it was agreed that a TCI field in DCI format 1_1/1_2 can indicate TCI state(s) for one or both of the two TRPs. In our view, the number of TCI states indicated in the TCI field should not intend the switching between STRP and MTRP modes. This is because all DL and/or UL channels are forcibly switched into MTRP mode even if the part of the DL and/or UL channels does not support MTRP scheme (e.g., SFN, SDM, STxMP, etc) due to the switching. For this reason, we don’t support that the switching between STRP and MTRP depends on the number of TCI states indicated in a beam indication DCI.

Proposal 6: We don’t support that the switching between STRP and MTRP depends on the number of TCI states indicated in a beam indication DCI.

For TCI state indication, there are two alternatives for update of the indicated TCI state as the following.

- Alt 1: The indicated TCI state(s) are applied, and the previous indicated TCI state(s) are discarded.

- Alt 2: The indicated TCI state(s) are applied, and the previous indicated TCI state(s) that are updated by the indicated TCI state(s) are discarded.

In our view, Alt 2 is reasonable because for separate DL/UL TCI mode, it is allowed that either of DL or UL TCI state(s) are independently updated.

Proposal 7: For single-DCI based MTRP, if the UE receives a TCI codepoint mapped with a sub-set for TCI state(s), the UE should apply the corresponding TCI state(s) in the sub-set and keep the currently applied TCI state(s) not in the sub-set.

Mapping of indicated TCI state

After activation of multiple joint/DL/UL TCI states by MAC CE, a TCI field in DCI format 1_1/1_2 with/without data scheduling can indicate one or two joint/DL/UL TCI states. However, when two UL TCI states are indicated and UL channel is scheduled, it is unknown that whether the UL channel apply to both or either of two the UL TCI states. For SDCI-MTRP, the mapping of the indicated TCI state to channels is needed to be defined.

PDCCH

Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported)

It was agreed that one or multiple TCI states are indicated in a CC/BWP by a TCI field in DCI. Namely, for one or more CORESETs in a BWP, one or multiple TCI state can be indicated. Furthermore, it was agreed to support to use RRC parameter(s) in a CORESET configuration to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH reception on the CORESET. In our view, the RRC configuration should inform that the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states on a CORESET, not a group of CORESETs.

Proposal 8: For single-DCI based MTRP, RRC configuration should inform that the UE shall apply the first one, second one, both, or none of the indicated joint/DL TCI states on a CORESET, not a group of CORESETs.

According to the following current spec, the application of the indicated TCI state depends on a search space set. Likewise, by reusing the legacy design, the application of the indicated TCI state(s) should depend on a search space set.

38.213 Clause 10.1 … If a UE is provided TCI-State in dl-OrJoint-TCIStateList, a DM-RS antenna port for PDCCH receptions in a CORESET, other than a CORESET with index 0, associated only with USS sets and/or Type3-PDCCH CSS sets, and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with reference signals provided by the indicated TCI-State [6, TS 38.214]. If a UE is provided followUnifiedTCIstate for a CORESET, other than a CORESET with index 0, associated at least with CSS sets other than Type3-PDCCH CSS sets, and if followUnifiedTCIstate is set as enabled, a DM-RS antenna port for PDCCH receptions in the CORESET and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with reference signals provided by the indicated TCI-State.

Proposal 9: For single-DCI based MTRP, if a CORESET other than a CORESET with index 0 is associated only with USS sets and/or Type3-PDCCH CSS sets, the CORESET should be configured by RRC to apply the first indicated joint/DL TCI state, the second indicated joint/DL TCI state, or both first and second indicated joint/DL TCI states to PDCCH reception on the CORESET.

Proposal 10: For single-DCI based MTRP, if a CORESET other than a CORESET with index 0 is associated at least with CSS sets other than Type3-PDCCH CSS sets and followUnifiedTCIstate = 'enabled' is configured for the CORESET, the CORESET is configured by RRC to apply the first indicated joint/DL TCI state, the second indicated joint/DL TCI state, or both first and second indicated joint/DL TCI states to PDCCH reception on the CORESET.

However, according to the following Rel-17 agreement, the UE does not expect that a CORESET with index 0 to be activated with two TCI states is associated with search space zero for Type 0/0A/2 CSS. Likewise, two indicated joint/DL TCI states should not be applied to the CORESET.

Agreement UE does not expect CORESET#0 to be activated with two TCI states when it is associated with SS#0 for Type 0/0A/2 CSS - Send an LS to inform RAN2 about this agreement - The final LS in R1-2208203

Proposal 11: For single-DCI based MTRP, if a CORESET with index 0 is configured with followUnifiedTCIstate = 'enabled' and associated with search space zero for Type 0/0A/2 CSS sets, the CORESET is configured by RRC to apply the first indicated joint/DL TCI state or the second indicated joint/DL TCI state to PDCCH reception on the CORESET.

PDCCH-MTRP scheme

[PDCCH repetition]

For search space linking (i.e., searchSpaceLinking configuration) for SDCI-MTRP, UE assumes that search space sets having the same search space linking ID are linked. Consequently, UE combines the two PDCCHs associated with the same search space linking ID after monitoring. Furthermore, one search space set is associated with one CORESET. Therefore, if more than one the indicated TCI states are applied to a CORESET, two beam indications by search space linking and Rel-18 Unified TCI framework for MTRP are duplicated. For this reason, if the single indicated TCI state is applied to the CORESET (i.e., RRC parameter in a CORESET configuration informs the UE which indicated joint/DL TCI state is applied to the corresponding PDCCH reception on the CORESET), UE should receive PDCCH repetition or a PDCCH, not PDCCH SFN.

[PDCCH SFN]

For PDCCH SFN, two joint/DL TCI states are activated by MAC CE. The UE receives two PDCCHs that have the same DCI in the same frequency/time resource based on the two activated TCI states.  Therefore, if the single indicated TCI state is applied to a CORESET, the UE cannot receive two PDCCHs with using SFN scheme. For this reason, if the multiple indicated TCI states are applied to the CORESET (i.e., RRC parameter in a CORESET configuration informs the UE that the indicated joint/DL TCI states are applied to the corresponding PDCCH reception on the CORESET), UE should receive PDCCH SFN, not PDCCH repetition and a PDCCH.

Proposal 12: For single-DCI based MTRP,

- If the single indicated TCI state is applied to a CORESET, the UE should receive a PDCCH repetition or STRP-PDCCH on the CORESET.

- If the multiple indicated TCI state is applied to a CORESET, the UE should receive a PDCCH SFN on the CORESET.

  - FFS: UE behavior when the single indicated TCI state is applied to a CORESET and PDCCH SFN is enabled.

PDSCH

Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value Agreement On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed)

For PDSCH, the following issues remain:

- DCI format 1_1/1_2

  - Offset >= threshold

    - w/ TCI selection field

      - if a PDSCH-MTRP scheme is enabled,

        - the single indicated TCI state is applied or not (Case 1).

    - w/o TCI selection field (Case 2)

      - Not yet

  - Offset < threshold (Case 3)

    - Not yet

- DCI format 1_0 (Case 4)

  - Not yet

(Case 1) w/ TCI selection field

[PDSCH FDM/TDM/SDM]

For SDCI-MTRP PDSCH FDM/TDM, two indicated TCI states can be applied to a PDSCH for FDM/TDM scheme A or two PDSCHs for FDM/TDM scheme B. For SDCI-MTRP PDSCH SDM, two indicated TCI states can be applied to a PDSCH with more than one CDM groups.

For PDSCH FDM scheme A/B, the UE receives a single or two PDSCH transmission occasion(s) by using two TCI states. To reuse Rel-16 rules, the first and second selected TCI states should be applied to the first and other non-overlapping frequency domain resources of the PDSCH transmission occasion(s), respectively. The selected TCI state means either or both indicated TCI states selected by the 2-bit TCI selection field.

For PDSCH TDM scheme A, the UE receives two PDSCH transmission occasions by using two TCI states. To reuse Rel-16 rules, the first and second selected TCI states should be applied to the first and other PDSCH transmission occasions, respectively.

For PDSCH SDM scheme, the UE receives a single PDSCH transmission occasion with more than one CDM groups by using two TCI states. To reuse Rel-16 rules, the first and second selected TCI states should be applied to the first and other CDM groups, respectively.

Proposal 13: For single-DCI based MTRP, the first and second selected TCI states should be mapped to

- the first and other non-overlapping frequency domain resources for PDSCH FDM scheme A/B, respectively.

- the first and other PDSCH transmission occasions for PDSCH TDM scheme A, respectively.

- the first and other CDM groups for PDSCH SDM scheme, respectively.

[PDSCH repetition]

For SDCI-MTRP PDSCH repetition, one or two indicated TCI state can be applied to PDSCH repetitions over multiple slots.

[PDSCH SFN]

For SDCI-MTRP PDSCH SFN with dynamic switching, one or two indicated TCI states can be applied to a PDSCH. For SDCI-MTRP PDSCH SFN without dynamic switching, two indicated TCI states can be applied to a PDSCH.

Proposal 14: For single-DCI based MTRP,

- If the single indicated TCI state is applied to the PDSCH(s), the UE should receive a PDSCH repetition or STRP-PDSCH.

- If the multiple indicated TCI state is applied to the PDSCH, the UE should receive a PDSCH repetition, or PDSCH SFN/FDM/TDM/SDM.

  - FFS: UE behavior when the single indicated TCI state is applied to a PDSCH and PDSCH SFN/FDM/TDM/SDM is enabled.

(Case 2) w/o TCI selection field

For Alt 2 and 3, the fixed rule is used to select one or both of the indicated joint/DL TCI state(s). However, the PDSCH-MTRP schemes (e.g., SFN/FDM/TFM/SDM) cannot be enabled in Alt 2, and the PDSCH-MTRP schemes are always applied in Alt 3. In our view, Alt 2 and Alt 3 are simple but not complete solutions. Therefore, at least Alt 2 and Alt 3 should be combined and be switched according to whether the PDSCH-MTRP schemes are enabled or not.

SFN and non-SFN can be dynamically switched subject to the UE capability. Therefore, any of Alt 2 and Alt 3 is not complete solution.

For Alt 4, the existing TCI field is used for not only PDSCH, and the existing TCI field can update the indicated TCI state(s) for all DL and/or UL channels/signals. For this reason, the existing TCI field should not be used to switch one of STRP and MTRP operations for only PDSCH.

For Alt 3A, the application of the indicated joint/DL TCI state(s) can be dynamically determined, but there is a misalignment case that PDCCH-SFN is enabled and PDSCH-SFN is not enabled. For this case, the indicated joint/DL TCI state(s) applied for the PDCCH should not be applied for the PDSCH.

Therefore, we think Alt 1 and the combined Alt 2 and Alt 3 are reasonable. Considering spec impacts, we slightly prefer Alt 1 to the combined Alt 2 and Alt 3.

Proposal 15: (1st preference) For single-DCI based MTRP, if a PDSCH is scheduled/activated by a DCI format 1_1/1_2 without the TCI selection field, a RRC configuration should inform that the UE shall apply the first one, the second one, or both of the indicated joint/DL TCI states to the scheduled/activated PDSCH reception (Alt 1).

Proposal 16: (2nd preference) For single-DCI based MTRP, if a PDSCH is scheduled/activated by a DCI format 1_1/1_2 without the TCI selection field,

- The UE shall apply the first indicated joint/DL TCI state to the PDSCH if the PDSCH SFN/FDM/TDM/SDM is not enabled (Alt 2).

- The UE shall apply both indicated joint/DL TCI states to the PDSCH if the PDSCH SFN/DM/TDM/SDM is enabled (Alt 3).

(Case 3) Offset < threshold and (Case 4) DCI format 1_0

To simplify the Rel-18 design, the default indicated TCI state(s) should be applied to a PDSCH in Case 3 and Case 4. Namely, the default indicated TCI state(s) should be explicitly or implicitly determined by a RRC configuration. For explicit determination, the RRC configuration informs that the default indicated TCI state(s) are the first one, the second one, or both of the indicated joint/DL TCI states. For implicit determination, the default indicated TCI state is the first indicated joint/DL TCI state if the PDSCH SFN/FDM/TDM/SDM is not enabled, and the default indicated TCI states are both indicated joint/DL TCI states if the PDSCH SFN/FDM/TDM/SDM is enabled.

Proposal 17: Default indicated TCI state(s) should be applied to a PDSCH in the following cases:

- The TCI selection field in a DCI format 1_1/1_2 scheduling the PDSCH is absent,

- The PDSCH is scheduled by a DCI format 1_0, and

- The offset between a reception of a DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold.

Furthermore, timeDurationForQCL defined in Rel-15 can be reused as the threshold. Since the application of the indicated TCI state(s) is done during beamAppTime in Rel-17, we think there is no additional time over head.

Proposal 18: For unified TCI framework extension, timeDurationForQCL in Rel-15 should be reused as a threshold.

Furthermore, regarding a beam application time, we think there is no additional time overhead. Therefore, for unified TCI framework extension, the indicated joint/DL/UL TCI state(s) should be updated based on beamAppTime in Rel-17.

Proposal 19: For unified TCI framework extension, the indicated joint/DL/UL TCI state(s) should be updated based on beamAppTime in Rel-17.

PUSCH

Agreement On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

PUSCH-MTRP scheme

[PUSCH repetition]

For SDCI-MTRP PUSCH repetition, one or two indicated TCI state(s) can be applied to a PUSCH over multiple slots. If two indicated TCI state(s) are applied to a PUSCH over multiple slots, TCI state mapping pattern depends on cycling mapping or sequential mapping.

[PUSCH STxMP]

In our view, for SDCI-MTRP PUSCH STxMP, two indicated TCI states can be applied to a PUSCH as SFN/SDM scheme.

Proposal 20: For single-DCI based MTRP,

- If the single indicated TCI state is applied to PUSCH(s), the UE should receive PUSCH repetition or a STRP-PUSCH.

- If the multiple indicated TCI state is applied to PUSCH(s), the UE should receive PUSCH repetition, or PUSCH STxMP.

  - FFS: UE behavior when the single indicated TCI state is applied to PUSCH(s) and PUSCH STxMP is enabled.

Spatial Tx filter determination for PUSCH and SRS

The UE transmits multiple SRS using spatial Tx filters, and the gNB feedback the SRS resource(s) to indicate one or two of the spatial Tx filters for PUSCH transmission. If an indicated joint/UL TCI state are applied to each SRS resource set, the SRS resource set indicator field in a DCI format 0_1/0_2 scheduling a PUSCH can select one or two SRS resource sets for the PUSCH. However, if the indicated joint/UL TCI state(s) are updated between a SRS transmission and a PUSCH transmission, the spatial Tx filter(s) used for the SRS is different from the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to the PUSCH. Since two spatial Tx filters between SRS and PUSCH are different, two channel states between SRS and PUSCH are also different. Therefore, after updating the indicated joint/UL TCI state(s), the SRS needs to be transmitted to perform channel sounding, link adaptation, and scheduling at the gNB side.

For a simple solution, the indicated joint/UL TCI state(s) should not be updated between a SRS transmission and a PUSCH transmission. If needed, when a DCI format 1_0/1_1 indicates to update the indicated joint/UL TCI state(s), the DCI format should request an aperiodic SRS such that the SRS is transmitted after the application of the indicated joint/UL TCI state(s).

Proposal 21: The indicated joint/UL TCI state(s) should not be updated between a SRS transmission and a PUSCH transmission.

Proposal 22: When a DCI format 1_0/1_1 indicates to update the indicated joint/UL TCI state(s), the DCI format should request an aperiodic SRS.

Power control

Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS

The indicated joint/UL TCI state(s) are NOT associated with an UL PC parameter setting

If one or both of the indicated joint/UL TCI state(s) applied to PUSCH/PUCCH/SRS transmission don’t include the UL PC parameter setting(s) for PUSCH/PUCCH/SRS, the default UL PC parameter setting in Rel-17 is not proper for two TRPs. Therefore, we can support two default UL PC parameter settings for PUSCH/PUCCH/SRS configured in BWP-UplinkDedicated.

Proposal 23: On unified TCI framework extension, if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUSCH/PUCCH/SRS, support two default UL PC parameter settings for PUSCH/PUCCH/SRS configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter settings for PUSCH/PUCCH/SRS configured in the corresponding UL BWP.

The indicated joint/UL TCI state(s) are associated with an UL PC parameter setting

For STxMP, if one closed-loop index is used for two simultaneous PUSCH transmissions, two UL PC parameter settings and two PL-RSs included in two indicated joint/UL TCI states are overlapped, and either of both needs to be dropped/ignored. To avoid this, two indicated joint/UL TCI states should have two closed loop indexes (i.e., l=0,1) if both indicated joint/UL TCI states are associated with UL PC parameter settings and PL-RSs. Regarding association of the indicated joint/UL TCI state with a closed loop index, “first” and “second” should simply correspond to l=0 and l=1.

Proposal 24: For single-DCI based MTRP, two indicated joint/UL TCI states should have two closed loop indexes (i.e., l=0,1) if both indicated joint/UL TCI states are associated with UL PC parameters.

Proposal 25: For unified TCI framework extension, the first and second indicated joint/UL TCI states should be associated with the first closed loop index (i.e., l=0) and the second closed loop index (i.e., l=1), respectively.

  - Others

CJT

Agreement On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT - Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1 Agreement On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework Agreement On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB

For CJT-based PDSCH, we think it depends on a scenario such as a large cell scenario, cell-free scenario, and so on. For this reason, we prefer to enable PDSCH-CJT and switching between other MTRP schemes via RRC.

Proposal 26: PDSCH-CJT should be enabled via RRC.

In our view, for PDSCH-CJT, there are two alternatives for QCL assumptions/types of both indicated joint TCI states as the following:

- Alt 1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA

- Alt 2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state

If the same DMRS port is QCLed with multiple DL RSs, we think the UE receives the same PDSCH DMRS for multiple reception processes by using multiple large-scale properties, and then combines the multiple received PDSCH DMRSs. For Alt 2, if the UE has the high speed, the doppler shift and doppler spread are estimated based on a TRP-specific TRS, and for the PDSCH-CJT transmission from multiple TRPs, one of large-scale parameters for multiple TRPs has no QCL assumption of doppler shift and doppler spread. However, we are still open to discuss about a use case of Alt 2, and at least support Alt 1 so far.

Proposal 27: On unified TCI framework extension, for PDSCH-CJT applying both indicated joint TCI states, we support at least that PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA.

Conclusion

In this contribution, we have the following proposal:

Observation 1: For multi-DCI based MTRP, since it is corner case that ackNackFeedbackMode = “joint”, in this case, it is enough that fixed rules is applied to a PUCCH (e.g., the indicated joint/UL TCI state specific to the CORESET pool index value 0 is applied to a PUCCH scheduled by a PDCCH).

Proposal 1: For multi-DCI based MTRP, the indicated TCI state specific to a CORESET pool index should be applied to a PUCCH scheduled by a PDCCH on a CORESET associated with the same CORESETE pool index.

Proposal 2: For multi-DCI based MTRP, the indicated TCI state specific to a CORESET pool index should be applied to a PUCCH on a PUCCH resource configured with the same CORESET pool index, if the PUCCH is not scheduled by a PDCCH.

Proposal 3: The indicated TCI state specific to a CORESET pool index is applied to a Type 1 CG-PUSCH corresponding to a ConfiguredGrantConfig including SRS_resource_set_index of a SRS resource set associated with the same CORESET pool index.

Proposal 4: For multi-DCI based MTRP, the indicated TCI state specific to a CORESET pool index should be applied to a SRS on a SRS resource set corresponding to the same CORESET pool index if the SRS resource set is configured to follow the indicated joint/UL TCI state by followingUnifiedTCIState.

Proposal 5: The size of the TCI field should remain 3 bits.

Proposal 6: We don’t support that the switching between STRP and MTRP depends on the number of TCI states indicated in a beam indication DCI.

Proposal 7: For single-DCI based MTRP, if the UE receives a TCI codepoint mapped with a sub-set for TCI state(s), the UE should apply the corresponding TCI state(s) in the sub-set and keep the currently applied TCI state(s) not in the sub-set.

Proposal 8: For single-DCI based MTRP, RRC configuration should inform that the UE shall apply the first one, second one, both, or none of the indicated joint/DL TCI states on a CORESET, not a group of CORESETs.

Proposal 9: For single-DCI based MTRP, if a CORESET other than a CORESET with index 0 is associated only with USS sets and/or Type3-PDCCH CSS sets, the CORESET should be configured by RRC to apply the first indicated joint/DL TCI state, the second indicated joint/DL TCI state, or both first and second indicated joint/DL TCI states to PDCCH reception on the CORESET.

Proposal 10: For single-DCI based MTRP, if a CORESET other than a CORESET with index 0 is associated at least with CSS sets other than Type3-PDCCH CSS sets and followUnifiedTCIstate = 'enabled' is configured for the CORESET, the CORESET is configured by RRC to apply the first indicated joint/DL TCI state, the second indicated joint/DL TCI state, or both first and second indicated joint/DL TCI states to PDCCH reception on the CORESET.

Proposal 11: For single-DCI based MTRP, if a CORESET with index 0 is configured with followUnifiedTCIstate = 'enabled' and associated with search space zero for Type 0/0A/2 CSS sets, the CORESET is configured by RRC to apply the first indicated joint/DL TCI state or the second indicated joint/DL TCI state to PDCCH reception on the CORESET.

Proposal 12: For single-DCI based MTRP,

- If the single indicated TCI state is applied to a CORESET, the UE should receive a PDCCH repetition or STRP-PDCCH on the CORESET.

- If the multiple indicated TCI state is applied to a CORESET, the UE should receive a PDCCH SFN on the CORESET.

  - FFS: UE behavior when the single indicated TCI state is applied to a CORESET and PDCCH SFN is enabled.

Proposal 13: For single-DCI based MTRP, the first and second selected TCI states should be mapped to

- the first and other non-overlapping frequency domain resources for PDSCH FDM scheme A/B, respectively.

- the first and other PDSCH transmission occasions for PDSCH TDM scheme A, respectively.

- the first and other CDM groups for PDSCH SDM scheme, respectively.

Proposal 14: For single-DCI based MTRP,

- If the single indicated TCI state is applied to the PDSCH(s), the UE should receive a PDSCH repetition or STRP-PDSCH.

- If the multiple indicated TCI state is applied to the PDSCH, the UE should receive a PDSCH repetition, or PDSCH SFN/FDM/TDM/SDM.

  - FFS: UE behavior when the single indicated TCI state is applied to a PDSCH and PDSCH SFN/FDM/TDM/SDM is enabled.

Proposal 15: (1st preference) For single-DCI based MTRP, if a PDSCH is scheduled/activated by a DCI format 1_1/1_2 without the TCI selection field, a RRC configuration should inform that the UE shall apply the first one, the second one, or both of the indicated joint/DL TCI states to the scheduled/activated PDSCH reception (Alt 1).

Proposal 16: (2nd preference) For single-DCI based MTRP, if a PDSCH is scheduled/activated by a DCI format 1_1/1_2 without the TCI selection field,

- The UE shall apply the first indicated joint/DL TCI state to the PDSCH if the PDSCH SFN/FDM/TDM/SDM is not enabled (Alt 2).

- The UE shall apply both indicated joint/DL TCI states to the PDSCH if the PDSCH SFN/DM/TDM/SDM is enabled (Alt 3).

Proposal 17: Default indicated TCI state(s) should be applied to a PDSCH in the following cases:

- The TCI selection field in a DCI format 1_1/1_2 scheduling the PDSCH is absent,

- The PDSCH is scheduled by a DCI format 1_0, and

- The offset between a reception of a DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold.

Proposal 18: For unified TCI framework extension, timeDurationForQCL in Rel-15 should be reused as a threshold.

Proposal 19: For unified TCI framework extension, the indicated joint/DL/UL TCI state(s) should be updated based on beamAppTime in Rel-17.

Proposal 20: For single-DCI based MTRP,

- If the single indicated TCI state is applied to PUSCH(s), the UE should receive PUSCH repetition or a STRP-PUSCH.

- If the multiple indicated TCI state is applied to PUSCH(s), the UE should receive PUSCH repetition, or PUSCH STxMP.

  - FFS: UE behavior when the single indicated TCI state is applied to PUSCH(s) and PUSCH STxMP is enabled.

Proposal 21: The indicated joint/UL TCI state(s) should not be updated between a SRS transmission and a PUSCH transmission.

Proposal 22: When a DCI format 1_0/1_1 indicates to update the indicated joint/UL TCI state(s), the DCI format should request an aperiodic SRS.

Proposal 23: On unified TCI framework extension, if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUSCH/PUCCH/SRS, support two default UL PC parameter settings for PUSCH/PUCCH/SRS configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter settings for PUSCH/PUCCH/SRS configured in the corresponding UL BWP.

Proposal 24: For single-DCI based MTRP, two indicated joint/UL TCI states should have two closed loop indexes (i.e., l=0,1) if both indicated joint/UL TCI states are associated with UL PC parameters.

Proposal 25: For unified TCI framework extension, the first and second indicated joint/UL TCI states should be associated with the first closed loop index (i.e., l=0) and the second closed loop index (i.e., l=1), respectively.

Proposal 26: PDSCH-CJT should be enabled via RRC.

Proposal 27: On unified TCI framework extension, for PDSCH-CJT applying both indicated joint TCI states, we support at least that PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA.

References

- [1]	RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Dec., 2021.

- [2]	RAN1 chair’s note, RAN1#112 e-meeting, Feb. 2023.