3GPP TSG RAN WG1 #112bis-e	R1-2303181

e-Meeting, April 17th – 26th, 2023

Source:	Sharp

Title:	SRS enhancement targeting TDD CJT and 8 TX operation

Agenda Item:	9.1.3.2

Document for:	Discussion and Decision

Background

In RAN#94e, the working item to enhance both downlink and uplink MIMO operations in Rel-18 was agreed [1].

According to the WID, the following item needs to be studied, and if justified, specified:

- enhancement of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints the 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

- SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle Industrial devices

Potential enhancements for 8Tx SRS

In up to RAN1#112 meeting, the following agreements were made in AI 9.1.3.2 for SRS enhancement [2].

Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), - Option 2: For comb 4, do not support 4 comb offsets.

  - Mapping of 8 SRS ports onto one OFDM symbol

For an 8-port SRS resource in one OFDM symbol, legacy mechanisms (i.e., FDM + Cyclic shift) can be reused as the followings:

|  | Comb 2 Max cyclic shift: 8 | Comb 4 Max cyclic shift: 12 | Comb 8 Max cyclic shift: 6 |
| --- | --- | --- | --- |
| 1 comb offset | 8 cyclic shift values |  |  |
| 2 comb offsets | 4 or 8 cyclic shift values | 4 cyclic shift values |  |
| 4 comb offsets |  |  | 2 cyclic shift values |

Cyclic shift

For the current spec, one cyclic shift value for one or multiple SRS ports is determined and SRS ports with having different cyclic shift values are multiplexed.

For comb 2, the legacy cyclic shift value determination can be reused as below:

nSRSCS,i=nSRSCS+nSRSCS,max(pi-1000)NapSRS mod nSRSCS,max, or

nSRSCS,i=nSRSCS+nSRSCS,max(pi-1000)/2NapSRS/2 mod nSRSCS,max.

For comb 4, the legacy cyclic shift value determination can be reused as below:

nSRSCS,i=nSRSCS+nSRSCS,max(pi-1000)/2NapSRS/2 mod nSRSCS,max.

For comb 8, since the maximum cyclic shift is 6 and it cannot be divided into 4, the enhancement on the cyclic shift value nSRSCS,i determination for comb 8 is needed. The cyclic shift value nSRSCS,i for each antenna port pi can be calculated as the following:

nSRSCS,i=nSRSCS+nSRSCS,max(pi-1000)/4NapSRS/4 mod nSRSCS,max.

Proposal 1: For mapping of 8 SRS ports onto one OFDM symbol, the cyclic shift value nSRSCS,i for comb 8 should be calculated as the following:

- nSRSCS,i=nSRSCS+nSRSCS,max(pi-1000)/4NapSRS/4 mod nSRSCS,max

FDM

For cyclic shift determination, some subsets of SRS ports have the same cyclic shift value. For this reason, the SRS ports within each subset need to have different comb offset values to multiplex on the frequency domain. For example, each SRS port can have the following cyclic shift value and comb offset value:

- Comb 2 (Case 1)

  - Cyclic shift value 0: {1000}

  - Cyclic shift value 1: {1001}

  - Cyclic shift value 2: {1002}

  - Cyclic shift value 3: {1003}

  - Cyclic shift value 4: {1004}

  - Cyclic shift value 5: {1005}

  - Cyclic shift value 6: {1006}

  - Cyclic shift value 7: {1007}

- Comb 2 (Case 2)

  - Cyclic shift value 0: {1000, 1001}

    - Comb offset value 0: {1000}

    - Comb offset value 1: {1001}

  - Cyclic shift value 1: {1002, 1003}

    - Comb offset value 0: {1002}

    - Comb offset value 1: {1003}

  - Cyclic shift value 2: {1004, 1005}

    - Comb offset value 0: {1004}

    - Comb offset value 1: {1005}

  - Cyclic shift value 3: {1006, 1007}

    - Comb offset value 0: {1006}

    - Comb offset value 1: {1007}

- Comb 4

  - Cyclic shift value 0: {1000, 1001}

    - Comb offset value 0: {1000}

    - Comb offset value 1: {1001}

  - Cyclic shift value 1: {1002, 1003}

    - Comb offset value 0: {1002}

    - Comb offset value 1: {1003}

  - Cyclic shift value 2: {1004, 1005}

    - Comb offset value 0: {1004}

    - Comb offset value 1: {1005}

  - Cyclic shift value 3: {1006, 1007}

    - Comb offset value 0: {1006}

    - Comb offset value 1: {1007}

- Comb 8

  - Cyclic shift value 0: {1000, 1001, 1002, 1003}

    - Comb offset value 0: {1000}

    - Comb offset value 1: {1001}

    - Comb offset value 2: {1002}

    - Comb offset value 3: {1003}

  - Cyclic shift value 1: {1004, 1005, 1006, 1007}

    - Comb offset value 0: {1004}

    - Comb offset value 1: {1005}

    - Comb offset value 2: {1006}

    - Comb offset value 3: {1007}

For comb 2 (Case 2), the comb offset value can be calculated as:

kTCpi=kTC+KTC/2 mod KTCif pi∈{1001, 1003, 1005,1007}kTCotherwise

For comb 4, the comb offset value can be calculated as:

kTCpi=kTC+KTC/2 mod KTCif pi∈{1001, 1003, 1005,1007}kTCotherwise

For comb 8, the comb offset value can be calculated as:

kTCpi=kTC+3KTC/4 mod KTCif pi∈{1003, 1007}kTC+KTC/2 mod KTCif pi∈{1002, 1006}kTC+KTC/4 mod KTCif pi∈{1001, 1005}kTCotherwise

Proposal 2: For mapping of 8 SRS ports onto one OFDM symbol, the comb offset values should be calculated as the following:

- Comb 2 and 4:

  - kTCpi=kTC+KTC/2 mod KTCif pi∈{1001, 1003, 1005,1007}kTCotherwise

- Comb 8:

  - kTCpi=kTC+3KTC/4 mod KTCif pi∈{1003, 1007}kTC+KTC/2 mod KTCif pi∈{1002, 1006}kTC+KTC/4 mod KTCif pi∈{1001, 1005}kTCotherwise

  - Mapping of 8 SRS ports onto multiple OFDM symbol

Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. - At least s = 2. - FFS: s = 4, s = 8. - m = 2,4,8, 10,12,14, and m is a multiple of s. - Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol. - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports. - FFS: down selection from existing resource mapping designs - FFS: which subset of 8/s ports are mapped onto each OFDM symbol. - FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern: - Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. - Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

Although it was agreed that 8 SRS ports are mapped onto s OFDM symbols (TDM factor s), s is FFS point. In terms of the cyclic shift determination, when s is 2, 4, or 8, the determination of cyclic shift nSRSCS,i for 4, 2, or 1 SRS port(s) can be reused, respectively. For this reason, we can support s is 2, 4, and 8.

Proposal 3: For mapping of 8 SRS ports onto s OFDM symbols, we can support TDM factor s = 2, 4, and 8.

For channel sounding, the CSI averaged across s OFDM symbols and Msc,bSRS subcarriers is calculated at the gNB side. For this reason, frequency hopping (e.g., partial frequency hopping), power control, and change of UL Tx spatial filter for SRS should be stopped during s OFDM symbols to achieve accurate channel sounding. In other words, the UE should keep power consistency and phase continuity during s OFDM symbols.

Proposal 4: For mapping of 8 SRS ports onto s OFDM symbols, the UE should keep power consistency and phase continuity during s OFDM symbols.

Likewise, a subset of SRS ports should be mapped to s OFDM symbols as close as possible. This is because a channel for 8 SRS ports is sounded at the gNB side by transmitting the s subsets of ports and the UE keeps power consistency and phase continuity during transmitting the s subsets of ports to not change the channel. Therefore, we prefer Option 2-1.

Proposal 5: For mapping of 8 SRS ports onto s OFDM symbols, the s subsets of ports are mapped cyclically as {1, 2, …, s, 1, 2, …, s} on the m OFDM symbols.

  - SRS resource set with usage ‘nonCodebook’

For an SRS resource set with usage ‘nonCodebook’, it was agreed to support 8 1-port SRS resources in the SRS resource set. For the current spec, up to N 1-port SRS resources in the SRS resource set occupy the same RBs in the same symbol, where N is UE capability (i.e., maxNumberSimulaneousSRS-ResourceTx). For accurate channel estimation, a time gap between SRS resources within the same SRS resource set should be as small as possible. Therefore, the maximum number of simultaneous SRS resources should be 8.

Proposal 6: For an SRS resource set with usage ‘nonCodebook’, the maximum number of simultaneously transmitted SRS resources should be 8.

SRI field

Agreement For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt1 is supported, that is - Alt1: A single SRS resource set configured with up to 8 single-port SRS resources - FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively.

For the maximum number of layers is 8, the enhancement on SRI field for nonCodebook is needed. For SRI for nonCodebook, parts or all of SRS resources within the SRS resource set are indicated, and the indicated SRS resources are used as SRS ports and the number of layers. Note that the number of SRS ports is the same as the number of DMRS ports for nonCodebook because precoder matrix W is identity matrix. When NSRS=8 and Lmax=8, the number of all combinations of the SRS resources is calculated as

C18+C28+C38+C48+C58+C68+C78+C88=8+28+56+70+56+28+8+1=255,

and 8 bits (additional 4 bits) are needed for SRI field. In our view, the SRS resource(s) need to be indicated among all combinations to maximize capacity and optimize rank because the optimal combination of the SRS resources depends on channel condition. Therefore, SRI field for nonCodebook should include all combinations of the SRS resources.

Besides, we think the second SRI field cannot be used for indication of part or all of 8 SRS resources within the SRS resource set with usage set to nonCodebook. This is because the second SRI field indicates the same number of layers as the SRI field, and it has lower flexibility for rank indication. In our view, two or four SRS resource sets for 8 SRS resources should be used for MTRP mode. For 8Tx in STRP mode, it is enough that single SRS resource set is used for 8 SRS resources.

Proposal 7: SRI field for nonCodebook should include all combinations of the SRS resources.

SRS capacity enhancement and/or interference randomization for SRS targeting TDD CJT

At RAN1#111 meeting, the following agreements were made in AI 9.1.3.2 for SRS enhancement [2].

Conclusion No consensus to support SRS TD OCC for TDD CJT SRS enhancement in Rel-18. Conclusion No consensus to support the following for TDD CJT SRS enhancement in Rel-18: - Further enhancements to frequency hopping - Sequence hopping/randomization, per-hop sequence from a long SRS sequence - Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment - Precoded SRS for DL CSI acquisition - Pseudo-random muting of SRS transmission for periodic and semi-persistent SRS - Configuration of v (sequence index within a group) per SRS resource - Multiplying mask sequence to the legacy SRS sequence Agreement For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set: - Option 1: - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha - Each transmission occasion of the SRS resource is towards multiple TRPs - Option 2: - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS - Different transmission occasions of the SRS resource can be towards different TRPs

Power control

We support Option 2 (i.e., Different transmission occasions of the SRS resource can be towards different TRPs).

In Option 1, the SRS transmission power P is expressed as [3]:

P=PO+10 log10M+1NTRPn=1NTRPαn*PLn+h

where PLn is a downlink pathloss for each TRP in dB. For this reason, αnPLn [dB] is averaged across multiple TRPs whose number is NTRP. In Figure 1, we show an example of Option 1 in a simple case that NTRP=2 and α=1. Since the averaged pathloss PL is 50 dB, a gap between PL1 that is pathloss from TRP#1 and PL is 30 dB. Therefore, to reach TRP#1, 210 repetitions are necessary to improve 30=10log10210 dB. Consequently, Option 1 potentially consumes a lot of UL resources and causes a lot of interference.

Furthermore, α should not be averaged across multiple TRPs. This is because it is beneficial that α is determined for each TRP. For example, in Figure 1, the UE is in the edge of Area#1 but is around the center of Area#2. Therefore, a case that α1<α2 is better than other case that (α1+α2)/2  to mitigate interference from TRP#1 to TRP#2. In other words, since α is beneficial to mitigate near-far problem, averaging α is unnecessary.

Figure 1: An example of Option 1.

On the other hand, in Option 2, the number of transmission occasions is always same as the number of TRPs. That is, 2 repetitions are enough in Figure 1. Additionally, α can be determined for each TRP. For this reason, we support Option 2.

Proposal 8: For per-TRP power control, different transmission occasions of the SRS resource should be towards different TRPs.

  - Cyclic shift hopping and comb offset hopping

Agreement For SRS interference randomization, support: - Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping - FFS: Associated UE capability Agreement For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs. - Option 1: Reuse the SRS sequence identity . - Option 2: Introduce new ID(s). - FFS: the value range, one new ID or two separate new IDs, default ID(s) Agreement For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index within a radio frame and OFDM symbol index , and select at least one of the following options: - Option 1: Within a slot, hopping based on the repetition factor and symbol index that is the same across the R repetitions. - Option 2: Within a slot, hopping based on only the symbol index . - Option 3: No intra-slot hopping. - FFS: Time domain hopping behaviour further depends on system frame number (SFN) . - FFS: reinitialization periodicity of N radio frames or reinitialization based on system frame number. - FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) - FFS: At least support reinitialization at the beginning of each radio frame.

Combined cyclic shift hopping and comb offset hopping

For combined cyclic shift hopping and comb offset hopping, we think there are no additional spec impacts other than UE capability for combined cyclic shift hopping and comb offset hopping. That is, these schemes can be separately configured and independently performed. Furthermore, in our view, a restriction that only one of cyclic shift hopping and comb offset hopping is performed at the same time is unnecessary if the UE has capability.

Proposal 9: For combined cyclic shift hopping and comb offset hopping, cyclic shift hopping and comb offset hopping can be separately configured and independently performed.

Hopping granularity

For hopping time-granularity for intra-slot hopping, there are two options as the following:

- Option 1: Granularity = adjacent R OFDM symbols

- Option 2: Granularity = 1 OFDM symbol

In our view, Option 1 has higher flexibility than Option 2 because R can be set to 1 or other values. Furthermore, R>1, the repetition gain can be achieved. For this reason, we support Option 1.

Proposal 10: For cyclic shift hopping and/or comb offset hopping, hopping time-granularity for intra-slot hopping should be adjacent R OFM symbols (i.e., Option 1).

Hopping pattern

Besides, if l' is OFDM symbol index, for Option 1, cyclic shift value and/or comb offset value are hopped at every (l0+l')/R symbol group index. The symbol group index is the same across the adjacent R OFDM symbols. For this, for cyclic shift hopping, hopping value nSRS,l'CS can be substituted into nSRSCS,i as the following:

nSRSCS,i=nSRSCS+nSRS,l'CS'+nSRSCS,max(pi-1000)/2NapSRS/2 mod nSRSCS,max, if NapSRS=4 and nSRSCS,max=6 nSRSCS+nSRS,l'CS'+nSRSCS,max(pi-1000)NapSRS mod nSRSCS,max, otherwise

Hopping value nSRS,l'CS (i.e., hopping pattern) can be from 0 to nSRSCS,max-1 and is determined based on pseudo-random sequence c(i). Therefore, hopping value nSRS,l'CS can be calculated as:

nSRS,l'CS=m=07c8ns,fμNsymbslot+l0+l'R+m2m mod nSRSCS,max

However, if using pseudo-random sequence c(i), initialization is needed (i.e., cinit). For initialization, there are two options as the following:

- Option 1: Use legacy ID nIDSRS.

- Option 2: Use new ID.

For SRS sequence group hopping/sequence hopping, the legacy ID nIDSRS is used for initialization of c(i). If two UEs are configured with the same ID nIDSRS, and in a case that two SRS resources have the same resource allocation, both UE have the same hopping pattern. However, since nIDSRS∈{0,1,…,65535}, we think this is corner case. Therefore, we have no strong preference but slightly prefer Option 1.

Proposal 11: For cyclic shift hoping and/or comb offset hopping, we slightly prefer to use the legacy ID nIDSRS to initialize the pseudo-random sequence c(i).

Conclusion

In this contribution, we have the following proposal:

Proposal 1: For mapping of 8 SRS ports onto one OFDM symbol, the cyclic shift value nSRSCS,i for comb 8 should be calculated as the following:

- nSRSCS,i=nSRSCS+nSRSCS,max(pi-1000)/4NapSRS/4 mod nSRSCS,max

Proposal 2: For mapping of 8 SRS ports onto one OFDM symbol, the comb offset values should be calculated as the following:

- Comb 2 and 4:

  - kTCpi=kTC+KTC/2 mod KTCif pi∈{1001, 1003, 1005,1007}kTCotherwise

- Comb 8:

  - kTCpi=kTC+3KTC/4 mod KTCif pi∈{1003, 1007}kTC+KTC/2 mod KTCif pi∈{1002, 1006}kTC+KTC/4 mod KTCif pi∈{1001, 1005}kTCotherwise

Proposal 3: For mapping of 8 SRS ports onto s OFDM symbols, we can support TDM factor s = 2, 4, and 8.

Proposal 4: For mapping of 8 SRS ports onto s OFDM symbols, the UE should keep power consistency and phase continuity during s OFDM symbols.

Proposal 5: For mapping of 8 SRS ports onto s OFDM symbols, the s subsets of ports are mapped cyclically as {1, 2, …, s, 1, 2, …, s} on the m OFDM symbols.

Proposal 6: For an SRS resource set with usage ‘nonCodebook’, the maximum number of simultaneously transmitted SRS resources should be 8.

Proposal 7: SRI field for nonCodebook should include all combinations of the SRS resources.

Proposal 8: For per-TRP power control, different transmission occasions of the SRS resource should be towards different TRPs.

Proposal 9: For combined cyclic shift hopping and comb offset hopping, cyclic shift hopping and comb offset hopping can be separately configured and independently performed.

Proposal 10: For cyclic shift hopping and/or comb offset hopping, hopping time-granularity for intra-slot hopping should be adjacent R OFM symbols (i.e., Option 1).

Proposal 11: For cyclic shift hoping and/or comb offset hopping, we slightly prefer to use the legacy ID nIDSRS to initialize the pseudo-random sequence c(i).

References

- [1]	RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Dec., 2021.

- [2]	RAN1 chair’s note, RAN1#112 meeting, Feb. 2023.

- [3]	R1-2210848, FUTUREWEI, “SRS enhancements for TDD CJT and 8TX operation”, Nov., 2022.