3GPP TSG RAN WG1 Meeting #112bis-e			    	R1-2303189

e-Meeting, April 17th – April 26th, 2023

Source 	: CAICT

Title 	: Considerations on channel access mechanism of SL-U

Agenda Item	: 9.4.1.1

Document for	: Discussion and decision

Discussions

CW Adjustment

Agreement The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows: - Option 1a - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed

CW Adjustment when SL-HARQ feedback is disabled

Proposal 3-2 (II)[5] - SL HARQ-ACK feedback disabled in SCI (i.e., all cast types) - Option 1: For every priority class p∈1,2,3,4, use the latest CWp. - Option 2: CW is adjusted according to number blind retransmissions of the TBs within a COT. - Option 3: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported in SL-U.

As for Option1/Option2, setting CW with the latest value, or based on the blind retransmission number of the TBs, obviously cannot dynamically adapt to the channel busy state, which is against the original purpose of CW adjustment and undesirable.

In our view, it is applicable and beneficial to use Option3 to adapt the CW size based on the channel busy ratio (CBR) measurement results, reacting to the channel busy state. When the measured channel is busy, UE can enlarge the CW size to reduce the channel collision possibility. And UE can reduce CW size to access the channel more quickly if the measured CBR level is low. The CBR measurement is available for current SL UE and usually used for channel congestion control to determine the transmission parameters including transmit power, maximum retransmission time, maximum and minimum MCS, subchannel number limit and CR limit. We can configure CW size as one of transmission parameters in channel congestion control which are also associated to the transmission priority or access category level to determine the CW size in case of disabled SL-HARQ feedback.

Proposal 1: For CW Adjustment when SL-HARQ feedback is disabled, propose to adopt Option3 and configure CW size as one of transmission parameters in channel congestion control based on CBR measurement, associated to the transmission priority or access category level.

CW Adjustment for groupcast option2

Proposal 3-2 (II) [5] - Only groupcast option 2 (ACK and NACK) within SL reference duration - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks. - Option 2: If at least one ‘ACK’ is received, CWp=CWmin,p for each priority class p∈1,2,3,4; otherwise, increase. - FFS whether the at least one ‘ACK’ is from just one UE or every UE

For Option1, it is difficult to set a single ratio for all cases with different number of receivers. In the same channel busy state, the groupcast with more receivers, the ratio of HARQ-ACKs should be set lower than the groupcast with less receivers. Therefore, we slightly prefer Option2, which aligns the basic rule of CW adjustment, setting CWp=CWmin,p once ACK is received. Whether the ACK is from one ‘UE’or every‘UE’ is depending on whether the corresponding retransmission is needed. If retransmission is triggered based on the HARQ feedback, then ACK from every ‘UE’ is required here to set CWp=CWmin,p.

Proposal2: For CW adjustment with groupcast option2, it is suggested to use Option2, considering one ratio in Option1 is not applicable for all cases with different number of receivers.

CW Adjustment for groupcast option1

Proposal 3-2 (II) [5] - Only groupcast option 1 (NACK-only) within SL reference duration - Option 1: For every priority class p∈1,2,3,4, use the latest CWp. - Option 2: If ‘NACK’ or a collision indicator (IUC scheme 2) is received, increase CWp; Otherwise, CWp=CWmin,p or use the latest CWp (FFS which). - Option 6: GC option 1 (NACK-only) is not supported in SL-U

We suggest to support groupcast option 1 (NACK-only) with SL-HARQ feedback considering that it is beneficial to reduce the cost for LBT procedures for PSFCH transmission and it can further reduce the LBT impact for inter-UE PSFCH transmissions.

Regarding Option1, we don’t think it is a good choice to use the latest CW value when a UE is able to determine the packet is well received though HARQ feedback. Therefore, we prefer to adopt Option2.

Proposal3: For CW adjustment with groupcast option1, it is suggested to use Option2.

LBT in Mode2 RA

Proposal 6 (I)[6] - To resolve the Type 1 LBT blocking issue, where one UE performing a Type 1 LBT procedure for using its own selected/reserved resource(s) is blocked by another UE’s SL transmission at least in a slot preceding to the selected/reserved resource and causing the LBT to fail. Further study at least the following solution options for down-selection in a future meeting. Other options are not precluded. - Option 1: - UE avoid selection of a resource before a reserved resource when the transmitting symbols of the selected resource overlap with Type 1 LBT of the reserved resource. - UE avoid selection of a resource after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. - FFS: the avoidance should be performed by L1 exclusion or L2 MAC selection - FFS: whether / how to achieve this in RA mode 1 - Option 2: - UE prioritizes/selects resource(s) for transmission in slot(s) after a reserved resource when transmission of the selected resource is able to share the initiated COT of the reserved resource (i.e., the selected resource(s) is within the COT duration of the reserved resource and the CAPC value of the selected resource(s) is equal to or higher than that of the reserved resource). - UE prioritizes/selects resource(s) for transmission in slot(s) before a reserved resource when transmission of the selected resource is able to share its initiated COT with the reserved resource (i.e., the reserved resource is within the CAT duration of the selected resource(s) and the CAPC value of the selected resource(s) is equal to or smaller than that of the reserved resource). - FFS whether / how to achieve this in RA mode 1. - Option 3: UE selects extra / more resources than required for transmitting a TB (i.e., overbooking) to accommodate potential Type 1 LBT failures. - Option 4: LBT duration is determined firstly, then resource selection takes into account of the LBT duration is performed. - Option 5: At MAC layer, selection of resource(s) among the reported set of candidate resources from L1 is up to UE implementation in mode 2 for SL-U, instead of random selection.

Collision Avoidance Principles for Resource Allocation

From our view, in order to avoid transmission collisions, it is necessary to define some principles for resource allocation in Mode2 RA taking the LBT procedure and COT sharing information into account. As mentioned in the Proposal 6 (I) [6], we think both Option1 and Option2 can be supported as the collision avoidance principles for resource allocation in Mode2 RA. However, more details need to be discussed. For example, if a UE cannot obey the collision avoidance principles due to the transmission latency requirements, where UE needs to select resources overlapping the LBT duration of reserved resources, or resources of which the corresponding LBT duration is overlapped with reserved resources, it is still feasible if LBT contention back-off continues during the inter-UE/intra-UE blocking slots as we analyse in Section 1.2.3. In addition, if to mandate to support Option1, it will result many unused resources cannot be selected which is undesirable. For example, the resources overlapping the LBT duration of reserved resources which cannot be selected could be unused. Therefore, we suggest to implement Option1 as prioritization behaviour as Option2 and can be violated due to the transmission latency requirements and transmission priority.

Observation1: When a UE violates Option1, it is still feasible if LBT contention back-off continues during the inter-UE/intra-UE blocking slots. And if to mandate to support Option1, it will result many unused resources cannot be selected, which is undesirable.

Proposal4: Propose to implement Option1 as prioritization behaviour as Option2 and can be violated due to the transmission latency requirements and transmission priority. Option1 can be modified as:

- UE priorities the avoidance of selection of a resource before a reserved resource when the transmitting symbols of the selected resource overlap with Type 1 LBT of the reserved resource.

- UE priorities the avoidance of selection of a resource after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource.

- FFS: the avoidance should be performed by L1 exclusion or L2 MAC selection

- FFS: whether / how to achieve this in RA mode 1

Timeline of LBT and Resource Allocation

For SL-U UE to access channel with Mode2 RA, it shall apply both LBT and resource allocation procedures. In our view, it is up to UE implementation to determine which procedure should be triggered first. However, triggering LBT procedure before resource allocation might be more beneficial. If a UE trigger LBT type1 procedure, we can estimate the earliest ending time of contention back-off, i.e., N0×Td，where N0 is generated by randn(0,CWp) , and the resource during the LBT contention back-off can be excluded, as shown in Fig.1. If triggering resource allocation procedure first, the reported candidate resource might locate in the LBT contention back-off duration and resource reselection will be triggered. As a result, it has to exclude the resource in the estimated shortest LBT contention back-off duration just as the case triggering LBT procedure before resource allocation procedure.

Fig. 1 Resource exclusion during LBT contention back-off.

Observation2: Triggering LBT procedure before resource allocation procedure is more beneficial than triggering resource allocation procedure before LBT procedure, considering the resource can be excluded in the estimated shortest LBT contention back-off duration.

Proposal5: It can be up to UE implementation to determine which procedure between LBT and resource allocation should be triggered first. Trigger LBT procedure before resource allocation procedure if specification is necessary.

LBT with intra-UE or inter-UE blocking

During LBT type1 contention back-off, when the channel is sensed busy, the back-off process will be resumed till the channel is sensed idle. However, for NR SL-U, a SL UE would encounter the issues of intra-UE blocking and inter-UE blocking during the contention back-off. For example, as shown in Fig.2., during the back-off, the channel might be sensed “busy” because of intra- NR SL transmissions from other SL UE or the UE itself.

Fig. 2 LBT with inter-UE blocking or intra-UE blocking.

How to deal with LBT contention back-off with inter-UE blocking or intra-UE blocking needs to be specified. In our observation, there are the following options:

Option1: the contention back-off is frozen till the channel is sensed idle.

Option2: the contention back-off continues in a slot if the SL UE can successfully decode the SCI transmitted from other UEs in the slot, or the SL UE sends its own data in the slot. Otherwise, the contention back-off is frozen in the slot.

For Option1, it means that a SL UE would better not select the resources in the same slot with the reserved resources indicated by the other UEs. Because in that case, the back-off process would be very likely to be suspended during the slot with reserved resources. Therefore, we prefer Option2. As shown in Fig.3, assuming that a UE’s LBT contention back-off encounters the inter-UE blocking in slot n-1, the back-off continues if SCI in slot n-1 is successfully decoded. And the back-off counter N0 should be updated by N0-DataDuration/Td, where DataDuration is the data duration in slot n-1 indicated by the associated SCI, and Td is the time duration used to sense the channel for back-off. If the back-off counter N0 just reaches zero at the end of slot n-1, the UE then can send data in slot n with the selected resource.

Fig. 3 UE’s LBT contention back-off encounters the inter-UE blocking in slot n-1.

Observation3: Options to deal with LBT contention back-off with inter-UE blocking or intra-UE blocking include:

Option1: the contention back-off is frozen till the channel is sensed idle.

Option2: the contention back-off continues in a slot if the SL UE can successfully decode the SCI transmitted from other UEs in the slot, or the SL UE sends its own data in the slot. Otherwise, the contention back-off is frozen in the slot.

Proposal6: For LBT contention back-off with inter-UE blocking or intra-UE blocking, the contention back-off continues in a slot if the SL UE can successfully decode the SCI transmitted from other UEs in the slot, or the SL UE sends its own data in the slot. Otherwise, the contention back-off is frozen in the slot.

UE-to-UE COT Sharing

Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated Agreement A responding UE’s SL transmission(s) within RB set(s) corresponding to a shared COT can be transmitted when the CAPC value(s) of the SL transmission(s) have an equal or smaller CAPC value than the CAPC value indicated in the COT sharing information. Agreement A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when, - In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported) - In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported) FFS: all other details and additional restrictions

Responding UE for COT Sharing

In our observation, the number of responding UE should be restricted. Otherwise, the responding UE might not be able to use Type2 LBT to access the channel within the COT due the hidden node problem. For example, as shown in Fig.4, UE1 initiates a COT1 and shares it to UE2 and UE3, where UE2 and UE3 cannot communicate with each other. In this situation, with sharing COT1, UE2 cannot sense the transmission from UE3 on slot3 and the responding channel can be occupied by other SL UEs or WiFi UEs and the left transmission on slot4 would be interrupted. The greater number of nodes to which COT is shared, the problem of COT interruption could be more serious. Therefore, we propose to limit the number of responding UE to share COT.

Fig. 4 COT interruption due to hidden node issue with more than one responding UEs.

Proposal7: Propose to limit the number of responding UE to share COT, considering the greater number of nodes to which COT is shared, the problem of COT interruption due to hidden node issue could be more serious.

The purpose to share a COT is to enable a UE could transmit or receive data consecutively based on its requirements. However, due to variant traffic requirements from different UEs, it is hard to maintain the transmission consecutiveness within a COT. Then the reserved transmission for SL -U can be included in a COT and support the case that a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission. The transmission in a COT is used for four purpose:1) to transmit data from the COT initiating UE, 2) to transmit data intended for the COT initiating UE, 3) to transmit data intended for the same destination of COT initiating UE’s SL transmissions, 4) to transmit data on the resources reserved by other SL UE before COT initiating. Therefore, additional IDs for COT sharing information can be set to the source IDs of reserved SL transmissions which happen within the sharing COT and can be used to maintain the consecutiveness of the SL transmissions within the sharing COT.

Fig. 5 COT including the chance reserved for transmission not intended to the COT initiating UE.

Proposal8: Additional IDs for COT sharing information can be set to the source IDs of reserved SL transmissions which happen within the sharing COT and can be used to maintain the consecutiveness of the SL transmissions within the sharing COT.

Power control for COT Sharing

We notice that whether to enable the responding UE to sense the transmissions from COT initiating UE needs FFS. For a unicast transmission, the transmission power is set based on the pass loss between the SL transmitter and receiver. For example, as shown in Fig.6, on slot1, if the COT initiating UE1 transmits data to UE3 other than UE2 which shares the COT in unicast way, based on the current power control mechanism, UE2 might not be able to sense the transmission between UE1 and UE3. As a result, the COT interruption problem occurs at UE2. From our view, UE2 could not assure whether the COT sharing on slot3 initiated by UE1 is still effective if the transmission from UE1 on slot1 cannot be sensed. If enabling UE2 to sense the transmission from UE1 on slot1, the power control mechanism therefore needs to be enhanced.

Fig. 6

Proposal9: FFS whether to enable the responding UE to sense the transmission from COT initiating UE.

COT Interruption due to COT Collision

In our observation, a responding UE might share more than one COTs, which could cause transmission collision on the same transmission resource due to device half-duplex issue or transmission resource over-lapping from different COTs. As a result, the transmission of COT with lower priority could be cancelled on the COT collision slot. How to treat the transmission in a COT after the COT collision slot and guarantee the transmission opportunity needs FFS. As shown in Fig.7, due to device half-duplex issue, if UE3 cancels the transmission to UE2 on slot3 with COT2, how to guarantee the transmission on slot4 of COT2 should be investigated. In addition, whether to indicate the effectiveness of transmissions in a COT after the COT collision slot should be FFS.

Fig. 7 COT collision due to half-duplex issue.

Proposal10: How to treat the transmission in a COT after the COT collision slot and guarantee the transmission opportunity needs FFS.

Proposal 11: Whether to indicate the effectiveness of transmissions in a COT after the COT collision slot should be FFS

Conclusions

In this contribution, we have presented our considerations on the FFS issues including CW adjustment, LBT in Mode2 RA, and UE-to-UE COT sharing, and put forward the following observations and proposals:

Proposal 1: For CW Adjustment when SL-HARQ feedback is disabled, propose to adopt Option3 and configure CW size as one of transmission parameters in channel congestion control based on CBR measurement, associated to the transmission priority or access category level.

Proposal2: For CW adjustment with groupcast option2, it is suggested to use Option2, considering one ratio in Option1 is not applicable for all cases with different number of receivers.

Proposal3: For CW adjustment with groupcast option1, it is suggested to use Option2.

Observation1: When a UE violates Option1, it is still feasible if LBT contention back-off continues during the inter-UE/intra-UE blocking slots. And if to mandate to support Option1, it will result many unused resources cannot be selected which is undesirable.

Proposal4: Propose to implement Option1 as prioritization behaviour as Option2 and can be violated due to the transmission latency requirements and transmission priority. Option1 can be modified as:

- UE priorities the avoidance of selection of a resource before a reserved resource when the transmitting symbols of the selected resource overlap with Type 1 LBT of the reserved resource.

- UE priorities the avoidance of selection of a resource after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource.

- FFS: the avoidance should be performed by L1 exclusion or L2 MAC selection

- FFS: whether / how to achieve this in RA mode 1

Observation2: Triggering LBT procedure before resource allocation procedure is more beneficial than triggering resource allocation procedure before LBT procedure, considering the resource can be excluded in the estimated shortest LBT contention back-off duration.

Proposal5: It can be up to UE implementation to determine which procedure between LBT and resource allocation should be triggered first. Trigger LBT procedure before resource allocation procedure if specification is necessary.

Observation3: Options to deal with LBT contention back-off with inter-UE blocking or intra-UE blocking include:

Option1: the contention back-off is frozen till the channel is sensed idle.

Option2: the contention back-off continues in a slot if the SL UE can successfully decode the SCI transmitted from other UEs in the slot, or the SL UE sends its own data in the slot. Otherwise, the contention back-off is frozen in the slot.

Proposal6: For LBT contention back-off with inter-UE blocking or intra-UE blocking, the contention back-off continues in a slot if the SL UE can successfully decode the SCI transmitted from other UEs in the slot, or the SL UE sends its own data in the slot. Otherwise, the contention back-off is frozen in the slot.

Proposal7: Propose to limit the number of responding UE to share COT, considering the greater number of nodes to which COT is shared, the problem of COT interruption due to hidden node issue could be more serious.

Proposal8: Additional IDs for COT sharing information can be set to the source IDs of reserved SL transmissions which happen within the sharing COT and can be used to maintain the consecutiveness of the SL transmissions within the sharing COT.

Proposal9: FFS whether to enable the responding UE to sense the transmission from COT initiating UE.

Proposal10: How to treat the transmission in a COT after the COT collision slot and guarantee the transmission opportunity needs FFS.

Proposal 11: Whether to indicate the effectiveness of transmissions in a COT after the COT collision slot should be FFS

References

- RP-220300, WID revision: NR sidelink evolution, OPPO, March 2022.

- 3GPP RAN1#110, RAN1 Chair’s Notes, Aug. 2022.

- R1-2207793 3GPP RAN1#110, FL summary for AI 9.4.1.1: SL-U channel access mechanism, Moderator (OPPO), Aug. 2022.

- R1-2210293 3GPP RAN1#110be, FL summary for AI 9.4.1.1: SL-U channel access mechanism, Moderator (OPPO), Oct. 2022.

- R1-2212802 3GPP RAN1#111, FL summary for AI 9.4.1.1: SL-U channel access mechanism, Moderator (OPPO), Nov. 2022.

- R1-2209236 3GPP RAN1#110be, Considerations on channel access mechanism of SL-U, CAICT, Oct. 2022.

- R1-2211917 3GPP RAN1#111, Considerations on channel access mechanism of SL-U, CAICT, Nov. 2022.

- R1-2301797 3GPP RAN1#111, FL summary for AI 9.4.1.1: SL-U channel access mechanism, Moderator (OPPO), Feb. 2023.

- R1-2301037 3GPP RAN1#110be, Considerations on channel access mechanism of SL-U, CAICT, Feb. 2023.