3GPP TSG RAN WG1 #112bis-e	R1-2303191

e-Meeting, April 17th – April 26th, 2023

Source:	Sharp

Title:	CSI enhancement

Agenda Item:	9.1.2

Document for:	Discussion and Decision

Background

In RAN#94e, a new work item “MIMO Evolution for Downlink and Uplink” was approved, and the following objectives were agreed for unified TCI framework extension [1].

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking … - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

Discussions

2.1	CSI enhancement for CJT mTRP

At RAN1#112, the following agreements were made for Type II codebook refinements for CJT mTRP.

Agreement On the Type-II codebook refinement for CJT mTRP, only support NL ={2,4} as additional candidate values to NL=1. - FFS: Additional restriction(s) depending on the configured value for NTRP Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {Ln} for the higher-layer-configured value of NTRP (FFS by RAN1#112: whether the bracketed permutations are also supported): - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {n} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement FFS: Whether the total number of Ln is a UE capability NTRP; {Ln} combination. 1; {2}. {4}. {6} (analogous to legacy, only for total # ports =32, rank 1-2, R=1. 2; {2,2}. {2,4}, [{4,2}]. {4,4}. 3; {2,2,2}. {2,2,4} [and its other permutations]. {4,4,4}. 4; {2,2,2,2}. {2,2,2,4} [and its other permutations]. {2,2,4,4} [and its other permutations]. {4,4,4,4}. Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {pv,} from where the value of {pv,} is gNB-configured via higher-layer (RRC) signaling: - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {M} for Rel-17-based refinement are derived from the supported combinations of {pv ,} for Rel-16-based refinement pv for layers 1-4; ; Condition(s). {1/8, 1/8, 1/16, 1/16}; ¼; --. ½; --. {1/4, 1/4, 1/8, 1/8}; ¼ (*); --. ½ (*); --. {1/4, 1/4, 1/4, 1/4}; ¾ (*); --. {1/2, 1/2, 1/2, 1/2}; ½; - Only applicable when NTRP≤3 and NL=1 - Optional. (*) Supported by legacy Rel-16 Agreement On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, down-select between the following three alternatives (by RAN1#112-bis where n denotes the n-th CSI-RS resource): - Alt1. Prio(,l,m,n)=( k=0n-12Lk) .N.RI.P(m)+N.RI.l(n)+N.n - Note: This implies that CSI-RS resource is designated the highest priority - Alt2. Prio(,l,m,n)=2L’.Qn).RI.N3+2L’.RI. P(m)+RI.l(n)+ - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis) - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI. - Alt3. Replace SD basis index l in legacy Prio calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+ FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m Agreement On the Type-II codebook refinement for CJT mTRP, regarding CBSR, at least for restricting SD basis selection, the legacy CBSR scheme is fully reused for each of the RRC-configured NTRP CSI-RS resources (resulting in CSI-RS-resource-specific SD beam group restriction) - FFS: Whether amplitude restriction is CSI-RS-resource-common or specific, and soft vs hard restriction - FFS: Whether CBSR can be configured to be off for a CSI-RS resource The same rank restriction is applied across NTRP CSI-RS resources Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, regarding the list of supported combinations of {Ln}, only support the following additional combinations: NTRP; {Ln} combination. 2; {4,2}. 3; {2,4,2}, {4,2,2}. No other permutations are supported. FFS: For NTRP>1, in addition to the supported combinations/permutations, whether to support at least one additional combination where at least one of the Ln values (n=1, …, NTRP) is 6 Agreement On the Parameter Combination of Type-II codebook refinement for CJT mTRP, support linkage between the list of supported {Ln} combinations and list of supported {pv,} combinations via pairing each combination for {pv,} with at least one combination for {Ln}, for each NTRP value. - FFS (by RAN1#112bis-e): The exact list of supported pairs/linkage, or restriction of {Ln} when paired to each of {pv,} - FFS (by RAN1#112bis-e): Whether/How to support configuration signalling for indicating the linkage - Note: While no additional codebook parameter will be introduced, the total number of SD basis vectors across CSI-RS resources can still be used as a criterion for choosing the supported pairs/linkage Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FD basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

The main discussion points are linkage of parameter combination and UCI omission. Regarding the parameter combinations, in our view, when NTRP > 1, we prefer not to support Ln = 6 from UE complexity perspective

Proposal 1: we prefer not to support Ln = 6 when NTRP > 1

Regarding the UCI omission, the following alternatives are on the table.

- Alt1. Prio(l,l,m,n)=( k=0n-12Lk) .N.RI.P(m)+N.RI.l(n)+N.l+n

  - Note: This implies that CSI-RS resource is designated the highest priority

- Alt2. Prio(l,l,m,n)=2L’.Q(n).RI.N3+2L’.RI. P(m)+RI.l(n)+l

  - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis)

  - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources

  - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI.

- Alt3. Replace SD basis index l in legacy Prio calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(l,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+ l

In our view, Alt. 2 or Alt. 3 is preferred, and the 1st preference is Alt. 2 because gNB can fallback to the single TRP.

Proposal 2: Prio(l,l,m,n)=2L’.Q(n).RI.N3+2L’.RI. P(m)+RI.l(n)+l is preferred for UCI omission (Alt. 2)

2.2	CSI enhancement for high/medium UE velocities

2.2.1 Type-II CSI refinement

For Type-II CSI refinement for high/medium UE velocities, the following agreement was made [2]:

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrix - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix - FFS: Whether/how to include CQI overhead reduction for X=2 Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme: - Alphabets for amplitude and phase - Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, for each layer: - One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors - One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2) - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported Conclusion For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding the parameter Q, there is no consensus in supporting additional candidate values Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter K (the number of AP-CSI-RS resources for the CMR), optionally support only K=12 as an additional candidate value Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), in addition to 0 and 2, δ=1 is additionally supported Conclusion For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), there is no consensus in supporting additional candidate values Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots), - for P/SP-CSI-RS, support d equal to the periodicity of the CSI-RS resource - for AP-CSI-RS, also support d =1 Conclusion On the Parameter Combination of Type-II codebook refinement for high/medium velocities, there is no consensus on including another non-UCI Doppler codebook parameter as a variable in the list of supported Parameter Combinations. - Note: This implies that other non-UCI Doppler codebook parameters will be a part of RRC configuration (either explicit or implicit) Agreement For the Type-II codebook refinement for high/medium velocities, - The constraint on the maximum number of non-zero coefficients (NZCs) per-layer (K0) is defined jointly across all Q DD basis vectors. - FFS: How K0 is calculated - Also support a constraint on the total number of non-zero coefficients (NZCs) summed across all Q DD basis vectors and across all layers: - Following the legacy specification, the maximum total number is 2K0 Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, CQI is defined per legacy CQI definition (ensuring at most 10% BLER) within the slot(s) which a CQI is associated with. Agreement For the Type-II codebook refinement for high/medium velocities, for N4>2 and Q=2, the selection of Q out of N4 DD basis vectors is indicated by a log2N4-1-bit indicator in CSI part 2 - Analogous to FD basis selection, DD basis index 0 (representing DC) is always selected. Agreement On the Type-II codebook refinement for high/medium velocities based on Rel-16 regular eType-II codebook (if supported), for the purpose of choosing the supported Parameter Combinations - Regarding the codebook parameter pv, in addition to the supported values from the legacy specification, introduce as additional candidate values - pv =1/8 for v=1,2 (hence 1/16 for v=3,4) - pv =1/2 for v=1,2,3,4 - Regarding the codebook parameter , in addition to the supported values from the legacy specification, introduce as an additional candidate value = 1/8 - Regarding the codebook parameter L, the supported values from the legacy specification apply Agreement On the Type-II codebook refinement for high/medium velocities, regarding UCI omission, down-select between the following three alternatives (by RAN1#112bis-e where q denotes the q-th DD basis vector): - Alt1. Prio(,l,m,q)=2L. Q.RI.P(m)+Q.RI.l+Q.q - Note: This implies that DD basis is designated the highest priority - Alt2. Prio(,l,m,q)=2L.S(q).RI.N3+2L.RI. P(m)+RI.l+ - Note: This implies that DD basis is designated the lower priority (after FD basis) - FFS: S(q) maps the index q according to a rule - Alt3. Prio(,l,m,q)=2L.RI.Mv.q + 2L.RI.P(m)+ RI.l +  - Note: This implies that DD basis is designated the least priority - Alt4. Prio(,l,m,q)=2L.P(m).RI.Q+2L.RI.S(q)+RI.l+ - Note: This implies that DD basis is designated with lower priority (after SD basis) and higher priority (before FD basis) - FFS: S(q) maps the index q according to a rule FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m q=0,…,Q-1 Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives (no later than RAN1#112bis-e): - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L – s1-s0+ mint1-t0, Q – t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively. Nokia/NSB, Samsung, vivo, and ZTE raised concerns that, in their understanding, Alt3A violates previous agreements for “Q different two-dimensional bitmaps” and/or common DD basis selection across SD/FD basis pairs and hence, to some extent, objective 1 of the WID. Agreement For the Type-II codebook refinement for high/medium velocities, regarding the down-selection of bitmap(s) for indicating the locations of the NZCs (in RAN1#112bis-e), the following is used as a guidance for evaluation: - Following the agreed EVM, use “UPT vs. overall overhead (including CQI and PMI)” to compare across alternatives, assuming at least FTP1 traffic model and Rel-16 Parameter Combinations (L, beta, pv) - Use only the supported codebook parameter values (e.g. Q, K, m, d, delta, N4) - Companies are to state their assumptions on UE-side prediction (e.g. ideal or realistic, CSI-RS type, CSI-RS overhead calculation in relation to UPT, assumptions on WCSI and l) and the use of rank adaptation

In our view, regarding the 2nd time-domain CQI, we prefer that the 2nd CQI is located on UCI part 2. Regarding whether differential CQI is applied or not, no differential CQI is supported.

Proposal 3:

- The 2nd CQI is located on UCI part 2

- No differential CQI is not supported for the 2nd CQI

2.2.2 Reporting of TRS-based time-correlation/Doppler information via TRS

At RAN1#112 meeting, the following conclusion were made for TDCP reporting.

Agreement For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - FFS: Candidate values for delay - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 - FFS: Value of Dbasic Agreement For the Rel-18 TRS-based TDCP reporting, the priority of the CSI report(s) associated with TDCP reporting is down-selected from the following alternatives: - Alt1. Lower than other CSI reports - Alt2. Same as CSI report(s) not carrying L1-RSRP or L1-SINR - Alt3. Higher than other CSI reports - Other alternatives are not precluded Agreement For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter Y for Y>1, down-select from the following alternatives: - Alt1. The value of Y is gNB-configured via higher-layer (RRC) signalling - Alt2. The value of Y follows the delays from the configured TRS resource - Alt3. The value of Y is UE-selected and reported The value of Y is a UE capability Agreement For the Rel-18 TRS-based TDCP reporting, support multiplexing TDCP reporting with other UCI parameters on PUSCH following the legacy UCI multiplexing rule for AP-CSI Agreement For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, by RAN1#112bis-e, decide between the following alternatives: - Alt1. Fully reuse legacy TRS - Alt2. Study enhancements on TRS (e.g. periodicities) Note. If there is no consensus on Alt2, Alt1 is the default outcome

Regarding TRS configuration for TDCP, we support KTRS ≥1 TRS resource set(s) is configured when reportQuantity is ‘tdcp’

Regarding amplitude quantization, we prefer to have size-Q quantization alphabet supported: 1-2-N-qs,   q=0,1,…,2Q-1.

Proposal 4:

- reportQuantity with ‘tdcp’ is supported

- KTRS ≥1 TRS resource set(s) can be configured when reportQuantity is ‘tdcp’

- size-Q quantization alphabet supported: 1-2-N-qs,   q=0,1,…,2Q-1 is supported.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: we prefer not to support Ln = 6 when NTRP > 1

Proposal 2: Prio(l,l,m,n)=2L’.Q(n).RI.N3+2L’.RI. P(m)+RI.l(n)+l is preferred for UCI omission (Alt. 2)

Proposal 3:

- The 2nd CQI is located on UCI part 2

- No differential CQI is not supported for the 2nd CQI

Proposal 4:

- reportQuantity with ‘tdcp’ is supported

- KTRS ≥1 TRS resource set(s) can be configured when reportQuantity is ‘tdcp’

- size-Q quantization alphabet supported: 1-2-N-qs,   q=0,1,…,2Q-1 is supported.

References

- [1]	RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Dec., 2021.

- [2]	RAN1 chair’s note, RAN1#112 meeting, Feb. 2023.