3GPP TSG RAN WG1 #112bis-e	R1-2303194

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.2.2.1

Source: 	ETRI

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:	Discussion

Introduction

This contribution presents ETRI’s views on the evaluation of AI/ML for CSI feedback enhancement use case for the AI/ML for NR Air Interface study [1].

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: o CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] o Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] o Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] …

Discussion

Evaluation methodologies

In this section, we discuss evaluation methodologies for AI/ML for CSI feedback enhancement.

Specific evaluation methodologies for AI/ML-based CSI compression sub use case

Quantization/dequantization

In RAN1 #110bis-e meeting, quantization/dequantization issues have been brought up, including whether/how to handle different quantization/dequantization methods between NW and UE [3].

By the limitation of uplink capacity to deliver CSI feedback from the UE, the size of the CSI feedback should be limited. To generate the limited length of CSI feedback, quantization should be performed on the UE side and dequantization should be performed on NW-side during the CSI compression procedure. The quantization/dequantization along with the AI/ML-based CSI feedback can be considered as following two types:

- Case 1: AI models perform compression and quantization simultaneously. Only a binary sequence of CSI payload is obtained by the UE-side model.

- Case 2: AI models perform compression, followed by quantization. An unquantized latent variable is additionally obtained by the UE-side model.

For Case 1, AI models simultaneously perform compression and quantization and generate CSI payloads of a binary sequence. In this case, AI models are trained along with the size of the CSI payload, i.e., quantization-aware training is allowed for AI model training. Moreover, different AI models are independently trained with different output sizes (i.e., CSI payload sizes). In this case, NW-side model can accept the binary CSI payload, and the quantization codebook is not required to be shared between UE and network-side models.

For Case 2, AI models perform compression and quantization sequentially. In this case, the quantization function can be separated from the compression function. An unquantized latent variable can be obtained by UE-side model and the variable is quantized by the quantization block to generate CSI payload. Various CSI payload sizes can be generated by controlling the quantization block. When the quantization is performed on each latent dimension, the quantization block is scalar quantization, otherwise, i.e., when the quantization is performed on multiple latent dimensions, the quantization block is vector quantization. Codebook for quantization/dequantization can be either fixed or dynamic throughout the training. For fixed codebook, the range of input and output of quantization can be pre-determined and does not necessarily be shared between both sides. For dynamic codebook, the relationship between input and output is continuously updated throughout the training and should be shared after each re-training. The AI model can be trained either with and without aware of the quantization block.

Proposal 1: For the evaluation of AI/ML-based CSI compression use case, companies to report the details on the quantization/dequantization including:

- Functional separability of compression and quantization

- Configuration of quantization/dequantization block (Scalar or vector quantization, fixed or dynamic codebook)

- Quantization aware/non-aware training of the AI models

Figure 1 shows an example of an AI model with a functionally separable quantization block. In the model, a trained UE-side AI model can be applied for different CSI payload sizes using different quantization configurations. For example, when the dimension of the latent variable is L, by applying Q bits scalar quantization for each latent variable’s dimension, CSI payload size of Q*L bits can be generated and delivered to the NW-side.

Figure 1. Supporting various CSI payload sizes using quantization

Observation 1: AI/ML models with a functionally separable quantization block can adjust CSI payload sizes by using different quantization configurations.

Performance upper bound of quantization

In RAN1 #112 meeting, an issue of performance upper bound of quantization is brought up (Issue #3-18 in [5]) as following.

Question 3.2.17: For the evaluation of quantization aware/non-aware training, on top of the agreed cases of quantization non-aware training (Case 1) and quantization aware training (Case 2-1, Case 2-2), do you think it is needed to introduce an additional upper bound case of Case 0: non-quantized inference (i.e., float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training and inference), to reflect the performance loss due to quantization?

In our view, the purpose of CSI compression sub-use case is to see recovery performance using quantized feedback information, and unquantized feedback information is unable to be delivered to CSI reconstruction part. In addition to the infeasibility, training and inference of AI/ML model using unquantized feedback information is not appropriate as an upper bound on performance because it tends to overestimate and does not provide much information on the performance of the AI/ML model. For example, when we consider the case of unquantization, an AI/ML model that doesn't compress the input information (e.g., precoding vector) at all can still perform perfect recovery.

Proposal 2: For the evaluation of CSI compression sub use-case with quantization aware/non-aware training, consider not introducing training and inference using unquantized latent variables for the performance upper bound.

Definition of pre and post-processing

In RAN1 #112 meeting, an issue of processing complexity is brought up (Issue #2-4 in [5]) as following.

Question 2.2.1: For the evaluation of CSI enhancements, when reporting the computational complexity including the pre-processing and post-processing, do you agree that we need to clarify the boundary of calculating the complexity metric (i.e., FLOPs)? E.g., - For the input of the CSI prediction, or input of the CSI generation part, the pre-processing starts at the raw channel matrix (i.e., includes the decomposition from channel matrix to eigenvectors) - For the input of the CSI reconstruction part, the pre-processing includes the dequantization. - For the output of the CSI generation part, the post-processing includes the quantization. - For the output of the CSI reconstruction part, the ends at the precoding vectors.

Based on the discussion so far, the inputs and outputs of an AI/ML model can be in the form of a precoding matrix or an explicit channel matrix. In our view, pre and post-processing are not meant to change the format of the inputs and outputs of an AI/ML model, but rather as a supplementary means to improve the performance of inference. From this perspective, we think that it is reasonable to limit pre and post-processing operations perform after the input or output data is prepared (transformed) in the respective input and output formats. In this regards, pre and post-processing not include quantization and dequantization.

Proposal 3: For the evaluation of CSI compression sub use-case, consider limiting the pre-processing operations as operations after the input data is prepared in the input format (e.g., precoding matrix, or explicit channel matrix).

Proposal 4: For the evaluation of CSI compression sub use-case, consider limiting the post-processing operations as operations after the output data is prepared in the output format (e.g., precoding matrix, or explicit channel matrix).

Specific evaluation methodologies for AI/ML-based CSI prediction sub use case

In RAN #111 meeting, time domain CSI prediction is selected as sub-use case for AI/ML CSI enhancement [6]. In this section, we discuss simulation results for the CSI prediction sub-use case to evaluate performance of AI/ML models.

Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer futher till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

CSI prediction represents that an AI/ML model inferences T number of future CSI information (RAW channel matrix or eigenvectors) based on K number of previous CSI information. The structure of CSI prediction is described in the following figure.

Figure 2. The structure of CSI prediction

In the case of UE sided CSI prediction which is selected as sub-use case, both eigenvectors and RAW channel matrix can be considered as input of AI/ML model. Because, the UE directly measures channel matrix by receiving CSI-RS signals from gNB. In view of eigenvectors as input of AI/ML model, eigenvectors may have smaller dimensions and data size compared to raw channel matrix as input of AI/ML model. Thus, computational complexity of AI/ML model could be decreased. However, due to loss of information during making eigenvectors from RAW channel matrix, performance of AI/ML model could be degraded. At this time, it is important to check the degree of performance improvement in AI/ML model compared to the baseline.

Proposal 5: For the evaluation of CSI prediction sub use-case, consider the input of AI/ML model for CSI prediction as RAW channel matrix.

Evaluations on AI/ML-based CSI compression

In this section, we provide initial evaluation results on CSI compression sub use case based on SLS (System level simulations).

Evaluation assumption

For evaluation of AI/ML based CSI compression sub use case, the wireless channel data generated using the SLS for AI/ML based CSI compression sub use case. The parameters used for the SLS are provided in Table 1.

Table 1. Parameters for the SLS

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Scenario | Dense Urban (Macro only) |
| BWP | 48 RBs |
| Subcarrier spacing | 15 kHz |
| Subband/PRG size | 4 RBs |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx Power | 44dBm for 20MHz |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 9dB |
| Slot/non-slot | 14 OFDM symbol slot |
| SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth | 20 MHz |
| Number of layers | 1 |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |
| Channel estimation | Ideal |

Autoencoder based AI model for CSI compression

For AI/ML based CSI compression sub use case, AI model at UE (Encoder) gets wireless channel information as the input and generates compressed feedback information as the output, and AI model at gNB (Decoder) gets compressed feedback information as the input and generates the original wireless channel information as the output.

Autoencoder architecture is deployed for the evaluation of CSI compression. The architecture of Autoencoder is in Figure 5.

Figure 3. Architecture of the Autoencoder

We set eigenvectors of subbands as input of the encoder and output of the decoder in the evaluation. The number of latent variables is M, and Q bits are used to quantize each latent variable, i.e., the size of CSI feedback information is MQ bits.

The Encoder and Decoder in Autoencoder architecture are deployed using neural networks and design choices of the neural networks can be made with considerations of performance and complexity. We use Transformer-based neural networks for both the encoder and decoder. Figure 4 shows the Transformer-based model for AI/ML based CSI compression.

Figure 4. Transformer-based model for AI/ML based CSI compression

In order to train the AI Model, training samples are collected by the SLS. The parameters of AI Model and training are in Table 2.

Table 2. Parameters of AI Models

| Parameter | Value |
| --- | --- |
| Total number of samples | 57e4 |
| Portion of validation samples | 0.1 |
| Batch size | 512 |
| Total number of epoches | 512 |
| Learning algorithm | Adam |
| Learning rate | 0.0003 |

Summary of evaluation results of AI/ML-based CSI compression

According to the initial template discussed at the last meeting, the initial evaluation results are summarized as follows.

Table 3. Evaluation results for CSI compression on 1-on-1 joint training

| Assumptions |  | ETRI #1 |
| --- | --- | --- |
| CSI generation part | AI/ML model backbone | Transformer |
|  | Pre-processing |  |
|  | Post-processing |  |
|  | FLOPs/M | 179 |
|  | Number of parameters/M | 7.15 |
|  | [Storage /Mbytes] |  |
| CSI reconstruction part | AI/ML model backbone | Transformer |
|  | [Pre-processing] |  |
|  | [Post-processing] |  |
|  | FLOPs/M | 179 |
|  | Number of parameters/M | 7.15 |
|  | [Storage /Mbytes] |  |
| Common description | Input type | Eigenvector |
|  | Output type | Eigenvector |
|  | Quantization /dequantization method | Uniform quantization (2bits per each latent dimension) |
|  | Rank/layer adaptation settings for rank>1 | N/A |
| Dataset description | Train/k | 570 |
|  | Test/k | 1 |
|  | Ground-truth CSI quantization method (including scalar/codebook based quantization, and the parameters) | Float32 |
|  | Overhead reduction compared to Float32 if high resolution quantization of ground-truth CSI is applied |  |
| [Other assumptions/settings agreed to be reported] |  |  |
| Benchmark |  | R16 eTypeII |
| Benchmark assumptions, e.g., CSI overhead calculation method (Optional) |  |  |
| SGCS of benchmark, [layer 1] | X: <=80bits | 0.717 |
|  | Y: 100bits-140bits | 0.79 |
|  | Z: >=230bits | 0.831 |
| Gain for SGCS, [layer 1] | X: <=80bits | 2.79% |
|  | Y: 100bits-140bits | 0% |
|  | Z: >=230bits | 3.13% |
| NMSE of benchmark, [layer 1] | X: <=80bits |  |
|  | Y: 100bits-140bits |  |
|  | Z: >=230bits |  |
| Gain for NMSE, [layer 1] | X: <=80bits |  |
|  | Y: 100bits-140bits |  |
|  | Z: >=230bits |  |
| Other intermediate KPI (description/value) (optional) |  |  |
| Gain for other intermediate KPI (description/value) (optional) |  |  |

Observation 2: In the CSI compression sub-use case, with a Transformer based Autoencoder, there are no or slight (2~3%) improvements in terms of SGCS compared to the baseline (R16 eTypeII) for payload sizes X/Y/Z.

Evaluations on AI/ML-based CSI prediction

In this section, we provide basic evaluation results on the CSI prediction sub-use case based on a dataset derived from our SLS platform.

Evaluation assumption and dataset generation

For the evaluation of AI/ML based CSI prediction sub-use case, wireless channel datasets are generated by the SLS platform. The detailed parameters used for evaluation are described in Table 4.

Table 4. Parameters for channel generation

| Parameter | Value |
| --- | --- |
| Scenario | Urban Macro |
| Carrier Frequency | 4GHz |
| Inter-BS distance | 200m |
| Channel Model | NR Uma |
| Subcarrier spacing | 15kHz |
| Antennas at gNB | 32 ports (8,8,2,1,1,2,8) |
| Antennas at UE | 4 ports (1,2,2,1,1,2,2) |
| UE distribution | 100% outdoor |
| UE velocity | 20 km/h |

Details of generated datasets in preliminary simulation are shown in Table 5. We generate channel sample of 570 UEs during 500ms with CSI-RS interval 5ms. We utilize 90% of samples as training datasets and other 10% samples are used as test datasets

Table 5. Description of datasets

| Parameter | Value |
| --- | --- |
| CSI-RS periodicity | 5ms |
| Simulation time | 500ms |
| Number of UEs | 570 |
| Number of RB | 52 |
| Number of subblock | 12 |
| Number of samples | 50000 |
| Training/Test sets split | 90% / 10% |

AI/ML model for CSI prediction

The AI/ML model for CSI prediction utilizes the MLP-Mixer as the backbone. As the name implies, the MLP-mixer is simply composed of MLP (multi-layer perceptrons). The detailed structure of MLP-Mixer is shown in below Figure 5. Original channel images(inputs) are separated into patches and patches are entered them in a sequential order. In mixer layers, MLPs are repeatedly applied across either spatial locations or feature channels.

Figure 5. Structure of MLP-mixer

The parameters of AI/ML model and training are in Table 6. In the input dimension, K means the number of the observed channel matrix. And, in the output dimension, T means the number of the predicted channel matrix. For simplicity, we perform simulations considering only one Rx antenna. Our AI/ML model for prediction has 8.77M parameters and 738.87M FLOPs of computations

Table 6. Parameters of AI/ML model

| Parameter | Value |
| --- | --- |
| Backbone | MLP-Mixer |
| Input / Output type | RAW channel matrix |
| Input dimension | K x 2 x 32 x 52 |
| Output dimension | T x 2 x 32 x 52 |
| Batch size | 128 |
| Number of epochs | 150 |
| Optimizer | Adam |
| Initial learning rate | 0.001 |
| Loss function | MSE |

Evaluation results on CSI prediction

In this chapter, we show evaluation results on CSI prediction with sample and hold baseline which stores the nearest historical CSI. We evaluate intermediate KPIs such as NMSE. Firstly, we simulate 5 historical CSI information as the input of AI/ML model to predict 3 future CSIs. The results show that the AI/ML based prediction method outperforms the baseline. And, it can be confirmed that the performance of AI/ML model is superior in the results of distant times. In other words, the performance of the baseline rapidly degrades over time.

Table 7. Evaluation result on CSI prediction with 5 historical CSI

|  | 5ms CSI |  | 10ms CSI |  | 15ms CSI |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | AI/ML model | Baseline | AI/ML model | Baseline | AI/ML model | Baseline |
| NMSE [dB] | -4.608 | 0.636 | -3.626 | 2.708 | -3.269 | 3.401 |

Observation 3: AI/ML based CSI prediction improves performance compared to the baseline.

Evaluation results changing the number of historical CSI

The next results show the performance of AI/ML based CSI prediction while changing the number of CSI information used as input of AI/ML model. According to the results, if more CSI inputs are given, the performance of AI/ML prediction model slightly increases.

Table 8. Evaluation result on AI/ML based CSI prediction with various number of historical CSI

|  | 5ms CSI |  | 10ms CSI |  | 15ms CSI |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 5 samples | 10 samples | 5 samples | 10 samples | 5 samples | 10 samples |
| NMSE [dB] | -4.608 | -4.823 | -3.626 | -3.701 | -3.269 | -3.346 |

Observation 4: AI/ML based CSI prediction slightly improves performance as the number of historical input CSIs increase.

Evaluation results using eigenvectors as input of model

The next results show the performance of AI/ML based CSI prediction using eigenvectors as input of AI/ML model instead of RAW channel matrix. As shown in the results, AI/ML based prediction method outperforms the baseline. However, the degree of improvement of AI/ML model as input of eigenvectors is slightly reduced compared to the AI/ML model as input of the RAW channel matrix.

Table 9. Evaluation result on CSI prediction using eigenvectors as input

|  | 5ms CSI |  | 10ms CSI |  | 15ms CSI |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | AI/ML model | Baseline | AI/ML model | Baseline | AI/ML model | Baseline |
| NMSE [dB] | -4.276 | -3.615 | -3.099 | -1.734 | -2.401 | -0.682 |

Observation 5: Performance improvement of AI/ML based CSI prediction as input of eigenvectors is slightly reduced compared to the AI/ML model as input of RAW channel matrix.

Summary on evaluation results of AI/ML-based CSI prediction

In this chapter, according to the template without generalization discussed at the last meeting, the simulation results described in the previous sections are summarized as follows.

Table 10. Evaluation results for CSI prediction without model generalization/scalability

|  |  | ETRI (Section 3.2.3) | ETRI (Section 3.2.5) |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone | MLP-Mixer | MLP-Mixer |
|  | [Pre-processing] | Min-Max | Min-Max |
|  | [Post-processing] | Min-Max | Min-Max |
|  | FLOPs/M | 738.87M | 91.86M |
|  | Parameters/M | 8.77M | 4.01M |
|  | [Storage /Mbytes] | - | - |
|  | Input type | RAW channel matrix | Eigenvector |
|  | Output type | RAW channel matrix | Eigenvector |
| Assumption | UE speed | 20 Km/h | 20 Km/h |
|  | CSI feedback periodicity | 5ms | 5ms |
|  | Observation window (number/distance) | 5 / 5ms | 5 / 5ms |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) | 3 / 5ms / 5ms | 3 / 5ms / 5ms |
|  | Whether/how to adopt spatial consistency | - | - |
| Dataset size | Train/k | 45K | 45K |
|  | Test/k | 5K | 5K |
| Benchmark 1 |  | Nearest historical CSI w/o prediction | Nearest historical CSI w/o prediction |
| NMSE of Benchmark 1 (1,…N, N is number of prediction instances) |  | 0.636, 2.708, 3.401 | -3.615, -1.734, -0.682 |
| Gain for NMSE over Benchmark 1 (1,…N, N is number of prediction instances) |  | 5.244 (-4.608), 6.334 (-3.626), 6.670 (-3.269) | 0.661 (-4.276), 1.365 (-3.099), 1.719 (-2.401) |

Generalization performances of AI/ML-based CSI prediction

In the last RAN1 meeting, FL summarized potential issues related to the generalization of CSI prediction as follows [5].

- Drawing observations - As the templates for the CSI compressions sub use case (including the basic 1-on-1 joint training table, the generalization/scalability verification table, and the multi-vendor joint training as well as the separate training table) and the CSI prediction sub use case (the basic table as well as the generalization/scalability verification table) have been accomplished this meeting, and to be reported by companies before the next meeting, it is the initial plan of Moderator to start drawing observations in the next meeting; probably first with generalization/scalability which has less controversy, then followed by the performance gain of CSI prediction (to facilitate the check point of this sub use case), the performance gain of CSI compression, and the comparison among training types.

Generalization performance over UE speeds

In this chapter, we provide some evaluation results on generalization performances over various UE speeds. The AI/ML model is trained using a training dataset of a certain UE speed and this model is tested to the dataset of different speeds. According to the simulation results, changes in the speed of UE significantly degrade the performance of AI/ML model.

Table 11. Evaluation result on CSI prediction with various UE speeds

| NMSE [dB] | Testing (UE speed) |  |  |  |
| --- | --- | --- | --- | --- |
| Training (UE speed) | 60km/h | 30km/h | 20km/h | 10km/h |
| 60km/h | -2.667 | 1.079 | 2.025 | 6.724 |
| 30km/h | 0.328 | -3.426 | 2.235 | 5.891 |
| 20km/h | 3.603 | 4.462 | -4.587 | 7.365 |
| 10km/h | 6.169 | 6.853 | 9.468 | -7.141 |

To overcome this performance degradation, we trained AI/ML model with a mixed dataset which equally combined different UE speeds (20km/h, 30km/h and 60km/h). The results show that the performance degradation is alleviated compared to the result of AI/ML model trained on a certain speed.

Table 12. Evaluation result on CSI prediction with various UE speeds using the mixed dataset

| NMSE [dB] | Testing (UE speed) |  |  |
| --- | --- | --- | --- |
| Training (UE speed) | 60km/h | 30km/h | 20km/h |
| Mixed datasets | -1.702 | -3.605 | -3.357 |

Observation 6: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of UE speeds.

Observation 7: Mixed datasets of different UE speeds can mitigate performance degradation compared to datasets of single UE speed.

Proposal 6: Further evaluate the AI/ML based CSI prediction over various UE speeds to overcome performance degradation in untrained UE speed.

Generalization performance over Carrier Frequencies

In this chapter, we provide some evaluation results on generalization performances over various carrier frequencies. The AI/ML model is trained using a training dataset of a certain UE speed and this model is tested to the dataset of different carrier frequencies. According to the simulation results, changes in carrier frequency significantly degrade the performance of AI/ML model.

Table 13. Evaluation result on CSI prediction with various carrier frequencies

| NMSE [dB] | Testing (Carrier Freq.) |  |  |
| --- | --- | --- | --- |
| Training (Carrier Freq.) | 2GHz | 3GHz | 4GHz |
| 2GHz | -7.611 | -0.827 | -0.026 |
| 3GHz | -0.394 | -5.998 | 2.942 |
| 4GHz | -1.497 | 1.612 | -4.587 |

To overcome this performance degradation, we trained AI/ML model with a mixed dataset which equally combined different carrier frequencies (2GHz, 3GHz, 4GHz). The results show that the performance degradation is alleviated compared to the result of AI/ML model trained on a certain speed.

Table 14. Evaluation result on CSI prediction with various carrier frequencies using the mixed dataset

| NMSE [dB] | Testing (Carrier Freq.) |  |  |
| --- | --- | --- | --- |
| Training (Carrier Freq.) | 2GHz | 3GHz | 4GHz |
| Mixed datasets | -6.583 | -4.860 | -2.143 |

Observation 8: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of carrier frequencies.

Observation 9: Mixed datasets of different carrier frequencies can mitigate performance degradation compared to datasets of single carrier frequency.

Proposal 7: Further evaluate the AI/ML based CSI prediction over various carrier frequencies to overcome performance degradation in untrained carrier frequency.

Summary on generalization results of AI/ML-based CSI prediction

In this chapter, according to the template with model generalization discussed at the last meeting, the simulation results described in the previous sections are summarized as follows.

Table 15. Evaluation results for CSI prediction with model generalization over UE speeds

|  |  | ETRI #1 | ETRI #2 | ETRI #3 | ETRI #4 |
| --- | --- | --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone | MLP-Mixer | MLP-Mixer | MLP-Mixer | MLP-Mixer |
|  | [Pre-processing] | Min-Max | Min-Max | Min-Max | Min-Max |
|  | [Post-processing] | Min-Max | Min-Max | Min-Max | Min-Max |
|  | FLOPs/M | 738.87M | 738.87M | 738.87M | 738.87M |
|  | Parameters/M | 8.77M | 8.77M | 8.77M | 8.77M |
|  | [Storage /Mbytes] | - | - |  |  |
|  | Input type | RAW channel matrix | RAW channel matrix | RAW channel matrix | RAW channel matrix |
|  | Output type | RAW channel matrix | RAW channel matrix | RAW channel matrix | RAW channel matrix |
| Assumption | UE speed | 20 Km/h | 20 Km/h | 20 Km/h | 20 Km/h |
|  | CSI feedback periodicity | 5ms | 5ms | 5ms | 5ms |
|  | Observation window (number/distance) | 5 / 5ms | 5 / 5ms | 5 / 5ms | 5 / 5ms |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) | 3 / 5ms / 5ms | 3 / 5ms / 5ms | 3 / 5ms / 5ms | 3 / 5ms / 5ms |
|  | Whether/how to adopt spatial consistency | - | - |  |  |
| Generalization Case 1 | Train (setting#A, size/k) | 20km/h, 45K | 30km/h, 45K | 60km/h, 45K | 20km/h+30km/h+60km/h, 15K+15K+15K |
|  | Test (setting#A, size/k) | 20km/h, 5K | 30km/h, 5K | 60km/h, 5K | 20km/h, 5K |
|  | NMSE (1,…N, N is number of prediction instances) | -4.587, -3.607, -3.252 | -3.426, -2.953, -2.665 | -2.667, -2.561, -2.541 | -1.702, -1.384, -1.144 |
| Generalization Case 2 | Train (setting#A, size/k) | 20km/h, 45K | 30km/h, 45K | 60km/h, 45K | 20km/h+30km/h+60km/h, 15K+15K+15K |
|  | Test (setting#A, size/k) | 30km/h, 5K | 20km/h, 5K | 20km/h, 5K | 30km/h, 5K |
|  | NMSE (1,…N, N is number of prediction instances) | 4.462, 2.804, 3.091 | 2.235, 2.571, 2.875 | 2.205, 2.058, 2.173 | -3.605, -2.946, -2.502 |
| Generalization Case 3 | Train (setting#A, size/k) | 20km/h, 45K | 30km/h, 45K | 60km/h, 45K | 20km/h+30km/h+60km/h, 15K+15K+15K |
|  | Test (setting#A, size/k) | 60km/h, 5K | 60km/h, 5K | 30km/h, 5K | 60km/h, 5K |
|  | NMSE (1,…N, N is number of prediction instances) | 3.603, 2.106, 2.522 | 0.328, 0.723, 1.153 | 1.079, 1.100, 1.251 | -3.357, -2.896, -2.662 |

Table 16. Evaluation results for CSI prediction with model generalization over carrier frequencies

|  |  | ETRI #1 | ETRI #2 | ETRI #3 | ETRI #4 |
| --- | --- | --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone | MLP-Mixer | MLP-Mixer | MLP-Mixer | MLP-Mixer |
|  | [Pre-processing] | Min-Max | Min-Max | Min-Max | Min-Max |
|  | [Post-processing] | Min-Max | Min-Max | Min-Max | Min-Max |
|  | FLOPs/M | 738.87M | 738.87M | 738.87M | 738.87M |
|  | Parameters/M | 8.77M | 8.77M | 8.77M | 8.77M |
|  | [Storage /Mbytes] | - | - |  |  |
|  | Input type | RAW channel matrix | RAW channel matrix | RAW channel matrix | RAW channel matrix |
|  | Output type | RAW channel matrix | RAW channel matrix | RAW channel matrix | RAW channel matrix |
| Assumption | UE speed | 20 Km/h | 20 Km/h | 20 Km/h | 20 Km/h |
|  | CSI feedback periodicity | 5ms | 5ms | 5ms | 5ms |
|  | Observation window (number/distance) | 5 / 5ms | 5 / 5ms | 5 / 5ms | 5 / 5ms |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) | 3 / 5ms / 5ms | 3 / 5ms / 5ms | 3 / 5ms / 5ms | 3 / 5ms / 5ms |
|  | Whether/how to adopt spatial consistency | - | - |  |  |
| Generalization Case 1 | Train (setting#A, size/k) | 2GHz, 45K | 3GHz, 45K | 4GHz, 45K | 2GHz+3GHz+4GHz, 15K+15K+15K |
|  | Test (setting#A, size/k) | 2GHz, 5K | 3GHz, 5K | 4GHz, 5K | 2GHz, 5K |
|  | NMSE (1,…N, N is number of prediction instances) | -7.611, -7.012, -5.418 | -5.998, -5.070, -4.188 | -4.587, -3.607, -3.253 | -6.583, -6.457, -5.477 |
| Generalization Case 2 | Train (setting#A, size/k) | 2GHz, 45K | 3GHz, 45K | 4GHz, 45K | 2GHz+3GHz+4GHz, 15K+15K+15K |
|  | Test (setting#A, size/k) | 3GHz, 5K | 2GHz, 5K | 2GHz, 5K | 3GHz, 5K |
|  | NMSE (1,…N, N is number of prediction instances) | -0.827, 1.028, 1.969 | -0.394, 0.199, 0.931 | -1.497, 0.050, 0.577 | -4.860, -4.052, -3.325 |
| Generalization Case 3 | Train (setting#A, size/k) | 2GHz, 45K | 3GHz, 45K | 4GHz, 45K | 2GHz+3GHz+4GHz, 15K+15K+15K |
|  | Test (setting#A, size/k) | 4GHz, 5K | 4GHz, 5K | 3GHz, 5K | 4GHz, 5K |
|  | NMSE (1,…N, N is number of prediction instances) | -0.026, 2.460, 1.867 | 2.942, 3.523, 4.163 | 1.612, 2.396, 2.584 | -2.413, -1.559, -0.819 |

Conclusion

In this contribution, ETRI’s views on the evaluation on AI/ML for CSI feedback enhancement were shown and the following proposals and observations were made:

Proposal 1: For the evaluation of AI/ML-based CSI compression use case, companies to report the details on the quantization/dequantization including:

- Functional separability of compression and quantization

- Configuration of quantization/dequantization block (Scalar or vector quantization, fixed or dynamic codebook)

- Quantization aware/non-aware training of the AI models

Observation 1: AI/ML models with a functionally separable quantization block can adjust CSI payload sizes by using different quantization configurations.

Proposal 2: For the evaluation of CSI compression sub use-case with quantization aware/non-aware training, consider not introducing training and inference using unquantized latent variables for the performance upper bound.

Proposal 3: For the evaluation of CSI compression sub use-case, consider limiting the pre-processing operations as operations after the input data is prepared in the input format (e.g., precoding matrix, or explicit channel matrix).

Proposal 4: For the evaluation of CSI compression sub use-case, consider limiting the post-processing operations as operations after the output data is prepared in the output format (e.g., precoding matrix, or explicit channel matrix).

Proposal 5: For the evaluation of CSI prediction sub use-case, consider the input of AI/ML model for CSI prediction as RAW channel matrix.

Observation 2: In the CSI compression sub-use case, with a Transformer based Autoencoder, there are no or slight (2~3%) improvements in terms of SGCS compared to the baseline (R16 eTypeII) for payload sizes X/Y/Z.

Observation 3: AI/ML based CSI prediction improves performance compared to the baseline.

Observation 4: AI/ML based CSI prediction slightly improves performance as the number of historical input CSIs increase.

Observation 5: Performance improvement of AI/ML based CSI prediction as input of eigenvectors is slightly reduced compared to the AI/ML model as input of RAW channel matrix.

Observation 6: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of UE speeds.

Observation 7: Mixed datasets of different UE speeds can mitigate performance degradation compared to datasets of single UE speed.

Proposal 6: Further evaluate the AI/ML based CSI prediction over various UE speeds to overcome performance degradation in untrained UE speed.

Observation 8: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of carrier frequencies.

Observation 9: Mixed datasets of different carrier frequencies can mitigate performance degradation compared to datasets of single carrier frequency.

Proposal 7: Further evaluate the AI/ML based CSI prediction over various carrier frequencies to overcome performance degradation in untrained carrier frequency.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- RAN#95-e Meeting Report

- R1-2210753, Summary#7 of [110bis-e-R18-AI/ML-02], Moderator(Huawei)

- R1-2212966, Summary#6 for CSI evaluation of [111-R18-AI/ML], Moderator (Huawei)

- R1-2301940, Summary#5 for CSI evaluation of [112-R18-AI/ML], Moderator (Huawei)

- 3GPP RAN1, Chairman’s Notes, 3GPP RAN WG1 #111, November 2022