3GPP TSG RAN WG1 #112bis-e	R1-2303195

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.2.2.2

Source: 	ETRI

Title:	Discussion on other aspects on AI/ML for CSI feedback enhancement

Document for:	Discussion

Introduction

This contribution presents ETRI’s views on the other aspects on AI/ML for CSI feedback enhancement use case.

Discussion

CSI compression sub-use case

CSI compression is one of the key sub-use cases of AI/ML for CSI feedback enhancement. In this section, we describe the potential specification impacts on the AI/ML-based CSI feedback for the CSI compression sub-use case.

Training dataset collection

During RAN1#112, an agreement on the data collection is made as follows [7].

Agreement - In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection - In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection

For AI model training, a large amount of training dataset is required. In our view, the training dataset can be naturally collected on UE-side. UE can measure the downlink channel using CSI-RS and generate channel information or eigenvectors. In the channel measurement, since UE only can apply realistic channel estimation in the real networks, the ideal channel information is hardly obtained for the training. The objective of the CSI compression sub-use case is the accuracy of reconstructed CSI to the target CSI from ideal channel estimation and is also captured as the intermediate KPI. It would be beneficial to study on methods to improve performance of AI/ML models with training using datasets from realistic channel estimation.

Proposal 1: Consider further studies on performance improvement of AI models with training datasets from realistic channel estimation.

One possible method is training AI models to additionally remove the noises from the realistic channel estimations. Although clear targets are normally required for training the denoising function, the clear target (i.e., target CSI from ideal channel estimation) is not available in general. In Noise2noise [2], a training method for denoising without clear targets is introduced, which utilizes sample pairs that experienced independent noise processes. To apply such a training method in training AI models for CSI compression, obtaining a training dataset of sample pairs is required.

Observation 1: One possible performance improvement of AI models with training datasets from realistic channel estimation, is training a denoising function additionally.

Observation 2: To train the additional denoising function of the AI model for CSI compression, obtaining a training dataset with pairs can be required.

UCI generation

During RAN1#112, an agreement on the CSI configuration and report is made as follows [7].

Agreement In CSI compression using two-sided model use case, further study the following aspects for CSI configuration and report: - NW configuration to determine CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration. - How UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network.

For the report of downlink CSI made by AI models, UCI needs to be generated on UE. CSI-related contents of UCI payload include RI, CQI, LI, PMI and etc.

For the UCI payload generation in AI/ML-based CSI feedback, there can be two cases that determine the actual UCI payload size as follows.

Case 1: NW configures UE to generate the UCI payload in a certain size

Case 2: NW configures and UE determines the actual UCI payload size

For the first case, NW determines the UCI payload size and configures UE to generate UCI payload with the UCI payload size, then UE generates the UCI payload according to the configurations by NW. To configure UCI payload size, NW designates a specific CSI generation model only or a specific CSI generation model with quantization information. In this case, NW can naturally infer the length of CSI feedback information without additional information.

For another case, NW can configure the maximum UCI payload size and UE generates the UCI payload within the maximum UCI payload size, as in the legacy CSI report. NW can also configure a set of possible CSI generation models and/or a set of possible quantization information regarding the models. The set of configurable quantization information may have been previously exchanged between NW and UE, for example, the configurable quantization information may have been delivered along with the model information. In this case, UE can dynamically generate various sizes of UCI payload by selecting CSI generation model and/or quantization. The details of UCI payload (e.g., CSI generation model, quantization-related information) should be informed to NW for proper reconstruction. This information can be included in the Part1 information of CSI feedback information.

Proposal 2: In CSI compression using two-sided AI model, further study the following potential specification impacts on UCI configuration.

- NW configures UE to generate the UCI payload in a certain size.

- UE generates the UCI payload within the maximum UCI payload size. UE delivers to or shares details of the UCI payload (including quantization-related information)

One example of how the UE can dynamically adjust size of UCI payload is by setting asymmetric quantization levels for the encoder output. For proper reconstruction in NW-side, information on quantization may be additionally delivered to NW-side, or NW may infer the quantization-related information using the size of the received UCI payload.

Observation 3: By setting asymmetric quantization levels for the encoder output allows dynamic adjustments of UCI payload.

Performance monitoring based model selection

During RAN1#112, an agreement on the intermediate KPI based monitoring is made as follows [7].

Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

As in the agreement, UE can monitor performance based on the intermediate KPIs (e.g., NMSE or SGCS) using CSI reconstruction model output. The results of performance monitoring can be used to select the AI model (i.e., CSI generation model) that the UE uses for CSI compression when the UE has more than one CSI reconstruction models. The model selection can be performed by the UE side, or the performance monitoring results can be delivered to the NW so that model selection can be performed in the NW.

Proposal 3: Consider further studies on potential specification impacts of model selection using the performance monitoring result.

Model selection can occur when AI/ML-based CSI feedback is enabled. The performance of the selected model may be periodically monitored after enabling of AI/ML-based CSI feedback and the model selection. For this purpose, NW may request UE to periodically monitor the performance of the selected AI/ML model and report the result of performance monitoring. When the performance of the selected model degrades significantly, UE or NW can change the AI/ML model, or configure UE to generate CSI using the legacy method (i.e., non-AI/ML).

Proposal 4: Consider further studies on potential specification impacts of model changes and fallback operation using the performance monitoring result.

CSI prediction sub-use case

In the RAN #111 meeting, time domain CSI prediction is selected as sub-use case for AI/ML CSI enhancement [6]. In this section, we describe the potential specification impacts on CSI prediction sub-use case even if the discussion was deferred until the RAN1 #112bis-e.

Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer futher till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

CSI feedback

In AI/ML based CSI prediction, an UE generates the prediction results (e.g. RAW channel matrix). And, in order to transfer these results to gNB, the format of CSI feedback can be modified. Each CSI feedback might contain channel information from multiple prediction windows.

The simplest way is to use the existing baseline scheme such as CSI enhancement which is currently discussed in R18 MIMO agenda. According to current discussions, the amount of data can be reduced through Doppler domain (time domain) compression. Even if R18 CSI enhancement is adopted, it may be possible to further compress feedback information using the characteristics of the AI/ML based CSI prediction method.

Moreover, AI/ML based CSI compression is also being studied. Thus, we can consider the method to combine benefits of CSI prediction and compression together to deliver CSI prediction results.

Proposal 5: Study the potential specification impacts on transferring results of CSI prediction.

Model monitoring

In RAN1#109-e meeting, the agreement about collaboration levels is made [2].

Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels - Level x: No collaboration - Level y: Signaling-based collaboration without model transfer - Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary

And, in the last meeting, FL in agenda item 9.2.2.1 summarized potential issues of a level y/z collaboration related to AI/ML based CSI prediction [8].

- CSI prediction: - Modeling of level-x/y/z based CSI prediction (Issue#4-2): As the inputs for this meeting is limited, for how to model the impact of LCM to the performance, we may discuss it further.

In our view, even if we utilize LCM procedure of other one-sided model cases, level y collaboration is needed in some situations. As defined in the agreement about sub-use case, the AI/ML model for CSI prediction is located in the UE. As shown in our evaluation result [9], the performance of CSI prediction is severely degraded according to the change in other variables such as UE speed and carrier frequency.

Therefore, UE has to monitor the performance to deal with performance degradation. On the other hand, the results of channel prediction are transferred to the gNB and can be used for downlink transmission. Therefore, the help of gNB is essential to estimate and monitor the performance of the AI/ML model used by UE. Since this corresponding procedure requires separate signaling between the UE and the gNB, it can be classified as level y collaboration. In this process, information that is not included in LCM procedure of other cases may be delivered through this signaling

Proposal 6: Study the potential specification impacts on the level y collaboration for model monitoring.

Observation 4: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of UE speeds and carrier frequency.

Conclusion

In this contribution, ETRI’s views on other aspects of AI/ML framework for CSI feedback enhancement use case were shown and the following proposals and observations were made:

Proposal 1: Consider further studies on performance improvement of AI models with training datasets from realistic channel estimation.

Observation 1: One possible performance improvement of AI models with training datasets from realistic channel estimation, is training a denoising function additionally.

Observation 2: To train the additional denoising function of the AI model for CSI compression, obtaining a training dataset with pairs can be required.

Proposal 2: In CSI compression using two-sided AI model, further study the following potential specification impacts on UCI configuration.

- NW configures UE to generate the UCI payload in a certain size.

- UE generates the UCI payload within the maximum UCI payload size. UE delivers to or shares details of the UCI payload (including quantization-related information)

Observation 3: By setting asymmetric quantization levels for the encoder output allows dynamic adjustments of UCI payload.

Proposal 3: Consider further studies on potential specification impacts of model selection using the performance monitoring result.

Proposal 4: Consider further studies on potential specification impacts of model changes and fallback operation using the performance monitoring result.

Proposal 5: Study the potential specification impacts on transferring results of CSI prediction.

Proposal 6: Study the potential specification impacts on the level y collaboration for model monitoring.

Observation 4: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of UE speeds and carrier frequency.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- 3GPP RAN1, Chairman’s Notes, 3GPP RAN WG1 #109-e, May 2022

- 3GPP RAN1, Chairman’s Notes, 3GPP RAN WG1 #110, August 2022

- 3GPP TS 38.213 V17.0.0.

- 3GPP RAN1, Chairman’s Notes, 3GPP RAN WG1 #110bis-e, October 2022

- 3GPP RAN1, Chairman’s Notes, 3GPP RAN WG1 #111, November 2022

- 3GPP RAN1, Chairman’s Notes, 3GPP RAN WG1 #112, March 2023

- R1-2301940, Summary#5 for CSI evaluation of [112-R18-AI/ML], Moderator (Huawei)

- R1-2303194, Evaluation on AI/ML for CSI feedback enhancement, ETRI, April 2023