3GPP TSG RAN WG1 #112bis-e			      R1-2303196

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.2.3.2

Source: 	ETRI

Title: 	Discussion on other aspects on AI/ML for beam management

Document for:	Discussion/Decision

Introduction

In the RAN#94 e-meeting, a new SI to study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface had been approved [1]. In the RAN1 #112 meeting [2], AI/ML model monitoring and information reported from UE to network including potential specification impacts for beam management have been discussed. In this contribution, we further discuss the details of performance metric(s) of AI/ML model monitoring and reported information from UE to network for BM-Case1 and BM-Case2 with a UE-side AI/ML model.

Discussion

- 2.1 AI/ML model monitoring

In the RAN1#112 meeting, the following agreement for the performance metric(s) of AI/ML model monitoring were made.

Agreement Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, study the following alternatives (including feasibility/necessity) with potential down-selection: - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - Other alternatives are not precluded - Note: At least the performance and spec impact should be considered

Alt.1 is to use beam prediction accuracy as the performance metric of AI/ML model monitoring. The primary objective of beam prediction based on AL/ML is to determine how well the AL/ML model predicts the optimal beam. The accuracy of beam prediction is the most objective performance metric that reflects this objective. Therefore, beam prediction accuracy should be the primary performance metric to consider for monitoring the AI/ML model.

Additionally, regarding Top-K beam prediction accuracy, the following additional considerations are possible: From the perspective of network optimization, where multiple UEs need to be considered simultaneously for beam management, providing information by predicting Top-K rather than only Top-1 would be more advantageous. Therefore, we support using Top-K beam prediction accuracy with K >= 1 as a performance metric. Furthermore, we can choose whether to predict the accurate ranking (i.e., Top-K/1) or to predict the top K beams that include the optimal beam regardless of their rank (i.e., Top-1/K). This means that we may use either Top-K/1 (a.k.a. Top-K Recall) as a performance metric, which indicates the probability that the model correctly predicts the optimal beam as the first answer among the top K predicted results, or Top-1/K (a.k.a. Top-K), which measures whether the predicted top K results contain the optimal beam only. Further discussion is needed to determine which performance metric to use.

Alt.2 is to use link quality related KPIs as the performance metrics of AI/ML model monitoring. By employing link quality-related indicators (e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER), we can evaluate the performance of AI/ML model from multiple perspectives. First of all, monitoring link quality metrics ensures relevance to actual communication performance. The primary goal of beam prediction models is to select appropriate beams for achieving better communication performance. Since link quality metrics directly reflect the performance of wireless communication, monitoring these metrics can help us evaluate the performance of beam prediction models in real communication environments. Moreover, link quality metrics can help assess the adaptability and robustness of the model in diverse environments. Link quality is influenced by various environmental factors and interference. By using link quality metrics to monitor model performance, we can assess the model's adaptability and robustness across different communication environments. As a result, link quality related KPIs can monitor system performance, which is difficult to judge only by the classification performance of AI/ML model, which can be confirmed by Alt.1's beam prediction accuracy metric.

Alt.3 is to use performance metric based on input/output data distribution of AI/ML model as the performance metrics of AI/ML model monitoring. Data distribution-based monitoring is a method of evaluating the performance and stability of AI/ML systems by analyzing the input and output data of the system. This monitoring can be mainly divided into two types: input-based and output-based monitoring. Input-based monitoring ensures the proper operation of the system by verifying the basic attributes of the input data used in the AI/ML system. For example, it evaluates the quality of input data by examining parameters such as SNR or delay spread. Therefore, input-based monitoring can be performed using some link quality related KPIs in Alt.2. Output-based monitoring detects changes in output data over time by checking how the output of an AI/ML system changes over time, enabling us to verify whether the generated values are within the expected range. Out-of-range values ultimately lead to performance degradation of AI/ML model. Therefore, whether the distribution of the output data is abnormal is reflected in the performance metrics of Alt.1. Monitoring the input/output data distribution of an AI/ML model is useful for stability verification, but it has limitations when it comes to evaluating the actual performance of the model. Therefore, sufficient discussion is needed on how to define the performance metric based on input/output data distribution of AI/ML model.

Alt.4 is to use the L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP as the performance metrics of AI/ML model monitoring. This performance metric is only available when the AI/ML model predicts RSRP. That is, it is difficult to utilize when the AI/ML model outputs an optimal beam ID or an optimal beam probability. Additionally, it should be considered that the use of reporting only the difference value without reporting individual RSRP is limited because it cannot be used for Alt.2 and is only available for Alt.4.

Based on the above discussion, we are therefore proposing to adopt Alt.1 and Alt.2 as a starting point for the further study.

Proposal 1: Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, support to study the following as a priority:

-	Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy

-	Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER

- 2.2 AI/ML model output at UE side

In the RAN1#112 meeting, the following was agreed on reporting information from the UE to the network for BM-Case1 and BM-Case2 with a UE-side AI/ML model:

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity, feasibility and the potential specification impact (if needed) of the following information reported from UE to network: - Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s) - Whether/how to differentiate predicted L1-RSRP and measured L1-RSRP - Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams) - FFS: Definition/content of confidence/probability information - Note: At least the performance and spec impact should be considered

The definition of probability associated with the output of an AI/ML model may vary depending on the purpose of the model. When the AI/ML model's ultimate goal is to predict Top-1 beam, the model is typically designed assuming a single-label problem, and the probability corresponding to each beam being the optimal beam is expressed in a dependent manner. In this case, the softmax activation function is used for the output layer of the AI/ML model, which means that the sum of probabilities corresponding to each beam is equal to 1. On the other hand, when the AI/ML model's ultimate goal is to predict Top-K beams, the model is designed assuming a multi-label problem for selecting K beams. The probability values of each beam for selecting K beams can be expressed in a dependent or independent manner. When the probability values corresponding to each beam for selecting K beams are expressed in a dependent manner, the softmax activation function is used for the output layer of the AI/ML model, and K beams are selected in order of the largest probability values. When the probability values of each beam for selecting K beams are expressed in an independent manner, the sigmoid activation function is used for the output layer of the AI/ML model, and the probability corresponding to each beam being included in the Top-K beams is independently predicted. In this case, the sum of probabilities corresponding to each beam may be greater than 1.

Alternatively, for Top-K beam prediction, we can also design an AI/ML model by using the probability corresponding to all cases of selecting K out of N total beams as the corresponding probability information for the output of the AI/ML model, classifying all cases of selecting K out of N total beams as individual classes. In this case, the number of output nodes in the AI/ML model is N-Choose-K, and the output value of each node represents the probability of the corresponding K beams being selected as Top-K beams. However, this model is difficult to apply when K is variable, and it cannot provide priority information within K beams. Additionally, the number of output nodes in the AI/ML model exponentially increases as K increases, leading to a significant increase in the model's complexity. Such an increase in complexity makes stable training difficult and also increases the time required for both training and inference. Therefore, we support defining the probability information related to the AI/ML model's output as the probability that each beam is the optimal beam.

Lastly, it is considered that there may not be a need for separate content for confidence in addition to probability. Confidence is a measure of certainty in a prediction, and it is generally represented by the highest probability value among the predicted classes. Therefore, if the UE reports probabilities to the NW, there may not be a need for separate confidence reporting.

Based on the above discussion, we suggest the following proposal:

Proposal 2: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, consider the following as the baseline for the information reported from UE to NW:

-	The probability information associated with the AI/ML model output can be defined as the probability of each beam being identified as the optimal beam.

-	In terms of reporting information, there may not be a need for individual content for confidence in addition to probability.

- 2.3 AI/ML model inference at UE side

In the RAN1#112 meeting, for BM-Case1 and BM-Case2 with a UE-side AI/ML model, it was agreed to study the potential specification impact of the network indicating association/mapping of beams within Set A and beams within Set B to UE. In the RAN1#110 meeting [3], the following alternatives had already been achieved for Set A and Set B. In this contribution, we further address the association/mapping information based on the relationship between Set A and Set B.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused). Agreement For the sub use case BM-Case1, support the following alternatives for further study: - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A) - Alt.2: Set B is a subset of Set A - Note1: Set A is for DL beam prediction and Set B is for DL beam measurement. - Note2: The beam patterns of Set A and Set B can be clarified by the companies. Agreement For the sub use case BM-Case2, further study the following alternatives: - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A) - Alt.2: Set B is a subset of Set A (Set A and Set B are not the same) - Alt.3: Set A and Set B are the same - Note1: The beam pattern of Set A and Set B can be clarified by the companies.

From the perspective of AI/ML model design, when using the measurements of Set B as model inputs and the information regarding the optimal beam from Set A as model outputs, if Set A and/or Set B are variable, the model should be individually designed for each case. Therefore, it seems necessary to define a model ID for each case to distinguish between AL/ML models. Consequently, it is considered efficient for the network to deliver this ID to the UE for the indication of the associated Set A. This approach can be applied equally regardless of the relationship between Set A and Set B (i.e., Alt.1~3). As a result, NW should provide this ID to UE to enable AI/ML model selection, and the UE can directly infer the relationship between Set A and Set B from the ID. Regarding this approach, it is believed that additional discussion may be necessary for potential specification issues.

Proposal 3. For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of the approach that distinguishes the relationship between Set A and Set B using AI/ML model's ID.

Conclusions

In this contribution, we provided our view on specification impact on AI/ML for beam management and the followings are proposed.

Proposal 1: Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, support to study the following as a priority:

-	Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy

-	Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER

Proposal 2: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, consider the following as the baseline for the information reported from UE to NW:

-	The probability information associated with the AI/ML model output can be defined as the probability of each beam being identified as the optimal beam.

-	In terms of reporting information, there may not be a need for individual content for confidence in addition to probability.

Proposal 3. For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of the approach that distinguishes the relationship between Set A and Set B using AI/ML model's ID.

References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#94e, Electronic Meeting, Dec. 6 - 17, 2021.

- Chairman’s notes, RAN1#112, February 27 - March 3, 2023.

- Chairman’s notes, RAN1#110, Toulouse, France, October 10 - 14, 2022.