3GPP TSG RAN WG1 Meeting #112bis-e  			    			             R1- 2303200

e-Meeting, April 17th – April 26th, 2023

# Source:	ETRI

# Title:		Discussion on co-channel coexistence for LTE sidelink and NR sidelink

# Agenda item:	9.4.2

# Document for:	Discussion

Introduction

In the last RAN1#112 meeting, the following agreements and working assumption for co-channel coexistence for LTE sidelink and NR sidelink have been made [1]:

Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied Working assumption - For dynamic resource pool sharing, select one option between 1-1 and 1-2, and select one option between 3-1 and 4-1-a, and define how the two selected options can be used in the operation: - Option 1-1: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is performed by the UE transmitting PSFCH - FFS whether/how to define condition(s) under which the UE transmitting PSFCH drops its PSFCH transmission overlapping with the LTE SL resources reserved for LTE SL transmission - Option 1-2: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is ensured by the UE transmitting PSSCH - FFS whether/how to define condition(s) of selecting NR SL candidate resources for NR SL PSCCH/PSSCH - Option 3-1: Additional PSFCH periodicity(ies) is introduced - Alt 3-1-a: 10 logical slots - Alt 3-1-b: 8 logical slots - Alt 3-1-c: 5 logical slots - FFS: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration - Option 4-1: PSFCH resources and LTE SL resources are TDMed - Alt 4-1-a: PSFCH resources is confined within the guard symbol of the LTE SL subframe in the time domain - Note: selecting a single option is not precluded Agreement In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module - Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

In the last RAN#99 meeting, the WID for NR sidelink evolution has been updated as follows [2]:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

Based on the above agreements/working assumption and the updated WID, we introduce our views on the co-channel coexistence of LTE sidelink and NR sidelink. According to the endorsed proposals in [3], the coexistence objective should be completed by June 2023. The revised WID limits the supporting SCSs to 15 and 30kHz, and provides solutions for supporting the SCSs as shown in the above. Therefore, we focus on the remaining issues on dynamic resource sharing in this contribution.

Co-channel coexistence of LTE sidelink and NR sidelink

Higher SCS

In addition to 15kHz, only 30kHz is allowed for dynamic resource pool sharing between LTE SL and NR SL. For NR PSCCH/PSSCH transmissions with 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer. In case that the first and the subsequent NR slots are selected for SL transmissions and the power difference between two slots is small, there will be no AGC issue (as shown in Figure 1 (a)). In case that only the first NR slot is selected and the subsequent NR slot is empty (as shown in Figure 1 (b)), it would result in the ADC gain being lower in the second half part of LTE SL subframe. Even though it might cause an increased quantization error, the impact due to the quantization error might not be as much as the impact of ADC saturation. However, if the subsequent overlapping NR slot is selected and the transmit power of the subsequent NR slot is much higher than that of the first NR slot, the AGC issue can be occurred due to the power difference between two slots (as shown in Figure 1 (c)). Therefore, certain restriction(s) for the selection should be applied in order to resolve the AGC issue. The restriction(s) can be limitation to the same destination UE for SL transmission in both slots, power control related parameters (e.g., the same number of RBs, DL or SL based power control, etc.), and others. Otherwise, the subsequent overlapping NR slot is selected by UE implementation for the simplicity.

(a)                                                        (b)                                                        (c)

Figure 1. Examples of AGC issues

Proposal 1: For the selection of the subsequent overlapping NR slot, in order to avoid AGC issue due to power difference between the first and the subsequent NR slots,

- Certain restriction(s) should be applied, e.g., limitation to the same destination UE for SL transmission in both slots, power control related parameters, and others

- Or the selection is up to UE implementation

Candidate resource determination of NR SL module

Based on the previous agreement, for determination of candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE(s) in NR SL resource (re-)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission. For the list of the above initial SL RSRP threshold, there are several alternatives. If both LTE and NR SCSs are the same, NR SL RSRP threshold list can be reused. However, if NR SCS is higher than LTE SCS, a separate SL RSRP list can be (pre-)configured considering different SCSs between LTE and NR. For determining the LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE should be determined in a same way as LTE legacy.

Proposal 2: For determination of candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE,

- For the list of the initial SL RSRP threshold,

  - In case that both LTE and NR SCSs are the same, NR SL RSRP threshold list (pre-)configured in a NR SL resource pool

  - In case that NR SCS is different from LTE SCS, SL RSRP threshold list separately (pre-)configured for dynamic resource pool sharing

- For the LTE SL periodic reserved resources by other LTE SL UE(s),

  - the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period based on the formula of Q in TS 36.213

For handling of resources for LTE SL module’s own LTE SL transmission and handling of non-monitored subframe of LTE SL module, the LTE SL UE cannot monitor certain subframe(s) in which the LTE SL UE is transmitting due to half-duplex restriction. And the subframe(s) can be considered as non-monitored subframe(s). At the same time, NR SL module also cannot monitor NR slot(s) corresponding to the LTE subframe(s). Therefore, we do not see the difference between non-monitored subframe(s) and subframe(s) used for its own transmission. For determination of repeatedly reserved resources, Q times according to the formula of Q in TS 36.213 can be also applied.

Proposal 3: For determination of candidate resource set for NR SL considering both the LTE SL reserved resources for LTE SL module’s own LTE SL transmission and/or the LTE SL resources associated with non-monitored subframe of LTE SL module,

- In NR SL resource (re-)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with the LTE SL reserved resources for LTE SL module’s own LTE SL transmission and/or the LTE SL resources associated with non-monitored subframe of LTE SL module

  - For determining the LTE SL resources reserved for LTE SL module’s own LTE SL transmission and/or the LTE SL resources associated with non-monitored subframe of LTE SL module, all the LTE SL resources in the above are assumed to be repeatedly reserved Q times according to the formula of Q in TS 36.213

For NR SL resource (re-)selection procedure in the above, at least following parameters should be included in the candidate information shared by LTE SL module. Regarding the LTE logical subframe related information we also agree that it is RAN1’s understanding that the NR SL module is aware of the LTE SL resource pool configurations as per Rel-16 in-device coexistence.

Proposal 4: At least the following parameters should be included in the candidate information shared by LTE SL module:

- Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs

- SL RSRP measurement results

- Resource reservation periods based on decoded SCI and for own LTE SL transmissions

- Priority based on decoded SCI and for own LTE SL transmissions

- Time and frequency location of resources used for own LTE SL transmissions

Timeline of Information Shared by LTE SL Module

Regarding the timeline to pass the candidate information from LTE SL module to NR SL module, it was agreed that the latest time for sharing is n-T which T ≤ Tmax ms, and Tmax is 4ms considering Rel-16 in-device coexistence framework. In addition to the latest time, the earliest time (i.e., n-Tvalid1) can be also considered to prevent usage of outdated information.

Figure 2. Example of timeline of sensing and resource (re-)selection procedure

Figure 2 shows an example of additional timeline for sharing the candidate information on top of legacy NR sidelink sensing and resource (re-)selection procedure. According to the additional timeline, NR SL module can use the candidate information shared by LTE SL module between n-T-Tvalid1 and n-T. It will be beneficial to prevent the candidate information to be outdated. The earliest time can be determined considering LTE sensing window. Therefore, Tvalid1 should be 1000ms.

Proposal 5: For dynamic resource pool sharing, the starting LTE SL subframe used to determine the information shared with the NR SL module is at most Tvalid1 ms prior to the time (n-T) where the shared information is known by the NR SL module.

- Tvalid1 is defined as 1000ms

Regarding Tvalid2 which is the time for the ending LTE SL subframe used to determine the information shared with the NR SL module, we already have the latest time constraint, i.e., T ≤ Tmax. and the latest time will be determined within the constraint by UE implementation. Therefore, we think no need to define Tvalid2 and it can be determined by UE implementation considering the latest time.

Proposal 6: Regarding the time for the ending LTE SL subframe used to determine the information shared with the NR SL module, i.e., Tvalid2, it can be determined by UE implementation.

Summary

In this contribution, we made the following proposals to support co-channel coexistence between LTE sidelink and NR sidelink.

Proposal 1: For the selection of the subsequent overlapping NR slot, in order to avoid AGC issue due to power difference between the first and the subsequent NR slots,

- Certain restriction(s) should be applied, e.g., limitation to the same destination UE for SL transmission in both slots, power control related parameters, and others

- Or the selection is up to UE implementation

Proposal 2: For determination of candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE,

- For the list of the initial SL RSRP threshold,

  - In case that both LTE and NR SCSs are the same, NR SL RSRP threshold list (pre-)configured in a NR SL resource pool

  - In case that NR SCS is different from LTE SCS, SL RSRP threshold list separately (pre-)configured for dynamic resource pool sharing

- For the LTE SL periodic reserved resources by other LTE SL UE(s),

  - the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period based on the formula of Q in TS 36.213

Proposal 3: For determination of candidate resource set for NR SL considering both the LTE SL reserved resources for LTE SL module’s own LTE SL transmission and/or the LTE SL resources associated with non-monitored subframe of LTE SL module,

- In NR SL resource (re-)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with the LTE SL reserved resources for LTE SL module’s own LTE SL transmission and/or the LTE SL resources associated with non-monitored subframe of LTE SL module

  - For determining the LTE SL resources reserved for LTE SL module’s own LTE SL transmission and/or the LTE SL resources associated with non-monitored subframe of LTE SL module, all the LTE SL resources in the above are assumed to be repeatedly reserved Q times according to the formula of Q in TS 36.213

Proposal 4: At least the following parameters should be included in the candidate information shared by LTE SL module:

- Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs

- SL RSRP measurement results

- Resource reservation periods based on decoded SCI and for own LTE SL transmissions

- Priority based on decoded SCI and for own LTE SL transmissions

- Time and frequency location of resources used for own LTE SL transmissions

Proposal 5: For dynamic resource pool sharing, the starting LTE SL subframe used to determine the information shared with the NR SL module is at most Tvalid1 ms prior to the time (n-T) where the shared information is known by the NR SL module.

- Tvalid1 is defined as 1000ms

Proposal 6: Regarding the time for the ending LTE SL subframe used to determine the information shared with the NR SL module, i.e., Tvalid2, it can be determined by UE implementation.

References

- Chairman’s notes, RAN1#112, February 2023.

- RP-230077, “WID revision: NR sidelink evolution,” OPPO

- RP-230769, “Way forward on R18 SL-Evo WI objectives,” OPPO