3GPP TSG RAN1 WG1 Meeting #112-bis-e 	R1-2303204

Apr 17  –  Apr 26, 2023

Source:	ETRI

Title:	Discussion on coverage enhancement for NR NTN

Agenda Item:	9.9.1 Coverage Enhancement for NR NTN

Document for:	Discussion/Decision

1	Introduction

The following conclusion and working assumptions were made in RAN1#112 [1].

Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g. DMRS ports Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. - Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design) Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

In this contribution, the issues for coverage enhancement would be discussed.

2	Discussion

2.1	PUCCH repetition for Msg4 HARQ-ACK

- Option A: PRACH preamble and/or occasion

  - FFS: whether PRACH resource partitioning is needed for indication of [repetition request  or capability report]

  - FFS: whether or not indication of repetition factor is assumed

  - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings

- Option B: Higher layer signaling in Msg3 PUSCH

  - FFS: which signaling is used

  - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2.

- Option C: Physical layer signaling in Msg3 PUSCH

FFS: which signaling is used, e.g. DMRS ports

In the previous RAN1 meeting #112, working assumptions were established for repetition request or capability report of PUCCH repetition for Msg4 HARQ-ACK. At the end of the meeting, following options were agreed:

For option A, existing mechanism for Msg3 repetition can be reused. In rel. 17 CE Msg3 repetition, UE requests its needs for Msg3 repetition after measuring RSRP of SSB. If result of RSRP measurement is below the threshold indicated by SIB, UE uses predefined preambles or accesses certain RACH occasions to request repeated transmissions for Msg3. The repetition request or capability report of PUCCH repetition for Msg 4 HARQ-ACK can be delivered similarly to this mechanism.

Proposal 1: If repetition request or capability report is indicated by PRACH preamble and/or occasion, existing mechanism for Msg3 repetition can be reused.

Suppose that RSRP threshold is configured for PUCCH repetition for Msg4 HARQ-ACK. UE can request PUCCH repetition for Msg 4 HARQ-ACK if measured RSRP is below the threshold. In rel. 17 CE Msg3 repetition, the threshold for Msg3 repetition is indicated in rsrp-ThresholdMsg3-r17. This threshold can be reused for PUCCH repetition for Msg 4 HARQ-ACK because transport channel state for Msg3 and PUCCH for Msg4 HARQ-ACK will have similar characteristics. By adding or subtracting some amount of constant to rsrp-ThresholdMsg3-r17, UE can interpret this calculation result as a threshold for PUCCH repetition for Msg 4 HARQ-ACK. Alternatively, gNB can add some RRC parameters to indicate rsrp-Threshold for PUCCH repetition for Msg4 HARQ-ACK like “rsrp-thresholdMsg4HARQ-Ack-r18”.

Observation 1: The threshold for Msg3 repetition can be reused for PUCCH retransmission for Msg4 HARQ-ACK.

If UE follows the required number of PUCCH retransmissions for Msg4 HARQ-ACK by the gNB’s signaling, UE only needs to inform the gNB its repetition capability or whether it needs repetition. UE can inform the gNB its repetition capability or repetition request by using predefined RO/preamble or by using high level/phy level signaling of Msg3.

However, since the method of accessing a particular RO or using a particular preamble to make a user's repeated transmission request has already been used in Rel.17, further subdivisions of RO or preambles for Msg4 may increase the probability of preamble collision.

Also, capability reporting or repetition request using Msg3 requires the consent or agreement of the higher layer (RAN2) for the high-level signaling procedure.

If UE needs to transmit the required number of retransmissions according to their measured RSRP, UE can inform the gNB its required number of repetitions by using predefined RO/preamble or by using high level/phy level signaling of Msg3.

However, since the method of accessing a particular RO or using a particular preamble to make a user's repeated transmission request has already been used in Rel.17, further subdivisions of RO or preambles for Msg4 may increase the probability of preamble collision.

Moreover, request the number of PUCCH for Msg4 HARQ-ACK at UE side through Msg1 preamble or particular RO can be waste of resource and increase collision probability because UEs in NTN environment experience similar channel conditions and may need similar number of retransmissions.

Also, capability reporting or repetition request using Msg3 requires the consent or agreement of the higher layer (RAN2) for the high-level signaling procedure, as in the above case.

Observation 2: Request the number of PUCCH for Msg4 HARQ-ACK at UE side through Msg1 preamble or particular RO can be waste of resource and increase collision probability. Because UEs in NTN environment experience similar channel conditions and may need similar number of retransmissions, there is a possibility that some Preambles and ROs will not be used.

Proposal 2: In case of indication repetition request or capability report using PRACH preamble and/or occasion, a method for minimizing collision probability for more detailed fragmentation should be discussed.

Proposal 3: Capability reporting or repetition request using Msg3 requires the consent or agreement of the higher layer (RAN2) for the high-level signaling procedure.

UE can report its repeated transmission capability or repetition request through Msg1 or Msg3. Even if the UE reports the number of repetitions, the required number of repeated transmissions according to the measurement result of the gNB may be different from this, and the necessary number of repeated transmissions may be delivered to the DCI of Msg4. When the UE informs the gNB that it needs repetition for PUCCH for Msg4 HARQ-ACK before receiving Msg4, some fields of DCI of Msg4 may be reused for this purpose. According to section 7-3 of TS 38.212, the DCI format for the given existing Msg4 is as follows (DCI 1_0 with CRC scrambled by TC-RNTI).

| Field (Item) | Bits |
| --- | --- |
| Identifier for DCI formats | 1 |
| Frequency domain resource assignment | Variable |
| Time domain resource assignment | 4 |
| VRB-to-PRB mapping | 1 |
| Modulation and coding scheme | 5 |
| New data indicator | 1 |
| Redundancy version | 2 |
| HARQ process number | 4 |
| Downlink assignment index | 2 |
| TPC command for scheduled PUCCH | 2 |
| PUCCH resource indicator | 3 |
| PDSCH-to-HARQ_feedback timing indicator | 3 |

Since the maximum repetition number is 4 times, 2 bits are required. As shown in the table above, in the field for DCI 0_0 with CRC scrambled by TC-RNTI, fields for MCS or HARQ process number or DAI may be useless. Also, HARQ-ACK transmission timing is predetermined [TS 38.213 Sec.8.4], PDSCH-to-HARQ_feedback timing indicator field can be reused. So, these fields can be reused. Defining new fields may be meaningless because there are relatively many fields that can be reused.

Observation 3: If gNB uses Msg4 to notice the UE to PUCCH for Msg4 HARQ-ACK repeated transmission, some of the fields in DCI 1_0 with CRC scrambled by TC-RNTI for existing Mag4 can be reused. Defining new fields may be meaningless because there are relatively many fields that can be reused.

Proposal 4: Among the field for DCI 1_0 with CRC scrambled by TC-RNTI, fields for MCS/HARQ process number/DAI/HARQ_feedback_timing can be reused.

There is a discussion to reuse some of the Field in DCI scheduling Msg3 PUSCH to use a mechanism similar to Msg3 repetitive transmission. According to section 7-3 of TS 38.212, the DCI format for the given existing Msg4 is as follows (DCI 0_0 with CRC scrambled by TC-RNTI).

| Field (Item) | Bits |
| --- | --- |
| Identifier for DCI formats | 1 |
| Frequency domain resource assignment | Variable |
| Time domain resource assignment | 4 |
| Frequency hopping flag | 1 |
| Modulation and coding scheme | 5 |
| New data indicator | 1 |
| Redundancy version | 2 |
| HARQ process number | 4 |
| TPC command for scheduled PUSCH | 2 |
| ChannelAccess-CPext | 2 |
| Padding bits | If required |
| UL/SUL indicator | 1 |

Among them, fields for MCS or HARQ process number can be reused or new field can be defined for dynamic indication of repetition factor which can cause impact on DCI design.

We think it is more advantageous to use the DCI scheduling Msg4 PDSCH. When using DCI scheduling Msg3 PUSCH, gNB only measures the user's preamble signal. However, when using Msg4's DCI, the two or more signals (for Msg3 repetition) sent by UE are measured, and more accurate results can be expected.

Since NTN channels have a long latency, it is likely that the channel measured at the time of the preamble transmission is different from the channel state at the time of transmitting the Msg4 HARQ. Therefore, there is a possibility that the required number of repeated transmissions for PUCCH for Msg4 HARQ-ACK may be incorrect when the number of repeated transmissions is indicated in Msg3 based on the signal measured by preamble only.

However, when gNB measures the Msg1 Preamble and Msg3 signals to indicate the number of repeated transmissions of Msg4 HARQ-ACK on DCI for scheduling Msg4 PDSCH, the two or more measurement results can be combined. In addition, since the result of signal measurement in a time closer to the transmission timing of Msg4 HARQ-ACK is reflected, much more accurate results can be expected.

Observation 4: If gNB uses DCI scheduling Msg3 PUSCH to notice the UE to PUCCH for Msg4 HARQ-ACK repeated transmission, some of the fields in DCI 0_0 with CRC scrambled by TC-RNTI for existing Mag4 can be reused or new field can be defined for dynamic indication of repetition factor which can cause impact on DCI design.

Proposal 5: Among the field for DCI 0_0 with CRC scrambled by TC-RNTI for Msg3 PUSCH scheduling, MCS or HARQ process number can be reused or new field can be defined for dynamic indication of repetition factor which can cause impact on DCI design.

Observation 5: Since NTN channels have a long latency, it is likely that the channel measured at the time of the preamble transmission is different from the channel state at the time of transmitting the Msg4 HARQ.

Observation 6: When using DCI scheduling the Msg4 PDSCH, the two or more signals sent by UE are measured, and more accurate results can be expected.

Proposal 6: It is more advantageous to use the DCI scheduling Msg4 PDSCH for dynamic indication of repetition factor from gNB.

2.2	DMRS bundling for PUSCH taking into account NTN-specifics

In RAN1#112, the feasibility on the NTN-specific PUSCH DMRS bundling was discussed in the point of 3 limits{timing error limit, frequency error limit, phase difference limit}, and it was concluded that, under the scenario (LEO-1200, elevation=30°, SCS=15 kHz) with no TA adjustment, the timing error limit (38.133 Table 7.1C.2-1) could be satisfied within at most 13 slots, and the frequency error limit (38.101-5 Section 6.4.1) could be satisfied within at most 32 slots. However, there is one left thing for checking the feasibility of the NTN-specific PUSCH DMRS bundling, which is the phase difference limit (38.101-1 Table 6.4.2.5-1).

In [3], the phase discontinuity caused by the TA variation was discussed, and it might be calculated by 1. In 1, dTAdt (TA variation) which is given in Figure 1 can be calculated by a function of (elevation, satellite altitude, UE velocity) as specified in R1-2103719, and LTDW,DMRS  is the time difference for measuring the phase discontinuity, and fs might be the farthest subcarrier frequency from the center subcarrier for the channel bandwidth (or the PUSCH transmission), and “delay drift” might be given by the TA variation, and “phase discontinuity” could be the maximum allowable phase difference for DMRS bundling as specified in 38.101-1 Table 6.4.2.5-1. By using 1, LTDW,DMRS could be a function of (elevation, satellite altitude, UE velocity, the farthest subcarrier frequency from the center subcarrier) as given in 2 where the maximum allowable phase difference for DMRS bundling might be considered as a constant (set to 30°) and LTDW,DMRS could be considered as the TDW length to guarantee DMRS bundling requirement in 38.101-1 Table 6.4.2.5-1. The TDW length for {LEO-600,LEO-1200} are given in Figure 2.

phase discontinuity °=delay drift*Δt*fs*360=dTAdt*LTDW,DMRS*ffarthest*360# SEQ Equation \* ARABIC 1

LTDW,DMRS ≤phase discontinuity °dTAdt*ffarthest*360=30dTAdt*NRB*12*15kHz2*360# SEQ Equation \* ARABIC 2

Figure 1: TA variation vs. elevation 
(Left) UE velocity=1200 km/h (Right) UE velocity=0 km/h

Figure 2: TDW length for DMRS bundling 
(Left) LEO-600 (Right) LEO-1200

By using 2 under the assumption (NRB=1), the TDW length for the DMRS bundling is given by 9.96 ms for {LEO-600, elevation=10°}, 11.3 ms for {LEO-600, elevation=30°}, and 12.7 ms for {LEO-1200, elevation=30°} respectively as presented in 3. In [4], companies used NRB from 1 to 6. Hence, the TDW length for the DMRS bundling might be less than 1.66~9.96 ms for {LEO-600, elevation=10°}, 1.89~11.33 ms for {LEO-600, elevation=30°}, and 2.13~12.79 ms for {LEO-1200, elevation=30°} respectively. On the other hands, under the assumption (NRB=25, which is the edge of the 5 MHz channel bandwidth), the TDW length for DMRS bundling might be less than 0.39 ms for {LEO-600, elevation=10°}, 0.45 ms for {LEO-600, elevation=30°}, and 0.51 ms for {LEO-1200, elevation=30°}. Consequently, if PUSCH is allocated at the edge of the 5 MHz channel bandwidth, the TDW length for the DMRS bundling might be less than 1 ms (1 slot for 15 kHz SCS), and the phase difference limit (38.101-1 Table 6.4.2.5-1) cannot be satisfied at the farthest subcarrier of the 5 MHz channel bandwidth (NRB=25).

LTDW,DMRS≤3093.0μssec*NRB*180kHz2*360=9.96 msNRB for LEO-600,α=10°,vUE=1200km/h3081.7μssec*NRB*180kHz2*360=11.33 msNRBfor LEO-600, α=30°,vUE=1200km/h3072.4μssec*NRB*180kHz2*360=12.79 msNRBfor LEO-1200, α=30°,vUE=1200km/h  # SEQ Equation \* ARABIC 3

On the other hands, assuming (UE velocity=0 km/h), dTAdt (TA variation) is 90.76 us/sec for {LEO-600, elevation=10°}, 79.81 us/sec for {LEO-600, elevation=30°}, and 70.51 us/sec for {LEO-1200, elevation=30°} respectively. In this case, the TDW length for DMRS bundling might be revised from 3 to 4. Comparing (UE velocity=0 km/h), dTAdt (TA variation) might be lower than that of (UE velocity=1200 km/h), and the TDW length for DMRS bundling might be higher than that of (UE velocity=1200 km/h).

LTDW,DMRS≤3090.76μssec*NRB*180kHz2*360=10.02 msNRB for LEO-600,α=10°,vUE=0km/h3079.81μssec*NRB*180kHz2*360=11.6 msNRBfor LEO-600, α=30°,vUE=0km/h3070.51μssec*NRB*180kHz2*360=13.1 msNRBfor LEO-1200, α=30°,vUE=0km/h  # SEQ Equation \* ARABIC 4

Consequently, the DMRS bundling/joint channel estimation (JCE) for NR NTN might be considered within the TDW length to guarantee the phase difference limit (38.101-1 Table 6.4.2.5-1), and the timing error limit (38.133 Table 7.1C.2-1), and the frequency error limit (38.101-5 Section 6.4.1). The TDW length for the phase difference limit (0.39~11.33 ms for LEO-600, 0.51~12.79 ms for LEO-1200) might be smaller than the TDW length for the timing error limit (11.55 ms for LEO-600, 13.03 ms for LEO-1200). Thus, the maximum TDW length might depend on LTDW,DMRS (the TDW length for the DMRS bundling), rather than LTDW,Te (the TDW length for the timing error limit). Considering the VoIP packet arrival time (20 ms), the timing error limit might have impact on the TDW length for the DMRS bundling because the PUSCH DMRS bundling over 20 slots might be not possible, on the other hands, the frequency error limit might have no impact on TDW length for the DMRS bundling. Thus, the maximum available TDW length might be determined as given in 5 depending on TA variation (which might depend on elevation angle, satellite altitude, UE velocity), Timing Error Limit (which might depend on SSB SCS, UL SCS), and the channel bandwidth or the PUSCH resource (which is depending on the farthest subcarrier frequency from the center subcarrier).

max TDW length≤min(10.15ms,9.96 msNRB)for LEO-600,α=10°min(11.55ms,11.33 msNRB)for LEO-600, α=30°min(13.03ms,12.79 msNRB)for LEO-1200, α=30°# SEQ Equation \* ARABIC 5

According to the above, because the NTN specific TA drift might have impact on the possible TDW length, the enhancement of nominal/actual TDW determination might be necessary in order to guarantee the RAN4 requirement and the same TDW understanding between gNB and UE. For the same TDW understanding, it might be most important to share TA compensation timing regardless of whether TA timing is indicated by gNB or determined by UE. However, the TA compensation timing and its period might be UE-specific rather than cell-specific because TA variation is a function of elevation and UE velocity. In that sense, even if TA compensation timing is indicated by gNB, the indication might be based on information reported by the UE. Thus, The NTN specific TA drift and/or its compensation might be the one of the events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions within the nominal TDW.

According to [4], most companies evaluated PUSCH VoIP with the 20 repetitions (20 ms), since the VoIP packet arrives at every 20 ms. During 20 repetitions, at least 1 TA adjustment might be necessary in order to guarantee the above 2 requirements because the TDW lengths might be less than 20 ms. Roughly, the TDW length might be half of 20 ms with 1 RB assumption. The bundling across all the 20 repetitions with DMRS bundling/JCE might be impossible and half of the bundling gain might be lost. Hence, the link budget gap to be enhanced might be degraded by more than 2~3 dB. According to [4], the average link budget gap to be enhanced for PUSCH VoIP might be around 2.64 dB. (if the best/worst are excluded, 2.45 dB). Thus, the average link budget gap for PUSCH VoIP might be more than 4.64~5.64 dB (if the best/worst are excluded, more than 4.45 dB~5.45 dB). In order to mitigate the link budget gap, the enhancement techniques including 2 transmit antenna, polarization type mismatch compensation might be considered in combination with the DMRS bundling/JCE.

Observation 7: The phase difference limit (38.101-1 Table 6.4.2.5-1) might be satisfied within the following value

- UE velocity=1200 km/h

  - LEO-600, elevation=10°: LTDW,DMRS≤9.96 msNRB= 0.39 ms (@ 25 RB)~9.96 ms (@ 1 RB)

  - LEO-600, elevation=30°: LTDW,DMRS≤11.33 msNRB= 0.45 ms (@ 25 RB)~11.33 ms (@ 1 RB))

  - LEO-1200, elevation=30°: LTDW,DMRS≤12.79 msNRB= 0.51ms (@ 25 RB)~12.79 ms (@ 1 RB)

- UE velocity=0 km/h

  - LEO-600, elevation=10°: LTDW,DMRS≤10.02 msNRB = 0.40 ms (@ 25 RB)~10.02 ms (@ 1 RB)

  - LEO-600, elevation=30°: LTDW,DMRS≤11.6 msNRB= 0.46 ms (@ 25 RB)~11.6 ms (@ 1 RB)

  - LEO-1200, elevation=30°: LTDW,DMRS≤13.1 msNRB= 0.52 ms (@ 25 RB)~13.1 ms (@ 1 RB)

Observation 8: The TDW length might be configured based on (all of the followings/the subset of the followings)

- TA variation (elevation, satellite altitude, UE velocity)

- Timing Error Limit (SSB SCS, UL SCS)

- Channel bandwidth or PUSCH resource (the farthest subcarrier freq.)

- Note: UE velocity might have less impact on TA variation than other 2 variables (elevation, satellite altitude)

Observation 9: The value of the TDW length might be configured

- LEO-600, elevation=10°: max TDW ≤min(10.15ms,9.96 msNRB)= 0 ms (@ 25 RB)~9 ms (@ 1 RB)

- LEO-600, elevation=30°: max TDW ≤min(11.55ms,11.33 msNRB)=0 ms (@ 25 RB)~11 ms (@ 1 RB)

- LEO-1200, elevation=30°: max TDW ≤min(13.03ms,12.79 msNRB)=0 ms (@ 25 RB)~12 ms (@ 1 RB)

Observation 10: The link budget gap for the PUSCH VoIP in RAN1#110-e might be degraded by more than 2~3 dB with 1 RB assumption because the TDW length might be less than 20 ms.

- The average gap with 20 repetition and DMRS bundling/JCE might be 2.64 dB

- The average gap with 10 repetition and DMRS bundling/JCE might be more than 4.64~5.64 dB.

Proposal 7: The DMRS bundling/the joint channel estimation for NR NTN might be considered within the TDW length to guarantee the followings.

- the phase difference limit (38.101-1 Table 6.4.2.5-1)

- the timing error limit (38.133 Table 7.1C.2-1)

- the frequency err limit (38.101-5 Section 6.4.1)

- Note: the phase difference limit (38.133 Table 6.4.2.5-1) cannot be satisfied over multiple slots, if the farthest subcarrier from the center is taken from the edge of the 5 MHz channel bandwidth.

- Note: the frequency error limit in 38.101-5 Section 6.4.1 might have no impact on the TDW length considering the VoIP packet arrival time (20 ms).

Proposal 8: The NTN-specific TDW length might be configured by sharing all/the subset among the followings between gNB and UE.

- TA variation (elevation, satellite altitude, UE velocity)

- Timing Error Limit (SSB SCS, UL SCS)

- Channel bandwidth or PUSCH resource (the farthest subcarrier freq. from the center)

Proposal 9: The enhancement of nominal/actual TDW determination is necessary to guarantee same TDW understanding between gNB and UE.

- The TA compensation timing might be shared between gNB and UE regardless of whether TA timing is indicated by gNB or determined by UE.

- The new event for TA compensation timing might be needed regardless of whether TA timing is indicated by gNB or determined by UE.

- Note: TA compensation timing and its period might be UE-specific rather than cell-specific.

- Note: Even if TA compensation timing is indicated by gNB, the indication might be based on information reported by the UE.

Proposal 10: To mitigate the link budget gap for the PUSCH VoIP, the following enhancement techniques might be considered in combination with the DMRS bundling/JCE.

- 2 transmit antenna

- polarization type mismatch compensation

3	Conclusion

Observation 1: The threshold for Msg3 repetition can be reused for PUCCH retransmission for Msg4 HARQ-ACK.

Observation 2: Request the number of PUCCH for Msg4 HARQ-ACK at UE side through Msg1 preamble or particular RO can be waste of resource and increase collision probability. Because UEs in NTN environment experience similar channel conditions and may need similar number of retransmissions, there is a possibility that a specific Preamble and RO may not be used.

Observation 3: If gNB uses Msg4 to notice the UE to PUCCH for Msg4 HARQ-ACK repeated transmission, some of the fields in DCI 1_0 with CRC scrambled by TC-RNTI for existing Mag4 can be reused. Defining new fields may be meaningless because there are relatively many fields that can be reused.

Observation 4: If gNB uses DCI scheduling Msg3 PUSCH to notice the UE to PUCCH for Msg4 HARQ-ACK repeated transmission, some of the fields in DCI 0_0 with CRC scrambled by TC-RNTI for existing Mag4 can be reused or new field can be defined for dynamic indication of repetition factor which can cause impact on DCI design.

Observation 5: Since NTN channels have a long latency, it is likely that the channel measured at the time of the preamble transmission is different from the channel state at the time of transmitting the Msg4 HARQ.

Observation 6: When using DCI scheduling the Msg4 PDSCH, the two or more signals sent by UE are measured, and more accurate results can be expected.

Observation 7: The phase difference limit (38.101-1 Table 6.4.2.5-1) might be satisfied within the following value

- UE velocity=1200 km/h

  - LEO-600, elevation=10°: LTDW,DMRS≤9.96 msNRB= 0.39 ms (@ 25 RB)~9.96 ms (@ 1 RB)

  - LEO-600, elevation=30°: LTDW,DMRS≤11.33 msNRB= 0.45 ms (@ 25 RB)~11.33 ms (@ 1 RB))

  - LEO-1200, elevation=30°: LTDW,DMRS≤12.79 msNRB= 0.51ms (@ 25 RB)~12.79 ms (@ 1 RB)

- UE velocity=0 km/h

  - LEO-600, elevation=10°: LTDW,DMRS≤10.02 msNRB = 0.40 ms (@ 25 RB)~10.02 ms (@ 1 RB)

  - LEO-600, elevation=30°: LTDW,DMRS≤11.6 msNRB= 0.46 ms (@ 25 RB)~11.6 ms (@ 1 RB)

  - LEO-1200, elevation=30°: LTDW,DMRS≤13.1 msNRB= 0.52 ms (@ 25 RB)~13.1 ms (@ 1 RB)

Observation 8: The TDW length might be configured based on (all of the followings/the subset of the followings)

- TA variation (elevation, satellite altitude, UE velocity)

- Timing Error Limit (SSB SCS, UL SCS)

- Channel bandwidth or PUSCH resource (the farthest subcarrier freq.)

- Note: UE velocity might have less impact on TA variation than other 2 variables (elevation, satellite altitude)

Observation 9: The value of the TDW length might be configured

- LEO-600, elevation=10°: max TDW ≤min(10.15ms,9.96 msNRB)= 0 ms (@ 25 RB)~9 ms (@ 1 RB)

- LEO-600, elevation=30°: max TDW ≤min(11.55ms,11.33 msNRB)=0 ms (@ 25 RB)~11 ms (@ 1 RB)

- LEO-1200, elevation=30°: max TDW ≤min(13.03ms,12.79 msNRB)=0 ms (@ 25 RB)~12 ms (@ 1 RB)

Observation 10: The link budget gap for the PUSCH VoIP in RAN1#110-e might be degraded by more than 2~3 dB with 1 RB assumption because the TDW length might be less than 20 ms.

- The average gap with 20 repetition and DMRS bundling/JCE might be 2.64 dB

- The average gap with 10 repetition and DMRS bundling/JCE might be more than 4.64~5.64 dB.

Proposal 1: If repetition request or capability report is indicated by PRACH preamble and/or occasion, existing mechanism of Msg3 repetition can be reused.

Proposal 2: In case of indication using PRACH preamble and/or occasion, a method for minimizing collision probability for more detailed fragmentation should be discussed.

Proposal 3: Capability reporting or repetition request using Msg3 requires the consent or agreement of the higher layer (RAN2) for the high-level signaling procedure.

Proposal 4: Among the field for DCI 1_0 with CRC scrambled by TC-RNTI, fields for MCS/HARQ process number/DAI/HARQ_feedback_timing can be reused.

Proposal 5: Among the field for DCI 0_0 with CRC scrambled by TC-RNTI for Msg3 PUSCH scheduling, MCS or HARQ process number can be reused or new field can be defined for dynamic indication of repetition factor which can cause impact on DCI design.

Proposal 6: It is more advantageous to use the DCI scheduling Msg4 PDSCH for dynamic indication of repetition factor from gNB.

Proposal 7: The DMRS bundling/the joint channel estimation for NR NTN might be considered within the TDW length to guarantee the followings.

- the phase difference limit (38.101-1 Table 6.4.2.5-1)

- the timing error limit (38.133 Table 7.1C.2-1)

- the frequency err limit (38.101-5 Section 6.4.1)

- Note: the phase difference limit (38.133 Table 6.4.2.5-1) cannot be satisfied over multiple slots, if the farthest subcarrier from the center is taken from the edge of the 5 MHz channel bandwidth.

- Note: the frequency error limit in 38.101-5 Section 6.4.1 might have no impact on the TDW length considering the VoIP packet arrival time (20 ms).

Proposal 8: The NTN-specific TDW length might be configured by sharing all/the subset among the followings between gNB and UE.

- TA variation (elevation, satellite altitude, UE velocity)

- Timing Error Limit (SSB SCS, UL SCS)

- Channel bandwidth or PUSCH resource (the farthest subcarrier freq. from the center)

Proposal 9: The enhancement of nominal/actual TDW determination is necessary to guarantee same TDW understanding between gNB and UE.

- The TA compensation timing might be shared between gNB and UE regardless of whether TA timing is indicated by gNB or determined by UE.

- The new event for TA compensation timing might be needed regardless of whether TA timing is indicated by gNB or determined by UE.

- Note: TA compensation timing and its period might be UE-specific rather than cell-specific.

- Note: Even if TA compensation timing is indicated by gNB, the indication might be based on information reported by the UE.

Proposal 10: To mitigate the link budget gap for the PUSCH VoIP, the following enhancement techniques might be considered in combination with the DMRS bundling/JCE.

- 2 transmit antenna

- polarization type mismatch compensation

4	Reference

- R1-2302230_ Summary EOM on 9.11.1 Coverage enhancement for NR NTN

- Chair’s notes RAN1#112 9.11 R18 NTN v04

- R1-2106289, Summary #4 of AI 8.15.2 Enhancements to time and frequency synchronization, RAN1#105-e

- R1-2208269, Summary #6 on 9.12.1 Coverage enhancement for NR NTN, RAN1#110-e