3GPP TSG RAN WG1 #112bis-e			      R1-2303205

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.9.2

Source: 	ETRI

Title: 	Discussion on Network verified UE location for NR NTN

Document for:	Discussion/Decision

Introduction

In Rel-18, the enhancement of New Radio (NR) Non-Terrestrial Network (NTN) began with the examination of the requirement for network-verified User Equipment (UE) location [1-2]. In this contribution, we discuss the network-verified UE location for NR NTN.

Discussion

In this section, we discuss the potential enhancements for network verified UE location based on the RAN1 discussion in the last meeting.

Definition of UE Rx-Tx time difference

In NR, a UE reports UE Rx-Tx time difference as in TS38.215, and also a UE reports its TA value to the gNB. Similar to the Rx-Tx time difference, the TA value the UE reports can be used to calculate the distance between the satellite and the UE as shown in Figure 1.

Figure 1. UE reports TA value

The current matter concerns the definition of UE Rx-Tx time difference in NTN. It is worth noting that 3GPP had already defined UE Tx-Rx time difference for TN in Rel-17, which could be considered as a possible option. Alternatively, a new measurement and report can be defined. Additionally, the TA values that have been defined for NTN can be utilized to derive UE Tx-Rx time difference.

In RAN1#112, the following agreement for UE Rx-Tx time difference were made.

Agreement Select one (or more) of the following options for enhancing UE Rx-Tx time difference in NTN - Option 1: The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: - UE Rx-Tx time difference is defined with respect to the Rx and Tx subframe timing associated with the TRP. For a Transmission Point - TUE-RX is the UE received timing of downlink subframe #i from this Transmission Point (TP), defined by the first detected path in time. - TUE-TX is the UE transmit timing of the uplink subframe corresponding to subframe #i received from the TP - One or multiple DL RS for positioning, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP. FFS: For a Transmission Point different from the serving cell (e.g. a DL-PRS-only TP) - Option 2: - For RTT measurement in NTN, support UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS. - FFS: details of report and the definition of UE Rx-Tx time difference - Option 3: The legacy R17 definition of UE Rx-Tx time difference is adopted for NTN with an offset that is determined based on one of the following options: - Option 3-1: This offset is reported as the nearest integer value in the unit of milliseconds by rounding the time difference of transmit timing of uplink subframe #i and receive timing of downlink subframe#i - Option 3-2: UE report the index of the subframe j that is closest in time to the subframe #i received from the TP and LMF can derive the offset - Option 3-3: TA report which corresponds to the time difference of received timing of downlink subframe #i and transmit timing of uplink subframe#i rounding up to slot granularity. - Option 4: - UE Rx – Tx time difference TUE-RX – TUE-TX can be directly derived from timing advance TTA - FFS: the granularity and the reporting range of TA. - Note: This implies that the existing framework for Multi-RTT positioning report can be used without need to specify a new TA report. Note: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

Option 1 is based on the existing definition of UE Tx-Rx time difference for TN. This can also be used for NTN as the form of Option 1, as the starting point. Among the options above, Option 2 proposes using a new UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS. This approach can potentially provide a more accurate measurement of UE Rx-Tx time difference in NTN, as it uses specific signals designed for positioning purposes. Option 2 also suggests that the details of the report and the definition of UE Rx-Tx time difference can be further discussed. This allows for more flexibility and customization in the final specification of this enhancement, and enables the possibility of addressing any potential issues or limitations that may arise during the discussion. Compared to the other options, Option 2 offers a more targeted and precise approach to UE positioning in NTN while Option 1 can provide much less spec impact.

Proposal 1. Support Option 1 and/or Option 2 for enhancing UE Rx-Tx time difference in NTN, i.e., support reusing the current UE Tx-Rx time difference or UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS.

Definition of gNB Rx-Tx time difference

At this stage, the discussion on defining gNB Rx-Tx time difference for NTN has led to four options below, which were agreed in the last meeting.

Select one (or more) of the following options for the enhancement of gNB Rx-Tx time difference in NTN - Option 1: The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: For a Transmission Point - TgNB-RX is the Transmission and Reception Point (TRP) received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. - TgNB-TX is the TRP transmit timing of the downlink subframe corresponding to uplink subframe #i received from the UE - Multiple SRS resources can be used to determine the start of one subframe containing SRS. FFS: For a Transmission Point different from the serving cell (e.g. a DL-PRS-only TP) - Option 2: - For RTT measurement in NTN, support gNB report of gNB Rx-Tx time as defined in 38.215 with the following change: - Only the SRS resource starting within a subframe can be used to determine the start of the subframe. - Option 3: - Keep the current gNB Rx-Tx definition, and report an offset which can covers the time duration corresponds to kmac if needed. - Option 4: - For RTT measurement in NTN, support gNB report that indicates the time difference between the transmit time of a DL RS for positioning and the arrival time of an SRS. - FFS: details of report. Note: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

Similar to UE Tx-Rx time difference, Option 1 can be a starting point for NTN. Among other options, we believe that Option 4 provides a clear and straightforward approach for gNB Rx-Tx time difference measurement similar to Option 2 of UE Rx-Tx time difference. Option 2 can be further discussed, especially whether the modification on the SRS resource is essential.

Proposal 2. Support Option 1(or Option 2) and/or Option 4 for enhancing gNB Rx-Tx time difference in NTN.

Reference Point of gNB Rx-Tx time difference

The reference point for measuring the gNB Rx-Tx time difference in TN is well defined in TS 38.215. However, in the case of NTN, the reference point for gNB Rx-Tx time difference measurement is yet to be discussed. The options for the reference point include onboard the satellite, the uplink time synchronization reference point, and on the gNB as agreed in the last meeting.

Agreement In NTN, for the position of the reference point for definition of gNB Rx – Tx time difference measurement, consider the following options: - Option 1: Onboard the satellite - Option 2: The uplink time synchronization reference point - Option 3: on the gNB

In Option 1, the reference point for measuring gNB Rx-Tx time difference is located onboard the satellite. This means that the satellite itself is responsible for measuring the time difference between the reception and transmission of signals from and to the gNB. In Option 2, the reference point is the point where the gNB receives the timing information from the UE to synchronize its own timing. This option would align with the current TN definition and would allow for consistent measurement of gNB Rx-Tx time difference. In Option 3, the gNB is responsible for measuring the time difference between the reception and transmission of signals from and to the satellite. RAN1 needs further discussion with and after the discussion of the definition of gNB Rx-Tx time difference. Based on which information is reported to LMF, each option can be considered to use. Option 3 can be a starting point similar to the case of TN, but Option 1 can be straightforward for positioning from a given satellite.

Proposal 3. Use Option 1 or Option 3 for the reference point of gNB Rx-Tx time difference, i.e., either onboard the satellite or the gNB.

Information from gNB/UE to LMF

The transfer of the value of 𝑘mac used by the gNB and the value of TACommon is necessary for multi-RTT positioning with a single satellite in NTN. This information is needed by the LMF to calculate the UE location accurately based on the time difference between the UE and the satellite. The serving satellite ID and the cell/beam reference point are required for the LMF to identify the serving satellite and cell/beam that are communicating with the UE. The ephemeris data in PVT state vector format or Keplerian format along with the associated epoch time are necessary for the LMF to determine the position and trajectory of the satellite accurately at the time of measurement. The UE self-estimated TA to pre-compensate for service link delay may be needed by the LMF when UE Rx-Tx time difference is directly derived from timing advance. This information is required for accurate positioning of the UE in NTN.

The transfer of satellite positions at which the PRS should be measured and the aperiodic SRS should be activated from the UE to the LMF is necessary for accurate positioning in NTN. The PRS and aperiodic SRS are used in the multi-RTT positioning method, where the RTT between the UE and satellite is calculated based on the time difference between the transmission and reception of these signals. To accurately calculate the RTT, it is important to know the precise location of the satellite at the time of signal transmission.

Proposal 4. RAN1 further discusses which information is needed to transfer from gNB/UE to LMF.

Multiple RTT Transmissions and Mirror-Image Ambiguity

Multiple RTT measurements from a single satellite can be used to estimate the potential UE location with its mirror image. One possible enhancement is for the gNB to configure that the UE reports the RTT measurements (e.g., Rx-Tx time difference, TA value) periodically. The intersection points of circles or ellipses depending on the definition of the RTT measurements could be the UE location by using the periodically reported RTT measurements. In order to resolve the mirror image ambiguity, the following agreement was made in the last meeting.

Agreement Study the following options to resolve the mirror positions ambiguity for multi-RTT positioning: - Option 1: gNB or LMF implementation to solve the mirror error issue. - FFS: whether there is spec impact - Option 2: Reuse existing ECID method (e.g. combine UE neighbor measurements to solve the ambiguity between mirror positions), with potential enhancements - Option 3: NR NTN UE should report the Doppler calculated on the service link - Option 4: a VSAT UE should report its beam pointing in respect to satellite beam line of sight - Option 5: Reporting of cell coverage information (e.g. cell footprint and reference point, or antenna pattern) to the LMF - Option 6: Support and potentially enhance the optional Rel-17 UL-AoA measurements defined for multi-RTT positioning Other solutions are not precluded

By using the multi-RTT configuration by gNB/LMF, the mirror image ambiguity can be resolved, i.e., Option 1. Of course, there could be some specification impacts, e.g., to configure the reference location of satellite and period for reporting. This option requires no additional specification enhancement, which may save time and resources. The gNB implementation can be tailored to specific network conditions and may provide a more efficient solution.

Option 2 leverages an existing method that has already been standardized, which may save time and resources. The ECID method has been used in other technologies and has been proven to be effective in resolving positioning ambiguities.

In Option 5, the cell coverage information can provide valuable context for resolving mirror point ambiguities and may lead to more accurate positioning results.

Proposal 5. Support Options 1, 2, and 5 above, i.e., gNB or LMF implementation to solve the mirror error issue with some spec impact, ECID method, and reporting cell coverage to LMF.

Figure 2. Periodic UE reporting of RTT when the RTT is obtained based on the past starting time

Multiple RTT measurements from a single satellite can be used to estimate the potential UE location with its mirror image. One possible enhancement is for the gNB to configure that the UE reports the RTT measurements (e.g., UE Rx-Tx time difference) periodically. The intersection points of circles or ellipses depending on the definition of the RTT measurements could be the UE location by using the periodically reported RTT measurements. Figure 2 shows the UE candidate location when the RTT is defined by using UE Rx-Tx time difference between the pre-defined DL slot and the UL slot. For the periodic reporting of UE Rx-Tx time difference, the issue of determining the triggering time of multi-RTT positioning can be treated by network/gNB/LMF implementation.

Proposal 6. RAN1 can consider periodic reporting of RTT, i.e., UE Rx-Tx time difference. In the case of determining the triggering time of multi-RTT positioning, it can be treated by network/gNB/LMF implementation.

Multi-RTT measurement with prediction

In the above, the multi-RTT measurements can be obtained by using satellite’s actual movement. This means when the satellite is moving, the satellite locations can be considered as the locations of different TRPs. But the satellite might not need to actually locate in the reference locations of the satellite for the multiple RTT, which means that it is possible for the UE to just assume that the satellite is located in those multiple locations. In NTN, a UE is configured/broadcasted with satellite ephemeris so that the UE is aware of the satellite location in a specific time. Then, the UE can report the multi-RTT measurement with pre-defined satellite locations on the ephemeris. This can lead to reduction of delay for the reporting of multi-RTT.

Figure 3. UE reports multi-RTT from the predicted satellite locations

In Figure 3, the locations of (b) and (c) are the predicted locations of the satellite on the ephemeris. Since the UE is already know the ephemeris, the UE can predict the RTT measurement according to (b) and (c) and can report them to satellite via (a).

Proposal 7. RAN1 considers multi-RTT measurement reporting with predicted satellite locations on the ephemeris.

Multi-RTT measurement with pre-defined measurements

In the above, the multi-RTT measurements with predicted satellite locations on the ephemeris is used. This method can be generalized to the multi-RTT measurement with arbitrarily pre-defined locations, which are not only for the pre-defined locations on a single line.

Figure 4. UE reports multi-RTT with arbitrarily pre-defined locations

As shown in Figure 4, the locations of (b) and (c) are the pre-defined/pre-configured locations. Note that the locations of the satellite, (b), and (c) are not in a single line. Note also that, three locations can be pre-defined. Then, the UE calculates the RTT based on the locations and report the multi-RTT according to (b) and (c) to satellite via (a). In this method, the network can estimate the UE location without the mirror-image ambiguity. The other advantage of this method is that, this can be applied to GEO satellite case.

Proposal 8. In order to avoid mirror-image ambiguity, RAN1 considers multi-RTT measurement reporting with pre-configured locations.

Conclusions

In this contribution, we discussed the multi RTT for UE location verification. Our proposals are summarized below.

Proposal 1. Support Option 2 for enhancing UE Rx-Tx time difference in NTN, i.e., support UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS

Proposal 2. Support Option 2 for enhancing gNB Rx-Tx time difference in NTN, i.e., support gNB report of gNB Rx-Tx time as defined in 38.215 with a modification.

Proposal 3. RAN1 needs further discussion with and after the discussion of the definition of gNB Rx-Tx time difference.

Proposal 4. RAN1 further discusses which information is needed to transfer from gNB/UE to LMF.

Proposal 5. Support gNB or LMF implementation to solve the mirror error issue with some spec impact.

Proposal 6. RAN1 considers periodic reporting of RTT, i.e., UE Rx-Tx time difference. In the case of determining the triggering time of multi-RTT positioning, it can be treated by network/gNB/LMF implementation.

Proposal 7. RAN1 considers multi-RTT measurement reporting with predicted satellite locations on the ephemeris.

Proposal 8. In order to avoid mirror-image ambiguity, RAN1 considers multi-RTT measurement reporting with pre-configured locations.

References

- RP-221820, “New SID: Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR”, Thales, 3GPP RAN#96.

- 3GPP TR 38.882 V18.0.0, “Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR,” June 2022.

- RP-230809, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales, 3GPP RAN#99, Mar., 2023.

- R1-2302223, “FL Summary #5: Network verified UE location for NR NTN”, Thales, 3GPP RAN1#112, Feb., 2023.