3GPP TSG RAN WG1 #112bis-e			                    	R1-2303209

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.12.1

Source: 		Quectel

Title: 	Discussion on PRACH coverage enhancements

Document for:	Discussion and decision

Introduction

In RAN1 #112 meeting, the following working assumptions and agreements were achieved regarding PRACH coverage enhancement [1].

Agreement For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions. - If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported. - FFS: details Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs. - Note: Separate RO means that the RO is separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Conclusion For multiple PRACH transmissions within one RACH attempt, they are only transmitted over ROs associated with the same SSB/CSI-RS. Note: This applies for multiple PRACH transmissions with same Tx beam, and also applies for multiple PRACH transmissions with different Tx beam (if supported). Agreement For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power ramping is not applied within one RACH attempt. Agreement For multiple PRACH transmissions with same Tx beam, only one RAR window is supported for RAR monitoring for one RACH attempt. - FFS: the start position of the RAR window. - FFS: RA-RNTI. Agreement For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions. Note 1: All ROs in one RO group is associated with the same SSB(s). Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. Note 3: whether/how to define “RO group” in specification will be discussed separately Note 4: Valid RO(s) refers to what is defined in existing specification FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH. FFS: the time span of RO group. FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions. FFS: other details Agreement Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx beams.

In this contribution, we focus on discussing the remaining issues of multiple PRACH transmissions with same beam in the following aspects.

Multiple PRACH transmissions with same beam

Differentiation between different number of PRACH transmissions

In RAN1#112 meeting, it was agreed that gNB can configure one or multiple values for the number of multiple PRACH transmissions with same Tx beam and if multiple values are configured, PRACH resource differentiation between multiple PRACH transmissions with different number of PRACHs is supported. To differentiate PRACH resources for multiple PRACH transmissions with different number of PRACHs, the following two options can be considered:

- Option 1: Multiple PRACH transmissions with different number of PRACHs can be differentiated by using separate preambles on partially shared ROs.

- Option 2: Multiple PRACH transmissions with different number of PRACHs can be differentiated by using separate ROs.

- If multiple values for the number of multiple PRACH transmissions with same Tx beam is configured, it is assumed that each SSB index mapped to different number of ROs of multiple PRACH transmissions. By Option 1, it is preferred to associate one SSB index with the number of shared RO(s) as much as possible, which facilitates the gNB using the same receiving beam to detect the preambles in the corresponding RO(s). While considering that the number of available preambles in one RO is limited, Option 2 is also necessary as a complement.

Proposal 1:

- To provide sufficient PRACH resources, it is suggested to support using both separate preambles on partially shared ROs and separate ROs to differentiate multiple PRACH transmissions with different number of PRACHs.

Starting RB of ROs at different time instances

RO group (ROG) is assumed for multiple PRACH transmissions with same Tx beam to specify a group/set of valid ROs, which is associated with the same SSB(s). It was agreed in RAN1#110b-e that at least ROs located at different time instances can be used for multiple PRACH transmissions with same beam. Therefore, ROG with multiple ROs located at different time instances is an essential configuration. When the parameter msg1-FDM is greater than one, multiple ROs can be configured at one time instance which are frequency multiplexed. Hence such configuration provides an opportunity for UE to achieve additional frequency diversity gain by varying starting RB of ROs at different time instances within the ROG, i.e., frequency hopping within ROG. However, the gain that the frequency hopping within ROG can achieve depends on the channel conditions and the frequency hopping pattern. When the channel condition is frequency flat, the frequency hopping gain is very limited. Therefore, to approach the maximum frequency diversity gain, we proposed that the starting RB of ROs at different time instances should be selected to maximize the frequency span of the ROG.

- For example, as illustrated in Figure 1, if Msg1-FDM = 8 ROs in frequency domain,

- for one ROG (e.g., ROG1) consisting of 2 valid ROs located at different time instances, the frequency hopping distance (Drog1) between the starting RBs of two contiguous ROs in time domain is determined by 8/2 = 4 ROs so as to guarantee the minimum frequency span of ROG1 up to 5 ROs;

- for one ROG (e.g., ROG2) consisting of 4 valid ROs located at different time instances, the frequency hopping distance (Drog2) is determined by 8/4 = 2 ROs so as to guarantee the minimum frequency span of ROG2 up to 7 ROs.

- for one ROG (e.g., ROG3) consisting of 8 valid ROs located at different time instances, the frequency hopping distance (Drog3) is determined by 8/8 = 1 RO so as to guarantee the minimum frequency span of ROG3 up to 8 ROs.

- This frequency hopping pattern enables the channels of different ROs of one ROG varying significantly in frequency domain so as to achieve maximum frequency diversity gain.

Figure 1: Different starting RB of ROs at different time instances

Proposal 2:

- Different RB of ROs at different time instances should be supported to provide an opportunity for frequency diversity. To approach the maximum frequency diversity gain, the frequency hopping distance should be selected to maximize the frequency span of one ROG.

SSB-to-RO group Association

Since the ROG is introduced to specify the group of valid ROs for multiple PRACH transmissions and all ROs in one ROG is associated with the same SSB(s). Therefore the legacy SSB-to-RO association method could be reused for SSB-to-ROG association where the RO is replaced by ROG to reduce the spec efforts. That is, for a specific value of number of multiple PRACH transmissions, the SSB (SS/PBCH block) indexes provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon are mapped to ROGs for multiple PRACH transmissions in the following order,

- First, in increasing order of preamble indexes within a ROG

- Second, in increasing order of frequency resource indexed for frequency multiplexed ROGs

- Third, in increasing order of time resource indexed for time multiplexed ROGs within a PRACH slot group

- Fourth, in increasing order of indexes for PRACH slot groups

- As one ROG consisting of multiple valid ROs located at different time instances may cover several PRACH slots, “PRACH slot group” is assumed to specify a group of PRACH slots corresponding to one ROG.

Proposal 3:

- PRACH slot group is assumed for multiple PRACH transmissions over a ROG, where the PRACH slot group consisting of PRACH slot(s) corresponding to the ROG . The legacy SSB-to-RO association method is reused for SSB-to-ROG association except that the RO is replaced by ROG.

RAR window and RA-RNTI calculation

It was agreed that only one RAR window is supported for RAR monitoring for one RACH attempt of multiple PRACH transmissions with same beam. For the starting position of the RAR window, mainly two options have been discussed in last meeting,

- Option 1: the starting position of RAR window is after the last symbol of the last RO of the corresponding ROG

- Option 2: the starting position of RAR window is after the last symbol of the first RO of the corresponding ROG

- Option 1 can achieve fully combined detection of multiple PRACH transmissions with same beam. However, a large number of multiple PRACH transmissions may span a long time covering multiple PRACH slots considering that some ROs in some PRACH slots are invalid. With Option 1, the monitoring of RAR can only start after a long time, which incurs a significant access delay. Option 2 can enable early termination of multiple PRACH transmissions in one RACH attempt, i.e., the UE can drop the unnecessary transmissions in advance, which can save the UE transmit power, reduce the collision in the ROs without unnecessary transmissions, and also avoid unnecessary access delay. Correspondingly, the RA-RNTI of multiple PRACH transmissions can be calculated based on the first RO of the corresponding ROG.

Proposal 4:

- Support the starting position of RAR window is after the last symbol of the first RO of the corresponding ROG .

Proposal 5:

- Support the RA-RNTI of multiple PRACH transmissions is calculated based on the first RO of the corresponding ROG .

Conclusion

In this contribution, we discussed the remaining issues of multiple PRACH transmissions with same beam, and our proposals are summarized below:

Proposal 1:

- To provide sufficient PRACH resources, it is suggested to support using both separate preambles on partially shared ROs and separate ROs to differentiate multiple PRACH transmissions with different number of PRACHs.

Proposal 2:

- Different RB of ROs at different time instances should be supported to provide opportunities of frequency diversity. To approach the maximum frequency diversity gain, the frequency hopping distance should be selected to maximize the frequency span of one ROG.

Proposal 3:

- PRACH slot group is assumed for multiple PRACH transmissions over a ROG, where the PRACH slot group consisting of PRACH slot(s) corresponding to the ROG . The legacy SSB-to-RO association method is reused for SSB-to-ROG association except that the RO is replaced by ROG.

Proposal 4:

- Support the starting position of RAR window is after the last symbol of the first RO of the corresponding ROG .

Proposal 5:

- Support the RA-RNTI of multiple PRACH transmissions is calculated based on the first RO of the corresponding ROG .

Reference

- RAN1 Chair’s notes RAN1#112 eom2, RAN1#112, Athens, Greece, Feb. 27th - Mar. 3rd, 2023.