3GPP TSG RAN WG1 #112bis-e	R1-2303211

# e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on collision handling between PRACH and NCD-SSB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For TDD system, to realize common SSB to PRACH association between UEs in a BWP without any SSB and UEs in a BWP with only NCD-SSB, a RedCap UE in a BWP operating in a DL BWP without CD-SSB but with NCD-SSB should apply CD-SSB for determining PRACH occasion validation (in Clause 8.1), and PUSCH occasion validation (in Clause 8.1A). However, collision of NCD-SSB and valid RO, valid PUSCH occasion may happen for such RedCap UEs. For collision between valid RO and NCD-SSB, clarification is needed for common understanding between gNB and RedCap UE. Since UEs without NCD-SSB configuration cannot realized the overlap between NCD-SSB and RO, the ROs in the overlapped symbols will be treated as valid RO, so RedCap UEs operating on such BWP configured with NCD-SSB should not receive NCD-SSB, so as to reduce impact to legacy UEs. For overlapping between valid PUSCH occasion and NCD-SSB, the collision handling in section 11.1 that CD-SSB is prioritized than PUSCH can be reused, with description in 17.1 that collision handling of NCD-SSB is the same as CD-SSB unless otherwise stated. So no modification is needed for this part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that when there is overlap between NCD-SSB and valid RO/symbols before the valid RO in unpaired spectrum, UE does not receive NCD-SSB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The behavior of UE is not clear when the set of symbols to transmit NCD-SSB is overlapped with valid PRACH occasions or symbols before the valid RO. If different UEs applies different collision handlings, UE reception of SSB may be interfered by PRACH transmission of legacy UEs or RedCap UEs which choose to sending PRACH on overlapped symbols. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Detailed discussion on this CR can be seen in R1-2303210. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

  - 11.1	Slot configuration

========================= Unchanged parts =========================

For a set of symbols of a slot corresponding to a valid PRACH occasion and  symbols before the valid PRACH occasion, as described in clause 8.1, the UE does not receive PDCCH, PDSCH, CSI-RS or NCD-SSB in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

========================= Unchanged parts =========================