3GPP TSG RAN WG1 #112bis-e						 R1-2303218

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.2

Title:	Discussion on CSI enhancement for high/medium UE velocities and CJT

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN#94e, the Rel-18 WID for NR MIMO evolution for downlink and uplink was approved [1], in which two potential enhancements are for CSI enhancement for high/medium UE velocities and coherent JT (CJT) as shown below:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we will share our views for the CSI enhancements for high/medium UE velocities and coherent JT scheme respectively.

- Type-II codebook refinement for CJT

  - FD basis selection scheme for CJT codebook structure mode-1

In RAN1#112 meeting [2], we have the following agreement on the FD basis selection scheme for CJT codebook structure mode-1:

Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FD basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

The main target scenario for CJT codebook mode-1 is inter-cell CJT, where there is big difference in the delay profile of the DL channels of different TRPs. So, it is more reasonable to adopt Alt 2 which facilitate independent FD basis selection for different TRPs, no matter small or large ISD scenarios. Therefore, our first preference on mode-1 codebook FD basis selection is Alt 2. We are also open for Alt 1 if enough SLS results sources do show good UPT vs PMI overhead trade-off under sufficiently diverse configurations.

Proposal 1: On the Type-II codebook refinement for CJT mTRP, for the FD basis selection scheme for mode-1, our first preference is Alt2:

- Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset)

  - Parameter Combination

In RAN1#112 meeting [2], we have the following agreement on the supported parameter combinations for CJT codebook:

Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, regarding the list of supported combinations of {Ln}, only support the following additional combinations: NTRP; {Ln} combination. 2; {4,2}. 3; {2,4,2}, {4,2,2}. No other permutations are supported. FFS: For NTRP>1, in addition to the supported combinations/permutations, whether to support at least one additional combination where at least one of the Ln values (n=1, …, NTRP) is 6

Regarding the supported combinations of {Ln} for SD basis number, for NTRP > 1, in addition to the supported combinations/permutations, we think at least one parameter combination where at least one of the Ln values (n=1, …, NTRP) is 6.

Considering it has been agreed that UE could perform and report TRP selection, it is possible that UE reports CSI with N =1 even when NTRP > 1 is configured by gNB. But based on current supported {Ln} combinations, although Ln=6 is supported when NTRP = 1, once NTRP > 1, Ln = 6 cannot be supported if single-TRP is selected and reported by UE. Thus, we support additional {Ln} combination with Ln = 6 for one of the Ln values, and this {Ln} combination can be selected by UE only when UE reports single TRP selection, i.e., N =1. Besides, considering UE SD basis searching complexity, Ln = 6 can be limited with NTRP < 4. One possible combination example for {Ln} can be {2,6} when NTRP = 2, {2, 2, 6} when NTRP = 3.

Proposal 2: On the Type-II codebook refinement for CJT mTRP, when NTRP > 1, support at least one parameter combination where at least one of the Ln values (n=1, …, NTRP) is 6.

  - UCI omission rule

In RAN1#112 meeting [2], we have the following agreement on the UCI omission rule for CJT codebook:

Agreement On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, down-select between the following three alternatives (by RAN1#112-bis where n denotes the n-th CSI-RS resource): - Alt1. Prio(,l,m,n)=( k=0n-12Lk) .N.RI.P(m)+N.RI.l(n)+N.n - Note: This implies that CSI-RS resource is designated the highest priority - Alt2. Prio(,l,m,n)=2L’.Qn).RI.N3+2L’.RI. P(m)+RI.l(n)+ - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis) - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI. - Alt3. Replace SD basis index l in legacy Prio calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+ FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m

For Alt 1 and Alt 3, UE can be severed by all cooperation TRPs after CSI omission. However, the performance of CJT transmission may be significantly degraded and can be even worser than Single-TRP transmission, since there are not too much non-zero coefficients for each TRP once CSI omission happens.

In Alt 2, when CSI omission happens, only the information of some TRP will be dropped firstly, it is just like passive fallback to single TRP transmission or CJT transmission with a smaller number of TRPs after UE’s dynamic TRP selection. And then based on the reported CSI and the system condition, gNB could choose single TRP transmission or CJT transmission with a smaller number of TRPs after receiving such omitted CSI and these omitted TRP(s) can be used for other UE’s transmission.

Proposal 3: On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, support Alt2:

- Alt2. Prio(,l,m,n)=2L’.Qn).RI.N3+2L’.RI. P(m)+RI.l(n)+

  - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis)

  - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources

  - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI.

- Type-II codebook refinement for high/medium UE velocities

  - Bitmap design for indicating the locations of the NZCs

In RAN1#112 meeting [2], we have the following agreement for indicating the locations of the NZCs for high/medium UE velocities:

Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives (no later than RAN1#112bis-e): - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L – s1-s0+ mint1-t0, Q – t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively. Nokia/NSB, Samsung, vivo, and ZTE raised concerns that, in their understanding, Alt3A violates previous agreements for “Q different two-dimensional bitmaps” and/or common DD basis selection across SD/FD basis pairs and hence, to some extent, objective 1 of the WID.

Among the above several solutions on NZCs indication bitmap design, Alt 1 is the simple extension of legacy design. The basic design principle of Alt 3A is assuming that all the NZCs locations indicated by the second bitmap can only be confined with the selected S basis vectors pairs. The gain of Alt 3A scheme much rely on the configurations of L, M, Q, S. The gain of it will be shown if the value of S is rather smaller than MQ. In another words, the selected S pairs of FD basis vector and DD basis vector should be sparse enough in the MQ basis vector pairs so that the gain can be demonstrated. But now we only agreed that the value of Q can be 2 without additional candidate values, so we think the gain of Alt 3A might be a little marginal in this case. Besides, the legacy CSI omission rules may also need reconsidering for Alt 3A, since the priorities of the first and the second bitmap might be different as the first bitmap can be considered to contain more important information on the NZCs locations indication.

Based on the above analysis, we prefer Alt 1 as the simple and straight forward extension of legacy design for NZCs locations indication.

Proposal 4: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, support Alt1:

- Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv

  - UCI omission rule

In RAN1#112 meeting [2], we have the following agreement on the UCI omission rule for high/medium UE velocities:

Agreement On the Type-II codebook refinement for high/medium velocities, regarding UCI omission, down-select between the following three alternatives (by RAN1#112bis-e where q denotes the q-th DD basis vector): - Alt1. Prio(,l,m,q)=2L. Q.RI.P(m)+Q.RI.l+Q.q - Note: This implies that DD basis is designated the highest priority - Alt2. Prio(,l,m,q)=2L.S(q).RI.N3+2L.RI. P(m)+RI.l+ - Note: This implies that DD basis is designated the lower priority (after FD basis) - FFS: S(q) maps the index q according to a rule - Alt3. Prio(,l,m,q)=2L.RI.Mv.q + 2L.RI.P(m)+ RI.l +  - Note: This implies that DD basis is designated the least priority - Alt4. Prio(,l,m,q)=2L.P(m).RI.Q+2L.RI.S(q)+RI.l+ - Note: This implies that DD basis is designated with lower priority (after SD basis) and higher priority (before FD basis) - FFS: S(q) maps the index q according to a rule FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m q=0,…,Q-1

Considering the maximum value of Q can only be 2, if one of the 2 DD basis related information is dropped, the performance of CSI codebook for high/medium velocities might suffer great degradation. Thus, we prefer put DD basis with the highest priority, i.e., Alt1.

Proposal 5: For the Type-II codebook refinement for high/medium velocities, regarding UCI omission, support Alt1:

- Alt1. Prio(,l,m,q)=2L. Q.RI.P(m)+Q.RI.l+Q.+q

  - Note: This implies that DD basis is designated the highest priority

  - CQI design for high/medium velocities

In RAN1#112 meeting [2], we have the following agreements on CQI reporting for high/medium UE velocities:

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrix - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix - FFS: Whether/how to include CQI overhead reduction for X=2

For X=2, 3 CQI format alternatives for the 2nd time-domain (TD) CQI have been proposed in RAN1#112:

- Alt1. Independent of the 1st TD CQI: A 4-bit wideband CQI and 2-bit sub-band CQIs

- Alt2. Differential reference CQI relative to the 1st TD CQI: A BR-bit wideband CQI and 2-bit sub-band CQIs

- Alt3. Differential reference and sub-band CQIs relative to the 1st TD CQI: A BR-bit wideband CQI and 1-bit sub-band CQIs

Where:

- 0≤BR<4 (0 implies that the reference CQI for the 2nd TD CQI is the reference CQI for the 1st TD CQI)

During the offline discussion pre-RAN1#112bis-e, there was also a proposal on this issue:

Proposal 2.F.2: For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI includes 4-bit wideband CQI and 2-bit sub-bands CQIs calculated independently from the 1st CQI

For Alt 2 and Alt 3, we think the overhead saving brought is very small and usually the CQI overhead contributes very small in the final CSI overhead for Type-II codebook family. Therefore, we prefer Alt1 for simplicity and support Offline Proposal 2.F.2.

Proposal 6: Support Offline Proposal 2.F.2:

For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI includes 4-bit wideband CQI and 2-bit sub-bands CQIs calculated independently from the 1st CQI

In addition, the location of the 2nd TD CQI in a UCI block was also discussed and there was 3 Alts in last meeting:

- Alt1. In UCI part 1

- Alt2. In UCI part 2

- Alt3. Wideband CQI in UCI part 1, sub-band CQIs in UCI part 2

During the offline discussion pre-RAN1#112bis-e, there was also a proposal on this issue:

Proposal 2.F.1: For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI is located in UCI part 2

In legacy CSI construction rule, if we have 2 CQIs (Type-I and rank > 4) in one CSI report, the 2nd CQI will be located in UCI part 2. So, we think reusing legacy design for the location of the 2nd TD CQI is enough for codebook for high/medium velocities, i.e., Alt2 and support Offline Proposal 2.F.1.

Proposal 7: Support Offline Proposal 2.F.1:

For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI is located in UCI part 2

- Conclusion

Proposal 1: On the Type-II codebook refinement for CJT mTRP, for the FD basis selection scheme for mode-1, our first preference is Alt2:

- Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset)

Proposal 2: On the Type-II codebook refinement for CJT mTRP, when NTRP > 1, support at least one parameter combination where at least one of the Ln values (n=1, …, NTRP) is 6.

Proposal 3: On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, support Alt2:

- Alt2. Prio(,l,m,n)=2L’.Qn).RI.N3+2L’.RI. P(m)+RI.l(n)+

  - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis)

  - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources

  - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI.

Proposal 4: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, support Alt1:

- Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv

Proposal 5: For the Type-II codebook refinement for high/medium velocities, regarding UCI omission, support Alt1:

- Alt1. Prio(,l,m,q)=2L. Q.RI.P(m)+Q.RI.l+Q.+q

  - Note: This implies that DD basis is designated the highest priority

Proposal 6: Support Offline Proposal 2.F.2:

For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI includes 4-bit wideband CQI and 2-bit sub-bands CQIs calculated independently from the 1st CQI

Proposal 7: Support Offline Proposal 2.F.1:

For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI is located in UCI part 2

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94-e, e-Meeting, December 6 - 17, 2021.

- 3GPP RAN1#112, RAN1 Chairman’s Notes, Athens, Greece, February 2023.