3GPP TSG RAN WG1 #112bis-e			    R1-2303219

e-Meeting, April 17th – April 26th, 2023

Source:	CMCC

Title:	Discussion on increased number of orthogonal DMRS ports

Agenda Item:	9.1.3.1

Document for:	Discussion and Decision

Introduction

In this contribution, we share our views on increasing number of orthogonal DMRS ports and DMRS enhancement for 8 TX UL transmission.

Larger number of DMRS ports for MU-MIMO

Rel-18 DMRS ports indication for PDSCH

Working Assumption@RAN1#112 For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs, - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0. Table 4-0: DMRS ports for 2CWs. Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8. 1; 2; 0,1,2,3,8,10. 2; 2; 0,1,2,3,8,9,10. 3; 2; 0,1,2,3,8,9,10,11.

The antenna ports indication for eType1 DMRS with maxLength=1 for PDSCH of S-TRP case has been agreed in RAN1#111 and RAN1#112 meeting [1-2], bus some DMRS port combinations are still controversial. For port combinations [8-10], [8-11] and [8,10] / [9,11], we think they are redundant to legacy port combinations [0-2], [0-3] and [0,2], and can be removed. For two codewords cases, the port combinations in the table of working assumption of RAN1#112 can be confirmed.

Proposal 1: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, the following table can be supported.

Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes |
| 0 | 1 | 0 | Cat. 1 | 0 | 2 | 0,1,2,3,8 | Rank 5-8 with one DMRS symbol |
| 1 | 1 | 1 |  | 1 | 2 | 0,1,2,3,8,10 |  |
| 2 | 1 | 0,1 |  | 2 | 2 | 0,1,2,3,8,9,10 |  |
| 3 | 2 | 0 |  | 3 | 2 | 0,1,2,3,8,9,10,11 |  |
| 4 | 2 | 1 |  |  |  |  |  |
| 5 | 2 | 2 |  |  |  |  |  |
| 6 | 2 | 3 |  |  |  |  |  |
| 7 | 2 | 0,1 |  |  |  |  |  |
| 8 | 2 | 2,3 |  |  |  |  |  |
| 9 | 2 | 0-2 |  |  |  |  |  |
| 10 | 2 | 0-3 |  |  |  |  |  |
| 11 | 2 | 0,2 |  |  |  |  |  |
| 12 | 1 | 8 | Cat.2 |  |  |  |  |
| 13 | 1 | 9 |  |  |  |  |  |
| 14 | 1 | 8,9 |  |  |  |  |  |
| 15 | 2 | 8 |  |  |  |  |  |
| 16 | 2 | 9 |  |  |  |  |  |
| 17 | 2 | 10 |  |  |  |  |  |
| 18 | 2 | 11 |  |  |  |  |  |
| 19 | 2 | 8,9 |  |  |  |  |  |
| 20 | 2 | 10,11 |  |  |  |  |  |
| 21 | [2] | [8-10] |  |  |  |  |  |
| 22 | [2] | [8-11] |  |  |  |  |  |
| 23 | [2] | [8, 10], [9, 11] |  |  |  |  |  |
| 24 | 1 | 0,1,8 | Cat.3 |  |  |  |  |
| 25 | 1 | 0,1,8,9 |  |  |  |  |  |
| 26 | 2 | 0,1,8 |  |  |  |  |  |
| 27 | 2 | 0,1,8,9 |  |  |  |  |  |
| 28 | 2 | 2,3,10 |  |  |  |  |  |
| 29 | 2 | 2,3,10,11 |  |  |  |  |  |

For the size of antenna ports field, three alternatives have been discussed in last meeting with Alt 0: The DCI size of DMRS port indication is the same as Rel.17, Alt.1: The DCI size of DMRS port indication is increased by M (M>=1) bit from Rel.17, Alt.3: The DCI size of DMRS port indication is increased by M (M>=0) bit, depending on RRC configuration. Based on the discussion in section 2.1, the codepoints increase with more DMRS ports, and it is natural to increase the DCI size. To consider the overhead, some DMRS port combinations with lower probability to be used can be removed. Besides, the DCI level dynamic indication of DMRS ports is benefit for network scheduling and flexible MU-MIMO pairing.

Proposal 2: For Rel.18 eType1/eType2 DMRS ports with maxLength=1/2 for PDSCH, the DCI size of DMRS port indication is increased by M (M>=1) bit from Rel.17.

Rel-18 DMRS ports indication for PUSCH

Agreement@RAN1#112 For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-8-X, Table 7.3.1.1.2-9-X, Table 7.3.1.1.2-10-X, and Table 7.3.1.1.2-11-X are supported. - FFS: Whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not. Table 7.3.1.1.2-8-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=1, rank = 1 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0. 1; 1; 1. 2; 2; 0. 3; 2; 1. 4; 2; 2. 5; 2; 3. 6; 1; 8. 7; 1; 9. 8; 2; 8. 9; 2; 9. 10; 2; 10. 11; 2; 11. 12-15; Reserved; Reserved. Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0,1. 1; 2; 0,1. 2; 2; 2,3. 3; 2; 0,2. 4; 1; 8,9. 5; 2; 8,9. 6; 2; 10,11. [7]; [2]; [8,10]. 8-15; Reserved; Reserved. Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-2. [1]; [2]; [8-10]. 2; 1; 0,1,8. 3; 2; 0,1,8. 4; 2; 2,3,10. 5-15; Reserved; Reserved. Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-3. [1]; [2]; [8-11]. 2; 1; 0,1,8,9. 3; 2; 0,1,8,9. 4; 2; 2,3,10,11. 5-15; Reserved; Reserved.

The antenna ports indication for eType1 DMRS with maxLength=1 for PUSCH of S-TRP case has been agreed in RAN1#112 meeting [2], bus some DMRS port combinations are still controversial. For port combinations [8,10], [8-10] and [8-11], we think they are redundant to legacy port combinations [0,2], [0-2] and [0-3] and can be removed.

Proposal 3: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, the port combinations [8,10], [8-10] and [8-11] with brackets can be removed.

DMRS enhancement for 8 TX UL transmission

Antenna port indication table for >4 layers PUSCH

Agreement@RAN1#111 - For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS. - For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following: - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook. - For Rel.18 eType1/eType2 DMRS ports, - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details). - Note: Whether the DMRS port combination allows to use single symbol DMRS for rank = 5,6,7,8 should be checked. - FFS: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group. - FFS: One or more than one DMRS port combination(s) for each rank and TPMI - Note: New DMRS port combinations above does not preclude the new antenna ports tables including the current DMRS port combination(s) for PDSCH for rank = 5,6,7,8 in Rel.15-17. - FFS: Whether the antenna ports combinations for rank = 5,6,7,8 can be indicated by the reserved entries of existing antenna ports tables for rank =1,2,3,4, if the rank is indicated together with DMRS antenna ports.

In RAN1#111 meeting [1], antenna ports tables based on Type 1/Type 2 Rel.15 DMRS ports has been discussed, it has been agreed that the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook, while the DMRS port combination(s) for partial coherent UL codebook is still FFS. In Rel-15, for dmrs-Type=1, maxLength=2, and rank=3 UL transmission, the DMRS port combination {0,1,2} can be indicated to UE, where DMRS ports 0 and 1 are within one CDM group, and DMRS port 2 is in another CDM group. The TPMI for three-layer transmission using four antenna ports (partial coherent) can be  or . It is shown that the second and third layers associated to the same antenna port group are multiplexed into different DMRS CDM group, and none of restriction is added to DMRS port combination(s) for partial coherent UL codebook. From our perspective, the restriction of DMRS port combination(s) for partial coherent UL codebook is also not needed for 8 TX UL transmission as well, and a unified design of DMRS port combination(s) for rank = 5,6,7,8 is used for full, partial, and non-coherent UL codebook.

Proposal 4: Support the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused for partial coherent UL codebook, and layers associated to the same antenna port group can be multiplexed into same or different DMRS CDM group.

In RAN1#111 meeting [1], the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 has been agreed, while some DMRS port combinations are still with brackets for further discussion. From our perspective, the DMRS port combinations in brackets for rank = 5,6,7,8 for PDSCH can be supported, and can be reused for full, partial, and non-coherent UL codebook.

Proposal 5: For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, reuse the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports at least for full or partial or non-coherent UL codebook.

Working Assumption@RAN1#112 To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables. - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table). - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussedin AI9.1.4.2 [or AI9.1.3.1].

In RAN1#112 meeting [2], separate or joint DMRS ports tables for rank 5,6,7,8 has been discussed, from our perspective, legacy separate scheme can be reused for rank 5,6,7,8.

In Rel-15, the available DMRS port combinations for DL are summarized in Table 1, which can be reused for UL. It is noted that for dmrs-Type=1and dmrs-Type=2, maxLength=1, only one DMRS ports combination is available for each rank. In other words, if the rank derived from TPMI is larger than 4, there is no need to additionally indicate the antenna ports. Then, the bits for antenna ports indication can be reused for other purpose, such as MCS, RV and NDI for second codeword. For dmrs-Type=2, maxLength=2, since two DMRS ports combinations are available for rank 5 and rank 6, one bit is needed for antenna ports indication, and the other bits can be reused for other purpose.

Table 1. DMRS port combinations for rank = 5,6,7,8 for PDSCH in Rel-15

| Rank | dmrs-Type=1, maxLength=2 | dmrs-Type=2, maxLength=1 | dmrs-Type=2, maxLength=2 |
| --- | --- | --- | --- |
| 5 layers | 0-4 | 0-4 | 0-4 |
|  |  |  | 0,1,2,3,6 |
| 6 layers | 0,1,2,3,4,6 | 0-5 | 0-5 |
|  |  |  | 0,1,2,3,6,8 |
| 7 layers | 0,1,2,3,4,5,6 |  | 0,1,2,3,6,7,8 |
| 8 layers | 0,1,2,3,4,5,6,7 |  | 0,1,2,3,6,7,8,9 |

Proposal 6: Confirm the working assumption of separate DMRS ports tables for rank 5,6,7,8 for PUSCH.

Proposal 7: For larger than 4 layers UL transmission with Type 1/Type 2 Rel.15 DMRS ports, the reserved bits for antenna ports indication can be reused for other purpose, such as MCS, RV and NDI for second codeword.

  - PTRS-DMRS association

In Rel-15, if a UE has reported the capability of supporting full-coherent UL transmission, the UE shall expect the number of UL PT-RS ports to be configured as one. For partial-coherent and non-coherent codebook based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI and/or number of layers as: PUSCH antenna port 1000 and 1002 in indicated TPMI share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI share PT-RS port 1. For codebook or non-codebook UL transmission scheduled by DCI format 0_1, the association between UL PT-RS port(s) and DM-RS port(s) is signalled by PTRS-DMRS association field in DCI format 0_1. In Rel-17, for M-TRP PUSCH repetition, when maxRank=2, the number of bits for the indication of PTRS-DMRS association is the same as Rel-15/16, MSB and LSB separately indicating the association between PT-RS port and DMRS port for two TRPs. When maxRank>2, a second PTRS-DMRS association field (similar to the existing field) is added in DCI format 0_1, and each field separately indicating the association between PT-RS port and DMRS port for two TRPs.

To support up to 8 layers UL transmission, PTRS-DMRS association should be enhanced to indicate the PT-RS port association with up to 8 DMRS ports. For partial-coherent and non-coherent PUSCH with rank 5-8, if four antenna groups are assumed for UE’s antenna layouts, where each group comprises coherent antennas, and across groups, antennas are non-coherent, the maximal PT-RS ports to be configured is 4. For codebook-based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI and/or number of layers as: PUSCH antenna port 1000 and 1004 in indicated TPMI share PT-RS port 0, and PUSCH antenna port 1001 and 1005 in indicated TPMI share PT-RS port 1, PUSCH antenna port 1002 and 1006 in indicated TPMI share PT-RS port 2, and PUSCH antenna port 1003 and 1007 in indicated TPMI share PT-RS port 3. As shown in Table 2, 4 bits of PTRS-DMRS association field is needed to indicate the association between PT-RS port and DMRS port pair. If two antenna groups are assumed for UE’s antenna layouts, the maximal PT-RS ports to be configured is 2, and the actual number of UL PT-RS port(s) is determined based on TPMI and/or number of layers as: PUSCH antenna port 1000, 1002, 1004 and 1006 in indicated TPMI share PT-RS port 0, and PUSCH antenna port 1001, 1003, 1005 and 1007 in indicated TPMI share PT-RS port 1. As shown in Table 3, 4 bits of PTRS-DMRS association field is needed to indicate the association between PT-RS port and DMRS port pair. For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant.

Table 2: PTRS-DMRS association for UL PTRS ports 0, 1, 2 and 3

| Value of lowest bit | DMRS port |  | Value of the third bit | DMRS port |
| --- | --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 |  | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 |  | 1 | 2nd DMRS port which shares PTRS port 1 |
| Value of second | DMRS port |  | Value of first bit | DMRS port |
| 0 | 1st DMRS port which shares PTRS port 0 |  | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 |  | 1 | 2nd DMRS port which shares PTRS port 1 |

Table 3: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port |  | Value of LSB | DMRS port |
| --- | --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 |  | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 |  | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 |  | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 |  | 3 | 4th DMRS port which shares PTRS port 1 |

For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0, for PUSCH corresponding to a configured grant Type 1 transmission, Rel-15 procedure can be reused without further enhancement.

Proposal 8: For partial-coherent and non-coherent UE, if four antenna groups are assumed for UE’s antenna layouts, the maximal PT-RS ports to be configured is 4, and if two antenna groups are assumed, the maximal PT-RS ports to be configured is 2. 4 bits of PTRS-DMRS association field is needed to indicate the association between PT-RS port and DMRS port pair.

Conclusion

In this contribution, we share our views on increasing number of orthogonal DMRS ports. The proposals are summarised as follows:

Proposal 1: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, the following table can be supported.

Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes |
| 0 | 1 | 0 | Cat. 1 | 0 | 2 | 0,1,2,3,8 | Rank 5-8 with one DMRS symbol |
| 1 | 1 | 1 |  | 1 | 2 | 0,1,2,3,8,10 |  |
| 2 | 1 | 0,1 |  | 2 | 2 | 0,1,2,3,8,9,10 |  |
| 3 | 2 | 0 |  | 3 | 2 | 0,1,2,3,8,9,10,11 |  |
| 4 | 2 | 1 |  |  |  |  |  |
| 5 | 2 | 2 |  |  |  |  |  |
| 6 | 2 | 3 |  |  |  |  |  |
| 7 | 2 | 0,1 |  |  |  |  |  |
| 8 | 2 | 2,3 |  |  |  |  |  |
| 9 | 2 | 0-2 |  |  |  |  |  |
| 10 | 2 | 0-3 |  |  |  |  |  |
| 11 | 2 | 0,2 |  |  |  |  |  |
| 12 | 1 | 8 | Cat.2 |  |  |  |  |
| 13 | 1 | 9 |  |  |  |  |  |
| 14 | 1 | 8,9 |  |  |  |  |  |
| 15 | 2 | 8 |  |  |  |  |  |
| 16 | 2 | 9 |  |  |  |  |  |
| 17 | 2 | 10 |  |  |  |  |  |
| 18 | 2 | 11 |  |  |  |  |  |
| 19 | 2 | 8,9 |  |  |  |  |  |
| 20 | 2 | 10,11 |  |  |  |  |  |
| 21 | [2] | [8-10] |  |  |  |  |  |
| 22 | [2] | [8-11] |  |  |  |  |  |
| 23 | [2] | [8, 10], [9, 11] |  |  |  |  |  |
| 24 | 1 | 0,1,8 | Cat.3 |  |  |  |  |
| 25 | 1 | 0,1,8,9 |  |  |  |  |  |
| 26 | 2 | 0,1,8 |  |  |  |  |  |
| 27 | 2 | 0,1,8,9 |  |  |  |  |  |
| 28 | 2 | 2,3,10 |  |  |  |  |  |
| 29 | 2 | 2,3,10,11 |  |  |  |  |  |

Proposal 2: For Rel.18 eType1/eType2 DMRS ports with maxLength=1/2 for PDSCH, the DCI size of DMRS port indication is increased by M (M>=1) bit from Rel.17.

Proposal 3: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, the port combinations [8,10], [8-10] and [8-11] with brackets can be removed.

Proposal 4: Support the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused for partial coherent UL codebook, and layers associated to the same antenna port group can be multiplexed into same or different DMRS CDM group.

Proposal 5: For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, reuse the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports at least for full or partial or non-coherent UL codebook.

Proposal 6: Confirm the working assumption of separate DMRS ports tables for rank 5,6,7,8 for PUSCH.

Proposal 7: For larger than 4 layers UL transmission with Type 1/Type 2 Rel.15 DMRS ports, the reserved bits for antenna ports indication can be reused for other purpose, such as MCS, RV and NDI for second codeword.

Proposal 8: For partial-coherent and non-coherent UE, if four antenna groups are assumed for UE’s antenna layouts, the maximal PT-RS ports to be configured is 4, and if two antenna groups are assumed, the maximal PT-RS ports to be configured is 2. 4 bits of PTRS-DMRS association field is needed to indicate the association between PT-RS port and DMRS port pair.

References

- 3GPP RAN1#111, RAN1 Chairman’s Notes, November 2022.

- 3GPP RAN1#112, RAN1 Chairman’s Notes, February 2023.