3GPP TSG RAN WG1 #112bis-e						 R1-2303220

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.3.2

Title:	Discussion on SRS enhancement targeting TDD CJT and 8 TX operation

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN#94e, the Rel-18 WID for NR MIMO evolution for downlink and uplink was approved [1], in which the potential enhancement is for SRS enhancement for TDD coherent JT (CJT) scenario and 8 TX operation as shown below:

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32 - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, we will discuss SRS enhancement for enabling 8 TX UL transmission and TDD coherent JT (CJT) scenario.

- SRS enhancement for enabling 8 Tx UL transmission

To enable 8 Tx UL transmission, up to 8 ports SRS design and SRS antenna switching for 8 Tx UE should be enhanced in Rel-18.

  - Detailed designs to support 8 Tx SRS

In Rel-15, when the number of SRS ports is less than 4, different cyclic shifts are used for different antenna ports. To consider the orthogonality between different antenna ports, when the number of SRS ports is equal to 4 and the number of transmission comb is equal to 2, if cyclicShift-n2 is larger than 4, in addition to different cyclic shifts are used for different antenna ports, SRS antenna ports {1002, 1004} and SRS antenna ports {1001, 1003} occupy different REs with comb transmission, both cyclic shift and comb-2 are used for orthogonal antenna ports, otherwise, only different cyclic shifts are used for orthogonal antenna ports. Similarly, when the number of SRS ports is equal to 4 and the number of transmission comb is equal to 4, if cyclicShift-n4 is larger than 6, both cyclic shift and comb-4 are used for orthogonal antenna ports, otherwise, only different cyclic shifts are used for orthogonal antenna ports. In Rel-17, the number of transmission comb equals to 8 is added for SRS, when the number of SRS ports is equal to 4 and the number of transmission comb is equal to 8, both cyclic shift and comb-8 are used for orthogonal antenna ports. The comb offset and cyclic shift allocation for 2 ports and 4 ports SRS in Rel-15/17 are summarized in Table 1.

Table 1. Comb offset and cyclic shift allocation for 2 ports and 4 ports SRS in Rel-15/17

| SRS | Comb | Comb offset | Cyclic shift | Allocation scheme |
| --- | --- | --- | --- | --- |
| 2 ports SRS | 2 | 1 | nSRScs,max=8 | 2 cyclic shifts |
|  | 4 | 1 | nSRScs,max=12 | 2 cyclic shifts |
|  | 8 | 2 | nSRScs,max=6 | 2 comb offsets |
| 4 ports SRS | 2 | 1 | nSRScs,max=8 cyclicShift-n2<4 | 4 cyclic shifts |
|  | 2 | 2 | nSRScs,max=8 cyclicShift-n2>4 | 4 cyclic shifts and 2 comb offsets |
|  | 4 | 1 | nSRScs,max=12 cyclicShift-n4<6 | 4 cyclic shifts |
|  | 4 | 2 | nSRScs,max=12 cyclicShift-n4>6 | 4 cyclic shifts and 2 comb offsets |
|  | 8 | 2 | nSRScs,max=6 | 2 cyclic shifts and 2 comb offsets |

For 8 ports SRS transmission, based on legacy comb and cyclic shift design, the port mapping scheme of 8 ports SRS are summarized in Table 2.

- For comb 2, as shown in Fig.1 (a), when the number of transmission comb is equal to 2, if cyclicShift-n2 is smaller than 4, 1 comb offset is used, and 8 different cyclic shifts can be used for orthogonal antenna ports. As shown in Fig.1 (b), if cyclicShift-n2 is larger than 4, in addition to different cyclic shifts are used for different antenna ports, SRS antenna ports {1002, 1004, 1006, 1008} and SRS antenna ports {1001, 1003, 1005, 1007} occupy different REs with comb transmission, as legacy scheme, 8 different cyclic shifts and 2 comb offsets are used for orthogonal antenna ports.

- For comb 4, as shown in Fig.1 (c), as legacy scheme, SRS antenna ports {1002, 1004, 1006, 1008} and {1001, 1003, 1005, 1007} occupy different REs with comb transmission, 8 different cyclic shifts and 2 comb offsets are used for orthogonal antenna ports.

- For comb 8, 4 comb offsets has been supported. As shown in Fig.1 (d), SRS antenna ports {1002, 1004}, {1001, 1003}, {1006, 1008}, {1005, 1007} occupy different REs with comb transmission, 2 cyclic shifts and 4 comb offsets are used for orthogonal antenna ports.

Table 2. Comb offset and cyclic shift allocation for 8 ports SRS

| SRS | Comb | Comb offset | Cyclic shift | Allocation scheme | Fig |
| --- | --- | --- | --- | --- | --- |
| 8 ports SRS | 2 | 1 | nSRScs,max=8 cyclicShift-n2<4 | 8 cyclic shifts | Fig.1 (a) |
|  | 2 | 2 | nSRScs,max=8 cyclicShift-n2>4 | 8 cyclic shifts and 2 comb offsets | Fig.1 (b) |
|  | 4 | 2 | nSRScs,max=12 cyclicShift-n4<6 | 8 cyclic shifts and 2 comb offsets | Fig.1 (c) |
|  | 8 | 4 | nSRScs,max=6 | 2 cyclic shifts and 4 comb offsets | Fig.1 (d) |

(a) 8 CSs for comb 2, (b) 8 CSs and 2 comb offsets for comb 2, (c) 8 CSs and 2 comb offsets for comb 4, (d) 2 CSs and 4 comb offsets for comb 8

Figure 1. Port mapping of 8 ports SRS

For comb 2, if cyclicShift-n2 is smaller than 4, 1 comb offset is used, and 8 different cyclic shifts are used for 8 ports SRS, if cyclicShift-n2 is larger than 4, 8 different cyclic shifts and 2 comb offsets are used for 8 ports SRS.

For comb 4, 8 different cyclic shifts and 2 comb offsets are used for 8 ports SRS.

For comb 8, 2 cyclic shifts and 4 comb offsets are used for 8 ports SRS.

The mapping from the ports to comb offsets are determined as follows: if k=2, ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, {1001, 1003, 1005, 1007} on the second comb offset; if k=4, ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} are mapped on the third comb offset, and {1003, 1007} on the fourth comb offset.

Furthermore, the SRS antenna ports combination that located in the same RE also needs discussion. The antenna ports that located in the same RE are distinguished by cyclic shift, to increase the orthogonality between different antenna ports, antenna ports with largest cyclic shift distance should be allocated in the same RE, such as antenna ports 1001 and 1003 are located in the same RE in Rel-15. Based on this principle, when comb-4 and cyclic shift are configured, antenna ports 1001 and 1004 should be configured in the same RE. However, to consider backward compatibility, antenna ports 1001 and 1003 should be configured in the same RE that same as Rel-15 for 4 ports SRS. Hence, backward compatibility should be considered for RE mapping of 8 ports SRS.

Backward compatibility should be considered for RE mapping of 8 ports SRS.

  - Enhancement on 8 ports SRS in multiple OFDM symbol

In RAN1#112 meeting [2], details of 8 ports SRS resource transmitted in multiple OFDM symbols has been discussed.

Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. - At least s = 2. - FFS: s = 4, s = 8. - m = 2,4,8, 10,12,14, and m is a multiple of s. - Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol. - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports. - FFS: down selection from existing resource mapping designs - FFS: which subset of 8/s ports are mapped onto each OFDM symbol. - FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern: - Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. - Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

In RAN1#112 meeting [2], different SRS ports mapped onto 2 adjacent OFDM symbols has been supported. While, whether TDM-4 and TDM-8 can be supported for 8 ports SRS is FFS. TDM-8 needs adjacent 8 OFDM symbols, which increase the UL resource overhead. To consider the scheduling flexibility of network, TDM-4 could be also supported to transmit 2 SRS antenna ports on each symbol.

For TDM-2, one symbol is used to transmit SRS antenna ports {1001, 1002, 1003, 1004}, and another symbol is used to transmit SRS antenna ports {1005, 1006, 1007, 1008}, and the RE mapping of each symbol is same as legacy 4 ports SRS. Similarly, for TDM-4, four symbols are used to transmit SRS antenna ports {1001, 1002}, {1003, 1004}, {1005, 1006}, and {1007, 1008} respectively, and the RE mapping of each symbol is same as legacy 2 ports SRS.

Besides, in RAN1#112 meeting [2], the TDM pattern of cyclically mapping and sequentially mapping have been discussed. To consider the phase consistence, cyclically mapping is benefit for gNB to estimate each 8-port SRS with more precise channel estimation performance. To consider the high-speed scenario, cyclically mapping is also benefit for gNB to estimate the Doppler property.

Support TDM-4 for 8 ports SRS.

For TDM-2, ports {1001, 1002, 1003, 1004} and ports {1005, 1006, 1007, 1008} are transmitted in adjacent symbols, and the RE mapping of each symbol is same as legacy 4 ports SRS. For TDM-4, four symbols are used to transmit SRS antenna ports {1001, 1002}, {1003, 1004}, {1005, 1006}, and {1007, 1008} respectively, and the RE mapping of each symbol is same as legacy 2 ports SRS.

Support cyclically mapping of TDM pattern for 8-port SRS.

Additionally, to improve the reliability of SRS, repetition can be also supported with TDM transmission. For frequency hopping, 8 ports of SRS are firstly mapped onto nrofTDM adjacent OFDM symbols, then mapped onto different frequency band based on frequency hopping procedure, where nrofTDM = {1,2,4} is the number of OFDM symbols for TDM. The combinations of nrofSymbols and repetitionFactor are listed in Table 1. It can be seen that the repetitionFactor×nrofTDM should be less than or equal to nrofSymbols, and nrofSymbols should be divisible by repetitionFactor×nrofTDM, where nrofTDM is the number of OFDM symbols for TDM. For TDM-4, when nrofSymbols=12, repetitionFactor = 3 can be supported for three times of frequency hopping, which has not been supported in current spec.

Table 1. Combinations of nrofSymbols and repetitionFactor for 8 ports SRS with TDM-2, TDM-4 and TDM-8

| nrofTDM | nrofSymbols | repetitionFactor |
| --- | --- | --- |
| TDM-2 | 2 | 1 |
|  | 4 | 1,2 |
|  | 8 | 1,2,4 |
|  | 10 | 1,2,4,5 |
|  | 12 | 1,2,4,5,6 |
|  | 14 | 1,2,4,5,6,7 |
| TDM-4 | 4 | 1 |
|  | 8 | 1,2 |
|  | 12 | 1,2,3 |

Support repetition and frequency hopping based on TDM transmission, where repetitionFactor×nrofTDM should be less than or equal to nrofSymbols, and nrofSymbols should be divisible by repetitionFactor×nrofTDM.

Support repetitionFactor=3 for TDM-4 with nrofSymbols=12.

In Rel-15, SRS is transmitted in one symbol, and UE splits the transmit power equally across the configured antenna ports for SRS. For 8-port SRS transmitted in multiple symbols, if UE splits the transmit power across 8 antenna ports, then the benefit to use multiple SRS symbols to improve transmit power for cell edge UE is not available. If UE splits the transmit power equally across the configured antenna ports for each symbol, the transmit power for each antenna port may exceed the PA capability. For example, a UE of PCMAX=23dB with PA capability {14, 14, 14, 14, 14, 14, 14, 14}. For SRS transmitted in two symbols, if UE splits the transmit power across 4 antenna ports for each symbol, then UE may need to transmit 17 dB for each antenna port, which exceeds the PA capability of 14dB. So, firstly, PCMAX should be split across the symbols for SRS transmission, secondly, UE splits the transmit power equally across the configured antenna ports for each symbol.

For 8 ports SRS with TDM transmission, PCMAX is firstly split across the symbols for SRS transmission, then UE splits the transmit power equally across the configured antenna ports for each symbol.

  - SRS enhancement for antenna switching

In RAN1#110 meeting [2], SRS enhancement for 8T8R ‘antennaSwitching’ has been identified.

Agreement@RAN1#110 For 8 Tx SRS, at least support - 8 ports in 1 SRS resource for ‘antennaSwitching’; - FFS 8 ports in one or multiple SRS resources for ‘codebook’ Above does not imply support for 8 ports in one or multiple OFDM symbols Agreement@RAN1#110 For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol. - FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols. Agreement@RAN1#110 For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR).

Currently, UE may be configured SRS for antenna switching depending on the indicated UE capability supportedSRS-TxPortSwitch. For 8T8R UE, 8 ports SRS resource can be configured in SRS resource set for antenna switching. Besides, whether downgrading configuration of SRS for antenna switching will be considered for 8T8R UE can be studied. For example, whether the SRS configurations for 4T8R or 4T4R can be also configured for 8T8R UE to reduce the potential high overhead of SRS resources and facilitate high UE power efficiency.

Whether downgrading configuration of SRS for antenna switching will be considered for 8T8R UE can be discussed.

- SRS enhancement for TDD coherent JT (CJT)

  - Randomized code-domain resource mapping and/or comb offset hopping

In RAN1#112 meeting [5], we have the following agreement on the enhancement for CJT SRS interference randomization:

Agreement For SRS interference randomization, support: - Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping - FFS: Associated UE capability

For SRS interference randomization, in last meeting we have agreed that cyclic shift hopping and comb offset hopping for SRS will be supported in Rel-18.

As for whether/how co-existence of comb offset hopping and cyclic shift hopping is supported, we think it is much related to the time-domain granularity for these two hopping methods. If the time domain granularities for comb offset hopping and cyclic shift hopping are the same, then the SRS orthogonality may be difficult to guarantee. Meanwhile, if the granularities for comb offset hopping and cyclic shift hopping are different, then the SRS orthogonality within the smaller granularity can still be guaranteed. For example, if the SRS is configured with repetition and the cyclic shift hopping is performed per symbol within a slot, while the comb offset hopping is performed per transmission occasion /slot, then the SRS is still orthogonal within a slot/transmission occasion. Hence, we think whether the SRS is orthogonal or not is much related to the time-domain granularity for hopping if both comb offset hopping and cyclic shift hopping are configured.

- If both comb offset hopping and cyclic shift hopping are configured, whether the SRS is orthogonal or not is much related to the time-domain granularity for the two hopping-based methods.

However, when only one of cyclic shift hopping and comb offset hopping is applied for C-JT SRS, usually the interference SRS suffered could already been alleviated enough. So, for simplicity, cyclic shift hopping and comb offset hopping should be supported separately. Cyclic shift hopping and comb offset hopping can be configured by RRC signalling, but them cannot be configured simultaneously. Only if the benefit of combined CS hopping and comb offset hopping is justified validly, combined hopping can be supported.

For SRS interference randomization, cyclic shift hopping and comb offset hopping should be supported separately.

Cyclic shift hopping and comb offset hopping can be configured by RRC signalling.

Regarding the issue on multiplexing with legacy UEs if cyclic shift hopping and/or comb offset hopping are enabled, gNB could make UE configured with cyclic shift hopping multiplexed with legacy UE via FDM, i.e., these two kinds of UE occupy different combs. And the UE configured with comb offset hopping can be multiplexed with legacy UE via CDM, i.e., these two kinds of UE occupy different cyclic shifts.

UE configured with cyclic shift hopping could be multiplexed with legacy UE via FDM, i.e., these two kinds of UE occupy different combs.

UE configured with comb offset hopping could be multiplexed with legacy UE via CDM, i.e., these two kinds of UE occupy different cyclic shifts.

As for the hopping details of cyclic shift hopping and comb offset hopping, we have the following agreement in previous meeting [4]:

Agreement @RAN1#111 For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, - FFS: Hopping pattern - Support at least hopping based on slot index, OFDM symbol index - FFS: Use of symbol group based on repetition factor - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource - FFS: Re-initialization periodicity - Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching - FFS: Other types of SRS - FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively - FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one

In RAN1#111 meeting, it has been agreed that SRS comb offset hopping and/or cyclic shift hopping is applicable to periodic/semi-persistent SRS with usage antennaSwitching. As for aperiodic SRS, since the motivation of SRS interference randomization is to alleviate continuous SRS collision, there seems no motivation for aperiodic SRS.

SRS comb offset hopping and/or cyclic shift hopping is not needed for aperiodic SRS.

As for the hooping pattern, we have the following agreements in last meeting [5]:

Agreement For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options: - Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions. - Option 2: Within a slot, hopping based on only the symbol index l'. - Option 3: No intra-slot hopping. - FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf. - FFS: reinitialization periodicity of N radio frames or reinitialization based on system frame number. - FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) - FFS: At least support reinitialization at the beginning of each radio frame. Agreement For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs. - Option 1: Reuse the SRS sequence identity nIDSRS. - Option 2: Introduce new ID(s). - FFS: the value range, one new ID or two separate new IDs, default ID(s)

For periodic/semi-persistent SRS, at least cyclic shift hopping and comb offset hopping could be performed with the time granularity of transmission slot, i.e. the hopping pattern design could be based on slot index at least. Besides, hopping could also be performed with the time granularity of symbol additionally, if the SRS resource take up multiple symbols, i.e., repetition factor R is configured larger than 1. In another word, we support intra-slot hopping and Option 3 could be excluded.

Cyclic shift hopping and comb offset hopping could be performed with the time granularity of symbol.

Cyclic shift hopping and comb offset hopping could be performed with the time granularity of slot.

For the case of intra-slot hopping, in Option 1, the hooping granularity is the repetition factor R symbols; while in Option 2, the SRS resource is hopped per symbol level. Although limiting the hooping with a larger granularity could release UE implementation complexity during performing hopping, however, it is achieved with the cost of hopping randomness, which will affect the gain brought from CS/Comb offset hopping. Besides, when the repetition factor R is configured with the same value of SRS occupation OFDM symbols NsymbSRS, the UE couldn’t perform CS/Comb offset hopping within a slot, which is an extra restriction for gNB’s scheduling. Therefore, we suggest hopping is based on only the symbol index l', i.e., Option 2.

For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index  ns,fμ  within a radio frame and OFDM symbol index l', support Option 2:

- Option 2: Within a slot, hopping based on only the symbol index l'.

For the pseudo-random sequence c(i) used for determining the hopping pattern, we have discussed 2 Options in previous meeting:

- Option 1: Reuse the SRS sequence identity nIDSRS.

- Option 2: Introduce new ID(s).

In the legacy sequence hopping and group hopping, the pseudo-random sequence is initialized with SRS sequence identity nIDSRSat the beginning of each radio frame. For simplicity, we think we could just follow the legacy hopping related behaviour and reuse the SRS sequence identity nIDSRS to initialize the pseudo-random sequence for CS/Comb offset hopping.

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, about initializing the pseudo-random sequence c(i) used for hopping pattern, support Option 1:

- Option 1: Reuse the SRS sequence identity nIDSRS.

  - Per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs

In RAN1#110bis-e meeting [3], we have agreed to study two options for per-TRP power control and/or power control for one or multiple SRS transmission occasions towards multiple TRPs:

Agreement @RAN1#110bis-e For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set: - Option 1: - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha - Each transmission occasion of the SRS resource is towards multiple TRPs - Option 2: - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS - Different transmission occasions of the SRS resource can be towards different TRPs

For Option 2, an SRS resource set is divided to more than 1 subset of SRS resource and each subset is associated with 1 UL power control parameter set. However, for xTxR antenna switching, where only 1 SRS resource can be configured, there will be only one subset of SRS resource, so, whether/how Option 2 could work need further study.

- For xTxR antenna switching, enabling more than 1 power control processes each for a subset of SRS resource of an SRS resource set need further study.

In addition, for the second bullet in Option 2, different transmission occasions of the SRS resource can be towards different TRPs, it seems we need to design power control process hopping pattern based on the more than 1 power control parameter sets, and it should be noticed that the power control process hopping pattern is specific to CJT scenario where up to 4 TRPs will be deployed.

- For power control-based method for CJT SRS, in Option2, to facilitate different transmission occasions of the SRS resource towards different TRPs, power control process hopping pattern need be studied based on the more than 1 power control parameter sets.

Nevertheless, in Option 1, just the number of DL pathloss RS and/or the alpha associated with an SRS resource set will be enhanced to be more than one. It seems the spec impact of Option 1 is much clearer and much less.

And each transmission occasion of the SRS resource is towards to multiple TRPs in Option 1, the overhead of SRS for CJT is less than in Option 2. Therefore, we think Option 1 for CJT specific SRS power control could be supported.

For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, support Option 1:

- Option 1:

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- Conclusion

Based on the above discussions, the proposals are as follows:

SRS-8T

- For comb 2, if cyclicShift-n2 is smaller than 4, 1 comb offset is used, and 8 different cyclic shifts are used for 8 ports SRS, if cyclicShift-n2 is larger than 4, 8 different cyclic shifts and 2 comb offsets are used for 8 ports SRS.

For comb 4, 8 different cyclic shifts and 2 comb offsets are used for 8 ports SRS.

For comb 8, 2 cyclic shifts and 4 comb offsets are used for 8 ports SRS.

The mapping from the ports to comb offsets are determined as follows: if k=2, ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, {1001, 1003, 1005, 1007} on the second comb offset; if k=4, ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} are mapped on the third comb offset, and {1003, 1007} on the fourth comb offset.

Backward compatibility should be considered for RE mapping of 8 ports SRS.

Support TDM-4 for 8 ports SRS.

For TDM-2, ports {1001, 1002, 1003, 1004} and ports {1005, 1006, 1007, 1008} are transmitted in adjacent symbols, and the RE mapping of each symbol is same as legacy 4 ports SRS. For TDM-4, four symbols are used to transmit SRS antenna ports {1001, 1002}, {1003, 1004}, {1005, 1006}, and {1007, 1008} respectively, and the RE mapping of each symbol is same as legacy 2 ports SRS.

Support cyclically mapping of TDM pattern for 8-port SRS.

Support repetition and frequency hopping based on TDM transmission, where repetitionFactor×nrofTDM should be less than or equal to nrofSymbols, and nrofSymbols should be divisible by repetitionFactor×nrofTDM.

Support repetitionFactor=3 for TDM-4 with nrofSymbols=12.

For 8 ports SRS with TDM transmission, PCMAX is firstly split across the symbols for SRS transmission, then UE splits the transmit power equally across the configured antenna ports for each symbol.

Whether downgrading configuration of SRS for antenna switching will be considered for 8T8R UE can be discussed.

SRS-CJT

For SRS interference randomization, cyclic shift hopping and comb offset hopping should be supported separately.

Cyclic shift hopping and comb offset hopping can be configured by RRC signalling.

UE configured with cyclic shift hopping could be multiplexed with legacy UE via FDM, i.e., these two kinds of UE occupy different combs.

UE configured with comb offset hopping could be multiplexed with legacy UE via CDM, i.e., these two kinds of UE occupy different cyclic shifts.

SRS comb offset hopping and/or cyclic shift hopping is not needed for aperiodic SRS.

Cyclic shift hopping and comb offset hopping could be performed with the time granularity of symbol.

Cyclic shift hopping and comb offset hopping could be performed with the time granularity of slot.

For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index  ns,fμ  within a radio frame and OFDM symbol index l', support Option 2:

- Option 2: Within a slot, hopping based on only the symbol index l'.

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, about initializing the pseudo-random sequence c(i) used for hopping pattern, support Option 1:

- Option 1: Reuse the SRS sequence identity nIDSRS.

For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, support Option 1:

- Option 1:

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- If both comb offset hopping and cyclic shift hopping are configured, whether the SRS is orthogonal or not is much related to the time-domain granularity for the two hopping-based methods.

- For xTxR antenna switching, enabling more than 1 power control processes each for a subset of SRS resource of an SRS resource set need further study.

- For power control-based method for CJT SRS, in Option2, to facilitate different transmission occasions of the SRS resource towards different TRPs, power control process hopping pattern need be studied based on the more than 1 power control parameter sets.

- References

- 3GPP RAN#94e, RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, December 2021.

- 3GPP RAN1#110, RAN1 Chairman’s Notes, August 2022.

- 3GPP RAN1#110bis-e, RAN1 Chairman’s Notes, September 2022.

- 3GPP RAN1#111, RAN1 Chairman’s Notes, November 2022.

- 3GPP RAN1#112, RAN1 Chairman’s Notes, Athens, Greece, February 2023.