3GPP TSG RAN WG1 #112bis-e						 R1-2303223

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.2.1

Title:	Discussion on general aspects of AI/ML framework

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN1#112 meeting [1], the following agreements have been approved.

Agreement To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side. Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. Note: The Case definition is NOT intended to introduce sub-levels of Level z. Note: Other cases may be included further upon interest from companies. FFS: Z4 and Z5 boundary Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model Agreement - AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Agreement - For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature. Agreement For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.

In this contribution, we present our views on network-UE collaboration levels, model life cycle management, framework for AI/ML for air-interface, and evaluation methodology of AI/ML for air interface.

- Network-UE collaboration levels

In RAN1#110bis-e meeting [2], the definitions of Level-y and Level-z were further discussed. According to the definitions of model transfer and model delivery, model transfer means model delivery over the air interface. Hence, we think the working assumption can be confirmed.

Proposal 1: Confirm the working assumption on Level y-z boundary.

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: Other procedures than model transfer/delivery are decoupled with collaboration level y-z.

- Clarifying note: Level y includes cases without model delivery.

Based on the discussion in previous meetings, several model delivery methods can be considered, which can be categorized as follows.

- Opt1. Proprietary-format based model transfer

- Opt2. Open-format based model transfer

- Opt3. OTT/OAM based model delivery

Different UE may support different AI/ML frameworks. For Opt1, UE can first report the supported AI/ML model format to NW, e.g., via UE capability report, then NW delivers the AI/ML model to UE over the air interface, and the model structure and model parameters are represented by the format the UE supports. For Opt2, model structure and model parameters are represented by an open-format or standard format, and NW delivers the open-format model to UE over the air interface. Both Opt1 and Opt2 should be viewed as Level-z, and the exact signaling for model transfer (e.g, via RRC, NAS,…) can be discussed in RAN 2. For Opt3, the model delivery may be transparent to air-interface, but whether other signaling enhancement is needed for Opt3 can be discussed.

Proposal 2: Study the following options and potential spec impact of model delivery.

- Opt1. Proprietary-format based model transfer

- Opt2. Open-format based model transfer

- Opt3. OTT/OAM based model delivery

- Model life cycle management

Life cycle management (LCM) of AI/ML model is necessary for obtaining satisfactory performance by applying AI/ML model, and several components of LCM have been discussed in the last meeting.

  - Data collection

Data collection is needed for model training, model inference, and model monitoring. For different purposes, the requirements and mechanisms for data collection can be different. For model training, a large size and high accuracy dataset may be needed. The dataset delivery can be done based on 3GPP signaling or transparent to 3GPP. For one-sided model, if the data collection and model training can be performed at the same side, non-3GPP based data collection mechanism can be used. But if data collection and model training are performed at different sides, 3GPP based data collection can be used to deliver the data from one side to the other side. For two-sided model with Type 2 or Type 3 training, a common dataset should be used at the UE side or network side. For Type 2 training, the common dataset will be used for training the AI/ML model at the UE side and network side. For Type 3 training, the common dataset will be delivered to UE if the first part of AI/ML model is trained by the network. In such case, we think the common dataset can be delivered via air-interface, and the potential spec impact should be studied.

Proposal 3: For data collection, study the potential spec impact of dataset delivery based on 3GPP signaling.

In last meeting, it was agreed to study potential specification impact needed to enable the development of a set of specific models. To develop scenario-/configuration/site-specific models, the scenario-/configuration/site-specific datasets are the perquisite. How to construct the dataset and inform the related information should be discussed. One potential way is to associate the dataset with a specific scenario/configuration/site, and the association can be informed to UE by 3GPP signaling.

Proposal 4: To enable the development of a set of specific models, study the way to associate the dataset with a specific scenario/configuration/site.

For different purposes in LCM, the requirements and mechanisms for data collection can be different. In the initial model training phase without fine-tuning, a dataset with huge size and high accuracy may be needed. This dataset can be a mixed dataset with several sub datasets generated under different scenarios, depending on the requirements on generalization performance. One dataset can be used for multiple usages, like model update and model monitoring. The AI/ML model could firstly use the dataset to check the performance of itself, and then the same dataset can be reused for updating (fine-tuning) this AI/ML model to improve inference performance. The collection of dataset could be periodic performed or triggered by gNB or UE, also dataset collection via broadcast is also possible, depending on the latency requirement. Taking beam management use case as an example, gNB may transmit lots of RS to assist model monitoring, which requires a large amount of RS resource. If different UEs request beam measurement at different time, and gNB response all these requests without any condition, the total RS consumption may cancel out the AI/ML model’s benefit on overhead reduction. To resolve this issue, gNB could transmit the RS only if the number of UE requests is larger than a threshold or the time intervals from last RS transmission to first measurement request reaches a limitation, rather than every time UE request.

Proposal 5: To further improve the system performance, study the mechanism to reduce overhead of data collection in LCM.

  - Model identification and Functionality identification

Model identification is the process of identifying an AI/ML model for the common understanding between the NW and the UE, and functionality identification is the process of identifying an AI/ML functionality. In our mind, the motivation of model identification or functionality identification is to align the understanding between the NW and the UE when referring to an AI/ML model or AI/ML functionality. For UE-sided model or UE-part model, if the model is also trained by UE side, the approach of model identification may include the following steps.

Step1: UE reports the model existence and related model description to NW.

Step2: NW assign an ID to each AI/ML model.

If model inference is performed at UE side and model training is performed at NW side, model identification and model delivery can be integrated into one procedure, since NW can provide model ID, model description along with model structure and parameters to UE. Based on the model ID provided or assigned by the NW, model-ID-based LCM procedure can be used.

The model description information may include the following aspects:

- Model functionality

- Model applicability scenarios, configurations

- Information on model input

- Information on model output

- Information on assistance information

For the above several kinds of model description information, the model functionality should be mandatorily provided during registration to make network understand which sub use case(s) this model can be applied to. And whether the model applicability scenarios/configurations are needed or not may be related to the generalization of this model, for the model with well generalization capability, this description information may not be needed. For the model input, in some case, like the generation part for CSI compression sub use case, the model input is related to the UE implementation and does not need standardization. As for the model output and the necessary assistance information of the model, whether and how to provide them is much related to their functionality and can be discussed in each sub use case.

For functional identification, UE shall report the AI/ML functionality related information to NW. One issue is what is the relation between model identification and functionality identification. Based on the above analysis of model identification, model functionality information shall be shared between UE and NW. In this sense, functionality identification and model identification performed simultaneously. If only functionality identification is performed, to facilitate functionality-based LCM procedure, individual functionality ID may still be needed. For example, UE may support several AI/ML functionalities, to facilitate model activation/deactivation/switch for a certain AI/ML functionality, the functionality ID can be used to indicate the model functionality. For UE-sided model or UE-part model, if one model or several models with the same functionality are trained by UE side, UE can report the functionality existence and related description to NW, and NW assign the functionality ID to UE. The related description reported by UE may include the following aspects:

- Model functionality

- Applicability scenarios, configurations of models for the functionality

- Information on model input type(s)

- Information on model output type(s)

- Information on assistance information

For the model input/output type(s), if UE trained multiple AI/ML models for the functionality, the model input/output for each AI/ML model may be different, and all the supported input/output type(s) for the functionality may need to be reported. If model inference is performed at UE side and model training is performed at NW side, it is more nature to perform model identification along with model delivery, whether functionality identification is proper to such case should be further discussed.

Proposal 6: For the model description information during model identification, the following aspects could be considered:

- Model functionality

- Model applicability scenarios, configurations

- Information on model input

- Information on model output

- Information on assistance information

Proposal 7: For functionality identification, the functionality ID can be assigned by the network to facilitate functionality-based LCM procedure.

Proposal 8: For the description information during functionality identification, the following aspects could be considered:

- Model functionality

- Applicability scenarios, configurations of models for the functionality

- Information on model input type(s)

- Information on model output type(s)

- Information on assistance information

As for the granularity of functionality, we think the granularity of functionality might be smaller than the meaning of UE feature. There may be multiple functionalities corresponding to multiple configurations/scenarios/sites, even within one same sub use case. So, for functionality identification, there could be at least one functionality defined within an AI/ML-enabled sub use case.

Proposal 9: For functionality identification, there could be at least one functionality defined within an AI/ML-enabled sub use case.:

  - Model selection, activation, deactivation, switching

It was agreed that for UE-sided models and two-sided models, the decision for model selection/switching/activation/deactivation can be made by the network or the UE. For mechanism that make decision by the UE, one option is UE-autonomous and UE’s decision is not reported to the network. We think this mechanism is not reasonable. The operation between UE and network should be aligned. Take AI-based beam management as an example, if UE-sided model is used for DL Tx beam prediction, the beam reporting quantity can be different from that of the legacy beam reporting. If the AI/ML model is deactivated by UE, the network and UE should align the beam reporting accordingly. We think UE can report the decision to the network, and the decision can be applied after UE receiving the acknowledgement from the network. The signaling to report the decision and the acknowledgement to UE’s decision can be based on RRC or MAC-CE signaling.

Proposal 10: For the mechanism of model selection, activation, deactivation, switching, and fallback, if the decision is made by UE, UE’s decision should be reported to the network.

  - Model monitoring

The goal of model monitoring is to evaluate the performance of the AI/ML model based on the defined metrics. The metrics can be directly or indirectly related to the AI/ML model performance and can be different for each use case. Based on the results of the model monitoring, model updating/switching/activation/deactivation/fallback operations may be triggered. For UE-sided AI/ML model, the following model monitoring mechanisms have been agreed in AI 9.2.3.2.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

These mechanisms can also be used for other UE-sided AI/ML models. For NW-side AI/ML model, it is natural that the AI/ML model is controlled by the network, and hence NW-side model monitoring can be considered.

Proposal 11: For NW-sided AI/ML model, study the following mechanism for model monitoring

- Atl1. NW-side Model monitoring

  - NW monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

  - UE capability

For AI-related UE capability, it may include the capability of training, power, computation, storage, and so on. How to define and report the corresponding capability should be studied. From the reported UE capability, the network can decide whether an AI/ML model or a set of AI/ML models can be simultaneously employed at the UE side and whether the AI/ML model can be updated/fine-tuned by the UE.

Proposal 12: For AI-related UE capability, how to define and report the capability of training, power, computation, storage should be studied.

- Considerations on framework for AI/ML for air-interface

A functional framework for AI-enabled RAN intelligence has been proposed as follow.

Fig. 1 Functional Framework for RAN Intelligence

The above functional framework comprises of four functions, data collection, model training, model inference and actor. The detailed descriptions of each function can refer to TR 37.817 [3]. In our view, the above framework is a common and general framework, whether it can characterize the representative use cases of AI/ML for air-interface should be discussed.

Most functions of lifecycle management have already been reflected in the functional framework for RAN intelligence. Whether model monitoring should be presented as a separate function in the framework for air-interface can be discussed. Based on the metrics and methods for model monitoring, the location nodes for model inference and model monitoring can be the same or different. Moreover, model deployment and model update can also be operated at different nodes. For example, model training is located in the gNB, and the model inference is located in the UE. For model updating, UE can fine-tune the model with a small number of data samples.

Proposal 13: On Rel-18 AI/ML for air interface, whether a new framework based on the functional framework for RAN intelligence is needed can be studied.

- Considerations on evaluation methodology of AI/ML for air interface

The performance of AI/ML model relates to many factors, including the data sets, the architecture of the model, the training method, the optimization of hyper parameters, etc. For fair comparison with traditional schemes, the evaluation methodology for AI/ML based schemes should be studied.

  - Common data set

Unlike traditional algorithms used in wireless communication, artificial intelligence/machine learning is a data-based science. The data is used for nearly all the stages of AI/ML, including model training, model inference, model updating and model monitoring. The construction of data set is essential for AI/ML for air interface.

For each use case, the evaluation assumptions for constructing the dataset for training, validation and test has been discussed in the corresponding agenda. Besides, we think a common data set for each selected use case should be encouraged to be constructed. The common dataset can be uploaded to 3GPP web or a third-party web site in a proper way and each company could download it for evaluation and cross-checking of performance. The common data set could come from some companies’ input. Moreover, there might need some criteria to assess the validity and sufficiency of common data set.

Proposal 14: A common data set for each use cased could be encouraged to be constructed for evaluation and cross-checking of performance.

  - KPIs and requirements

In RAN1#110 meeting, an initial list of common KPIs has been listed for evaluating performance benefits of AI/ML. In this list, the generalization performance has been listed as a performance KPI for AI/ML based method. The generalization capability is to verify whether a model trained under a certain assumption can be applied well under different assumptions. The different assumptions may be different scenarios (e.g., Uma, Umi) or different configurations (e.g., different bandwidth, different number of antenna ports).

To improve the practical application of AI/ML based method, the AI model should be applicable to different configurations / scenarios. One possible way is to train and storage configuration / scenario specific model, however, the training and memory storage cost may be unacceptable. Therefore, the generalization of the AI model over different configurations/scenarios should be studied. When comparing the performance with baseline, instead of evaluating the performance under one single configuration/scenario, the average performance under multiple configurations/ scenarios should be evaluated.

On the other hand, the performance loss of intermediate or eventual performance KPIs using configurations / scenarios-common models over configurations/scenarios-specific models can also be adopted as the metric for evaluating the generalization performance.

Proposal 15: The average performance under multiple configurations / scenarios should be evaluated to evaluate the generalization capability of AI/ML model.

Proposal 16: The performance loss of intermediate or eventual performance KPIs using configurations / scenarios-common models over configurations / scenarios-specific models can also be adopted as the metric for evaluating the generalization performance.

In addition, the overhead of model delivery/transfer is also adopted as one part of over-the-air overhead. In our view, it is more related to the detailed model delivery formats. For instance, if the model is transferred using network/UE specific format, the specific format will impact on the overhead of model delivery/transfer. And if the model is transferred with a standard format, like ONNX, the overhead might be different from that of network/UE specific format. Besides, the information for delivery is also an important factor of model delivery overhead, i.e., whether both model structure and model parameters or only either one of them need be transferred will influence the model delivery overhead. In the initial phase, the model size can be simply adopted as one representative KPI to evaluate the overhead of model delivery/transfer.

Proposal 17: The model size can be adopted as one representative KPI to evaluate the overhead of model delivery/transfer.

Another open issue in RAN1#110 meeting is whether and how to evaluate the inference latency of AI/ML model. Inference latency is not only related to the model complexity, but also related to the hardware platform. In our view, it is difficult to evaluate the inference latency using a mathematical method. We think some representative chipset(s) can be selected to assess the inference latency of AI/ML model, both numerical calculation and prototype experiments method can be considered.

Proposal 18: The inference latency can be adopted as one common KPI when evaluating the performance of AI/ML model.

In RAN1#110bis e-meeting, the LCM related complexity and storage overhead are additionally adopted as common KPIs to evaluate the benefits of AI/ML. In our view, the calculation of the storage and computation for model monitoring is much related to the monitoring metrics/methods. For example, when the monitoring metric/method is the input or output data-based monitoring, such as data drift between training dataset and observed dataset or out-of-distribution detection, this kind of monitoring methods usually need large number of data samples to perform data drift or out-of-distribution detection. It will bring extra overhead of storing these data and extra complexity to compute the input or output data based KPIs for model monitoring.

Proposal 19: When the performance monitoring metric/method is the input or output data-based monitoring method, such as data drift between training dataset and observed dataset or out-of-distribution detection, the overhead of storing these data and the complexity to compute the input or output data-based KPIs need to be considered.

- Conclusion

Proposal 1: Confirm the working assumption on Level y-z boundary.

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: Other procedures than model transfer/delivery are decoupled with collaboration level y-z.

- Clarifying note: Level y includes cases without model delivery.

Proposal 2: Study the following options and potential spec impact of model delivery.

- Opt1. UE specific format based model transfer

- Opt2. Standard format based model transfer

- Opt3. OTT/OAM based model delivery

Proposal 3: For data collection, study the potential spec impact of dataset delivery based on 3GPP signaling.

Proposal 4: To enable the development of a set of specific models, study the way to associate the dataset with a specific scenario/configuration/site.

Proposal 5: To further improve the system performance, study the mechanism to reduce overhead of data collection in LCM.

Proposal 6: For the model description information during model identification, the following aspects could be considered:

- Model functionality

- Model applicability scenarios, configurations

- Information on model input

- Information on model output

- Information on assistance information

Proposal 7: For functionality identification, the functionality ID can be assigned by the network to facilitate functionality-based LCM procedure.

Proposal 8: For the description information during functionality identification, the following aspects could be considered:

- Model functionality

- Applicability scenarios, configurations of models for the functionality

- Information on model input type(s)

- Information on model output type(s)

- Information on assistance information

Proposal 9: For the mechanism of model selection, activation, deactivation, switching, and fallback, if the decision is made by UE, UE’s decision should be reported to the network.

Proposal 10: For the mechanism of model selection, activation, deactivation, switching, and fallback, if the decision is made by UE, UE’s decision should be reported to the network.

Proposal 11: For NW-sided AI/ML model, study the following mechanism for model monitoring

- Atl1. NW-side Model monitoring

  - NW monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

Proposal 12: For AI-related UE capability, how to define and report the capability of training, power, computation, storage should be studied.

Proposal 13: On Rel-18 AI/ML for air interface, whether a new framework based on the functional framework for RAN intelligence is needed can be studied.

Proposal 14: A common data set for each use cased could be encouraged to be constructed for evaluation and cross-checking of performance.

Proposal 15: The average performance under multiple configurations / scenarios should be evaluated to evaluate the generalization capability of AI/ML model.

Proposal 16: The performance loss of intermediate or eventual performance KPIs using configurations / scenarios-common models over configurations / scenarios-specific models can also be adopted as the metric for evaluating the generalization performance.

Proposal 17: The model size can be adopted as one representative KPI to evaluate the overhead of model delivery/transfer.

Proposal 18: The inference latency can be adopted as one common KPI when evaluating the performance of AI/ML model.

Proposal 19: When the performance monitoring metric/method is the input or output data-based monitoring method, such as data drift between training dataset and observed dataset or out-of-distribution detection, the overhead of storing these data and the complexity to compute the input or output data-based KPIs need to be considered.

- References

- 3GPP RAN1#112, RAN1 Chairman’s Notes, Athens, Greece, February 2023.

- 3GPP TR 37.817, v17.0.0, April 2022.