3GPP TSG RAN WG1 #112bis-e				                        R1-2303231

e-Meeting, April 17th – April 26th, 2023

Source: 	CMCC

Title:	Updated summary on SLS calibration results for NR duplex evolution

Agenda item:	9.3

Document for:	Discussion

Introduction

The SI Study on evolution of NR duplex operation was approved in RAN plenary #94-e meeting [1], and the latest updated SID was approved in RAN plenary #97 e-meeting [2].

In RAN1#110b e-meeting, 3GPP agreed to conduce SLS calibration for evaluation of SBFD operation [4]

Agreement RAN1 to conduct a SLS calibration for evaluation of SBFD operation.

The detailed simulation assumption has been established in the last several meetings [3]-[5]. Before RAN1#112, a SLS calibration for NR duplex was conducted on a voluntary basis and an email discussion was kicked off to collect the SLS calibration results. The calibration results were uploaded under the following draft folder.

- ftp://ftp.3gpp.org/tsg_ran/WG1_RL1/TSGR1_111/Inbox/drafts/9.3(FS_NR_duplex_evo)/9.3.1/Calibration#112/

This document provides the updated summary of the calibration activities and the collected results.

SLS calibration methodology

Calibration scenarios

The SLS calibration focuses on the following scenarios of SBFD Deployment Case 1:

- FR1

  - Urban Macro

  - Indoor office

- FR2-1

  - Dense Urban Macro layer

  - Indoor office

Calibration metrics

The following metrics are considered for SLS calibration:

- CDF of gNB-UE coupling loss

- CDF of Inter-gNB coupling loss

- CDF of Inter-UE coupling loss

Table 1  Statistic method for SLS calibration

| SLS calibration metrics | Statistic method |
| --- | --- |
| CDF of gNB-UE coupling loss | Only the coupling losses between each UE and its serving cell are collected for CDF statistic. - wA and gB are determined by selecting the best beam pair of the UE and its serving cell with the criteria of maximizing receive power of the UE. |
| CDF of gNB-gNB coupling loss | For one SLS drop, generate channels among gNBs, calculate and collect the coupling loss for each gNB pair - The two gNBs in each gNB pair should be from different sites. - Both wA and gB are randomly selected for calculating the coupling loss for each gNB pair. Companies to run enough SLS drops and report the number of SLS drops when plotting the CDF using the collected coupling losses. |
| CDF of UE-UE coupling loss | For one SLS drop, drop UEs in the network and generate channels among UEs, calculate and collect the coupling loss for each UE pair - If the 2D distance between two UEs in a UE pair is larger than 50m, the UE pair is not considered for statistic. - For each UE, wA and gB is determined based on the best beam pair of the UE and its serving cell. Companies to run enough SLS drops and report the number of SLS drops when plotting the CDF using the collected coupling losses. |
| NOTE 1: Formula (2) for CL with averaging across all the Tx/Rx ports is used for coupling loss calculation above, i.e., CLA,BwA,gB=1Sp=0S-11Uu=0U-1CLp,uA,BwA,gB wherein, CLp,uA,BwA,gB=PL∙SF∙α'0,u,p2#2 α'0,u,p=Frx,u,θθLOS,ZOA,ϕLOS,AOAFrx,u,ϕθLOS,ZOA,ϕLOS,AOAT∙100-1∙Ftx,p,θθLOS,ZOD,ϕLOS,AODFtx,p,ϕθLOS,ZOD,ϕLOS,AOD#3 NOTE 2: The beams for above cases are selected based on a defined set of beams for FR1 and FR2 in the table for calibration assumptions. |  |

SLS calibration assumptions

Table 2: SLS calibration assumptions

|  | Urban Macro (FR1) | Dense Urban Macro Layer (FR2-1) | Indoor office (FR1) | Indoor office (FR2-1) |
| --- | --- | --- | --- | --- |
| Carrier frequency | 4 GHz | 30GHz | 4 GHz | 30GHz |
| System bandwidth | 100MHz | 100MHz | 100MHz | 100MHz |
| Numerology | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 120kHz | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 120kHz |
| BS transmit power for SBFD | 53 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 40 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 24 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 23 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD |
|  | Assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. |  |  |  |
| UE Tx power | 23dBm | 23 dBm. EIRP should not exceed 43 dBm | 23dBm | 23 dBm. EIRP should not exceed 43 dBm |
| Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  | 12 TRPs per 120m x 50m x 3m - X-axis is pointing down to the floor - The antenna array is mounted in the Y-Z plane with boresight along the X-axis - The X-axis/Y-axis/Z-axis refer to LCS | 12 TRPs per 120m x 50m x 3m - X-axis is pointing down to the floor - The antenna array is mounted in the Y-Z plane with boresight along the X-axis - The X-axis/Y-axis/Z-axis refer to LCS |
| Inter-BS (2D) distance | 500m | 200m | 20m |  |
| Minimum BS-UE (2D) distance | 35m | 35m | 0m |  |
| Minimum UE-UE (2D) distance | 1m | 1m | 1m |  |
| BS antenna height | 25m | 25m | 3m |  |
| UE distribution | UE Clustering | UE Clustering | Uniform |  |
| UE number per macro/indoor TRP (per direction) (M) | 20 | 10 | 10 |  |
| UE cluster number per macro cell (X) | 2 | 1 | - |  |
| UE outdoor/indoor proportion | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h | 100% outdoor without car penetration loss: 3km/h | 100% indoor in houses: 3km/h |  |
| Indoor UE height (m) | 1.5m | 1.5m | 1.5m |  |
| Outdoor UE height (m) | 1.5m | 1.5m | - |  |
| Radius of cluster (R) | 25m | 20m | - |  |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | 60m | 55m | - |  |
| Minimum distance between two UE cluster centers (Dinter-cluster) | 50m | - | - |  |
| gNB-UE Channel model | Macro-to-UE: UMa in TR 38.901 For FR1, gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |  | TRP-to-UE: InH-Office in TR 38.901 Penetration loss is not modelled. |  |
| gNB-gNB Channel model (large-scale) | Macro-to-Macro: UMa in TR 38.901 (hUE =25m) LOS probability: If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 0.75; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. |  | TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) Penetration loss is not modelled. |  |
| UE-UE Channel model (large-scale) | UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m). For FR1, penetration loss between UEs follows Table A.2.1-12 in TR38.802 |  | UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) |  |
| BS antenna array configuration for Legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization | (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | (M,N,P,Mg,Ng;Mp,Np) = (4,4,2,1,1; 4,4) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | (M,N,P,Mg,Ng;Mp,Np) =(16,8,2,1,1; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization |
| BS antenna array configuration for SBFD | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (8,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (4,16,2,2,2). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (4,4,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (16,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ |
| BS antenna radiation pattern | reuse Table 9 in Report ITU-R M.2412 |  | reuse Table 10 in Report ITU-R M.2412 |  |
| UE antenna configuration | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain | reuse Table 11 in Report ITU-R M.2412 | Omni-directional with 0 dBi element gain | reuse Table 11 in Report ITU-R M.2412 |
| BS receiver noise figure | 5dB | 7 dB | 5dB | 7 dB |
| UE receiver noise figure | 9 dB | 13 dB | 9 dB | 13 dB |
| Open loop power control parameters | P0= -80 dBm, alpha = 0.8 | P0= -86 dBm, alpha = 0.9 | P0= -60 dBm, alpha = 0.6 |  |
| Handover margin (dB) | 3 dB | 3 dB | 3 dB | 3 dB |
| UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. - Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. - Single gNB panel is used for UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. - Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. - Single gNB panel is used for UE attachment |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |  |  |  |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) | 180° in GCS (pointing to the ground) | 180° in GCS (pointing to the ground) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") | (According to Zenith angle in "Beam set at TRxP") | 90° in LCS | (According to Zenith angle in "Beam set at TRxP") |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = {5*pi/8, 7*pi/8} NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = {pi/4, 3*pi/4} NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |
| NOTE 1: The UE cluster is totally confined within the macro cell geographical area (i.e. a cluster cannot be partially overlap with adjacent cell area) |  |  |  |  |

Calibration results

The calibration results are provided in the attachment. Twenty-one 3GPP members provided the calibration results, including CMCC, Samsung, Spreadtrum, Huawei, Nokia, CATT, Xiaomi, ZTE, Sharp, Qualcomm, MTK, Ericsson, CEWiT, vivo, Intel, Sony, IDC, Fujitsu, OPPO, MediaTek, DOCOMO.

According to the agreed SLS calibration assumption, only large scale fading is modelled for SLS calibration, i.e., using formula (2) in Section 2, which can be denoted as “L”. Summary is draw only based on the results for “L” case.

In addition, some companies also provide results for both large scale fading and small scale fading are modelled, which is denoted as “L+S”. The results for “L+S” case will not be considered in this summary.

A summary on samples collected of each calibration scenario is shown in Appendix A.

Attachment: Calibration results for evaluation of SBFD operation.

Summary

In this contribution, the calibration methodology and calibration parameters are discussed and captured in section 2 and 3. The calibration results are provided in section 4. It is observed that

- For Urban Macro (FR1),

  - From 18 sources out of 19 sources, the calibration results for gNB-UE from the independent samples are within 3.57 dB of the average CL

  - From 15 sources out of 18 sources, the calibration results for gNB-gNB from the independent samples are within 2.30 dB of the average CL

  - From 17 sources out of 18 sources, the calibration results for UE-UE from the independent samples are within 3.39 dB of the average CL

- For Dense Urban Macro Layer (FR2-1),

  - From all 11 sources, the calibration results for gNB-UE from the independent samples are within 3.03 dB of the average CL

  - From 7 sources out of 11 sources, the calibration results for gNB-gNB from the independent samples are within 3.83 dB of the average CL

  - From all 9 sources out of 11 sources, the calibration results for UE-UE from the independent samples are within 3.02 dB of the average CL

- For Indoor office (FR1),

  - From all 17 sources, the calibration results for gNB-UE from the independent samples are within 2.41 dB of the average CL

  - From 15 sources out of 17 sources, the calibration results for gNB-gNB from the independent samples are within 2.46 dB of the average CL

  - From all 17 sources, the calibration results for UE-UE from the independent samples are within 2.50 dB of the average CL

- For Indoor office (FR2-1),

  - From 11 sources out of 12 sources, the calibration results for gNB-UE from the independent samples are within 1.85 dB of the average CL

  - From 8 sources out of 12 sources, the calibration results for gNB-gNB from the independent samples are within 3.33 dB of the average CL

  - From 10 sources out of 12 sources, the calibration results for UE-UE from the independent samples are within 3.73 dB of the average CL

References

- RP-213591, New SI: Study on evolution of NR duplex operation, CMCC

- RP-222110, Revised SID: Study on evolution of NR duplex operation, CMCC

- Chairman's Notes RAN1#111, Toulouse, France, November 14th – 18th, 2022.

- Chairman's Notes RAN1#110b-e, e-Meeting, October 10th – 19th, 2022.

- Chairman's Notes RAN1#110, Toulouse, France, August 22nd – 26th, 2022.

- Annex A: Collected samples

Twenty-one 3GPP members provided the calibration results, including CMCC, Samsung, Spreadtrum, Huawei, Nokia, CATT, Xiaomi, ZTE, Sharp, Qualcomm, MTK, Ericsson, CEWiT, vivo, Intel, Sony, IDC, Fujitsu, OPPO, MediaTek, DOCOMO. A summary of the collected samples of each calibration scenario is shown inTable 3.

Table 3: Sample statistics for calibration scenarios

| calibration scenario | Channel | Total number of samples | CL difference compared to average CL (at 50%-tile CDF point) |
| --- | --- | --- | --- |
| Urban Macro (FR1) | gNB-UE | 19 | 4.18 dB (19 sources), 3.57 dB (18 sources) |
|  | gNB-gNB | 18 | 10.77 dB (18 sources), 2.30 dB (15 sources) |
|  | UE-UE | 18 | 4.70 dB (18 sources), 3.39 dB (17 sources) |
| Dense Urban Macro Layer (FR2-1) | gNB-UE | 11 | 3.03 dB (11 sources) |
|  | gNB-gNB | 11 | 8.51 dB (11 sources), 3.83 dB (7 sources) |
|  | UE-UE | 11 | 13.23 dB (11 sources), 3.02 dB (9 sources) |
| Indoor office (FR1) | gNB-UE | 17 | 2.41 dB (17 sources) |
|  | gNB-gNB | 17 | 6.70 dB (17 sources), 2.46 dB (15 sources) |
|  | UE-UE | 17 | 2.50 dB (17 sources) |
| Indoor office (FR2-1) | gNB-UE | 12 | 4.54 dB (10 sources), 1.85 dB (11 sources) |
|  | gNB-gNB | 12 | 5.70dB (12 sources), 3.33 dB (8 sources) |
|  | UE-UE | 12 | 8.47 dB (12 sources), 3.73 dB (10 sources) |