3GPP TSG RAN WG1 #112bis-e				                        R1-2303232

e-Meeting, April 17th – April 26th, 2023

Source: 	CMCC

Title:	Evaluation on NR duplex evolution

Agenda item:	9.3.1

Document for:	Discussion & Decision

Introduction

Up to RAN1#112 meeting [1]-[6], progress has been made to establish a comprehensive SLS evaluation methodology. In this contribution, we discuss the remaining issues on SLS and LLS evaluation methodology. Some initial evaluation results are also provided.

General

Consideration on evaluation for dynamic/flexible TDD in AI 9.3.3

Regarding potential enhancements on dynamic/flexible TDD, the following gNB-to-gNB / UE-to-UE co-channel CLI handling schemes are being discussed in AI 9.3.3

- For gNB-to-gNB CLI handling

  - Scheme 1: gNB-to-gNB CLI/channel measurement, e.g., uplink resources muting

  - Scheme 2: Coordinated scheduling

  - Scheme 3: Spatial domain enhancements

  - Scheme 4: Advanced receiver

  - Scheme 5: Enhance power control mechanism

- For UE-to-UE CLI handling

  - Scheme 1: Potential enhancements to UE-to-UE CLI measurement/reporting, e.g., L1/L2 based UE-to-UE CLI measurement and reporting

  - Scheme 2: Coordinated scheduling

  - Scheme 3: Spatial domain enhancements

  - Scheme 4: Enhance UL power control mechanism

- Other schemes and combinations are not precluded.

For the above schemes, it may not always be the case that SLS is used as the tool for evaluation. For some schemes, maybe LLS or even analysis is more suitable, and for some other schemes SLS is more suitable. If SLS is used for evaluation, the basic evaluation methodologies and assumptions for dynamic/flexible TDD that we have established in AI 9.3.1 can be used. For some CLI schemes, additional scheme-specific assumptions may be needed. It can be up to companies to report these scheme-specific assumptions.

Proposal 1: Regarding SLS of potential enhancements on dynamic/flexible TDD in AI 9.3.3,

- The basic evaluation methodologies and assumptions for dynamic/flexible TDD agreed in AI 9.3.1 can be used.

- If additional scheme-specific assumptions are needed for some CLI schemes, it is up to companies to report the scheme-specific assumptions.

For some CLI schemes, it is up to companies to report additional scheme-specific assumptions Evaluation results collection

Annex A “Format for capturing evaluation results in TR 38.858” is for clarifying our initial thinking on how to capture the evaluation results and observations under TR clause 7.3 for SBFD.

Considering there are a lot of parameters with different options and different companies may select different values for each of these parameters, it would be better that companies’ final results can be categorized into several sub-cases based on some key assumptions, with each sub-case corresponding to one combination of key assumptions. An example is shown below.

Table X: Sub-cases for Urban Macro in FR1 in SBFD Deployment Case 1.

| Key assumptions Sub-cases | Interference modelling (e.g., Co-site: Spatial isolation + digital isolation) |  |  |  | SBFD slot configuration |  | BS transmit power |  | SBFD antenna configuration |  | Packet Size |  | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 75dB | 93dB | 100dB | 100+10 dB | Alt-2: {DDDSU} vs. {XXXXU} | Alt-4: {DDDSU} vs. {XXXXX} | 53dBm | 49dBm | Twice area&same TxRUs | Same area&same TxRUs | DL: 4Kbytes, UL: 1Kbyte | DL: 0.5Mbytes, UL: 0.125Mbyte |  |
| SBFD#1_UMa_FR1_Sub#1 |  |  |  | ○ | ○ |  | ○ |  | ○ |  |  |  | Source [X], … |
| SBFD#1_UMa_FR1_Sub#2 |  |  |  | ○ |  | ○ | ○ |  | ○ |  |  |  | Source [X], … |

The number of sub-cases should not be too large, otherwise it will be difficult to summarize and draw conclusions. However, currently we cannot determine how many sub-cases will be categorized into and which assumptions should be used for the categorization. One way is that we first collect all the evaluation results and corresponding assumptions from different companies for each scenario in a single table, and then based on the assumptions we can decide how many sub-cases to be determined and which assumptions are used as key assumptions for the categorization.

Based on the above consideration, four subfolders, each contains one excel file, are uploaded to the FTP draft folder with the link (ftp://ftp.3gpp.org/tsg_ran/WG1_RL1/TSGR1_112/Inbox/drafts/9.3(FS_NR_duplex_evo)/9.3.1/Evaluation Results/) to collect the SLS simulation results for NR duplex, wherein,

- SBFD-Case1 subfolder: for SBFD deployment Case 1

- SBFD-Case3-2 subfolder: for SBFD deployment Case 3-2

- SBFD-Case4 subfolder: for SBFD deployment Case 4

- DTDD subfolder: for dynamic/flexible TDD

In these excel sheets, we encourage companies to provide both their assumptions and evaluation results. Each company can input multiple columns, and each column corresponds to one kind of assumption and the corresponding result. Typically, one company may only change some of the parameters, you can just copy the parameters in column G (i.e., the exemplary values), and modify the necessary ones.

Regarding the performance metrics, considering there are several metrics for UPT and latency, companies may not provide the results for all these metrics. Moderator recommends companies to at least provide Average-UPT CDF related metrics and Packet-Latency CDF related metrics for drawing conclusions, and other metrics (e.g., Tail-UPT, Median-UPT, UE-Average-Latency) are up to companies. In addition, moderator recommends companies to at least provide relative gains of SBFD compared to legacy TDD for UPT and latency related metrics, and the absolute values are up to companies.

For each excel file, every time when update the results, moderator recommends companies to add one new row in the sheet named “Revision comments” to briefly indicate what changes have been made at this time.

Proposal 2: Companies are encouraged to upload evaluation results to the FTP draft folder with the link (ftp://ftp.3gpp.org/tsg_ran/WG1_RL1/TSGR1_112/Inbox/drafts/9.3(FS_NR_duplex_evo)/9.3.1/Evaluation Results/)

- In the excel sheets, companies are encouraged to provide both assumptions and evaluation results. Each company can input multiple columns, and each column corresponds to one kind of assumption and the corresponding result

- Regarding the performance metrics, it is recommended to at least provide Average-UPT CDF related metrics and Packet-Latency CDF related metrics for drawing conclusions, and other metrics (e.g., Tail-UPT, Median-UPT, UE-Average-Latency) are up to companies. In addition, it is also recommended to at least provide relative gains of SBFD compared to legacy TDD for UPT and latency related metrics, and the absolute values are up to companies

- For each excel file, every time when update the results, companies are recommended to add one new row in the sheet named “Revision comments” to briefly indicate what changes have been made at this time

Proposal 3: For duplex evolution evaluation, the evaluation results are categorized into X sub-cases (as shown in below table for example) based on the different key assumptions. Each sub-case is based on one combination of key assumptions.

- Note: How many sub-cases will be determined and which assumptions will be used for the categorization can be discussed based on the final evaluation results and assumptions submitted by companies.

Table X: Sub-cases for Urban Macro in FR1 in SBFD Deployment Case 1.

| Key assumptions Sub-cases | Interference modelling (e.g., Co-site: Spatial isolation + digital isolation) |  |  |  | SBFD slot configuration |  | BS transmit power |  | SBFD antenna configuration |  | Packet Size |  | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 75dB | 93dB | 100dB | 100+10 dB | Alt-2: {DDDSU} vs. {XXXXU} | Alt-4: {DDDSU} vs. {XXXXX} | 53dBm | 49dBm | Twice area&same TxRUs | Same area&same TxRUs | DL: 4Kbytes, UL: 1Kbyte | DL: 0.5Mbytes, UL: 0.125Mbyte |  |
| SBFD#1_UMa_FR1_Sub#1 |  |  |  | ○ | ○ |  | ○ |  | ○ |  |  |  | Source [X], … |
| SBFD#1_UMa_FR1_Sub#2 |  |  |  | ○ |  | ○ | ○ |  | ○ |  |  |  | Source [X], … |

Proposal 4: For each sub-case, the performance gains of SBFD over legacy TDD are summarized (as shown in below table for example). For each performance metric, the {mean, <min~max>} is used to represent the mean value and the value range from the sources.

Table-Y: Summary of results for sub-case XX.

| Simple description for the sub-case (e.g., 100dB co-site inter-sector co-channel inter-subband isolation, SBFD Alt2, 49dBm gNB Tx power, Twice area&same TxRUs, DL: 4Kbytes, UL: 1Kbyte, UE clustering,…) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT (Mbps) | Mean | {mean, <min~max>} = {3%, <-10%~20%>} |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Average-UPT (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 |  |  |  |  |  |  |  |  |  |
|  | Type-2 |  |  |  |  |  |  |  |  |  |
| UL RU (%) | Type-1 |  |  |  |  |  |  |  |  |  |
|  | Type-2 |  |  |  |  |  |  |  |  |  |
| Note: - For Average-UPT / Packet-Latency / RU, the gain can be calculated as: Gain (%) = SBFD UPT/latency/RUTDD UPT/latency/RU-1 |  |  |  |  |  |  |  |  |  |  |

Proposal 5: The following table is used to capture companies’ evaluation results in the Annex of TR 38.858. Companies are encouraged to provide evaluation results in their submitted contribution with the following table.

Table D.1.2: Evaluation results for Urban Macro in FR1 in SBFD Deployment Case 1

| Tdoc/Source | Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
|  | DL Average-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Tail-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Median-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Average-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Tail-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Median-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | Type-1 RU (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Type-2 RU (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Unfinished/dropped Packet Rate (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Additional comments: e.g., Layout and UE distribution - Macro Layer: e.g., Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - UE distribution: e.g., UE clustering distribution with M=20, X=2 Interference Modelling - gNB self-interference: e.g., based on 1 dB UL desense - Co-site inter-sector co-channel inter-subband CLI: e.g., 100dB (spatial isolation), 10dB digital isolation - UE-UE co-channel inter-subband CLI: e.g., 33 dBc SBFD subband and slot configuration - SBFD slot configuration: Alt 2 (higher priority): Legacy TDD: {DDDSU}; SBFD: {XXXXU} - SBFD Subband configuration: e.g., <ND, NU, NG >=<104, 55, 5> - Guard symbol number: - UL resource percentage per TDD period (%): - DL resource percentage per TDD period (%): BS transmit power & antenna configuration - BS transmit power for legacy TDD: e.g., 53dBm - BS transmit power for SBFD: e.g., Option-1: Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems) - BS antenna configuration for legacy TDD: e.g., (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) , (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization - BS antenna configuration for SBFD: e.g., Twice area&same TxRUs (higher priority): SBFD antenna configuration Option 2 - BS antenna radiation pattern: e.g., Table 9 in Report ITU-R M.2412 - UE antenna configuration: e.g., 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization; 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization Traffic Model - DL/UL traffic assignment for the same UE: e.g., Option 2: Each UE is assigned both UL traffic and DL traffic - DL/UL FTP packet size: 4Kbytes for DL and 1Kbyte for UL Channel model - gNB-gNB: e.g., Both Large scale fading and small scale fading - UE-UE: e.g., Large scale fading only - UE-UE details: e.g., TR 38.901 Others - Open loop power control parameters: e.g., P0= -80 dBm, alpha = 0.8 - UE receiver: e.g., MMSE-IRC - Channel estimation: e.g., Ideal - Transmission scheme: e.g., SU-MIMO - Overhead: |  |  |  |  |  |  |  |  |  |  |
| Tdoc/Source | Reported Parameters |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
|  |  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
|  | DL Average-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Tail-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Median-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Average-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Tail-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Median-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | Type-1 RU (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Type-2 RU (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Unfinished/dropped Packet Rate (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Additional comments: e.g., Layout and UE distribution - Macro Layer: e.g., Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - UE distribution: e.g., UE clustering distribution with M=20, X=2 Interference Modelling - gNB self-interference: e.g., based on 1 dB UL desense - Co-site inter-sector co-channel inter-subband CLI: e.g., 100dB (spatial isolation), 10dB digital isolation - UE-UE co-channel inter-subband CLI: e.g., 33 dBc SBFD subband and slot configuration - SBFD slot configuration: Alt 4: Legacy TDD: {DDDSU}; SBFD: {XXXXX} - SBFD Subband configuration: e.g., <ND, NU, NG >=<104, 55, 5> - Guard symbol number: - UL resource percentage per TDD period (%): - DL resource percentage per TDD period (%): BS transmit power & antenna configuration - BS transmit power for legacy TDD: e.g., 53dBm - BS transmit power for SBFD: e.g., Option-1: Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems) - BS antenna configuration for legacy TDD: e.g., (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) , (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization - BS antenna configuration for SBFD: e.g., Twice area&same TxRUs (higher priority): SBFD antenna configuration Option 2 - BS antenna radiation pattern: e.g., Table 9 in Report ITU-R M.2412 - UE antenna configuration: e.g., 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization; 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization Traffic Model - DL/UL traffic assignment for the same UE: e.g., Option 2: Each UE is assigned both UL traffic and DL traffic - DL/UL FTP packet size: 4Kbytes for DL and 1Kbyte for UL Channel model - gNB-gNB: e.g., Both Large scale fading and small scale fading - UE-UE: e.g., Large scale fading only - UE-UE details: e.g., TR 38.901 Others - Open loop power control parameters: e.g., P0= -80 dBm, alpha = 0.8 - UE receiver: e.g., MMSE-IRC - Channel estimation: e.g., Ideal - Transmission scheme: e.g., SU-MIMO - Overhead: |  |  |  |  |  |  |  |  |  |  |

UL/DL resource percentage per TDD period

UL (or DL) resource percentage per TDD period can be a useful metric to investigate the net performance gain/loss of SBFD over legacy TDD. For example, when compared with legacy TDD, if the UL UPT of SBFD is increased by X% (e.g., 100%), while the available UL resources is increased by Y% (e.g., 80%), then the net UL UPT gain of SBFD over legacy TDD is about (X-Y)% (e.g., 100% - 80% = 20%).

UL (or DL) resource percentage per TDD period can be calculated as follows:

- UL resource percentage per TDD period = (Number of UL RBs per cell per TDD period excluding guard bands and guard symbols) / (Total number of RBs per cell per TDD period including DL, UL, guard bands and guard symbols )

- DL resource percentage per TDD period = (Number of DL RBs per cell per TDD period excluding guard bands and guard symbols) / (Total number of RBs per cell per TDD period including DL, UL, guard bands and guard symbols )

Note: DL+UL resource percentage is not always 100% since guard bands and guard symbols are assumed.

Since the UL (or DL) resource percentage per TDD period is related to the guard symbols assumed by companies, which is also related to the assumed SBFD antenna configurations, it is recommended that companies can report the UL (or DL) resource percentage per TDD period together with the evaluation results. An example of UL/DL resource percentage per TDD period for SBFD slot configuration Alt 2 and Alt 4 is provided in 错误!未找到引用源。 with the assumption that SBFD antenna configuration option 2 is assumed.

Table 1 One example of UL/DL resource percentage per TDD period for SBFD slot configuration Alt 2 and Alt 4 with SBFD antenna configuration option 2 is assumed.

|  | Legacy TDD | SBFD slot configuration Alt 4 with no guard symbol | SBFD slot configuration Alt 2 with 2 guard symbols | SBFD Alt 2 slot configuration with no guard symbol |
| --- | --- | --- | --- | --- |
| slot configuration | DDDSU,S=12:2:0 | XXXXX | XXXXU with 2 guard symbols between X and U | XXXXU with no guard symbol |
| DL only symbol | 54 | 0 | 0 | 0 |
| UL only symbol | 14 | 0 | 14 | 14 |
| SBFD symbol | 0 | 70 | 54 | 56 |
| DL RB number in DL only symbol | 273 | 273 | 273 | 273 |
| UL RB number in UL only symbol | 273 | 273 | 273 | 273 |
| DL RB number in SBFD symbol |  | 208 | 208 | 208 |
| UL RB number in SBFD symbol |  | 55 | 55 | 55 |
| Guard RB number in SBFD symbol |  | 10 | 10 | 10 |
| Total DL RB in 5 slots | 14742 | 14560 | 11232 | 11648 |
| Total UL RB in 5 slots | 3822 | 3850 | 6792 | 6902 |
| Total RB in 5 slots | 19110 | 19110 | 19110 | 19110 |
| DL resource percentage per TDD period | 77.1% | 76.2% | 58.8% | 61.0% |
| UL resource percentage per TDD period | 20.0% | 20.1% | 35.5% | 36.1% |
| DL resource percentage gain over legacy TDD | - | -1.2% | -23.8% | -21.0% |
| UL resource percentage gain over legacy TDD | - | 0.7% | 77.7% | 80.6% |

Proposal 6: It is recommended that companies to report the UL/DL resource percentage per TDD period together with the evaluation results, which can be calculated as follows:

- UL resource percentage per TDD period = (Number of UL RBs per cell per TDD period excluding guard bands and guard symbols)  /(Total number of RBs per cell per TDD period including DL, UL, guard bands and guard symbols )

- DL resource percentage per TDD period = (Number of DL RBs per cell per TDD period excluding guard bands and guard symbols) / (Total number of RBs per cell per TDD period including DL, UL, guard bands and guard symbols )

Remaining issues for SLS

Interference modelling for SBFD

As shown in Table 2, during the past RAN1 meetings, there are many LS interactions between RAN1 and RAN4 to confirm interference modelling understanding for SBFD.

Table 2  LS interaction between RAN1 and RAN4.

| RAN1 meeting | LS from RAN1 to RAN4 | Reply LS from RAN4 to RAN1 |
| --- | --- | --- |
| RAN1#109-e | R1-2205543 LS on interference modelling for duplex evolution RAN1, CMCC |  |
| RAN1#110bis-e | R1-2210602 LS on interference modelling for duplex evolution RAN1, CMCC | R1-2208347 Reply LS on interference modelling for duplex evolution (Reply to R1-2205543) RAN4, Samsung, CMCC, Qualcomm |
|  | R1-2210671 LS on maximum number of UL subbands for duplex evolution RAN1, CATT |  |
| RAN1#111 | R1-2212963 LS on interference modelling for duplex evolution RAN1, CMCC |  |
| RAN1#112 | R1-2302087 LS on interference modelling for duplex evolution RAN1, CMCC | R1-2300025 LS response to RAN1 for interference modelling and Sub-band configuration (Reply to R1-2205543, R1-2210602, R1-2210671) RAN4, Samsung |
| RAN1#112bis-e |  | R1-2302262 Reply LS on interference modelling (Reply to R1-2212963) RAN4, CATT |

Based on RAN4’s reply LS, agreements on interference modelling for SBFD can be confirmed or updated.

gNB self-interference modelling

Observation 1: The following agreements related to the modelling of gNB self-interference were confirmed by RAN4 according to R1-2300025.

[RAN1#110b] Agreement For SLS of SBFD in RAN1, candidate values for αSI at least can be determined based on the assumption that UL receiver sensitivity degradation due to self-interference is 1dB. - FFS: UL receiver sensitivity degradation due to self-interference is 0.8dB and 0.1dB - The value of αSI can be calculated based on the UL receiver sensitivity degradation, noise floor of UL subband and maximum gNB DL Tx Power as below - αSI=PtxmaxISImax*NDLRBNULRB - For example, for sensitivity degradation of 1dB, ISImax can be computed based on 10*log10ISImax+NN=1dB, where N is the noise floor over the UL subband given by N(dB)=-174 + 10*log10 (20e6) + 5 = -96 dBm, assuming 20MHz UL subband and 5dB noise figure. - Note: the feasibility of the determined αSI values can be discussed separately - Companies shall report what values of the individual components are assumed in order to achieve the alpha_SI value corresponding to 1 dB desense - Other approaches of determining values for αSI are not precluded and can be used and reported by companies. Send LS to RAN4 to confirm RAN1’s understanding. Agreement RAN1 assumes frequency isolation value in the overall RSI value ranges provided by RAN4 is based on the assumption of SBFD subband configuration with {DUD=40MHz:20MHz:40MHz} at least for FR1 and all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power. - For SLS of SBFD in RAN1, the RSI is modelled as frequency flat within the UL subband. - Using αSI to denote the overall RSI value provided by RAN4, RAN1 makes the following assumption - ISImax=PtxmaxαSI*NULRBNDLRB - ISImax is the residual self-interference power on the UL subband when all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power (in linear scale). - Ptxmax is the maximum gNB DL Tx Power on the two DL subbands (in linear scale). - NDLRB is the total number of DL RBs in the DL subbands. - NULRB is the total number of UL RBs in the UL subband. - Note: αSI is in linear scale - RAN1 further makes a simple assumption that αSI doesn’t change when DL RBs are not fully allocated for DL transmission, and the residual self-interference power on one UL RB when DL RBs are not fully allocated for DL transmission is computed by - ISIper-RB=Ptxper-RBαSI*Nused-DL-RBNDLRB - Ptxper-RB is DL transmission power of gNB per RB, Ptxper-RB=Ptxmax/NDLRB. - Nused-DL-RB is the number of DL RBs allocated for DL transmission. - Send LS to RAN4 to confirm RAN1’s assumptions and the subband configuration assumed for FR1/FR2 - Also ask RAN4 if the above is applicable to other subband configurations

Regarding the candidate values for αSI,

- For BS transmit power for SBFD Option-1 (Baseline): no power boosting

  - Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems), i.e., BS transmit power spectrum density per Tx chain is kept the same for SBFD symbols and DL-only symbols

  - In this case

PtxmaxNDLRB=PtxTDDNRBTDD

αSI=PtxmaxISImax*NDLRBNULRB=PtxmaxNDLRBISImaxNULRB=PtxTDDNRBTDD10Desence (dB)10-1*10-174+NF(dB)10*12*SCS

- For BS transmit power for SBFD Option-2 (Optional): with power boosting

  - Power boosting is assumed for SBFD symbols compared to DL-only symbols, i.e.,

    - DL symbols in SBFD operation have the same PSD as used in TDD DL symbols

    - For SBFD symbols, its PSD is scaled according to the number of RBs in DL subband(s)

  - In this case,

Ptxmax=PtxTDD

αSI=PtxmaxISImax*NDLRBNULRB=PtxmaxNDLRBISImaxNULRB=PtxTDDNDLRB10Desence (dB)10-1*10-174+NF(dB)10*12*SCS

The agreed NDLRB, NULRB, NF (noise figure) and BS transmit power for legacy TDD are shown below

| Parameters | FR1 | FR2 with 200MHz |
| --- | --- | --- |
| NDLRB | 104 | 52 |
| NULRB | 55 | 26 |
| NG | 5 | 1 |
| Total RB number for legacy TDD | 273 | 132 |
| SCS (kHz) | 30 | 120 |
| BS receiver noise figure (dB) | 5 | 7 |

|  | FR1 | FR2-1 |
| --- | --- | --- |
| Urban macro | - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz | - N.A. |
| Dense Urban Macro layer | - Option 1: 53 dBm for 100MHz - Option 3: 44 dBm for 100MHz | - 43dBm for 200MHz |
| Dense Urban Micro layer | - 38 dBm for 100MHz | - 33dBm for 200MHz. EIRP should not exceed 68 dBm. |
| Indoor hotspot | - 24 dBm for 100MHz | - 23 dBm for 200MHz. EIRP should not exceed 58 dBm. |

Assume 1dB UL receiver sensitivity degradation and SBFD antenna configuration Option-2, the candidate values for αSI can be derived as below.

Proposal 7: For SLS of SBFD, with the assumption of 1dB UL receiver sensitivity degradation and SBFD antenna configuration Option-2, consider the below candidate values for RSI αSI.

|  | no power boosting (BS transmit power for SBFD Option-1) |  |  |  | with power boosting (BS transmit power for SBFD Option-2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FR1 (100 MHz), <104, 55, 5>, 30KHz |  | FR2-1(200 MHz), <52,26,1>, 120KHz |  | FR1 (100 MHz), <104, 55, 5>, 30KHz |  | FR2-1(200 MHz), <52,26,1>, 120KHz |  |
|  | Tx Power (dBm) | αSI (dB) | Tx Power (dBm) | αSI (dB) | Tx Power (dBm) | αSI (dB) | Tx Power (dBm) | αSI (dB) |
| Urban macro | 53 | 147.9 |  |  | 53 | 149.1 |  |  |
|  | 49 | 143.9 |  |  | 49 | 145.1 |  |  |
| Dense Urban Macro layer | 53 | 147.9 | 43 | 133.1 | 53 | 149.1 | 43 | 134.1 |
|  | 44 | 138.9 |  |  | 44 | 140.1 |  |  |
| Dense Urban Micro layer | 38 | 132.9 | 33 | 123.1 | 38 | 134.1 | 33 | 124.1 |
| Indoor hotspot | 24 | 118.9 | 23 | 113.1 | 24 | 120.1 | 23 | 114.1 |

Co-site inter-sector co-channel inter-subband CLI

Observation 2: The following agreement related to the modelling of co-site inter-sector co-channel inter-subband CLI was confirmed by RAN4 according to R1-2302262.

[RAN1#111] Agreement For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling as follows. Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB - Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB. - Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale). - NDLRB is the total number of DL RBs in the DL subbands. - Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x. - αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI. - 10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+… - Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment. - Companies shall report the value of αco-site assumed in the simulations with feasibility of how these values were derived. - Send LS to RAN4 confirming the model and asking the value ranges for spatial isolation, and values of 10*log10ACLRBS and 10*log10ACSBS.

The value ranges for spatial isolation proposed by RAN4 according to R1-2302262 has been taken into account in the below working assumption made in RAN1#112.

Proposal 8: Confirm the below working assumption related to modelling of co-site inter-sector co-channel inter-subband CLI.

[RAN1#112] Working Assumption: For co-site inter-sector co-channel inter-subband CLI modelling, before receiving RAN4’s reply on the value of αco-site, RAN1 assume the following only for evaluation: - FR1: - 75dB for spatial isolation (RAN4 typical value). - 93dB for spatial isolation (RAN4 best value). - 100dB for spatial isolation - FR2: - 88dB for spatial isolation (RAN4 typical value). - 98dB for spatial isolation (RAN4 best value). - 105dB for spatial isolation - In addition to spatial isolation and frequency isolation, companies can use digital cancelation and report the value, e,g., 10dB. Above does not imply that RAN1 assumes or does not assume digital cancelation is feasible. - The feasibility of these values is up to RAN4. These values can be revisited based on further RAN4 inputs. - The 100dB/105dB isolation values for FR1 and FR2 are not from RAN4, but based on RAN4 input that some companies have proposed that isolating material could be added between sectors to increase the isolation. RAN4 has not yet discussed the details whether such approaches can be applied to outdoor sites.

The values of ACLRBS and ACSBS proposed by RAN4 according to R1-2302262 have been agreed in RAN1#112.

[RAN1#112] Agreement For SLS of SBFD, use the following values for BS ACLR/ACS (ACLRBS and ACSBS). FR1; FR2-1. BS ACLR; 45 dB; 28 dB. BS ACS; 46 dB; 23.5 dB.

Inter-site gNB-gNB co-channel inter-subband CLI

Observation 3: The following agreements related to the modelling of inter-site gNB-gNB co-channel inter-subband CLI were confirmed by RAN4 according to R1-2300025.

[RAN1#110b] Agreement For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for gNB-gNB co-channel channel model, the power of inter-site gNB-gNB co-channel inter-subband CLI experienced by the victim gNB on each receiver chain at one UL RB can be modelled as - IInter-Site-CLIA'→A,per-RB=PtxBS A',per-RB*Nused-DL-RBBS A'*CLBS A'→BS A*1ACLRBS+1ACSBS* 1NDLRB - IInter-Site-CLIA'→A,per-RB is the power of inter-site gNB-gNB co-channel inter-subband CLI from gNB A' to gNB A on each receiver chain at one UL RB (linear value) - PtxBS A',per-RB is DL transmission power of gNB A' across all transmit chains per RB (linear value). PtxBS A',per-RB=Ptxmax/NDLRB. - Nused-DL-RBBS A' is the number of DL RBs allocated for DL transmission by gNB A' - CLBS A'→BS A is the coupling loss between gNB A' and gNB A (linear value), accounting for beamforming at the aggressor gNB and victim gNB. - FFS: the detailed definition of the coupling loss, which can be discussed later - NDLRB is the total number of DL RBs in the DL subbands - Note: ACLRBS and ACSBS are in linear scale. In RAN4 reply LS, gNB ACLR (i.e., ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment. - Note: the model is based on the assumption that the same transmission power across different DL RBs is used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS. - Note: This model is not applicable to the RBs in the guardband. - Note: This model is not applicable for some candidate gNB-gNB CLI handling schemes (for example, spatial digital beam coordination, advanced receivers) - Send LS to RAN4 to confirm RAN1’s understanding Agreement For SLS in RAN1, if both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the inter-site gNB-gNB co-channel inter-subband CLI signal across all Rx chains at UL RB n at victim gNB can be modeled as IInter-Site-CLIn=Ileakagen+Iselectivity, where, - Ileakagen=HCLInWyn is the first part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at UL RB n, caused by power leakage at aggressor gNB, - HCLIn is the NR×NT channel matrix between aggressor gNB and victim gNB at UL RB n, the beamforming of the aggressor gNB and the victim gNB can be taken into account by HCLIn, - yn=y0n,y1n,…,yNT-1nT is the unwanted emission across all Tx chains at UL RB n at aggressor gNB, - NT is the number of Tx chains at aggressor gNB, - ykn~N0,σy,n2, k=0,1,…,NT-1, is modelled as white Gaussian noise, - σy,n2=Ptxper-RB*Nused-DL-RBACLRBS* 1NDLRB is the total leakage power at UL RB n at aggressor gNB, - Ptxper-RB is the DL power transmitted across all Tx chains at one DL RB at aggressor gNB, Ptxper-RB=Ptxmax/NDLRB, - Nused-DL-RB is the number of DL RBs scheduled for DL transmission by aggressor gNB, - NDLRB is the total number of DL RBs in the DL subbands - W is the NT×NT normalized identity matrix with unit norm, WF=1, - FFS whether W can be other values and corresponding conditions - FFS for Iselectivity - Note: ACLRBS and ACSBS are in linear scale. In RAN4 reply LS, gNB ACLR (i.e., ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment. - Note: the model is based on the assumption that the same transmission power across different DL RBs are used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS. - Note: This model is not applicable to the RBs in the guardband. - Send LS to RAN4 to confirm RAN1’s understanding.

Observation 4: Regarding the below agreement related to the modelling of inter-site gNB-gNB co-channel inter-subband CLI, RAN1’s assumption on ICSBS (in channel selectivity) to be given by the value of gNB ACS was confirmed by RAN4 according to R1-2302262.

[RAN1#111] Agreement Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB, can be modelled as Iselectivity=z0n,z1n,…,zNR-1nT - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise - σz,n2=PblockerICSBS×NDLRB - Pblocker=1NR*m∈Used DL RBsHCLImWmsm2 - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm, - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1, - sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB. - NDLRB is the total number of DL RBs in the DL subbands, - RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS unless further RAN4 guidance is received. - Send LS to RAN4 to confirm RAN1 understanding and check whether ICSBS can be modelled depending on the value of the blocker interference, e.g., ICSBS= ICS1 Pblocker<P1 ICS2, P1< Pblcoker<P2ICS3 Pblocker>P2 - Note: Pblocker can be reported by companies

Nevertheless, RAN4 suggests the following noise figure model to be used for simulation purpose. Although it was not mentioned whether the following values (i.e., A/B/C/D) can be used for only FR1 or both FR1 and FR2, it is our understanding that the following values should be used for only FR1 since C=5dB is what we usually assumed for FR1.

[R1-2302262] Answer from RAN4: From RAN4 perspective, the following model is provided for simulation purpose: - The noise figure model is provided as below: - X-axis: Total received power is the linear sum of all received power, including wanted signal, self-interference, inter-gNB interference and inter-sector interference. - Y-axis: noise figure - The values of A, B, C and D: - A = -43dBm - B = -25dBm - C = 5dB - D = 14dB - If the total received power is larger than B, the receiver will be blocked.

Proposal 9: Update the agreement in RAN1#111 as below:

Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB, can be modelled as

Iselectivity=z0n,z1n,…,zNR-1nT

  - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise

  - σz,n2=PblockerICSBS×NDLRB

  - Pblocker=1NR*m∈Used DL RBsHCLImWmsm2

  - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm,

  - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1,

  - sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB.

  - NDLRB is the total number of DL RBs in the DL subbands,

  - RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS unless further RAN4 guidance is received.

    - Send LS to RAN4 to confirm RAN1 understanding and check whether ICSBS can be modelled depending on the value of the blocker interference, e.g.,

ICSBS= ICS1            Pblocker<P1 ICS2,   P1< Pblcoker<P2ICS3             Pblocker>P2

  - Note: Pblocker can be reported by companies.

Proposal 10: For SLS in RAN1, the BS noise figure is modelled as below

    - X-axis: Total received power is the linear sum of all received power, including wanted signal, legacy UE-gNB interference, self-interference, inter-gNB interference and inter-sector interference.

    - Y-axis: noise figure

    - The values of A, B, C and D:

      - A = -43dBm for FR1, FFS for FR2-1

      - B = -25dBm for FR1, FFS for FR2-1

      - C = 5dB for FR1, FFS for FR2-1

      - D = 14dB for FR1, FFS for FR2-1

    - If the total received power is larger than B, the receiver will be blocked.

UE-UE co-channel inter-subband CLI

Observation 5: Regarding the modelling of UE-UE co-channel inter-subband CLI, according to R1-2302262, RAN4 also adopts IBE-based model, and RAN4 has not reached the agreement for an equivalent frequency flat model.

[RAN1#111] Agreement For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, Option 1 is used as starting point. - Option 1: RAN1 to take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point. - Send LS to RAN4 to ask them whether it can be modelled as an equivalent frequency flat model (e.g., IBEUE,ave) based on RAN4 IBE requirement, and if possible, what is the value of IBEUE,ave

Observation 6: The following agreements and working assumptions related to the modelling of UE-UE co-channel inter-subband CLI still wait for RAN4’s confirm.

[RAN1#112] Agreement For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE co-channel channel model, the power of UE-UE co-channel inter-subband CLI experienced by the victim UE on each receiver chain at DL RB n can be modelled as IUE-UE-CLIB'→B,per-RBn=IUE-UE,IBEn*CLUE B'→UE B+PtxUE B'*CLUE B'→UE B*1ICSUE*1NULRB where - IUE-UE-CLIB'→B,per-RBn is the power of UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B on each receiver chain at one DL RB n (linear value). - PtxUE B' is UL transmission power of UE B' across all transmit chains over the allocated UL RBs (linear value) - CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE - NULRB is the total number of UL RBs in the UL subband - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input. - 10*log10IUE-UE,IBEn=maxPtxUE B',per-RB-30dB,10*log10IGeneral+IIQfor FR1maxPtxUE B',per-RB-25dB,10*log10IGeneral+IIQfor FR2-1, wherein, - For SBFD Subband configuration with {DUD} pattern, IIQ can be ignored - 10*log10IGeneral=PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-3-5⋅ΔRB-1LCRB,-57dBm+10log10SCS/15kHz-PtxUE B',per-RBfor FR1PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-5⋅ΔRB-1LCRB,-55.1dBm-PtxUE B',per-RBfor FR2-1 - PtxUE B',per-RB is UL transmission power of UE B' across all transmit chains per RB (linear value). PtxUE B',per-RB=PtxUE B'/ LCRB, and LCRB is the number of UL RBs allocated for UL transmission of UE B'. - NRB is the Transmission Bandwidth Configuration, referring to Table 5.3.2-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1. - NRB=273 for FR1 with 100MHz transmission bandwidth and 30kHz SCS - NRB=132 for FR2-1 with 200MHz transmission bandwidth and 120kHz SCS - ΔRB is the starting frequency offset between the allocated UL RBs and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated UL RBs) - EVM is the limit specified in Table 6.4.2.1-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1 for the modulation format used in the allocated RBs. Include the above in the LS to RAN4 to inform them of the agreement and to check if the RAN1 agreement is in line with RAN4’s understanding. Working assumption: For SLS in RAN1, if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as: IInter-UE-CLIn=Ileakagen+Iselectivity, where, - Ileakagen=HCLIn W yn is the first part of UE-UE co-channel inter-subband CLI across all Rx chains at DL RB n, caused by power leakage at aggressor UE, - HCLIn is the NR×NT channel matrix between aggressor UE and victim UE at DL RB n, the beamforming of the aggressor UE and the victim UE can be taken into account by HCLIn - NR is the number of Rx chains and NT is the number of Tx chains - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1. - yn=y0n,y1n,…,yRnT, - ykn~N0,IUE-UE,IBEn, k=0,1,…,R-1, is modelled as white Gaussian noise - IUE-UE,IBEn has the same meaning as in the agreement for the case only large-scale fading is modelled - Iselectivity is modelled as frequency flat Iselectivity=z0n,z1n,…,zNR-1nT - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise, σz,n2=PblockerICSUE×NULRB - Pblocker=1NR*m∈Used UL RBsHCLImWsm2 - HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim gNB can be taken into account by HCLIm, - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1 - sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE B',per-RB. - PtxUE B',per-RB has the same meaning as in the agreement for the case only large-scale fading is modelled - NULRB is the total number of UL RBs in the UL subbands, - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input. Include the above in the LS to RAN4 to inform them of the agreement and to check if the RAN1 agreement is in line with RAN4’s understanding.

Proposal 11: Confirm the below working assumption related to the modelling of UE-UE co-channel inter-subband CLI.

Working assumption:

For SLS in RAN1, if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as:

IInter-UE-CLIn=Ileakagen+Iselectivity, where,

- Ileakagen=HCLIn W yn is the first part of UE-UE co-channel inter-subband CLI across all Rx chains at DL RB n, caused by power leakage at aggressor UE,

  - HCLIn is the NR×NT channel matrix between aggressor UE and victim UE at DL RB n, the beamforming of the aggressor UE and the victim UE can be taken into account by HCLIn

  - NR is the number of Rx chains and NT is the number of Tx chains

  - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1.

  - yn=y0n,y1n,…,yRnT,

    - ykn~N0,IUE-UE,IBEn, k=0,1,…,R-1, is modelled as white Gaussian noise

    - IUE-UE,IBEn has the same meaning as in the agreement for the case only large-scale fading is modelled

- Iselectivity is modelled as frequency flat

Iselectivity=z0n,z1n,…,zNR-1nT

  - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise, σz,n2=PblockerICSUE×NULRB

  - Pblocker=1NR*m∈Used UL RBsHCLImWsm2

  - HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim gNB can be taken into account by HCLIm,

  - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1

  - sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE B',per-RB.

    - PtxUE B',per-RB has the same meaning as in the agreement for the case only large-scale fading is modelled

  - NULRB is the total number of UL RBs in the UL subbands,

  - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input.

Co-site gNB-gNB adjacent-channel CLI

Observation 7: Regarding the below agreement related to the modelling of co-site gNB-gNB adjacent-channel CLI, the value of αadj, co-site-x still wait for RAN4’s inputs.

[RAN1#112] Agreement For SLS in RAN1, for co-site gNB-gNB adjacent-channel CLI modelling, reuse similar method as co-site inter-sector co-channel inter-subband CLI modeling as follows. Iadj, co-siteper-RB=Iadj, co-site-sector-1per-RB+Iadj, co-site-sector-2per-RB+Iadj, co-site-sector-3per-RB Iadj,co-site-sector-xper-RB=Ptxadj,sector-x, per-RBαadj,co-site-x*Nused-DL-RBadj, sector-xNDLRBadj - Ptxadj, sector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxadj,sector-x, per-RB=Ptxadj,sector-x, max/NDLRBadj. - Ptxadj,sector-x, max is the maximum DL Tx Power of sector x in adjacent channel (in linear scale). - NDLRBadj is the total number of DL RBs in adjacent channel. - Nused-DL-RBadj,sector-x is the number of DL RBs allocated for DL transmission of sector x in adjacent channel. - αadj, co-site-x is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI between the aggressor sector x and the victim sector. - 10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS+… - FFS the concrete value of αadj, co-site-x - ACLRBS and ACSBS are in linear scale. Send an LS to RAN4 to inquire on the value of αadj, co-site-x.

SBFD subband and slot configurations

Proposal 12: For SBFD evaluation, deprioritize SBFD subband configuration#2 with {DU} pattern.

Initial SLS evaluation results for SBFD

Indoor Office (FR1)

Evaluation assumptions

The evaluation assumption for Indoor Office (FR1) is given in Table 3.

Table 3  Evaluation assumptions for Indoor Office (FR1).

|  | Parameters | CMCC |
| --- | --- | --- |
| Interference Modelling | gNB self-interference - αSI | (Baseline) Based on 1 dB UL desense |
|  | UE-UE co-channel inter-subband CLI | 33 dBc |
| SBFD subband and slot configuration | SBFD slot configuration | - Alt 2 (higher priority): Legacy TDD: {DDDSU}; SBFD: {XXXXU} - Alt 4 (higher priority): Legacy TDD: {DDDSU}; SBFD: {XXXXX} |
|  | SBFD Subband configuration | Option 1 (Baseline): <ND, NU, NG >=<104, 55, 5> |
|  | Guard symbols | 2 symbols at the last X slot for Alt 2 {XXXXU} |
|  | UL/DL resource percentage per TDD period | Legacy TDD: DL: 77.1%, UL: 20% Alt 2 {XXXXU} with 2 guard symbols: DL: 58.8% (-23.8% loss over legacy TDD), UL: 35.5% (77.7% gain over legacy TDD) Alt 4 {XXXXX}: DL: 76.2% (-1.2% loss over legacy TDD), UL: 20.1% (0.7% gain over legacy TDD) |
| BS transmit power & antenna configuration | BS transmit power for SBFD | (Baseline): Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems) |
|  | BS antenna configuration for legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (4,4,2,1,1; 4,4) , (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization |
|  | BS antenna configuration for SBFD | Twice area&same TxRUs (higher priority): SBFD antenna configuration Option 2 |
|  | BS antenna radiation pattern | (Baseline): the ceiling-mount antenna pattern in Table 10 in Report ITU-R M.2412 |
|  | UE antenna configuration | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization |
| Traffic Model | DL/UL traffic assignment for the same UE | Option 2: Each UE is assigned both UL traffic and DL traffic. |
|  | DL/UL FTP packet size | (higher priority): 0.5Mbyte for DL and 0.125 Mbytes for UL |
|  | DL/UL traffic load for legacy TDD | {DL:UL}={Low, Low} / {Medium, Medium} / {High, High} |
| Channel model | gNB-gNB | (higher priority): Both Large scale fading and small scale fading |
|  | UE-UE | Large scale fading only |
|  | UE-UE details | (baseline): TR 38.901 |
| Others | Open loop power control parameters | P0= -60 dBm, alpha = 0.6 |
|  | UE receiver | (Baseline): MMSE-IRC |
|  | Channel estimation | Ideal |
|  | Transmission scheme | SU-MIMO for both UL and DL |

Evaluation results

For Indoor Office (FR1) in SBFD Deployment Case 1, assuming gNB self-interference isolation based on 1 dB UL desense, SBFD antenna configuration Option 2, 0.5Mbyte FTP packet size for DL and 0.125 Mbytes for UL, both SBFD slot configuration Alt 2 and Alt 4 are evaluated with evaluation results summarized in Table 4.

Table 4  Evaluation results for Indoor Office (FR1) in SBFD Deployment Case 1

| R1-2303232 / CMCC | Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} with 2 guard symbols (UL/DL resource percentage in a TDD period = {20.0%, 77.1%} for TDD, {35.5%, 58.8%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
|  | DL Average-UPT CDF (Mbps) | Mean | 225.72 | 180.64 | -19.97% | 194.29 | 148.29 | -23.68% |  |  |  |
|  |  | 5% | 212.57 | 168.12 | -20.91% | 165.76 | 125.76 | -24.13% |  |  |  |
|  |  | 50% | 226.88 | 182.07 | -19.75% | 194.66 | 148.92 | -23.50% |  |  |  |
|  |  | 95% | 233.24 | 188.07 | -19.37% | 217.82 | 168.2 | -22.78% |  |  |  |
|  | UL Average-UPT CDF (Mbps) | Mean | 37.11 | 67.51 | 81.92% | 34.43 | 64.85 | 88.35% |  |  |  |
|  |  | 5% | 29.94 | 59.94 | 100.20% | 26.41 | 56.47 | 113.82% |  |  |  |
|  |  | 50% | 37.99 | 68.28 | 79.73% | 34.91 | 65.51 | 87.65% |  |  |  |
|  |  | 95% | 42.19 | 71.71 | 69.97% | 40.07 | 69.74 | 74.05% |  |  |  |
|  | DL Packet-Latency CDF (ms) | Mean | 9.6 | 11.8 | 22.92% | 12.7 | 16.9 | 33.07% |  |  |  |
|  |  | 5% | 8.5 | 10.5 | 23.53% | 8.5 | 10.5 | 23.53% |  |  |  |
|  |  | 50% | 8.5 | 10.5 | 23.53% | 9 | 12.5 | 38.89% |  |  |  |
|  |  | 95% | 13.5 | 18 | 33.33% | 25 | 33.5 | 34.00% |  |  |  |
|  | UL Packet-Latency CDF (ms) | Mean | 16.5 | 8.7 | -47.27% | 18.9 | 9.4 | -50.26% |  |  |  |
|  |  | 5% | 11.5 | 7 | -39.13% | 11.5 | 7 | -39.13% |  |  |  |
|  |  | 50% | 14 | 8 | -42.86% | 15.5 | 8 | -48.39% |  |  |  |
|  |  | 95% | 25.5 | 12 | -52.94% | 33 | 14.5 | -56.06% |  |  |  |
|  | Type-1 RU (%) | DL | 3.43% | 3.43% | 0.00% | 14.23% | 14.60% | 2.60% |  |  |  |
|  |  | UL | 1.13% | 1.01% | -10.62% | 4.08% | 3.63% | -11.03% |  |  |  |
|  | Type-2 RU (%) | DL | 4.31% | 5.47% | 26.91% | 17.92% | 23.25% | 29.74% |  |  |  |
|  |  | UL | 5.47% | 2.70% | -50.64% | 19.82% | 9.77% | -50.71% |  |  |  |
|  | Unfinished/dropped Packet Rate (%) | DL | 0.03% | 0.08% | 166.67% | 0.08% | 0.26% | 225.00% |  |  |  |
|  |  | UL | 0.07% | 0.09% | 28.57% | 0.19% | 0.24% | 26.32% |  |  |  |
|  | Additional comments: Layout and UE distribution - Layout: Indoor floor: (12BSs per 120m x 50m) - UE distribution: Uniform UE distribution, 10 users per TRP per direction Interference Modelling - gNB self-interference: based on 1 dB UL desense - UE-UE co-channel inter-subband CLI: 33 dBc SBFD subband and slot configuration - SBFD slot configuration: Alt 2 (higher priority): Legacy TDD: {DDDSU}; SBFD: {XXXXU} - SBFD Subband configuration: <ND, NU, NG >=<104, 55, 5> - Guard symbol number: 2 - UL resource percentage per TDD period (%): 35.5% - DL resource percentage per TDD period (%): 58.8% BS transmit power & antenna configuration - BS transmit power for legacy TDD: 24dBm - BS transmit power for SBFD: Option-1: Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems) - BS antenna configuration for legacy TDD: (M,N,P,Mg,Ng;Mp,Np) = (4,4,2,1,1;4,4) , (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization - BS antenna configuration for SBFD: Twice area&same TxRUs (higher priority): SBFD antenna configuration Option 2 - BS antenna radiation pattern: Table 9 in Report ITU-R M.2412 - UE antenna configuration: 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization Traffic Model - DL/UL traffic assignment for the same UE: Option 2: Each UE is assigned both UL traffic and DL traffic - DL/UL FTP packet size: 0.5Mbytes for DL and 0.125Mbyte for UL Channel model - gNB-gNB: Both Large scale fading and small scale fading - UE-UE: Large scale fading only - UE-UE details: TR 38.901 Others - Open loop power control parameters: P0= -60 dBm, alpha = 0.6 - UE receiver: MMSE-IRC - Channel estimation: Ideal - Transmission scheme: SU-MIMO - Overhead: |  |  |  |  |  |  |  |  |  |  |
| R1-2303232 / CMCC | Reported Parameters |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} (UL/DL resource percentage in a TDD period = {20.0%, 77.1%} for TDD, {20.1%, 76.2%} for SBFD) |  |  |  |  |  |  |  |  |
|  |  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
|  | DL Average-UPT CDF (Mbps) | Mean | 225.72 | 227.77 | 0.91% | 194.29 | 195.1 | 0.42% |  |  |  |
|  |  | 5% | 212.57 | 213.96 | 0.65% | 165.76 | 167.47 | 1.03% |  |  |  |
|  |  | 50% | 226.88 | 229.16 | 1.00% | 194.66 | 193.74 | -0.47% |  |  |  |
|  |  | 95% | 233.24 | 235.54 | 0.99% | 217.82 | 217.27 | -0.25% |  |  |  |
|  | UL Average-UPT CDF (Mbps) | Mean | 37.11 | 40.22 | 8.38% | 34.43 | 37.81 | 9.82% |  |  |  |
|  |  | 5% | 29.94 | 37.01 | 23.61% | 26.41 | 33.46 | 26.69% |  |  |  |
|  |  | 50% | 37.99 | 40.4 | 6.34% | 34.91 | 38.07 | 9.05% |  |  |  |
|  |  | 95% | 42.19 | 41.93 | -0.62% | 40.07 | 40.38 | 0.77% |  |  |  |
|  | DL Packet-Latency CDF (ms) | Mean | 9.6 | 9.3 | -3.12% | 12.7 | 12.4 | -2.36% |  |  |  |
|  |  | 5% | 8.5 | 8.5 | 0.00% | 8.5 | 8.5 | 0.00% |  |  |  |
|  |  | 50% | 8.5 | 8.5 | 0.00% | 9 | 9 | 0.00% |  |  |  |
|  |  | 95% | 13.5 | 13 | -3.70% | 25 | 23.5 | -6.00% |  |  |  |
|  | UL Packet-Latency CDF (ms) | Mean | 16.5 | 14.3 | -13.33% | 18.9 | 16 | -15.34% |  |  |  |
|  |  | 5% | 11.5 | 13 | 13.04% | 11.5 | 13 | 13.04% |  |  |  |
|  |  | 50% | 14 | 13 | -7.14% | 15.5 | 13 | -16.13% |  |  |  |
|  |  | 95% | 25.5 | 19 | -25.49% | 33 | 25 | -24.24% |  |  |  |
|  | Type-1 RU (%) | DL | 3.43% | 3.43% | 0.00% | 14.23% | 14.26% | 0.21% |  |  |  |
|  |  | UL | 1.13% | 0.94% | -16.81% | 4.08% | 3.39% | -16.91% |  |  |  |
|  | Type-2 RU (%) | DL | 4.31% | 4.34% | 0.70% | 17.92% | 18.03% | 0.61% |  |  |  |
|  |  | UL | 5.47% | 4.47% | -18.28% | 19.82% | 16.19% | -18.31% |  |  |  |
|  | Unfinished/dropped Packet Rate (%) | DL | 0.03% | 0.02% | -33.33% | 0.08% | 0.14% | 75.00% |  |  |  |
|  |  | UL | 0.07% | 0.17% | 142.86% | 0.19% | 0.76% | 300.00% |  |  |  |
|  | Additional comments: Layout and UE distribution - Layout: Indoor floor: (12BSs per 120m x 50m) - UE distribution: Uniform UE distribution, 10 users per TRP per direction Interference Modelling - gNB self-interference: based on 1 dB UL desense - UE-UE co-channel inter-subband CLI: 33 dBc SBFD subband and slot configuration - SBFD slot configuration: Alt 4 (higher priority): Legacy TDD: {DDDSU}; SBFD: {XXXXX} - SBFD Subband configuration: <ND, NU, NG >=<104, 55, 5> - Guard symbol number: 0 - UL resource percentage per TDD period (%): 20.1% - DL resource percentage per TDD period (%): 76.2% BS transmit power & antenna configuration - BS transmit power for legacy TDD: 24dBm - BS transmit power for SBFD: Option-1: Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems) - BS antenna configuration for legacy TDD: (M,N,P,Mg,Ng;Mp,Np) = (4,4,2,1,1;4,4) , (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization - BS antenna configuration for SBFD: Twice area&same TxRUs (higher priority): SBFD antenna configuration Option 2 - BS antenna radiation pattern: Table 9 in Report ITU-R M.2412 - UE antenna configuration: 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization Traffic Model - DL/UL traffic assignment for the same UE: Option 2: Each UE is assigned both UL traffic and DL traffic - DL/UL FTP packet size: 0.5Mbytes for DL and 0.125Mbyte for UL Channel model - gNB-gNB: Both Large scale fading and small scale fading - UE-UE: Large scale fading only - UE-UE details: TR 38.901 Others - Open loop power control parameters: P0= -60 dBm, alpha = 0.6 - UE receiver: MMSE-IRC - Channel estimation: Ideal - Transmission scheme: SU-MIMO - Overhead: |  |  |  |  |  |  |  |  |  |  |

Observation 8: For Indoor Office (FR1) in SBFD Deployment Case 1, assuming gNB self-interference isolation based on 1 dB UL desense, SBFD antenna configuration Option 2, 0.5Mbyte FTP packet size for DL and 0.125 Mbytes for UL,

- For SBFD slot configuration Alt 2: {DDDSU} vs. {XXXXU} with 2 guard symbols,

  - For UL/DL resource percentage per TDD period

    - The DL resource percentage per TDD period is decreased by around 23.8%

    - The UL resource percentage per TDD period is increased by around 77.7%

  - For {DL : UL} traffic load for legacy TDD = {Low : Low}

    - DL performance of SBFD compared with legacy TDD

      - The mean value of DL average-UPT CDF of SBFD is decreased by around 19.97%

      - The 5% of DL average-UPT CDF of SBFD is decreased by around 20.91%

      - The mean value of DL packet-latency CDF of SBFD is increased by around 22.92%

      - The 5% of DL packet-latency CDF of SBFD is increased by around 23.53%

      - The DL Type-1 RU of SBFD almost keeps unchanged

      - The DL Type-2 RU of SBFD is increased by around 26.91%

    - UL performance of SBFD compared with legacy TDD

      - The mean value of UL average-UPT CDF of SBFD is increased by around 81.92%

      - The 5% of UL average-UPT CDF of SBFD is increased by around 100.20%

      - The mean value of UL packet-latency CDF of SBFD is decreased by around 47.27%

      - The 5% of UL packet-latency CDF of SBFD is decreased by around 39.13%

      - The UL Type-1 RU of SBFD is decreased by around 10.62%

      - The UL Type-2 RU of SBFD is decreased by around 50.64%

  - For {DL : UL} traffic load for legacy TDD = {Medium : Medium}

    - DL performance of SBFD compared with legacy TDD

      - The mean value of DL average-UPT CDF of SBFD is decreased by around 23.68%

      - The 5% of DL average-UPT CDF of SBFD is decreased by around 24.13%

      - The mean value of DL packet-latency CDF of SBFD is increased by around 33.07%

      - The 5% of DL packet-latency CDF of SBFD is increased by around 23.53%

      - The DL Type-1 RU of SBFDs increased by around 2.60%

      - The DL Type-2 RU of SBFD is increased by around 29.74%

    - UL performance of SBFD compared with legacy TDD

      - The mean value of UL average-UPT CDF of SBFD is increased by around 88.35%

      - The 5% of UL average-UPT CDF of SBFD is increased by around 113.82%

      - The mean value of UL packet-latency CDF of SBFD is decreased by around 50.26%

      - The 5% of UL packet-latency CDF of SBFD is decreased by around 39.13%

      - The UL Type-1 RU of SBFD is decreased by around 11.03%

      - The UL Type-2 RU of SBFD is decreased by around 50.71%

- For SBFD slot configuration Alt 4: {DDDSU} vs. {XXXXX}

  - For UL/DL resource percentage per TDD period

    - The DL resource percentage per TDD period is decreased by around 1.2%

    - The UL resource percentage per TDD period is increased by around 0.7%

  - For {DL : UL} traffic load for legacy TDD = {Low : Low}

    - DL performance of SBFD compared with legacy TDD

      - The mean value of DL average-UPT CDF of SBFD is increased by around 0.91%

      - The 5% of DL average-UPT CDF of SBFD is increased by around 0.65%

      - The mean value of DL packet-latency CDF of SBFD is decreased by around 3.12%

      - The 5% of DL packet-latency CDF of SBFD almost keeps unchanged

      - The DL Type-1 RU of SBFD almost keeps unchanged

      - The DL Type-2 RU of SBFD is increased by around 0.70%

    - UL performance of SBFD compared with legacy TDD

      - The mean value of UL average-UPT CDF of SBFD is increased by around 8.38%

      - The 5% of UL average-UPT CDF of SBFD is increased by around 23.61%

      - The mean value of UL packet-latency CDF of SBFD is decreased by around 13.33%

      - The 5% of UL packet-latency CDF of SBFD is increased by around 13.04%

      - The UL Type-1 RU of SBFD is decreased by around 16.81%

      - The UL Type-2 RU of SBFD is decreased by around 18.28%

  - For {DL : UL} traffic load for legacy TDD = {Medium : Medium}

    - DL performance of SBFD compared with legacy TDD

      - The mean value of DL average-UPT CDF of SBFD is increased by around 0.42%

      - The 5% of DL average-UPT CDF of SBFD is increased by around 1.03%

      - The mean value of DL packet-latency CDF of SBFD is decreased by around 2.69%

      - The 5% of DL packet-latency CDF of SBFD almost keeps unchanged

      - The DL Type-1 RU of SBFD is increased by around 0.21%

      - The DL Type-2 RU of SBFD is increased by around 0.61%

    - UL performance of SBFD compared with legacy TDD

      - The mean value of UL average-UPT CDF of SBFD is increased by around 9.82%

      - The 5% of UL average-UPT CDF of SBFD is increased by around 26.69%

      - The mean value of UL packet-latency CDF of SBFD is decreased by around 15.34%

      - The 5% of UL packet-latency CDF of SBFD is increased by around 13.04%

      - The UL Type-1 RU of SBFD is decreased by around 16.91%

      - The UL Type-2 RU of SBFD is decreased by around 18.31%

Remaining issues for LLS

Coverage performance evaluation methodology

In RAN1#112, the following high level agreements were achieved for link level evaluation of coverage performance.

Agreement For link level evaluation of coverage performance, focus on Urban Macro scenario for FR1 and Dense Urban Macro Layer scenario for FR2-1. Agreement For link level evaluation of coverage performance, focus on the following uplink channels. - PUSCH with 1Mbps target data rate for FR1 - PUSCH with 5Mbps target data rate for FR2-1 - FFS: PUCCH - Note: the data rate is based on TR38.830 Agreement For link level evaluation of coverage performance for SBFD, the following interference components are added per each receive chain to the UL channel at SBFD symbols: - Self-interference, modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense similar to SLS. - Co-site inter-sector interference, modelled as additive white gaussian noise with fixed INR = - X dB based on assumption of co-site isolation αco-site - Inter-site gNB-gNB co-channel inter-subband CLI, - Alt-1: the value of interference power is selected according to the INR distribution drawn based on the statistics from SLS. - Alt-2: the value of interference power is determined based on the inter-site gNB-gNB co-channel inter-subband CLI model agreed for SLS taking into account the locations of victim gNB and several aggressor gNBs, and the gNB-gNB channel model - FFS: Receiver blocking model Agreement For link level evaluation of coverage performance, use Alt 2 defined in SLS. - Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth. Agreement Regarding the schemes for link level evaluation of PUSCH coverage performance, - For baseline legacy TDD, consider - Single slot PUSCH transmission - For SBFD, consider the following techniques of coverage enhancement: - Case 2: SBFD with PUSCH repetition type A - Case 3: SBFD with TBoMS PUSCH - Case 4: SBFD with PUSCH repetition type A and joint channel estimation - FFS: Joint channel estimation across SBFD and non-SBFD slots - Case 5: SBFD with TBoMS PUSCH and joint channel estimation - FFS: Joint channel estimation across SBFD and non-SBFD slots - UL coverage metrics are obtained using link budget template and TDD/SBFD required SINR for target data rate. Note: Evaluation accounts for different SINR level between SBFD and non-SBFD slots

However, the detailed assumptions for LLS and Link budget analysis haven’t been agreed.

Proposal 13: For coverage performance evaluation for SBFD, consider the following simulation assumptions for LLS.

Table 5  Simulation assumption for coverage performance evaluation for SBFD for LLS.

| Parameter | FR1 | FR2-1 |
| --- | --- | --- |
| Scenario and frequency | Urban Macro (ISD = 500m): 4GHz | Dense Urban Macro Layer (ISD = 200m): 30GHz |
| SBFD subband and slot configurations | Legacy TDD: DDDSU, S=[12D:2G:0U] SBFD: XXXXU, where X denotes SBFD slot. For SBFD slot, {DUD} pattern is assumed, For FR1, 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <104, 55, 5> For FR2-1, 200MHz channel bandwidth and 120kHz SCS (132 PRB): < ND, NU, NG > = <52, 26, 1> |  |
| Target data rates for eMBB | PUSCH with 1Mbps target data rate | PUSCH with 5Mbps target data rate |
| Pathloss model (select from LoS or NLoS) | Urban: NLoS |  |
| System bandwidth | 100MHz | 200MHz |
| Channel model for link-level simulation | TDL-C for NLOS | CDL- A, TDL-A, [urban/suburban: TDL-C] Note: company can provide simulation results based on either TDL channel or CDL model |
| Delay spread | 300ns | 100ns |
| UE velocity | 3km/h for indoor | 3km/h for indoor, 30km/h for outdoor. |
| Number of antenna elements for BS (Legacy TDD) | 128 antenna elements, (M,N,P,Mg,Ng) = (8,8,2,1,1) | 512 antenna elements, (M,N,P,Mg,Ng) = (4,16,2,2,2) |
| Number of TxRUs for BS (Legacy TDD) | gNB architectures to study: 32TxRUs per panel, (Mp,Np)=(2,8) gNB modeling in LLS for TDL: - Option 1: 2 or 4 or 8 gNB RF chains per panel in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. | gNB architectures to study: 2TxRUs per panel gNB modeling in LLS: gNB RF chains per panel in LLS Note: Analog beamforming is assumed. |
| BS antenna configuration (SBFD) | Same area & same TxRUs (higher priority): SBFD antenna configuration Option 2 |  |
| Frequency hopping | w/ or w/o frequency hopping |  |
| BLER | 10% iBLER. |  |
| Number of antenna elements for UE | 2 antenna elements, (M,N,P,Mg,Ng) = (1,1,2,1,1) | 32 antenna elements, (M,N,P,Mg,Ng) = (2,4,2,1,2) |
| Number of TxRUs for UE | UE architectures to study: 2TxRUs per panel, (Mp,Np)=(1,1) gNB modeling in LLS for TDL: - Option 1: 1 UE RF chains in LLS. - Option 2 (Optional): Number of UE RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. | UE architectures to study: 2TxRUs per panel, (Mp,Np)=(1,1) gNB modeling in LLS for TDL: - Option 1: 1, 2 UE RF chains in LLS. - Option 2 (Optional): Number of UE RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For 30km/h (optional: 120km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping for PUSCH: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH/PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) | DFT-s-OFDM |
| SCS | 30kHz | 120kHz. |
| PUSCH duration | 14 OS | 14 OS |
| Repetitions | Legacy TDD: w/o repetition SBFD: up to company to report |  |
| HARQ configuration | whether HARQ is adopted is up to company to report |  |
| PRBs/TBS/MCS | Any value of PRBs, and corresponding MCS index, up to company to report TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |  |

For coverage performance evaluation of SBFD, the link budget template in Table A.3 in TR 38.830 with necessary modification as shown in Annex B can be used as a starting point.

Proposal 14: For coverage performance evaluation for SBFD, the link budget template in Table A.3 in TR 38.830 with necessary modification as shown in Annex B can be used as a starting point.

Figure 1  Equivalent interference modelling in LLS.

The following high-level evaluation method is used for coverage performance evaluation

- Step 1: For legacy TDD system, assume the SNR in UL only slot is SNRLLS,TDD=SLLSNLLS,TDD. NLLS,TDD is modelled as gaussian noise. Perform LLS to get the target SNR (SNRLLS,TDD) with which UE can achieve a certain bit rate in UL, wherein,

  - Single slot PUSCH transmission is assumed

  - Note that from SLS view, NLLS,TDD=N+IUE-gNBTDD, and N is the noise, and IUE-gNBTDD is the legacy UE-gNB interference in TDD from SLS. But in LLS, as shown in Figure 1 (a), legacy UE-gNB interference is not explicitly modelled

- Step 2: For SBFD system, assume the SNR in UL only slot is SNRLLS,SBFD-UL=SLLSNLLS,TDD, and in SBFD slot SNRLLS,SBFD-X=SLLSNLLS,TDD+ILLS,SBFD. Perform LLS to get the target SNR (SNRLLS,SBFD-UL) with which UE can achieve a certain bit rate in UL for a given SBFD coverage enhancement scheme (e.g., SBFD with PUSCH repetition type A, etc.), wherein,

  - SBFD frame structure (XXXXU) is used for LLS

  - Note that from SLS view, ILLS,SBFD=NLLS,SBFD-NLLS,TDD=IgNB-gNB+Ico-site+ISI+IUE-gNBSBFD-IUE-gNBTDD. But in LLS, SBFD interferences (i.e., IgNB-gNB+Ico-site+ISI) are implicitly modelled as an additional equivalent interference ILLS,SBFD.

  - As shown in Figure 1 (c), if we define 10*log10ILLS,SBFD=10*log10NLLS,TDD+∆, then ∆=10*log10ILLS,SBFDNLLS,TDD=10*log10IgNB-gNB+Ico-site+ISI+IUE-gNBSBFD-IUE-gNBTDDN+IUE-gNBTDD

  - ∆ is obtained from SLS, wherein,

      - ISI is the residual self-interference.

      - Ico-site is the sum of co-site inter-sector interference

      - IgNB-gNB is the inter-site gNB-gNB co-channel inter-subband CLI with a specific interference power

      - IUE-gNBTDD is the legacy UE-gNB interference with a specific interference power in UL only symbols

      - IUE-gNBSBFD is the legacy UE-gNB interference with a specific interference power in SBFD symbols

      - N is the white noise in SLS

- Step 3: Perform Link budget analysis by reusing the link budget template in TR 38.830 as much as possible to obtain MPL, MCL, and MIL for legacy TDD and SBFD.

  - For legacy TDD, SNRLLS,TDD obtained in Step 1 is used to calculate MPL, MCL, MIL.

  - For SBFD, SNRLLS,SBFD-UL obtained in Step 2 is used to calculate MPL, MCL, MIL.

Regarding how to obtain ∆ from SLS, the following method is used.

- Step 1: Perform SLS for SBFD with frame structure (XXXXU), and log the post MMSE interference component in each slot, e.g.,

  - In X slot, for the scheduled UL RBs, log the per RB average post MMSE interference power of ISI, Ico-site, IgNB-gNB, IUE-gNBSBFD

  - In UL slot, for the scheduled UL RBs, log the per RB average post noise power N, and the interference power IUE-gNBTDD

- Step 2: Calculate ∆k per TDD period, i.e., in each TDD period,

  - Obtain the per RB average post MMSE interference power of ISI, Ico-site, IgNB-gNB, IUE-gNBSBFD over all X slots

    - Note: One slot may be excluded from statistics if any above interference component has invalid value, e.g., due to no UL/DL scheduling

  - Calculate ∆k=10*log10IgNB-gNB+Ico-site+ISI+IUE-gNBSBFD-IUE-gNBTDDN+IUE-gNBTDD, wherein, subscript k denotes the TDD period index

    - Note: One TDD period may be excluded from statistics if any above interference component has invalid value

- Step 3: Obtain the average ∆ over all ∆k

The post-MMSE ∆ value obtained from SLS for low load, medium load, and high load are summarized in Table 6.

Table 6  The average ∆ from SLS for Urban Macro (FR1).

|  | Low load | Medium load | High load |
| --- | --- | --- | --- |
| Post MMSE ∆ (dB) | -0.13 | 1.43 | 3.26 |

Initial evaluation results for SBFD coverage performance

Initial LLS evaluation was performed for SBFD coverage performance assuming

- PUSCH with 1Mbps target data rate for Urban Macro scenario for FR1

- Baseline legacy TDD with {DDDSU} and SBFD with {XXXXU}

  - For baseline legacy TDD, single slot is used for PUSCH transmission

  - For SBFD, coverage enhancement technique of Case 2 (i.e., SBFD with PUSCH repetition type A) is assumed, wherein, PUSCH for SBFD is repeated across 4 SBFD slots and one UL slot with RV {0,2,3,1,0}

    - As shown in Figure 1 (c), in SBFD slot, an additional equivalent interference ILLS,SBFD is considered, wherein, 10*log10ILLS,SBFD=10*log10NLLS,TDD+∆, and ∆ is obtained from SLS.

    - The ∆ values in Table 6 is used for LLS evaluation

- For both baseline legacy TDD and SBFD, PUSCH with 24 PRB and MCS = 5 is considered to achieve 1Mbps target data rate

- Regarding antenna configuration, 2T32R is considered for architectures to study as shown in Table 5, while 1T8R is modelled in our LLS. The practical antenna gain can be reflected in link budget template

The initial LLS evaluation results are given in Table 7 and Figure 2.

Considering post-MMSE interferences, it can be observed that the required SNR for SBFD with repetition across 4 SBFD slots and one UL slot is much smaller than legacy TDD, and the difference (which may be seen as required SNR gain) decreases with the traffic load increases,

- For low load, required SNR gain of SBFD over legacy TDD is about 4.17 dB for ∆ = -0.13 dB

- For medium load, required SNR gain of SBFD over legacy TDD is about 3.58 dB for ∆ = 1.43 dB

- For high load, required SNR gain of SBFD over legacy TDD is about 2.95 dB for ∆ = 3.26 dB

Table 7  Required SNR in UL only slot assume different interference.

|  | ∆ (dB) | Required SNR(dB) in UL only slot (1Mbps with 10% iBLER) | Required SNR gain (dB) |
| --- | --- | --- | --- |
| Legacy TDD (single UL slot) | - | -7.8 | - |
| SBFD (XXXXU) for Low Load (consider post-MMSE interference) | -0.13 | -11.97 | 4.17 |
| SBFD (XXXXU) for Medium Load (consider post-MMSE interference) | 1.43 | -11.38 | 3.58 |
| SBFD (XXXXU) for High Load (consider post-MMSE interference) | 3.26 | -10.75 | 2.95 |

Figure 2  BLER over required SNR.

Coverage metrics, e.g., MPL, MCL, MIL, and Maximum range, are obtained from Link budget analysis (with Link budget template shown in Annex B) based on the required SNR for both legacy TDD and SBFD obtained in LLS (as shown in Table 7).

The PUSCH coverage performance of SBFD with 1Mbps target data rate for Urban Macro O2I scenario for FR1, assuming 24 PRB and MCS = 5, legacy TDD with {DDDSU} and SBFD with {XXXXU}, SBFD coverage enhancement technique of Case 2 (SBFD with PUSCH repetition type A, wherein, PUSCH for SBFD is repeated across 4 SBFD slots and one UL slot with RV {0,2,3,1,0}) is summarized in Table 8. The detailed Link budget analysis can be found in Annex C.

Table 8  Coverage performance of SBFD.

|  | Required SNR(dB) in LLS | MPL (dB) | MCL (dB) | MIL (dB) | Maximum range (m) |
| --- | --- | --- | --- | --- | --- |
| Legacy TDD (single UL slot) | -7.8 | 128.9 | 142.0 | 111.2 | 155.3 |
| SBFD (XXXXU) for Low Load | -11.97 | 133.1 | 146.1 | 115.4 | 198.6 (gain = 27.8%) |
| SBFD (XXXXU) for Medium Load | -11.38 | 132.5 | 145.5 | 114.8 | 191.8 (gain = 23.5%) |
| SBFD (XXXXU) for High Load | -10.75 | 131.9 | 144.9 | 114.2 | 184.8 (gain = 19.0%) |

Observation 9: For PUSCH coverage performance with 1Mbps target data rate for Urban Macro O2I scenario for FR1, assuming 24 PRB and MCS = 5, legacy TDD with {DDDSU} and SBFD with {XXXXU}, SBFD coverage enhancement technique of Case 2 (SBFD with PUSCH repetition type A, wherein, PUSCH for SBFD is repeated across 4 SBFD slots and one UL slot with RV {0,2,3,1,0}),

- The coverage performance of SBFD is much better than legacy TDD

- The coverage performance gain of SBFD over legacy TDD decreases with the traffic load increases

- Regarding MPL metric,

  - The MPL for legacy TDD is 128.9dB,

  - The MPL for SBFD is 133.1dB, 132.5dB and 131.9dB for low load, medium load, and high load, respectively

- Regarding MCL metric,

  - The MCL for legacy TDD is 142.0dB,

  - The MCL for SBFD is 146.1dB, 145.5dB and 144.9dB for low load, medium load, and high load, respectively

- Regarding MIL metric,

  - The MIL for legacy TDD is 111.2dB,

  - The MIL for SBFD is 115.4dB, 114.8dB and 114.2dB for low load, medium load, and high load, respectively

- Regarding maximum range metric,

  - The maximum range for legacy TDD is 155.3m,

  - The maximum range for SBFD is 198.6m (gain = 27.8%), 191.8 (gain = 23.5%) and 184.8 (gain = 19.0%) for low load, medium load, and high load, respectively

Conclusions

In this contribution, remaining issues on SLS and LLS evaluation methodology are discussed. Some initial evaluation results are also provided. The following observations and proposals are made.

Observation 1: The following agreements related to the modelling of gNB self-interference were confirmed by RAN4 according to R1-2300025.

[RAN1#110b] Agreement For SLS of SBFD in RAN1, candidate values for αSI at least can be determined based on the assumption that UL receiver sensitivity degradation due to self-interference is 1dB. - FFS: UL receiver sensitivity degradation due to self-interference is 0.8dB and 0.1dB - The value of αSI can be calculated based on the UL receiver sensitivity degradation, noise floor of UL subband and maximum gNB DL Tx Power as below - αSI=PtxmaxISImax*NDLRBNULRB - For example, for sensitivity degradation of 1dB, ISImax can be computed based on 10*log10ISImax+NN=1dB, where N is the noise floor over the UL subband given by N(dB)=-174 + 10*log10 (20e6) + 5 = -96 dBm, assuming 20MHz UL subband and 5dB noise figure. - Note: the feasibility of the determined αSI values can be discussed separately - Companies shall report what values of the individual components are assumed in order to achieve the alpha_SI value corresponding to 1 dB desense - Other approaches of determining values for αSI are not precluded and can be used and reported by companies. Send LS to RAN4 to confirm RAN1’s understanding. Agreement RAN1 assumes frequency isolation value in the overall RSI value ranges provided by RAN4 is based on the assumption of SBFD subband configuration with {DUD=40MHz:20MHz:40MHz} at least for FR1 and all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power. - For SLS of SBFD in RAN1, the RSI is modelled as frequency flat within the UL subband. - Using αSI to denote the overall RSI value provided by RAN4, RAN1 makes the following assumption - ISImax=PtxmaxαSI*NULRBNDLRB - ISImax is the residual self-interference power on the UL subband when all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power (in linear scale). - Ptxmax is the maximum gNB DL Tx Power on the two DL subbands (in linear scale). - NDLRB is the total number of DL RBs in the DL subbands. - NULRB is the total number of UL RBs in the UL subband. - Note: αSI is in linear scale - RAN1 further makes a simple assumption that αSI doesn’t change when DL RBs are not fully allocated for DL transmission, and the residual self-interference power on one UL RB when DL RBs are not fully allocated for DL transmission is computed by - ISIper-RB=Ptxper-RBαSI*Nused-DL-RBNDLRB - Ptxper-RB is DL transmission power of gNB per RB, Ptxper-RB=Ptxmax/NDLRB. - Nused-DL-RB is the number of DL RBs allocated for DL transmission. - Send LS to RAN4 to confirm RAN1’s assumptions and the subband configuration assumed for FR1/FR2 - Also ask RAN4 if the above is applicable to other subband configurations

Observation 2: The following agreement related to the modelling of co-site inter-sector co-channel inter-subband CLI was confirmed by RAN4 according to R1-2302262.

[RAN1#111] Agreement For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling as follows. Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB - Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB. - Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale). - NDLRB is the total number of DL RBs in the DL subbands. - Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x. - αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI. - 10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+… - Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment. - Companies shall report the value of αco-site assumed in the simulations with feasibility of how these values were derived. - Send LS to RAN4 confirming the model and asking the value ranges for spatial isolation, and values of 10*log10ACLRBS and 10*log10ACSBS.

Observation 3: The following agreements related to the modelling of inter-site gNB-gNB co-channel inter-subband CLI were confirmed by RAN4 according to R1-2300025.

[RAN1#110b] Agreement For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for gNB-gNB co-channel channel model, the power of inter-site gNB-gNB co-channel inter-subband CLI experienced by the victim gNB on each receiver chain at one UL RB can be modelled as - IInter-Site-CLIA'→A,per-RB=PtxBS A',per-RB*Nused-DL-RBBS A'*CLBS A'→BS A*1ACLRBS+1ACSBS* 1NDLRB - IInter-Site-CLIA'→A,per-RB is the power of inter-site gNB-gNB co-channel inter-subband CLI from gNB A' to gNB A on each receiver chain at one UL RB (linear value) - PtxBS A',per-RB is DL transmission power of gNB A' across all transmit chains per RB (linear value). PtxBS A',per-RB=Ptxmax/NDLRB. - Nused-DL-RBBS A' is the number of DL RBs allocated for DL transmission by gNB A' - CLBS A'→BS A is the coupling loss between gNB A' and gNB A (linear value), accounting for beamforming at the aggressor gNB and victim gNB. - FFS: the detailed definition of the coupling loss, which can be discussed later - NDLRB is the total number of DL RBs in the DL subbands - Note: ACLRBS and ACSBS are in linear scale. In RAN4 reply LS, gNB ACLR (i.e., ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment. - Note: the model is based on the assumption that the same transmission power across different DL RBs is used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS. - Note: This model is not applicable to the RBs in the guardband. - Note: This model is not applicable for some candidate gNB-gNB CLI handling schemes (for example, spatial digital beam coordination, advanced receivers) - Send LS to RAN4 to confirm RAN1’s understanding Agreement For SLS in RAN1, if both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the inter-site gNB-gNB co-channel inter-subband CLI signal across all Rx chains at UL RB n at victim gNB can be modeled as IInter-Site-CLIn=Ileakagen+Iselectivity, where, - Ileakagen=HCLInWyn is the first part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at UL RB n, caused by power leakage at aggressor gNB, - HCLIn is the NR×NT channel matrix between aggressor gNB and victim gNB at UL RB n, the beamforming of the aggressor gNB and the victim gNB can be taken into account by HCLIn, - yn=y0n,y1n,…,yNT-1nT is the unwanted emission across all Tx chains at UL RB n at aggressor gNB, - NT is the number of Tx chains at aggressor gNB, - ykn~N0,σy,n2, k=0,1,…,NT-1, is modelled as white Gaussian noise, - σy,n2=Ptxper-RB*Nused-DL-RBACLRBS* 1NDLRB is the total leakage power at UL RB n at aggressor gNB, - Ptxper-RB is the DL power transmitted across all Tx chains at one DL RB at aggressor gNB, Ptxper-RB=Ptxmax/NDLRB, - Nused-DL-RB is the number of DL RBs scheduled for DL transmission by aggressor gNB, - NDLRB is the total number of DL RBs in the DL subbands - W is the NT×NT normalized identity matrix with unit norm, WF=1, - FFS whether W can be other values and corresponding conditions - FFS for Iselectivity - Note: ACLRBS and ACSBS are in linear scale. In RAN4 reply LS, gNB ACLR (i.e., ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment. - Note: the model is based on the assumption that the same transmission power across different DL RBs are used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS. - Note: This model is not applicable to the RBs in the guardband. - Send LS to RAN4 to confirm RAN1’s understanding.

Observation 4: Regarding the below agreement related to the modelling of inter-site gNB-gNB co-channel inter-subband CLI, RAN1’s assumption on ICSBS (in channel selectivity) to be given by the value of gNB ACS was confirmed by RAN4 according to R1-2302262.

[RAN1#111] Agreement Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB, can be modelled as Iselectivity=z0n,z1n,…,zNR-1nT - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise - σz,n2=PblockerICSBS×NDLRB - Pblocker=1NR*m∈Used DL RBsHCLImWmsm2 - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm, - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1, - sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB. - NDLRB is the total number of DL RBs in the DL subbands, - RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS unless further RAN4 guidance is received. - Send LS to RAN4 to confirm RAN1 understanding and check whether ICSBS can be modelled depending on the value of the blocker interference, e.g., ICSBS= ICS1 Pblocker<P1 ICS2, P1< Pblcoker<P2ICS3 Pblocker>P2 - Note: Pblocker can be reported by companies

Observation 5: Regarding the modelling of UE-UE co-channel inter-subband CLI, according to R1-2302262, RAN4 also adopts IBE-based model, and RAN4 has not reached the agreement for an equivalent frequency flat model.

[RAN1#111] Agreement For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, Option 1 is used as starting point. - Option 1: RAN1 to take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point. - Send LS to RAN4 to ask them whether it can be modelled as an equivalent frequency flat model (e.g., IBEUE,ave) based on RAN4 IBE requirement, and if possible, what is the value of IBEUE,ave

Observation 6: The following agreements and working assumptions related to the modelling of UE-UE co-channel inter-subband CLI still wait for RAN4’s confirm.

[RAN1#112] Agreement For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE co-channel channel model, the power of UE-UE co-channel inter-subband CLI experienced by the victim UE on each receiver chain at DL RB n can be modelled as IUE-UE-CLIB'→B,per-RBn=IUE-UE,IBEn*CLUE B'→UE B+PtxUE B'*CLUE B'→UE B*1ICSUE*1NULRB where - IUE-UE-CLIB'→B,per-RBn is the power of UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B on each receiver chain at one DL RB n (linear value). - PtxUE B' is UL transmission power of UE B' across all transmit chains over the allocated UL RBs (linear value) - CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE - NULRB is the total number of UL RBs in the UL subband - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input. - 10*log10IUE-UE,IBEn=maxPtxUE B',per-RB-30dB,10*log10IGeneral+IIQfor FR1maxPtxUE B',per-RB-25dB,10*log10IGeneral+IIQfor FR2-1, wherein, - For SBFD Subband configuration with {DUD} pattern, IIQ can be ignored - 10*log10IGeneral=PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-3-5⋅ΔRB-1LCRB,-57dBm+10log10SCS/15kHz-PtxUE B',per-RBfor FR1PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-5⋅ΔRB-1LCRB,-55.1dBm-PtxUE B',per-RBfor FR2-1 - PtxUE B',per-RB is UL transmission power of UE B' across all transmit chains per RB (linear value). PtxUE B',per-RB=PtxUE B'/ LCRB, and LCRB is the number of UL RBs allocated for UL transmission of UE B'. - NRB is the Transmission Bandwidth Configuration, referring to Table 5.3.2-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1. - NRB=273 for FR1 with 100MHz transmission bandwidth and 30kHz SCS - NRB=132 for FR2-1 with 200MHz transmission bandwidth and 120kHz SCS - ΔRB is the starting frequency offset between the allocated UL RBs and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated UL RBs) - EVM is the limit specified in Table 6.4.2.1-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1 for the modulation format used in the allocated RBs. Include the above in the LS to RAN4 to inform them of the agreement and to check if the RAN1 agreement is in line with RAN4’s understanding. Working assumption: For SLS in RAN1, if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as: IInter-UE-CLIn=Ileakagen+Iselectivity, where, - Ileakagen=HCLIn W yn is the first part of UE-UE co-channel inter-subband CLI across all Rx chains at DL RB n, caused by power leakage at aggressor UE, - HCLIn is the NR×NT channel matrix between aggressor UE and victim UE at DL RB n, the beamforming of the aggressor UE and the victim UE can be taken into account by HCLIn - NR is the number of Rx chains and NT is the number of Tx chains - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1. - yn=y0n,y1n,…,yRnT, - ykn~N0,IUE-UE,IBEn, k=0,1,…,R-1, is modelled as white Gaussian noise - IUE-UE,IBEn has the same meaning as in the agreement for the case only large-scale fading is modelled - Iselectivity is modelled as frequency flat Iselectivity=z0n,z1n,…,zNR-1nT - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise, σz,n2=PblockerICSUE×NULRB - Pblocker=1NR*m∈Used UL RBsHCLImWsm2 - HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim gNB can be taken into account by HCLIm, - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1 - sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE B',per-RB. - PtxUE B',per-RB has the same meaning as in the agreement for the case only large-scale fading is modelled - NULRB is the total number of UL RBs in the UL subbands, - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input. Include the above in the LS to RAN4 to inform them of the agreement and to check if the RAN1 agreement is in line with RAN4’s understanding.

Observation 7: Regarding the below agreement related to the modelling of co-site gNB-gNB adjacent-channel CLI, the value of αadj, co-site-x still wait for RAN4’s inputs.

[RAN1#112] Agreement For SLS in RAN1, for co-site gNB-gNB adjacent-channel CLI modelling, reuse similar method as co-site inter-sector co-channel inter-subband CLI modeling as follows. Iadj, co-siteper-RB=Iadj, co-site-sector-1per-RB+Iadj, co-site-sector-2per-RB+Iadj, co-site-sector-3per-RB Iadj,co-site-sector-xper-RB=Ptxadj,sector-x, per-RBαadj,co-site-x*Nused-DL-RBadj, sector-xNDLRBadj - Ptxadj, sector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxadj,sector-x, per-RB=Ptxadj,sector-x, max/NDLRBadj. - Ptxadj,sector-x, max is the maximum DL Tx Power of sector x in adjacent channel (in linear scale). - NDLRBadj is the total number of DL RBs in adjacent channel. - Nused-DL-RBadj,sector-x is the number of DL RBs allocated for DL transmission of sector x in adjacent channel. - αadj, co-site-x is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI between the aggressor sector x and the victim sector. - 10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS+… - FFS the concrete value of αadj, co-site-x - ACLRBS and ACSBS are in linear scale. Send an LS to RAN4 to inquire on the value of αadj, co-site-x.

Observation 8: For Indoor Office (FR1) in SBFD Deployment Case 1, assuming gNB self-interference isolation based on 1 dB UL desense, SBFD antenna configuration Option 2, 0.5Mbyte FTP packet size for DL and 0.125 Mbytes for UL,

- For SBFD slot configuration Alt 2: {DDDSU} vs. {XXXXU} with 2 guard symbols,

  - For UL/DL resource percentage per TDD period

    - The DL resource percentage per TDD period is decreased by around 23.8%

    - The UL resource percentage per TDD period is increased by around 77.7%

  - For {DL : UL} traffic load for legacy TDD = {Low : Low}

    - DL performance of SBFD compared with legacy TDD

      - The mean value of DL average-UPT CDF of SBFD is decreased by around 19.97%

      - The 5% of DL average-UPT CDF of SBFD is decreased by around 20.91%

      - The mean value of DL packet-latency CDF of SBFD is increased by around 22.92%

      - The 5% of DL packet-latency CDF of SBFD is increased by around 23.53%

      - The DL Type-1 RU of SBFD almost keeps unchanged

      - The DL Type-2 RU of SBFD is increased by around 26.91%

    - UL performance of SBFD compared with legacy TDD

      - The mean value of UL average-UPT CDF of SBFD is increased by around 81.92%

      - The 5% of UL average-UPT CDF of SBFD is increased by around 100.20%

      - The mean value of UL packet-latency CDF of SBFD is decreased by around 47.27%

      - The 5% of UL packet-latency CDF of SBFD is decreased by around 39.13%

      - The UL Type-1 RU of SBFD is decreased by around 10.62%

      - The UL Type-2 RU of SBFD is decreased by around 50.64%

  - For {DL : UL} traffic load for legacy TDD = {Medium : Medium}

    - DL performance of SBFD compared with legacy TDD

      - The mean value of DL average-UPT CDF of SBFD is decreased by around 23.68%

      - The 5% of DL average-UPT CDF of SBFD is decreased by around 24.13%

      - The mean value of DL packet-latency CDF of SBFD is increased by around 33.07%

      - The 5% of DL packet-latency CDF of SBFD is increased by around 23.53%

      - The DL Type-1 RU of SBFDs increased by around 2.60%

      - The DL Type-2 RU of SBFD is increased by around 29.74%

    - UL performance of SBFD compared with legacy TDD

      - The mean value of UL average-UPT CDF of SBFD is increased by around 88.35%

      - The 5% of UL average-UPT CDF of SBFD is increased by around 113.82%

      - The mean value of UL packet-latency CDF of SBFD is decreased by around 50.26%

      - The 5% of UL packet-latency CDF of SBFD is decreased by around 39.13%

      - The UL Type-1 RU of SBFD is decreased by around 11.03%

      - The UL Type-2 RU of SBFD is decreased by around 50.71%

- For SBFD slot configuration Alt 4: {DDDSU} vs. {XXXXX}

  - For UL/DL resource percentage per TDD period

    - The DL resource percentage per TDD period is decreased by around 1.2%

    - The UL resource percentage per TDD period is increased by around 0.7%

  - For {DL : UL} traffic load for legacy TDD = {Low : Low}

    - DL performance of SBFD compared with legacy TDD

      - The mean value of DL average-UPT CDF of SBFD is increased by around 0.91%

      - The 5% of DL average-UPT CDF of SBFD is increased by around 0.65%

      - The mean value of DL packet-latency CDF of SBFD is decreased by around 3.12%

      - The 5% of DL packet-latency CDF of SBFD almost keeps unchanged

      - The DL Type-1 RU of SBFD almost keeps unchanged

      - The DL Type-2 RU of SBFD is increased by around 0.70%

    - UL performance of SBFD compared with legacy TDD

      - The mean value of UL average-UPT CDF of SBFD is increased by around 8.38%

      - The 5% of UL average-UPT CDF of SBFD is increased by around 23.61%

      - The mean value of UL packet-latency CDF of SBFD is decreased by around 13.33%

      - The 5% of UL packet-latency CDF of SBFD is increased by around 13.04%

      - The UL Type-1 RU of SBFD is decreased by around 16.81%

      - The UL Type-2 RU of SBFD is decreased by around 18.28%

  - For {DL : UL} traffic load for legacy TDD = {Medium : Medium}

    - DL performance of SBFD compared with legacy TDD

      - The mean value of DL average-UPT CDF of SBFD is increased by around 0.42%

      - The 5% of DL average-UPT CDF of SBFD is increased by around 1.03%

      - The mean value of DL packet-latency CDF of SBFD is decreased by around 2.69%

      - The 5% of DL packet-latency CDF of SBFD almost keeps unchanged

      - The DL Type-1 RU of SBFD is increased by around 0.21%

      - The DL Type-2 RU of SBFD is increased by around 0.61%

    - UL performance of SBFD compared with legacy TDD

      - The mean value of UL average-UPT CDF of SBFD is increased by around 9.82%

      - The 5% of UL average-UPT CDF of SBFD is increased by around 26.69%

      - The mean value of UL packet-latency CDF of SBFD is decreased by around 15.34%

      - The 5% of UL packet-latency CDF of SBFD is increased by around 13.04%

      - The UL Type-1 RU of SBFD is decreased by around 16.91%

      - The UL Type-2 RU of SBFD is decreased by around 18.31%

Observation 9: For PUSCH coverage performance with 1Mbps target data rate for Urban Macro O2I scenario for FR1, assuming 24 PRB and MCS = 5, legacy TDD with {DDDSU} and SBFD with {XXXXU}, SBFD coverage enhancement technique of Case 2 (SBFD with PUSCH repetition type A, wherein, PUSCH for SBFD is repeated across 4 SBFD slots and one UL slot with RV {0,2,3,1,0}),

- The coverage performance of SBFD is much better than legacy TDD

- The coverage performance gain of SBFD over legacy TDD decreases with the traffic load increases

- Regarding MPL metric,

  - The MPL for legacy TDD is 128.9dB,

  - The MPL for SBFD is 133.1dB, 132.5dB and 131.9dB for low load, medium load, and high load, respectively

- Regarding MCL metric,

  - The MCL for legacy TDD is 142.0dB,

  - The MCL for SBFD is 146.1dB, 145.5dB and 144.9dB for low load, medium load, and high load, respectively

- Regarding MIL metric,

  - The MIL for legacy TDD is 111.2dB,

  - The MIL for SBFD is 115.4dB, 114.8dB and 114.2dB for low load, medium load, and high load, respectively

- Regarding maximum range metric,

  - The maximum range for legacy TDD is 155.3m,

  - The maximum range for SBFD is 198.6m (gain = 27.8%), 191.8 (gain = 23.5%) and 184.8 (gain = 19.0%) for low load, medium load, and high load, respectively

Proposal 1: Regarding SLS of potential enhancements on dynamic/flexible TDD in AI 9.3.3,

- The basic evaluation methodologies and assumptions for dynamic/flexible TDD agreed in AI 9.3.1 can be used.

- If additional scheme-specific assumptions are needed for some CLI schemes, it is up to companies to report the scheme-specific assumptions.

Proposal 2: Companies are encouraged to upload evaluation results to the FTP draft folder with the link (ftp://ftp.3gpp.org/tsg_ran/WG1_RL1/TSGR1_112/Inbox/drafts/9.3(FS_NR_duplex_evo)/9.3.1/Evaluation Results/)

- In the excel sheets, companies are encouraged to provide both assumptions and evaluation results. Each company can input multiple columns, and each column corresponds to one kind of assumption and the corresponding result

- Regarding the performance metrics, it is recommended to at least provide Average-UPT CDF related metrics and Packet-Latency CDF related metrics for drawing conclusions, and other metrics (e.g., Tail-UPT, Median-UPT, UE-Average-Latency) are up to companies. In addition, it is also recommended to at least provide relative gains of SBFD compared to legacy TDD for UPT and latency related metrics, and the absolute values are up to companies

- For each excel file, every time when update the results, companies are recommended to add one new row in the sheet named “Revision comments” to briefly indicate what changes have been made at this time

Proposal 3: For duplex evolution evaluation, the evaluation results are categorized into X sub-cases (as shown in below table for example) based on the different key assumptions. Each sub-case is based on one combination of key assumptions.

- Note: How many sub-cases will be determined and which assumptions will be used for the categorization can be discussed based on the final evaluation results and assumptions submitted by companies.

Table X: Sub-cases for Urban Macro in FR1 in SBFD Deployment Case 1.

| Key assumptions Sub-cases | Interference modelling (e.g., Co-site: Spatial isolation + digital isolation) |  |  |  | SBFD slot configuration |  | BS transmit power |  | SBFD antenna configuration |  | Packet Size |  | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 75dB | 93dB | 100dB | 100+10 dB | Alt-2: {DDDSU} vs. {XXXXU} | Alt-4: {DDDSU} vs. {XXXXX} | 53dBm | 49dBm | Twice area&same TxRUs | Same area&same TxRUs | DL: 4Kbytes, UL: 1Kbyte | DL: 0.5Mbytes, UL: 0.125Mbyte |  |
| SBFD#1_UMa_FR1_Sub#1 |  |  |  | ○ | ○ |  | ○ |  | ○ |  |  |  | Source [X], … |
| SBFD#1_UMa_FR1_Sub#2 |  |  |  | ○ |  | ○ | ○ |  | ○ |  |  |  | Source [X], … |

Proposal 4: For each sub-case, the performance gains of SBFD over legacy TDD are summarized (as shown in below table for example). For each performance metric, the {mean, <min~max>} is used to represent the mean value and the value range from the sources.

Table-Y: Summary of results for sub-case XX.

| Simple description for the sub-case (e.g., 100dB co-site inter-sector co-channel inter-subband isolation, SBFD Alt2, 49dBm gNB Tx power, Twice area&same TxRUs, DL: 4Kbytes, UL: 1Kbyte, UE clustering,…) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT (Mbps) | Mean | {mean, <min~max>} = {3%, <-10%~20%>} |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Average-UPT (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 |  |  |  |  |  |  |  |  |  |
|  | Type-2 |  |  |  |  |  |  |  |  |  |
| UL RU (%) | Type-1 |  |  |  |  |  |  |  |  |  |
|  | Type-2 |  |  |  |  |  |  |  |  |  |
| Note: - For Average-UPT / Packet-Latency / RU, the gain can be calculated as: Gain (%) = SBFD UPT/latency/RUTDD UPT/latency/RU-1 |  |  |  |  |  |  |  |  |  |  |

Proposal 5: The following table is used to capture companies’ evaluation results in the Annex of TR 38.858. Companies are encouraged to provide evaluation results in their submitted contribution with the following table.

Table D.1.2: Evaluation results for Urban Macro in FR1 in SBFD Deployment Case 1

| Tdoc/Source | Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
|  | DL Average-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Tail-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Median-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Average-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Tail-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Median-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | Type-1 RU (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Type-2 RU (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Unfinished/dropped Packet Rate (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Additional comments: e.g., Layout and UE distribution - Macro Layer: e.g., Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - UE distribution: e.g., UE clustering distribution with M=20, X=2 Interference Modelling - gNB self-interference: e.g., based on 1 dB UL desense - Co-site inter-sector co-channel inter-subband CLI: e.g., 100dB (spatial isolation), 10dB digital isolation - UE-UE co-channel inter-subband CLI: e.g., 33 dBc SBFD subband and slot configuration - SBFD slot configuration: Alt 2 (higher priority): Legacy TDD: {DDDSU}; SBFD: {XXXXU} - SBFD Subband configuration: e.g., <ND, NU, NG >=<104, 55, 5> - Guard symbol number: - UL resource percentage per TDD period (%): - DL resource percentage per TDD period (%): BS transmit power & antenna configuration - BS transmit power for legacy TDD: e.g., 53dBm - BS transmit power for SBFD: e.g., Option-1: Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems) - BS antenna configuration for legacy TDD: e.g., (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) , (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization - BS antenna configuration for SBFD: e.g., Twice area&same TxRUs (higher priority): SBFD antenna configuration Option 2 - BS antenna radiation pattern: e.g., Table 9 in Report ITU-R M.2412 - UE antenna configuration: e.g., 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization; 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization Traffic Model - DL/UL traffic assignment for the same UE: e.g., Option 2: Each UE is assigned both UL traffic and DL traffic - DL/UL FTP packet size: 4Kbytes for DL and 1Kbyte for UL Channel model - gNB-gNB: e.g., Both Large scale fading and small scale fading - UE-UE: e.g., Large scale fading only - UE-UE details: e.g., TR 38.901 Others - Open loop power control parameters: e.g., P0= -80 dBm, alpha = 0.8 - UE receiver: e.g., MMSE-IRC - Channel estimation: e.g., Ideal - Transmission scheme: e.g., SU-MIMO - Overhead: |  |  |  |  |  |  |  |  |  |  |

Proposal 6: It is recommended that companies to report the UL/DL resource percentage per TDD period together with the evaluation results, which can be calculated as follows:

- UL resource percentage per TDD period = (Number of UL RBs per cell per TDD period excluding guard bands and guard symbols)  /(Total number of RBs per cell per TDD period including DL, UL, guard bands and guard symbols )

- DL resource percentage per TDD period = (Number of DL RBs per cell per TDD period excluding guard bands and guard symbols) / (Total number of RBs per cell per TDD period including DL, UL, guard bands and guard symbols )

Proposal 7: For SLS of SBFD, with the assumption of 1dB UL receiver sensitivity degradation and SBFD antenna configuration Option-2, consider the below candidate values for RSI αSI.

|  | no power boosting (BS transmit power for SBFD Option-1) |  |  |  | with power boosting (BS transmit power for SBFD Option-2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FR1 (100 MHz), <104, 55, 5>, 30KHz |  | FR2-1(200 MHz), <52,26,1>, 120KHz |  | FR1 (100 MHz), <104, 55, 5>, 30KHz |  | FR2-1(200 MHz), <52,26,1>, 120KHz |  |
|  | Tx Power (dBm) | αSI (dB) | Tx Power (dBm) | αSI (dB) | Tx Power (dBm) | αSI (dB) | Tx Power (dBm) | αSI (dB) |
| Urban macro | 53 | 147.9 |  |  | 53 | 149.1 |  |  |
|  | 49 | 143.9 |  |  | 49 | 145.1 |  |  |
| Dense Urban Macro layer | 53 | 147.9 | 43 | 133.1 | 53 | 149.1 | 43 | 134.1 |
|  | 44 | 138.9 |  |  | 44 | 140.1 |  |  |
| Dense Urban Micro layer | 38 | 132.9 | 33 | 123.1 | 38 | 134.1 | 33 | 124.1 |
| Indoor hotspot | 24 | 118.9 | 23 | 113.1 | 24 | 120.1 | 23 | 114.1 |

Proposal 8: Confirm the below working assumption related to modelling of co-site inter-sector co-channel inter-subband CLI.

[RAN1#112] Working Assumption: For co-site inter-sector co-channel inter-subband CLI modelling, before receiving RAN4’s reply on the value of αco-site, RAN1 assume the following only for evaluation: - FR1: - 75dB for spatial isolation (RAN4 typical value). - 93dB for spatial isolation (RAN4 best value). - 100dB for spatial isolation - FR2: - 88dB for spatial isolation (RAN4 typical value). - 98dB for spatial isolation (RAN4 best value). - 105dB for spatial isolation - In addition to spatial isolation and frequency isolation, companies can use digital cancelation and report the value, e,g., 10dB. Above does not imply that RAN1 assumes or does not assume digital cancelation is feasible. - The feasibility of these values is up to RAN4. These values can be revisited based on further RAN4 inputs. - The 100dB/105dB isolation values for FR1 and FR2 are not from RAN4, but based on RAN4 input that some companies have proposed that isolating material could be added between sectors to increase the isolation. RAN4 has not yet discussed the details whether such approaches can be applied to outdoor sites.

Proposal 9: Update the agreement in RAN1#111 as below:

Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB, can be modelled as

Iselectivity=z0n,z1n,…,zNR-1nT

  - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise

  - σz,n2=PblockerICSBS×NDLRB

  - Pblocker=1NR*m∈Used DL RBsHCLImWmsm2

  - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm,

  - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1,

  - sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB.

  - NDLRB is the total number of DL RBs in the DL subbands,

  - RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS unless further RAN4 guidance is received.

    - Send LS to RAN4 to confirm RAN1 understanding and check whether ICSBS can be modelled depending on the value of the blocker interference, e.g.,

ICSBS= ICS1            Pblocker<P1 ICS2,   P1< Pblcoker<P2ICS3             Pblocker>P2

  - Note: Pblocker can be reported by companies.

Proposal 10: For SLS in RAN1, the BS noise figure is modelled as below

    - X-axis: Total received power is the linear sum of all received power, including wanted signal, legacy UE-gNB interference, self-interference, inter-gNB interference and inter-sector interference.

    - Y-axis: noise figure

    - The values of A, B, C and D:

      - A = -43dBm for FR1, FFS for FR2-1

      - B = -25dBm for FR1, FFS for FR2-1

      - C = 5dB for FR1, FFS for FR2-1

      - D = 14dB for FR1, FFS for FR2-1

    - If the total received power is larger than B, the receiver will be blocked.

Proposal 11: Confirm the below working assumption related to the modelling of UE-UE co-channel inter-subband CLI.

Working assumption:

For SLS in RAN1, if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as:

IInter-UE-CLIn=Ileakagen+Iselectivity, where,

- Ileakagen=HCLIn W yn is the first part of UE-UE co-channel inter-subband CLI across all Rx chains at DL RB n, caused by power leakage at aggressor UE,

  - HCLIn is the NR×NT channel matrix between aggressor UE and victim UE at DL RB n, the beamforming of the aggressor UE and the victim UE can be taken into account by HCLIn

  - NR is the number of Rx chains and NT is the number of Tx chains

  - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1.

  - yn=y0n,y1n,…,yRnT,

    - ykn~N0,IUE-UE,IBEn, k=0,1,…,R-1, is modelled as white Gaussian noise

    - IUE-UE,IBEn has the same meaning as in the agreement for the case only large-scale fading is modelled

- Iselectivity is modelled as frequency flat

Iselectivity=z0n,z1n,…,zNR-1nT

  - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise, σz,n2=PblockerICSUE×NULRB

  - Pblocker=1NR*m∈Used UL RBsHCLImWsm2

  - HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim gNB can be taken into account by HCLIm,

  - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1

  - sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE B',per-RB.

    - PtxUE B',per-RB has the same meaning as in the agreement for the case only large-scale fading is modelled

  - NULRB is the total number of UL RBs in the UL subbands,

  - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input.

Proposal 12: For SBFD evaluation, deprioritize SBFD subband configuration#2 with {DU} pattern.

Proposal 13: For coverage performance evaluation for SBFD, consider the following simulation assumptions for LLS.

Table  Simulation assumption for coverage performance evaluation for SBFD for LLS.

| Parameter | FR1 | FR2-1 |
| --- | --- | --- |
| Scenario and frequency | Urban Macro (ISD = 500m): 4GHz | Dense Urban Macro Layer (ISD = 200m): 30GHz |
| SBFD subband and slot configurations | Legacy TDD: DDDSU, S=[12D:2G:0U] SBFD: XXXXU, where X denotes SBFD slot. For SBFD slot, {DUD} pattern is assumed, For FR1, 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <104, 55, 5> For FR2-1, 200MHz channel bandwidth and 120kHz SCS (132 PRB): < ND, NU, NG > = <52, 26, 1> |  |
| Target data rates for eMBB | PUSCH with 1Mbps target data rate | PUSCH with 5Mbps target data rate |
| Pathloss model (select from LoS or NLoS) | Urban: NLoS |  |
| System bandwidth | 100MHz | 200MHz |
| Channel model for link-level simulation | TDL-C for NLOS | CDL- A, TDL-A, [urban/suburban: TDL-C] Note: company can provide simulation results based on either TDL channel or CDL model |
| Delay spread | 300ns | 100ns |
| UE velocity | 3km/h for indoor | 3km/h for indoor, 30km/h for outdoor. |
| Number of antenna elements for BS (Legacy TDD) | 128 antenna elements, (M,N,P,Mg,Ng) = (8,8,2,1,1) | 512 antenna elements, (M,N,P,Mg,Ng) = (4,16,2,2,2) |
| Number of TxRUs for BS (Legacy TDD) | gNB architectures to study: 32TxRUs per panel, (Mp,Np)=(2,8) gNB modeling in LLS for TDL: - Option 1: 2 or 4 or 8 gNB RF chains per panel in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. | gNB architectures to study: 2TxRUs per panel gNB modeling in LLS: gNB RF chains per panel in LLS Note: Analog beamforming is assumed. |
| BS antenna configuration (SBFD) | Same area & same TxRUs (higher priority): SBFD antenna configuration Option 2 |  |
| Frequency hopping | w/ or w/o frequency hopping |  |
| BLER | 10% iBLER. |  |
| Number of antenna elements for UE | 2 antenna elements, (M,N,P,Mg,Ng) = (1,1,2,1,1) | 32 antenna elements, (M,N,P,Mg,Ng) = (2,4,2,1,2) |
| Number of TxRUs for UE | UE architectures to study: 2TxRUs per panel, (Mp,Np)=(1,1) gNB modeling in LLS for TDL: - Option 1: 1 UE RF chains in LLS. - Option 2 (Optional): Number of UE RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. | UE architectures to study: 2TxRUs per panel, (Mp,Np)=(1,1) gNB modeling in LLS for TDL: - Option 1: 1, 2 UE RF chains in LLS. - Option 2 (Optional): Number of UE RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For 30km/h (optional: 120km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping for PUSCH: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH/PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) | DFT-s-OFDM |
| SCS | 30kHz | 120kHz. |
| PUSCH duration | 14 OS | 14 OS |
| Repetitions | Legacy TDD: w/o repetition SBFD: up to company to report |  |
| HARQ configuration | whether HARQ is adopted is up to company to report |  |
| PRBs/TBS/MCS | Any value of PRBs, and corresponding MCS index, up to company to report TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |  |

Proposal 14: For coverage performance evaluation for SBFD, the link budget template in Table A.3 in TR 38.830 with necessary modification as shown in Annex B can be used as a starting point.

References

- Chairman's Notes RAN1#112, Athens, Greece, February 27th – March 3rd, 2023.

- Chairman's Notes RAN1#111, Toulouse, France, November 14th – 18th, 2022.

- Chairman's Notes RAN1#110b-e, e-Meeting, October 10th – 19th, 2022.

- Chairman's Notes RAN1#110, Toulouse, France, August 22nd – 26th, 2022.

- Chairman's Notes RAN1#109-e, May 9th – 20th, 2022.

- Chairman's Notes RAN4#104-e, October 10th – 21th, 2022.

- Chairman's Notes RAN4#105, Toulouse, France, November 14th – 18th, 2022.

- R1-2212993, Final Summary on evaluation on NR duplex evolution, Moderator (CMCC).

- R1-2300025, LS response to RAN1 for interference modelling and Sub-band configuration, RAN4, Samsung

- Annex A: Format for capturing evaluation results in TR 38.858

Note: The followings are moderator’s initial thinking on how to capture the evaluation results and observations under TR clause 7.3 for SBFD.

7.3	Performance evaluation results

7.3.1	SBFD Deployment Cases 1

7.3.1.1	Indoor Office-FR1

…

7.3.1.2	Urban Macro-FR1

For Urban Macro in FR1 in SBFD Deployment Case 1, the evaluation results are included in Annex D.X. These evaluation results are categorized into X sub-cases in Table-X based on the different key assumptions. Each sub-case is based on one combination of key assumptions.

Note: How many sub-cases will be determined and which assumptions will be used for the categorization can be discussed based on the final evaluation results and assumptions submitted by companies. Here we just provide one example for the categorization.

Table X: Sub-cases for Urban Macro in FR1 in SBFD Deployment Case 1.

| Key assumptions Sub-cases | Interference modelling (e.g., Co-site: Spatial isolation + digital isolation) |  |  |  | SBFD slot configuration |  | BS transmit power |  | SBFD antenna configuration |  | Packet Size |  | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 75dB | 93dB | 100dB | 100+10 dB | Alt-2: {DDDSU} vs. {XXXXU} | Alt-4: {DDDSU} vs. {XXXXX} | 53dBm | 49dBm | Twice area&same TxRUs | Same area&same TxRUs | DL: 4Kbytes, UL: 1Kbyte | DL: 0.5Mbytes, UL: 0.125Mbyte |  |
| SBFD#1_UMa_FR1_Sub#1 |  |  |  | ○ | ○ |  | ○ |  | ○ |  |  |  | Source [X], … |
| SBFD#1_UMa_FR1_Sub#2 |  |  |  | ○ |  | ○ | ○ |  | ○ |  |  |  | Source [X], … |

7.1.1.2.1	SBFD#1_UMa_FR1_Sub#1

For sub-case SBFD#1_UMa_FR1_Sub#1, [X] sources provided the results. The performance gains of SBFD over legacy TDD are summarized in Table-Y. For each performance metric, the {mean, <min~max>} is used to represent the mean value and the value range from the sources.

Table-Y: Summary of results for sub-case SBFD#1_UMa_FR1_Sub#1.

| Simple description for the sub-case (e.g., 100dB co-site inter-sector co-channel inter-subband isolation, SBFD Alt2, 49dBm gNB Tx power, Twice area&same TxRUs, DL: 4Kbytes, UL: 1Kbyte, UE clustering,…) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT (Mbps) | Mean | {mean, <min~max>} = {3%, <-10%~20%>} |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Average-UPT (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 |  |  |  |  |  |  |  |  |  |
|  | Type-2 |  |  |  |  |  |  |  |  |  |
| UL RU (%) | Type-1 |  |  |  |  |  |  |  |  |  |
|  | Type-2 |  |  |  |  |  |  |  |  |  |
| Note: - For Average-UPT / Packet-Latency / RU, the gain can be calculated as: Gain (%) = SBFD UPT/latency/RUTDD UPT/latency/RU-1 |  |  |  |  |  |  |  |  |  |  |

[One Example for observation]: For subcase SBFD#1_UMa_FR1_Sub#1, assuming UE clustering distribution, 100dB co-site inter-sector co-channel inter-subband isolation, …, key findings are summarized below.

- DL performance of SBFD compared with legacy TDD

  - The mean value of DL average-UPT CDF of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

  - The 5% of DL average-UPT CDF of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

  - The mean value of DL packet-latency CDF of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

  - The 5% of DL packet-latency CDF of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

  - The DL Type-1 RU of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

  - The DL Type-2 RU of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

- UL performance of SBFD compared with legacy TDD

  - The mean value of UL average-UPT CDF of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

  - The 5% of UL average-UPT CDF of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

  - The mean value of UL packet-latency CDF of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

  - The 5% of UL packet-latency CDF of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

  - The UL Type-1 RU of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

  - The UL Type-2 RU of SBFD is increased/decreased by around XX%, with the value range as {XX%~XX%}

7.1.1.2.2	: SBFD#1_UMa_FR1_Sub#2

…

Annex D: Evaluation results for SBFD

Annex D.1: SBFD deployment cases 1

Annex D.1.1: Indoor office-FR1

…

Annex D.1.2: Urban Macro-FR1

Note: Considering there are a lot of performance metrics for UPT and latency, companies may not provide the results for all the metrics. Moderator recommends companies to at least provide Average-UPT CDF related metrics for drawing conclusions, and other metrics (e.g., Tail-UPT, Median-UPT, UE Average-Latency) are up to companies. In addition, moderator recommends companies to at least provide relative gains of SBFD compared to legacy TDD for UPT and latency related metrics in the following table.

Table D.1.2: Evaluation results for Urban Macro in FR1 in SBFD Deployment Case 1

| Tdoc/Source | Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
|  | DL Average-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Tail-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Median-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Average-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Tail-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Median-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | Type-1 RU (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Type-2 RU (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Unfinished/dropped Packet Rate (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Additional comments: e.g., Layout and UE distribution - Macro Layer: e.g., Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - UE distribution: e.g., UE clustering distribution with M=20, X=2 Interference Modelling - gNB self-interference: e.g., based on 1 dB UL desense - Co-site inter-sector co-channel inter-subband CLI: e.g., 100dB (spatial isolation), 10dB digital isolation - UE-UE co-channel inter-subband CLI: e.g., 33 dBc SBFD subband and slot configuration - SBFD slot configuration: Alt 2 (higher priority): Legacy TDD: {DDDSU}; SBFD: {XXXXU} - SBFD Subband configuration: e.g., <ND, NU, NG >=<104, 55, 5> - Guard symbol number: - UL resource percentage per TDD period (%): - DL resource percentage per TDD period (%): BS transmit power & antenna configuration - BS transmit power for legacy TDD: e.g., 53dBm - BS transmit power for SBFD: e.g., Option-1: Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems) - BS antenna configuration for legacy TDD: e.g., (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) , (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization - BS antenna configuration for SBFD: e.g., Twice area&same TxRUs (higher priority): SBFD antenna configuration Option 2 - BS antenna radiation pattern: e.g., Table 9 in Report ITU-R M.2412 - UE antenna configuration: e.g., 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization; 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization Traffic Model - DL/UL traffic assignment for the same UE: e.g., Option 2: Each UE is assigned both UL traffic and DL traffic - DL/UL FTP packet size: 4Kbytes for DL and 1Kbyte for UL Channel model - gNB-gNB: e.g., Both Large scale fading and small scale fading - UE-UE: e.g., Large scale fading only - UE-UE details: e.g., TR 38.901 Others - Open loop power control parameters: e.g., P0= -80 dBm, alpha = 0.8 - UE receiver: e.g., MMSE-IRC - Channel estimation: e.g., Ideal - Transmission scheme: e.g., SU-MIMO - Overhead: |  |  |  |  |  |  |  |  |  |  |
| Tdoc/Source | Reported Parameters |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} (UL/DL resource percentage in a TDD period = {XX%, YY%} for TDD, {ZZ%, MM%} for SBFD) |  |  |  |  |  |  |  |  |
|  |  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
|  | DL Average-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Tail-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Median-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Average-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Tail-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Median-UPT CDF (Mbps) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  |  | 5% |  |  |  |  |  |  |  |  |  |
|  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 95% |  |  |  |  |  |  |  |  |  |
|  | Type-1 RU (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Type-2 RU (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Unfinished/dropped Packet Rate (%) | DL |  |  |  |  |  |  |  |  |  |
|  |  | UL |  |  |  |  |  |  |  |  |  |
|  | Additional comments: e.g., Layout and UE distribution - Macro Layer: e.g., Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - UE distribution: e.g., UE clustering distribution with M=20, X=2 Interference Modelling - gNB self-interference: e.g., based on 1 dB UL desense - Co-site inter-sector co-channel inter-subband CLI: e.g., 100dB (spatial isolation), 10dB digital isolation - UE-UE co-channel inter-subband CLI: e.g., 33 dBc SBFD subband and slot configuration - SBFD slot configuration: Alt 4: Legacy TDD: {DDDSU}; SBFD: {XXXXX} - SBFD Subband configuration: e.g., <ND, NU, NG >=<104, 55, 5> - Guard symbol number: - UL resource percentage per TDD period (%): - DL resource percentage per TDD period (%): BS transmit power & antenna configuration - BS transmit power for legacy TDD: e.g., 53dBm - BS transmit power for SBFD: e.g., Option-1: Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems) - BS antenna configuration for legacy TDD: e.g., (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) , (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization - BS antenna configuration for SBFD: e.g., Twice area&same TxRUs (higher priority): SBFD antenna configuration Option 2 - BS antenna radiation pattern: e.g., Table 9 in Report ITU-R M.2412 - UE antenna configuration: e.g., 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization; 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization Traffic Model - DL/UL traffic assignment for the same UE: e.g., Option 2: Each UE is assigned both UL traffic and DL traffic - DL/UL FTP packet size: 4Kbytes for DL and 1Kbyte for UL Channel model - gNB-gNB: e.g., Both Large scale fading and small scale fading - UE-UE: e.g., Large scale fading only - UE-UE details: e.g., TR 38.901 Others - Open loop power control parameters: e.g., P0= -80 dBm, alpha = 0.8 - UE receiver: e.g., MMSE-IRC - Channel estimation: e.g., Ideal - Transmission scheme: e.g., SU-MIMO - Overhead: |  |  |  |  |  |  |  |  |  |  |

……

- Annex B: Link budget template in Table A.3 in TR 38.830

Table B.1  Link budget template (refer to Table A.3 in TR 38.830 with modification)

| System configuration |  |
| --- | --- |
| Channel for evaluation | PUSCH FFS: PUCCH |
| Scenarios and Carrier frequency (GHz) | For FR1: - Urban 4 GHz For FR2: - Urban 30 GHz |
| BS antenna heights (m) | 25m |
| UT antenna heights (m) | Reported by companies, 1.5m can be used as a starting point. |
| Cell area reliability (%) | Reported by companies, 95% for control channel, 90% for data channel can be used as a starting point. |
| Lognormal shadow fading std deviation (dB) | Reported by companies |
| Tx Diversity | Reported by companies |
| Number of SSB | Reported by companies |
| Transmitter |  |
| (1) Number of transmit antenna elements | For FR1 UE: - 2 For FR2 UE: - 32 |
| (2) Number of transmit TxRUs Note: this row is void (left empty) for uplink |  |
| (2a) Number of transmit chains modelled in LLS | For FR1 UE: - Option 1: 1 UE RF chains in LLS. - Option 2 (Optional): Number of UE RF chains = number of TXRUs in LLS For FR2 UE: - Option 1: 1, 2 UE RF chains in LLS. - Option 2: Number of UE RF chains = number of TXRUs in LLS. |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | For FR1 UE: - 23 dBm for UE For FR2 UE: - 23 dBm for UE |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | For uplink: - Occupied bandwidth is reported by companies |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: no PSD constraint for uplink |  |
| (3c) Bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. For uplink, (3a) = (3c) |  |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) |  |
| (4) Total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) – (4b) (dB) |  |
| (4a) Antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4c) + 10 log ( (1) / (2a) ) (dB) for uplink |  |
| (4b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB) | Reported by companies |
| (4c) Gain of antenna element (dBi) | For UE: - 0 dBi for FR1 - 5 dBi for FR2 |
| (5) Total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink |  |
| (5a) Antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) Note: zero for uplink |  |
| (5b) Antenna gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink | Reported by companies |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | Reported by companies |
| (9) EIRP = (3bis) + (4) + (5) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antenna elements | For FR1 BS: 128 antenna elements For FR2 BS: - 512 antenna elements |
| (10a) Number of receive TxRUs Note: this row is void (empty) for downlink | FR1 BS: - 32TxRUs per panel FR2 BS: - 2 TxRUs per panel |
| (10b) Number of receive chains modelled in LLS | For FR1 BS: - Option 1: 2 or 4 or 8 gNB RF chains per panel in LLS. - Option 2 (optional): Number of gNB receive chains = number of TXRUs FR2 BS: - 2 gNB RF chains per panel in LLS |
| (11) Total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB) |  |
| (11a) Antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11c) + 10 log ( (10)/(10a) ) (dB) for uplink |  |
| (11b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB) | Reported by companies |
| (11c) Gain of antenna element (dBi) | For BS: - 8 dBi or reported by companies |
| (11bis) Total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink |  |
| (11bis-a) Antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for downlink |  |
| (11bis-b) Antenna gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | Reported by companies |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | Reported by companies |
| (13) Receiver noise figure (dB) | Reported by companies |
| (14) Thermal noise density (dBm/Hz) | Reported by companies |
| (15) Receiver interference density (dBm/Hz) | Reported by companies |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) |  |
| (18) Effective noise power = (16) + 10 log ((3c)) (dBm) |  |
| (19) Required SNR (dB) |  |
| (20) Receiver implementation margin (dB) | Reported by companies |
| (21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS | Reported by companies |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) |  |
| (22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB) | (22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB) |
| (23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) − (12) − (22) (dB) Note: MIL can also be derived by (22bis) + (4) – (8) + (11) − (12) | (23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) − (12) − (22) (dB) Note: MIL can also be derived by (22bis) + (4) – (8) + (11) − (12) |
| Calculation of available pathloss |  |
| (25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation) (dB) | Reported by companies |
| (26) BS selection/macro-diversity gain (dB) | Reported by companies |
| (27) Penetration margin (dB) | Reported by companies |
| (28) Other gains (dB) (if any please specify) | Reported by companies |
| (29) MPL: Available path loss = (23) – (25) + (26) – (27) + (28) (dB) |  |
| Range/coverage efficiency calculation |  |
| (30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m) |  |

- Annex C: Link budget analysis

Table B.1  Link budget analysis

| System configuration | CMCC PUSCH for O2I-UMa-FR1 |  |  |  |
| --- | --- | --- | --- | --- |
| Channel for evaluation | Legacy TDD | SBFD for Low Load | SBFD for Medium Load | SBFD for High Load |
| Scenarios and Carrier frequency (GHz) | 4 | 4 | 4 | 4 |
| BS antenna heights (m) | 25 | 25 | 25 | 25 |
| UT antenna heights (m) | 1.5 | 1.5 | 1.5 | 1.5 |
| Cell area reliability (%) | - | - | - | - |
| Lognormal shadow fading std deviation (dB) | - | - | - | - |
| Tx Diversity | - | - | - | - |
| Number of SSB | - | - | - | - |
| Transmitter |  |  |  |  |
| (1) Number of transmit antenna elements | 2 | 2 | 2 | 2 |
| (2) Number of transmit TxRUs Note: this row is void (left empty) for uplink | - | - | - | - |
| (2a) Number of transmit chains modelled in LLS | 1 | 1 | 1 | 1 |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | 23 | 23 | 23 | 23 |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 8640000 (24 RB) | 8640000 (24 RB) | 8640000 (24 RB) | 8640000 (24 RB) |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) | 13.63 | 13.63 | 13.63 | 13.63 |
| (3c) Bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. For uplink, (3a) = (3c) | 8640000 | 8640000 | 8640000 | 8640000 |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) | 23 | 23 | 23 | 23 |
| (4) Total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) – (4b) (dB) | 3.01 | 3.01 | 3.01 | 3.01 |
| (4a) Antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4c) + 10 log ( (1) / (2) ) (dB) for downlink, and = (4c) + 10 log ( (1) / (2a) ) (dB) for uplink | 3.01 | 3.01 | 3.01 | 3.01 |
| (4b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB) | 0 | 0 | 0 | 0 |
| (4c) Gain of antenna element (dBi) | 0 | 0 | 0 | 0 |
| (5) Total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink | 0 | 0 | 0 | 0 |
| (5a) Antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) Note: zero for uplink | 0 | 0 | 0 | 0 |
| (5b) Antenna gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink | 0 | 0 | 0 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 1 | 1 | 1 | 1 |
| (9) EIRP = (3bis) + (4) + (5) – (8) dBm | 25.01 | 25.01 | 25.01 | 25.01 |
| Receiver |  |  |  |  |
| (10) Number of receive antenna elements | 128 | 128 | 128 | 128 |
| (10a) Number of receive TxRUs Note: this row is void (empty) for downlink | 32 | 32 | 32 | 32 |
| (10b) Number of receive chains modelled in LLS | 8 | 8 | 8 | 8 |
| (11) Total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB) | 14.02 | 14.02 | 14.02 | 14.02 |
| (11a) Antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11c) + 10 log ( (10)/(10a) ) (dB) for uplink = (11c) + 10 log ( (10)/(10b) ) (dB) for downlink | 14.02 | 14.02 | 14.02 | 14.02 |
| (11b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB) | 0 | 0 | 0 | 0 |
| (11c) Gain of antenna element (dBi) | 8 | 8 | 8 | 8 |
| (11bis) Total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink | 6.02 | 6.02 | 6.02 | 6.02 |
| (11bis-a) Antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for downlink | 6.02 | 6.02 | 6.02 | 6.02 |
| (11bis-b) Antenna gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | 0 | 0 | 0 | 0 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 3 | 3 | 3 | 3 |
| (13) Receiver noise figure (dB) | 5 | 5 | 5 | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 |
| (15) Receiver interference density (dBm/Hz) | -1E+99 | -1E+99 | -1E+99 | -1E+99 |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) | -169 | -169 | -169 | -169 |
| (18) Effective noise power = (16) + 10 log ((3c)) (dBm) | -99.63 | -99.63 | -99.63 | -99.63 |
| (19) Required SNR (dB) | -7.8 | -11.97 | -11.38 | -10.75 |
| (20) Receiver implementation margin (dB) | 2 | 2 | 2 | 2 |
| (21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS | 0.5 | 0.5 | 0.5 | 0.5 |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) | -105.9 | -110.1 | -109.5 | -108.9 |
| (22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB) | 128.9 | 133.1 | 132.5 | 131.9 |
| (23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) - (12) - (22) (dB) Note: MIL can also be derived by (22bis) + (4) – (8) + (11) - (12) | 142.0 | 146.1 | 145.5 | 144.9 |
| Calculation of available pathloss |  |  |  |  |
| (25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation) (dB) | 4.48 | 4.48 | 4.48 | 4.48 |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 | 0 | 0 |
| (27) Penetration margin (dB) | 26.3 | 26.3 | 26.3 | 26.3 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 | 0 | 0 |
| (29) MPL: Available path loss = (23) – (25) + (26) – (27) + (28) (dB) | 111.24 | 115.41 | 114.82 | 114.19 |
| Range/coverage efficiency calculation |  |  |  |  |
| (30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m) | 155.3 | 198.6 | 191.8 | 184.8 |