3GPP TSG RAN WG1 #112-bis-e			                        R1-2303234

e-Meeting, April 17th – April 26th, 2023

Source: 	CMCC

Title:	Discussion on potential enhancements on flexible/dynamic TDD

Agenda item:	9.3.3

Document for:	Discussion & Decision

- Introduction

In last RAN1 meeting, the following agreements were made [1]:

Agreement

For the study of gNB-to-gNB co-channel interference measurement, it is assumed that both CD-SSB and NCD-SSB can be used for gNB-to-gNB CLI measurement.

Agreement

Study the benefit of knowledge among gNBs of configurations such as

- SBFD time/frequency configuration

Agreement

For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, DL Tx beam information of the gNB can be exchanged between gNBs. Reference signal resource ID (e.g., NZP-CSI-RS resource ID, SSB index) can be used as beam information exchange between gNBs.

Agreement

For L1/L2 based UE-to-UE CLI measurement, SRS-RSRP and CLI-RSSI are to be further studied as baseline metrics.

Agreement

For the study of L1/L2 based UE-to-UE co-channel CLI measurement, measurement resource for CLI-RSSI measurement as defined in Rel-16 and SRS resource for SRS-RSRP measurement as defined in Rel-16 can be considered. Enhancement of measurement resource can be studied.

Agreement

For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the following measurement and report framework.

- Use existing CSI framework as the baseline.

- Others are not precluded.

Agreement

For spatial domain enhancement of gNB-to-gNB CLI handling, study the benefit and the procedure of the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs, based on the beam information exchanged between gNBs.

In this contribution, the study of inter-UE intra-subband and inter-gNB intra-subband CLI handling on flexible/dynamic TDD will be discussed.

- Inter-UE CLI handling enhancement

2.1 L1/L2 CLI measurement and reporting

In the last meeting, companies agreed to use the existing CSI framework as the baseline for L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism. In the existing CSI measurement and reporting framework, the CSI measurement RS is configured and transmitted by gNB and UE will measure the CSI resource and report on configured or indicated PUCCH/PUSCH resource. However, for L1/L2 UE-to-UE CLI measurement and reporting, the inter-UE CLI measurement SRS is transmitted by inter-cell aggressor UEs. Then victim UE will report measurement results to its serving gNB. Therefore, enhancements to the existing CSI measurement and reporting framework should be considered for CLI measurement and reporting.

In the CSI framework, the CSI reporting can be periodic, semi-persistent, and aperiodic. Similarly, for L1/L2 UE-to-UE CLI reporting, periodic, semi-persistent, and aperiodic reporting can be considered.

1) Periodic reporting:

Victim UE measures the pre-configured periodic CLI measurement resource and reports the measurement results to its serving gNB via periodic PUCCH resources. In this scheme, gNB can acquire the measurement results immediately but the measurement resource and report resource overhead are also very huge because each victim UE may need to perform extensive measurements and reports corresponding to lots of aggressor UEs. Therefore, how to reduce the inter-UE CLI report overhead, e.g., CLI measurements results content compression should be studied for L1/L2 based inter-UE CLI periodic reporting.

Proposal 1: For L1/L2 based periodic inter-UE CLI reporting, how to reduce the CLI report overhead should be studied.

2) Aperiodic reporting:

Similar to aperiodic CSI feedback, gNB can transmit DCI signaling to trigger UE’s inter-UE CLI measurement and report on PUSCH resources. Compared with periodic schemes, aperiodic schemes can reduce the measurement and report resource overhead since gNB can trigger the CLI reporting when needed, but this scheme will lead to increased latency and reduced efficiency of inter-UE CLI measurement and report. That is because gNB does not know the exact inter-UE CLI situation in real time and the trigger time of inter-UE CLI measurement and report may not match the exact time when inter-UE CLI occurs.

For the CLI measurement resource, periodic, semi-persistent, or aperiodic measurement resources can all be considered. However, how to trigger aperiodic CLI reporting is a critical issue that needs further study. In legacy aperiodic CSI report with aperiodic CSI-RS transmission, a single DCI signaling is used to trigger CSI-RS as well as indicate the reported PUSCH resource. However, in inter-UE CLI measurement and reporting, the UE that transmits CLI SRS is different from the UE that measures the CLI SRS. If UE-specific signaling is used to trigger SRS transmission or indicate CLI report resource, the overall signaling overhead may be huge in a large UE number scenario.

In addition, in order to perform efficient CLI measurement and report for inter-cell UE, UE’s SRS configurations should be exchanged among gNBs over the Xn/F1 interface, which may lead to a large processing delay. Therefore, the effect of information exchange time between gNBs on L1/L2 based UE-to-UE CLI measurement and reporting should be studied.

3) Semi-persistent reporting:

Similar to semi-persistent CSI feedback, gNB can transmit MAC CE or DCI signaling to activate/trigger UE’s inter-UE CLI measurement and report on PUCCH/PUSCH resource. For the CLI measurement resource, periodic and semi-persistent measurement resource can be considered. Semi-persistent reporting suffers from similar issues as aperiodic reporting. Therefore, detailed triggering signaling for semi-persistent and aperiodic CLI reports and methods to reduce signaling overhead should be studied.

Proposal 2: For L1/L2 based semi-persistent/aperiodic inter-UE CLI reporting, the information exchange time between gNBs should be considered for the design of CLI measurement and reporting timeline.

Proposal 3: For L1/L2 based semi-persistent/aperiodic inter-UE CLI reporting, the detailed SRS resource triggering signaling and report resource indication signaling should be studied, as well as how to reduce the signaling overhead.

4) Event-triggered reporting:

In Rel-16 L3 inter-UE CLI handling, both periodic and event-triggered report are supported. Similarly, the event-triggered scheme can be supported for L1/L2 based UE-to-UE CLI measurement and report.

In this scheme, the measurement report is triggered by events, e.g., the inter-UE CLI is higher than a threshold. The report resource overhead will be reduced than periodic reporting because UE only needs to report the CLI measurement results on some occasions, which is the most efficient way to let gNB acquire the accurate inter-UE CLI measurement results. Therefore, the L1/L2 based event triggered reporting should also be supported in inter-UE CLI measurement and reporting.

Different from pre-configured PUCCH resource in periodic reporting and the indicated PUSCH resource in semi-persistent/aperiodic reporting, one thing that needs further study is how to indicate gNB the inter-UE CLI report request and the report resource configuration by L1/L2 signaling. For L1 based event triggered reporting, SR and associated PUCCH resource can be used, for example, UE first informs gNB of the CLI measurement reporting request on SR resource and then reports the measurement results on associated PUCCH resource. For L2 based event triggered reporting, UE can use MAC-CE on CG PUSCH to report the measurement results.

Proposal 4: For L1/L2 based inter-UE CLI measurement and reporting, event triggered reporting can be supported. The following reporting triggering method can be further studied as examples:

- For L1 based event triggered reporting, SR resource can be used for UE to inform gNB the CLI measurement results reporting and PUCCH can be used as reporting resource.

- Foe L2 based event triggered reporting, MAC-CE on CG PUSCH can be used by UE to convey measurement results.

In the existing CSI measurement and reporting framework, CSI may consist of CQI, PMI, CRI, SSBRI, L1-RSRP, L1-SINR, etc. When the PUCCH or PUSCH configured for CSI feedback could not contain all the CSI reports overlapping in time, some of the CSI reports will be dropped according to the priority value, which is calculated by the reporting type, carried information type, serving cell index, reporting configuration ID with pre-defined rules. Similarly, if PUCCH or PUSCH is insufficient to transmit all the CSI reports and CLI reports overlapping in time, it’s necessary to determine which measurement reports should be transmitted with higher priority.

Proposal 5: For L1/L2 based inter-UE CLI measurement and reporting, the priority of CLI reports relative to the existing CSI reports should be studied.

2.2 Coordinated scheduling

In Rel-16 CLI, only TDD configurations are exchanged between gNBs. As shown in Figure 1, in order to better support inter-vendor inter-UE CLI cooperation, the backhaul signaling can be enhanced to exchange other necessary information, including inter-UE CLI measurement resource configuration (e.g., CLI SRS or RSSI resource configurations) to assist the network to identify the aggressor UEs, and the corresponding coordinated scheduling information. For instance,

- Example 1 (2-step negotiation): As shown in Figure 1 (a),

  - Step 0 (backhaul): The serving cell for aggressor UE informs CLI-SRS resource configuration to the serving cell for victim UE

  - Step 1: The serving cell for victim UE indicates the victim UE to perform intra-subband CLI measurement, and identify the strong interference UE pair based the reported CLI-SRS-RSRP from the victim UE

  - Step 2 (backhaul): The serving cell for victim UE requests the serving cell for aggressor UE to avoid scheduling the aggressor UE (associate with certain CLI-SRS index) at certain pre-configured resources in time/frequency domain

  - Step 3: The serving cell for aggressor UE avoids scheduling the aggressor UE (associate with certain CLI-SRS index) to transmit PUSCH at the requested resources

- Example 2 (3-step negotiation): As shown in Figure 1 (b),

  - Step 0 (backhaul): The serving cell for aggressor UE informs CLI-SRS resource configuration to the serving cell for victim UE

  - Step 1: The serving cell for victim UE indicates the victim UE to perform intra-subband CLI measurement, and identify the strong interference UE pair based the reported CLI-SRS-RSRP from the victim UE

  - Step 2 (backhaul): The serving cell for victim UE requests the serving cell for aggressor UE for the scheduling information of the aggressor UE (associate with certain CLI-SRS index)

  - Step 3 (backhaul): The serving cell for aggressor UE informs the pre-choregraphed scheduling information of the aggressor UE (associate with certain CLI-SRS index) to the serving cell for victim UE

  - Step 4: The serving cell for victim UE avoids scheduling the victim UE to receive PDSCH at the informed resources

- Example 3 (1-step negotiation): As shown Figure 1 (c),

  - Step 0 (backhaul): The serving cell for aggressor UE informs CLI-SRS resource configuration as well as the corresponding pre-configured candidate PDSCH resources subset for the associated aggressor UE to the serving cell for victim UE

  - Step 1: The serving cell for victim UE indicates the victim UE to perform intra-subband CLI measurement, and identify the strong interference UE pair based the reported CLI-SRS-RSRP from the victim UE

  - Step 2: The serving cell for victim UE avoids scheduling the victim UE to receive PDSCH at the pre-configured resources subset associated with the aggressor UE

Figure 1. Coordinated scheduling for Inter-UE CLI handling

Proposal 6: For coordinated scheduling for inter-UE intra-subband CLI handling, support to enhance the backhaul signaling to exchange necessary information, e.g.,

- Example 1 (2-step negotiation): CLI-SRS resource configuration and the request for scheduling avoidance of the aggressor UE (associate with certain CLI-SRS index) at certain pre-configured resources in time/frequency domain

- Example 2 (3-step negotiation): CLI-SRS resource configuration, the request for scheduling information of the aggressor UE (associate with certain CLI-SRS index) and the information of pre-choregraphed scheduling information of the aggressor UE (associate with certain CLI-SRS index)

- Example 3 (1-step negotiation): CLI-SRS resource configuration and the corresponding pre-configured candidate DL resources subset for the associated aggressor UE

Another coordinated scheduling techniques is in spatial domain, victim UE can report the recommended beams for inter-UE CLI measurement along with the CLI measurement results reporting.

Proposal 7: For coordinated scheduling for inter-UE intra-subband CLI handling in spatial domain, victim UE can report the recommended beams along with the CLI measurement results.

2.3 UL power control

Both in SBFD system and flexible/dynamic TDD system, the inter-UE interference situation varies in different slots, e.g., the SBFD slots and normal UL slots in SBFD, and the slots with same direction or conflict direction in flexible/dynamic TDD. In current UE UL power control framework, two power control loops can be configured to UE which is a good solution to effectuate different UL power in different slot types in SBFD and flexible/dynamic TDD to avoid the severe inter-UE interference. However, some further enhancements may be needed for CG PUSCH or PUCCH/PUSCH repetition across different slot types, because only one power control loop can be configured to one CG configuration in current NR design.

Proposal 8: Two power control loops can be used for different slot types, e.g., SBFD slot and normal UL slot in SBFD system or aligned and unaligned slot in flexible/dynamic TDD system.

- FFS: Enhancements on CG PUSCH or PUCCH/PUSCH repetition across different slot types.

- Inter-gNB CLI handling enhancement

3.1 Inter-gNB CLI measurement and reporting

Some guidance of inter-gNB CLI measurement and reporting study were achieved in previous RAN1 meetings and further details are discussed as the following:

1) Measurement resource:

In previous RAN1 meetings, NZP CSI-RS/SSB were agreed as the baseline for inter-gNB CLI measurement and report. Similar to inter-UE CLI measurement, there are two types of measurement resource in inter-gNB CLI measurement, which the first type is for interference power measurement, e.g., SSB/CSI-RS, and the second type is for interference strength measurement, e.g., RSSI resource. From our point of view, RSSI-like interference strength measurement can also be supported except for interference power measurement considering the interference strength measurement complicity is simpler, e.g., don’t need the accurate timing to receive the measurement RS.

Proposal 9: For inter-gNB intra-subband CLI measurement and reporting, RSSI-like measurement can also be supported.

2) Aggressor gNB identification:

In order to identify the aggressor gNB, some coordination among gNBs is needed, especially for RSSI based interference strength measurement because the victim gNB can only measure the interference strength on these blank resources but cannot identify which and the number of gNBs cause interference on these RSSI resources. One solution for aggressor gNB identification is to coordinate the transmission of different aggressor gNBs on different RSSI measurement resources in TDM/FDM manner. That is for a given RSSI resource/occasion, only one aggressor gNB can transmit and other gNBs are muted. Therefore, the victim gNB can identify the strongest aggressor gNB by measuring the interference strength on different RSSI resources/occasions.

Proposal 10: For inter-gNB intra-subband CLI measurement and reporting, the transmission of different aggressor gNBs are coordinated on different RSSI resources/occasions in TDM/FDM manner.

3) UL resource muting:

For victim gNB, UE’s resources muting in UL transmission may be needed for more accurate inter-gNB CLI measurement when the UL transmission of target UE is overlapped with the inter-gNB CLI measurement resource. Considering the coexistence with legacy UEs, two methods can be considered, which the first is non-transparent UL resource muting method by indicating UE the UL rate-matching/muting pattern. Another method is transparent UL resource muting method by gNB’s scheduling to avoid such resource, but this method may cause the reduction of available PUSCH resources when DFT-S-OFDM is used since non-continuous resource allocation is not supported in this waveform.

Proposal 11: For inter-gNB intra-subband CLI measurement and reporting, both transparent and non-transparent UL resource muting method should be considered, e.g., define UL rate-matching/muting pattern or avoid the scheduling on measurement resource.

3.2 Coordinated scheduling

From operator’s point of view, the mainly interested and realistic scenario for dynamic/flexible TDD deployment is to apply different TDD frame structures for outdoor macro network and indoor hotspot in the same frequency carrier to fit the different UL/DL traffic statistic ratios. In this scenario, it is highly probable that the macro gNBs and indoor gNBs are from different vendors, so specification support may be needed to handle the inter-gNB CLI in this scenario. Thus, some backhaul signalling enhancement should be considered to support inter-vendor cooperation.

Proposal 12: For coordinated scheduling for inter-gNB intra-subband CLI handling, support to enhance the backhaul signaling to exchange necessary information, e.g., scheduling information in time-domain, frequency-domain and power domain.

3.3 UE and gNB transmission and reception timing

As the illustration of Figure 2, due to the TA of UE UL transmission, for victim gNB, the timing between the received UL transmission of target UEs and the CLI interference from aggressor gNB is not aligned, e.g., NTA,offset=13us, which will complicate the inter-gNB CLI mitigation as well as the UL reception. Thus, some methods need to be applied to align the timing between the received UL transmission of target UE and the CLI interference from aggressor gNB. For example, set NTA,offset=0 via information n-TimingAdvanceOffset in SIB1 or define negative NTA,offset.

Figure 2. Unalignment between UE and gNB transmission and reception timing

Proposal 13: For inter-gNB intra-subband CLI handling, UE and gNB transmission and reception timing alignment can be further studied, e.g., set NTA,offset=0 via information n-TimingAdvanceOffset or define negative NTA,offset.

- Conclusions

In this contribution, the study on potential enhancements on flexible/dynamic TDD are discussed and the following proposals are made.

Inter-UE CLI handling

Proposal 1: For L1/L2 based periodic inter-UE CLI reporting, how to reduce the CLI report overhead should be studied.

Proposal 2: For L1/L2 based semi-persistent/aperiodic inter-UE CLI reporting, the information exchange time between gNBs should be considered for the design of CLI measurement and reporting timeline.

Proposal 3: For L1/L2 based semi-persistent/aperiodic inter-UE CLI reporting, the detailed SRS resource triggering signaling and report resource indication signaling should be studied, as well as how to reduce the signaling overhead.

Proposal 4: For L1/L2 based inter-UE CLI measurement and reporting, event triggered reporting can be supported. The following reporting triggering method can be further studied as examples:

- For L1 based event triggered reporting, SR resource can be used for UE to inform gNB the CLI measurement results reporting and PUCCH can be used as reporting resource.

- Foe L2 based event triggered reporting, MAC-CE on CG PUSCH can be used by UE to convey measurement results.

Proposal 5: For L1/L2 based inter-UE CLI measurement and reporting, the priority of CLI reports relative to the existing CSI reports should be studied.

Proposal 6: For coordinated scheduling for inter-UE intra-subband CLI handling, support to enhance the backhaul signaling to exchange necessary information, e.g.,

- Example 1 (2-step negotiation): CLI-SRS resource configuration and the request for scheduling avoidance of the aggressor UE (associate with certain CLI-SRS index) at certain pre-configured resources in time/frequency domain

- Example 2 (3-step negotiation): CLI-SRS resource configuration, the request for scheduling information of the aggressor UE (associate with certain CLI-SRS index) and the information of pre-choregraphed scheduling information of the aggressor UE (associate with certain CLI-SRS index)

- Example 3 (1-step negotiation): CLI-SRS resource configuration and the corresponding pre-configured candidate DL resources subset for the associated aggressor UE

Proposal 7: For coordinated scheduling for inter-UE intra-subband CLI handling in spatial domain, victim UE can report the recommended beams along with the CLI measurement results.

Proposal 8: Two power control loops can be used for different slot types, e.g., SBFD slot and normal UL slot in SBFD system and aligned and unaligned slot in flexible/dynamic TDD system.

- FFS: Enhancements on CG PUSCH or PUCCH/PUSCH repetition across different slot types.

Inter-gNB CLI handling

Proposal 9: For inter-gNB intra-subband CLI measurement and reporting, RSSI-like measurement can also be supported.

Proposal 10: For inter-gNB intra-subband CLI measurement and reporting, the transmission of different aggressor gNBs are coordinated on different RSSI resources/occasions in TDM/FDM manner.

Proposal 11: For inter-gNB intra-subband CLI measurement and reporting, both transparent and non-transparent UL resource muting method should be considered, e.g., define UL rate-matching/muting pattern or avoid the scheduling on measurement resource.

Proposal 12: For coordinated scheduling for inter-gNB intra-subband CLI handling, support to enhance the backhaul signaling to exchange necessary information, e.g., scheduling information in time-domain, frequency-domain and power domain.

Proposal 13: For inter-gNB intra-subband CLI handling, UE and gNB transmission and reception timing alignment can be further studied, e.g., set NTA,offset=0 via information n-TimingAdvanceOffset or define negative NTA,offset.

- References

- Chairman’s notes, RAN1#112, February 27th - March 3rd, 2023.