3GPP TSG RAN WG1 #112bis-e			R1-2303237

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.4.2

Source: 	CMCC

Title: 	Discussion on co-channel coexistence for LTE sidelink and NR sidelink

Document for:	Discussion and Decision

Introduction

In RANP#99 meeting, the objective in NR sidelink evolution SID/WID [1] has been updated as follow, which leads RAN1 to continue the study and work on co-channel coexistence for LTE sidelink and NR sidelink:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

Since the RAN guidelines in above updated WID have already provided the solutions for addressing the issues caused by NR PSFCH and higher SCS. Then in this contribution we just need to focus on other remaining issues for the co-existence scenario.

Discussion

UE type and operating combination

According to the updated SID/WID, Type A devices and operating combination A will have higher priority to be considered during RAN1’s work on dynamic resource pool sharing, which means that other UE types and operating combinations are not completely precluded in Rel-18. However, considering the progress of another existing objective (SL-U) and there are two objectives which will be potentially started after RANP #98 (FR2 and CA), we propose not to consider other UE types and operating combinations at least in Rel-18.

Proposal 1: RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID only for Type A devices and operating combination A.

- Other UE types and operating combinations should not be considered at least in Rel-18.

Enhancements on NR sidelink to support the coexistence with LTE sidelink

One remaining aspect should be considered is that in LTE sidelink, the resource pool can be configured as non-adjacent which means in an LTE SL resource pool, the PSCCH resource pool is separately configured in frequency domain and is not adjacent to the PSSCH resource. Then, if a NR SL PSSCH can be transmitted in LTE SL PSCCH resource pool, relatively more collisions may occur between LTE and NR SL UEs, because one NR PSSCH may overlap with multiple LTE PSCCH transmissions. Therefore, one of the most straightforward methods is that the NR resource pool does not contain the resource in the LTE SL PSCCH resource pool in frequency domain, if the LTE SL resource pool is configured as non-adjacent.

Figure 1: Illustration of NR SL RP configuration when LTE SL RP is configured as non-adjacent

Proposal 2: If the LTE SL resource pool is configured as non-adjacent, the configuration should ensure that NR resource pool does not contain the resource in the LTE SL PSCCH resource pool in frequency domain.

In RAN1#112 meeting, the following agreement has been achieved for how the NR SL module uses the candidate information shared by the LTE SL module:

Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module FFS whether/how LTE SL RSRP is applied

From our perspective, for the RSRP threshold list, Alt 2 is the most straightforward way because it can guarantee that the LTE SL transmission is not affected by NR SL transmission. For Alt 3 and Alt 4, both of them need further workload in RAN2 to discuss the exact values in the newly introduced RSRP threshold list, which should not be preferred due to the stressful time.

Proposal 3: For the list of the initial SL RSRP threshold, Alt 2 should be selected, i.e., LTE SL RSRP threshold list (pre)configured in an LTE SL resource pool.

Regarding how to treat the LTE SL module’s own LTE SL transmission, the following agreement has been achieved in last meeting.

Agreement In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module - Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

In our view, if NR SL module needs to exclude the all the candidate resources in the slots where LTE SL module would perform LTE SL transmission, this is somehow an excessive exclusion, which may result in too few resources available for NR SL transmission, and increasing the probability of resource collisions between NR SL transmissions. For instance, anyway this issue can be addressed by Rel-16 NR SL in-device coexistence rule, so we think that Option 2 in the above agreement should be selected.

Proposal 4: NR SL module does not take the LTE SL module’s own LTE SL transmission into account during the resource selection procedure, i.e., Option 2 should be selected.

Another aspect which should be further discussed is how to treat the LTE SL resources associated with non-monitored subframe of LTE SL module, in NR SL module’s resource selection procedure, unfortunately, there is no agreements have been achieved and the latest proposal from FL is shown in below.

Draft proposal 2-3(II): - In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources associated with non-monitored subframe of LTE SL module - Option 1: - The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module - For determining the LTE SL resources associated with non-monitored subframe of LTE SL module, - All the LTE SL resources in the non-monitored subframes are assumed to be repeatedly reserved Q times for each LTE SL resource reservation period (pre)configured in a LTE SL resource pool - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module according to Step 5 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL resources associated with non-monitored subframe of LTE SL module used in the above procedure is shared from LTE SL module to NR SL module - Option 2: - The PHY layer of NR SL module does not perform NR SL candidate resource exclusion based on the information regarding LTE SL resources associated with non-monitored subframe of LTE SL module

From our perspective, if resource selection procedure does not treat the non-monitored subframe of LTE SL module, probably there will be some resource collisions between LTE SL transmissions and NR SL transmissions, which will lead to the performance degradation of both LTE SL and NR SL. Therefore, we think Option 1 is more feasible and should be selected. Moreover, to guarantee the performance of LTE SL transmission, we prefer to use the formula of Q in Section 14.1.1.6 in TS 36.213 in the resource selection procedure. Besides, we think step 5a) is still applicable after the procedure in Option 1 is further introduced, i.e.,

5a)	 If the number of candidate single-slot resources Rx,y remaining in the set SA is smaller than X⋅Mtotal, the set SA is initialized to the set of all the candidate single-slot resources as in step 4.

Proposal 5: NR SL module should take the non-monitored subframe of LTE SL module into account during the resource selection procedure, i.e., Option 1 should be selected.

- The formula of Q in Section 14.1.1.6 in TS 36.213 should be used;

- Step 5a) is still applicable after the procedure in Option 1 is further introduced.

Conclusions

In this contribution, we have shared our views on the remaining issues of supporting dynamic resource sharing b/w LTE SL and NR SL transmission, the following proposals are provided:

Proposal 1: RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID only for Type A devices and operating combination A.

- Other UE types and operating combinations should not be considered at least in Rel-18.

Proposal 2: If the LTE SL resource pool is configured as non-adjacent, the configuration should ensure that NR resource pool does not contain the resource in the LTE SL PSCCH resource pool in frequency domain.

Proposal 3: For the list of the initial SL RSRP threshold, Alt 2 should be selected, i.e., LTE SL RSRP threshold list (pre)configured in an LTE SL resource pool.

Proposal 4: NR SL module does not take the LTE SL module’s own LTE SL transmission into account during the resource selection procedure, i.e., Option 2 should be selected.

Proposal 5: NR SL module should take the non-monitored subframe of LTE SL module into account during the resource selection procedure, i.e., Option 1 should be selected.

- The formula of Q in Section 14.1.1.6 in TS 36.213 should be used;

- Step 5a) is still applicable after the procedure in Option 1 is further introduced.

References

- RP-230077, WID revision: NR sidelink evolution, 3GPP TSG RAN Meeting #99