3GPP TSG RAN WG1 #112bis-e			R1-2303239

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.5.1.1

Source: 	CMCC

Title: 	Discussion on SL positioning reference signal

Document for:	Discussion and Decision

Introduction

In RAN#98-e meeting, a new WID on expanded and improved NR positioning was approved [1], in which the application of NR positioning is further expanded to enable sidelink positioning and ranging, improved accuracy, integrity and power efficiency, and positioning for RedCap UEs. To support sidelink positioning and ranging, the following objectives were identified:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we further provide our views on SL PRS design based on the outcomes of RAN1#112 meeting [2].

SL PRS design

- 2.1 Sequence design

In the last RAN1 meeting, the following agreements were achieved regarding sequence design for SL PRS:

Agreement SL PRS sequence is generated based on Gold sequence: rn=121-2⋅c2n+j121-2⋅c2n+1, where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211. Agreement - For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS. - The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined. Agreement Range of the parameter nID,seqSL-PRS is: nID,seqSL-PRS∈0,1,…,4095

From the agreements, one remaining issue was how to determine the sequence ID and 7 options were listed for further study. Regarding these options, Option 1 basically follows the configuration logic of the sequence ID for DL PRS, Option 2 reuses the sequence ID derivation mechanism for reference signals in sidelink, other options are combinations or enhancements of sequence ID configuration in Uu and/or sidelink. In our views, the configuration of sequence ID should consider at least two aspects, one is to provide sufficient orthogonality and the other is to ensure the privacy (i.e., the sequence ID is not required to be identified by all the surrounding UEs). From the former perspective, Option 6 should not be pursued, since we have agreed that the range of the sequence ID is {0,…,4095}. From the later perspective, Options 2/5/7 share the same issue that any UE can obtain the sequence ID of SL PRS. Hence, we are open to further discuss Option 1 and Option 3.

In addition, regarding Option 1 and Option 3, we think that the understanding on higher layer, whether it is per UE configured or per resource pool (pre-)configured should be clarified. From our understanding, similar as the sequence ID configuration for DL PRS, the sequence ID for SL PRS can be higher layer configured per UE.

Proposal 1: For SL PRS generation, down-select between the following two options, and Option 1 is slightly preferred:

  - Option 1: nID,seqSL-PRS is a higher layer configured parameter

  - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission

Proposal 2: The sequence ID of SL PRS should be higher layer configured per UE.

- 2.2 SL PRS configuration

In RAN1#112 meeting, the following agreements were made regarding the SL PRS configuration structure, including whether SL PRS PFL is needed and the numerology of SL PRS:

Agreement A SL PFL is not defined. SL positioning RS are defined directly with respect to and contained within a single SL BWP and carrier. Agreement Support SCS values for SL PRS include: - 15 kHz, 30 kHz, 60 kHz for FR1, and 60 kHz, 120 kHz for FR2 - Which SCS values are required, and which ones are optional follow Rel-16 UE capabilities.

### 2.2.1 SL PRS resource and/or resource set

During the meeting, the issue on whether to define SL PRS resources and/or resource sets were discussed without consensus reached, and the latest proposal was recapped as follows:

### [Medium] FL1 Proposal 2.1-2 - As a starting point, SL PRS resources and SL PRS resource sets are defined similar to the definitions for DL PRS resources and DL PRS resource sets respectively. - FFS: Detailed definitions and configuration parameters.

In our views, the motivation to introduce SL PRS resource set(s) are not clear. For the configuration of reference signals in Uu, such as DL PRS and UL SRS, the reason to introduce resource set is that resources within a resource set share same parameters such as resource type, periodicity and slot offset, etc. In sidelink, however, these configurations per resource set level is not necessary. At least, SL PRS resources should be defined, and configuration details are further discussed in the following section.

Proposal 3: At least support to define SL PRS resources similar to the definitions for DL PRS resources.

### 2.2.2 SL PRS configuration

For the configuration/activation/deactivation/triggering of SL-PRS, the following two options were agreed for further down-selection in SI:

Agreement With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options: - Option 1: High-layer-only signaling involvement in the SL-PRS configuration - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS. - Based on the study, this option may correspond to - A SL-PRS configuration that is a single-shot or multiple shots - A high-layer configuration that may be received from an LMF, a gNB, or a UE - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/ activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS.

Before making the down-selection between option 1 and option 2, two questions should be clarified since the current descriptions in the above agreements may cause some confusions among companies. Some understanding should be aligned during RAN1 discussion in order to select a better option.

Q1:  What is the definition of high-layer configuration for SL-PRS?

In our views, there are several categories of high-layer configuration. One is as in NR sidelink, some parameters for sidelink data transmission is up to BWP level configuration, e.g., StartSymbol and LengthSymbol for PSSCH transmission, or resource pool level configuration, e.g., PSSCH-DMRS-TimePattern, which is (pre-)configured by the NW. Another is as for SL CSI-RS, the configuration is conveyed by PC5 RRC signalling. In addition, the SL-PRS configuration may also depend on SLPP signalling, which is under the discussion in RAN2 and follows the Uu positioning principle. In our views, all these configuration methods are not precluded.

Observation 1: The high-level configuration signalling of SL-PRS may include one or a combination of resource pool/BWP level configuration, PC5-RRC signalling or SLPP signalling which is potentially designed in RAN2.

Note that since no gNB scheduling is included in Scheme 2 resource allocation, the SL PRS configuration in a dedicated RP cannot be supported as so much flexible as NR DL PRS and/or UL SRS pos; otherwise, some big issues such as resource collision, AGC performance degradation, etc., may be caused, and these issues may have negative impacts on the positioning performance. In this sense, we propose to (pre-)configure parameters of SL PRS in a resource pool level, including at least SL PRS pattern (i.e., number of symbols, comb size) and the bandwidth of the SL PRS.

Proposal 4: Parameters of SL-PRS, e.g., SL PRS pattern (incl. number of symbols, comb size), and bandwidth, should be (pre-)configured in a resource pool level.

The second question regarding the agreement on SL PRS configuration/activation/deactivation/triggering is that:

Q2:  What is the definition of low-layer activation/indication/reservation/triggering for SL-PRS?

First of all, from our understanding, activation/deactivation mechanism does not exist in sidelink because this kind of semi-persistent mechanism by sending MAC-CE is not supported. Next, SCI is used to indicate the PSSCH transmission within the same slot of the PSCCH, for the UE(s) with a specific destination ID, and it can also reserve further resources for the subsequent transmission. From a receiver UE’s perspective, its PSSCH reception is based on the decoded SCI within the same slot, and is achieved by identifying that it is the intended receiver of the corresponding destination ID indicated in the SCI; however, the resource reservation is only for other transmitter UE’s resource exclusion procedure, not the receiver UE’s reception because the transmission may be dropped even the resource has been reserved, or the reserved resource may also be changed by the pre-emption procedure.

Observation 2: In NR sidelink, SCI is used to indicate the PSSCH transmission within a same slot of PSCCH, and is also used to reserve further resource for subsequent transmission.

Observation 3: In NR sidelink, from the perspective of receiver UE(s), its reception is achieved by identifying that it is the intended receiver of the corresponding destination ID indicated in the SCI within the same slot; not based on the reservation information of a transmitter UE.

Observation 4: In NR sidelink, from the perspective of other transmitter UE(s), the resource reservation information is used for resource exclusion procedure.

Based on the above analysis, there are two alternatives for SL-PRS reception. The first one is to follow NR sidelink principle, then the aforementioned mechanism for NR sidelink data should be reused; the second one is to follow NR Uu principle, where SCI can be used as “triggering” for one or more transmissions to notify the receiver UE the corresponding resources to perform reception, in this case one SCI can be used to trigger multiple PSSCH transmission, i.e., there is no need to transmit a PSCCH along with each PSSCH. However, since the SL resources may need to be changed, more collision and redundant reservation issues will be caused accordingly.

Moreover, regardless of which alternatives above is finally selected, option 2 in the agreement should be agreed since SCI has been involved in both of the two alternatives.

Proposal 5: RAN1 should further study the following alternatives for SL-PRS reception:

- Alt 1: The NR sidelink principle is followed.

  - SCI is used to indicate the current PSSCH for receiver UE, and reserve subsequent resources for the resource exclusion procedure of other transmitter UE(s) only.

- Alt 2: The NR Uu principle is followed.

  - SCI is used to trigger one or more transmissions and to notify the receiver UE where to perform resource reception.

Proposal 6: With regards to the configuration/activation/deactivation/triggering of SL-PRS, option 2 in the agreements from RAN1#109-e should be selected.

### 2.2.3 SL PRS bandwidth

Regarding the bandwidth of SL PRS, the issue on the configurations of SL PRS BW in shared RP was discussed; however, no agreement was reached. The latest intermediate proposal was provided in the following. In addition, the SL PRS BW in dedicated RP should be further discussed as well.

### [Medium] FL3 Proposal 2.3-3 - For shared resource pools, - SL PRS BW can be smaller than resource pool BW. - FFS: SL PRS BW is same as the BW indicated for PSSCH if/when PSSCH is present in a shared resource pool.

For the bandwidth of SL-PRS in a dedicated RP, the following agreement was made during the study item:

Agreement At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

In our views, it should be directly equal to the bandwidth of the resource pool, in order to provide higher positioning accuracy. Regarding Alt. 1, some companies preferred to leave configuration flexibility of the SL PRS BW to be smaller than that of the resource pool, to our understanding, it may scatter the resources and further reduce the resource utilization efficiency. Therefore, we propose to support Alt. 2, i.e., the SL PRS BW in the dedicated RP should be the same as that of the resource pool.

Proposal 7: To improve the positioning accuracy, it can be considered defining the BW of SL-PRS to be equal to that of the resource pool (Support Alt. 2).

- 2.3 SL PRS slot structure

In R16/17 NR sidelink, only slot-based scheduling is specified. In this sense, slot-based scheduling granularity for SL-PRS should be supported as starting point, and FFS whether sub-slot granularity can be considered, which further details can be found in Section 2.5.

Proposal 8: Slot-based scheduling granularity for SL-PRS is supported as the starting point.

- FFS: Sub-slot scheduling granularity for SL-PRS.

For the slot-based slot structure of SL-PRS, the discussion is related to the issue of which channels can be included in the resource pool in addition to SL-PRS. During the last RAN1 meeting, the following agreement was made under AI 9.5.1.3 that option 1, i.e., no other channel in addition to SL PRS can be included, is not pursued further:

- Agreement For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

Based on the above agreement, we think that the following SL PRS slot structure should be adopted as a starting point. Similar as the design principle in sidelink, PSCCH occupies 2 or 3 symbols. In addition, the 1st symbol of PSCCH and SL-PRS should be used as the AGC symbol, which is the duplication of the 2nd symbol of the channel/RS. A gap symbol for Rx-Tx turnaround should be included at the end of the SL-PRS symbol.

Figure 1: Illustration of SL PRS slot structure consists of SL-PRS and its associated PSCCH

Proposal 9: A SL-PRS slot should at least consist of SL PRS and its associated PSCCH, and it includes:

- PSCCH occupies 2 or 3 symbols.

- The 1st symbol of SL-PRS should be used as the AGC symbol, which is the duplication of the 2nd symbol of SL-PRS.

- A GAP symbol for Rx-Tx turnaround is also required immediately after the last symbol of SL-PRS.

- 2.4 Mapping to physical resources

During the last RAN1 meeting, the following agreements were made regarding the SL PRS patterns:

Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering.

For the comb size N of the SL PRS, there were FFS bullets on whether comb size 6 is supported for shared RP and whether comb size 1 is supported for dedicated RP. Our views are that it is not necessary to make distinctions of the supported comb size for different resource pools. First, regarding whether comb size 6 is supported for shared RP, during the last RAN1 meeting, some companies argued that supporting comb size 6 may cause large Tx power variance for different symbols and propose not to support comb 6 in the shared RP. We think that this issue can be solved by enhancing SL PRS power control, which is within the scope of this AI. In addition, regarding whether comb size 1 is supported for dedicated RP, opponents argued that it may cause SL PRS symbol waste in a dedicated RP. However, we think that this issue can be resolved if the SL PRS pattern (pre-)configured per resource pool level is supported (e.g., by proper configuring starting symbol and length of the dedicated resource pool, or by enabling TDM-based multiplexing of SL PRS resources from different UEs). Furthermore, regarding the FFS bullet on whether comb size 8 is supported, considering the slot structure illustrated in Figure 1, as PSCCH occupying 2 or 3 symbols (as defined in sidelink), 2 AGC symbols with one for PSCCH and the other for SL-PRS, and 1 GAP symbol at the end of SL-PRS, a full staggering patten with N = 8 and M = 8 can be supported.

Proposal 10: For SL-PRS in shared and dedicated resource pools:

- Comb size 6 is supported in shared resource pool.

- Comb size 1 is supported in dedicated resource pool.

- Comb size 8 is supported.

For the number of SL PRS symbols M excluding AGC and GAP symbols, there was an FFS bullet on the constraints on maximum effective comb size. The distance corresponding to the search window without ambiguity can be computed as follows:

W(m)=c*TsymbolComb Size=cComb Size*SCS(Hz

The distances with respect to different SCS and comb size are listed in the following table:

| SCS | Comb 1 | Comb 2 | Comb 4 | Comb 6 | Comb 8 | Comb 12 |
| --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 20000m | 10000m | 5000m | 3333.33m | 2500m | 1666.67m |
| 30kHz | 10000m | 5000m | 2500m | 1666.67m | 1250m | 833.33m |
| 60kHz | 5000m | 2500m | 1250m | 833.33m | 625m | 416.67m |

For a typical use case of sidelink positioning, such as V2X, the communication distance between UEs can be 2.5 times of the relative moving speed (i.e., 2~300 meters), and the maximum delay spread for a highway or outdoor scenario can be up to several hundreds of ns, therefore, we think that on a safer side, a maximum effective comb size [6] with a minimum ambiguity window of 833 m can be supported without ambiguity of timing measurements. The following partial staggering (M,N) pairs with uniform sampling in frequency domain can be further supported: (M, N) = (1, 6), (2, 6), (1, 4), (2, 8).

Proposal 11: For SL PRS in shared and dedicated resource pools:

- At least a maximum effective comb size [6] can be supported.

- SL PRS patterns with partial staggering are further supported for the following (M,N) pairs:

    - (M, 6) with M = {1, 2}

    - (M, 4) with M = {1}

    - (M, 8) with M = {2}

Furthermore, the issue on whether non-consecutive symbols SL-PRS can be supported for shared resource pool was raised during the study item phase. The proponents’ view to support the non-consecutive symbols of SL-PRS is the SL-PRS may overlap with other physical channel and signal in time domain, e.g., DMRS, PSCCH, etc. In our view, however, take SRS in Uu as an example, in this case, the symbols of SRS overlaps with other channel/signal are just “punctured” due to priority rule, which does not mean the symbols of this RS itself are “non-consecutive”. On the other hand, even if shared resource pool is included in the WID scope, this condition can still be avoided by proper configuration, for example, when SL-PRS and SL data are symbol level semi-statically TDMed in a shared resource pool (details please refer to our companion’s paper [3]), in that case, the issue will never exist.

Proposal 12: RAN1 does not consider non-consecutive symbols SL-PRS for shared resource pool.

- 2.5 SL PRS resource multiplexing

In RAN1#112 meeting, the following agreements were achieved for SL PRS resource multiplexing including both comb-based multiplexing and TDMed multiplexing:

Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability

For comb-based multiplexing of SL PRS from different UEs in a slot, we prefer to only support a single (M,N) value. If multiple (M,N) values are allowed, it may increase the resource selection complexity and reduce the resource utilization efficiency, while no significant advantages can be observed.

Proposal 13: For comb-based multiplexing of SL PRS from different UEs in a slot, only support a single (M,N) value.

Regarding the TDM-based multiplexing of SL PRS from different UEs in a slot, it was agreed to support it at least for dedicated resource pools and other details were left for FFS including the resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, the applicable resource allocation schemes, etc.

Regarding the 2nd FFS bullet, to enable TDM-based multiplexing of SL PRS from different UEs, as different SL PRS resources within a slot are transmitted by different Tx UEs, from a Rx UE perspective, the reception powers on different symbols may varies significantly. To solve this issue, there may have two possible solutions, which is also related to the SL PRS slot structure. One is that all the PSCCH associated with SL PRS resources from different UEs within a same slot is located at the beginning of the 2 or 3 symbols of the SL PRS slot, and for each SL PRS resource from a UE, the 1st symbol is used for AGC purpose which is the duplication of the 2nd symbol of the SL PRS resource, as shown in Figure 2(a). Note that a UE capability to adjust AGC within a slot has already been supported in NR sidelink. For a sidelink slot contains PSFCH, a sidelink UE is able to adjust AGC within a slot to receive PSCCH/PSSCH and PSFCH. Alternatively, another solution is that the PSCCH is of its associated SL PRS resources are located together, in which power control of PSCCH can be enhanced to ensure that the Tx power of symbols carrying PSCCH is the same as that carrying its associated SL PRS resource, and no additional AGC symbol is needed in front of each SL PRS resource, as shown in Figure 2(b). However, it should be noted that such UE capability of a sub-slot level PSCCH blind detection is not supported in NR sidelink and corresponding UE feature should be enhanced if such slot structure is introduced to support TDM-based multiplexing of SL PRS resources from different UEs.

Figure 2: Illustration of SL PRS slot structure of TDM-based multiplexing of SL PRS resources from different UEs within a slot

Observation 5: For TDM-based multiplexing of SL PRS resources from different UEs within a slot, introducing AGC symbols at the beginning of each SL PRS resource can solve the reception power variation on different symbols, and corresponding UE capability to perform sub-slot level AGC has been supported in NR sidelink.

Observation 6: For TDM-based multiplexing of SL PRS resources from different UEs within a slot, the solution to locate PSCCH and its associated SL PRS resource from a UE together within a slot can solve the reception power variation on different symbols but corresponding UE capability to perform sub-slot level PSCCH blind detection has not been supported in NR sidelink.

Proposal 14: For the TDM-based multiplexing of SL PRS resources from different UEs within a slot, the following two alternatives should be considered to solve the potential reception power variation on different symbols:

- Alt. 1: The PSCCH of associated SL PRS resources from different UEs are located at the first 2 or 3 symbols within the SL PRS slot. For each SL PRS resource, the 1st symbol is used as the AGC symbol which is the duplication of the 2nd symbol.

- Alt. 2: The PSCCH of its own associated SL PRS resource are located together within the SL PRS slot.

    - FFS details on PSCCH power control to maintain same Tx power of symbols carrying PSCCH and that carrying its associated SL PRS resource.

    - UE capability of sub-slot level PSCCH blind detection should be further enhanced.

Regarding the 4th FFS bullet, as in sidelink, only slot-based scheduling and resource allocation have been specified, to support TDM-based multiplexing of SL PRS from different UEs, it may have large specification impact, at least on Scheme 2 resource allocation mechanism. For example, issues including whether to support TDM-based multiplexing from different UEs in a slot using a single or multiple (M,N) pair(s), how to determine the candidate resource set, the potential issues on resource scattering and the reduction of resource utilization efficiency, etc, should be further studied and introduce large workload in RAN1. Our preference is to treat TDM-based multiplexing of SL PRS from different UEs for resource allocation Scheme 1 as 1st priority and to treat that for resource allocation Scheme 2 as 2nd priority.

Proposal 15: Considering TDM-based multiplexing of SL PRS from different UEs for resource allocation Scheme 1 as 1st priority and for resource allocation Scheme 2 as 2nd priority.

- 2.6 Power control

In the last RAN1 meeting, the following agreement was generally made on the transmit power control of SL PRS:

Agreement The OLPC framework defined for PSSCH/PSCCH is considered as a starting point for OLPC for SL PRS.

In NR sidelink, OLPC based on DL pathloss and/or SL pathloss are specified. For the power control of SL-PRS, to mitigate the potential interference to both UL or SL, we think that same mechanism of NR sidelink OPLC should be supported as the starting point, i.e., supporting both DL pathloss-based and SL pathloss-based OLPC

Proposal 16: Both DL pathloss-based and SL pathloss-based OLPC should be supported for SL-PRS.

In addition, for the PSCCH associated with the SL PRS, the power control mechanism should be considered. In NR sidelink, the transmit power of the PSCCH is determined based on its associated PSSCH. A straightforward solution is that the transmit power of the PSCCH is determined based on its associated SL PRS as well. Details including its impact on resource exclusion can be further studied.

Proposal 17: The transmit power of the PSCCH is determined based on its associated SL PRS. FFS details.

Conclusions

In this contribution, we have shared our views on SL positioning reference signal, the following observations and proposals are provided:

Observation 1: The high-level configuration signalling of SL-PRS may include one or a combination of resource pool/BWP level configuration, PC5-RRC signalling or SLPP signalling which is potentially designed in RAN2.

Observation 2: In NR sidelink, SCI is used to indicate the PSSCH transmission within a same slot of PSCCH, and is also used to reserve further resource for subsequent transmission.

Observation 3: In NR sidelink, from the perspective of receiver UE(s), its reception is achieved by identifying that it is the intended receiver of the corresponding destination ID indicated in the SCI within the same slot; not based on the reservation information of a transmitter UE.

Observation 4: In NR sidelink, from the perspective of other transmitter UE(s), the resource reservation information is used for resource exclusion procedure.

Observation 5: For TDM-based multiplexing of SL PRS resources from different UEs within a slot, introducing AGC symbols at the beginning of each SL PRS resource to solve the reception power variation on different symbols and corresponding UE capability to perform sub-slot level AGC has been supported in NR sidelink.

Observation 6: For TDM-based multiplexing of SL PRS resources from different UEs within a slot, the solution to locate PSCCH and its associated SL PRS resource from a UE together within a slot to solve the reception power variation on different symbols and corresponding UE capability to perform sub-slot level PSCCH blind detection has not been supported in NR sidelink.

Proposal 1: For SL PRS generation, down-select between the following two options, and Option 1 is slightly preferred:

  - Option 1: nID,seqSL-PRS is a higher layer configured parameter

  - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission

Proposal 2: The sequence ID of SL PRS should be higher layer configured per UE.

Proposal 3: At least support to define SL PRS resources similar to the definitions for DL PRS resources.

Proposal 4: Parameters of SL-PRS, e.g., SL PRS pattern (incl. number of symbols, comb size), and bandwidth, should be (pre-)configured in a resource pool level.

Proposal 5: RAN1 should further study the following alternatives for SL-PRS reception:

- Alt 1: The NR sidelink principle is followed.

  - SCI is used to indicate the current PSSCH for receiver UE, and reserve subsequent resources for the resource exclusion procedure of other transmitter UE(s) only.

- Alt 2: The NR Uu principle is followed.

  - SCI is used to trigger one or more transmissions and to notify the receiver UE where to perform resource reception.

Proposal 6: With regards to the configuration/activation/deactivation/triggering of SL-PRS, option 2 in the agreements from RAN1#109-e should be selected.

Proposal 7: To improve the positioning accuracy, it can be considered defining the BW of SL-PRS to be equal to that of the resource pool (Support Alt. 2).

Proposal 8: Slot-based scheduling granularity for SL-PRS is supported as the starting point.

- FFS: Sub-slot scheduling granularity for SL-PRS.

Proposal 9: A SL-PRS slot should at least consist of SL PRS and its associated PSCCH, and it includes:

- PSCCH occupies 2 or 3 symbols.

- The 1st symbol of SL-PRS should be used as the AGC symbol, which is the duplication of the 2nd symbol of SL-PRS.

- A GAP symbol for Rx-Tx turnaround is also required immediately after the last symbol of SL-PRS.

Proposal 10: For SL-PRS in shared and dedicated resource pools:

- Comb size 6 is supported in shared resource pool.

- Comb size 1 is supported in dedicated resource pool.

- Comb size 8 is supported.

Proposal 11: For SL PRS in shared and dedicated resource pools:

- At least a maximum effective comb size [6] can be supported.

- SL PRS patterns with partial staggering are further supported for the following (M,N) pairs:

    - (M, 6) with M = {1, 2}

    - (M, 4) with M = {1}

    - (M, 8) with M = {2}

Proposal 12: RAN1 does not consider non-consecutive symbols SL-PRS for shared resource pool.

Proposal 13: For comb-based multiplexing of SL PRS from different UEs in a slot, only support a single (M,N) value.

Proposal 14: For the TDM-based multiplexing of SL PRS resources from different UEs within a slot, the following two alternatives should be considered to solve the potential reception power variation on different symbols:

- Alt. 1: The PSCCH of associated SL PRS resources from different UEs are located at the first 2 or 3 symbols within the SL PRS slot. For each SL PRS resource, the 1st symbol is used as the AGC symbol which is the duplication of the 2nd symbol.

- Alt. 2: The PSCCH of its own associated SL PRS resource are located together within the SL PRS slot.

    - FFS details on PSCCH power control to maintain same Tx power of symbols carrying PSCCH and that carrying its associated SL PRS resource.

    - UE capability of sub-slot level PSCCH blind detection should be further enhanced.

Proposal 15: Considering TDM-based multiplexing of SL PRS from different UEs for resource allocation Scheme 1 as 1st priority and for resource allocation Scheme 2 as 2nd priority.

Proposal 16: Both DL pathloss-based and SL pathloss-based OLPC should be supported for SL-PRS.

Proposal 17: The transmit power of the PSCCH is determined based on its associated SL PRS. FFS details.

References

- RP-223549, New WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, Ericsson, 3GPP TSG RAN Meeting 98-e.

- RAN1 Chair’s Note, 3GPP RAN1#112 v20.

- R1-2303241, Discussion on resource allocation for SL positioning reference signal, CMCC, 3GPP RAN1#112bis meeting.