3GPP TSG RAN WG1 #112bis-e			                                    R1-2303247

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.7.1

Title:	Discussion on network energy saving techniques in spatial and power domains

Source:	CMCC

Document for:	Discussion and Decision

# 1. Introduction

In the RAN1#112 meeting, the techniques of network energy saving from the spatial domain and power domains was discussed. A few agreements have been achieved [1]. The details of the agreements are listed in the corresponding sections.

In this contribution, we provide our views on the network energy saving techniques in spatial and power domains.

# 2. Network energy saving techniques in spatial domain

It was agreed in the last meeting that both Type 1 and Type 2 spatial domain adaptation are supported for the NES. The agreement is as below. In the Type 1 spatial adaptation, if the elements are turned off, the corresponding logical antenna ports will be closed. UE cannot detect or measure the antenna ports which are closed. In the Type 2 spatial adaptation, part or the subset of the antenna elements would be turned off, the corresponding antenna ports still exist. But due to the antenna elements change, the antenna pattern would change which may have impacts to the CSI and TCI states.

Agreement For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases - Type 1: all antenna elements associated to a logical antenna port is disabled/enabled - Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

## 2.1 Enhancements on measurement for CSI and beam management

When gNB mutes some spatial elements for network energy saving, the corresponding PAs may be turned off that leads to power backoff, and/or the corresponding antenna elements may be turned off that leads to EIRP back off, which finally lead to the power backoff of signal transmission, including CSI-RS and PDSCH, etc. Dynamic spatial elements on/off may result in changes to the transmission power of the reference signal or channel, which may further introduce additional measurement and reporting problem for UE.

- Firstly, the CSI-RS measurement and L3 related measurements may be affected. As shown in Fig. 1, when timeRestrictionForChannelMeasurements for CSI-RS measurement is set to "notConfigured", UE derives the channel measurements for computing CSI value reported in uplink slot n based on multiple CSI-RS resources no later than the CSI reference resource. If gNB turns off some spatial elements at slot n0, it may lead to the CSI-RS transmission power backoff after slot n0. If UE does not know the power changing of CSI-RS resource before and after slot n0, UE still derives the CSI value by averaging the CSI-RS measurements before and after slot n0, the CSI value may be not corrected. On one hand, the timing of spatial elements on/off, or the timing restriction of measurements can be configured to UE to indicate that the CSI-RS transmitted before TxRUs on/off cannot be used for measurements. However, this may result in less CSI-RS for measurements, or if the timing of spatial elements on/off is later than CSI reference resource, none of CSI-RS can be used for measurements. On the other hand, some enhancements can be studied to enable UE to jointly measure the CSI-RS transmitted before and after spatial elements on/off. Similarly, the L3 related measurements, such as PL RS measurement, may also have such problem.

Figure 1. CSI-RS measurement with timeRestrictionForChannelMeasurements set to "notConfigured"

- Secondly, beam failure recovery or radio link monitoring procedure may be affected. For beam failure recovery procedure, UE assesses the radio link quality according to periodic CSI-RS and/or SSB against the threshold Qout,LR for beam failure detection, and UE finds new candidate beam according to the L1-RSRP measurements of periodic CSI-RS and/or SSB that are larger than or equal to the threshold Qin,LR, where Qout,LR and Qin,LR correspond to the value of rlmInSyncOutOfSyncThreshold and rsrp-ThresholdSSB configured by RRC, respectively. If gNB turns off some spatial elements, it may lead to CSI-RS transmission power backoff. If the threshold is not changed accordingly, the power backoff may leads to the L1-RSRP of CSI-RS less than the threshold Qout,LR, especially for cell-edge UEs, and results in frequently beam failure detection, or UE could not finds new candidate beam that larger than or equal to the threshold Qin,LR, and results in frequently contention-based initial access procedure. Hence, the threshold Qout,LR and threshold Qin,LR may be needed to update together with spatial elements on/off. Similarly, radio link monitoring procedure may also have such problem.

- Thirdly, dynamic spatial elements on/off may also have impact on CSI feedback. For CSI reporting, UE performs channel estimation and SVD decomposition based on CSI-RS, and reports CQI, RI, and PMI in CSI reporting. However, if gNB turns off part of spatial elements for network energy saving, such as 64 TxRUs mutes to 32 TxRUs, whether the 8-port CSI-RS transmitted through 64 TxRUs or through 32 TxRUs has impact on the CQI, RI, or PMI reporting can be discussed. Enhancements can be considered to enable adaptation of CQI, RI, or PMI calculation with spatial elements on/off.

Proposal 1:

Enhancements can be studied to enable UE to jointly measure CSI-RS or PL RS transmitted before and after spatial elements on/off.

Proposal 2:

Threshold for beam failure recovery or radio link monitoring may be needed to update together with spatial elements on/off.

Proposal 3:

Enhancements on adaptation of CQI, RI, or PMI calculation with spatial elements on/off.

## 2.2 Signaling to enable dynamic logical port adaptation for CSI-RS

In current network, the overhead of CSI-RS transmission is non-negligible, especially for periodic CSI-RS. Reducing the overhead of CSI-RS transmission is essential for network energy saving. Currently, the ports number and RE location of CSI-RS within a slot is configured by RRC. If gNB turns off some spatial elements that cannot transmit all the ports of CSI-RS, RRC reconfiguration is needed for CSI-RS adaptation, which may lead to extra delay for network energy saving. Dynamic switching and the indication of different spatial domain adaptations can reduce the delay and overhead caused by RRC reconfiguration.

For example, as shown in Fig. 2, an 8-port CSI-RS is configured to UE, if gNB turns off 64 TxRUs to 32 TxRUs, the 8-port CSI-RS can still be transmitted. But the antenna pattern of the 8 port CSI-RS would be changed, which may induce the change of the QCL state. The dynamic indication of the change of spatial adaptation pattern would let the UE know that the CSI would be changed due to the change of the antenna elements or the TxRUs. On the other hand, if gNB turns off the 8 TxRUs to 2 TxRUs, then only 2-port CSI-RS can be transmitted. In this case, the dynamic indication of the spatial adaptation would let the UE know that the CSI-RS ports has changed.

For aperiodic CSI-RS, multiple CSI-RS resource sets are configured by RRC, DCI could trigger a suitable CSI-RS resource set, so, enhancement is not needed. For semi-persistent and periodic CSI-RS, multiple port configurations can be semi-statically configured by RRC for each CSI-RS resource, and MAC CE or DCI can be used for dynamic CSI-RS port adaptation.

Figure 2. CSI-RS port adaptation for network energy saving

Proposal 4:

Dynamic adaptation for CSI-RS should be supported for semi-persistent and periodic CSI-RS.

## 2.3 Enhancements on CSI reporting

CSI reporting enhancement can be considered for UE to provide antenna muting pattern recommendations to gNB, which ensures the performance of UE while achieving network energy saving gain. One approach is that different CSI reporting can be configured to UE with different antenna muting pattern assumptions. However, the number of CSI-reportConfig including P/SP/A-CSI reportconfig per BWP is 4. If coupling of CSI reporting and antenna muting pattern for network energy saving, the flexibility of CSI reporting configuration will be restricted.

Another approach is that UE reports multiple CSIs with different antenna muting pattern assumptions in one CSI reporting. In this case, multiple CSI-RS resources can be configured with different antenna muting pattern assumption. However, this may lead to higher CSI-RS overhead. Another way under this approach is that multiple CSI-RS port patterns can be configured to represent different antenna muting pattern assumptions. In this case, the UE can report multiple CSIs corresponding to multiple CSI-RS port patterns in one CSI reporting. And the multiple CSIs within the CSI reporting could be used for the recommendation of muting pattern to gNB.

Proposal 5:

UE reports multiple CSIs with different antenna muting pattern assumptions in one CSI reporting.

Proposal 6:

The multiple CSIs within the CSI reporting could be used for the recommendation of muting pattern to gNB.

In the last meeting, two agreements related to the CSI report configuration and CSI-RS resource setting have been achieved.

Agreement For spatial element adaptation, further study the following - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern - FFS: Details on how the association is done - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns - FFS: Details on how the association is done - FFS: Details on the definition of “spatial adaptation patterns” Agreement For spatial element adaptation, further study the following - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern - FFS: Details of sub-configuration Agreement For spatial and power domain adaptation, solution(s) based on adaptation within an active BWP is considered as baseline.

For the CSI report configuration, two alternatives were discussed. Alt 2-1 is independent/separated CSI report configuration are supported where each CSI report configuration corresponds to one spatial adaptation configuration pattern. As discussed above, currently, the number of CSI-reportConfig including P/SP/A-CSI reportConfig per BWP is 4. And the legacy UE CSI/CSI-RS capabilities cannot be extended according to the WID [2]. Then if the one CSI report configuration can only corresponds to one spatial adaptation, it is very limited for CSI report configuration to support both legacy CSI reporting and the NES related CSI reporting. Then it is more flexible to support one CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation patterns.

Proposal 7:

One CSI report configuration containing multiple CSI report sub-configuration where each sub-configuration corresponds to one spatial adaptation pattern, i.e. Alt 2-2, should be supported.

For the association between CSI-RS resource configuration (CSI-RS resource/resource set/resource setting) and spatial adaptation pattern, if each CSI-RS resource configuration can only associate with one spatial adaptation pattern, multiple CSI-RS resource configurations should be configured to support the multiple adaptation patterns. And the mapping between the CSI-RS resources within different configurations which corresponds to different adaptation patterns should be defined. Not only relation between the spatial adaptation patterns, e.g. un-muted patten and muted pattern, but also the relation between the CSI-RS resources should be clarified and defined in the specifications, which greatly increase the specification efforts.

If each CSI-RS configuration can be associated with one or more spatial adaptations, within one CSI-RS resource set, multiple resources can associate with multiple spatial adaptation patterns. And also multiple CSI-RS ports within one CSI-RS resource can be associated with multiple spatial adaptation patterns. For example, 32 CSI-RS ports are supported for the un-muted spatial pattern and a subset of the 32 ports could be used to present the muted spatial pattern for the network energy saving. Thus, the Alt 1-2 is preferred.

Proposal 8:

Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns, i.e. A1-2, should be supported.

## 2.4 Enhancements on beam management

When gNB turns on/off the spatial elements for network energy saving, the beam of reference signal or channel may be changed. As shown in Fig. 3, CSI-RS is generated by 32 TxRUs originally, when gNB turns on 32 TxRUs to 64 TxRUs, CSI-RS may be generated by 64 TxRUs and the beam of CSI-RS may be narrowed down. If CSI-RS is used as the reference RS of TCI states, CSI-RS 1 may be the most suitable beam originally, when the beam of CSI-RS is narrowed down, the most suitable beam for UE may be changed to CSI-RS 2. In this case, the indicated TCI states may be needed to update simultaneously with the adaptation of spatial elements.

Figure 3. TCI states adaptation for network energy saving

Proposal 9:

TCI states may be needed to update simultaneously with the adaptation of spatial elements.

## 2.5 Enhancement on uplink power control

For the spatial element adaption, whether uplink and downlink spatial adaption can be applied separately needs to be discussed. For example, to realize energy saving, gNB can mute part of spatial elements for downlink transmission, but still use all of the spatial elements for uplink reception. If this case is allowed, the pathloss estimation based on downlink reference signals may be impacted.

Since UE estimates the pathloss based on PathlossReferenceRS, when part of downlink spatial elements are turned off, the transmitted power of PathlossReferenceRS may also be reduced. If the reduced power of dynamic adaption is not informed to UE, and UE still use the semi-static RRC configured power to calculate the pathloss, i.e. pathloss = referenceSignalPower – higher layer filtered RSRP, the pathloss estimation will be higher than actual pathloss. When UE control the uplink PUSCH, PUCCH and SRS transmission power based on the calculated pathloss, while gNB still uses all the spatial elements for uplink reception, the uplink power of such channels will be higher than the actual needed power. To solve this problem, uplink power control enhancement is needed to get a correct uplink transmission power for separate uplink and downlink spatial adaption case.

Proposal 10:

Uplink power control enhancement is needed for separate uplink and downlink spatial adaption case.

- Network energy saving techniques in power domain

In the last meeting, a few agreements have been achieved for the power domain enhancements.

Agreement For adaptation of power offset values between PDSCH and CSI-RS, further study the following - Where/how to configure multiple power offset values - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS - Whether other UE report content can be included

## 3.1 Dynamic indication of powerControlOffset

As specified in TS 38.214, the power offset between PDSCH EPRE and CSI-RS EPRE is given by parameter powerControlOffset provided by higher layers. UE receives the CSI-RS and calculates CSI based on the configured ratio. When gNB dynamically reduces the transmission power of PDSCH, the value of powerControlOffset can be also updated accordingly, to obtain more accurate CSI reporting for gNB to determine compatible RI, MCS and precoder. Multiple values could be configured for different power saving mode or scenarios.

Both MAC CE and DCI can be considered for the indication of the power offset values. MAC CE based indication or activation could be used to update the power offset assumptions which will be used for the following transmission and the CSI reporting. And DCI based indication has the most flexibilities and could also be used for the aperiodic CSI reporting. It could also be considered that the dynamic indication of the power offset in the DCI to facilitate the PDSCH demodulation of the UE. The signaling of dynamic power offset indication could be further discussed. And whether UE specific indication or the group common indication for multiple UEs can be further discussed.

Proposal 11:

Multiple power offset values can be configured and dynamically indicated and/or activated.

Proposal 12:

Dynamic indication the value of powerControlOffset can be applied for the adaptation of PDSCH transmission power.

## 3.2 Enhancements on CSI reporting

Similarly, as the CSI reporting enhancement for spatial domain, potential feedback from UE to provide assistance information for adjustment of transmission power can be considered for network energy saving, which ensures the performance of UE while achieving network energy saving gain. One approach is that different CSI reporting can be configured to UE with different transmission power assumptions, however, the number of CSI-reportConfig including P/SP/A-CSI reportconfig per BWP is 4, if coupling of CSI reporting and transmission power related parameters for network energy saving, the flexibility of CSI reporting configuration will be restricted. Another approach is that UE reports multiple CSIs with different transmission power assumptions in one CSI reporting. One CSI report can contains multiple CSIs under different transmit power assumptions. The common part of multiple CSIs could be concise and indicated in one field for all the CSI reports. The delta parts could be present with additional indication of which assumption is used. In this case, multiple CSI-RS resources can be configured with different transmission power assumptions, and CSI reporting enhancement can be used for UE to feedback one or multiple CQI, LI-RSRP, L1-SINR, or PHR like value to indicate the power headroom for gNB to adjust DL transmission power.

Proposal 13:

CSI reporting enhancement can be considered for gNB to adjust DL transmission power.

Proposal 14:

Multiple CSI reports within one CSI reporting should be supported at least for the power domain enhancements.

## 3.3 Joint adaptation of spatial domain and power domain configurations

In addition to the above discussion of spatial domain and power domain enhancement, joint adaptation of spatial domain and power domain configurations to avoid coverage loss can be also considered. If some of the spatial elements are turned off, the corresponding PAs may be turned off that leads to power backoff, and/or the corresponding antenna elements may be turned off that leads to EIRP back off, which finally lead to the reduction of gNB maximum output power. Therefore, in order to keep the coverage, gNB may adjust the power configuration to achieve an acceptable power spectrum density level. Then, joint adaptation of spatial and power domain configurations is beneficial and flexible for network energy saving. It is possible to have separate indication of spatial and power configurations. However, from the cell level, the adaptation of spatial domain and power domain will affect all UEs in the cell, then, separate indication may increase the indication overhead and cause PDCCH/PDSCH congestion. Besides, separate indication may result in different activation time of spatial domain and power domain, and cause performance degradation for a period of time. Therefore, joint adaptation of spatial domain and power domain configurations can be considered to avoid coverage loss.

Proposal 15:

Joint adaptation of spatial domain and power domain configurations can be considered to avoid coverage loss

# 4. Conclusion

In this contribution, we provide our views on the network energy saving techniques in spatial and power domains. The proposals are as follows:

Spatial domain:

Proposal 1:

Enhancements can be studied to enable UE to jointly measure CSI-RS or PL RS transmitted before and after spatial elements on/off.

Proposal 2:

Threshold for beam failure recovery or radio link monitoring may be needed to update together with spatial elements on/off.

Proposal 3:

Enhancements on adaptation of CQI, RI, or PMI calculation with spatial elements on/off.

Proposal 4:

Dynamic adaptation for CSI-RS should be supported for semi-persistent and periodic CSI-RS.

Proposal 5:

UE reports multiple CSIs with different antenna muting pattern assumptions in one CSI reporting.

Proposal 6:

The multiple CSIs within the CSI reporting could be used for the recommendation of muting pattern to gNB.

Proposal 7:

One CSI report configuration containing multiple CSI report sub-configuration where each sub-configuration corresponds to one spatial adaptation pattern, i.e. Alt 2-2, should be supported.

Proposal 8:

Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns, i.e. A1-2, should be supported.

Proposal 9:

TCI states may be needed to update simultaneously with the adaptation of spatial elements.

Proposal 10:

Uplink power control enhancement is needed for separate uplink and downlink spatial adaption case.

Power domain:

Proposal 11:

Multiple power offset values can be configured and dynamically indicated and/or activated.

Proposal 12:

Dynamic indication the value of powerControlOffset can be applied for the adaptation of PDSCH transmission power.

Proposal 13:

CSI reporting enhancement can be considered for gNB to adjust DL transmission power.

Proposal 14:

Multiple CSI reports within one CSI reporting should be supported at least for the power domain enhancements.

Proposal 15:

Joint adaptation of spatial domain and power domain configurations can be considered to avoid coverage loss

# 5. References

- Chair’s notes RAN1#112, 3GPP TSG RAN WG1 #112, Athens, Greece, February 27th – March 3rd, 2023

- RP-223540, “New WID: Network energy savings for NR”, 3GPP RAN#98-e, December 2022.

# 6. Annex A: WID

The objectives of the work item are the following: - Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]