3GPP TSG RAN WG1 #112bis-e	                              R1-2303250

e-Meeting, April 17th – April 26th, 2023

Source: 	CMCC

Title:	Discussion on coverage enhancement for NR NTN

Agenda item:	9.9.1

Document for:	Discussion & Decision

- Introduction

During RAN#94-e and RAN#95-e meeting, a new work item is proposed to define further enhancements for NR NTN, and it is concluded that the coverage enhancement of PUCCH for Msg4 HARQ-ACK and discussion of DMRS-bundling for PUSCH will be focused on during previous RAN plenary meetings according to the observation and evaluation on the NTN coverage performance [1]. The justification and objectives that the work item focuses on are listed as follows.

Justification In Release 17, a work item was carried out to define solutions enabling New Radio and NG-RAN to support Non-Terrestrial Networks (NTN): - Transparent payload based GSO and NGSO network scenarios addressing at least 3GPP power class 3 UE with GNSS capability in both Earth fixed and/or moving cell configurations As part of Release 18, a new work item is proposed to define enhancements for NG-RAN based Non-Terrestrial Networks in order to: - Support new scenarios to cover deployments in frequency bands above 10 GHz - Offer optimized performance especially when addressing handset terminals (including smartphones with more realistic assumptions on antenna gains instead of 0 dBi antenna gain with the specific realistic antenna gain assumption to be determined at the working group level) w.r.t. coverage considering the NTN characteristics such as large propagation delay and satellite movement. - Provide mobility and service continuity enhancements considering the NTN characteristics such as large propagation delay and satellite movement. - Address requirements, if needed based on the FS_NR_NTN_netw_verif_UE_loc study outcome, which mandate the network operator to cross check the UE location reported by the UE, which needs to be carried out in order to fulfil the regulatory requirements (e.g., Lawful intercept, emergency call, Public Warning System, …) regarding a network verified UE location i.e., to be able to check the UE reported location information (e.g. estimate UE location at the network side) and specify if needed mechanisms to fulfil the regulatory requirements. Objective The work item aims at specifying enhancements for NG-RAN based NTN (non-terrestrial networks) according to the following assumptions with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios: - GSO and NGSO (LEO and MEO) with transparent payload. - Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS capabilities - Both “VSAT” devices with directive antenna (including fixed and moving platform mounted devices and commercial handset terminals (e.g. Power class 3) are supported in FR1 - Only “VSAT” devices with directive antenna (including fixed and moving platform mounted devices) are supported in above 10 GHz bands. Note 1: In Rel-17 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion). Note 2: The Rel-17 NTN architecture is assumed. This WI is adding functionality in REL-18 to enhance features that were introduced in REL-15/16/17: The first objective is to study the coverage enhancement to NTN, including: The Rel-18 NTN objectives are focused on the applicability of the “solutions developed by general NR coverage enhancement” (NR_cov_enh) to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). - Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

During last RAN1 meeting, the detailed procedure and issues on coverage enhancement schemes for NTN scenarios were discussed and some progress was achieved [2]. In this contribution, we will further study and share our views on the detailed coverage enhancement schemes for PUCCH enhancements for Msg4 HARQ-ACK and consideration of DMRS bundling for PUSCH in NTN.

- Discussion

  - 2.1 PUCCH enhancement for Msg4 HARQ-ACK

### 2.1.1 Repetition request or capability report

Regarding the enhancement scheme of PUCCH repetition for Msg4 HARQ-ACK in NTN scenarios, the procedure of PUCCH repetition for Msg4 HARQ-ACK was discussed and it is concluded that one or more repetition factors can be configured via SIB in the working assumption shown below[3].

RAN1#111 Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - One or more repetition factors may be configured via SIB - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor - FFS: whether UE requests repetition or indicates repetition capability - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB - FFS: whether UE requests repetition or indicates repetition capability - FFS: whether repetition factor is indicated by UE - FFS: UE behavior when repetition factor is not configured via SIB - FFS: whether one or more UE capabilities are needed for the above is for further discussion

For the cases when one or multiple repetition factors are configured, it is efficient that UE indicates its repetition request in advance to enable gNB know that the UE needs to transmit PUCCH repetition via corresponding PUCCH resource, or report its repetition capability to inform gNB the UE capability of performing PUCCH repetition for Msg4 HARQ-ACK.

Regarding whether UE requests repetition or indicates repetition capability, the main difference is the UE which has repetition capability but does not request for the former manner. In this case, UE may report repetition capability after RACH procedure to let gNB have a better understanding. It is recommended in the last meeting that both request repetition and report repetition capability can be taken into account based on some conditions, e.g., the configuration of SSB RSRP threshold, which is aligned with Rel-17 Msg3 PUSCH repetition mechanism. The detailed RSRP threshold and conditions needs to be further discussed based on the working assumption made in the RAN1#112 meeting.

RAN1#112 Working assumption - For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

From our understanding, the value of RSRP threshold for determining whether UE requests repetition or reports capability can also be configured as RSRP-Range = 127 (infinity). If the RSRP threshold is configured among the candidates of RSRP-Range, UE can request Msg4 HARQ-ACK repetition when the measured RSRP is lower than the configured RSRP threshold. Otherwise, if the RSRP threshold is not configured, or configured as the infinity value, UE is allowed to report its capability on PUCCH repetition for Msg4 HARQ-ACK.

Another issue is when UE requests PUCCH repetition for HARQ-ACK, whether additional configured RSRP threshold or the same RSRP threshold as R17 Msg3 repetition should be used for UE requesting repetitions. Considering that the payload of Msg4 HARQ-ACK is usually smaller than that of Msg3, and the coverage performance is different between PUCCH Msg4 HARQ-ACK and Msg3, it is reasonable that UE decides whether to request PUCCH repetition and Rel-17 Msg3 repetition based on different RSRP threshold. Thus, separate RSRP threshold can be configured for requesting repetition of Msg4 HARQ-ACK and PUSCH Msg3 respectively.

- Proposal 1. For PUCCH repetition for Msg4 HARQ-ACK,

- A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB.

  - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold.

  - If the RSRP threshold is not configured, or if the configured RSRP threshold is X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK.

  - The value of X (the maximum configurable value of the RSRP threshold) is the infinity value, e.g., RSRP-Range = 127.

  - For PUCCH repetition request for Msg4 HARQ-ACK, different RSRP threshold from R17 Msg3 repetition can be configured.

- Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report.

### 2.1.2 How UE transmits information

For the issue on how UE requests repetition before Msg4 transmission, considering the UL transmissions in a RACH procedure are the Msg1 and the Msg3 transmission, the following options for the repetition request or capability report indicated by UE can be further discussed.

RAN1#112 Working assumption - For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g. DMRS ports

For Option A, the mechanism of Rel-17 Msg3 repetition can be reused for PUCCH repetition for Msg4 HARQ-ACK. That is UE requests Msg4 HARQ-ACK repetition or reports capability in Msg1 by using a specific preamble when measured SSB-RSRP is lower than the threshold, which is a straightforward way to transmit repetition related indications. While there are some concerns on further PRACH fragmentation in a shared RACH occasion and the impact of R18 NR coverage enhancements for PRACH may also be taken into consideration. To avoid further partitioning of PRACH resource, it is possible for UE to transmit repetition indication of Msg3 PUSCH and Msg4 HARQ-ACK by sharing the same allocated PRACH resource based on the pre-defined mapping rule. However, using the shared PRACH resource is based on the assumption that both Rel-17 Msg3 repetition and PUCCH repetition for Msg4 HARQ-ACK may be required due to suffering the poor channel condition in some cases. Because if using exactly the same PRACH preamble or if UE reports repetition capability based on the similar RSRP configuration, gNB can not figure out whether Msg3 repetition or PUCCH repetition for Msg4 HARQ-ACK, or both are required by UEs.

Regarding the issue on whether the repetition factor indicated by UE can be supported, from our perspective, it is gNB’s responsibility to determine whether and how the UE performs PUCCH repetition for Msg4 HARQ-ACK after the UE requests repetition or reports capability. The gNB is also aware of the UL channel condition more clearly since paired spectrum is applied in the NTN system. Besides, UE indicating repetition factor may also needs more signalling to report different number of repetitions, the increased payload should also be avoided considering the NTN UE is more likely suffered from a bad channel condition. Thus, indication of repetition factor seems unnecessary for requesting PUCCH repetition for Msg4 HARQ-ACK.

- Proposal 2. For PUCCH repetition for Msg4 HARQ-ACK, UE can request repetition or report capability dynamically through Msg1 by using a dedicated PRACH preamble or RACH occasion.

- No further PRACH resource partitioning is needed for indication of repetition request or capability report

- Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings.

Considering using Msg3 PUSCH related signalling in Option B and Option C to indicate the information related to repetition, it is preferred the higher layer signalling in Msg3 PUSCH, e.g., the reserved bits in MAC sub-header, can be considered for requesting repetition or reporting capability, which is simpler and whether the related high layer signalling is available and the details can be further discussed in RAN2. For Option C, we prefer not to use physical layer signalling, such as the dedicated scrambling to Msg3 PUSCH or using a dedicated DMRS ports of Msg3 PUSCH, which will introduce network implementation complexity and significant standard effort.

- Proposal 3. For PUCCH repetition for Msg4 HARQ-ACK, higher layer signaling in Msg3 PUSCH for UE to indicate repetition request or capability report can be considered, the feasibility and detailed signalling can be discussed in RAN2.

### 2.1.3 Dynamic indication by gNB

It is supported that one or multiple repetition factors are configured via SIB, and PUCCH repetition for Msg4 HARQ-ACK can be determined and dynamically indicated by gNB. One key issue needs to be further discussed is how gNB indicates the information of PUCCH repetition, several options have been proposed in the last RAN1 meeting and the following alternatives for dynamic indication of repetition factor can be further discussed.

Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. - Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design)

Regarding Alt 1, DCI scheduling the Msg4 PDSCH could be used to indicate the repetition factor for PUCCH repetition for Msg4 HARQ-ACK. In the Rel-17 Msg3 repetition mechanism, part of the MCS bit field in RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI is repurposed for indicating Msg3 repetition factor. To avoid significant payload increasing and DCI format spec impact, similar design spirit could be reused, the 2 MSB of MCS field in the DCI format 1_0 scheduling Msg4 PDSCH can be re-interpreted to indicate repetition factor for Msg4 HARQ-ACK.

If UE requests Msg4 HARQ-ACK repetition in Msg1 by using a specific preamble can be supported, considering UL coverage limitation happens in both Msg3 PUSCH and Msg4 HARQ-ACK, a joint indication of repetition factor by the reinterpreted bits of MCS field in DCI scheduling Msg3 PUSCH can be used. Furthermore, as the payload of PUSCH carrying Msg3 is usually larger than that of Msg4 HARQ-ACK，the repetition factor of Msg4 HARQ-ACK can be same or the fraction of the Msg3 repetition factor by pre-defined mapping rule.

Proposal 4. For PUCCH repetition for Msg4 HARQ-ACK, the following alternatives for dynamic indication of repetition factor from gNB can be considered.

- Alt 1: Reinterpret the 2 MSB of MCS field in DCI format 1_0 scheduling the Msg4 PDSCH to indicate PUCCH repetition factor.

- Alt 2: A joint indication for both Msg3 and Msg4 HARQ-ACK repetitions in DCI format 0_0 can be configured by pre-defined mapping rule.

Another issue on PUCCH repetition for Msg4 HARQ-ACK is the configuration/indication of the repetition factor. The repetition factor 2, 4, 8 are acceptable according to the conclusion achieved [4], and whether/how single PUCCH transmission can be configured and/or indicated needs to be further discussed.

RAN1#110bis-e Agreement For PUCCH transmission for Msg4 HARQ-ACK, supported number of transmissions are 1, 2, 4, 8. - Note: single PUCCH transmission is performed as in the existing specification, and/or (if supported for single PUCCH transmission) according to configuration/indication e.g., in signaling with respect to number of transmissions. - FFS: whether larger number of transmissions is supported - FFS: whether/how single PUCCH transmission can be configured and/or indicated

From our perspective, the same issue has been discussed in Rel-17 Msg3 repetition enhancement, and this issue is also related to the indicator mechanism. Depending on whether the indication bit field is limited, repetition factor 1 and larger number more than 8 may be supported in different ways. Considering the procedure that one or more repetition factors may be configured via SIB, if only one repetition is configured in SIB information, repetition factor 1 could be achieved by not configuring the repetition configuration field. If multiple repetition factors are configured via SIB, after UE requests repetition/indicates repetition capability, it may assume that the repurposed bit field/ new bit field would be given. In this case, the single PUCCH transmission should be indicated by the same way as repetition factor does. If larger number of repetition factor need to be supported, the payload of configuration and indication should be studied, because this is the coverage-limited scenario.

Proposal 5. Discussion about the indication of single PUCCH transmission and larger number of repetitions should be deprioritized until the procedure to perform repetitions is confirmed.

  - 2.2 DMRS bundling

It is important to study and clarify the necessity as well as issues to apply DMRS bundling in NTN scenarios, especially for fast time-variant channel. The observation and conclusions of PUSCH DMRS bundling during previous RAN1 meetings are shown as below.

RAN1#110bis-e Agreement For NTN-specific PUSCH DMRS bundling, - Discuss further the need of enhancement in consideration of at least the following: - Phase difference due to timing drift and/or doppler shift. - e.g., whether/how long a UE can meet phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133, whether RAN1 should introduce enhancement to meet the requirement and/or recommend RAN4 to update the requirement or UE should pre-compensate phase difference by UE implementation, etc. - An event which causes power consistency and phase continuity not to be maintained. - e.g., whether the new event is necessary to determine actual TDW(s) from each nominal TDW or the existing specification can work without any specification change or whether such event may not occur depending on implementations, etc. - Note: baseline performance for legacy UEs can include antenna switching RAN1#111 Conclusion For the study of NTN-specific PUSCH DMRS bundling, RAN1’s understanding is that Phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1. RAN1#111 Conclusion RAN1 concluded that PUSCH DMRS bundling with sufficient TDW size should be applicable in NTN to meet the performance requirement for VoIP - FFS: How to determine TDW size, including UE capability. - Note: The above does not mean the performance requirements will be satisfied with DMRS bundling RAN1#112 Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit

Considering NTN is a FDD system which has consecutive slots, more opportunities can be expected for DMRS bundling. The main challenge for DMRS bundling in NTN scenarios is whether the channel correlation can be ensured between slots for fast time-variant channel with large time and frequency shift, which need more evaluation and discussion.

According to the timing error requirements specified in TS 38.133, the maximum timing error boundary that the UE is required to adjust its timing is about 29*64*Tc ≈ 0.928 us. Taking the worst case when the satellite moves towards/away from the UE at 10⁰ elevation angle and a maximum 93 us/s delay variation is assumed based on TR 38.821 as an example, only about 10 ms continuous transmission time can be maintained. The observation on timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed was also concluded in the last RAN1 meeting. Therefore, the TDW size need to be reconsidered by taking NTN-specific requirements into consideration.

Observation 1. 	Considering the large timing drift variation with the timing error requirement that up to 29Ts, about 10 ms continuous transmission time can be maintained in the LEO-1200 satellite scenarios.

Proposal 6. For the enhancement of PUSCH DMRS bundling, it is necessary to take timing drift into consideration when determine the TDW size in NTN scenarios.

It is recommended that PUSCH DMRS bundling with sufficient TDW size should be applicable in NTN to meet the performance requirement for VoIP. One main issue is how to handle the impact on DMRS bundling in NTN LEO scenarios considering the time and frequency pre-compensation, since the phase discontinuity due to applying TA pre-compensation may be very significant for large UL bandwidth.

The issue on how to determine the actual TDW for NTN-specific PUSCH DMRS bundling has been discussed during previous meetings, and no consensus on whether information related to TA pre-compensation timing reported by UE is necessary or not, the following options proposed in the last meeting needs to be further discussed [6].

Proposal 2-3_v5 For NTN-specific PUSCH DMRS bundling, further discuss whether nominal/actual TDW determination defined in R17 can be reused for NTN or not, e.g., the following options. - Option 1: Actual TDW determination based on timing of TA pre-compensation update causing phase discontinuity and power inconsistency. UE determines the timing. - Nominal TDW is configured by gNB as in the existing spec. - UE reports information on the timing. FFS details - Actual TDW is determined by the reported information and events defined in R17. - New event is defined based on TA pre-compensation update. FFS details - Option 2: Nominal/actual TDW determination based on gNB configuration and/or indication - Option 2-1: Determined as in the existing spec. No spec change is assumed. - Nominal TDW is configured by gNB as in the existing spec. - Note: Based on gNB implementation, nominal TDW length can be configured to current timing drift, derived from satellite ephemeris, to allow UE to update time/frequency pre-compensation sufficiently often. - Actual TDW is determined by events defined in R17. - Note: no new event is defined for the TA pre-compensation update. - Option 2-2: Determined by new gNB configuration/indication. - FFS details - e.g., TA pre-compensation timing is indicated by gNB - e.g., further segmented configuration is provided by gNB - FFS: Whether new event is defined based on new gNB configuration/indication. - FFS: whether to define another new event to decide actual TDW (e.g., antenna switching, update of epoch time, etc.)

For Option 1, the actual TDW is decided based on the timing of TA pre-compensation update causing phase discontinuity and power inconsistency, it can be regarded as a event which cause power consistency and phase continuity not to be maintained when the UE changes its time and/or frequency pre-compensation. For Option 2, actual/nominal TDW is determined based on gNB configuration. The difference between Option 2-1 and Option 2-2 is no specification impact on Option 2-1, but segmentation for pre-compensation needs to be further considered for determining actual TDW in Option 2-2.

From our understanding, a UE self-estimated TA to pre-compensate for the service link delay, which is calculated based on the UE position and the serving satellite ephemeris, may be transparent for gNB. In this case, gNB and UE may have misunderstanding about the actual TDW. Therefore, we prefer Option 1, information reporting about pre-compensation timing by UE is necessary to help gNB understand when UE perform pre-compensation and then determine the actual TDWs accordingly.

Otherwise, as in Option 2, if actual/nominal TDW is determined as in the existing specification or by taking segmentation into consideration, UE will be limited to only change its self-estimated TA between two actual TDWs, which may lead to a late pre-compensation and intolerable phase difference for DMRS bundling.

Proposal 7. For NTN-specific PUSCH DMRS bundling, nominal TDW is configured by gNB as in the existing specification, actual TDW can be determined based on reported information regarding when UE performs pre-compensation update causing phase discontinuity and power inconsistency.

- Conclusions

In this contribution, we share our views on some issues on coverage enhancement to NTN scenarios. The observation and proposals are summarized as follows:

- Proposal 1. For PUCCH repetition for Msg4 HARQ-ACK,

- A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB.

  - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold.

  - If the RSRP threshold is not configured, or if the configured RSRP threshold is X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK.

  - The value of X (the maximum configurable value of the RSRP threshold) is the infinity value, e.g., RSRP-Range = 127.

  - For PUCCH repetition request for Msg4 HARQ-ACK, different RSRP threshold from R17 Msg3 repetition can be configured.

- Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report.

- Proposal 2. For PUCCH repetition for Msg4 HARQ-ACK, UE can request repetition or report capability dynamically through Msg1 by using a dedicated PRACH preamble or RACH occasion.

- No further PRACH resource partitioning is needed for indication of repetition request or capability report.

- Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings.

- Proposal 3. For PUCCH repetition for Msg4 HARQ-ACK, higher layer signaling in Msg3 PUSCH for UE to indicate repetition request or capability report can be considered, the feasibility and detailed signalling can be discussed in RAN2.

Proposal 4. For PUCCH repetition for Msg4 HARQ-ACK, the following alternatives for dynamic indication of repetition factor from gNB can be considered.

- Alt 1: Reinterpret the 2 MSB of MCS field in DCI format 1_0 scheduling the Msg4 PDSCH to indicate PUCCH repetition factor.

- Alt 2: A joint indication for both Msg3 and Msg4 HARQ-ACK repetitions in DCI format 0_0 can be configured by pre-defined mapping rule.

Proposal 5. Discussion about the indication of single PUCCH transmission and larger number of repetitions should be deprioritized until the procedure to perform repetitions is confirmed.

Proposal 6. For the enhancement of PUSCH DMRS bundling, it is necessary to take timing drift into consideration when determine the TDW size in NTN scenarios.

Proposal 7. For NTN-specific PUSCH DMRS bundling, nominal TDW is configured by gNB as in the existing specification, actual TDW can be determined based on reported information regarding when UE performs pre-compensation update causing phase discontinuity and power inconsistency.

- References

- RP-230809, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, RAN#99.

- Chairman's Notes RAN1#112.

- Chairman's Notes RAN1#111.

- Chairman's Notes RAN1#110bis-e.

- TS 38.133, Requirements for support of radio resource management.

- R1-2302230, Summary EOM on 9.11.1 Coverage enhancement for NR NTN, Athens, Greece, February 27th – March 3rd, 2023